Input paper:   ENG17-3.1.2.10

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					□  Input

□  ENAV	□  VTS							X  Information

Agenda item 			3.1

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			Manuel López / European Union Space Programme Agency (EUSPA)

Pedro Pintor/GNSS Service Center

Emilio Gonzalez/GNSS Service Center

Jan Safar / GLA R&D (GRAD)

Ronan Boyle / Irish Lights

# The Galileo High Accuracy Service: Maritime Testing Campaign 2023

# Summary

This paper introduces the Galileo High Accuracy Service (HAS), which was declared operational in January 2023. As one of the services offered by Galileo, HAS is an open-access and free-of-charge service.

The Galileo HAS provides precise corrections (orbit, clock, biases) transmitted via the Galileo E6 signal (E6-B, data component [1]) from a subset of Galileo satellites as well as through the internet, allowing users of both Galileo and GPS to achieve improved positioning performance. Users require only one correction channel.

The precise corrections provided by the Galileo HAS allow users to reduce errors associated with the orbit and clock information provided through the Galileo Open Service [2] broadcast navigation messages and the GPS Standard Positioning Service [3] navigation data. These corrections, along with the biases provided by HAS, enable the computation of a high-accuracy Position Velocity and Time (PVT) solution in real-time.

Galileo HAS implementation proceeds in three phases: testing and experimentation; an initial Galileo HAS for a reduced number of signals, with relaxed performance targets and a reduced coverage area (currently ongoing); and a full Galileo HAS, which will offer two service levels and global coverage. Service Level 1 will provide global corrections, achieving 20 cm / 40 cm (95%) horizontal/vertical precision with < 300 s convergence time, while Service Level 2 will additionally offer atmospheric corrections for Europe, reducing the convergence time to 100 s.

The Galileo HAS is under responsibility of the European Union Agency for the Space Program (EUSPA) as Galileo Service Provider.

The Galileo HAS service reference documentation is available in the GNSS Service Centre website (https://www.gsc-europa.eu/).

With the Galileo High Accuracy Service (HAS), Galileo pioneers a worldwide, free high-accuracy positioning service aimed at applications that require higher performance than that offered by the Galileo Open Service and GPS Standard Positioning Service.  A very first testing campaign has been conducted at sea with a HAS-enabled receiver in collaboration with Irish Lights and the GLA R&D Directorate (GRAD), yielding promising results for maritime operations.

## Purpose of the document

The purpose of this document is to provide information to the IALA ENG Committee WG3 Radionavigation Services regarding the Galileo HAS and the first tests conducted at sea.

## Related documents

No related documents.

# Background

In 2016, Galileo joined other GNSS as part of the World Wide Radio Navigation System (WWRNS). The IMO’s Maritime Safety Committee (MSC), during its 96th session (11-20 May 2016), recognised the Galileo Open and Search and Rescue (SAR) services as part of the WWRNS.

While GNSS technology supports all phases of navigation and is an integral component of AIS and, consequently, VTS systems, standalone GNSS accuracy is often insufficient for specific operations. Multiple technologies have been developed to achieve regional and local accuracy improvements, such as Differential GNSS (DGNSS), Satellite Based Augmentation Service (SBAS) and Real Time Kinematic (RTK).

To meet the global demand for high-accuracy positioning, the European Commission proposed the provisioning of the Galileo HAS. Galileo is the first constellation capable of providing a worldwide high-accuracy service directly through the Signal in Space (SIS) and Internet Data Distribution (IDD).

# Discussion

This paper provides an overview of the Galileo HAS, covering features such as service levels, target performance, the roadmap and documentation. In addition, it presents the results of the first maritime testing campaign using Galileo HAS.

# References

Galileo E6-B/C Codes Technical Note, Issue 1.0, January 2019.

Galileo Open Service – Service Definition Document (OS-SDD) Issue 1.2, November 2021.

Global Positioning System Standard Positioning Service Performance Standard, 5th Edition, April 2020.

Galileo High Accuracy Service Service Definition Document (HAS SDD), Issue 1.0. January 2023.

Galileo High Accuracy Service (HAS) Info Note, Issue 1.0. 2020.

Galileo High Accuracy Service Signal-In-Space Interface Control Document (HAS SIS ICD), Issue 1.0. May 2022.

Galileo High Accuracy Service Internet Data Distribution Interface Control Document (HAS IDD ICD), Issue 1.0. January 2023.

# Action requested of the Committee

The Committee is requested to:

- Note the information within this paper.

- Annex

This Annex presents the Galileo HAS and the performance results obtained in a maritime campaign in 2023.

- Galileo HIGH ACCURACY service

This annex presents the Galileo High Accuracy Service (HAS) declared operational in January 2023. As one of the services offered by Galileo, the Galileo HAS is an open access and free of charge service.

The Galileo HAS is based on the provision of precise corrections (orbit, clock, biases) transmitted in the Galileo E6 signal (E6-B, data component [1]) from a subset of the Galileo satellites as well as via the internet, allowing Galileo and GPS users to achieve improved positioning performance. Only one correction channel is required by users.

The precise corrections provided by the Galileo HAS allow users to reduce the error associated with the orbit and clocks provided through the Galileo Open Service [2] broadcast navigation messages and the GPS Standard Positioning Service [3] navigation data. These corrections, together with the biases provided by the service and when processed by an appropriate algorithm, enable the computation of a High Accuracy Position Velocity and Time (PVT) solution in real-time.

  - Galileo HAS roadmap

It is planned that the implementation of the Galileo High Accuracy Service is staggered, based on the three phases depicted in Figure 1. The main elements and objectives of each phase are as follows [5]:

- Phase 0 (HAS testing and experimentation). Activities aimed at validating Galileo’s dissemination capabilities through the E6B channel and performing initial high-accuracy testing. Internal testing is ongoing since 2019. HAS SiS tests were executed from Q1 2021.

- Phase 1 (HA Initial Service) declared in January 2023. Provision of an initial Galileo High Accuracy Service resulting from the implementation of a high-accuracy data generation system processing Galileo system data only. The HA initial service delivers Service Level 1 only with a reduced performance (below the full service’s targets).

- Phase 2 (HA Full Service). Full provision of the Galileo High Accuracy Service, Service Level 1 and Service Level 2, fulfilling its target performance (e.g. 20 cm positioning performance). Phase 2 will include global coverage, better accuracy, ionospheric corrections in Europe, authentication and error characterization.

At the time of writing, September 2023, the Galileo HAS is in its Phase 1 (HA Initial Service).

Figure 1 Galileo HAS Service roadmap [5]

  - Galileo HAS characterization

The HAS comprises two service levels for global and regional coverage [5]:

- Service Level 1 (SL1): with global coverage; providing high accuracy corrections (orbits, clocks) and biases (code and phase) for Galileo E1/E5b/E5a/E6 and E5AltBOC and GPS L1/L5/L2 signals.

- Service Level 2 (SL2): with regional coverage; providing SL1 corrections plus atmospheric (at least ionospheric) corrections and potential additional biases.

Table 1 summarises the HAS full service characteristics and main targets for both service levels. Note that the Galileo HAS Initial Service currently provides a reduced performance level, as described in Annex 1.1.

Table 1: Main HAS characteristics and target performances [5]

  - Galileo HAS service area

The Galileo HAS service area is defined as the geographical area in which the Galileo HAS users can expect the fulfilment of minimum performance levels and typical positioning performances specified in the Galileo HAS SDD [4].

Figure 2 Galileo HAS Service area [4]

  - Galileo HAS relevant documentation and interfaces available

Detailed documentation for Galileo HAS is available on the GNSS Service Centre website: https://www.gsc-europa.eu/electronic-library/programme-reference-documents. This includes:

- Galileo HAS Service Definition Document (HAS SDD) [4].

- Galileo HAS Info Note [5].

- Galileo HAS Signal-In-Space Interface Control Document (HAS SIS ICD) [6].

- Galileo HAS Internet Data Distribution Interface Control Document (HAS IDD ICD) [7].

For general information about Galileo, you can visit:

- GNSS Service Centre website, https://www.gsc-europa.eu/

- Galileo helpdesk, https://www.gsc-europa.eu/helpdesk/

- Galileo HAS MARITIME CAMPAIGN

The objective of this campaign was to assess the accuracy obtained by using the Galileo HAS corrections over E6-B and the Galileo HAS User Terminal on an operational vessel.

  - Vessel and test equipment

The vessel used for this campaign was the Irish Lights Vessel (ILV) Granuaile (IMO: 9192947), as shown in Figure 3. It is a Buoy-Laying Vessel that was built in 2000 and is sailing under the flag of Ireland. Her carrying capacity is 1132 t DWT and her current draught is reported to be 5 meters. Her length overall (LOA) is 79.63 meters and her width is 16.08 meters. The vessel is property of the Commissioners of Irish Lights.

Figure 3 ILV Granuaile

The ILV Granuaile was already equipped with experimental GLA equipment, which included an E6 capable GNSS antenna (Figure 4), a TNC cable running from the antenna to the equipment room, a GNSS amplifier to compensate for cable losses, a cellular router for internet connectivity over LTE/5G and an Ethernet switch to connect devices to the cellular router. Spaceopal installed a Galileo HAS User Terminal (Figure 4) along with a laptop used for configuring and downloading data from the User Terminal and uploading data to the Spaceopal FTP server and other minor equipment (a power strip, cables, etc…).

Figure 4 Antenna installation (left) and Galileo HAS User Terminal (right)

The User Terminal is the first Galileo HAS receiver. It hosts a real-time Galileo HAS PPP algorithm, working in dual- and triple-frequency Galileo-only and Galileo and GPS modes, with corrections obtained from either the E6-B signal or the internet. In 2021, Spaceopal and partners developed the first HAS receiver in only 12 months for EUSPA to support HAS initial service deployment.

  - Trajectory

Figure 5 presents the position of ILV Granuaile at 0.00h UTC every day throughout the campaign, which ran from July 19th to August 16th (days 1-29), and the route followed by ILV Granuaile during this period.

Figure 5 Position of ILV Granuaile at 0.00h UTC every day and the route followed

The User Terminal was activated on July 19th in Dun Laoghaire and turned off in Killibegs on August 17th. Spaceopal monitored the User Terminal operations daily and uploaded data through the internet connection. Initially, the User Terminal was configured to operate in Galileo E1/E5a + GPS L1/L2C mode. On August 3rd, the operating mode was changed to Galileo E1/E5a/E6-B + GPS L1/L2C, and the User Terminal was restarted at 15.00UTC on that day.

  - Reference trajectory

The vessel’s reference trajectory was calculated using NovAtel GrafNav post-processing software (version 8.90). GrafNav was configured in kinematic differential mode, and data was processed in both forward and backward directions. The software was fed with raw GNSS observables from the Galileo HAS User Terminal and selected reference stations within the HxGN SmartNet network. Typical baseline lengths were suboptimal, averaging around 40 km but occasionally extending up to 65 km. The GrafNav processing was performed by GRAD.

  - RF environment

As a cargo vessel with a gross tonnage exceeding 300, ILV Granuaile is required to carry Long Range Identification and Tracking (LRIT) equipment. The LRIT system enables global ship identification and tracking, enhancing shipping security and contributing to safety and marine environment protection.

Shipborne LRIT equipment transmits within the 1600 MHz to 1630 MHz frequency band, which is close to the GPS/Galileo L1/E1 band (centred at 1575.42MHz), and it operates at a power level exceeding 1.5 W. These transmissions occur at least every six hours.

Every day at approximately 0.50 UTC, 6.50 UTC, 12.50 UTC and 18.50 UTC, the User Terminal observations were interrupted for a few seconds with a similar pattern.

The LRIT antenna location in relation to the GNSS antenna used in the campaign is 2.5 m below the GNSS antenna. The recommended distance between an LRIT antenna and any GNSS antenna is 30 m. While it is not confirmed that LRIT is responsible for the interference events, there is a high likelihood that it is the cause. In Figure 6, the reader can observe an example of the position degradation occurring every 6 hours.

Figure 6: Real-time User Terminal accuracy vs time using Galileo HAS corrections for July 26th 2023. Periodic degradation occurs every 6 hours.

The impact of these events on overall performance is significant, occurring every 6 hours and resulting in a notable degradation of position accuracy for an undetermined duration. Performance is affected not only during the event but also after it has finished, primarily because the selected positioning technique for Galileo HAS, Precise Point Positioning, uses a Kalman filter. These events also impacted the calculation of the reference trajectory using GrafNav.

  - Performance assessment

Table 2 presents results for the campaign. The User Terminal was initially configured to operate in Galileo E1/E5a + GPS L1/L2C mode, starting from July 19th. On August 3rd, the operating mode was changed to Galileo E1/E5a/E6-B + GPS L1/L2C. These two days have been excluded from the results to mitigate the impact of the convergence process. The results have not been modified to remove any possible external LRIT RF interference effects.

Table 2: Position accuracy using Galileo HAS

| Campaign day | Date | Granuaile location at 0.00 UTC | Horizontal 68% | Vertical 68% | Horizontal 95% | Vertical 95% |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 20/07/2023 | Dun Laoghire | 0.13 | 0.14 | 0.2 | 0.28 |
| 3 | 21/07/2023 | Dun Laoghire | 0.16 | 0.13 | 0.30 | 0.32 |
| 4 | 22/07/2023 | Kilmore Quay | 0.12 | 0.19 | 0.26 | 0.44 |
| 5 | 23/07/2023 | Bere Island | 0.09 | 0.12 | 0.19 | 0.26 |
| 6 | 24/07/2023 | Bere Island | 0.17 | 0.2 | 0.38 | 0.41 |
| 7 | 25/07/2023 | Ballingskelig Bay | 0.18 | 0.28 | 0.31 | 0.60 |
| 8 | 26/07/2023 | Mouth of Shannon estuary | 0.14 | 0.19 | 0.22 | 0.40 |
| 9 | 27/07/2023 | Aran Islands | 0.13 | 0.16 | 0.29 | 0.38 |
| 10 | 28/07/2023 | Aran Islands | 0.15 | 0.26 | 0.30 | 0.49 |
| 11 | 29/07/2023 | Aran Islands | 0.15 | 0.23 | 0.25 | 0.50 |
| 12 | 30/07/2023 | Aran Islands | 0.12 | 0.20 | 0.21 | 0.33 |
| 13 | 31/07/2023 | Clew Bay | 0.16 | 0.20 | 0.38 | 0.65 |
| 14 | 01/08/2023 | Clew Bay | 0.15 | 0.21 | 0.26 | 0.46 |
| 15 | 02/08/2023 | Clew Bay | 0.18 | 0.15 | 0.58 | 0.43 |
| 17 | 04/08/2023 | Mouth of Shannon Estuary | 0.14 | 0.28 | 0.27 | 0.5 |
| 18 | 05/08/2023 | Foynes | 0.12 | 0.20 | 0.21 | 0.29 |
| 19 | 06/08/2023 | Mouth of Shannon Estuary | 0.19 | 0.18 | 0.31 | 0.43 |
| 20 | 07/08/2023 | Aran Islands | 0.13 | 0.14 | 0.18 | 0.39 |
| 21 | 08/08/2023 | Aran Islands | 0.16 | 0.16 | 0.28 | 0.38 |
| 22 | 09/08/2023 | Inishkea Islands | 0.13 | 0.16 | 0.22 | 0.38 |
| 23 | 10/08/2023 | Broadhaven bay | 0.13 | 0.18 | 0.26 | 0.43 |
| 24 | 11/08/2023 | Sligo | 0.16 | 0.20 | 0.39 | 0.5 |
| 25 | 12/08/2023 | Sligo | 0.18 | 0.18 | 0.29 | 0.45 |
| 26 | 13/08/2023 | Sligo | 0.18 | 0.22 | 0.52 | 0.51 |
| 27 | 14/08/2023 | Killibegs | 0.12 | 0.09 | 0.22 | 0.22 |
| 28 | 15/08/2023 | Killibegs | 0.13 | 0.11 | 0.24 | 0.21 |
| 29 | 16/08/2023 | Killibegs | 0.08 | 0.09 | 0.18 | 0.19 |
|  |  | All days | 0.14 | 0.17 | 0.29 | 0.42 |

Table 3 presents the position availability results from the campaign. Two sets of thresholds have been selected. The first set is 0.15 m horizontal and 0.20 m vertical from the Galileo HAS SDD [4], while the second set, 0.20 m horizontal and 0.40 m vertical, is from the Galileo HAS Info Note [5]. The results have not been modified to remove any possible external LRIT RF interference effects.

Table 3: Position availability using Galileo HAS

| Campaign day | Date | Granuaile location at 0.00 UTC | Position availability % H 0.2m V 0.4m | Position availability % H 0.15m V 0.20m |
| --- | --- | --- | --- | --- |
| 2 | 20/07/2023 | Dun Laoghire | 94 | 64 |
| 3 | 21/07/2023 | Dun Laoghire | 76 | 54 |
| 4 | 22/07/2023 | Kilmore Quay | 86 | 58 |
| 5 | 23/07/2023 | Bere Island | 96 | 85 |
| 6 | 24/07/2023 | Bere Island | 71 | 51 |
| 7 | 25/07/2023 | Ballingskelig Bay | 62 | 36 |
| 8 | 26/07/2023 | Mouth of Shannon estuary | 89 | 53 |
| 9 | 27/07/2023 | Aran Islands | 81 | 62 |
| 10 | 28/07/2023 | Aran Islands | 75 | 41 |
| 11 | 29/07/2023 | Aran Islands | 83 | 45 |
| 12 | 30/07/2023 | Aran Islands | 92 | 61 |
| 13 | 31/07/2023 | Clew Bay | 77 | 52 |
| 14 | 01/08/2023 | Clew Bay | 78 | 50 |
| 15 | 02/08/2023 | Clew Bay | 67 | 49 |
| 17 | 04/08/2023 | Mouth of Shannon Estuary | 76 | 39 |
| 18 | 05/08/2023 | Foynes | 94 | 72 |
| 19 | 06/08/2023 | Mouth of Shannon Estuary | 69 | 45 |
| 20 | 07/08/2023 | Aran Islands | 94 | 66 |
| 21 | 08/08/2023 | Aran Islands | 80 | 56 |
| 22 | 09/08/2023 | Inishkea Islands | 89 | 63 |
| 23 | 10/08/2023 | Broadhaven bay | 84 | 55 |
| 24 | 11/08/2023 | Sligo | 81 | 47 |
| 25 | 12/08/2023 | Sligo | 77 | 48 |
| 26 | 13/08/2023 | Sligo | 66 | 39 |
| 27 | 14/08/2023 | Killibegs | 91 | 73 |
| 28 | 15/08/2023 | Killibegs | 89 | 68 |
| 29 | 16/08/2023 | Killibegs | 97 | 89 |
|  |  | All days | 81 | 55 |

  - Summary of the test campaign

When comparing the reference trajectory to the real-time trajectory from the User Terminal, the results indicate a 68% horizontal accuracy of 0.14 m and a vertical accuracy of 0.17 m. The 95% accuracy values were 0.29 m horizontally and 0.42 m vertically.

In terms of position availability, 0.15 m horizontal and 0.20 m vertical position error thresholds were achieved 55% of the time, while 0.20 m horizontal and 0.40 m vertical thresholds were met for 81% of the time.

The overall performance, including the calculation of the reference trajectory, was significantly impacted by periodic interference occurring daily at 0.50 h, 6.50 h, 12.50 h and 18.50 h. This interference was likely generated by LRIT transmissions. LRIT broadcasts are comparatively strong (several watts), and the antenna was positioned just 2.5 m from the GNSS antenna location.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain