Agenda item	11

Task Number

Author(s)	Captain Jean-Charles CORNILLOU

GMDSS Report from Rapporteur

# Summary

The latest development on the modernization of GMDSS has been presented in the last report of the correspondence group on the review and modernization of GMDSS, chaired by Bob Markle (RTCM) on behalf of the USA, in a document referred NCSR 2/9. This document is still pending for there was few time for discussion on it during NCSR2, from 9 to 13 March 2015, because the working group in charge of communications and technical subjects was as usually always overloaded by work in comparison to the other working groups.

The main concerns in the modernization of GMDSS are actually the followings:

- How to change A3 and A4 sea areas to accommodate with other recognized mobile satellite systems as requested by MSC. Irridium is already under an audit process to check its compatibility with the GMDSS requirements.

- Asserting that distress and safety traffic from other satellite systems gets to RCCs.

- Governments’ reluctance to pay for duplicate satellite broadcasting MSI.

- How to sustain MF / HF services with the decline of MF / HF Public Coast Stations?

- The role of VDES in GMDSS?

- Next generation of MEOSAR satellites to support COSPAS-SARSAT EPIRB detection.

- There are few requirements concerning the shore side in the present GMDSS, despite the basic concept is that search and rescue authorities ashore will be rapidly first alerted to a distress incident so that they can assist in a co-ordinated SAR operation with the minimum delay.

During the same week of ENAV16, there is a CEPT (European Conference on Postal and Telecommunication Administrations) meeting in Clug-Napoca (Romania). Christian Rissone and some other members of ENAV participate to this meeting to prepare WRC15 in November this year. Main subjects for modernization of GMDSS which will be discussed during preparation and WRC15 will be VDES, Digital HF including NAVDAT HF and Review of DSC recommendation.