VTS18 / 8 / 3

Information Document from IEC TC80 WG13

Draft Annex A and B from IEC Draft Standard 62288

Harmonization of Terminology and Symbols

|  | Issue 5 For IEC use only 2003-02-20 |
| --- | --- |

INTERNATIONAL ELECTROTECHNICAL COMMISSION

TECHNICAL COMMITTEE No 80: Maritime navigation and radiocommunication equipment and systems

WORKING GROUP No 13: Presentation of navigation related information

Working Paper for the meeting

to be held at the Bundesamt Für Seeschiffahrt und Hydrographie (BSH) in Hamburg, Germany

from 18th to 20th February 2003.

_________________________

Title:	Draft Input Paper to NAV 49 accompanying the Draft IMO PS

Author:	Lee Alexander, Dave Blevins, Mathias Jonas

SUB-COMMITTEE ON SAFETY OF	NAV 49/?/?

NAVIGATION	19 February 2003

49th session	Original: ENGLISH

Agenda item 4

Requirements for the display and use of AIS information on shipborne navigational displays

Harmonization of Terms and Symbols used to Present Navigation Related Information

Submitted by the International Electrotechnical Commission (IEC)

SUMMARY

Executive summary:	Report on progress within the IEC on standards for the presentation of navigational information.

Action to be taken:	Paragraph 6

Related documents:	Report of the forty-eighth session of the Sub-Committee on Safety of Navigation (NAV 48/19) paragraphs 4.23 through, 4.26

Reference the other paper here

1.	The Sub-Committee on Safety of Navigation at its forty-seventh session invited IEC to set up a Working Group to develop a standard for the presentation of navigational information, being of the opinion that this standard should harmonize the following:

.1	display and interaction objects;

.2	multifunction displays;

.3	co-location, merging, processing, fusion of graphical information; and

.4	indication of quantity, status, integrity and accuracy of information.

2.	This paper reports on the progress made to harmonise terms and symbols, and other presentation characteristics which should be used uniformly by all navigation equipment installed on the bridge of a ship.

3.	Attached are draft annexes to the proposed IEC standard 62288:

Annex A 	Terms and numerical quantities presented on the display

Annex B	Symbols presented on the display

4.	These annexes are a compilation and harmonisation of presentation related objects, and support the draft recommendation on performance standards for the Presentation of Navigation Related Information.

5.	The approach taken to harmonize the symbols has been to:

- base the work on existing symbology currently in use

- make minimum changes to existing symbols

- modify or introduce additional symbols only when needed

- organize the symbols into logical groups with a common base symbol

- keep the total number of different symbols as small as possible

- use as few colours as possible (most symbols should be useable in B&W displays)

- make use of colours specified by IHO for ECDIS

- design coding by shape or outline rather than colour alone

- strive for consistency between symbol types

# Action requested of the Sub-Committee

6.	The Sub-Committee is invited to comment on the IEC compilation and provide any guidance as appropriate.

|  | Issue 1 For IEC use only 2003-02-20 |
| --- | --- |

INTERNATIONAL ELECTROTECHNICAL COMMISSION

TECHNICAL COMMITTEE No 80: Maritime navigation and radiocommunication equipment and systems

WORKING GROUP No 13: Presentation of navigation related information

Working Paper for the meeting

held at the Bundesamt Für Seeschiffahrt und Hydrographie (BSH) in Hamburg, Germany

from 18th to 20th February 2003.

_________________________

Title:	62288 Draft Annex A

Author:	Hans Ramsvik, Simon Cooke

- (normative)
Terms and numerical quantities
presented on the display

  - Standard Terms and Abbreviations

    - Code of Practice

Navigation related information shall be presented using the standard terms or abbreviations defined in clause  or symbol defined in Annex B.

When a standard abbreviation is not defined, the standard term shall not be abbreviated. When the meaning of the standard abbreviation is not clear from its context, the standard term shall not be abbreviated. Standard terms and abbreviations may be combined e.g., “CPA LIM”.

Where a standard term is not defined another term or abbreviation may be used providing it does not conflict with the standard terms or abbreviations, and providing the meaning is clear. Standard marine terminology shall be used for this purpose.

Unless otherwise specified in clause  the standard terms and abbreviations shall be presented using upper or lower case. When the standard abbreviation is an acronym (formed from the first letters of each word in the standard term) it shall be presented in upper case.

The User Manual or Quick Reference Guide shall include a list of all abbreviations used together with the terms to which they relate.

List of Standard Terms and Abbreviations

Table A.1 - Standard Terms and Abbreviations

| Term | Abbreviation |  | Term | Abbreviation |
| --- | --- | --- | --- | --- |
| ACKNOWLEDGE | ACK |  | AUTOMATIC RADAR PLOTTING AID | ARPA |
| ACQUIRE | ACQ |  | AUTOMATIC TRACKING AID | ATA |
| ACQUISITION ZONE | AZ |  | AUTOPILOT | AP |
| ADJUST | ADJ |  | AVAILABLE | AVAIL |
| AFT |  |  | AZIMUTH INDICATOR | AZI |
| ALARM | ALM |  | BACKGROUND | BKGND |
| ANCHOR WATCH | ANCH |  | BEARING | BRG |
| ANTENNA | ANT |  | BEARING WAYPOINT TO WAYPOINT | BWW |
| APRIL | APR |  | BOW CROSSING RANGE | BCR |
| ASTRONOMICAL | A |  | BOW CROSSING TIME | BCT |
| AUDIBLE | AUD |  | BUILT IN TEST EQUIPMENT | BITE |
| AUGUST | AUG |  | CALIBRATE | CAL |
| AUTOMATIC | AUTO |  | CARRIED | C |
| AUTOMATIC IDENTIFICATION SYSTEM | AIS |  | CANCEL | CNCL |
|  |  |  | CENTRAL PROCESSING UNIT | CPU |
| CHANGE | CHG |  | CENTRE | CENT |
| CIRCULARLY POLARISED | CP |  | DIFFERENTIAL GNSS | DGNSS |
| CLEAR | CLR |  | DIFFERENTIAL GPS | DGPS |
| CLOSEST POINT OF APPROACH | CPA |  | DIGITAL SELECTIVE CALLING | DSC |
| COASTGUARD STATION | CG |  | DISPLAY | DISP |
| COMPACT DISC READ ONLY MEMORY | CDROM |  | DISPLAY BRILLIANCE | BRILL |
| COMPASS |  |  | DISTANCE | DIST |
| CONTRAST | CONTR |  | DISTANCE INTERVAL | DIST INT |
| COORDINATED UNIVERSAL TIME | UTC |  | DISTANCE ROOT MEAN SQUARED | DRMS |
| CORRECTION | CORR |  | DRIFT |  |
| COURSE | CSE |  | DROPPED | D |
| COURSE OVER THE GROUND | COG |  | EAST | E |
| COURSE TO STEER | CTS |  | ECHO REFERENCE | REF |
| COURSE UP | C UP |  | ECHO REFERENCE SPEED | REF SOG |
| CROSS TRACK DISTANCE | XTD |  | ELECTRONIC BEARING LINE | EBL |
| CURSOR | CURS |  | ELECTRONIC CHART DISPLAY AND INFORMATION SYSTEM | ECDIS |
| CURVED HEADING LINE | CHL |  | ELECTRONIC CHART SYSTEM | ECS |
| DATA |  |  | ELECTRONIC NAVIGATIONAL CHART | ENC |
| DATE |  |  | ELECTRONIC PLOTTING AID | EPA |
| DAY/NIGHT | DAY/NT |  | ELECTRONIC POSITION FIXING SYSTEM | EPFS |
| DEAD RECKONING | DR |  | ELECTRONIC RANGE AND BEARING LINE | ERBL |
| DECCA | DError: Reference source not found |  | EMERGENCY POSITION INDICATING RADIO BEACON | EPIRB |
| DECEMBER | DEC |  | ENHANCE | ENH |
| DECREASE | DECR |  | ENTER | ENT |
| DEEP WATER | DWError: Reference source not found |  | EQUIPMENT | EQUIP |
| DEGAUSS |  |  | ERROR | ERR |
| DEGREESError: Reference source not found | DEG |  | ESTIMATED POSITION | EP |
| DELAY |  |  | ESTIMATED TIME OF ARRIVAL | ETA |
| DELETE | DEL |  | EUROPEAN GEO-STATIONARY NAVIGATIONAL OVERLAY SYSTEM | EGNOS |
| DEPTH | DPTH |  | EVENT |  |
| DESTINATION | DEST |  | EXCLUSION ZONE | EZ |
| Differential | dError: Reference source not found |  | ESTIMATED TIME OF DEPARTURE | ETD |
| DIFFERENTIAL GLONASS | DGLONASS |  | EXTERNAL | EXT |
| FEBRUARY | FEB |  | FATHOMS | F |
| FOREWARD | FWD |  | INPUT | IN |
| FULL |  |  | INPUT/OUTPUT | I/O |
| GAIN |  |  | INTEGRATED BRIDGE SYSTEM | IBS |
| GEOGRAPHICS | GEOG |  | INTEGRATED NAVIGATION SYSTEM | INS |
| GEOMETRIC DILUTION OF PRECISION | GDOP |  | INTEGRATED RADIO COMMUNICATION SYSTEM | IRCS |
| GLOBAL MARITIME DISTRESS AND SAFETY SYSTEM | GMDSS |  | INTERFERENCE REJECTOR | IR |
| GLOBAL NAVIGATION SATELLITE SYSTEM | GNSS |  | INTERSWITCH | ISW |
| GLOBAL POSITIONING SYSTEM | GPS |  | JANUARY | JAN |
| GLOBAL POSITIONING SYSTEM | GError: Reference source not found |  | JULY | JUL |
| GLOBAL ORBITING NAVIGATION SATELLITE SYSTEM | GLO or GLONASS |  | JUNE | JUN |
| GLOBAL ORBITING NAVIGATION SATELLITE SYSTEM | GlError: Reference source not found |  | KNOTS | KN |
| GREAT CIRCLE | GC |  | KILOMETER | kmError: Reference source not found |
| GRID |  |  | LABEL | LBL |
| GROUND STABILIZED | GND STAB |  | LATITUDE | LAT |
| GROUND TRACK | GND TRK |  | LATITUDE/LONGITUDE | L/L |
| GROUNDING AVOIDANCE SYSTEM | GAS |  | LEEWAY | LWY |
| GUARD ZONE | GZ |  | LIMIT | LIM |
| GYRO | GYRO |  | LINE OF POSITION | LOP |
| HEADING | HDG |  | LOG |  |
| HEADING LINE | HL |  | LONG PULSE | LP |
| HEADING LINE OFF | HL OFF |  | LONGITUDE | LON |
| HEAD UP | H UP |  | LORAN | LOR |
| HIGH SPEED CRAFT | HSC |  | LORAN/TCHAIKA | LError: Reference source not found |
| HORIZONTAL DILUTION OF PRECISION | HDOP |  | LOST TARGET | LOST TGT |
| HOURS | HR |  | MAGNETIC | MAG |
| IDENTIFICATION | ID |  | MAGNETIC VARIATION | MAG VAR |
| INCREASEError: Reference source not found | INCR |  | MAN OVERBOARD | MOB |
| INFORMATION | INFO |  | MANOEUVRE TIME | MVR TIME |
| INFORMATION | iError: Reference source not found |  | MANUAL | MAN |
| INFRARED | INF RED |  | MARCH | MAR |
| INITIALISATION | INIT |  | MAP LINES |  |
| MARKER | MKR |  | MAPS | MAP |
| MASTER | MSTR |  | PERSONAL IDENTIFICATION NUMBER | PIN |
| MAXIMUM | MAX |  | PORT |  |
| MAY |  |  | POSITION | POSN |
| MEDIUM PULSE | MP |  | POSITION APPROXIMATE | PAError: Reference source not found |
| MENU |  |  | POSITIONAL DILUTION OF PRECISION | PDOP |
| METERS | MError: Reference source not found |  | POWER | PWR |
| MFDF | MError: Reference source not found |  | PREDICTED AREA OF DANGER | PAD |
| MINIMUM | MIN |  | PREDICTED POINT OF COLLISION | PPC |
| MINUTESError: Reference source not found | MIN |  | PULSE LENGTH | PL |
| MISSING |  |  | PULSE REPETITION FREQUENCY | PRF |
| MUTE |  |  | PULSES PER REVOLUTION | PPR |
| NAUTICAL MILE | NM |  | RADAR | RDR |
| NAVIGATION | NAV |  | RADAR | RError: Reference source not found |
| NORMAL | NORM |  | RADAR PLOTTING | RP |
| NORTH | NError: Reference source not found |  | RADIUS | RAD |
| NORTH UP | N UP |  | RAIN |  |
| NOT LESS THAN | NLTError: Reference source not found |  | RANGE | RNG |
| NOT MORE THAN | NMTError: Reference source not found |  | RANGE RINGS | RR |
| NOT UNDER COMMAND | NUC |  | RASTER CHART DISPLAY SYSTEM | RCDS |
| NOVEMBER | NOV |  | RASTER NAVIGATIONAL CHART | RNC |
| OCTOBER | OCT |  | RATE OF TURN | ROT |
| OFF |  |  | REAL TIME KINEMATIC | RTK |
| OFF TRACK | OFF TRK |  | RECEIVER | RX |
| OFF CENTRE | OFF CENT |  | RECEIVER AUTONOMOUS INTEGRITY MONITORING | RAIM |
| OFFSET |  |  | RELATIVE | REL or R |
| ON |  |  | RELATIVE BEARING | R BRG |
| OUTPUT | OUT |  | RELATIVE COURSE | R CSE |
| OWN SHIP | OS |  | RELATIVE MOTION | RM |
| PANEL ILLUMINATION | PANEL or PANEL DIM |  | RELATIVE VECTOR | R VECT |
| PARALLEL INDEX LINE | PI |  | REVOLUTIONS PER MINUTE | RPM |
| PAST POSITIONS | PAST POSN |  | RHUMB LINE | RL |
| PERFORMANCE MONITOR | PM |  | RELATIVE MOTION (RELATIVE TRAILS) | RM (R) |
| PERMANENT | PERM |  | RELATIVE MOTION (TRUE TRAILS) | RM (T) |
| ROUTE | RTE |  | ROLL ON, ROLL OFFError: Reference source not found | RoRoError: Reference source not found |
| S BAND | SError: Reference source not found |  | TIME TO WHEEL OVER LINE | TWOL |
| SAFETY CONTOUR | SAF CON |  | TRACK | TRK |
| SCAN TO SCAN | SC/SC |  | TRACK CONTROL SYSTEM | TCS |
| SEA Term relates to sea clutter. |  |  | TRACKING | TRKG |
| SEARCH AND RESCUE | SAR |  | TRAILS |  |
| SECONDS | SEC or S |  | TRANSCEIVER | TX/RX |
| SELECT | SEL |  | TRANSFERRED POSITION LINE | TPL |
| SEPTEMBER | SEP |  | TRANSMIT | TX |
| SEQUENCE | SEQ |  | TRANSMITTER | TX |
| SET (Used in the context of “set and drift” or “setting” a value.) |  |  | TRANSPONDER | TPR |
| SHORT PULSE | SP |  | TRIAL MANOEUVRE | TRIAL |
| SIGNAL TO NOISE RATIO | SNR |  | TRIGGER PULSE | TRIG |
| SIGNAL STATION | SSError: Reference source not found |  | TRUE | T |
| SIMULATION | SIM |  | TRUE BEARING | T BRG |
| SLAVE |  |  | TRUE COURSE | T CSE |
| SOUTH | SError: Reference source not found |  | TRUE MOTION | TM |
| SPEED | SPD |  | TRUE SPEED | T SPD |
| SPEED AND DISTANCE MEASURING EQUIPMENT | SDME |  | TRUE VECTOR | T VECT |
| SPEED OVER THE GROUND | SOG |  | TUNE | TUNE |
| SPEED THROUGH THE WATER | STW |  | UNINTERRUPTIBLE POWER SUPPLY | UPS |
| STABILISED | STAB |  | UNIVERSAL TRANSVERSE MERCATOR | UTM |
| STANDBY | STBY |  | UNSTABILISED | UNSTAB |
| STARBOARD | STBD |  | VARIABLE RANGE MARKER | VRM |
| SYMBOLS OFF | SYM OFF |  | VECTOR | VECT |
| SYNCHRONISATION PULSE | SYNC |  | VECTOR TIME | VECT TIME |
| SYSTEM ELECTRONIC NAVIGATIONAL CHART | SENC |  | VESSEL TRAFFIC SERVICES | VTS |
| SYSTEM RASTER NAVIGATIONAL CHART | SRNC |  | VIDEO | VID |
| TARGET | TGT |  | VISUAL | VError: Reference source not found |
| TEST TARGET | TEST TGT |  | VISUAL DISPLAY UNIT | VDU |
| TIME |  |  | VOYAGE | VOY |
| TIME DILUTION OF PRECISION | TDOP |  | WAYPOINT | WPT |
| TIME TO CLOSEST POINT OF APPROACH | TCPA |  | WAYPOINT CLOSURE VELOCITY | WCV |
| TIME TO GO | TTG |  | WEST | WError: Reference source not found |
| WHEEL OVER POINT | WOP |  | WHEEL OVER LINE | WOL |
| WORLD GEODETIC SYSTEM | WGS |  |  |  |
| X BAND | XError: Reference source not found |  |  |  |

Shall only be used in conjunction with the ECDIS position fix symbol (Symbol 6 of “Route monitoring and route planning symbols“ in IEC 61174). Shall be presented using the case indicated.

Only to be used for display of S52 chart symbols as defined in “Colour & Symbol Specifications For ECDIS, Special Publication No. 52 Appendix 2”.

Shall be presented using the abbreviation in the case indicated.

Alternatively a symbol may be used.

Shall be presented using the case indicated when used for display of S52 chart symbols as defined in “Colour & Symbol Specifications For ECDIS, Special Publication No. 52 Appendix 2”.

  - Display of Numerical Quantities

    - Decimal Separator

A decimal point shall be used as the decimal separator. Where a digit-grouping symbol (used for separating thousands or thousandths of a unit) is used, it shall be a space character.

    - Units

The units of measurement shall be presented where not otherwise obvious. Where the operator needs to compare different values, the same units shall be used for the presentation of these values.

Values shall be presented using either nautical units or the metric system.

    - Resolution

Values shall be presented to a resolution appropriate to that required by the operator. Values shall be presented to a resolution that is appropriate to the accuracy to which the value is known.

Own ship data shall be presented to the minimum resolutions defined in the table below if the value is known with sufficient accuracy.

Table A.2 - Minimum Resolution of Own Ship Data

Own Ship DataMinimum ResolutionSpeed 1/10 knotHeading 0.1 degreePosition 1/1000 of a minuteDepth0.1 meters for values below 10 meters

1 meters for values 10 meters and aboveCourse 0.1 degree

    - Angle

Angles shall be presented with three digits before the decimal separator. Angles shall be presented in the range 0 ≤ Θ < 360. Relative angles may alternatively be presented in the range –180 ≤ Θ < 180. The sign of relative angles may be indicated by the use of positive or negative numbers (i.e. positive for starboard, forward etc), standard terms or abbreviations (e.g. STBD and FWD), or, in the case of port and starboard, colour (red for port, green for starboard).

    - Time/Date

Time of day shall be presented using the 24-hour clock convention. The time zone shall be indicated if not UTC. The display of time of day shall be formatted HH:MM:SS or HH:MM. HH = hour, MM = minute, SS = second. Each field shall be presented using two digits using a zero for padding where required. E.g. “08:30:04” or “18:30”.

Date shall be presented using the format DD MMM YYYY. The day field (DD) shall be one or two digits and may be padded with a zero. The month field (MMM) shall use the standard term or the standard abbreviation for months. The year field (YYYY) shall be the four digits of the Gregorian calendar year. E.g. “6 JUNE 2004” or “05 Jan 2003”.

    - Latitude and Longitude

Where a position is expressed in Latitude and Longitude the format DDD MM.MMM’ shall be used. Where latitude and longitude are presented together, latitude shall be presented either above or to the left of longitude.

The latitude and longitude fields shall be preceded by the standard term or standard abbreviation indicating the sign of the value (e.g. N, S, E or W).

The degree field (DDD) shall be a positive integer representing the number of degrees in the range 0 to 90 for latitude, or 0 to 180 for longitude. The degree field may be padded with leading zeros to give two digits for latitude and three digits for longitude. The degree field shall be terminated by a degree symbol () or the standard abbreviation for degrees.

The minutes field (MM.MMM’) shall be a positive number representing the number of minutes. It shall be padded with leading zeros to give two digits before the decimal separator. It may be terminated by a minutes symbol (‘) or the standard abbreviation for minutes.

Where the datum of a position expressed in latitude and longitude is not WGS 84, the datum shall be indicated unless clearly evident from the context. The NIMA datum name, acronym or code shall be used for this purpose, as defined in the document National Imagery and Mapping Agency Department of Defense World Geodetic System 1984 (TR8350.2).