VTS18/output/04

# THE DISPLAY OF AIS AND OTHER INFORMATION IN VTS CENTRES

## Introduction

IALA Recommendation A-123 (Introduction of Shore Based Automatic Identification System Facilities) on the considerations relating to the provision of shore-based AIS notes that the provision of shore-based AIS will be necessary to attain the full benefit of the 1974 SOLAS Convention (as amended).  For seagoing vessels, there are many compelling reasons for the international standardisation of methods of display of electronic sensors, including AIS. At VTS Centres, however, there may be differing or additional requirements for the display of information that may be port, country or area specific.

## Aim

The aim of this paper is to provide guidelines for the display of AIS and other information in VTS Centres.

## Broad Policy

The display of on-board AIS defined in IHO S52/S57, IALA VTS Manual Ed. 2002, Art 4.4.3 as endorsed by IMO Resolution A.857(20) and being developed under IEC TC 80-WG13-06-WP16 should be used as the basis from which shore-based VTS requirements may be developed.  The standard AIS message identifies fixed fields and it will not be possible to exchange data between ships and VTS Centres that are not accommodated within the standard AIS binary message string.  VTS Centres may wish to adapt symbology for their own more detailed needs.  In deciding to modify symbology, VTS Centres should be guided by the following principles:

- Any change from on-board symbology should be carefully considered and justified.

- Symbology already identified for existing on-board use should not be assigned a different meaning.

## Factors affecting the use of AIS in a VTS Centre

The contribution of AIS to VTS and the overall traffic image should be kept in context; responsibility for maintenance of a good lookout and for proper navigation of a vessel will always remain with the master.  Whilst AIS data in the VTS context is likely to be fused with other sources, in some cases such as monitoring of coastal and inland waterways, AIS may be the sole source of positional data available. The degree of accuracy required may vary dependent on the use to which the AIS data is being put.  In assessing the degree of reliance that can be placed on the information displayed, it will be important to establish the need for an appropriate level of validation.

There is an expectation that AIS will dramatically reduce operator loading.  Whilst this may be an aspiration for the future, it is possible that there could even be an increase in loading in the short to medium term.  Potential benefits that may be realised at an early stage, however, could include:

- Greater confidence in accurate identification,

- Greater system resilience to avoid “lost targets” and “target swap”,

- Greater positional accuracy.

VTS Operators will need to be alert to the fact that:

- Not all vessels are required to fit AIS,

- In the period of AIS introduction, not all vessels may have a fully operational system,

- Only a Minimum Keyboard Display (MKD) is mandated on board vessels,

- Those with a working AIS may not be familiar with the entry of the variable voyage related data.

This leads to the conclusion that not only should the received data be treated with care, but VTS Operators (VTSOs) should not assume that AIS data is presented in an immediately available form to mariners at sea.  Although AIS may change the way VTSOs interact with ships, there is unlikely to be a significant reduction in the verbal exchange of VHF information in the short to medium term.  The concept of a “silent VTS” may be some way off.

In the development of AIS, Human Factors within a VTS should be fully assessed.  This paper is directed at the integration of AIS with the existing VTS traffic Image.  It is possible that a standalone presentation of AIS data may be considered as an interim measure, but this is not as desirable as integrated displays.  In assessing the practicality of such a solution, VTS Centres should carefully review the ability for operators to assimilate the information provided.  The presentation of AIS data has been highlighted throughout this paper.  It is vital that the presentation and ease of access to the increased data available be carefully addressed if the introduction of AIS is not to lead to increased loading on VTSOs.

## Detailed Considerations

In translating the practical considerations in the use of AIS into equipment specifications for local or national requirements, the following broad issues may need to be considered:

- Operational issues

- Geographical issues

- Environmental issues

- Commercial issues

- Training issues

Whilst the actual method of display and symbology used will be for individual nations and/or ports to decide, the following checklist is provided to assist VTS authorities in developing their requirements for the identification of specific attributes for which VTSO awareness may be aided through the provision of appropriate display equipment and symbology.  Not all of these are directly related to AIS, nor are the lists all-inclusive. Where symbology is linked to other charting standards, such as IHO, care may have to be taken to ensure that any symbols selected solely for use in VTS are compatible with those that have already been specified under other standards.

## Operational.

- Type of VTS provided – consideration may have to be given to the equipment provided and the detail displayed either in the basic symbol or accompanying data dependent on the type and complexity of service offered.

- Pilot on board – symbology may be used to indicate the status and requirements of vessels having a pilot onboard.

- Pilot Exemptions – symbology may be used to define the status and area of authorisation for individual Pilot Exemption Certificate (PEC) holders.

- Traffic density – consideration may have to be given to the level of detail and the area being covered in deciding the amount of AIS detail displayed directly on the screen or available through pop – up menus/data fields

- Size of vessel – restrictions dictated by the geography of the port or surveillance area may have to be considered in deciding whether additional attributes are required to the Symbology dependent on the size of vessels operating in the area or passing in or through a channel.

- Type of Vessel/ Vessel Characteristics – symbology may be used to provide a clear indication of specialist vessels or those for whom special operational considerations may be appropriate.  For example:

- Dangerous Cargoes/Goods – Ports may have their own regulations that may determine the requirement for the subdivision of vessel or cargo types through the colour or shape of the target image.

- Harbour Authority Vessels (harbour launch, pilot vessel, survey vessel, salvage etc)

- Restricted in ability to manoeuvre

- Constrained by draught

- Quality of Survey/Background Charting and Display - symbology may be used in the form of an overlay or area designation to indicate the accuracy of survey information being displayed on the background chart in use by VTS operators.

- Security – symbology could be used to provide visual cues to VTS operators in support of evolving security considerations.

- Non- standard targets – it may be appropriate to develop a symbol to indicate the existence of hazards such as floating or semi-submerged obstructions.

- Defects or Deficiencies – symbology can be utilised to indicate vessels with defects or deficiencies.

- Emergency situations – symbology may be utilised to indicate the status, duties or tasking of assets involved in emergency situations, such as Search and Rescue (SAR).

- Port State Control – identification of vessels of special interest or non-compliance under Port State Control may be indicated through symbology.

- ATON – the status of Aids to Navigation may be indicated through appropriate symbology.

- Size of symbols – the display of symbols should be dependent on range scale in use.  For example, it may be found more appropriate for AIS symbols to reduce proportionately to the range scale of display in use to reduce screen clutter.

## Geographical

- Port access – the number and design of approach channels should be taken into account when deciding display presentation and the size/scale of symbology used.

- Berth locations – adjacent berths may dictate the need to reduce scale/size and amount of AIS information displayed.

- Temporary Danger Areas – areas that temporarily become dangerous and should be avoided by ships may be displayed using appropriate symbology.

- Naval Exercises Areas – appropriate symbology may be required to indicate naval activities that may impact on the control of vessel traffic through the area.

- Recreational Areas – symbology may be required to indicate areas where recreational activities are taking place.

- Sites of Special Scientific Interest (SSSIs)/Fish farms/Ecological – symbology may be developed to indicate the existence of sites where environmental considerations may required some form of traffic restriction, regulation or control.

- Mobile sea bed/sandbanks – areas subject to changes in seabed structure may require to be identified (see “quality of survey” above)

- Oil fields/Oil and Energy Installations (OEIs)

- Location of pipelines – the existence and presence of pipelines may be indicated by Symbology if not already disseminated through chart amendments.

### Environmental

- Hydrological – tide, current

- Meteorological – wind, visibility, sea-state

- Ice – ports that experience the seasonal phenomenon of sea ice may require to develop symbology to identify the areas affected.

## Commercial.

- Prioritisation – symbology may be utilised to indicate movement priorities.

- Information sharing – a need to exchange/share the presentation of the traffic image with other parties or users may dictate the choice/development of specific symbology.

Training.

- Standardisation – impact of locally developed symbology on training and local examination/certification in the IALA V-103 process.

General Issues.

There are a number of general issues that may need to be taken into account when considering the presentation of information to VTS Operators.  These include, Data Filtering and Track Labelling, Correlation, and the presentation of Safety Related Messages.  These are addressed below.

Data Filtering and Track Labelling.  The AIS and associated binary message will permit the transmission of increased information associated with each and every AIS target.  VTS Centres should consider carefully the number and arrangement of VTS operator displays and how much information on individual tracks is displayed.  Whilst it may be valuable to have detailed information on-screen, equally it may tend to clutter the screen in ports with high densities of vessel traffic or where individual VTS operators may need to carry out surveillance over a large area.  Technical solutions that may include pop-up displays or other means of displaying the details of individual track may need to be considered.

Correlation.  Correlation between AIS and other sensor information needs to be considered.  Systems may be capable of automating the correlation process and it may be appropriate to make the VTSO aware of the source(s) of information being presented.  AIS signals may be lost and consideration should be given to the presentation to the operator of the currency of the information presented and any automatic change between sensors.

Safety Related Messages.  AIS specifies the provision of short safety related messages, and it is important that a method is identified to draw the attention of the VTS operator to the receipt of such a message.  Experience has yet to be gained on the use of this feature but there is no provision to limit its use by the mariner.  In considering how these messages may be displayed, a balance may need to be struck between drawing attention to the presence of a safety related message and the need not to overload or distract the VTS operator. The advantages and disadvantages of including safety related messages and alertment superimposed on the traffic image or on a separate display may also need to be considered.

Industry Expectation.

Industry has stated a clear need for a standard to be set for the exchange of data.  They have stated a preference for only broad guidance on the equipment presentation and symbology actually used and the ability to adapt these for individual requirements.  This will encourage the development of equipment to meet emerging technology.