VTS 18 / 9 / 1

Data Exchange between VTS Centres

(Norcontrol IT, Øyvind Taugl)

# Data exchange between VTS centres

## Introduction

There is an increasing interest in exchanging data between VTS centres. Even if several systems have been interfaced, there is no standard available covering the different types of data available in a VTS. There is neither a standard making it easier for the vendors to communicate. To make a standard describing data exchange between VTS centres, it is important to choose a way forward that will take into account the ever evolving need of new type of information to exchange. A static description will only lead to proprietary solutions needed to cater for different types of data. At the same time, it is necessary to make a foundation describing a minimum requirement so interfaces between different vendors will work together without further refinement.

## Types of data to exchange

As mentioned, there seems to be an ever increasing load of information, but let’s try to split the information.

### Meteorological and hydrological information

This information can vary quite a lot according to the different types of sensors and information. To have a standard for exchange of data might be difficult, but there exists some work in this area already.

### Digitized radar video

This is the representation of the radar image, and is used both for presentation and for the base of tracking in a VTS system. Ideally, the tracking system should take care of all targets of interest in a system, and therefore the radar video should not be of interest. However, there might be different levels of interest, i.e. a commercial focused operator looks for something different than a security focused operator. At the moment transferring and display of radar video is probably one of the most proprietary areas for the VTS vendors, but still it should be possible to describe a format for radar video making it possible to transfer. Radar video might imply the need for a high bandwidth, but it should not limit the possibility of describing an interface.

### Radar tracks

This is dynamical information related to a target being followed by a tracking system. Up to now, a VTS gets track information for the most from a tracking system based on radars. Even if there exists different philosophies on how to best track a target, the basic parameters involved are very similar between tracking systems. Position, speed, bearing and identification with a few additions makes the parameter set involved in tracking. Tracks are usually considered for data exchange between systems, and there should be relatively easy to develop a standard where track data is one of the base elements.

### AIS tracks

AIS will become very important in a very short time, and information from AIS systems are also due for exchange between systems. The information is similar to radar tracks with a few important differences. AIS can directly identify a target, it can give additional information regarding cargo and route, and the positional accuracy is usually much better than radar based tracking systems. Even so, the basic information is very close to radar tracks.

### Voyage related information

A VTS usually contains a database system, and even if the systems in use are very different, there are a lot of common parameters.

## Technologies

There are several technologies existing today making it possible to exchange data. The most common one is based on NMEA over a serial connection. This is simple and secure, as the two systems are just connected over a serial line, making the need of firewalls unnecessary. AIS base stations are interfaced through NMEA over a serial line, which shows that relatively recent technologies also use this connection method. Saying this, I will describe the use of XML, and why I think this is the right choice for the base of a standard

### XML(eXtensible Markup Language)

The eXtensible Markup Language -- has recently emerged as a new standard for data representation and exchange on the Internet. The basic ideas underlying XML are very simple: tags on data elements identify the meaning of the data, rather than, e.g., specifying how the data should be formatted (as in HTML), and relationships between data elements are provided via simple nesting and references. Yet the potential impact is significant: Web servers and applications encoding their data in XML can quickly make their information available in a simple and usable format, and such information providers can interoperate easily. Information content is separated from information rendering, making it easy to provide multiple views of the same data.

Currently there are several EU projects focusing on standardization in a maritime environment. Even if IALA is not a European organisation, the result from the initiatives might be a good starting point for IALA. These projects use XML as the base, and it seems naturally to involve some of this work to extract the knowledge gained, but also to be able to form an overall approach where the different standards fits in.

An approach using XML Schemas for message validation will make the transfer of data easier. The Schemas can be made as part of the standard, and when validating against these both for sending and receiving, the different providers and readers will have a certain level of quality of the data. There are tools available for the creation of schemas, which also feature creation of test data. There are also high level tools available for generating code for reading and creating correct data in accordance to the XML Schema.

## Example of a simple XML implementation of tracks exchange

### XML Schema

Figure 1: Example of Positional data within XML Schema

Figure 2: Example of Static data within XML Schema

## Example of XML content

<?xml version="1.0" encoding="UTF-8"?>

<!--Sample XML file generated by XMLSPY v5 rel. 3 U (http://www.xmlspy.com)-->

<TraficImageMessage xmlns="http://www.kystverket.no/" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="http://www.kystverket.no/ TraficImage.xsd">

<PoistionList>

<Position>

<TimeStamp>2001-12-17T09:30:47-05:00</TimeStamp>

<Coordinate>

<Longitude>+180</Longitude>

<Latitude>+90</Latitude>

<Altitude>3.14</Altitude>

</Coordinate>

<Movement xsi:type="MovementType">

<SOG>3.1</SOG>

<COG>360</COG>

</Movement>

</Position>

<Position>

<TimeStamp>2001-12-17T09:30:47-05:00</TimeStamp>

<Coordinate>

<Longitude>+180</Longitude>

<Latitude>+90</Latitude>

<Altitude>3.14</Altitude>

</Coordinate>

<Movement xsi:type="MovementType">

<SOG>3.1</SOG>

<COG>360</COG>

</Movement>

</Position>

<Position>

<TimeStamp>2001-12-17T09:30:47-05:00</TimeStamp>

<Coordinate>

<Longitude>+180</Longitude>

<Latitude>+90</Latitude>

<Altitude>3.14</Altitude>

</Coordinate>

<Movement xsi:type="MovementType">

<SOG>3.1</SOG>

<COG>360</COG>

</Movement>

</Position>

<Position>

<TimeStamp>2001-12-17T09:30:47-05:00</TimeStamp>

<Coordinate>

<Longitude>+180</Longitude>

<Latitude>+90</Latitude>

<Altitude>3.14</Altitude>

</Coordinate>

<Movement xsi:type="AisMovementType">

<SOG>3.1</SOG>

<COG>360</COG>

<ROT>4.0</ROT>

<Heading>200.0</Heading>

</Movement>

</Position>

<Position>

<TimeStamp>2001-12-17T09:30:47-05:00</TimeStamp>

<Coordinate>

<Longitude>+180</Longitude>

<Latitude>+90</Latitude>

<Altitude>3.14</Altitude>

</Coordinate>

<Movement xsi:type="AisMovementType">

<SOG>3.1</SOG>

<COG>360</COG>

<ROT>4.0</ROT>

<Heading>200.0</Heading>

</Movement>

</Position>

</PoistionList>

</TraficImageMessage>