Draft recommendation on performance standard for VTS radar services

(Recommendation [leave number blank, to be filled by IALA Secretariat])

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING that [to be completed by Secretariat - for example - IMO has concluded that AIS will improve the safety of navigation and the protection of the environment];

RECOGNISING ALSO that [to be completed by Secretariat - for example - there is a mandatory carriage requirement for AIS equipment on SOLAS Convention vessels that entered into force on 1 July 2002 and will be complete by [2008]];

RECOGNISING FURTHER that [to be completed by Secretariat - for example - documentation of IMO, ITU and IEC refer to the provision of an AIS shore infrastructure as part of the overall operational system];

NOTING the [as above];

NOTING ALSO that;

NOTING FURTHER that;

CONSIDERING that to be completed by Secretariat - for example.

ADOPTS the [name of document] in the annex of this recommendation; and,

RECOMMENDS that National Members and other appropriate Authorities providing marine aids to navigation services [action to be taken].

*    *    *

Annex

Performance standard for VTS radar services

List of contents

1.	Introduction	4

2.	Background	5

2.1	Radar Target Characteristics	5

2.2	Dynamic requirements	6

2.3	Noise and clutter affecting radar performance	6

2.3.1	Thermal noise	7

2.3.2	Sea Clutter	7

2.3.3	Sea breezes	8

2.3.4	Precipitation	8

2.3.5	Man made noise	9

2.4	Radar Propagation	9

3.	Benefits	10

4.	Recommendations to functions and performance	11

4.1	Radar sensor functions	11

4.1.1	Reduction of operator workload	11

4.1.2	Redundancy	11

4.1.3	Various	11

4.2	Detection Performance	12

4.3	Target separation	12

4.4	Side lobe suppression	13

4.5	Target extraction	13

4.5.1	Normal VTS (IS/NAS/TOS)	13

4.5.2	Security (optional)	13

4.6	Tracking	13

4.6.1	Normal VTS (IS/NAS/TOS)	13

4.6.2	Security (optional)	13

4.7	Availability and reliability	13

4.8	Environmental Specifications and Safety	13

4.9	Related services	14

4.9.1	signal distribution and display	14

5.	Performance verification methods	15

In operation test	15

5.1	System design	15

5.1.1	Parameters to be considered	15

5.1.2	Analysis	15

5.1.3	Simulations	16

5.1.4	Type tests	16

5.2	Factory Acceptance Test	16

5.2.1	System functional tests and measurements	16

5.2.2	Antenna measurements	16

5.2.3	Transceiver measurements	16

5.3	Site Acceptance Test	16

5.3.1	Functional Tests and Checks	16

5.4	In operation tests	16

5.4.1	Acquisition and analysis of statistical data from the system	16

5.4.2	Record and analysis of events	16

# Introduction

[anchor the document in regulations / documentation]

The objective of this document at its present state is to act as model for the structure VTS performance standards based on the VTS functional model illustrated below and eventually be developed into the performance standard for VTS radar services.

For the Radar Service the performance recommendations are divided into 3 levels, where:

- Standard recommendations are applicable to all 3 levels of VTS, Information Service, Navigational Assistance Service and Traffic Organisational Service for areas with medium to high traffic density and without special navigation hazards.

- Relaxed recommendations are applicable to VTS, Information Service and Navigational Assistance Service for areas with low to medium traffic density.

- Extended recommendations are applicable to VTS areas with high traffic density and/or special navigation hazards and /or requirements to detect very small vessels for safety or security reasons.

The individual VTS authority should decide on the level of specification and define requirements related to local conditions such as climate and topography as further advised in this document.

# Background

[provide any background information required to ensure complete understanding of the document]

The implementation of radar services for VTS require a broad spectrum of knowledge about:

- Target characteristics and coverage

- Accuracy and resolution

- Assessment of interference risks

- Weather influence including propagation

- External interface design

- System design and program planning

An often over-looked aspect is the high dynamic range required by VTS radars. Elevated antennas with high gain looking into the sea, surroundings with large structures and the need to operate simultaneously on short and long range results in dynamic requirements far beyond that required onboard a ship.

Ducting can have a serious – positive or negative – influence on the radar system performance and its behaviour is highly dependent on the climate.

Good clutter suppression is needed for sea clutter and, for most parts of the world, rain clutter.

The introduction of AIS further develops the VTS into a modern information system, and the presentation of radar information needs to follow the trend, setting new demands for the radar performance. Antenna side lobes and ghost targets (multiple reflections) may lead to false and dangerous results and high precision is a must to allow for unambiguous correlation of position obtained from the two information sources.

Furthermore, the need to see small targets in rough weather conditions become essential if objectives include detection of targets for security purposes.

## Radar Target Characteristics

A scientific reference is the experimental study of the radar cross-section (RCS) of maritime targets by P.D.L. Williams, H.D, Cramp and Kay Curtis, ELECTRONIC CIRCUITS AND SYSTEMS, July 1978. Vol 2. No 4.

This does in combination with measurements from trials made by an IALA industrial member give rules of thumb for determination of RCS as outlined in Table 2.1.

Table 2.1 , Typical target characteristics

| Target types |  | Average RCS at X band | Fluctuations etc. |
| --- | --- | --- | --- |
| 1 | Navigational aids without radar reflector | Up to 1 m2 | Rapidly fluctuating, highly dependent on sea characteristics. |
| 2 | Navigational aids with radar reflector | 10 – 100 m2 | Rapidly fluctuating, wind and currents may tilt to blind angles and lobbing may cause reflectors to be in blind spots |
| 3 | Small open boat, fibreglass, wood or rubber with outboard motor and at least 2 persons onboard. No radar reflector, low speed | 0.5 – 2 m2 , 0.5 –1 m ASL | Rapidly fluctuating, may be hidden behind waves up to 50 % of the time |
| 4 | Small fast open boat, small speedboat, small fishing vessels or small sailing boats without radar reflector | 1 – 3 m2 , 0.5 –1 m ASL |  |
| 5 | Inshore fishing vessels, sailing boats, speed boats, equipped with radar reflector | 3 – 10 m2 , 1 – 2 m ASL | Rapidly fluctuating |
| 6 | Small metal ships, fishing vessels, patrol vessels and the like | 10 – 100 m2 , 2 - 4 m ASL |  |
| 7 | Coasters and the like | 100 – 1000 m2 | Moderate fluctuations |
| 8 | Large coasters, Bulk carriers, cargo ships and the like | 1000 – 10000 m2 |  |
| 9 | Container carriers, tankers etc. | 10000 – 1000000 m2 | Insignificant |
|  | Buildings, cranes. Stacks of containers and other large structures | Up to 1000000 m2 |  |
|  | Floating items, oil drums and the like. Birds, floating or flying. | up to 1 m2 | Rapidly fluctuating, highly dependent on sea characteristics. |
|  | Flocks of birds. | up to 3 m2 | Rapidly fluctuating, flight paths tend to be characteristic of a given area of interest |

REMEMBER: Cross references to IMO classes

Remember other  Frequency Bands

## Dynamic requirements

Simultaneous coverage from short to long range using one radar set-up sets high demands to dynamic range for VTS radar services.

Targets of interest are often seen to range from below 1 m2  to 1.000.000 m2 in RCS. Ranges vary from typically 50 meters to 15 – 20 NM

Minimum desirable signals to be seen at the receiver input and processed onwards lie typically at -90 to - 105  dBm in VTS applications or maybe as low as –115 dB for security applications.

Very large targets at close range are, however, not fully within the beam at short range, limiting the input at the receiver to about + 10 dBm

Consequently, the overall dynamic range hall between 100 and 125 dB, for  the individual radar.

## Noise and clutter affecting radar performance

This section shall be further clarified

Targets as well as various sources of noise behave differently, and knowledge of this is essential to obtain adequate performance of the radar system. The ability to detect target echoes depends not only on the signal power, but also on the competing noise present in the receiver. The sources of this noise lie both within the radar and in the outside environment as illustrated in Figure 1.

Figure 1 Noise sources.

### Thermal noise

Historically, the sensitivity (or Minimum Detectable Signal, MDS) of radar systems has been limited by the thermal noise of the receiver, expressed by the noise figure. However, processing techniques becoming common during the 1990’s can remove the white noise and reduce the radar system MDS below the level determined by the noise figure.

The noise figure can therefore not be used as a single parameter to compare radar systems anymore.

### Sea Clutter

Sea and wind relations vary dependent on the geographical location. The Ocean differs from other waters and the same seams to be the case for the definition of sea states.

Tender material often relates the Sea State to the Beaufort Scale, definitions in the  CARPET programme or the Douglas scale.

The approximate relations between the scales, based on various publications are illustrated in Table 2.2.

In order to avoid confusion, it is suggested to study the subject further and find one common definition to be used.

Table 2.2 : Sea state – approximate relations between Beaufort scale, the Douglas scale and CARPET Definitions

### Sea breezes

The large temperature variations between day and night in desert areas and associated pressure differences between land and sea tend to cause strong sea breezed during summertime. This generates, in combination with layered distribution of air, eddy’s over the sea, often forming distinctive sea clutter patters

### Precipitation

#### Rain clutter and penetration

The amounts of rain clutter return from radars are dependent on:

- The resolution cell size, meaning that influence is reduced with decreasing antenna beam width and decreased with decreased transmitter pulse width.

- The antenna polarisation, where circular polarisation is preferred over vertical or horizontal polarisation.

- Frequency, where lower frequencies penetrate precipitation better than higher frequencies and give less clutter return: However, lower frequencies are in most cases associated with severe drawbacks in respect to resolution and cost.  Therefore, the use of higher frequencies and circular polarised antennas in combination with processing techniques tend to be preferred for VTS except for long range installations in areas with extreme tropical rainfall.

#### Snow and hail penetration

Text to be added

### Man made noise

Man made noise consist primary of signals from other nearby radars. However, applying noise reductions techniques allow for several radars near each other to operate on the same frequency without interference.

## Radar Propagation

In some areas of the world, especially those with adverse climate, propagation effects cause radar performance to vary drastically over time – sometimes from hour to hour.

Temperature and humidity gradients in the air above the sea surface cause the radar beam to be bent up or down - in most cases down and thereby elongated to follow the earth’s curvature – generating a duct.

A change in the moisture distribution without an accompanying temperature change form a so-called evaporation duct, always leads to extended radar range if present.  The air in contact with the ocean’s surface is saturated with water vapour. A few meters above the surface the air is not usually saturated, so there is a decrease of water vapour pressure from the surface to some value well above the surface.

The atmosphere may also form a trapping layer and generate a waveguide like duct – a surface based duct or an elevated duct.

Under trapping layer conditions a radar antenna situated within the duct may receive extremely long-range returns. The result is also considerably increased amounts of noise and false echoes generated by objects beyond the radar ambiguity range (2nd and multiple time around echoes).

An Antenna situated above a trapping layer, or even worse above more than one trapping layer, exhibits only very short range targets since most of the energy bounces off the upper limit of the duct and is reflected into space or into an elevated duct, potentially causing very difficult situations.

# Benefits

- [include as appropriate – the benefit of adhering to the information contained in the document]

# Recommendations to functions and performance

## Radar sensor functions

### Reduction of operator workload

Radar sensors are recommended to have as much automatic adoption to weather as possible to optimise performance and minimise operator workload.

Only basic functions such as start and stop should be controllable by operators.

Built in test features should include monitoring of functions and performance  and it is recommended to make the results accessible for remote monitoring, especially in the case of radars being installed at locations difficult to access.

### Redundancy

The redundancy listed in Table 4.3 is appropriate for the majority of radar sites.

Table 4.3 Redundancy

| Recommendation level |  | Standard |  |  |  | Relaxed |  |  |  | Extended |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VTS Area |  | Inland Waterways | Harbours | Littoral Waters | OFF Shore | Inland Waterways | Harbours | Littoral Waters | OFF Shore | Inland Waterways | Harbours | Littoral Waters | OFF Shore |
| Redundancy | Not required | X | X |  |  | X | X | X | X |  |  |  |  |
|  | Dual Transmitters |  |  | X | X |  |  |  |  | X | X | X | X |
|  | Stereographic view |  |  |  |  |  |  |  |  | X | X |  |  |

The use two or more radars to overlook an area (stereographic view) will increase the availability of radar service and it is beneficial in congested areas to eliminate effects of multi path propagation.

### Various

De-icing of antennas may additionally be required in areas with icing conditions.

## Detection Performance

It is recommended that VTS shall be capable of detecting the following type of targets:

Table 4.4

| Recommendation level | Standard | Relaxed | Extended |
| --- | --- | --- | --- |
| Inland waterways |  |  |  |
| Harbours |  |  |  |
| Littoral Waters |  |  |  |
| Offshore |  |  |  |

This section shall be further extended to include:

- Detection ranges versus target size

- Sea clutter suppression

- Rain penetration and rain clutter suppression

- Adaptation to varying propagation conditions

## Target separation

It is recommended that a VTS shall be capable of separating the following type of targets:

| Azimuth separation for for detection targets at distances beyond 1 NM. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Target 1 |  | 3 m2 | 10 m2 | 10 m2 | 10.000 m2 |
| Target 2 |  | 3 m2 | 10 m2 | 10.000 m2 | 10.000 m2 |
| Specification | Standard |  |  |  |  |
|  | Relaxed |  |  |  |  |
|  | Extended |  |  |  |  |

Azimuth Separation for targets within the coverage area and located at distances below 1 NM shall not be less than for targets at 1 NM.

| Range separation of 2 small point targets aligned in azimuth |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Standard specification | Relaxed specification | Extended specification |
| Application | Inland waterways |  |  |  |
|  | Harbours |  |  |  |
|  | Littoral Waters |  |  |  |
|  | Offshore |  |  |  |

| Range separation of 2 large targets aligned in azimuth |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Standard specification | Relaxed specification | Extended specification |
| Application | Inland waterways |  |  |  |
|  | Harbours |  |  |  |
|  | Littoral Waters |  |  |  |
|  | Offshore |  |  |  |

## Side lobe suppression

To be specified

## Target extraction

Including facilities to reduce / discriminate from noise.

Remember to mention Network Performance

### Normal VTS (IS/NAS/TOS)

To be specified

### Security (optional)

To be specified

## Tracking

### Normal VTS (IS/NAS/TOS)

To be specified

### Security (optional)

To be specified

Remember track management, correlation and association with other radars and other sensors

## Availability and reliability

To define requirements to availability of individual radar sensors

Also to suggest on redundancy

Stability, UPS etc.to be mentioned

Preventive and corrective maintenance to be discussed

## Environmental Specifications and Safety

OOB emissions and spurious

EMC/EMI

Temperature, humidity, etc

Acoustic Noise

Storage and transportation

Safety

Electric shock

Lightning protection

Rotating machines etc.

Radiation Hazards

Protection against other radars

## Related services

### signal distribution and display

To be specified

Remember to include definition for update rate requirements

# Performance verification methods

The high dynamic characteristics and dependence on propagation conditions furthermore adds uncertainty to tests on installed radar systems ad it can be difficult to reproduce tests as conditions may vary substantially from day to day. Therefore a combination of tests and calculations is recommended for verification of radar services.  Please refer to Table 5.5

Table 5.5, performance verification methods

|  | Design / verification method |  |  |  |
| --- | --- | --- | --- | --- |
|  | During system design | Factory Acceptance Test | Site Acceptance Test | ### In operation test |
| Overall functionality | Analysis | Functional test | Functional test |  |
| Coverage | Analysis including weather influence and propagation |  | Functional test |  |
| Detection performance and dynamic requirements | Simulation including weather influence and propagation | Measurement of antenna and transmitter-receiver parameters, losses etc. | Functional evaluation, compared to weather condition |  |
| Target separation | Analysis | Measurement of individual antenna and transceiver parameters | Check using known structures or controlled targets. |  |
| Tracking performance | Analysis or Simulation |  | Functional test | Acquisition and analysis of statistical data from the system |
| Environmental and Safety issues | Analysis and type tests | Check of workmanship and measurement of parameters | Check of Installation |  |
| Availability | Analysis |  |  | Record and analysis of events |

Methods suggested for the individual item is described in the following paragraphs.

## System design

### Parameters to be considered

To be specified

### Analysis

To be specified

### Simulations

To be specified

### Type tests

To be specified

## Factory Acceptance Test

### System functional tests and measurements

To be defined, possibly by references to other standards

### Antenna measurements

At least Including:

- Gain or directivity

- Radiation patterns, sidelobes

- Integrated cancellation ratio, in main beam, outside main beam (only circular polarised antennas)

### Transceiver measurements

At least including:

- Transmitter pulse characteristics

- Transmitter peak power,

- Transmitter rise and fall time

- Receiver Noise Figure (Overall) and/or Minimum Detectable Signal  (Relations to noise floor to be described)

- Receiver dynamic range , being the ability to receive pulses without degradation (collapse & pulse stretch)

## Site Acceptance Test

### Functional Tests and Checks

- Check of Transmission line losses etc.

## In operation tests

To be specified

### Acquisition and analysis of statistical data from the system

To be specified

### Record and analysis of events

To be specified