| From: AIS Committee | AIS12/output/03 |
| --- | --- |
| To: VTS18, RNAV18, EEP2, ANM2 | Date: 07-03-2003 |

Liaison Note

MMSI Number Assignment for Synthetic AIS and Virtual AtoN AIS

- Introduction

The Preliminary Draft Recommendation on AIS for AtoNs (AIS12/output/02) discusses the application of AIS to AtoN.  During the preparation of this recommendation questions arose regarding the assignment of MMSI numbers as follows.

Message 21 must include an MMSI number. For AtoN AIS, this number will be the MMSI number of the AtoN AIS station. However, for Synthetic or Virtual AtoN AIS the question arises of whether the Message 21 MMSI number should be the MMSI number of the AIS station, such as a base station which broadcasts the Message 21, or should be another MMSI number.

- Proposal

- MMSI numbers should be used for real AIS stations as noted in the introduction above. An AtoN AIS station will have an MMSI, and this should be used. (For retransmission see item 3 below).

- When a base station is sending synthetic information the base station’s MMSI should be used. To identify the Synthetic AtoN target an identification should be used in accordance with section 3.1.1 of the Preliminary Draft IALA Recommendation on AIS  for Aids to Navigation.

- Where an AtoN station exists, and it is required to retransmit its information, the AtoN station’s MMSI should be sent in the message.

- If there is a need to send a virtual AIS target for a short period of time (temporary situation) it is felt that the identification of the AtoN table 34 in the Draft IALA Recommendation on AIS for Aids to Navigation should be used. If there is a need for a virtual target to be generated for a long period of time, the Competent Authority should assign an identification similar to an existing aid.

- Action Requested

The VTS, RNAV, ANM & EEP Committees are invited to respond to these comments.

The IALA Secretariat is asked to forward comments on the draft Recommendation from the VTS18, RNAV18, EEP2 and ANM2 to the Chairman of the AIS Technical Working Group aa(JOltmann@wsd-nord.de) by 30-06-03.