### Meteorological system

### The measurement sensors must be installed and located according to recommendations from a meteorologist

- The most common measurements are: wind speed, wind direction, air temperature, relative humidity, air pressure, water temp and seawater level

- The results of the measurements must be presented in approved (SI) units

- The measurements must be available to the traffic operators at all time and be presented numerically and/or by a pointer indicator

- A log of the latest 24 hour measurements, must be available to the operators either numerically or graphically.

- External installation of electronic must be installed in an housing satisfy the IP65 requirements.

- The accuracy of the measurements must satisfy the following minimum requirements:

  - Wind speed		+/- 0,5 m/s

  - Wind direction 	better than +/- 5°

  - Air temperature	resolution better than 0,1°C

  - Air pressure		+/- 0,2 hPa

  - Air humidity		+/- 3%

  - Water temperature	better than 0,1°C