| From: AIS Committee | AIS12/output/07 |
| --- | --- |
| To: VTS18 | Date: 07-03-2003 |

Liaison Note

Draft – Frequently Asked Questions (FAQ) on AIS to assist in broadening awareness and understanding of the capabilities of AIS by the maritime community

- Introduction

Upon review of document AIS12/4/5 “Report on a Workshop on the Training of VTS Personnel for the AIS World” and hearing comments from participants, the OPSWG was of the opinion that there was a need to provide educational materisl for potential users of AIS.

- Proposal

- The AIS OPSWG has begun development of a series of FAQs reflecting issues raised in the Report, using the collective experience of WG members.  This list of FAQs will be added to the AIS page on the new VTS web site.

- Action Requested

The VTS Committee is asked to review the questions and add to the list as appropriate.

The IALA Secretariat is asked to forward the FAQ list to AIS13 for further review.

Questions & Answers

Will AIS replace Radar?

- No.  There always will be ships without AIS onboard.  Radar detects targets independent of the targets onboard-equipment.

How can the AIS assist the Radar?

- In addition to radar, AIS has following benefits:

- Automatic and immediate vessel identification

- Improved vessel tracking (no track swap)

- Wider geographical coverage

- Greater positional accuracy

- Absence of radar shadow area

- Real time manouvering data

How does the master know that his AIS unit is working properly?

- Other than a power loss the proper operation and information validity requires a verification from another AIS station.

Can I send a letter to my agent?

- AIS is mainly for monitoring other ships and allows only the transmission of safety related messages on AIS designated frequencies.

Is a vessel permitted to sail without an operational AIS equipment?

- It depends on regulations of  the national maritime administrations (port state control).  According to SOLAS Ch.V Reg.16 the vessel is allowed to proceed to a port where repairs can be carried out.

Why do AIS messages not include the rudder angle?

- It is deliberately avoided to send the rudder angle.  This information does not give an accurate indication of the actual path of the vessel.

NOTE:

The rudder is just a tool to get the ship to start a turn.  Due to shallow water, constellation of propellers, one or two rudders, open or closed skegg, dynamically stable or unstable ship as well as wind direction, the rudder angle could be very ambiguous.  E.g. a dynamically unstable ship turning to SB could e.g. need 10 degrees port to avoid a ship to turn fast to starboard.  If the rudder angle was sent on the AIS it could confuse an observer an another ship or ashore.

Can I display the AIS targets on my existing onboard radar?

- Type approved AIS-capable radars do not exist yet.  Manufacurers are the best source of information on this burgeoning state of the art.

How are ARPA and AIS vectors distinguished on a common onboard display?

- Using appropriate symbology.

Can I connect my old GPS to the AIS?

- The GPS connected to the AIS must fulfill the IMO performance standard.  It means inter alia that the output from the GPS must be in WGS 84 and that there is no dead reckoning of the position.

What is the accuracy of information provided?

- The accuracy of nautical information, as position, speed etc., depends on the accuracy of the sensors used.  There is an indication about the accuracy transmitted by AIS.  Further data accuracy, as voyage related data, depends on the accuracy this data is typed in.

Will AIS become a recognised anti-collision device?

- This would introduce the question of revising the collision regulations, which would be a lengthy process.  That is likely after vessels´ officers are trained in using it in conjunction with COLREG.

Is there an alarm to indicate that the AIS unit is not powered on?

- Depends on the onboard installation.

Under what circumstances is it allowed to switch off the AIS?

- Where the master believes it is necessary for the safety of the crew and/or the vessel and/or the cargo.

Is the AIS data stored somewhere?

- Ashore this is a matter for national administrations.

Onboard it depends on the equipment installed.

For what purposes will the stored or recorded data be used?

- Accident investigation, traffic monitoring, planning and optimising, statistics.

Are naval ships required to fit AIS transponders?

- No.  They are outside the SOLAS Convention, but Administrations may dictate naval vessel use of AIS.

Why is VHF as a transmission medium for AIS used, rather than radar?

- The secondary radar option (used by aviation) was rejected by the radar manufacturers, because it was thought it would add too much cost to the radar installations on ships.

What are the differences between AIS Class A transponders and AIS Class B transponders?

- Class B is intended to be a simpler, less expensive version for the use on non-SOLAS vessels.

Should the non-SOLAS maritime community wait for the development of Class B, particularly if it will cost less?

- Class B is intended primarily for recreational crafts.  It will be determined by Administrations whether non-SOLAS vessels are required to carry Class A or Class B AIS equipment.

Why does a vessel with a sophisticated ECDIS or ECS need to carry a minimum keyboard & display?

- Because type-approved ECDIS/ECS with AIS capabilities do not exist yet.

Will non-SOLAS vessels be required to carry a Minimum Keyboard and Display (MKD) as an AIS display device?

- Regulations for non-SOLAS vessels are a matter for Administrations.

Is the MKD intended as a navigation device?

- No.

How are GPS and chart datum collocated?

- AIS co-ordinates are in WGS-84, the co-ordinate system used by GPS.

What methods are available to prevent misuse of AIS data?

- This is a matter for Administrations to police.

How is AIS derived data controlled?

- This is a matter for Administrations.

How is the AIS data limited to safety information?

- ITU-R recommendation M.1371-1 limits AIS transmission on AIS designated channels to maritime safety related messages.

Is there a limit on the scope and content of AIS messages?

- Some messages are defined, others have limited capacity for text content.

Will AIS messaging grow to include for commercial need?

- That is likely;  however, commerical messages cannot be transmitted on designated, safety related frequencies, e.g. AIS1 and AIS2.

How do we detect/prevent transmission of false messages?

- This is a matter for national administrations to police.

Should AIS data be integrated on the same display as other sensors information in a VTS?

- Yes, all sensor information ought to be integrated on a single display.

Can I get DGNSS corrections over the AIS link?

- Yes, if Administration decides to do so, the message structure and technical approach are defined.

How is meteorological/hydrological information displayed?

- There are prototype ways to display, a message structure is being proposed to IMO.

Is there any calibration (regular/frequent/interval/initial) of the AIS equipment required?

- No.

Is AIS mandatory for inland waterways?

- This is a matter for Administrations.

Should I use a certified equipment?

- Yes, to comply with the international standard´s approval.

Is there any training available?