The following functions and features requirements are recommended for CCTV cameras

- Waterproof housing

- Satisfactory picture quality although exposed to saltwater and icing

- Heating element, wipers and/or washer if this is a standard equipment from the supplier

- Pan-tilt unit, remotely controlled in both the vertical plan (y) and in the horizontal plan (x).

The pan-tilt requirements are:

- 360 degree pan angle

- 120 degree tilt angel (+45-75)

- Very high resistance to vibrations

- Both monochrome and color cameras can be used. The monochrome is more light sensitivity and thus best suited for working in darker environments. The illumination requirements (the relationship between light lx  and the area of the enlighten surface) ought to E< lx for the black and white camera.

- Automated lens and electronic protection from intensive light should be fitted (example direct sunlight).

- Minimum lens zoom ratio of 1:10

- Module for storing the picture images. The require updating rate is minimum once per second.

- The camera must have an function for handling zoom and focus. If an automated iris control are not built into the system, this should be included as a function. The system should also have functionalities for programming prepositions.

- The pan-tilt platform should respond to position guidance given by the traffic system in latitude and longitude coordinates, and position the respective camera according to this position. Zoom and focus should be a part of the camera maneuver. Every fulfilled camera responce should be confirmed by the camera system to the operators.

- The picture from at least one arbitrary camera must be shown in minimum 4 gray tone  scale (black, white, light and dark) in a separate window on the traffic monitor with an update rate of minimum once per second. The resolution should be 1:1.

- Frame-Grabbing techniques may be used for transferring video (a digitized picture taken every “n” second. The parameter “n” should be remotely controlled)

- The system should be able to displayed video on a separate monitor where the updating rate is only restricted by the transferring capacity.

- The camera should have the abillity to be remotely controlled to follow a target  in movement or focus on a permanent position predetermined by the supplier or specified by the traffic operator.

- Presentation of camera bearing and elevation

- The camera should have the abillity to be positioned based on the traffic situation

- The camera should allow the current camera image to be frozen and stored to a file