VHF recommendations

- Simplex radio bases should have a multi channel function and be programmable for all simplex channels in the maritime VHF-band

- Duplex radio bases should be equipped with “hot standby” bases and be programmable with primary working channels

- Duplex radio bases should be operated as semi-duplex with priority given to the traffic operators

- Duplex radio bases should be operated with a repeat function in order for ship to shore calls on one channel to be repeated on a shore to ship frequency for the same channel

- Channel status should be automatically displayed at startup, on request or at regular intervals in order to avoid the possibility for displaying incorrect channel. A four tone feedback or equivalent, should be heard by the operator each time a new channel is selected.

- The area covered by the antenna system should be simulated with an appropriate software tool. Antenna covering diagram from current used VHF at the same location must be obtained

- The antenna covering diagram should have lower radiation in areas where VHF coverage is not needed.

- The suppliers design and configuration of the antennas together with an VHF coverage diagram, should be presented for approval before installation.

- When several radio bases are connected to a combined antenna system, the vertical separation between receiver and transmitter antennas should be minimum 12 meters in order to avoid interference. The same separation should be used by VHF antenna systems at the same location used by other operators. In order to avoid interference, a filter in the receiver equipment has to be considered.

- The following should be monitored and give an alarm if status is changed:

  - Operational voltages for the radio bases

  - Operation status for the radio bases

  - Denial of channel selection

  - Output power from the radio bases

  - Antenna adjustment (3 dB limit)

  - The programmed ‘hot standby’ function not accepted

  - Eruptive communication between the central unit and the radio bases

  - Central unit

- Two loudspeakers with individual volume control and light indication should be connected respectively to the primary channel and the secondary channel.

- The system should monitor all selected channels and show active channel status (scan modus)

- The system should fulfill all the requirement according to GMDSS

- All communication functions should be monitored from one or more windows on a pc monitor or from a dedicated monitor panel. The monitoring should minimum include the following:

  - Selection of radio bases for transmitting or receiving, only receiving or independent. The channels for transmitting/receiving must be changeable

  - Keying of selected radio base(s)

  - An option for connection the radio base(s) to PABX and a telecommunication network

  - Monitoring of alarm/status of remote radio bases

  - The operators must have access to the remote radio bases log in order to confirm alarms

- Separate microphone with a key function, loudspeaker and a wireless headset should be an option

- The wireless headset should have an transmission key option