- (normative)
Symbols and colours presented on the display

  - Symbols

| Table B.1 - Summary of symbol definitions for operational information in existing standards |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Topic | Radar 60936 | Radar plotting 60872 | Chart radar 60936-3 | ECDIS 61174 | IHO S52 | AIS IMO circ. 217 | 60945 and Other | Harmonized Symbology Description | Harmonised Symbology |
| Ship Symbol Own Ship |  |  |  |  |  |  |  |  |  |
| Symbol Own Ship | Double circle Undefined colour. Colour to be determined dependent on the background. | As 60936. Undefined colour Colour to be determined dependent on the background. | As 60936. Undefined colour Colour to be determined dependent on the background. | As 60936. colour=ships (background dependent) | NA | NA | NA | Double circle, located at own ships reference position. | No changes needed |
| True scale outline Own Ship | Outline shape Colour to be determined dependent on the background. | As 60936 Colour to be determined dependent on the background. | As 60936 Colour to be determined dependent on the background. | As 60936 colour=ships (background dependent) | NA | NA | NA | True scale outline on small ranges/large scales if individual standard requires | No changes needed |
| Antenna Position | (draft new radar standard) Colour to be determined dependent on the background. | (draft new radar standard) Colour to be determined dependent on the background. | (draft new radar standard) | Not defined colour=ships (background dependent) | NA | NA | NA | Located on true scale outline of own ship. | No changes needed |
| Heading line Own Ship | Solid line Colour to be determined dependent on the background. | As 60936 Colour to be determined dependent on the background. | As 60936 | As 60936 colour=ships (background dependent) Change options of heading line length (App.E.2.2) | NA | NA | NA | Solid line thinner than speed vector line style, drawn to the bearing ring or of fixed length if bearing ring not displayed. Origin at own ship’s reference point | Changes needed |
| Beam line Own Ship | Solid line (option) | As 60936 | As 60936 | As 60936 | NA | NA | NA | Solid line of fixed length. Midpoint at own ships reference point. | No changes needed |
| Ship symbol ARPA Targets |  |  |  |  |  |  |  | On chart display as background use S-52 colour token arpat – Colours of AIS and ARPA targets should be identical |  |
| Normal state ARPA | NA | Dot or Small circle | As 60872 | As 60872 colour=arpat | NA | NA |  | Solid filled or unfilled circle 60872 specifies luminance control independent of radar video | No changes needed |
| Acquire state ARPA | NA | Dotted square (manual or automatic target acquisition) Change of the outline of the symbol required | As 60872 | Not defined | NA | NA |  | Circle segments colour=arpat | Changes needed |
| Danger state ARPA | NA | Triangle flash until acknowledged, then may be steady Change of the outline of the symbol required | As 60872 | NA | NA | NA |  | Bold red solid circle (may be larger) and vector if displayed, both flashing until acknowledged. colour=dnghl | Changes needed |
| Lost state ARPA | NA | Diamond flash until acknowledged, then may be steady Change of the outline of the symbol required | As 60872 | NA | NA | NA |  | Bold lines across the circle flash until acknowledged colour=arpat | . Changes needed. |
| Target in guard zone ARPA | NA | Triangle apex down flash until acknowledged, then may be steady Change of the outline of the symbol required | As 60872 | NA | NA | NA |  | Bold red circle segments (may be larger), flashing until acknowledged. colour=dnghl | Changes needed. |
| Selection symbol ARPA | NA | Solid square Change of the outline of the symbol required | As 60872 | NA | NA | NA |  | Square indicated by its corners Colour=arpat | Changes needed |
| Heading line ARPA | NA | NA | NA | NA | NA | NA |  |  |  |
| Beam line ARPA | NA | NA | NA | NA | NA | NA |  |  |  |
| AIS contacts Ship symbol |  |  |  |  |  |  |  |  |  |
| Default state AIS | Per Circ 217 | Per Circ 217 | Per Circ 217 | Per Circ 217 | NA | Triangle, smaller, oriented by Heading (or COG if heading missing) |  | Triangle, oriented by heading or COG if heading missing. No heading line or vector may be shown. Colour=arpat This symbol may be smaller than the minimum symbol size (defined below) The reported position should be located at centre and half the height of the triangle | No changes needed |
| Active state AIS | Per Circ 217 | Per Circ 217 | Per Circ 217 | Per Circ 217 | NA | Triangle, oriented by heading (or COG if heading missing) |  | Triangle, oriented by heading or COG if heading missing. The COG/SOG and heading are displayed Colour=arpat The reported position should be located at centre and half the height of the triangle | No changes needed |
| Active state AIS – true scale outline | Per Circ 217 | Per Circ 217 | Per Circ 217 | Per Circ 217 | NA | Triangle symbol may be replaced by true scale outline on small ranges/large scales |  | Triangle symbol may be replaced by true scale outline on small ranges/large scales Colour=arpat | No changes needed |
| Danger state AIS | Not defined | Not defined | Not defined | Not defined | NA | Bold red triangle |  | Bold red triangle, heading and vector flashing until acknowledged, size may be increased Colour=dnghl | No changes needed |
| Lost state AIS Position data not received. | Not defined | Not defined | Not defined | Not defined | NA | Triangle with perpendicular thick solid line |  | Triangle with bold solid cross, flashing until acknowledged. Triangle oriented per last known value. Cross has fixed orientation. Colour=arpat | No changes needed |
| Selection symbol AIS | Not defined | Not defined | Not defined | Not defined | NA | Square indicated by corners |  | Square indicated by corners drawn around the target symbol Colour=arpat | No changes needed |
| Headingline AIS |  |  |  |  |  |  |  |  |  |
| Heading line for activated state of AIS | 25mm solid line Change required to the length of the heading line | NA | As 60936 | As 60936 | NA | Solid line |  | Solid line thinner than speed vector line style, length twice of the length of the triangle symbols. Origin of the heading line is the apex of the triangle. If true scale outline is used the origin is the reported position. Colour=arpat | Changes of individual standards needed |
| Indicating turn | Not defined | NA | Not defined | Not defined | NA | Turn indicated by flag |  | Turn indicated by flag of fixed length Colour=arpat |  |
| Beam line AIS | Not defined | NA | Not defined | Not defined | NA | Not defined |  |  |  |
| Speed Vector Own Ship |  |  |  |  |  |  |  |  |  |
| Default state | Solid line with solid line minute marks There is a plan to change this to dashed line for Radar | As 60936 | As 60936 | As 60936 | NA | NA | NA | Dashed line thicker than heading line style The length of one dash plus one space represent a one minute increment of the selected time interval. Consequently the vector line starts with a space from the own ship reference position and ends with a dash. Colour=ships | Changes of individual standards needed |
| Water/Ground stabilization. | Water =single arrowhead. Ground =double arrowhead Change use of arrowheads to an option | As 60936 | As 60936 | As 60936 Change use of arrowheads to an option | NA | NA | NA | Water stabilised: Optionally one arrow Ground stabilised: optionally two arrows Colour=ships | No extra graphic representation required. Indication of stabilisation mode within the user interface is required. Optionally arrowheads may be used – one for water stabilisation – two for ground stabilisation. Changes needed |
| Path prediction | not defined | not defined | not defined | not defined | NA | NA | NA | A curved vector may be provided as a path predictor Colour=ships | No changes |
| Danger State | not defined | not defined | not defined | not defined | NA | NA | NA |  |  |
| Speed Vector ARPA Targets |  |  |  |  |  |  |  |  |  |
| Default state | NA | Solid line with optional arrows (single or double) and optional minute marks. Interval between “minute” marks is not specified. spacing 1 minute between marks with heavy mark each 6 minutes. luminance control independent of radar video | As 60872 | As 60872 | NA | NA | NA | Dashed line. The length of one dash plus one space represents a one-minute increment of the selected time interval. Consequently the vector line starts with a space from the reported position and ends with a dash. Colour= arpat | Changes needed |
| Speed Vector AIS contacts |  |  |  |  |  |  |  |  |  |
| Activated state | Solid line with open triangle minute marks, at 6-minute interval. | NA | As 60936 | As 60936 | NA | Dashed line |  | Dashed line The length of one dash plus one space represents a one-minute increment of the selected time interval. Consequently the vector line starts with a space from the reported position and ends with a dash. Colour= arpat | No changes |
| Activated state AIS target Water/Ground stab. | Double arrowhead Change: Remove requirement for display of arrowhead | NA | As 60936 | As 60936 Change: Remove requirement for display of arrowhead | NA | Not indicated |  | No symbol | No extra graphic representation required. Indication of stabilisation mode within the user interface is required. Changes needed |
| Activated state AIS target indicating turn | Not defined | NA | Not defined | Not defined | NA | Curved vector |  | A path predictor may be provided as curved vector Colour=arpat | No changes |
| Activated state AIS target for which course or speed or heading are not received. | Not defined | NA | Not defined | Triangle with question mark | Not defined | Not defined | Not defined | Dashed ships outline Colour=arpat [Reconsideration of the need of this symbol if more operational experience has gained – consider possible alarm conditions resulting from this as well] | Changes needed |
| Past Track Own Ship |  |  |  |  |  |  |  |  |  |
| Primary source | As 61174 | NA | As 60936 | Thick solid line with time marks Color=pstrk | NA | NA | NA | Thick line optionally added by time marks Color=pstrk | No changes needed |
| Second source | As 61174 | NA | As 60936 | Thin solid line with time marks Colour=sytrk | NA | NA | NA | Thin line optionally added by time marks Colour=sytrk | No changes needed |
| Past Track ARPA Target |  |  |  |  |  |  |  |  |  |
| Normal | NA | Dots | As 60872 | Not defined | NA | NA | NA | Dots Colour=arpat | No changes needed |
| Past Track AIS contacts |  |  |  |  |  |  |  |  |  |
| Normal | Not defined | NA | Not defined | Not defined | NA | Not defined |  | Dots Colour=arpat | Changes needed |
| AIS based AtoN |  |  |  |  |  |  |  |  | Text to identify AtoN |
| Real position of charted object | Not defined | NA | Not defined | Not defined | NA | Not defined |  | Diamond around charted object symbol – for radar alternatively without charted object symbol Colour=[resbl] |  |
| Virtual position | Not defined | NA | Not defined | Not defined | NA | Not defined |  | Diamond around virtual object symbol – for radar alternatively without virtual object symbol Colour=[resbl] |  |
| Navigation Tools |  |  |  |  |  |  |  |  |  |
| Cursor | Crosshair (two alternatives, one with open center) | As 60936 | As 60936 | As 60936 | As 60936. Colour=cursr (96,97,219) | NA | NA | Colour=cursr | Crosshair (two alternatives, one with open centre) No changes needed |
| EBL | Manufacturer selectable linestyle and colour/shape (additional rules for linestyle for 60936-2) | As 60936 | As 60936-2 | As 60936-2 | As 60936-2 Colour=ninfo (117) | NA | NA | Colour=ninfo | No changes needed |
| Origin of off- centered EBL | Not defined | Not defined | Not defined | As S52 ed3.2 | Dot Colour=ninfo (117) | NA | NA | Colour=ninfo | Dot No changes needed |
| Range marker (VRM) | Manufacturer selectable linestyle and colour/shape (additional rules for linestyle for 60936-2) | As 60936 | As 60936-2 | As 60936-2 | As 60936-2 Colour=ninfo (181) | NA | NA | Optional second VRM (if required by individual standards) visually distinguishable from the primary VRM Colour=ninfo | No changes needed |
| Range rings | Solid circle | As 60936 | As 60936 | Not defined | Not defined | NA | NA | Solid circles Colour= ninfo | No changes needed |
| Parallel index lines | At least two lines and distinguishable from EBL and maplines | As 60936 | As 60936 | Not defined | Not defined, | NA | NA | Distinguishable from EBL and maplines by line style. Colour= ninfo | No changes needed. |
| Map lines and Nav lines | Ref 60936-1 Annex C4. | As 60936 | As 60936 | Not defined | Not defined, | NA | NA | Per 60936 only allowed to be displayed on radar without chart background | No changes needed |
| Additional chart information | Not defined | Not defined | Not defined, | Colour=adinf | Colour=adinf | NA | NA | Colour=adinf | No changes needed |
| Minimum width of safety contour | NA | NA | At least 4 pixels change: reduce to 3 pixel with | Fixed 3 pixel Colour=depsc | Fixed 3 pixel Colour=depsc | NA | NA | Fixed 3 pixel Colour=depsc | changes needed |
| Alternate route | Orange solid line WP is circle etc. “monitored route”, Change of colour and line style | As 60936 | As 60936 | As S52 ed3.2 | Orange small dotted line WP is circle etc. Colour=aplrt | NA | NA | Dotted line, WP as circles Colour=aplrt [colour coordinates to be reconsidered] | Changes needed |
| Monitored route | Red solid line WP is circle etc Change of colour and line style | As 60936 | As 60936 | Red bold dotted line WP is circle etc. Change of colour and line style | Red bold dotted line WP is circle etc. Colour= plrte | NA | NA | Dashed bold line, WP as circles Colour=plrte | Changes needed |
| ARPA Predicted area of danger (PAD) – to be used for AIS as well | Optional hexagon or ellipse Luminance control independent of radar video [Change: delete requirement for independent luminance control] | As 60936 | As 60936 | Not defined | Not defined | NA | NA | [Luminance control independent of radar video] Optional solid line hexagon or ellipse Colour=ninfo | changes needed |
| ARPA Acquisition area | Luminance control independent of radar video Solid line boundary for an area [Change: delete requirement for independent luminance control] | As 60936 | As 60936 | Not defined | Not defined | NA | NA | [Luminance control independent of radar video] Solid line boundary for an area Colour=ninfo | changes needed |
| ARPA Guard zones | Luminance control independent of radar video Solid line boundary for an area [Change: delete requirement for independent luminance control] | As 60936 | As 60936 | Not defined | Not defined | NA | NA | [Luminance control independent of radar video] Solid line boundary for an area Colour=ninfo | changes needed |
| Searchlight (area of interest) | NA | NA | NA | 4.10.5.3 & 4.10.5.4 Symbol not defined | Not defined | NA | NA | Solid line boundary for an area. Colour=dnghl | No changes needed |
| ARPA Trial manoeuvre | NA | Large T on screen | As 60872 | Not defined | Not defined | NA | NA | Large T on screen Colour=ninfo | No changes needed |
| ARPA Test target | NA | Letter X, XX, XT, XXT | As 60872 | Not defined | Not defined | NA | NA | Letter X, XX, XT, XXT on screen Colour=ninfo | No changes needed |
| Event Mark | NA | NA | Not defined | As S52 | Colour=ninfo | NA | Not defined | Rectangle with diagonal line, added by text “MOB” for man over board cases Colour=ninfo | No change needed |
| Mariner’s Caution, Info Note | NA | NA | Not defined | As S52 | Colour=ninfo | NA | Not defined | Colour=ninfo | No change needed |
| Manufacturer’s Info Note | NA | NA | Not defined | As S52 | Colour=adinf | NA | Not defined | Colour=adinf | No change needed |

  - Colours

| Table B.2 - Summary of colour definitions for operational information in existing standards |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Topic | Radar 60936 | Radar plotting 60872 | Chart radar 60936-3 | ECDIS 61174 | IHO S52 | AIS IMO circ. 217 | Other 60945 | INS 61924 | Comment |
| Day and night operation |  |  |  |  |  |  |  |  |  |
| Day | Provision shall be made for day and night operation | As 60936 | Depth areas require mostly dark areas (equal to Day Blackback) | Daybright require mostly white depth areas. Day Blackback require mostly black depth areas | Daybright require mostly white depth areas. Day Blackback require mostly black depth areas | NA | NA |  | [Propose standard Day Blackback. If no chart, then option blue background] |
| Dusk, night | Provision shall be made for day and night operation | As 60936 | Depth areas require mostly dark areas (Dusk , Night) | Called as Dusk, Night | Called as Dusk, Night | NA | NA |  |  |
| No data area | NA | NA | The “dash area pattern” is prohibited | Filled area Colour=nodta with dash area pattern | Filled area Colour=nodta with dash area pattern | NA | NA |  | (ECDIS Day Blackback nodta fill-pattern is too bright) |
| Radar echo | Not defined (green and yellow widely used | Not defined (green and yellow widely used) | Not defined (yellow widely used) | Green Colour=radlo, colour=radhi (shades between are permitted) | Green Colour=radlo, colour=radhi (shades between are permitted) | NA | NA |  | [Proposed: If no chart option to use Yellow with Blue background] |
| Radar echo trails | Not defined (but blue or echo colour widely used) | Not defined (but blue or echo colour widely used) | Not defined (but blue or echo colour widely used) | Green Colour=radlo, colour=radhi (shades between are permitted) | Green Colour=radlo, colour=radhi (shades between are permitted) | NA | NA |  |  |
| Use of RNC raster charts | NA | NA | Not allowed | RNC colours are not controlled. This can make it very hard to see radar overlay, AIS and ARPA symbols | Not defined |  |  |  | [Proposed: Consider allowing RNC raster charts only using grey scale to display the RNC?] |

  - Other observations

| Table B.3 - Other observations, comparing existing standards |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Topic | Radar 60936 | Radar plotting 60872 | Chart radar 60936-3 | ECDIS 61174 | IHO S52 | AIS IMO circ. 217 | Other 60945 | INS 61924 | Comment |
| Pixel resolution |  |  |  |  |  |  |  |  |  |
| Minimum symbols size | Cursor at least 10mm Radar map alphanumeric labels and symbols a size such that clearly legible; | Following in 340 mm screen ARPA target circle at least 2 mm | At least 9 pixels | CHKSYM01 at least 16 pixel | CHKSYM01 at least 16 pixel | NA | NA |  |  |
| Minimum upper case character size | 3.5 mm | As 60936 | As 60936 | As 60936 | As 60936 | NA | 3.5 mm |  | No change. Addressed in the body of 62288 sec5.2.2] |
| Text Font | Per 60945 | Per 60945 | Per 60945 | Per 60945 | Sans serif Non-italic No computer 0 . |  | Font clearly Distinguish between X and K, T and Y, I and L, I and 1, 0, O and Q, S and 5 and U and V |  | No change. Addressed in the body of 62288 sec5.2.2] |
| Non north up presentation | Not defined (head up widely used) | As 60936 | There are no definition | As S52 | Symbols shall stay upright |  |  |  | Text and rectangular symbols must remain upright. AIS symbols must maintain correct orientation. Provision must be made to prevent “twitching” due to small changes in heading. |
| Flashing | Alarmed information, Highlighted danger area may flash. | Alarmed information, Highlighted danger area may flash. | Alarmed information, Highlighted danger area may flash. | Not defined | Not defined | Lost target, dangerous target. | Alarm may flash |  |  |
| Use of grey shade for chart symbol | NA | NA | Permitted | Not allowed | Not allowed |  |  |  |  |
| Colour definitions | Common use: radar echo colours (green, yellow) | Not defined | Radar map differences from 61174: Coastline OS safety contour Fairway/channel Radar backgound(day) | Per S52 | Fully defined | Red: dangerous target | Alarm indications Red or highlighted. High contrast on a low-reflectance background which emits negligible light at night. |  |  |
| Audible alarm | Per 60945 | Per 60945 | Per 60945 | acoustic alarm level must be capable of the maximum defined in IEC 60945 but may be adjustable below this limit. | Not defined | Per 60945 | Not defined |  |  |
| Alarm colours |  |  |  |  |  |  |  | 1) Red, blinking and audible Active alarm status, not acknowledged 2) Red, static Active alarm status acknowledged 3) Yellow, static Warning message active or status below normal but not critical, may be accompanied by a short or single audible signal 4) Green or no status indication Normal status. |  |
|  | Radar map: additional symbols may be used for other navigational purposes provided that they do not conflict with the ECDIS navigational and chart symbols or radar electronic plotting symbols; | RP:If two or more symbols simultaneously apply to a target, then the symbols may be displayed together, provided that they are clearly distinguishable. |  |  |  |  |  |  |  |