IALA S-240 DGNSS Station Almanac

Product Specification

Draft  0.0.2 – March 2016

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

Document Revisions	2

1.	Overview	5

1.1	Introduction …		5

1.2	References		5

1.2.1	Normative references		5

1.2.2	Informative references		5

1.3	Terms, Definitions & Abbreviations		6

1.3.1	Terms and Definitions		6

1.3.2	Abbreviations		8

1.4	Product specification metadata		8

1.5	IALA Product Specification Maintenance		8

1.5.1	Introduction		9

1.5.2	New Edition		9

1.5.3	Revisions		9

1.5.4	Clarification		9

1.5.5	Version Numbers		9

2.	Specification Scope	10

3.	Data product identification	11

4.	Data Content and structure	12

4.1	Introduction		12

4.2	Application Schema		12

4.3	Feature Catalogue		13

4.4	Feature Types		13

4.4.1	Meta Feature Types		13

4.4.2	Geographic Feature Types		13

4.4.3	Information Types		13

4.5	Attributes		13

4.5.1	Simple Attributes		14

4.5.2	Complex Attributes		14

5.	Coordinate Reference Systems (CRS)	15

5.1	Introduction		15

5.2	Horizontal reference system		15

5.3	Projection		15

5.4	Vertical coordinate reference system		15

5.5	Temporal reference system		16

6.	Data Quality	17

6.1	Quality, Reliability and Accuracy of Data		17

7.	Data Maintenance	18

7.1	Data Encoding and Product Delivery		18

7.1.1	Data Encoding		18

7.1.2	Data Product Delivery Information		18

7.1.3	Exchange Set		19

7.1.4	Support Files		19

7.1.5	Support File Naming Convention		20

7.1.6	Dataset Naming Convention		20

7.1.7	Catalogue File Naming Convention		21

7.1.8	Dataset Maintenance		21

8.	Portrayal	22

9.	Data Product format (encoding)	23

10.	Data Product Delivery	24

10.1	Feature and Portrayal Catalogue Delivery		24

11.	Metadata	25

11.1	Introduction		25

11.2	Dataset Metadata		25

11.3	Exchange Set Metadata		27

11.4	Catalogue File Metadata		28

ANNEX A	Data classification and Encoding Guide	29

# Overview

GNSS (Global Navigation Satellite System) is a satellite system that is used to pinpoint the geographic location of a user's receiver anywhere in the world. DGNSS (Differential GNSS) is an enhancement to GNSS that provides improved location accuracy and integrity.

DGNSS uses a network of fixed, ground-based reference stations to broadcast the difference between the positions indicated by the GNSS (satellite) systems and the known fixed positions. These stations broadcast the difference between the measured satellite pseudoranges and actual (internally computed) pseudoranges, and receiver stations may correct their pseudoranges by the same amount. The digital correction signal is typically broadcast over ground-based transmitters with ranges up to 250 km.

## Introduction

The DGNSS Station Almanac is a document published and maintained by IALA, which contains information about the worldwide DGNSS stations. The DGNSS Station Almanac Product Specification provides a common structure for the exchange of information about DGNSS stations. The product contains the station name, identification numbers of reference stations and transmitting station, positions, nominal range, operation status, transmitted message types, frequency, bit rate, remarks related to a DGNSS station.

The Product Specification can be used to exchange DGNSS station information in a consistent form between IALA, Lighthouse Authorities, Hydrographic Offices and other organizations (to include commercial and professional agencies). This information can also be provided as a document converted from the dataset.

## References

### Normative references

IHO S-100 IHO Universal Hydrographic Data Model, Edition 2.0.0, June 2015

### Informative references

ITU-R M.823-3 Technical characteristics of differential transmissions for global navigation satellite systems from maritime radio beacons in the frequency band 283.5-315 kHz in Regiion 1 and 285-325 kHz in Region 2 and 3

RTCM 10402.3 RTCM Recommended Stardards for Differential GNSS(Global Navigation Satellite Systems) Service version 2.3

IALA Recommendation R-121 On the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5-325 kHz (Ed. 2.0, May 2015)

IALA DGNSS Station Almanac (2015)

IALA Guideline 1106 on Producing an IALA S-100 Product Specification (Ed. 1, December 2013)

IALA NAVGUIDE (Ed. 7, 2014)

IALA International Dictionary of Marine Aids to Navigation (2012)

## Terms, Definitions & Abbreviations

### Terms and Definitions

Terms and definitions in addition to those in Annex A of S-100 are as given in the IALA Navguide and the IALA Dictionary.

application

manipulation and processing of data in support of user requirements (ISO 19101)

application schema

conceptual schema for data required by one or more applications (ISO 19101)

conceptual model

model that defines concepts of a universe of discourse (ISO 19101)

conceptual schema

formal description of a conceptual model (ISO 19101)

coverage

feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

EXAMPLE Raster image, polygon overlay, digital elevation matrix.

data product

dataset or dataset series that conforms to a data product specification

data product specification

detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

NOTE: A data product specification provides a description of the universe of discourse and a specification for mapping the universe of discourse to a dataset. It may be used for production, sales, end-use or other purpose.

dataset

identifiable collection of data (ISO 19115)

NOTE: A dataset may be a smaller grouping of data which, though limited by some constraint such as spatial extent or feature type, is located physically within a larger dataset. Theoretically, a dataset may be as small as a single feature or feature attribute contained within a larger dataset. A hardcopy map or chart may be considered a dataset.

dataset series

collection of datasets sharing the same product specification (ISO 19115)

domain

well-defined set (ISO/TS 19103)

NOTE: Well-defined means that the definition is both necessary and sufficient, as everything that satisfies the definition is in the set and everything that does not satisfy the definition is necessarily outside the set.

feature

abstraction of real world phenomena (ISO 19101)

NOTE:  A feature may occur as a type or an instance. Feature type or feature instance shall be used when only one is meant.

feature association

relationship that links instances of one feature type with instances of the same or a different feature type (ISO19110)

NOTE 1; A feature association may occur as a type or an instance. Feature association type or feature association instance is used when only one is meant.

NOTE 2: Feature associations include aggregation of features.

feature attribute

characteristic of a feature (ISO 19101)

NOTE 1: A feature attribute may occur as a type or an instance. Feature attribute type or feature attribute instance is used when only one is meant.

NOTE 2: A feature attribute type has a name, a data type and a domain associated to it. A feature attribute for a feature instance has an attribute value taken from the domain.

geographic data

data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

NOTE: Geographic information is also used as a term for information concerning phenomena implicitly or explicitly associated with a location relative to the Earth.

metadata

data about data (ISO 19115)

model

abstraction of some aspects of reality (ISO 19109)

portrayal

presentation of information to humans (ISO 19117)

quality

totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

universe of discourse

view of the real or hypothetical world that includes everything of interest (ISO 19101)

### Abbreviations

IALA		International Association of Marine Aids to Navigation and Lighthouse Authorities

CRS		Coordinate Reference System

ECDIS		Electronic Chart Display and Information System

ENC		Electronic Navigational Chart

IHO		International Hydrographic Organization

IMO		International Maritime Organization

ISO		International Organization for Standardization

DGNSS           Differential Global Navigation Satellite System

WGS84           World Geodetic System 1984

## Product specification metadata

Title:		DGNSS Station Almanac

S-240 Version:          0.0.2

Identifier:  		S-240

S-100 Version:  		2.0.0

Date:  		March 2016

Language:  		English

Classification:  		001 – unclassified

Contact:		IALA-AISM

10, rue des Gaudines

78100, Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01 Fax: +33 1 34 51 82 05

URL:  		http://www.iala-aism.org/IALA-240

Maintenance:  		IALA e-Navigation Committee

## IALA Product Specification Maintenance

This chapter is for clarification only on PS Maintenance.

### Introduction

Changes to a product specification will be released by IALA-AISM as a new edition, revision, or clarification.

### New Edition

New editions of a product specification introduce significant changes. New editions enable new concepts, such as the ability to support new functions or applications, or the introduction of new constructs or data types.

### Revisions

Revisions are defined as substantive semantic changes to a product specification. Typically, revisions will change a product specification to correct factual errors; introduce necessary changes that have become evident as a result of practical experience or changing circumstances. A revision must not be classified as a clarification. Revisions could have an impact on either existing users or future users of a product specification. All cumulative clarifications must be included with the release of approved corrections.

Changes in a revision are minor and ensure backward compatibility with the previous versions within the same edition. Newer revisions, for example, introduce new features and attributes. Within the same edition, a data product of one version could always be processed with a later version of the feature and portrayal catalogues.

### Clarification

Clarifications are non-substantive changes to a product specification. Typically, clarifications:

remove ambiguity; correct grammatical and spelling errors; amend or update cross references; insert improved graphics, spelling, punctuation and grammar. A clarification must not cause any substantive semantic change to a product specification.

Changes in a clarification are minor and ensure backward compatibility with the previous versions within the same edition. Within the same edition, a data product of one clarification version could always be processed with a later version of the feature and portrayal catalogues, and a portrayal catalogue can always rely on earlier versions of the feature catalogues.

### Version Numbers

The associated version control numbering to identify changes (n) to a product specification must be as follows:

- New editions denoted as n.0.0

- Revisions denoted as n.n.0

- Clarifications denoted as n.n.n

# Specification Scope

This product specification describes one data product and therefore requires only one scope which is described below:

Scope ID:  		DGNSS Station Almanac datasets.

Hierarchical level: 		MD_ScopeCode - XXX

Hierarchical level name: 	dataset.

Level description:		information applies to the dataset

Extent:	EX_GeographicExtent  - Global coverage of maritime areas.
EX_TemporalExtent - Not defined for this product specification.

EX_VerticalExtent - Not defined for this product specification.

# Data product identification

A data set that conforms to this product specification will be identifiable by the discovery metadata that supports it.

Title: 				         DGNSS Station Almanac

Alternate Title:	                     IALA – DGNSS

Abstract: 	Name, Identification numbers, positions, operation status of DGNSS stations. Additional general information can also be included.

Topic Category:

Geographic Description:             Areas specific to marine and inland navigation.

Spatial Extent:		        Description: Global, marine, inland waterways

East Bounding Longitude: 180

West Bounding Longitude: -180

North Bounding Latitude: 90

South Bounding Latitude: -90

Spatial Resolution:		        DGNSS Station Almanac is scale independent

Purpose:                           The purpose of a DGNSS Station Almanac dataset is to exchange name, identification numbers and operation status of DGNSS station. Additional general information can also be included.

Language: 			          English

Classification: 		          Unclassified

Spatial Representation Type:     Vector (Point)

Point of Contact: 		        Producer

Use Limitation: 		          None

# Data Content and structure

## Introduction

A DGNSS Station Almanac product is based on the S-100 General Feature Model. A General Feature Model is a meta-model of feature types. A feature may have properties that may be operations, attributes or associations. Any feature may have a number of attributes.  A feature is not defined in terms of a single geometry, but rather as a conceptually meaningful object within a particular domain, one or more of whose properties may be geometric. The 'Feature' is the fundamental unit of geospatial information, so the Feature Model is the fundamental meta-model used for developing an Application Schema.

The following sub-sections contain the product application schema expressed in UML and an associated feature catalogue.  The feature catalogue provides a full description of each feature type including its attributes and attribute values in the data product.

## Application Schema

The UML model shown in Figure 4.1 below illustrates an application schema of DGNSS Station Almanac. It includes a general description of elements used to construct the application schema, and the relationships between them. These elements include features types, information types, simple attributes, complex attributes, aggregations and associations.  A brief description of these is provided below and the full description is included in the feature catalogue.

Figure 4.1 - Application Schema of DGNSS Station Almanac

A feature is an abstraction of real world phenomena. A certain feature type is the class used to describe all instances of that kind of real world object. In object-oriented modelling, feature types are equivalent to classes and feature instances are equivalent to objects.

An information type is an identifiable object that can be associated with features in order to carry information pertaining to the associated features.

Simple attributes can be enumerations, code lists or simple types (e.g. integer or character string).

Complex attributes are properties of a feature which can be divided into multiple sub attributes and are used where objects have properties that better fit a hierarchical structure.  They provide a better construct for encoding list attributes on objects such as light sectors.

An association is a relationship that links instances of one feature or information type with instances of the same or different feature and information types. Each relationship has a name and two roles thus giving a more detailed representation of the real world relationships within the dataset.

## Feature Catalogue

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The S-240 Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema. A Feature Catalogue in XML format shall be available from IALA with an accompanying XSL Stylesheet.

Name: DGNSS Station Almanac Feature Catalogue

Scope:

Field of application:

Version Number: 0.1

Producer: IALA

Functional Language: English

## Feature Types

### Meta Feature Types

No Meta Feature Types are included in this product.

### Geographic Feature Types

Geographic feature types form the content of the data product and are fully defined in the Feature Catalogue.

### Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

## Attributes

S-240 defines attribute as either simple or complex. Attributes guideline is shown below:

Numeric Attribute Values

Floating point or integer attribute values must not be padded by non-significant zeroes.

Text Attribute Values

Character strings must be encoded using the character set specified in Unicode Transformation Format-8 (UTF-8).

Mandatory Attribute Values

Where attributes determine the display of a Feature, or are essential to the overall feature type construction they may be mandatory. All mandatory attributes are identified in the feature catalogue.

### Simple Attributes

S-240 uses several types of simple attributes; they are listed in the following table;

Table 4.1 – Simple feature attributes

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| CharacterString | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |

### Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

# Coordinate Reference Systems (CRS)

## Introduction

The location of an object in the S-100 standard is defined by means of coordinates which relate a feature to a position. The coordinate reference system used for this product specification is World Geodetic System 1984 (WGS 84) which is defined by the International Terrestrial Reference Frame 2008(or similar - North American Datum 1983 / Canadian Spatial Reference System).

Spatial data are expressed as latitude (φ) and longitude (λ) geographic coordinates. Latitude values are stored as a negative number to represent a position south of the Equator. Longitude values are stored as a negative number to represent a position west of the Prime Meridian. Coordinates are expressed as real value, degree / degree decimal format. Datasets conforming to this product specification are not projected.

Horizontal coordinate reference system:	WGS 84

Projection: 					None

Vertical coordinate reference system: 	Although all coordinates in a data set must refer to the same horizontal CRS different Vertical Datums can be used for the depth component of a coordinate tuple. Therefore the vertical CRS can be repeated. For each Vertical CRS a unique identifier is defined. Those identifiers will be used to indicate which Vertical CRS is used. Units must be in meters.

Temporal reference system: 		Gregorian calendar

Coordinate reference system registry: 	ITRF 2008

Date type (according to ISO 19115):  	002 - publication

## Horizontal reference system

Positional data is expressed in latitude and longitude geographic coordinates to one of the reference horizontal reference systems.  Unless otherwise defined, the World Geodetic System 84 (WGS 84) will be used for S-240 data products.

## Projection

S-240 data products are un-projected.

## Vertical coordinate reference system

Although all coordinates in a data set must refer to the same horizontal CRS different Vertical Datums can be used for the depth component of a coordinate tuple. Therefore the vertical CRS can be repeated. For each Vertical CRS a unique identifier is defined. Those identifiers will be used to indicate which Vertical CRS is used. Units must be in meters.

## Temporal reference system

Time is measured by reference to Calendar dates and Clock time in accordance with ISO 19108:2002 Temporal Schema clause 5.4.4.

# Data Quality

## Quality, Reliability and Accuracy of Data

Attributes at the feature level indicate method of positioning and date of observation or confirmation of operation. These attributes are defined in the Feature Catalogue.

Positions of stations are given with a resolution of four decimal places of a degree (latitude & longitude), or the equivalent in degrees and minutes. This is sufficient for station determination in the almanac.

# Data Maintenance

S-240 products must be based on data sources released by an appropriate DGNSS Station defining authority.  Data source must be described in each data product.

The production process used to generate S-240 products may be described in the dataset metadata.

Table 7.1 — Data capture information

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| dataSource | Identification of the kinds of data sources usable to product datasets compliant with the considering specification | 0..* | CharacterString |
| productionProcess | Link to a textual description of the production process (including encoding guide) applicable to the datasets compliant with the considering specification | 0..* | CharacterString (URL) |

## Data Encoding and Product Delivery

### Data Encoding

The principal encoding will be the Open Geospatial Consortium (OGC), Geography Markup Language (GML) format. GML is an XML grammar designed to express geographical features. It serves as a modelling language for geographic systems as well as an open interchange format for geographic transactions.

### Data Product Delivery Information

This data product specification defines GML as the primary format in which S-240 data products are delivered. The delivery format information should include the following items (from ISO 19131:2005 with some changes of obligation):  format name, version, specification, language, character set.  File structure and units of delivery can also be included if required.

Table 7.1 – Data Product Delivery

| Name | ISO 19131 Elements | Value |
| --- | --- | --- |
| Format name | DPS_DeliveryInformation.deliveryFormat > DPS_DeliveryFormat.formatName | GML* |
| Version | DPS_DeliveryInformation.deliveryFormat > DPS_DeliveryFormat.version | 3.2.1 |
| Specification description | DPS_DeliveryInformation.deliveryFormat > DPS_DeliveryFormat.specification | GML* |
| Language | DPS_DeliveryInformation.deliveryFormat > DPS_DeliveryFormat.language | English English |
| Character set | DPS_DeliveryInformation.deliveryFormat > DPS_DeliveryFormat.characterSet > MD_CharacterSetCode | 004 – utf8 |

Table x -

* GML is an XML encoding for the transport and storage of geographic information, including both the geometry and the properties of geographic features, between distributed systems. The XML Schema for the GML application schema is provided in a single schema document S-240.xsd. Feature instance shall validate against S-240.xsd and conform to all other requirements specified in this data product specification including all constraints not captured in the XML Schema document.

### Exchange Set

An exchange set will consist of one or more S-240 datasets.  An exchange set may also include one or more support files containing supplementary information encoded in separate files. These are linked to the S-240 dataset features, using the attributes described below. Each exchange set will include a single (XML) catalogue file containing discovery metadata for each S-240 dataset as well as support files.

### Support Files

Support files contain ancillary textual or graphic information in separate (linked) files. Information should be encoded in a structured format as defined by W3C.  The following formats would be suitable for graphics:

- Portable Network Graphics (PNG) [Edition 2.0]

- Scalable Vector Graphics (SVG) [Edition 1.1]

- Tagged Image File Format (TIFF) [Edition 6.0]

- Joint Photographic Experts Group (JPEG) [Edition 1.02 ]

Note: PNG is an extensible file format designed for lossless, portable storage of raster images. It provides a patent-free replacement for the GIF format and also replicates many common uses of TIFF. The PNG edition 2 format has been adopted as an ISO standard, (ISO/IEC 15948:2003).  SVG is a language for describing two-dimensional graphics in XML [XML10]. SVG allows for three types of graphic objects: vector graphic shapes (e.g., paths consisting of straight lines and curves), images and text. The JPEG standard specifies the codec, which defines how an image is compressed into a stream of bytes and decompressed back into an image, but not the file format used to contain that stream. (The term "JPEG" is an acronym for the Joint Photographic Experts Group, which is the body that created the standard).

### Support File Naming Convention

All support files will have unique world-wide file identifiers. The file identifier of support information should not be used to describe the physical content of the file. The support file metadata that accompanies the file will inform the user of the name and purpose of the file (new, replacement and deletion).

In this encoding the support files are named according to the specifications given below:

AABBBCCCXXXXXXXX.YYY

The main part forms an identifier where:

- the first two characters identify the issuing agency

- the third to fifth characters are indication on the domain

- the sixth to eighth characters are indication on the product to identify that this is DGNSS Station Almanac information,

- the ninth up to the sixteenth character can be used in any way by the producer to provide a unique file name for the dataset. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- .YYY – support file extension.

### Dataset Naming Convention

All dataset files will have unique world-wide file identifiers. The file identifier of the dataset should not be used to describe the physical content of the file. The dataset file metadata that accompanies the file will inform the user of the name and purpose of the file (new, replacement and deletion).

In this encoding the dataset files are named according to the specifications given below:

AABBBCCCXXXXXXXX.GML

The main part forms an identifier where:

- the first two characters identify the issuing agency

- the third to fifth characters are indication on the domain

- the sixth to eighth characters are indication on the product to identify that this is DGNSS Station Almanac information,

- the ninth up to the sixteenth character can be used in any way by the producer to provide a unique file name for the dataset. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

### Catalogue File Naming Convention

The exchange catalogue acts as the table of contents for the exchange set. The catalogue file of the exchange set must be named CATALOG.10X. No other file in the exchange set may be named CATALOG.10X. The content of the exchange catalogue file is described in Section 16.2.1.

### Dataset Maintenance

Datasets are maintained as needed. Data updates will be made by new editions.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The maintenance and update frequency of S-240 data sets should be defined by the producers (official national authority) implementing this specification.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime.  This should specify the expected frequency of updates.

Table 7.2 - Maintenance and Update Frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

# Portrayal

A Portrayal Catalogue defines the display rules and symbology which apply to the Features defined in this specification.

The Portrayal Catalogue contains portrayal functions that map features to symbology. It also contains symbol definitions, colour definitions, portrayal parameters and portrayal management concepts such as viewing groups. Each catalogue should include a reference to a portrayal library that contains a set of portrayal rules and a set of portrayal specifications (Table 9.1). This information should be included in the product specification for each identified scope.

Table 8.1 - Portrayal Information

| Item Name | Description | M/O | Card | Type |
| --- | --- | --- | --- | --- |
| portrayalLibraryCitation | Bibliographic reference to the portrayal library | O | 0..1 | CI_Citation (ISO 19115) |

# Data Product format (encoding)

Table 9.1 – Data product format

| formatName | S-100 GML |
| --- | --- |
| version | 1.0 |
| characterSet | UTF-8 |
| specification | S-100 GML Profile 1.0 |

The S-100 GML Profile 1.0 was included in the S-100 ver. 2.0. A GML application schema .XSD document is also provided to support data validation. An example of how this encoding could be implemented is shown in Annex A.

# Data Product Delivery

Table 10.1 – Data product delivery

| Item Name | Description |
| --- | --- |
| unitsOfDelivery | Datasets |
| transferSize | Undefined |
| mediumName | Undefined |
| otherDeliveryInformation |  |

## Feature and Portrayal Catalogue Delivery

This specification include a Feature Catalogue and Portrayal Catalogue. The Feature and Portrayal Catalogues can be also provided on the IALA website and users informed by a change notice on that site when any changes occur.

# Metadata

## Introduction

The S-240 metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

## Dataset Metadata

Dataset metadata is intended to describe information about a dataset or data resource. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset (and / or data resource). Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained. Information about the documented metadata (if provided as a separate resource)

Table 11.1 – Dataset Metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| DataSetDiscoveryMetadata |  |  |  |  |
| metadataFileIdentifier | 1 |  | CharacterString | See section 15 |
| metadataPointOfContact | 1 |  | CI_ResponsibleParty | Contact information for the data production authority |
| metadataDateStamp | 1 |  | Date | When the dataset was created |
| metadataLanguage | 1..2 | English or French | CharacterString | All data sets conforming to this PS must use English and optionally French |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString |  |
| dataProtection | 1 | {1} or {2} | CharacterString | 1. Encrypted 2. Unencrypted |
| purpose | 1 | {1} or {2} | CharacterString | 1. New dataset 2. Terminated |
| specificUsage | 0..1 |  | CharacterString | brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 | S-240 version XXX | CharacterString | This must be encoded as S-240 XXX |
| producingAgency | 1 |  | CI_ResponsibleParty | Party responsible for generating the dataset. |
| horizontalDatum | 1 | WGS84 | CharacterString | The datum for latitude/longitude. EPSG:4326 |
| verticalDatum | 1 | WGS84 | CharacterString | EPSG:4326 |
| dataType | 1 | GML 3.2.1 | CharacterString |  |
| boundingBox | 0..1 |  | EX_GeographicBoundingBox | minimum bounding rectangle within which data is available |
| boundingPolygon | 1..* |  | EX_BoundingPolygon | boundary enclosing the dataset, expressed as the closed set of (x,y) coordinates of the polygon (last point replicates first point) |
| geographicDescription | 0..1 | e.g. Norway | EX_GeographicDescription | description of the geographic area within which data is available |
| comment | 0..1 |  | CharacterString | Any additional Information |

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 2 (terminated), and the editionNumber metadata field is set to 0.  All other metadata fields must be blank.

Note 3: The implication of only updating by new edition is that, if a support file is terminated, a new edition of the dataset is required.

## Exchange Set Metadata

Frequently datasets are packaged and distributed as composite exchange sets by third party vendors. An exchange set could contain many different types of datasets, sourced from different data producers. For example an exchange set may contain numerous dataset files, ancillary data files, discovery metadata files and others.  Exchange set metadata contains metadata about the contents of the exchange set and metadata about the data distributor.

Figure 11.1 - Exchange Set Metadata

## Catalogue File Metadata

All S-240 Catalogue metadata files must contain at least the following metadata elements.

Table 11.2 - Catalogue File Metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| abstract | 0..1 |  | CharacterString | Description of what the exchange catalogue contains |
| AlgorithmMethod | 0 ..1 | e.g. ZIP Compression | CharacterString | algorithms or processes that are applied to read or expand resources to which compression techniques have been applied |
| metadataFileIdentifier | 1 |  | CharacterString | See section 15 |
| metadataPointOfContact | 1 |  | CI_ResponsibleParty | Contact information for the data production authority |
| metadataDateStamp | 1 |  | Date | Date when the dataset was created |
| metadataLanguage | 1..2 | English, French | CharacterString | All data sets conforming to this PS must use English language. The catalogue file must be in English with the optional addition of French. |
| name | 1 | CATALOG.XXX | CharacterString | Catalogue filename |
| path | 1 | URI | CharacterString | Path to filename |
| productSpecification | 1 | S-122 | CharacterString | Product specification Version Number |
| comment | 0..1 |  | CharacterString | Any additional Information |

# ANNEX A	Data classification and Encoding Guide

| Definition: DGNSS STATION REGION Information about a region that each DGNSS station is located |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-240 Information Feature: DgnssStationRegion |  |  |  |  |  |
| Primitives: None |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-240 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Date of issue |  |  |  | (S) DA | 1,1 |
| Date of last update |  |  |  | (S) DA | 1,1 |
| country |  |  |  | (S) TE | 1,* |
| Encoding Guide |  |  |  |  |  |

| Definition: DGNSS STATION ALMANAC Information about each DGNSS station |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-240 Information Feature: DGNSSStationAlmanac |  |  |  |  |  |
| Primitives: None |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-240 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Bit rate |  |  |  | IN | 1,1 |
| Signal Frequency |  |  |  | RE | 1,1 |
| Nominal Range At |  |  |  | IN | 1,1 |
| Nominal Range Km |  |  |  | IN | 1,1 |
| Radiobeacon Health |  | 1 : Radio Beacon Operation Normal 2 : No Integrity Monitor Operating 3 : No Information Available 4 : Don’t Use This Radio Beacon |  | EN | 1,1 |
| Reference Station IDs |  |  |  | (S) TE | 0,2 |
| Station Name |  |  |  | (S) TE | 1,1 |
| Transmitted Messages Types |  | 1 : Differential GPS Corrections 2 : Delta Differential GPS Corrections 3 : GPS Reference Station Parameters 4 : Reference Station Datum 5 : GPS Constellation Health 6 : GPS Null Frame 7 : DGPS Radio beacon Almanac 8 : Pseudolite Almanac 9 : GPS Partial Correction Set 10 : P-Code Differential Corrections 11 : C/A-Code L1L2 Delta Corrections 12 : Pseudolite Station Parameters 13 : Ground Transmitter Parameters 14 : GPS Time of Week 15 : Ionospheric Delay Message 16 : GPS Special Message 17 : GPS Ephemerides 18 : RTK Uncorrected Carrier Phases 19 : RTK Uncorrected Pseudoranges 20 : RTK Carrier Phase Corrections 21 : RTK/Hi-Accuracy Pseudo range Corrections 22 : Extended Reference Station Parameters 23 : Antenna Type Definition Record 24 : Antenna Reference Point(ARP) 27 : Extended Radio beacon Almanac 31 : Differential GLONASS Corrections 32 : Differential GLONASS Reference Station Parameters 33 : GLONASS Constellation Health 34 : GLONASS Partial Differential Correction Set 35 : GLONASS Radio beacon Almanac 36 : GLONASS Special Message 37 : GNSS System Time Off set 59 : Proprietary Message 60 : Multipurpose Usage |  | EN | 1,* |
| Transmitting Station ID |  |  |  | (S) TE | 0,1 |
| Encoding Guide |  |  |  |  |  |

| Definition: SUPPLEMENTARY INFORMATION Information about a feature or a number of features additional to that able to be encoded using other feature attributes. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-240 Information Feature: SupplementaryInformation |  |  |  |  |  |
| Primitives: None |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-240 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Information |  |  |  | C | 1,* |
| Language |  |  |  | (S) TE | 0,1 |
| Text |  |  |  | (S) TE | 1,1 |
| Pictorial Representation |  |  |  | (S) TE | 0,1 |
| Textual Description |  |  |  | C | 1,* |
| File reference |  |  |  | (S) TE | 1,1 |
| Language |  |  |  | (S) TE | 0,1 |
| Encoding Guide |  |  |  |  |  |

| IHO Definition: RADIO STATION. A place equipped to transmit radio waves. Such a station may be either stationary or mobile, and may also be provided with a radio receiver. In British terminology, also called w/t station. (IHO Dictionary – S-32). |  |  |  |
| --- | --- | --- | --- |
| S-240 Geo Feature: Radio station (RDOSTA) |  |  |  |
| Primitives: Point |  |  |  |
| Real World | Paper Chart Symbol | ECDIS Symbol |  |
| S-101 Attribute | Allowable Encoding Value | Type | Multiplicity |
| Call sign |  | TE | 0,1 |
| Category of radio station | 5 : radio direction-finding station 8 : Decca 9 : Loran C 10 : Differential GNSS 11 : Toran 12 : Omega 13 : Syledis 14 : Chaika (Chayka) 15 : radio telephone station 16 : AIS base station | EN | 0,1 |
| Communication channel |  | TE | 0,* |
| Estimated range |  | RE | 0,1 |
| Feature name |  | C | 0,* |
| Display name |  | (S) BO | 0,1 |
| Language | ISO 639-3 | (S) TE | 0,1 |
| Name |  | (S) TE | 1,1 |
| Fixed date range |  | C | 0,1 |
| Date end | ISO 8601:1988 | (S) DA | 0,1 |
| Date start | ISO 8601:1988 | (S) DA | 0,1 |
| Periodic date range |  | C | 0,* |
| Date end | ISO 8601:1988 | (S) DA | 1,1 |
| Date start | ISO 8601:1988 | (S) DA | 1,1 |
| Signal frequency |  | IN | 0,1 |
| Status | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private | EN | 0,* |
| Scale minimum | See clause X.X | IN | 0,1 |
| INT 1 Reference: S 10-16 ### Radio stations (see S-4 – B-480-484) Transmissions from radio stations may provide mariners with a line of position. Most radio position fixing systems require Radio Direction Finding (RDF) equipment to determine the bearing of the transmitting device; such equipment is generally no longer fitted on vessels. The exception is “emergency use only” VHF-based direction finding services (which do not use RDF equipment. Consequently, the following radio position-fixing stations are now obsolete and there is no longer any value in encoding them on ENCs: - Circular (non-directional) (RC), directional (RD) and rotating pattern (RW) marine radiobeacons; - Consol beacons (Consol); - Aeronautical radiobeacons (Aero RC); - Radio direction-finding stations (except VHF-based emergency stations) (RG); - Coast Radio Stations providing ‘QTG’ service (R). The feature “radio station” is used to encode the point of transmission of the signal. If it is required to encode a radio station, it must be done using the feature Radio Station. Remarks: - The Radio Station must only be used to encode the technical equipment itself, independent of the building or structure in which it is installed. If it is required to encode the building or structure (e.g. mast, tower, radar dome), it must be done using an appropriate feature (e.g. Building, Landmark). - Further information (e.g. transmission characteristic) may be encoded using complex attributes information or textual description. - Each VHF-channel should be indicated, using the attribute communication channel (see clause X.X). - If it is required to encode a DGPS station, it must be done using Radio Station, with attribute category of radio station = 10 (Differential GNSS). - Where required, the attribute signal frequency must be quoted in Hertz, e.g. a signal frequency of 950 MHz must be encoded as 950000000. ### Radio direction-finding stations (see S-4 – B-483) If it is required to encode a radio direction-finding station, it must be done using a Radio Station feature, with attribute category of radio station = 5 (radio direction-finding station). The identification signal may be encoded using the attribute call sign. Remarks: - Direction–finding is now only provided as an emergency service by VHF. Distinction: Radar station; radio calling-in point. |  |  |  |