AISM Association Internationale de Signalisation Maritime     IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

Working Towards a

IALA Guideline

[Number]

On

The support of the Common Shore-based System Architecture (CSSA) for the Maritime Service Portfolios (MSPs) and for the ‘Maritime Cloud’

Edition 1.0

[xxxx]

Initial Release

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01    Fax: +33 1 34 51 82 05

e-mail: contact@iala-aism.org       Internet:  www.iala-aism.org

This needs to be updated to the new IALA branding template

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| 2015-April | All sections | Initial discussion at ENAV16 |

Table of Contents

Document Revisions	1

– Application Note - The support of the CSSA for the Maritime Service Portfolios (MSPs) and for the ‘Maritime Cloud’	4

1	Introduction	4

2	Mapping of the Maritime Service Portfolios (MSPs) to the generic architectures	5

2.1	Mapping to the overarching architecture	5

2.2	Mapping to the CSSA	6

2.3	Some considerations on a transnational MSPs Registry and its interaction with stakeholders and their systems	7

2.3.1	Features of the MSPs Registry	7

2.3.2	The need for a transnational MSPs Registry	8

3	Mapping of the ‘Maritime Cloud as a technical framework’	10

3.1	The technical entities of the MC introduced	10

3.2	Mapping to the overarching architecture	12

3.3	Mapping to the CSSA	12

3.4	The functionality of ‘Application Interfaces (API)’ of MC client components	14

# – Application Note - The support of the CSSA for the Maritime Service Portfolios (MSPs) and for the ‘Maritime Cloud’

# Introduction

This Appendix is an introduction into how the CSSA supports the MSPs and the emerging concept of the ‘Maritime Cloud.’ This addresses the domain of technical services proper, namely those of technical telecommunication services, and presents an underlying digital IT framework which provides connectivity for several of the operational services defined in the MSPs. This technical framework concept is called the ‘Maritime Cloud’ (MC) and described here in an overview manner.

Note: The ‘Maritime Cloud’ should not be confused with ‘cloud computing.’

The MC is a contribution to one of the key elements of e-Navigation, namely ‘a communication infrastructure providing authorized seamless information transfer on board ship, between ships, between ship and shore and between shore authorities and other parties with many related benefits’ (IMO MSC85/26, Add. 1, Annex 20, para 4.1.3; emphasis added).

The MC consists of standards, infrastructure and governance that facilitate secure interoperable information exchange between stakeholders in the maritime community by the principles of Service Oriented Architectures (SOA). The core of the MC consists of three key infrastructural components providing central framework services (compare following Table A1).

Of course, any and all telecommunications services providing the actual physical links in the maritime domain, namely satellite, cellular, VHF-data, AIS, etc., are required to support the MC concept regarding the physical transmission of digital data.

| Maritime Service Portfolio Registry Encounter point for those that consume, provide or specify services in the maritime domain. It enables service standardization, and automatic service provision and discovery. | Maritime Identity Registry Provides all maritime stakeholders with a basic Maritime Identity and basic methods for authentication, integrity and confidentiality in information transfer through the use of digital certificates in a Public-Key Infrastructure (PKI). | Maritime Messaging Service Geo-aware messaging service taking into account the needs of ships in terms of achieving interoperability across varying data links with varying availability, technical characteristics and limited bandwidth. Allows geo-casting, a broadcasting method addressing receivers within a certain geographical area. |
| --- | --- | --- |

Table A1: the three key infrastructure components of the Maritime Cloud

# Mapping of the Maritime Service Portfolios (MSPs) to the generic architectures

## Mapping to the overarching architecture

The MSPs are directly defined in the overarching architecture which are to be eventually defined in any relevant detail by IMO by means of an envisaged IMO Resolution on Maritime Service Portfolios (MSPs) (compare SIP, Task 17 (NCSR1/28, Annex 7, Table 7; as adopted by MSC94)).

Figure A1-1: Mapping of the MSPs to the overarching architecture

Figure A1-1 shows the MSPs highlighted (in dark red). Since the MSPs are, from an architectural point of view, purposeful bundles of services, both operational and/or technical, the services and the associated information requests associated with the MSPs are also highlighted (in light red):

- In a general and simplistic sense the operational services are requesting the technical services.

- All technical services deliver the technical functions as requested, using their specific means, e.g. their functional and physical links to the vessels as shown in the picture.

- Also note that for simplicity’s sake the client-server-relationships amongst different operational services on one hand and amongst different technical services on the other hand are not shown here. The latter will come to the fore when considering further candidate solutions.

- The field of the MSPs is presently internationally only defined at its highest level (compare SIP, paras 17ff+Annex 2 (NCSR1/28, Annex 7; as adopted by MSC94)). This level of representation does not directly support the system design of those systems supporting the respective MSPs. In order to arrive at a (nearly) complete and also transparent derivation chain of client-server-relationships, i.e. requirements and fulfillment functionality descriptions between the different services within the MSPs, thus constituting the internal structure of the MSPs, more work is required. Obviously, the internal structure of the MSPs, encapsulated by the simple dotted line in Figure A1-1, exhibits a certain degree of complexity.

- The organisations that provide services, e.g. by employing the CSS, are service providers.

Note: The “Transnational Maritime Service Portfolio Registry” will be discussed in the following sections when addressing the shore-side.

## Mapping to the CSSA

An appropriate IT representation of the MSPs is indispensable to use the MSPs concept in any IT system, i.e. no IT-based functionality can be based on the above conventional definitions of MSPs directly. Rather, the MSPs need to be translated

- into a MSPs related contribution to the CMDS, i.e. into a MSPs meta-level data model in accordance with IHO S-100 standards, as stipulated by IMO, and

- into associated functions and algorithms of the technical entities dealing with this meta-level description correspondingly on the ship-board side and on the shore side.

Thus the MSPs functionality would be mapped to corresponding functions of both the ‘shipboard technical equipment supporting e-Navigation’ as well as the CSS.

On the shore side the MSPs meta-level data model as a whole would reside in a technical service within in the group of the Value-added Data Processing services, in a dedicated Maritime Portfolio Registry Service (MPR) (compare Figure A1-2).

This technical service would keep and maintain

- an own CSS’s copy of the MSPs master meta-level data model as harmonized by the competent transnational body (international or regional),

- an own CSS’s copy of any relevant MSPs meta-level data models of different individual relevant providers (e.g. in neighbouring countries or even in the same country),

- but may be in a position to amend that meta-level data in accordance with the own CSS’s service provision situation at run-time.

This technical service would deliver the relevant MSPs meta-level data

- to own CSS’s portrayal and display as status information to those primary users supported directly by own CSS, e.g. at a VTS center, via the own User Interaction Service and/or

- to external shore-based system(s) via the Gateway Service, upon request and/or continuously, and/or

- to vessels’ systems (see above) via appropriate Data Transfer Services.

Figure A1-2: Mapping of the MSPs to the generic CSSA
(Note: Interaction with ship systems not highlighted; for this compare ‘Maritime Cloud.’)

The above own CSS’s offline copy of external MSPs meta-level data can be called ‘MSPs Registry Almanac.’ Hence, the Maritime Portfolio Registry Service of a service providers contains and administers the own CSS’ MSPs Registry Almanac.

## Some considerations on a transnational MSPs Registry and its interaction with stakeholders and their systems

### Features of the MSPs Registry

The MSPs Registry is intended to facilitate the implementation of the MSPs concept as explained above by providing a repository for the meta-level specification of operational and technical services and provisioned service instances, thereby making it a single reference point for provision and discovery of meta-level descriptions.

The MSPs Registry thus contains generic service specifications from a data modelling point of view contained in relevant international standards and also provisioned service instances implemented according to a service specification.

The MSPs Registry aims at improving the visibility and accessibility of available maritime information and services. This enables service providers, consumers, and regulatory authorities to share a common view on service standards and provisioned services (compare Figure A1-3).

Figure A1-3: The different access roles of stakeholders to the MSPs Registry

As depicted in Figure A1-3, the MSPs Registry enables the ‘provider’ to ‘publish’ information related to its service instances so that the ‘consumer’ is able to ‘discover’ them and obtain everything (e.g. interface information) required to ultimately use those services.

The MSPs Registry supports some of the cornerstones of SOA: Service loose coupling, abstraction, reusability, autonomy, composability, discoverability and standardized service contracts.

The MSPs Registry does not provide maritime information but a meta-level specification of services and the information/data they carry, and the technical means to obtain it. The MSPs Registry provides the mechanisms to manage the life cycle of meta-level service specifications and service instances, from a data modelling point of view.

### The need for a transnational MSPs Registry

The MSPs concept is an integral part of IMO’s e-Navigation strategy because it has the potential to massively contribute to the harmonisation which is the most fundamental goal of that strategy. To exploit that potential, a transnational MSPs Registry is required considering the pre-dominantly national scope of shore-based service providers today. Ideally, transnational would mean international, i.e. global, though, to achieve the maximum harmonisation.

Consequentially, IMO has asserted the role to govern the definition of the MSPs by taking ‘initial action’ (compare Table 9 of SIP (NCSR1/28, Annex 7, Table 9; as adopted by MSC94)). When this action will have been fulfilled, there will have been developed the concept of an International MSPs Registry, i.e. one globally visible MSPs Registry which would contain at least the generic meta-level service descriptions and the meta-level descriptions of the information/data these services provide or handle, as explained above.

The Maritime Portfolio Registry Services of the CSSs of the various providers of any service to shipping would download these generic meta-level descriptions, i.e. would create their respective MSPs Registry Almanacs, and would use them for definition and deployment of their instances of those services in their respective service areas.

An International MSPs Registry may not be easy to implement from the outset for a completely new paradigm like the MSPs. Hence, a migration path towards such an International MSPs Registry may need to be considered. This migration path would lead, for instance, via Regional MSPs Registries in different regions across the world, while regions setting up those MSPs Registries would strive to harmonise their meta-level descriptions from the outset. Considering e.g. Europe as a region, this would render an European MSPs Registry. As another example, considering only the Asia-Pacific, would render an Asia-Pacific MSPs Registry.

Considering the massive efforts needed to set up all the generic meta-level service descriptions to be contained in any (generic) MSPs Registry and considering also the fact that e-Navigation is most desired by those regions where there is a high demand due to the traffic situation present and future, it appears prudent to start with setting up a regional MSPs Registry as a first step for a much broader international development to come and finally replace the regional MSPs Registry.

Note: In addition to the above, the individual service providers’ instances of their MSPs may also be uploaded to a transnational or even international location, thus creating a collection of most likely unrelated MSPs instances which may be useful for retrieval purposes. But that is neither a requirement nor a replacement for the desired Internationally MSPs Registry.

# Mapping of the ‘Maritime Cloud as a technical framework’

## The technical entities of the MC introduced

The framework of the ‘Maritime Cloud (MC)’ is used as the technical basis for several other applications in the context of e-Navigation. Its major goal is to allow for seamless optimum communications between all participating maritime stakeholders’ technical systems, i.e. the MC’s major goal is to provide the required technical communication connectivity for participating maritime stakeholders.

The MC – from an architectural or ontological point of view – is a technical framework that brings together the functionality of several technical entities in a coordinated and harmonised way, and – by doing so – achieve synergy. The technical entities brought together by the MC are specifically:

- The technical telecommunications means as such, i.e. those technologies which provide the physical and functional links between ships, ship-shore, shore-ship, and shore-shore, including their shipboard and shore-based technology-specific terminal equipment, namely radio stations or base stations proper together with all necessary HF radio front end installation like antennas.

  - This terminal equipment would need to have standardised M2M interfaces to connect, in particular, to the ‘Maritime Cloud client component' as follows. Hence, the technology specific terminal equipment is not the same as this MC client component and should not be confused with it.

  - Each communication technology will impose technology and situation specific limitations in terms of restrictions to capabilities, bandwidth availability, size of transferrable data packages, latencies, etc. – but basic transfer of text or structured data will be possible.

- The ‘Maritime Cloud client component’: The component makes it possible to keep the MC abstracted from the physical components and encapsulates the complexities of communication roaming. The component will function as a local information hub, connected to relevant sensors, HMIs and communication equipment.

The main tasks of the MC client component would be – in a transmitter-to-recipient order of appearance –

- on the transmitter side the compilation of appropriate messages from received input data,

- the selection of the optimum physical telecommunications route for the transmission of the compiled message based on specific selection criteria,

- the establishment of a functional link via the selected (optimum) physical telecommunications route,

- the release of the compiled message via the established functional link to physical transmission,

- the decomposition of the received message on the recipient’s side and

- forwarding of the received data to the entity addressed as the recipient.

Summarizing, the MC client component is intended to ensure seamless information transfer across different communication links in a carrier agnostic manner.

The MC client component protocol will be based on internet connectivity, yet any number of alternative telecommunication technologies could in principle be connected to and utilized either by directly connecting to the appropriate technology specific terminal equipment or via dedicated gateways.

Thus, when a maritime actor wishes to transfer information to another maritime actor not within range of a compatible direct telecommunication link, or in need of multicasting information to a group of actors not within range of one single telecommunication link, the MC client component can ensure delivery across whichever telecommunication link is currently active to each relevant actor.

In case an actor temporarily has no active communication link, the MC client component will function as a prioritized store-and-forward queue of messages where the validity period can be defined on messages. Through mechanisms of protocol level acknowledgements, the delivery of information via the MC client component can be quality assured.

These mechanisms introduced above require that each actor in the MC maintains a persistent connection or regularly establishes a connection via their own MC client component to the MC client components of other actors, in particular to the MC client component of shore-based providers.

- A new ‘Maritime Identity (MI)’ to be stored in a ‘Maritime Identity Registry (MIR)’: The identity of a ship is often addressed in terms of a ship’s name and IMO number. On telecommunication systems, the identity of a ship may be a call sign, MMSI number or system specific terminal number. These identifiers are however just numbers – and there is no guarantee that a signal identified by a specific call sign or MMSI number corresponds correctly to a unique ship. None of these identity systems or registers takes into account the need for dealing with actors who are not ships and don’t necessarily have their own radio station, such as ship owners or service providers.

The MC will need a MI stored for visibility in a MIR, enabling access to:

- to other MC client components of the various marine actors;

- the (transnational) MSPs Registry for downloading own MSPs Registry Almanac;

- certificates in a Public-Key Infrastructure (PKI) that enable secure data communication with other maritime stakeholders over any communication channel.

All actors may maintain their own contact information (such as VHF working channel, e-mail address, Phone or Fax No., etc.), while other attributes may origin from authoritative registers (such as IMO number or MMSI number), and there may be even introduced a new brand of maritime identifier, i.e. the MI. This way the MIR will provide updated ‘phone book’ contact information readily available to SAR and VTS authorities, or to other maritime professionals if marked as ‘public.’

A development regarding the transnational nature of the MI and of the MIR like in the case of the MSPs Registry above needs to take place, ideally resulting in an internationally i.e. globally defined MI and internationally i.e. globally set-up MIR. However, there needs to be defined, again similarly to the MSPs Registry a migration path which may lead via regional MI/MIRs or via internationally defined MIs with regional MIRs.

Similarly to the MSPs Registry Almanac (see above), the MIR can be downloaded by actors and stored in their local systems, thus rendering a MIR Almanac. It is an offline digital version of the public parts of MIR. It will function as a ‘phone book’ of registered maritime actors and allow offline use of central framework services like service discovery and secure communication. The MIR Almanac will limit the need for especially mobile actors to search online for contact information, but rather to update the publication upon request, or at regular intervals, when communication links are available at low cost.

- The MSPs Registry and the associated MSPs Registry Almanac: see previous chapter.

These entities brought together by the framework of the MC exist or would exist (if only postulated at this point in time) otherwise independently of the MC: I.e. the entities introduced above do not ‘need’ the MC, but the MC ‘needs’ them to achieve its (full) capabilities.

By bringing them together in the framework of the MC, the details of technical communications can be encapsulated towards the requesting operational service(s) to a certain degree, ideally to the extent that the technical communications process appears to be ‘invisible’ to the requesting operational service(s) and in particular the shipboard or shore-based user(s).

Also, benefits regarding the quality parameters of the technical communications may be achieved, e.g. reliability improvements of message transmission due to selection of best available technical communication path for the message.

## Mapping to the overarching architecture

Bringing the above entities together renders the mapping of the MC on the level of detail of the overarching architecture as depicted in Figure A1-4. Note that more detail will need to be added when zooming in on the shipboard and shore sides.

Figure A1-4: Mapping of the MC to the overarching architecture

## Mapping to the CSSA

When zooming to the shore-based technical environments, the details of the MC client components become visible. The MC client component for the shore-side would be a dedicated ‘Maritime Messaging Service (MMS)’ of the CSS (compare Figure A1-5). The above description of the MC client component would be applicable, as far as relevant for the shore side.  In particular, the MMS would host the shore-side MIR Almanac. Figure A1-5 introduces the MMS amongst the Value-added Data Processing services.

Figure A1-5: Mapping of the MC to the generic CSSA

While the terminal equipment of the various telecommunications technologies employed by own CSS of a provider would reside in the Data Collection and Data Transfer Services, external connectivity provided by external telecommunications providers, such as satellite communications with traffic objects, would be accessed through the Gateway Service.

The shore-based Maritime Messaging Service needs to maintain geographical awareness of the mobile actors. The geographic awareness enables geo-casting – i.e. actors may logically ‘broadcast to’ or ‘listen to’ an area around their own position, regardless of which communication link is used for broadcasting or listening in to the broadcast. This may be achieved by a position update sent by the mobile actor (i.e. by its MC client component) to the Maritime Messaging Service directly. Alternatively, the Maritime Messaging Service may acquire the position of mobile actors by sensors within the own CSS, like its own Radar and/or AIS Services, providing high resolution but requiring no additional communication. Yet another alternative would be for the Maritime Messaging Service to connect via the Gateway Service to a long range telecommunication service provider, such as an satellite based telecommunication service.

Certainly, there would be a need for the shore-based user to configure the MC client component according to needs and to receive appropriate status information on a HMI. This would reside in the User Interaction Service.

For a description of the MSPs part of the mapping of the MC compare description above.  For simplicity’s sake the connectivity of the Maritime Portfolio Registry Service is shown only simplified.

## The functionality of ‘Application Interfaces (API)’ of MC client components

When interfacing a MC client component to other entities of own system (either shipboard or shore-based), it would need to have an ‘Application Interface (API).’ The API of any MC client component will provide services for

- security through online use of the MIR or offline use provided by their copy of the MIR, i.e. their MIR Almanacs;

- service discovery through online use of the MSPs Registry or offline use provided by their MSPs Registry Almanacs;

- provision of dynamic functionality to another participant of the MC, e.g. sending relevant data;

- communication through generic communication primitives seamlessly roamed to appropriate available communication systems based on a user defined rule base.

Note that the above ‘services’ of the API have been called ‘Basic Services’ (of a shore-based technical service) in the context of the CSSA to avoid confusion with the notion of services within the MSPs.

It should be noted that there are complementary points of view for service specification, for instance the physical implementation point of view, prompting life cycle management concepts such as ITIL V3. Those views need to and can be harmonized.