COMSAR 8/13/..

# MEASURES TO ENHANCE MARITIME SECURITY

Long-range identification and tracking of ships

Submitted by the International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA)

Summary

Executive summary:	The document proposes improvement in long-range identification and tracking by making use of some of the information which is already available in the AIS.

Actions to be taken:	Paragraph 6

Related documents:	IMO Diplomatic Conference on SOLAS (December 2002) resolutions 3 and 10, NAV49/19 Annex 13

1     IALA noted the Draft Recommendation on Functional Requirements for Long-range Identification and Tracking of Ships, as last amended by NAV (NAV49/19 Annex 13).  As an association whose members are expected to discharge security responsibilities, IALA wishes to submit to the COMSAR Sub-Committee the following comments for consideration.

2     The Draft Recommendation requires that the long-range identification and tracking system should provide the competent authority of a State with the identity, position, data/time of position, course and speed of the ship.  This limited information may not be sufficient to support an assessment for security purposes or to make the long range tracking picture comprehensible.

3    To support an initial assessment of the intentions of a ship and to identify those ships which merit further scrutiny, shore authorities will require the following additional information:

- Destination

- Estimated time of arrival

- Types of hazardous cargo being carried

In order for shore authorities to receive, evaluate and react accordingly, this necessary information will have to be provided in sufficient time and at appropriate intervals.  Neither AIS nor any other installed long-range communication system is capable of meeting these requirements as a standalone system.  However, AIS in concert with a long range communication system, such as INMARSAT-C, will meet these requirements.

In order for shore authorities to discharge their full security responsibilities they may require further detailed information not necessarily available from AIS.

4     AIS, in the context of long-range identification and tracking, can be used as a data collector and message generator for certain relevant information concerning the ship. AIS transmits messages over short distances by VHF, but also has an output enabling transmission of specific messages with selected information over longer distances when linked to a long-range communication system.  These AIS messages have already been defined and can therefore readily be generated by the AIS and transmitted once the AIS is connected to a long-range communication system.

5    AIS has an output that can be used for connection to a long-range communication system, requiring only physical connection and software.  With this connection between the AIS and the long-range communication system the competent authority can also access other information about the ship available in the AIS automatically without further involving and burdening the crew.

Action requested of the Sub-Committee

6    The Sub-Committee is invited to consider these comments and to decide as appropriate.