# PERFORMANCE STANDARDS FOR AUTOMATIC IDENTIFICATION SYSTEMS

### 1	OBJECTIVES OF AUTOMATIC IDENTIFICATION SYSTEMS

AIS is intended to enhance: safety of life at sea; the safety and efficiency of navigation; and the protection of the marine environment. SOLAS regulation V/19 requires that AIS exchange data ship-to-ship and with shore-based facilities. Therefore, the purpose of AIS is to help identify vessels; assist in target tracking; simplify information exchange (e.g. reduce verbal mandatory ship reporting); and provide additional information to assist situation awareness. In general, data received via AIS will improve the quality of the information available to the OOW, whether at a shore surveillance station or on board a ship. AIS should become a useful source of supplementary information to that derived from navigational systems (including radar) and therefore an important ‘tool’ in enhancing situation awareness of traffic confronting users.

AIS should:

.1 provide automatically to appropriately equipped shore stations, other

vessels and aircraft information, including the craft’s identity, type,

position, course, speed, navigational status and other safety-related

information;

.2 receive automatically such information from similarly fitted vessels

and craft;

.3 monitor and track vessels; and

.4 exchange data with shore based facilities.

The requirements of 13.15.2 should not be applied to cases where

international agreements, rules or standards provide for the protection of navigational information.

AIS should be operated taking into account the guidelines developed by the

Organization.

### 2	APPLICATION

These performance standards specify the requirements for AIS for use in a Vessel Traffic Service.  The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

- in a ship-to-ship mode for collision avoidance;

- as a means for littoral States to obtain information about a ship and its cargo; and

- as a VTS tool, i.e. ship-to-shore (traffic management).

The AIS should be capable of providing to ships and to competent authorities, information from the ship, automatically and with the required accuracy and frequency, to facilitate accurate tracking.  Transmission of the data should be with the minimum involvement of ship's personnel and with a high level of availability.

The installation, in addition to meeting the requirements of the Radio Regulations, applicable ITU-R Recommendations and the general requirements as set out in resolution A.694 (17), should comply with the following performance standards.

Mandating AIS carriage and establishing a network to receive, process and distribute the AIS signals received from vessels will enhance safety and security and will improve a shore facility’s ability to manage traffic.  The shore facility that receives and processes the AIS signal can vary in size and mission ranging from a small boat station, a Vessel Traffic Service (VTS), a Captain of the Port (COTP), or a regional data fusion center.  Some shore facilities may need to act on the information, another may need to monitor AIS and maintain an information database or archive.  For these reasons, a nationwide network must be scalable and adaptable to a range of demands and requirements.

The network must also be capable of information exchange and distribution among several users ashore and afloat.  Government agencies outside the VTS and other commercial maritime interests all may have justifiable needs for AIS data.  The network must be able to accommodate all these users, allied services and industry partners.

Vessel Traffic Services have been established in various ports to respond to documented navigation safety needs, and the contributions of AIS as a VTS tool are well understood.  Therefore, as a matter of priority, these ports should be outfitted with AIS receiving and transmitting capability as soon as practicable.  Other ports, where there is no compelling federal need to operate a VTS, will want to take advantage of AIS capabilities in order to meet other operational demands.  AIS receiving and transmitting capability will be established in these places based on operational needs and exigency.

### 3	RELATED REQUIREMENTS INVOLVED

IMO Recommendation on Performance Standards for a Universal Shipborne

Automatic Identification System (AIS), (MSC. 74(69), Annex 3)

IMO SOLAS Convention Chapter V

ITU Radio Regulations, Appendix S18, Table of Transmitting Frequencies in the

VHF Maritime Mobile Band

ITU Recommendation on the Technical Characteristics for a Universal Shipborne

Automatic Identification System (AIS) Using Time Division Multiple Access in the Maritime Mobile Band (ITU-R M.1371)

IEC Standard 61993 Part 2: Universal Shipborne Automatic Identification System (AIS) Operational and Performance Requirements, Methods of Testing and required Test Results.
Relevant SOLAS requirements;

General resolutions, such as resolution A.694(17) - General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids;

Resolutions with more specific requirements, such as:

Resolution A.529(13) - Accuracy Standards for Navigation;

Resolution A.688(17) - Code on Alarms and Indicators;

Resolution MSC.39(63) - Adoption of amendments to the Code on Alarms and Indicators.

### 4 	DEFINITIONS

### COMPONENTS

In general, an AIS consists of:

- antennas;

- one VHF transmitter;

- two multi-channel VHF receivers;

- one channel 70 VHF receiver for channel management;

- a central processing unit (CPU);

- an electronic position-fixing system, Global Navigation Satellite System (GNSS)

receiver for timing purposes and position redundancy;

- interfaces to heading and speed devices and to other shipborne sensors;

- interfaces to radar/Automatic Radar Plotting Aids (ARPA), Electronic Chart

System/Electronic Chart Display and Information System (ECS/ECDIS) and

Integrated Navigation Systems (INS);

- BIIT (built-in integrity test); and

- minimum display and keyboard to input and retrieve data.

With the integral minimum display and keyboard unit, the AIS would be able to operate as a stand-alone system. A stand-alone graphical display or the integration of the AIS data display into other devices such as INS, ECS/ECDIS or a radar/ARPA display would significantly increase the effectiveness of AIS, when achievable.

All onboard sensors must comply with the relevant IMO standards concerning availability, accuracy, discrimination, integrity, update rates, failure alarms, interfacing and type-testing.

AIS provides:

- a built in integrity test (BIIT) running continuously or at appropriate intervals;

- monitoring of the availability of data;

- an error detection mechanism of the transmitted data; and

- an error check on the received data.

### CONNECTIONS

The connection of AIS to external navigational display systems :

The AIS can be connected either to an additional dedicated AIS display unit, possibly one

with a large graphic display, or to an existing navigational system such as radar or an electronic chart, but in the later case only as part of an integrated navigation system.

The connection of AIS to external portable navigational equipment :

It is becoming common practice for pilots to possess their own portable navigational equipment, which they carry on board. Such devices can be connected to shipborne AIS equipment and display the targets they receive.

The connection of AIS to external long-range radiocommunication devices :

AIS is provided with a two-way interface for connecting to long-range

radiocommunication equipment. Initially, it is not envisaged that AIS would be able to be directly connected to such equipment.  A shore station would first need to request that the ship makes a long-range AIS information transmission. Any ship-to-shore communication would always be made point-to-point, and not broadcast, and once communication had been established, the ship would have the option of setting its AIS to respond automatically to any subsequent request for a ship report from that shore station.

Users are reminded that SOLAS regulation V/11.10 provides that the participation of ships in IMO-adopted ship reporting systems shall be free of charge to the ships concerned.

### 5	FUNCTIONAL REQUIREMENTS

An AIS Service should be developed in such a way that it: minimizes life cycle and developmental costs, meets the operational requirements of its clients, and provides acceptable performance thresholds and optimum performance objectives.

The system should maximize the use of commercial off-the-shelf (COTS) equipment and software.  The architecture should maximize the use of open systems standards to the extent possible and use modular design to extend capabilities and accommodate the insertion of advanced technology or expand capacity.

The AIS Service should employ information network technology to connect client processes or users with the service’s reception and broadcast functions.  The system will accomplish this by providing multi-tiered service (i.e., supporting users with varying levels of equipment sophistication or software), and by supporting users afloat and ashore.  The service should be capable of conveying information on demand provided that primary AIS functions (surveillance and safety of navigation) are not impaired, and subject to operational control by the service’s management function.  The service must employ methods that assess and report the integrity of the information distributed from its reception functions.

The system should be capable of operating in a number of modes:

- an "autonomous and continuous" mode for operation in all areas. This mode should be capable of being switched to/from one of the following alternate modes by a competent authority;

- an "assigned" mode for operation in an area subject to a competent authority responsible for traffic monitoring such that the data transmission interval and/or time slots may be set remotely by that authority; and

- a "polling" or controlled mode where the data transfer occurs in response to interrogation  from a ship or competent authority.

5.1	Required functions and their availability

The AIS should comprise:

- a communication processor, capable of operating over a range of maritime frequencies, with an appropriate channel selecting and switching method, in support of both short and long range applications;

- a means of processing data from an electronic position-fixing system which provides a resolution of one ten thousandth of a minute of arc and uses the WGS-84 datum.;

- a means to automatically input data from other sensors meeting the provisions as specified in paragraph 6.2;

- a means to input and retrieve data manually;

- a means of error checking the transmitted and received data; and

- built in test equipment (BITE).

The AIS should be capable of:

- providing information automatically and continuously to a competent authority and other ships, without involvement of ship's personnel;

- receiving and processing information from other sources, including that from a competent authority and from other ships;

- responding to high priority and safety related calls with a minimum of delay; and

- providing positional and maneuvering information at a data rate adequate to facilitate accurate tracking by a competent authority and other ships.

### User interface

To enable a user to access, select and display the information on a separate system, the AIS should be provided with an interface conforming to an appropriate international marine interface standard.

### Identification

For the purpose of ship and message identification, the appropriate Maritime Mobile Service

Identity (MMSI) number should be used.

Date to be established by receiving equipment. *

Field not provided in basic message. **

As required by competent authority. ***

### Information

The information provided by the AIS should include

Static:

- IMO number (where available)

- Call sign & name

- Length and beam

- Type of ship

- Location of position-fixing antenna on the ship (aft of bow and port or starboard of centerline)

Dynamic:

- Ship's position with accuracy indication and integrity status

- Time in UTC *

- Course over ground

- Speed over ground

- Heading

- Navigational status (e.g. NUC, at anchor, etc. - manual input)

- Rate of turn (where available)

- Optional - Angle of heel (where available) **

- Optional - Pitch and roll (where available) **

Voyage related:

- Ship's draught

- Hazardous cargo (type) ***

- Destination and ETA (at masters discretion)

- Optional - Route plan (waypoints) **

Short safety-related messages

Information update rates for autonomous mode

The different information types are valid for a different time period and thus need a different update rate:

- Static information: Every 6 min and on request

- Dynamic information: Dependent on speed and course alteration

- Voyage related information: Every 6 min, when data has been amended and on request

- Safety-related message: As required

Security

A security mechanism should be provided to detect disabling and to prevent unauthorized alteration of input or transmitted data. To protect the unauthorized dissemination of data, the IMO guidelines (Guidelines and Criteria for Ship Reporting Systems) should be followed. *

### Permissible initialization period

The installation should be operational within 2 min of switching on.

5.2	Reliability, accuracy and discrimination

RELIABILITY
In this section the reliability, minimum requirements for accuracy and discrimination, and other relevant information that are necessary for the safe use of the navigational equipment should be clearly determined.
A combination of system maintainability characteristics and the use of redundancies in the design may be used to optimize reliability of the system and individual installations.  System refers to the entire set of AIS hardware and software on a national basis while an installation is a specified subset tailored to the needs of an individual port.  Each installation will have the capability to monitor, analyze, and report the condition of its components.  AIS facilities will primarily consist of off-the-shelf equipment.  Provided that operational availability requirements are met, the inherent reliability characteristics of components will be accepted.

Reliability against critical and major failures will be designed to meet the availability requirements outlined in Section II.D, below. Since the system will be dependent on commercially provided support services (i.e. GPS, communication links, electricity), alternatives that minimize the impact of the loss of those support services will be considered.

## Availability

The AIS service should be designed to provide an operational availability of at least 99.75% on a monthly basis.  The critical capabilities necessary to provide this level of service are the communications network connecting the AIS facilities and continuous operation of the electronics installed at the AIS facilities.

The system must be available 24 hours a day, 7 days a week to support a continuous maritime domain awareness picture of the entire coastal zone and to provide a communications path for transmission of meteorological, hydrological and/or other types of messages to mariners within specific regions. This requires that the system demonstrate a high level of operational availability (Ao), defined as follows:

(To + Ts)

Ao = —————————

(To + Ts + Tm + Tl + Ta)

where

To = operating time

Ts = standby time

Tm = maintenance time

Tl = logistical delays

Ta = administrative delays

The system must be designed to provide an average availability of 99.75% for all critical functions [listed in the Appendix?] on a monthly basis within a specified geographic region, such as within the area of responsibility of an Activity, Group or Station. The system design will consider the use of redundancies and/or alternate communication paths to achieve the required availability.

## Maintainability

Provided that availability requirements are met, the inherent maintainability characteristics of components will be accepted.  Maintenance may be provided through contracted services, which will be performed under a Maintenance Plan.

The maintenance plan for this system must reduce the time the system is not available to meet system availability requirements. This includes outages associated with preventative and casualty maintenance in addition to logistic and administrative delays. Redundancy of system functions will be balanced against the need to minimize maintenance. It is a goal to minimize maintenance costs, including costs associated with personnel, while providing the required level of system availability.

The system design should provide self and remote access diagnostics, to include automated system monitoring, diagnostic self-tests, and recording and reporting of major component status and system availability. The system design must be field-repairable and/or replaceable to the assembly level.

## Survivability

The system design will consider all potential failures, inherent or causative, natural or man-made, including sabotage and vandalism.  AIS installations will be designed to operate in severe weather conditions and survive extreme weather conditions (e.g. hurricane force winds) in all marine environments.

Following a Class 4 storm, the system must be fully functional within 48 hours.

5.3	Malfunctions, warnings, alarms and indications

In this section reference should be made to relevant requirements of resolution A.686(17) and resolution MSC.39(63). Depending on the individual type of the navigational equipment requirements in addition to resolutions A.686(17) and MSC.39(63) should be determined if necessary.

5.4	Software requirements

Until the Organization has adopted software requirements for navigational equipment, if necessary in this section software requirements concerning the following items should be determined for the individual navigational equipment:

demarcation of closed systems from open systems; correctness, stability and consistency of software; stability, security of certain system data; ability for a new startup of the system after a breakdown of power supply over a short period;

data and software update management; test procedures;

failure handling and release of warnings and alarms.

6	OPERATIONAL REQUIREMENTS

AIS should always be in operation at a VTS.  However, it should be noted that if a ship’s master believes that the continual operation of AIS might compromise the safety or security of his/her ship, the AIS may be switched off. This might be the case in sea areas where pirates and armed robbers are known to operate. Actions of this nature should always be recorded in the ship’s logbook together with the reason for doing so. The master should however restart the AIS as soon as the source of danger has disappeared. If the AIS is shut down, static data and voyage related information remain stored. Restart is accomplished by switching on the power to the AIS unit. Ship’s own data will be transmitted after a two-minute initialization period. In ports AIS operation should be in accordance with port requirements.

## Data

The AIS service will obtain and distribute data for VTS, but may include port security and state control, asset tracking, and maritime law enforcement.  Data will include, but not be limited to, the items specified below:

- Vessel Data.  Vessel data will be transmitted over the VHF marine band and will include data elements as required by international regulation.  Vessel static data will be reported as a vessel enters an AIS service area and can be updated through AIS polling techniques.  Transit-specific data will be provided continuously according to the reporting scheme defined in the international standard and will include vessel type, vessel position, speed over ground (SOG), and course over ground (COG).

- Voyage Data.  The AIS service will take advantage of the vessel AIS data to monitor mandatory participants and selected vessels within the service area to ensure their compliance with regulations or adherence to stated intentions.

- Anchorage Data.  The AIS service will obtain information about mandatory participant and selected vessels in or planning to enter or depart coastal zone anchorage areas, and will continuously monitor these areas.  Vessels in anchorage areas will continue to provide AIS updates at the intervals defined by international standard.

- Special Activity Data.  The AIS service may obtain information on and monitor the presence of activities in the coverage area that affect port security and vessel traffic flow.

- Oceanographic and Hydrographic Data.  The AIS may gather and disseminate relevant meteorological, oceanographic and hydrographic information within the AIS service area.

- Port Geography and Port Structures.  The AIS service will be used to provide timely information updates on coastal zone and port geography, waterway lock and bridge openings, and shore-based marine facilities as they relate to maritime safety and security throughout the AIS service area.  Geographic data will contain parameters normally included on navigational charts.

- Aids to Navigation.  The AIS will obtain and monitor the status of aids to navigation throughout the AIS service area and will report status information to users.

- Rules and Regulations.  The AIS service will be used to monitor rule and regulation compliance by mandatory participant and selected vessels.

- Incident Data. The AIS service may be used to communicate relevant traffic incident information to service users and maritime participants operating within service area.

- Record Data.  The AIS service will record vessel track data to support port safety and security mission procedures.  The AIS service will support incident analysis to clarify the causal factors and logical sequence of events surrounding an incident, and overall mission analysis to improve maritime safety and security.

### Manual input of data

The VTS may manually input the following data using an input device such as a keyboard:

- hydrographical and meteorological messages;

-

- short safety-related messages.

### Check of information

To ensure that own ship’s static information is correct and up-to-date, the OOW should check the data whenever there is a reason for it. As a minimum, this should be done once per voyage or once per month, whichever is shorter. The data may be changed only on the authority of the master.

The OOW should also periodically check the following dynamic information:

- positions given according to WGS 84;

- speed over ground; and

- sensor information.

After activation, an automatic built-in integrity test (BIIT) is performed. In the case of any AIS malfunction an alarm is provided and the unit should stop transmitting.

The quality or accuracy of any sensor data input into AIS would not however be checked by the BIIT circuitry before being broadcast to ships and other shore stations. The ship should therefore carry out regular routine checks during a voyage to validate the accuracy of the information being transmitted. The frequency of those checks would need to be increased in coastal waters.

### AIS INFORMATION RECEIVED FROM SHIPS

#### Ship’s data content

The AIS information transmitted by a ship is of three different types:

- fixed or static information, which is entered into the AIS on installation and need only be changed if the ship changes its name or undergoes a major conversion from one ship type to another;

- dynamic information, which, apart from ‘Navigational status’ information, is automatically updated from the ship sensors connected to AIS; and

- voyage-related information, which might need to be manually entered and updated during the voyage.

#### Information item Information generation, type and quality of information

Static

MMSI (Maritime Mobile Service Identity)

Set on installation

Note that this might need amending if the ship changes ownership

Call sign and name set on installation

Note that this might need amending if the ship changes ownership

IMO Number Set on installation

Length and beam Set on installation or if changed

Type of ship Select from pre-installed list

Location of position-fixing antenna

Set on installation or may be changed for bi-directional vessels or those fitted with multiple antennae

##### Dynamic

Ship’s position with accuracy indication and integrity status

Automatically updated from the position sensor connected to AIS

The accuracy indication is for better or worse than 10 m.

Position Time stamp in UTC

Automatically updated from ship’s main position sensor connected to AIS

Course over ground (COG) Automatically updated from ship’s main position sensor connected to AIS, if that sensor calculates COG

This information might not be available

Speed over ground (SOG) Automatically updated from the position sensor connected to AIS

This information might not be available

Heading Automatically updated from the ship’s heading sensor connected to

AIS

Navigational status Navigational status information has to be manually entered by the

OOW and changed as necessary, for example:

- underway by engines

- at anchor

- not under command (NUC)

- restricted in ability to maneuver (RIATM)

- moored

- constrained by draught

- aground

- engaged in fishing

- underway by sail

In practice, since all these relate to the COLREGs, any change that is

needed could be undertaken at the same time that the lights or shapes were changed

Rate of turn (ROT) Automatically updated from the ship’s ROT sensor or derived from the gyro

This information might not be available

##### Voyage-related

Ship’s draught To be manually entered at the start of the voyage using the maximum

draft for the voyage and amended as required (e.g. – result of de-ballasting prior to port entry)

Hazardous cargo (type) To be manually entered at the start of the voyage confirming whether or not hazardous cargo is being carried, namely:

DG (Dangerous goods)

HS (Harmful substances)

MP (Marine pollutants)

Indications of quantities are not required

Destination and ETA To be manually entered at the start of the voyage and kept up to date as necessary

Route plan (waypoints) To be manually entered at the start of the voyage, at the discretion of the master, and updated when required

##### Short safety-related messages

Free format short text messages would be manually entered,

addressed either a specific addressee or broadcast to all ships and shore stations

Table 1 - Data sent by ship

13 The data is autonomously sent at different update rates:

- dynamic information dependent on speed and course alteration (see table 2),

- static and voyage-related data every 6 minutes or on request (AIS responds automatically without user action).

##### Type of ship General reporting interval

Ship at anchor 3 min

Ship 0-14 knots 12 sec

Ship 0-14 knots and changing course 4 sec

Ship 14-23 knots 6 sec

Ship 14-23 knots and changing course 2 sec

Ship >23 knots 3 sec

Ship >23 knots and changing course 2 sec

Table 2 - Report rate of dynamic information

##### Short safety-related messages

Short safety-related messages are fixed or free format text messages addressed either to a specified destination (MMSI) or all ships in the area. Their content should be relevant to the safety of navigation, e.g. an iceberg sighted or a buoy not on station. Messages should be kept as short as possible. The system allows up to 158 characters per message but the shorter the message the more easily it will find free space for transmission. At present these messages are not further regulated, to keep all possibilities open.

Operator acknowledgement may be requested by a text message.

Short safety-related messages are only an additional means of broadcasting maritime safety information. Whilst their importance should not be underestimated, use of such messages does not remove any of the requirements of the Global Maritime Distress and Safety System  (GMDSS).

The operator should ensure that he displays and considers incoming safety-related messages and should send safety-related messages as required.

According to SOLAS regulation V/31 (Danger messages)

“The master of every ship which meets with dangerous ice, a dangerous derelict, or any other direct danger to navigation, or ...is bound to communicate the information by all the means at his disposal to ships at his vicinity, and also to the competent authorities...”.

Normally this is done via VHF voice communication, but “by all the means” now implies the additional use of the AIS short messages application, which has the advantage of reducing difficulties in understanding, especially when noting down the correct position.

##### Confidentiality

When entering any data manually, consideration should be given to the confidentiality of this information, especially when international agreements, rules or standards provide for the protection of navigational information.

6.1 Ergonomy

In this section general ergonomic requirements for the operation of AIS and the presentation of the information is provided.

6.2 Operational controls

Depending on the individual type of the navigational equipment, requirements in addition to resolution A.694(17), such as direct operations and automatic controls should be determined if necessary.

6.3 Presentation of information

Depending on the individual type of the navigational equipment, requirements in addition to resolution A.694(17) should be determined, such as settings (e.g. continuous, variable and default settings), maximum permissible time for operation, information to be displayed at all times and dedicated display area.
The AIS will provide timely, relevant and accurate information to users to support their decision-making processes.  The AIS also provides information to VTS and allied services to support their missions.

- Basic information needs of the VTS.  Representative users identified the following as the basic information needs for vessel tracking and port security related missions.

- Up-to-date knowledge and/or information regarding the route to be transited.

- Timely, relevant, and accurate information about other vessels within the area that might affect safety, security, or the decision making of the operational commander.

- Timely information about emergency and environmental conditions that might affect safety or the decision making of the operational commander.

- Where required, the transmission of relevant information to the mariner in a manner that does not distract from the task at hand, particularly in narrow, confined channels where there is heavy traffic.

- Existing navigation aids and tools, pilotage systems, navigation management systems, and regulations provide information to the mariner but these systems require integrity monitoring to ensure the information they impart is accurate.

b.   AIS Watchstander Decision Support System.  The AIS service shall provide automatic vessel position reports and movement information as it is received at remote sites throughout the service area.

- Integrated Operations.  Where applicable, AISs will support regional Vessel Traffic Services between adjoining VTSs, both domestic and foreign.

- Incident Analysis Support.  The AIS service shall support incident response and analysis.

- Immediately after notification of an incident, the AIS service shall alert vessels in or planning to enter the area of concern with particulars of the incident.

- The AIS service will provide for review of supporting information and events that occurred before, during, and after an incident.

- The AIS service will be able to replay a recorded incident.

- The AIS service will have the ability to generate and execute scenarios that simulate events to assist in analysis, planning, and readiness.

- Operations Support.  The AIS service may support port operations by providing information to appropriate shore facilities.  In support of incident response, the AIS service will coordinate with the port authority and provide information about traffic and the corresponding situational information.  In cooperation with the port authority, the system will monitor safety and security zone boundaries that are established in connection with an incident.

- Other Agency Support.  The AIS service will provide information to other agencies and allied services when a particular vessel of interest has been identified.

The AIS provides data that can be presented on the minimum display or on any suitable display device as described in annex 1.

### Minimum display

The minimum mandated display provides not less than three lines of data consisting of bearing, range and name of a selected ship. Other data of the ship can be displayed by horizontal scrolling of data, but scrolling of bearing and range is not possible. Vertical scrolling will show all the other ships known to the AIS.

### Graphical display

Where AIS information is used with a graphical display, the following target types are recommended for display:

Sleeping target A sleeping target indicates only the presence of a vessel equipped with AIS in a certain location. No additional information is presented until activated, thus avoiding information overload.

Activated target If the user wants to know more about a vessel’s motion, he has simply to activate the target (sleeping), so that the display shows immediately:

- a vector (speed and course over ground),

- the heading, and

- ROT indication (if available) to display actually initiated course changes.

Selected target If the user wants detailed information on a target (activated or sleeping), he may select it. Then the data received, as well as the calculated CPA and TCPA values, will be shown in an alphanumeric window.  The special navigation status will also be indicated in the alphanumeric data field and not together with the target directly.

Dangerous target If an AIS target (activated or not) is calculated to pass pre-set CPA and TCPA limits, it will be classified and displayed as a dangerous target and an alarm will be given.

Lost target If a signal of any AIS target at a distance of less than a preset value is not received, a lost target symbol will appear at the latest position and an alarm will be given.

Symbols The user should be familiar with the symbology used in the graphical display provided.

6.4	Software requirements

Until the Organization has adopted software requirements for the navigational equipment, if necessary in this section software requirements concerning the following items should be determined for the individual navigational equipment:

### 7	DESIGN AND INSTALLATION

7.1	Durability and resistance to environmental conditions

As regards environmental conditions the IEC requirements' should be applied as far as relevant. Depending on the individual type of the navigational equipment requirements concerning the durability and resistance to environmental conditions in addition to resolution A.b94(17) and the relevant IEC requirements' should be determined if necessary.
The following table illustrates the extremes of climatological and other environmental parameters within which AIS installations will function, and be capable of surviving:

| AIS Environmental Requirements |  |  |
| --- | --- | --- |
| Parameter | Operational | Survivable |
| Temperature | -25 to 50 deg. C* | -40 to 70 deg. C* |
| Humidity | 20%to 100% condensing | 1%to 100% condensing |
| Wind speed | 65kts sustained, 100kt gusts | 65 kts sustained, 140 gusts |
| Weather | Rain, fog, snow, sleet, freezing rain, other conditions that affect visibility | Rain, fog, snow, sleet, freezing rain, other conditions that affect visibility |
| Other | Salt spray, sand, dust, static discharge, and vibration. | Lightning strikes, saltwater immersion, shock. |
| *Temperature parameters shown do not include the effects of solar radiation. These effects will be considered. |  |  |

7.2 Interference

In this section reference should be made to [NAV 40125, annex 17 - General Requirements for Electromagnetic Compatibility (EMC) for Electrical and Electronic Ship's Equipment -] and as regards interference the IEC requirements' should be applied as far as relevant. Depending on the individual type of the navigational equipment, requirements in addition to resolution A.fi94(17), [NAV 44125, annex 17] and relevant IEC requirements' should be determined if necessary.

7.3	Power supply

As regards power supply, the IEC requirements' should be applied as far as relevant. Depending on the individual type of the navigational equipment, requirements in addition to resolution A.b94(17) and relevant IEC requirements' should be determined if necessary.

7.4 Installation

Depending on the individual type of the navigational equipment, requirements concerning the installation of sensors, wiring and the arrangement of the AIS equipment should be determined.

AIS Installation guidelines in IMO SN/Circ. 227, where applicable.

IEC 529 "Degrees of protection provided by enclosures (IP Code)"

IEC 721-3-6 "Classification of environmental conditions, Part 3: Classification of groups of environmental parameters and their severities; Ship environment"

IEC 945 "Maritime navigation and radiocommunication equipment and systems - General requirements, methods of testing and required test results"

7.5 Maintenance

Depending on the individual type of the navigational equipment, requirements in addition to resolution A.694(17) concerning maintenance should be determined if necessary.

### 8 	INTERFACING

Requirements concerning the correctness and unambiguity of input and output data of the individual components should be determined. Relevant existing standards' should be taken into account until the Organization has adopted relevant requirements.

### 9	BACK-UP AND FALL-BACK ARRANGEMENTS

Depending on the individual type of the navigational equipment, requirements concerning back-up and fall-back arrangements should be determined if necessary.

### 10	SAFETY PRECAUTIONS

Depending on the individual type of the navigational equipment, requirements in addition to resolution A.694(17) concerning safety precautions should be determined if necessary.

### II	MARKING AND IDENTIFICATION

Depending on the individual type of the navigational equipment, requirements in addition to resolution A.694(17) should be determined if necessary, e.g., for the identification of software.

### 12 	DOCUMENTATION

In this section the contents of the documentation (e.g. Operator's Manual, Installation Manual, Service Manual) necessary for the operation of the navigational equipment should be determined. Depending on the individual type of the navigational equipment, the documentation should contain the following items:

General:

manufacturer, type of equipment and the serial number; system description;

specification of the navigational equipment; supplementary equipment;

'For example:

National Marine Electronics Association (NMEA)

Radio Technical Commission for Maritime Services (RTCM) Radio Technical Commission for Aeronautics (RTCA) American National Standard Institute (ANSI)

Electronic Industries Association (EIA)

Installation:

physical characteristics of the navigational equipment;

requirements concerning installation (e.g. supply voltage and frequency, grounding, power consumption, heat dissipation);

safety information;

Operation:

operational controls, functions and presentation of information; startup and shutdown procedures;

tests, failures, calibration procedures and alarms; interfaces and software description;

of updating procedures; safety information;

Failure Elimination, Maintenance and Service:

required tools and materials for failure elimination and maintenance on board; access for maintenance;

preventive maintenance;

replacement, repair, failure elimination, adjustment; information concerning external service.

### 13 	MISCELLANEOUS

In this section additional requirements, which are necessary for the individual navigational equipment but could not be subsumed under another headline should be determined here.