Vessel Traffic Services Risk Mitigation Levels

The IALA Working Group on the development of the Generic Port and Waterway risk model noted the four levels of risk reduction measures proposed by the VTS Committee, but felt that these divisions did not conform to either IMO Resolution A857(20) or the IALA VTS Manual.  In their liaison note of September 2003, the Working Group requested that the VTS Committee review the proposal, taking into account the relevant IMO and IALA publications, and provide input to the next scheduled meeting of the Working Group, Oct. 2003.

The VTS Committee re-examined the original proposal and felt that the four levels of risk reduction potential for VTS are in conformance with relevant publications and are appropriate for use in risk mitigation modelling.

The original proposal was based on the VTS Committee’s consideration of the extent to which a vessel must rely on a shoreside organization to safely complete its passage.  The Committee recognizes that there are many waterways in which vessels can safely operate independently under all conditions of traffic density, traffic mix and weather likely to be encountered.  However, there are waterways with traffic congestion, physical limitations, adverse weather or vulnerable environments in which vessels are dependent on some level of interaction with persons ashore.  In the extreme case, there are areas in which vessels must rely entirely on information, advice or directions from ashore in order to complete a safe transit.

Taking the two extremes of vessels operating completely independent from shore-side influences and the situation in which navigation cannot proceed without intervention from ashore gives two end points upon which a continuum of VTS effectiveness can be established.  Within this continuum are defining intervals that describe various levels of VTS according to their respective capability to mitigate risk.  These intervals were derived based on the IALA/IMO definition of VTS and the services and operating options for VTS as described in the VTS Manual.

Using this continuum, waterways are initially divided into non- VTS areas and VTS areas; then further subdivided depending on the amount of information, advice, or instruction that is sent from the shore.  This process has resulted in a hierarchy of four levels that can be used in the course of a risk assessment or mitigation exercise to determine whether a VTS is necessary, and at what level of capability and competence.  The levels are defined as:

I	Unaided (non VTS)  Vessels navigating in these areas can safely proceed in all likely conditions of traffic and weather without any assistance from ashore.  In this scenario, the vessel can obtain all the information it needs to complete a passage by using its own on board resources, including pilots.

II	Port/Local Information (non VTS)  This case introduces a resource ashore that provides information to the bridge team.  However, services from such a resource are limited to information only.  This organization does not maintain a traffic image.  It does not interact with traffic, nor does it respond to developing traffic situations.  The organization does not participate in onboard decision-making and lacks the authority to direct the movement of a vessel.

III.  Information Service (VTS)  This level introduces a competent authority with resources to maintain a traffic image, interact with traffic and respond to developing traffic situations.  However, this level does not participate in on board decision-making and lacks the authority to direct the movement of a vessel.  All or certain classes of vessels may be required to provide information to this VTS and to establish and maintain communication during their transit.

IV.  Traffic Organization Service (VTS)  This level has all the capability of an Information Service (level III) and the authority to direct the movement of a vessel using results oriented instructions.  All or certain classes of vessels will be required to participate in this service and shall not transit the VTS area without clearance.

Information and Traffic Organization Services (levels III and IV) may have the additional capability to provide Navigation Assistance Service to individual vessels.  Navigation Assistance Service may be limited to the provision of factual navigation information to assist the onboard decision making process, or it may be a more participatory service in which the VTS provides navigational advice.

It is important to note that only levels III and IV constitute a VTS as defined in IMO Resolution A857(20) and described in IALA publications.

The VTS Committee developed these four levels based on a consideration of VTS operations and defined levels of service.  Each of these four levels describes an incremental degree of influence over marine traffic and, therefore, each level has a different impact on reducing navigation risk.  The Committee feels that applying this continuum of VTS effectiveness would be useful in a risk assessment exercise.  However, the VTS Committee cannot assign absolute values to these levels.  This can only be done in the context of the local risk assessment model being used.  Furthermore, the value of the VTS effectiveness factor would change under the influence of various and variable factors such as weather, traffic volume, or port conditions.

# Action

- The IALA Secretariat bring this paper to the attention of the next session of the Port and Waterways Risk Management Working Group.

- The Port and Waterways Risk Management Working Group is invited to use this clarification on the mitigation levels and consider how they may be used in the development of the Risk Model.