## 3.2	PROVISION OF FACILITIES

### 3.2.1	Infrastructure to support a VTS

The infrastructure of a VTS comprises the provision of operational, technical and administrative facilities and resources, all of which need to be integrated fully to provide an efficient and effective service.

### 3.2.2	Operational aspects of the infrastructure

In considering the operational aspects of the infrastructure of a VTS it is first necessary to decide a centralised VTS system which gathers information and disseminates from one central location or a decentralised VTS system which gathers information and disseminates it from at least two locations.

In making the decision on whether a centralised or decentralised system should be provided, technical, financial and environmental aspects will need to be taken into account, although the final decision may be dependent upon other factors such as international or national boundaries.  The factors to be considered when making the decision include:

the size and geographical configuration of the VTS area;

the surveillance and communication requirements to cover the VTS area;

the cost of providing several centres compared with the cost of facilities to permit the operation of surveillance and communication equipment at remote sites and for transmitting the information to the VTS centre;

which of the two options would be most economic to operate, particularly in terms of human resources;

The operational disadvantages and advantages of using one centre

With regard to the operational disadvantages and advantages of using one centre, the following should be taken into account:

The operational disadvantages include:

the cost of providing remote control facilities from the sensors and radio communication equipment to the VTS centre;

there is unlikely to be direct visual monitoring from the VTS centre of the movement of traffic in large sections of the VTS area;

in case of total power failure at the VTS centre there is unlikely to be any  possibility that even a limited VTS service could be provided until the power has been restored.

The operational advantages include:

VTS operators can be changed from one from one sector to another thereby providing greater job satisfaction;

better control of the performance of all VTS operators by VTS Supervisors;

on the job training is readily available for all VTS personnel at the special training console;

if there is equipment failure in one sub-area or sector, an adjacent sub-area or sector can be extending temporarily until defective equipment has been is repaired;

better utilisation of staff resources and reduction of capital costs and administrative support costs.

VTS centres should, preferably, be custom designed with adequate accommodation for all the operational and supporting tasks to be performed properly.  Sufficient VTS Operator positions should be provided to cater for all the sub-areas and sectors for which the centre has responsibility together with a spare position for use in the event of failure of a main position, in the case of emergency incidents and for training purposes.

In considering the design of a VTS centre, it should be noted that it may be desirable to view as much of the VTS area as is practicable from the operations room.  The facilities available should include telecommunication services, rest rooms for operation staff including catering arrangements for night duties, and accommodation for administrative, support and maintenance staff.  If the VTS centre is also used by other port or harbour services, for example, a pilotage service, sufficient accommodation must also be provided for personnel of these other services.  In addition, facilities should be provided for visitors to the centre and, where appropriate, for the conduct of emergency operations such as search and rescue or oil pollution activities.

Environmental factors are likely to influence the location of the VTS centres and other associated buildings as well as the siting of remote stations for radar and communication equipment.  In addition, environmental considerations might influence the use of microwave links or land lines for the transmission of radar data from remote stations to the VTS centre.

### 3.2.3	Technical aspects of the infrastructure

The major technical factor is likely to be an assessment of the coverage of the VTS area [which is likely to be obtained by radar.  This might indicate that two or more radars are necessary to provide complete coverage of the VTS area. The information detected by several radars can be transmitted to one VTS centre if required.][sensors, traffic analysis req’d]

Other technical aspects of the infrastructure relate mainly to the full integration of data sensors with the means provided for analysing the data and the consequent transmission of the results of the analysis.  When a VTS is being planned it is essential that the intended availability is defined and assessment made into the most cost effective means of ensuring that the required level of availability is likely to be met.  This assessment may result in the need to duplicate some power supplies, operational equipment and communication links.

When the technical infrastructure is being designed, consideration also needs to be given to the provision of facilities to store and replay information displayed at the VTS operator positions and communications received and transmitted by radiotelephone and telephone[other comms].  These facilities are essential in the event of an emergency incident and for analysing the performance of the VTS centre.

The technical infrastructure includes the communication facilities to other VTS centres and to local authorities, support organisations and related services.  Consideration needs to be given to the most appropriate means for such communications, for example, some of the communication links may use dedicated telephone [or telex lines], others may use the public switched telex and telephone services and still others may use radio services.  The choice will probably depend upon the required availability of the particular communication link and the facilities provided at the VTS centre and at other stations.

The maintenance of the technical facilities of a VTS is an important aspect of the technical infrastructure.  There are several methods of performing the maintenance of the system.  One is to employ a complete maintenance team with sufficient knowledge and experience to repair and service all the equipment used by the VTS.  A second method is to place contracts with local maintenance organisations to undertake the work either on an on-call basis or by the provision of a permanent small team of contractors’ personnel at the VTS centre with back-up being provided by other personnel of the contractor when necessary.  A third method is for a small permanent team of maintenance personnel to be employed by the VTS and undertake routine maintenance tasks, minor repairs and diagnose major faults. This small team would be supplemented by contractual arrangements for major repairs and other tasks which were not within the competence of the maintenance team.  The choice of which method to use will depend upon  the anticipated failure rate of equipment, the cost of the different methods, whether there is a need for the VTS to be self-sufficient and the availability of suitable contractors in the area concerned. [move to 3.3.6?]

### 3.2.4	Administrative aspects of the infrastructure

The administrative aspects of the infrastructure provide the means which enable the continued operation of the VTS.  These include:-

- the preparation of procedures for operation of the service, and their periodical review:

- the recruitment and training of personnel, including operational, maintenance and support staff, the preparation of personnel structures and leave and duty rosters;

- the repair and cleanliness of the VTS centre and its out-stations;

- the provision of facilities necessary to support the operations of the VTS centre, such as vehicles, car parking facilities for staff and visitors, typing, photocopying etc; and,

- financial matters including the funding of the VTS, staff payments, contractual arrangements and other budgetary matters.

In some cases part of these administrative duties may be retained by the authority responsible for establishing the VTS or may be shared between two or more VTS centres where these are operated by a common authority.  However, these matters are essential to the proper operation of a VTS and need to be taken into account in the planning and running of a VTS scheme.