# CHAPTER 4 - AIS  -  A VTS TOOL[separate annex, e.g. AIS perf std? compress this section, include in ch3?]

## 4.1 GENERAL

A VTS should at all times be capable of generating a comprehensive overview of the traffic in its service area combined with all traffic influencing factors.  The VTS should be able to compile the traffic image, which is the basis for the VTS capability to respond to traffic situations developing in the VTS area.  The traffic image allows the VTS operator to evaluate situations and make decisions accordingly.  Data should be collected to compile the traffic image.  This includes:

- Data on the fairway situation, such as meteorological and hydrological conditions and he operational status of aids to navigation;

- Data on the traffic situation, such as vessel positions, movements, identities and intentions with respect to manoeuvres, destination and routing;

- Data of vessels in accordance with the requirements of ship reporting and, if necessary, any additional data required for the effective operations of VTS.

In the future AIS is expected to contribute significantly to the achieving those tasks.

## 4.2 Benefits of AIS

### 4.2.1 Automatic Vessel Identification

AIS can be expected to bring many benefits to the mariner.  Principal amongst these is the automatic and immediate provision of vessel identity (MMSI, call sign etc), thereby facilitating rapid radio communication where necessary.  This benefit is also of great value to VTS authorities.

Most VTS organisations require vessels to report to the VTS centre on approaching or entering the VTS area.  Achieving vessel identity relies on such vessels reporting both identity and location to the VTS centre, and the VTS operator then correlating this information with an unassigned radar track.

The process is time consuming and wholly reliant on the co-operation of participating vessels.  It is not uncommon for some vessels to fail to comply with this requirement, thereby creating a potentially dangerous situation, and creating further distraction for the VTS operator.  Even where VHF direction finding equipment is fitted, the VTS traffic image is still reliant on vessels reporting identity via VHF thereby permitting the correlation of identity with the radar track identified by DF.  AIS will help to overcome the safety weaknesses and time consuming procedures, inherent in the present arrangements.

### 4.2.2.	VHF communications

A major benefit of AIS is likely to be the reduction of VHF voice messages.   The use of AIS would also minimise language problems and reduce the chances of vessels misunderstanding messages from a VTS centre and vice versa

### 4.2.3	Improved Vessel Tracking

4.2.3.1	Wider geographical coverage.

AIS data will be received by other AIS transponders, or by base or repeater stations.  Thus where a VTS organisation is fitted with such equipment, it will be capable of receiving both identity and precise location of a vessel at the maximum reception range of the radio communications frequency in use.  Often this will be VHF, and as a consequence will normally permit detection of vessel target outside of radar coverage.  Even where this is not possible due to the need to screen base stations from adjacent VHF interference, extended VTS detection range may be achieved by the installation of additional base or repeater stations connected into a network at much lower cost than radar.

#### 4.2.3.2	 Positional accuracy.

AIS aims to achieve positional accuracy better than 10 metres when associated with DGNSS correction signals.

4.2.3.3	Absence of “radar shadow” areas.

In coastal and harbour waters radar tracking of vessels can be masked, or otherwise affected by the proximity of land and buildings.  The resultant “shadow” areas can cause radar based VTS to lose track, thereby denying the VTS centre the ability to monitor accurately vessel movement.  The loss of tracking may also result in the need to reacquire and re-identify lost tracks.

Whilst AIS tracking is expected to avoid the majority of such effects, the very close proximity of buildings and bridges can cause difficulties for AIS transponders in heavily built-up areas.  This is a consequence of inhibiting either the reception of the differential GNSS signal by the AIS transponder, or the transmission of the subsequent AIS message.

#### 4.2.3.4	Traffic image accuracy

Vessel tracking can similarly be interrupted when two vessels pass close to one another, with the result that the radar tracking of one contact may be confused by the proximity of the other.  This may result in the identity of one track transferring or “swapping” to the other.  The more precise tracking associated with AIS has been shown to prevent the incidence of “track swap”.

#### 4.2.3.5	Real time manoeuvring data.

Radar based VTS systems will typically provide details of a vessel’s course and speed over the ground.  Of necessity, this information is historical in that it is calculated from the track made good by a vessel.  However, AIS in addition to course and speed information can provide elements of real time manoeuvring data such as Ships Heading and Rate of Turn.  These are derived directly from the vessel navigation systems and are included automatically in the Dynamic Message broadcast by the AIS.

#### 4.2.3.6	 Weather effects on tracking performance.

Navigational radar performance is often adversely affected by precipitation as a function of the radio frequency on which it operates.  In heavy rain or snow, effective radar tracking is sometimes unachievable, even with the use of modern suppression techniques. However, VHF radio transmissions are not so attenuated and consequently a VTS centre is much more likely to maintain an accurate traffic image in adverse weather where that tracking is based on AIS data.

VHF radio transmissions can be affected by atmospheric ducting.  In these conditions,   VHF reception ranges can be greatly extended.   Where AIS messages are received at such an enhanced reception range, the system will automatically overcome ignore signals originating from vessels at long range

4.2.3 7	Provision of more precise navigational advice.

Where a VTS centre is able to receive AIS information from vessels within or adjacent to its area, it is expected that the quality, accuracy and reliability of vessel tracking will be improved markedly.  The VTS centre would therefore be able to provide more precise navigational advice, as and when required, or when deemed necessary.   In addition, the availability of certain real time manoeuvring data within the VTS centre is expected to enable VTS operators to appreciate more rapidly, and in greater detail, actual vessel movement.

#### 4.2.3.8	Electronic transfer of port passage information

If AIS is integrated into a VTS system, it would become possible for suitably equipped vessels and the VTS centre to exchange passage information such as intended way points, provided the appropriate software is available.

### 4.2.3.9	Electronic transfer of safety messages.

The facility available within AIS for the transmission of short safety messages makes possible the electronic broadcasting from a VTS centre of local navigation warnings, and similar safety related messages to suitably equipped ships.

VTS centres could also the capability of broadcasting local chart corrections to ECDIS fitted ships via AIS.

#### 4.2.3.10	Automatic indication of Voyage Related Information ( cargoes, dangerous substances etc)

Vessels are normally required to report to VTS authority any dangerous substances being carried.  The AIS voyage related message will permit the inclusion and automatic transmission of this information.

#### 4.2.3.11	Archiving data

The automatic availability of AIS data for suitably equipped vessels in a VTS Centre would facilitates the rapid and comprehensive recording, replay and archiving of their data.

#### 4.2.3.12	System redundancy

By equipping VTS centres with AIS, an alternative method of tracking and monitoring the navigation of suitably equipped would be introduced, thereby improving system redundancy significantly.

#### 4.2.3.13	Potential for interaction within regional AIS network

Increasing emphasis is being placed on networking VTS centres on a regional basis.  Such an arrangement facilitates greater efficiency by making possible the rapid transfer of vessel details between different centres.  This benefit may be enhanced by the provision of AIS within the relevant VTS centres

#### 4.2.3.14	Improved SAR management

Many marine authorities are expected to equip SAR units, including aircraft and helicopters, with AIS transponders.  The AIS voyage related message permits vessels to transmit the number of persons onboard a vessel.  Whilst this is not mandatory for vessels at sea, it can be made a formal requirement in a VTS area.   The provision of such details, and the ready identification and location of SAR units is expected to facilitate the management and evaluation of any SAR response.

## 4.3 	Installation of AIS into a VTS - issues to be considered

### 4.3.1	Number/location of base stations/repeaters

In deciding the size, and thus cost, of integrating AIS into a VTS system, a careful study needs to be undertaken to establish the number and location of base and repeater stations required to achieve full and reliable coverage.  Although VHF reception is greatly influenced by antenna location and height, operation in a heavy electronic environment may necessitate the installation of additional base stations in order to reduce susceptibility to interference.

### 4.3.2	Operability with adjacent VTS organisations

Where it proves necessary to use more than one base station, or where a VTS organisation involves more than one VTS centre, the method of connecting the component elements into a local network needs to be given careful consideration.  In particular, the existence of, or plans for a regional network may necessitate using a local networking solution that is compatible with national and international networks.

4.3.3	Availability of suitable VHF Communications channels.

Two maritime VHF Channels have been allocated by the ITU for the international use of AIS in its primary ship-to-ship mode.  It is not yet certain whether additional local channel will need to be allocated to support the operation of VTS within certain congested VTS port environments.  The need for such additional channels will be at its most acute where large numbers of recreational craft fitted with AIS navigate within a VTS area, and where the VTS centre has a particular interest in deriving vessel identity at maximum range.  AIS in an overload situation will discount transponder signals received from the extremity of an area, before those emanating from vessels or craft close to the receiving station.  Alternatively, this can be achieved by the manual suppression of AIS signals.

### 4.3.4	Availability of national/regional/local DGNSS corrections

In order to monitor vessel navigation with the 10 metre precision required for port approach and harbour navigation, a reliable DGNSS correction signal will need to available to all vessels throughout the VTS area and such services are provided nationally or regionally in many areas.  However, where such a service does not exist, a VTS organisation may consider providing this service by transmitting the relevant corrections using the AIS system.

## 4.4	Other issues to be taken into consideration

### 4.4.1	Integration of AIS into existing radar based systems

Radar based VTS systems often differ in the way radar video is handled and processed prior to presentation of the traffic image at the operator console.  System design and age are thus likely to influence the options for successfully integrating AIS.  A full appreciation of those options, together with any consequences, will normally only be possible after consultation with the relevant manufacturers.

In many VTS areas, vessel traffic is varied and includes both SOLAS and non-SOLAS vessels, as well as recreational craft.  In these circumstances, radar will remain the primary sensor for detecting vessels not fitted with AIS.  Economies in infrastructure are therefore unlikely.  In ports where the traffic image is composed solely of SOLAS vessels, the possibility may exist, albeit not in the immediate future, for a reduction in the use of radar.

AIS data is transmitted at variable rates depending upon vessel speed and manoeuvre.  In contrast, radar data is generated at a constant rate as defined by the antennal rotation speed.   The integration of AIS into a radar based VTS system thus needs to be capable of achieving and maintaining the correlation of AIS and radar data originating from the same vessel, despite unpredictable variations in data rates.   The potential benefits of AIS would be quickly overshadowed, should the process of integration result in the generation of numerous false tracks.

### 4.4.2	Use of electronic charts[revisit]

VTS systems have traditionally used a diagrammatic representation of the geographical and hydrographical features of the relevant area as the background to the traffic image.  The accuracy of such representations, however, is not suitable for precise navigation.  With the advent of electronic charts, benefit may be gained by using such charts as the background to the traffic image.  By so doing, vessel navigation may be monitored, and advised, in relation to precise charted features.  In VTS systems not fitted with electronic charts, such advice can only be given in relation to radar detectable features, such as coastline or navigational buoys, or as depicted on existing VTS display diagrams.

Where reliance is to be placed on electronic charts for this purpose, it is important that they are accurate, up to date, and identical to those in use onboard vessels.  They should therefore be the official IHO S57v.3 chart for the area, as issued by the national hydrographic office

In confined waters, it is likely that VTS operators in monitoring vessel manoeuvres will occasionally reduce the scale of their displays.  In such circumstances, it will be important that the electronic chart acting as the background to the traffic image is capable of showing increasing levels of survey detail, as operators reduce the scale on their displays.  This will only be possible where the electronic chart is compiled from source survey data, rather than from an existing paper chart.  In these circumstances, it will also be important that the charted location of radar sites is accurate to a maximum of 10 metres, if errors between radar and AIS generated tracks, which will be more obvious at reduced range scales, are to be avoided.

IHOS52 defines the standards for symbols and colours on official electronic charts.  Four variations of the basic colour scheme are available.  These colour schemes, whilst optimised for navigation in varying light conditions on the bridge of a vessel, may not be suitable for VTS purposes ashore, particularly where operators are required to study a display constantly for long periods.

### 4.4.3	Choice of VTS Symbols [revisit]

The following table shows the choice of AIS symbols allocated for use onboard vessels:

These symbols may be found to be unsuitable for VTS purposes, for two reasons.   Firstly, those selected to represent AIS tracks may need to be accommodated logically within an existing framework of symbols.  Secondly, VTS centres will often have need to represent visually on the traffic image, a much wider range of information than is necessary onboard a vessel.  For example traffic management may necessitate the use of symbols which depict different types and sizes of vessels.  Alternatively, it may be necessary to show which vessels have pilots embarked, and which do not.

Where it is required to transmit the position of a vessel to an AIS/ECDIS fitted vessel, it will be necessary for that position to be transmitted in terms which will be recognised by the vessel, regardless of how it is represented internally within a VTS.

## 4.5	AIS and Aids to Navigation [reference IALA AIS documents; consider deleting?]

- A further potential application of AIS is as an aid to navigation. When positioned at a significant geographic point or danger to navigation, an AIS transponder could provide information and data that would serve to:

- complement or replace an existing aid to navigation;

- provide identity, state of “health” and other information such as real time tidal height, tidal stream and local weather to surrounding ships or back to the shore authority;

- provide the position of floating aids (primarily buoys) by transmitting an accurate position (based on DGPS corrections) to monitor that they are “on station”;

- provide information for performance monitoring, with the connecting data link serving to remotely control changes of navaid parameters or switching in back-up equipment;

- as a supplement to radar transponder beacons (racons), providing longer range detection and identification in all weather conditions; and,

- as a data gathering tool, providing information on all AIS fitted shipping traffic passing within VHF range of the site.

The remote control and monitoring of aids to navigation has been developed primarily to enable service providers to ensure that aids and supporting systems are functioning correctly and where necessary to organise maintenance and defect repair effectively.  The introduction of AIS will present an opportunity to provide such information to service providers and mariners, using internationally standardised and recognised equipment, message protocols and frequencies.

## 4.6	AIS for Meteorological and Hydrological information [reference IALA AIS documents; consider deleting]

Another potential application is the transmission of meteorological and/or hydrological data. However, where such an application is intended for international use, the message format shall be registered by IALA prior to being made available to system manufacturers. This will facilitate the correct presentation of the information on systems from different manufacturers.

Options for implementing this application include:

- Connecting a measuring station directly to a local AIS-unit, which the broadcasts the relevant information.

- Several measuring stations can be connected to a base station network via a data communication system. Information can then be broadcast from appropriate base stations.

- A measuring station can be co-located with an Aid to Navigation equipped with AIS. The AIS-unit can then be used to broadcast both the Aid to Navigation information and meteorological and/or hydrological information using separate messages.

The information to be broadcast will depend on the operational requirement and the availability of measuring and processing equipment.  Examples include:

- Wind speed, average and gust values

- Wind direction

- Water level

- Water temperature

- Air temperature

- Current speed and direction on different depths

- Tide information

The availability of such data would permit the presentation of real time information at receiving stations, including onboard ships within VHF range.

## 4.7	Short term action by VTS authorities [consider deleting; overtaken by events?]

AIS equipment is to be implemented as a mandatory carriage requirement under SOLAS for newly constructed vessels from 1 July 2002 and progressively thereafter to other types and sizes of SOLAS Convention vessels up to 1 July 2008.

VTS Authorities should consider as matter of priority whether they intend integrating AIS into their VTS system.

AIS also has the potential to improve efficiency in vessel traffic and port management.  The degree to which this potential may be realised will vary depending on the operational circumstances.  It is for each Port Authority in consultation with the VTS Authority to make that assessment, possibly using cost benefit analysis techniques.

In order to avoid a situation whereby AIS fitted vessels incorrectly believe that a VTS Centre is capable of receiving data being transmitted via the AIS, VTS Authorities should consider publishing by appropriate means their status in respect of AIS.  Where applicable, the date on which they intend to incorporate AIS into the VTS should also be promulgated well in advance.