DRAFT GUIDELINES ON THE EXCHANGE OF DATA BETWEEN VTS CENTRES

### 1	OBJECTIVES OF DATA EXCHANGE

There is an increasing interest in exchanging data between VTS centres. Even if several systems have been interfaced, there is no standard available covering the different types of data available in a VTS. There is neither a standard making it easier for the vendors to communicate. To make a standard describing data exchange between VTS centres, it is important to choose a way forward that will take into account the ever evolving need of new type of information to exchange. A static description will only lead to proprietary solutions needed to cater for different types of data. At the same time, it is necessary to make a foundation describing a minimum requirement so interfaces between different vendors will work together without further refinement.

Several methods of data exchange were discussed, including NMEA, EDI, XML, as well as NATO standards.  The WG discussed the role of the Committee in defining data exchange for VTS.  Once having identified under which conditions VTSs would need to exchange data, the WG felt it could develop guidelines for the content of the data to be exchanged.   The WG decided that perhaps this task was becoming overtaken by these events and that a review of EU efforts, including SafeSeaNet, by the Committee was warranted before continuing any effort to develop.   It was agreed that in an exchange of data it was best to exchange validated and correlated data as is typically processed at a VTS, rather than raw, unprocessed data from disparate sensors or other networks (such as AIS networks).  Experience has shown that exchange of unprocessed data will often cause network problems due to unvalidated, uncorrelated, and often redundant data.

### 2	CONDITIONS UNDER WHICH VTS CENTRES NEED TO EXCHANGE DATA

This document addresses the exchange of data between VTS centers.  In this section the application and the limits of application of the individual performance standards for the navigational equipment should be determined. It was agreed that the Committee should identify the conditions under which several VTSs might want to exchange data,  e.g., overlapping areas of responsibility or regional coverage. It must also be determined when and under what conditions would require the exchange of “real time” data.  Further discussion noted EU efforts, including SafeSeaNet, that are mandating what data is to be exchanged between Administrations in Europe.

### 3	TYPES/CLASSES OF DATA TO BE EXCHANGED

Processed/raw, static/dynamic/voyage-specific, real time/archived, validated/unvalidated, strategic/tactical, etc……

Traffic data, geographic data, meteorological/hydrological data.

### Meteorological and hydrological information

This information can vary quite a lot according to the different types of sensors and information. To have a standard for exchange of data might be difficult, but there exists some work in this area already.

### Digitized radar video

This is the representation of the radar image, and is used both for presentation and for the base of tracking in a VTS system. Ideally, the tracking system should take care of all targets of interest in a system, and therefore the exchange of radar video between VTSs should not be of interest. However, there might be different levels of interest, i.e. a commercial focused operator looks for something different than a security focused operator. At the moment transferring and display of radar video is probably one of the most proprietary areas for the VTS vendors, but still it should be possible to describe a format for radar video making it possible to transfer. Radar video might imply the need for a high bandwidth, but it should not limit the possibility of describing an interface.

### Radar tracks

This is dynamical information related to a target being followed by a tracking system. Up to now, a VTS gets track information for the most from a tracking system based on radars. Even if there exists different philosophies on how to best track a target, the basic parameters involved are very similar between tracking systems. Position, speed, bearing and identification with a few additions makes the parameter set involved in tracking. Tracks are usually considered for data exchange between systems, and there should be relatively easy to develop a standard where track data is one of the base elements.

### AIS tracks

AIS will become very important in a very short time, and information from AIS systems are also due for exchange between systems. The information is similar to radar tracks with a few important differences. AIS can directly identify a target, it can give additional information regarding cargo and route, and the positional accuracy is usually much better than radar based tracking systems. Even so, the basic information is very close to radar tracks.

### Voyage related information

A VTS usually contains a database system, and even if the systems in use are very different, there are a lot of common parameters.

### 4 	DATA TO BE EXCHANGED

The data to be exchanged is grouped according to the class/type of data as discussed above: static, dynamic, or voyage-related data.  This data is generally based on the AIS classes of messages and the data associated with them.

##### Static

MMSI (Maritime Mobile Service Identity);Set on installation

Note that this might need amending if the ship changes ownership

Call sign and name; Set on installation

Note that this might need amending if the ship changes ownership

IMO Number; Set on installation

Length and beam; Set on installation or if changed

Type of ship; Select from pre-installed list

Location of position-fixing antenna; Set on installation or may be changed for bi-directional vessels or those fitted with multiple antennae

##### Dynamic

Ship’s position with accuracy indication and integrity status

Automatically updated from the position sensor connected to AIS

The accuracy indication is for better or worse than 10 m.

Position Time stamp in UTC; Automatically updated from ship’s main position sensor connected to AIS

Course over ground (COG); Automatically updated from ship’s main position sensor connected to AIS, if that sensor calculates COG

This information might not be available

Speed over ground (SOG); Automatically updated from the position sensor connected to AIS

This information might not be available

Heading Automatically updated from the ship’s heading sensor connected to AIS

Navigational status Navigational status information has to be manually entered by the OOW and changed as necessary, for example:

- underway by engines

- at anchor

- not under command (NUC)

- restricted in ability to manoeuvre (RIATM)

- moored

- constrained by draught

- aground

- engaged in fishing

- underway by sail

In practice, since all these relate to the COLREGs, any change that is needed could be undertaken at the same time that the lights or shapes were changed

Rate of turn (ROT) Automatically updated from the ship’s ROT sensor or derived from the gyro

This information might not be available

Radar target tracks

Digital/Analog Radar video

##### Voyage-related

Ship’s draught To be manually entered at the start of the voyage using the maximum draft for the voyage and amended as required (e.g. – result of de-ballasting prior to port entry)

Hazardous cargo (type); To be manually entered at the start of the voyage confirming whether or not hazardous cargo is being carried, namely:

DG (Dangerous goods)

HS (Harmful substances)

MP (Marine pollutants)

Indications of quantities are not required

Destination and ETA; To be manually entered at the start of the voyage and kept up to date as necessary

Route plan (waypoints); To be manually entered at the start of the voyage, at the discretion of the master, and updated when required

##### Short safety-related messages (text messages)

Free format short text messages would be manually entered, addressed either a specific addressee or broadcast to all ships and shore stations

The data is autonomously sent at different update rates:

- dynamic information dependent on speed and course alteration,

- static and voyage-related data every 6 minutes or on request (AIS responds automatically without user action).

Short safety-related messages are fixed or free format text messages addressed either to a

specified destination (MMSI) or all ships in the area. Their content should be relevant to the safety of navigation, e.g. an iceberg sighted or a buoy not on station. Messages should be kept as short as possible. The system allows up to 158 characters per message but the shorter the message the more easily it will find free space for transmission. At present these messages are not further regulated, to keep all possibilities open.

Operator acknowledgement may be requested by a text message.

Short safety-related messages are only an additional means of broadcasting maritime

safety information. Whilst their importance should not be underestimated, use of such messages does not remove any of the requirements of the Global Maritime Distress and Safety System  (GMDSS).

The operator should ensure that he displays and considers incoming safety-related

messages and should send safety-related messages as required.

According to SOLAS regulation V/31 (Danger messages)

“The master of every ship which meets with dangerous ice, a dangerous derelict, or any

other direct danger to navigation, or ...is bound to communicate the information by all

the means at his disposal to ships at his vicinity, and also to the competent authorities...”.

Normally this is done via VHF voice communication, but “by all the means” now implies

the additional use of the AIS short messages application, which has the advantage of reducing difficulties in understanding, especially when noting down the correct position.

### 5	TECHNOLOGIES

There are several technologies existing today making it possible to exchange data. AIS base stations are interfaced through NMEA over a serial line, which shows that relatively recent technologies also use this connection method.

##### NMEA 0183
The most common one is based on NMEA over a serial connection. This is simple and secure, as the two systems are just connected over a serial line, making the need of firewalls unnecessary.

##### Electronic Data Interface (EDI)

##### eXtensible Markup Language(XML)

The eXtensible Markup Language -- has recently emerged as a new standard for data representation and exchange on the Internet. The basic ideas underlying XML are very simple: tags on data elements identify the meaning of the data, rather than, e.g., specifying how the data should be formatted (as in HTML), and relationships between data elements are provided via simple nesting and references. Yet the potential impact is significant: Web servers and applications encoding their data in XML can quickly make their information available in a simple and usable format, and such information providers can interoperate easily. Information content is separated from information rendering, making it easy to provide multiple views of the same data.

Currently there are several EU projects focusing on standardization in a maritime environment. Even if IALA is not a European organisation, the result from the initiatives might be a good starting point for IALA. These projects use XML as the base, and it seems naturally to involve some of this work to extract the knowledge gained, but also to be able to form an overall approach where the different standards fits in.

An approach using XML Schemas for message validation will make the transfer of data easier. The Schemas can be made as part of the standard, and when validating against these both for sending and receiving, the different providers and readers will have a certain level of quality of the data. There are tools available for the creation of schemas, which also feature creation of test data. There are also high level tools available for generating code for reading and creating correct data in accordance to the XML Schema.