## 3.3	PROVISION OF EQUIPMENT [be more generic, reference VTS Perf Stds]

### 3.3.1	General

During the requirement definition, feasibility study and project definition phases a number of matters will have been considered which will influence the decisions on the equipment needed by a VTS to enable it properly to perform its service options [properly].

Before making any decisions about the equipment to be provided more detailed consideration should be given to the following subject matters:

- division of a VTS area into sub-areas or sectors;

- surveillance requirements for the VTS area;

- communication requirements;

- additional equipment requirements;

- required equipment reliability;

- selection of equipment;

- documentation requirements.

- [add in VTS19/8/3 figure page 6?]

### 3.3.2	Division of a VTS area into sub-areas or sectors

The necessity to divide a VTS area into sub-areas or sectors or is dependent mainly upon:

- the size and geographical configuration of the area

- the traffic density

- the VTS services to be provided

Where the VTS area is to be divided into sub-areas or sectors, each sub-area or sector should have such dimensions that a VTS operator can cope with all the traffic in it.  This implies that the less VTS services provided the larger the sector may be.  In principle each sector has its own operator, who may have one or more VHF channels depending on the geography.

In a VTS which could not be covered by one radar, several radar installations could be provided each covering a part of the total area. The siting of each radar is important and should ensure that collectively the entire area is covered.  Care should also be taken to ensure that where a VTS area includes places where navigation is critical or where severe restrictions are placed on ship movements, such places are covered by one VTS operator and not shared between two operator positions.  An example of such an arrangement can be seen in Figure 5. [radar focus only]

Radar Sensor  1

Radar Sensor  2

Radar Sensor  3

Radar Sensor  4

Radar Sensor  5

Operator		      Operator	    	        Operator

Sector 1		       Sector 3		         Sector 3

VTS CENTRE

Figure 5.	Example of possible arrangement of sectors and radar sensors.

### 3.3.3	Surveillance requirements for the VTS area [Structure – radar, ais, cctv, etc?]

#### 3.3.3.1	Surveillance Coverage of the area

There is a need to take into account the extent of the VTS area with regard to the surveillance equipment necessary.  In principle the equipment should be able to cover an area well in excess of the designated VTS area, so as to allow for any decrease in performance in poor weather conditions.

Figure 6: VTS Radar, Vancouver Mt. Park site. Courtesy of the Canadian Coast Guard

The surveillance equipment in most common use is radar although other systems, such as the Automatic Identification System (AIS) (see Chapter 4), can be expected to be used with good effect[ in the future].  Therefore, depending on the services that a VTS is to carry out the radar coverage can be:

- nil (automatic identification systems, voice communication[other sensors] and reporting only)

- partly (covered areas chosen intentionally with some blind sectors)

- totally by one radar sensor (without any blind sectors)

- totally by two or more radar sensors (for large VTS areas and to cover for shadow effects of other vessels).

The required radar coverage of a VTS area should be determined by an assessment of the service to be provided, the safety level to be reached and the needs of the users of the VTS system.  Subsequently suitable positions for the radar equipment should to be determined by site-tests to ensure, as far as practicable, that the required radar coverage will be provided.

When determining the position and range of radar equipment, the possibility of malfunction of a radar should be taken into account, and where practicable the arrangements should enable another radar, or AIS where appropriate, to provide cover for the sub-area or sector affected.  The maintenance and availability of remote sensors should be given special consideration and adequate redundancy should be provided.

#### 3.3.3.2	Operational conditions

The operational conditions for VTS equipment will vary from one place to another.  In some countries the availability and reliability requirements need to take into account extremes of temperature or precipitation, in others special attention may need to be paid to wind force (including gusts) or earthquake resistance.

#### 3.3.3.3	Detection Performance [restructure thru 3.3.3.8, be more generic, reference VTS Perf Stds]

Radar should be able to detect for subsequent display, all moving or stationary targets which satisfy the detection criteria within its coverage area during all specified operating circumstances.  Therefore depending on the detection criteria the necessary type of radar can then be chosen for each location.

Although the detection performance may vary with the specific requirements for a particular VTS, the following criteria should be used as a basis for evaluating the detection performance of radar.

For stationary floating objects each radar should be capable of detecting all presently established floating aids equipped with an octahedral cluster radar reflector, with a diameter of 1 metre, located within 8 miles of  its antenna site, in visibility of 5 cables, or rain up to 10 mm/hr and in sea conditions of sea state 1.

For vessels, the detection performance needs to take into account the most frequent types of vessels operating in the VTS and Figure 7 should be used as a guide when determining the performance of a radar when detecting vessels of different sizes.

In the Example Performance Requirements section of Figure 8, “x” means the radar should be able to detect as a minimum the targets in 5 out of every 10 scans, averaged over a 2 minute period, and make them available for display.  If the rainfall is greater than 25mm/h a radar operating in the 2,900 - 3,100 MHz band (S-band) will need to be used to meet these requirements.  Every radar should be designed and equipped with devices to reduce the adverse effects of rain and sea clutter and enhance the probability of target detection per scan.  The radar should also be designed and installed so as to eliminate to the maximum extent possible, false echoes caused by side lobes or reflections from nearby structures.

#### 3.3.3.4	Radar Accuracy [update]

Range and bearing resolution precision is necessary in order to have a clear and distinct appreciation of the movement of vessels, including those which are at anchor.  The resolution necessary is dependant on the VTS services to be provided.  An ultimate operational test could be the capability of the radar and other sub-systems to maintain a clear definition of vessels manoeuvring within a predefined area at close distances to each other.  In order to measure the resolution objectively, two radar reflectors, equal to the radar cross-section of two chosen vessels, both at a specified height above the sea level, could be used.

#### 3.3.3.5	Range Discrimination and Accuracy [update]

Although the range discrimination and accuracy may vary in some applications the following should be used as a guide for normal circumstances:-

Discrimination:	The radar should be able to discriminate between two 20 metre steel vessels on the same radial bearing in rainfall conditions of up to 25 mm/h, and separated by 30 metres at a range of 6 nautical miles.

Accuracy:	The radar should be able to determine range with an accuracy of ± 30 metres at a range of 6 nautical miles.

#### 3.3.3.6	Bearing Discrimination and Accuracy [update]

Although the required bearing discrimination and accuracy may vary in some applications the following should be used as a guide in normal circumstances:

Discrimination. 	The radar should be able to resolve:

- in rainfall conditions up to 25 mm/h. two 20 metre steel vessels at a range of 6 nautical miles, when separated in azimuth by 75 metres; and,

2.	in rainfall conditions of 80 mm/h, two 20 metre steel vessels at a range of 6 nautical miles, when separated in azimuth by 185 metres.

Accuracy:	The radar should be able to determine the bearing of a target with an accuracy of 0.6º for a radar operating in the 9300 - 9500 MHz band (X-band) and 1º for a radar operating in the 2900 - 3100 MHz band (S-band).

#### 3.3.3.7	Tracking

When computers are used to establish and maintain the VTS traffic image the VTS display may be dependent completely on the information supplied by the target analysis computer and only synthetic images of the traffic are displayed.  In the case of a breakdown of this major sub-system the information available on the VTS display is not likely to be of use.  Therefore unless redundancy is provided in a synthetic target display system, a combination of raw radar video and information supplied by a target analysis computer is recommended.

To achieve its operational purpose a target analysis computer should be able, at all times, to acquire and maintain “track” on any target detected by a radar in at least 5 out of every 10 scans averaged over a 2 minute period.

A target analyser should incorporate a sufficient level of signal processing to eliminate, to the maximum degree possible, any spurious signals.  The detection probability and the false alarm rate should be specified during the project definition phase and evaluated during the commissioning and acceptance phase.

The system should allow for target acquisition to be made manually or automatically.  Manual target acquisition may be achieved by means of a trackball (roller-ball) or light-pen combined with functional keys.  In some systems, after automatic acquisition, the operator can manually give the target an identity (ID-tag), which may be derived from an abbreviation of the vessels name.  Sometimes an ID-tag is derived from information out of the VTS database or it could be the vessels International Radio Call Sign.

The capacity of the target analyser, for both “stationary” and “moving” objects should be adequate for the traffic density of the area.  Minimum and maximum tracking distance should be defined as well as the capability of acquiring, analysing, predicting and maintaining track of specified types of vessel moving and turning at specific speeds, without a track-loss of more than 10-15%, depending on the fairway and types of vessels.

The time for automatic re-acquisition in the event of track loss should be defined.  A failure to reacquire a target automatically in a specific time limit should be brought to the attention of the VTS operator.

The target analyser should, within the limits of the sensor equipment, be capable of providing data to a preset accuracy.  This data may include: position information, target speed, target course, ETA at specific geographical locations, CPA and TCPA with other vessels.  This data should be displayed either on the VTS display using synthetic symbols near the vessel or in tabular form in an un-used part of the VTS display.

It is recommended that means of providing an automatic collision warning be included where CPA’s can be defined, depending on the local area, before an alert is given.  When vessels are on approximately parallel courses the target analyser should be capable of calculating the time at which vessels meet or overtake each other.

| EXAMPLES OF VESSELS USED FOR DETERMINING PERFORMANCE:- |  |  |
| --- | --- | --- |
| VESSEL SIZE | RADAR CROSS SECTION | HEIGHT ABOVE SEA LEVEL |
| 6 Metres Wood | 6m2 | 1 metres |
| 20 Metres Wood | 25m2 | 3 metres |
| 80 Metres Steel | 300m2 | 8 metres |
| 10.000 GRT Steel | 1000m2 | 15 metres |
| For the craft specified, later a summary of performance standards per radar site can be developed, depending on the operational requirements. |  |  |

Figure 7      Guidance for determining the performance of radar when detecting vessels of different sizes

| EXAMPLE PERFORMANCE REQUIREMENTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| ANTENNA SITE: “..........” |  |  |  | MAXIMUM RANGE “….” |
|  | 6m. wood | 20m. wood | 80m. steel | 10k GRT |
| Radar range 6 nm 12 nm 18 nm 24 nm | x | x x | x x x x | x x x x |
| Visibility clear 0.5 nm | x | x x | x x | x x |
| Rainfall p/h 18mm 25mm 60mm |  | x x | x x x | x x x |
| Seastate 1 2 3 4 5 | x | x x | x x x x | x x x x x |

Figure 8 Example performance requirements for a VTS radar

If the target analyser is capable of calculating the TCPA for vessels on a parallel course, it should also be capable of doing the same for vessels on a converging or crossing course.  It should be noted that because of the track filtering in the radar data processing any changes in speed or course or a target are only noticeable after a certain delay (about 10 antenna rotations) and that the indicated values for position, course, speed and CPA are behind the real data.

Some target analysers have the capability of alerting the operator when a vessel deviates from a pre-defined route or strays outside a designated traffic lane.

The minimum response time of the target analysis computer should be defined under circumstances when a large number of targets are being processed.

Despite all the automatic functions that can be incorporated into the VTS display, a normal electronic cursor that can be positioned on the VTS display at any position, in order to measure direction and distance is an advantage for manually analysing traffic situations especially in case of failure of the tracking sub-system.

#### 3.3.3.8	Display

A VTS display mapping graphics, analytical graphics and overlay information should be provided to enable a VTS Operator to have a concise picture of the geographical features, waterways and navigational lanes.  When this information is being displayed, in many cases the rw radar video echoes of the coastline should be suppressed beyond the coastline, making this area of the display available for other synthetic information.  Definitions of the requirements and accuracy of all the graphic possibilities need to be developed during the Project Definition phase.

If VHF direction finders are included in the system, the bearing lines should be shown on the VTS display, with the possibility for the operator to switch them off.

Radar and other electronic displays of the VTS traffic image should be equipped with recorders.  The procedures which should be applied to the custody and operation of the recorders are as follows:

- radar and other electronic displays of the VTS traffic image should be recorded continuously;

- tapes or discs should be changed daily;

- recorded tapes or discs should be numbered, dated and placed in locked storage, the control and access to which should be under the control of the VTS;

- all recorded tapes or discs should be kept for a minimum of 30 days;

- tapes or discs should be retained by the VTS whenever an investigation or judicial inquiry has been ordered or is anticipated.

### 3.3.4	Communication requirements

Communications in a VTS area are presently heavily biased towards voice systems, mostly using VHF.  This requires a complex combination of procedural and language skills, which must be thoroughly understood by all VTS Personnel.

A VTS Operator will need to communicate with shipping for which he has responsibility and may also need to communicate with other operators or allied services.  The ideal situation would be for the control of all communication equipment to be integrated into each VTS Operators console.  To ensure adequate availability and reliability, all operational VTS communication services should have back-up systems with a power source independent from the normal power supply should.

When communicating with a vessel, the VTS Operator and the vessel should be alert to the problems involved with verbal communication.  Confirmation should be requested whenever doubts exist.  The English language should normally be used to enable the VTS and the vessel to understand each other clearly.  In particular, the IMO Standard Maritime Communication Phrases (SMCP), should be used where difficulties exist, and when requested by the master or the VTS Operator.  In a local area where there is little international shipping the national language may be the appropriate working language of the VTS.

#### 3.3.4.1	[VHF Voice]Communication with shipping in the VTS area

It is of the utmost importance that the VHF communication network of a VTS is set up properly, with high quality and reliable equipment and interconnection.  The number of VHF channels required for specific VTS usage should be assessed during the Project Definition phase and arranged with the Telecommunication Authorities well in advance of the Commissioning and Acceptance phase.  Often a VTS has, apart from the sub-area or sector frequencies, one or more general reporting or emergency frequencies.  The number of channels required is dependent upon the number of the various sub-areas and sectors that will be used and the overall traffic density.   The VTS Authority should also seek permission from their Telecommunication to use VHF Channel 16 for emergencies and for calling ships which are not participating in the VTS

Normally the communication coverage of the VTS area will be achieved by VHF using telephony or data transmissions and will involve:

- Communication between shore and participating vessels, using appropriate international VHF channels.  This communication can take place on simplex as well as duplex channels.  In the case of duplex channels, re-transmission from the shore may need to be carried out if the information received is of interest to other vessels to enable them to better comprehend the traffic situation in the their vicinity.

- Private channels may also be available for use between VTS centres and local service craft.  However in this case local service craft should always be able to communicate with shipping on the appropriate international VHF channel.

[Shipborne VHF equipment is normally capable of transmitting and receiving on the international channels 1-29 and 60-89 at full power [(25W)] and reduced power (1W).  Where practicable it is advisable to request low power transmission from ships.  Communications with vessels should be recorded, for accident investigation, disputes, etc.necessary?]

#### 3.3.4.2	Other Communication

The following general types of communication should be available within a VTS:

- between VTS operators in the same VTS centre

- where appropriate, between VTS operators in different VTS centres belonging to the same VTS;

- between VTS operators of any adjacent VTS;

- with vessels about to enter a VTS area

- with Pilotage, tugboats, SAR, Port Authorities and other Authorities.

These types of communication can be performed by using the following equipment:

- public telephone lines

- private telephone lines

- telex

- facsimile

- radio telephony [and microwave links, network?]

- automated data transfer systems [including AIS, DSC, GMDSS?????].

- [internet, email, satellite, etc.]

Where there is a need for communication between a VTS and a shore authority to have a high degree of operational importance, consideration should be given to providing direct telephone lines to increase the reliability and decrease the time of the connection.

The internal communication within a VTS is of the utmost importance.  If there is more than one VTS Centre it may be necessary for the operators in one VTS centre to communicate easily with operators in other centres so that specific traffic information can be passed from one operator to another with the minimum of delay.

#### 3.3.4.3	Communication recording facilities [Structure? Separate section for all recording capability?]

Operational communication links established at a VTS centre should be equipped with instant replay recorders.  The procedures which should be applied to the custody and operation of the recorders are as follows:

- all conversations on VTS operational communication links should be recorded;

- tapes or discs should be changed daily[depending on media];

- recorded tapes or discs should be numbered, dated and placed in locked storage, the control and access to which should be under the control of the VTS;

- all recorded tapes or discs should be kept for a minimum of 30 days;

- Tapes or discs should be retained by the VTS whenever an investigation or judicial inquiry has been ordered or is anticipated.

### 3.3.5	Additional Equipment

#### 3.3.5.1	VTS Operator position

A console should be provided at every VTS operator position with the equipment integrated in the most ergonomic arrangement.  The illumination of the operator position should be such that all relevant equipment such as the VTS monitor, communication equipment, target analyser etc can be monitored effectively and administrative tasks can be carried out at the same time.  Daylight displays for radar equipment have the advantage that they can be operated efficiently in normal room light levels.

Care should be when choosing the site for the consoles that VTS operator positions do not interfere with each other especially regarding sound.

In addition to the radar and the communication equipment, other equipment might be necessary at a VTS operator’s position for reference purposes for providing information to shipping.  These include:-

- data base relevant to the VTS;

- closed circuit television (additional to or instead of an outside view);

- VHF direction finding equipment (mainly for sea-areas, used for positive identification of vessels);

- remote indication of hydrological data (water height at specific positions; current and tidal data);

- remote meteorological data (visibility, wind direction and speed at specific points);

- communication recording equipment;

#### 3.3.5.2	Additional  console

A spare VTS operator console should be provided which:

- could substitute for any console that is unserviceable;

- be used for On-the-Job training;

- where a need exists, be used by a VTS supervisor;

- where appropriate, be used in the co-ordination of emergencies.

### 3.3.6	Availability and reliability of equipment[revisit; complete system vs individual components; definition to include availability over time period]

The equipment performance parameters are strongly dependant upon the services to be provided which influences the Mean Time Between Failures (MTBF) and the availability of the service.  An availability of 99.9% or better is recommended for the complete system.  Information on reliability methods is given in the IALA Guide to the Reliability and Availability of Aids to Navigation.

For the assessment of the equipment to be installed, manufacturers should provide an estimation of the reliability of the equipment based on the following formula:

| A calc = | MTBF | x 100% |
| --- | --- | --- |
|  | MTBF + MTTR |  |

Where:-

A calc		=	Calculated reliability (%)

MTBF		=	Mean Time Between Failures in hours

MTTR		=	Mean Time To Repair in hours

During the acceptance trials phase the VTS Authority should measure the actual availability of each equipment using the following formula:

| A actual = | T - B | x 100% |
| --- | --- | --- |
|  | T |  |

Where:-

A actual		=	Actual achieved reliability (%)

T		=	the number of minutes per calendar month

B		=	the number of minutes per calendar month the 							system is unavailable

A reliability of 99.9%, which represents a failure of less than 9 hours per year, should be specified.  Care should be taken not to overestimate the benefit of a stated MTBF because this is a statistical value and of little relevance in the case of a single equipment.

### 3.3.7	Selection of equipment

The Project Definition phase will normally result in a document for tender.  When the bids for tender have been received the proposals need to be studied in conjunction not only with regard to the capital costs but the running and maintenance costs as well.

Most equipment for a VTS, including operator consoles, is available “off the shelf” and normally can be modified to meet the particular requirements of a VTS Authority.  However if any additional equipment needs to be incorporated into the system it might be necessary to have the console custom designed.  This also applies in many instances to the equipment hardware and software.

It should be noted that where the operational parameters cannot be met by “off the shelf” equipment and custom designed equipment has to be ordered, the total cost may be increased substantially.

If possible suitable equipment should be viewed in an operational VTS centre before a final selection is made.

### 3.3.8	Documentation

A very important part of any VTS is the manuals and documentation of all equipment and the following should be available and centrally maintained:-

- [contract, SOW]

- ILSP

- System description by main-contractors of all subsystems in the configuration

- Functions of the equipment in the subsystems

- Interfaces between subsystems

- Available test facilities

- Equipment positioning

- Power distribution

- [Applicable documentation?]

- Operational User’s Handbook*

- Training Manual*

- [Technical Engineering Document]

- Maintenance Manual

- Test manual for pre-delivery and on-site testing, including required test results*

- *These documents would normally be prepared by or on behalf of the VTS Authority.