IALA Guideline

G1023

Design of Leading Lines

Edition 2.0

Date (of approval by Council)

urn:mrn:iala:pub:g1023

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
| December 2001 | 1st Edition | December 2001 |
| December 2005 | 1.1: Entire document reformatted to reflect IALA Documentation Hierarchy | December 2005 |
| 2024 | DRAFT 2.0: Entire document |  |

1.	Introduction	6

2.	Purpose	6

3.	Background	6

4.	History	7

5.	Definitions and Parameters	7

5.1.	Geometry, Plan View	8

5.2.	Geometry, Side View	8

5.3.	Photometry	9

5.4.	Miscellaneous	10

6.	Basic Equations	11

6.1.	Tower Position	11

6.2.	Luminous intensity	12

6.2.1.	Minimum Luminous Intensity	12

6.2.2.	Recommended Luminous Intensity of Front Light (Night)	12

6.2.3.	Recommended Luminous Intensity of Rear Light (Night)	12

6.2.4.	Maximum Luminous Intensity (Night)	14

6.3.	Elevation Angles	14

6.4.	Bearing Angles	15

6.5.	Minimum Elevation Difference	15

6.6.	Sensitivity of a Leading Line	16

6.7.	Cross-Track Factor	17

6.8.	Geographical Range	17

6.9.	Heights	18

6.9.1.	Front Light	18

6.9.2.	Rear Light	20

6.10.	Daymark	21

6.10.1.	Guideline 2001 Design	21

6.10.2.	Alternative Size Calculation	22

6.10.3.	Alternative Daymark Designs	24

7.	Design Methodology	26

7.1.	Preliminary Assessment of Tower Positions	27

7.2.	Iteration Input Parameters	27

7.3.	Iteration Results	28

7.3.1.	Daymark Size (optional)	28

7.3.2.	Minimum Intensity	28

7.3.3.	Recommended Intensity	28

7.3.4.	Maximum Intensity	28

7.3.5.	Intensity Ratio	28

7.3.6.	Recommended Heights	28

7.3.7.	Illuminance at the Eye of the Observer	28

7.3.8.	Minimum Elevation Difference	29

7.3.9.	Cross-Track Factor	29

7.4.	Iteration Assessment	29

7.4.1.	Blur Test	29

7.4.2.	Cross-Track Factor	29

7.4.3.	Brightness of Front Light	29

7.4.4.	Brightness of Rear Light	29

7.4.5.	Glare Test for Front Light	30

7.4.6.	Glare Test for Rear Light	30

7.4.7.	Check Daymark Size Front Light	30

7.4.8.	Check Daymark Size Rear Light	30

7.4.9.	Rear Light above Front Light considering Blur	30

7.4.10.	Front Light above Safe Height above Water	30

7.4.11.	Front Light above Horizon	30

7.4.12.	Obstruction not obscuring Front Light	31

7.4.13.	Obstruction not obscuring Rear Light	31

7.4.14.	Obstruction not obscuring Front Light Daymark	31

7.4.15.	Front Light not obscuring Daymark of Rear Light	31

7.4.16.	Obstruction not obscuring Rear Light Daymark	31

7.5.	Final Leading Line Design	31

7.5.1.	Lateral Sensitivity	32

7.5.2.	Maximum Intensity less than Minimum Intensity	32

7.5.3.	Maximum Intensity less than Recommended Intensity	33

7.5.4.	Compromises during Leading Line Design	33

8.	supplemental INFORMATION	33

8.1.	Lights	33

8.1.1.	Fan versus Pencil Beam Lights	33

8.1.2.	Beam Width of Pencil Beam Lights	33

8.1.3.	Additional Lights	34

8.2.	Daymarks versus Daytime Lights	34

8.3.	Colour and Flashes	35

8.4.	Tower Placement	35

8.5.	Beacon Placement	35

8.6.	Servicing Considerations	36

8.7.	Construction Details	36

8.8.	Safety	36

8.9.	Daymark Mounting	36

8.10.	Sensitivity of a Leading Line	36

9.	references	37

10.	Abbreviations and Technical Quantities	38

10.1.	Roman	38

10.2.	Greek	41

List of Tables

Es konnten keine Einträge für ein Abbildungsverzeichnis gefunden werden.

List of Figures

Figure 1	Geometrie of a leading line and appearance for observer, when off centreline	6

Figure 2	Geometrie of a leading line and appearance for observer, when at centreline	6

Figure 3	Channel parameters (plan view)	8

Figure 4	Observer's parameters (plan view)	8

Figure 5	Vertical plane (side view)	8

Figure 6	Heights of lights and daymarks	9

Figure 7	Elevation and heights	14

Figure 8	Bearing difference	15

Figure 9	'Apparent magnitude of front and rear light'	16

Figure 10	Bearing and elevation difference	17

Figure 11	Minimum height of front light considering an obstruction	19

Figure 12	US Coast Guard Daybord	22

Figure 13	Comparison of recommended daymark lengths	23

Figure 14	Design process	26

Figure 15	Two range designs having the same CTF at the far end but different CTF at the near end	32

Figure 16	Beam Width of Leading Lights	34

Figure 17	Tower Placement	35

Figure 18	Required intensities when calculated with different input for V and E	43

Figure 19	Recommended Intensity for a meteorological visibility Vmin=10 M (red dotted line)	44

Figure 20	Luminous intensity of German leading lights	45

Figure 21	Recommended luminous intensity (simple method)	46

# Introduction

A leading line is a straight line produced by the alignment of (usually) two navigational marks (leading marks) or lights (leading lights) and used for marking straight sections of fairways or channels. Vertical alignment of the marks and/or lights defines the centreline of a channel. The part of the line intended for navigational use is called the  useful segment” of the leading line.

The design of a leading line has to consider:

geometrical parameters and the layout of the leading line;

the luminous intensity of the ligh;t and

dimensions of the daymark.

The performance of a leading line can only be guaranteed when all parameters work together. Therefore, extensive and iterative calculations are usually necessary.

Geometry of a leading line and its appearance for observer, when off centreline

Geometry of a leading line and its appearance for observer, when at centreline

# Purpose

The purpose of this document is to provide the IALA membership with guidance on the principles of operation of leading lines and the calculation used for their design.

This document does not provide guidance on the nautical requirements for design of leading lines.

# Background

Complete text from E-112 Annex 1, cannot be improved

Leading lines allow ships to navigatewith precision along straight sections of a route. As a Marine Aid to Navigation they are reliable and simple to use providing visibility is sufficient to see the leading marks/lights..

It is therefore common practice to guide ships along narrow fairways by means of leading lights or, more generally, a series of leading lights in succession, the useful segments of which form a continuous series of straight lines. This is similar to the way the centrelines of artificial channels are laid out in a series of straight lines along which ships will be guided by leading lights. The availability of suitable sites for the leading marks may influence the selection of ship courses in natural, narrow fairways or in the layout of artificial channels.

In order to reach the useful segment of a leading line, it will often be necessary to observe at least one of the leading marks or lights whilst in a region to seaward or to the side of it. This region is called the “acquisition region”.

To reach the first useful segment of a leading line, it is necessary to observe both leading marks or lights while in a seaward region. This region is called the "acquisition region." Observing both marks or lights will allow the navigator to perform the maneuver with sufficient safety margin to properly navigate the channel. The size of the acquisition zone must be taken into account in the calculation of the main line (at least in terms of intensities and divergences).

When using a leading line, the determination of the “usable width of a channel” or of the axis to be marked in a natural channel for each particular case requires taking into account not only of the inaccuracy (sensitivity) of the relevant aid to navigation (leading line), but also different 'nautical margins', such as those resulting from the breadth of ships, extra widths required for vessels to pass each other, the drift angle between the shipping route and the head of the ship when a ship is submitted to the effect of transverse winds and currents, uncertainties resulting from inaccuracies in hydrographic surveys or possible changes in the sea bottom since the last survey, etc.

This guideline is only related to the inaccuracy of the aid to navigation proper, leaving out the different 'nautical margins' that should be taken into account but which depend on local conditions and can only be determined after considering each particular case.

# History

Although leading lines play an important role for navigation in narrow? channels, there was no internationally published design methodology or specification before 1998. The appearance and the design of leading lines is therefore very different at each nation.

The first standardization was published by IALA with Recommendation E-112 in 1998 [1]. The recommendation includes basic requirements on the position, height and luminous intensity of leading lights only. As a design methodology was missing a spreadsheet program for leading line calculation was created and the manual for the use of the spreadsheet became IALA Guideline 1023 in 2001 [2]. In addition to the methodology from the Recommendation, the Guideline includes a proposal for daymarks. Both recommendation and guideline are mainly influenced by the standard design of the United States Coast Guard.

Since then the IALA documents have been widely used for the design of new leading lines. However, many leadings lines were built long before 1998 and some nations still use their national design tools or specifications, when a leading line must be changed or newly built.

The present update of the 2001 guideline follows the idea to present the various aspects, which should be looked at, when designing a leading line. The original procedure is still present, but explanations and discussions are given to enable the users to adapt their own methodologies to the IALA procedure.

# Definitions and Parameters

The calculation of the design of a leading line needs the variables described in this section. The channel is idealised to a horizontal rectangle, which is symmetrical about the centreline (see Figure 3).

For the front daymark and the front light the abbreviation FL is used and for the rear daymark and light RL.

The units used follow the SI-Standard (length in metres [m], Illuminance in luxes [lx], luminous intensity in candelas [cd]) and the IHO Chart Specifications (nautical mile is abbreviated to a capital 'M': 1 M=1852 m). All heights are referenced to the datum given by IHO Chart Specifications [5].

To simplify the calculations all the angles are in radians, unless stated otherwise. In many cases the angles are small and therefore some approximations are valid (αradians≈sinα≈tanα and cosα≈1).

The updated guideline avoids 'hidden dimensions'. Hidden dimensions have been in use in IALA documents for many decades. For example in ex, it was assumed in the guideline that the value of x is put in as metres or nautical miles without the dimension. This may cause problems, especially when further explanations are missing. To avoid this situation 'hidden dimensions' are not used but all dimensions are named explicitly.

Instead of writing ex the term exdU is used. This will work for x both in metres and in nautical miles: e1000 m1 M=e1000 m1852 m= e0,54 M1 M=e0,54.

This is done for distance with dU=1 M=1852 m, height hU=1 m and illuminance EU=1 lx.

## Geometry, Plan View

C	Length of useful segment

L	Distance to front structure from far end to useful segment (L=C+M)

M	Distance to front structure from near end to useful segment

R	Distance between leading line structures

W	Channel width

Channel parameters (plan view)

x	Distance of observer (vessel) from front tower (parallel along centreline)

y	Distance of observer from centerline of channel (perpendicular to centreline)

θ	Bearing difference (horizontal angular difference between front and rear light)

Observer's parameters (plan view)

## Geometry, Side View

The height of the observer on the vessel is referenced to sea level (at vessel’s location?). All other heights are referenced to mean high water (MHW), or mean sea level (MSL), when the mean tidal range is small or zero (MTR<0.3 m).

Vertical plane (side view)

Heights of lights and daymarks

Hv	Height of observer (on vessel) above sea level

HFL	Height of front light above MHW or MSL

HRL	Height of rear light above MHW or MSL

Hshaw	Safe height above water

LFL,DM	Vertical length of the front light daymark

LRL,DM	Vertical length of the rear light daymark

MHW	Mean high water

MLW	Mean low water

MSL	Mean sea level

MTR	Mean tidal range

ZFL	Dip of horizon at front light

ZRL	Dip of horizon at rear light

γ	Elevation difference (vertical angular difference between front and rear light)

## Photometry

Basic information on the photometric calculations needed in design of leading lines can be found in IALA Guideline G1148 - Determination of Required Luminous Intensity for Marine Signal Lights [1]. Based on this guideline the following parameters are / have to be considered in calculation of leading lights… or some other introduction to the following.

EFL	Illuminance at the eye of the observer produced by the front light

ERL	Illuminance at the eye of the observer produced by the rear light

Er	Required minimum illuminance at the eye of the observer
The required minimum illuminance at the eye of the observer depends on background lighting.  Er=10-6 lx is used for situations without background lighting, Er=10-5 lx for minor and Er=10-4 lx for substantial background lighting. For a daytime light the required illuminance is Er=10-3 lx.

Emax	Allowed maximum illuminance at the eye of the observer
The allowed maximum illuminance at the eye of the observer is needed to avoid glare.  Emax=0.01 lx is used for situations without background lighting, Emax=0.1 lx with background lighting. It is very unlikely to cause glare with daytime lights, so there is no need to check for and no maximum value for the illuminance is defined for daytime lights.

IFL	Luminous intensity, front light

IRL	Luminous intensity, rear light

Vmin	Minimum Meteorological visibility
The minimum visibility should be based on the historic value of meteorological visibility at the site that is met or exceeded 90% of the time. This value is used to establish the minimum luminous intensities, required to ensure that the leading lights are usable as leading line signals at least 90% of the time. Typical values are in the range of Vmin=3⋯5 M=5556 ⋯9260 m.

Vdsg	Design Meteorological visibility
The design visibility was originally conceived as the median value of meteorological visibility for the site. That is, the value met or exceeded 50% of the time. Design visibility is used to establish the recommended ratio of luminous intensities of the front and the rear light. As a practical consideration, use of a fixed value Vdsg=10 M=18 520 m is recommended.

Vmax	Maximum Meteorological visibility
The potential for glare produced by the lights is calculated with the maximum meteorological visibility. Typically Vmax=20 M=37 040 m is chosen.

## Miscellaneous

Some symbols are defined in individual chapters and their meaning is explained in the context. For these the following predefinitions are used.

DDM	range of a daymark (usually according to US Coast Guard convention)

DM	indicates the use of daymarks (yes/no)

DTL	indicates the use of daytime lights (yes/no)

E	illuminance

H	height

I	luminous intensity

L	(vertical) length of a daymark, distance from the front mark to the far end

r	ratio

Rg	geographical range

S	distance between an obstruction and front light (parallel to centreline)

V	visibility

Windex	extension or width, when used with index, usually for the daymark

x	distance (parallel to centreline)

y	distance (perpendicular to centreline)

Z	dip of horizon

α	factor for dip calculation

ϕ	angle used to describe horizontal divergence

The following indices are used.

DM	daymark

dsg	design

far	far, referencing the far end of the channel

FL	front light

initial	initial iteration value

max	maximum

mid	middle, referencing the middle of the channel

min	minimum

min1, min2…	minimum according to the requirement referenced

near	near, referencing the near end of the channel

obs	obstruction

rec	recommended

RL	rear light

sel	selected

shaw	safe height above water

v	vessel, referencing the observer at the vessel

# Basic Equations

The equations are presented in the order of the IALA design methodology. However, the competent authority may use a different design methodology.

## Tower Position

To start a calculation an initial value for the tower positions the horizontal dimensions of the channel (C, W) must be given.

When the locations of the towers are not prescribed by geographical or other restrictions the following formulas can be used as an initial determination of their positions.

Distance from the near end to the front tower:	Minitial=C5=0.2∙C	(1)

Distance between the towers:	
Rinitial=a+C1000*W3.4∙a+C-1=L1000∙W3.4∙L-1	(2)

Explanation for Rinitial:

For the calculation of the initial value of Rinitial it is assumed that the observer is able to detect that he is distance YD=0.1∙W apart from the centerline at the far end of the channel. Furthermore it is assumed that the elevation difference at this position is γ=1.5∙10-3 rad (≈5') and the bearing difference becomes θ=0.34∙10-3 rad.

## Luminous intensity

The intensity is calculated with Allard’s Law and needs the input of a distance x, a visibility Vand a threshold for the illuminance E at the eye of the observer.

I=E∙x2∙0.05-xV	(3)

It is recommended to put in the values with SI units (metre, lux, candela).

When an intensity is fixed, the illuminance at the eye of the observer is:

E=I∙x-2∙0.05xV	(4)

### Minimum Luminous Intensity

The required minimum intensity of the front and rear light are calculated for the minimum visibility (worst atmospheric condition), required illuminance at the eye of the observer and for the maximum distance (far end of the channel.)

Minimum intensity (FL, night):	IFL,min=Er∙M+C2*0.05-M+CVmin	(5)

Minimum intensity (RL, night):	IRL,min=Er∙R+M+C2*0.05-R+M+CVmin	(6)

where	Er	required illuminance at the eye of the observer;

Vmin	minimum visibility;

M+C	distance from far end of channel to front light; and

R+M+C	distance from far end of channel to rear light.

### Recommended Luminous Intensity of Front Light (Night)

The recommended luminous intensity of the front light should be above the required minimum value. It may be derived with the methodology tools of IALA Guideline G1148 - Determination of Required Luminous Intensity for Marine Signal Lights [3].

For short ranges it has been proposed in the '2001 IALA Guideline' that, in order to provide a good, not just adequate signal, the recommended luminous intensity of the light is ten times the minimum value. However, for channel lengths above 5 M the factor 10 causes very intense lights and therefore high costs. In these cases a factor 'two' may be sufficient.

An alternative way to determine the luminous intensity for the front light is a simplified method derived from analysis of data of existing “good”? leading lines (in Germany?) and this empirical method does not need an input of a minimum visibility and background illumination (APPENDIX 1).

The recommended intensity of the front light (FL, night) according to this method is:

IFL,rec=10∙IFL,min	(suitable for a range up to 5 M)	(7)

IFL,rec=2∙IFL,min	(suitable for a range above 5 M)	(8)

IFL,rec=Irec,M∙M+CdU3	(9)
(simplified equation where Irec,M=1000 cd and dU=1852 m=1 M)

In any case, the designer of the leading line may choose any intensity, as long as it is between the minimum and maximum value.

### Recommended Luminous Intensity of Rear Light (Night)

The illuminances at the eye of the navigator within the useful channel segment provided by the two lights should be as nearly equal as possible. This requires a suggested intensity ratio between the front and rear lights and determining the intensity of the rear leading light based on the intensity of the front light. Allard’s law is used to determine the ideal ratio.

The illuminance values for the design visibility Vdsg are:

EFL=IFL∙x-2∙0.05xVdsg	(10)

ERL=IRL∙R+x-2*0.05R+xVdsg	(11)

The ratio of illuminations is:	EFLERL=IFLIRL∙R+x2x2∙0.05xVdsg0.05R+xVdsg=IFLIRL∙R+x2x2∙0.05-RVdsg

The ideal ratio for the illuminance should be equal to one:  EFLERL=1.

This gives a recommended ratio for the intensities r:

r=IRLIFL=R+x2x2∙0.05xVdsg0.05R+xVdsg=R+x2x2∙0.05-RVdsg

However the ratio depends on the observer's distance from the (front) light x. Therefore the ratio IRLIFL is calculated twice, first for a position in the middle of the channel and secondly for a position at the far end.

Middle of the channel:

The distance from the middle of the channel to

the front light (FL) is:	XFL,mid=M+C2

the rear light (RL) is:		XRL,mid=R+M+C2

With these equations the ratio becomes:	r1=IRLIFL=XRL,mid2XFL,mid2∙0.05XFL,midVdsg0.05XRL,midVdsg=R+M+C22M+C22∙0.05-RVdsg

Far end of the channel (here the reciprocal ratio is calculated):

The distance from the far end of the channel to

the front light (FL) is:	XFL,far=M+C

the rear light (RL) is: 	XRL,far=R+M+C

With these the ratio of intensities becomes:	r2=IFLIRL=XFL,far2XRL,far2∙0.05XRL,farVdsg0.05XFL,farVdsg=M+C2R+M+C2∙0.05RVdsg

Recommended ratio of intensities:

The recommended ratio between the luminous intensities of front and rear light is then selected as follows:

if r1<2r2  the recommended ratio is r=r1
	If r1≥2r2  the recommended ratio is r=2r2

And the recommended night time intensity for the rear light is :	IRL,rec=r∙IFL,rec	(12)

To Pärtel:

- I added two alternatives for the recommended value.

- Are there other alternatives for a recommended intensity?

At daytime the luminous intensity of a light needs to be magnitudes higher than at nighttime. This may cause very complex lighting and power systems. Therefore recommended luminous intensity is not calculated for daytime lights. Instead it is assumed that the selected intensity will be nearby the minimum value.

### Maximum Luminous Intensity (Night)

To avoid glare the luminous intensity should not exceed a certain maximum value. The maximum allowed intensity is calculated for both front and rear light for very good atmospheric viewing conditions (Vmax=120 M) and at the near end of the channel:

Maximum intensity (FL, night):	IFL,max=Ermax∙M2∙0.05-MVmin	(13)

Maximum intensity (RL, night):	IRL,max=Ermax∙R+M2∙0.05-R+MVmin	(14)

where	Ermax	maximum allowed illuminance at the eye of the observer;

Vmaxin	maximum visibility;

M	distance from near end of channel to front light; and

R+M	distance from near end of channel to rear light.

Because of practical reasons it is nearly impossible to produce glare at daytime. Therefore, a maximum intensity is not determned for daytime lights.

## Elevation Angles

The elevation angle is the vertical angle of a light or point to the horizontal plane of the observer. It can be calculated from the heights of the observer and the light or point, the tidal range and the distance between them. For a leading line the difference of the elevation angles of front and rear light plays an important role for correct design.

The equations necessary are derived from Figure 7.

Elevation and heights

At mean low water, the elevation angles are:

γFL≈tanγFL=HFL-ZFL-(Hv-MTR)x	(15)

γRL≈tanγRL=HRL-ZRL-Hv-MTRx+R	(16)

At longer distances the height of a light above horizon is lowered by the part of the height that is hidden behind earth curvature (dip of the horizon). The value of dip of the horizon depends on the distance x between the observer and the light.

ZFL≈α∙x2	(17)

ZRL≈α∙x+R2	(18)

where α=6.75∙10-8m-1.

The elevation difference (vertical angle between front and rear light as seen from the observer) is then:

γ=γRL-γFL=HRL-ZRL-Hv+MTRx+R-HFL-ZFL-Hv+MTRx →

γ=HRL-α∙x+R2-Hv+MTRx+R-HFL-α∙x2-Hv+MTRx →

γ=HRL-Hv+MTRx+R-HFL-Hv+MTRx-α∙R	(19)

where α=6.75∙10-8m-1.

The elevation difference depends on the current sea level, the observer height and the distance.

## Bearing Angles

The bearing angle describes the horizontal angular deviation of the direction the lights are seen to the centreline. The bearing difference is the horizontal angle between front and rear light seen from the observer. It is linked to the horizontal distance of the vessel from the centerline.

The equations necessary are derived from Figure 8.

Bearing difference

From Figure 8:

θFL≈tanθFL=yx

θRL≈tanθRL=yx+R

θ=θFL-θRL=yx-yx+R=y∙Rx∙(x+R)	(20)

The relation between the distance from the axis y and the horizontal bearing difference θ is then:

y≈Θ∙x*R+xR=Θ∙x∙1+xR	(21)

## Minimum Elevation Difference

For the elevation difference between front and rear light two different requirements exist:

The 'precision' of a leading line depends on the elevation difference and is better when both lights appear very close (see 6.6).

There is a minimum value  γmin for the difference, the limit where? the lights do not yet overlap but can still be clearly seen as two separated lights.

The minimum required? allowed? acceptable? difference γmin between the lights depends on the illuminance at the eye of the observer generated by the lights. In general all maritime lights are seen under a viewing angle below the eye resolution (approx. 1'=0.291∙10-3rad). However the lights appear larger or smaller for the observer's eye according to their brightness. This effect is known from astronomy as the 'apparent magnitude of a star'.

'Apparent magnitude of front and rear light'

To take that effect into account and calculate the minimum required difference γmin the factual luminous intensity of front and rear light must be selected.

selected luminous intensity FL:	IFL,sel

selected luminous intensity RL:	IRL,sel

The illuminance at the eye of the observer is then calculated for very good viewing conditions (Vmax=20 M, Tmax=0.861) and for the 10 equally spaced positions along the channel.

illuminance from FL:	EFL=IFL,sel∙x-2∙0.05xVmax	(22)

illuminance from RL:	ERL=IRL,sel∙x+R-2∙0.05x+RVmax	(23)

The minimum difference angle is then:

γmin=2.4-0.06∙logERLEFL+0.26∙logERLEFL2+logE+EU∙0.2-0.02∙logERLEFL-0.02∙logERLEFL2∙10-3rad	(24)

where E+=maxEFL,ERL at nighttimemaxEFL,ERL10000 at daytime and EU=1 lx.

Ask Pärtel:

Some nations use fixed values for γmin (e.g. Germany), should we mention them?

## Sensitivity of a Leading Line

The sensitivity (or precision) of a leading line is  the magnitude of the minimum value of the bearing difference ΘD of the two lights, when an off axis deviation in a definite direction is considered as detected with certainty by the observer.

Bearing and elevation difference

The bearing difference  depends on the elevation angle γ (vertical angle) and is calculated using the formulae:

ΘD=maxΘ'1,Θ'2	(25)

where

Θ'1=0.16∙10-3 rad+0.12*γ	for	 γ≤5∙10-3 rad;

Θ'1=0.31∙10-3 rad+0.09*γ	for	 5∙10-3<γ≤20∙10-3 rad;

Θ'2=0.224∙γmin	(see 6.5).

Ask Pärtel:

In Germany we have a different equation for sensitivity.

Are there others? Should we mention them?

## Cross-Track Factor

The cross-track factor CTF is an instrument to classify the sensitivity or precision of a leading line design. The cross-track factor is calculated as the ratio of the distance YDthe navigator can detect with certainty that the vessel is not on the channel centreline YD, divided by the half-width of the channel expressed as a percentage.

From equations (21) and (25) YD is:

YD≈ΘD∙x∙1+xR	(26)

The CTF then becomes:	(27)

CTF=100%∙YD0.5*W

For a fixed leading line design, the cross-track factor varies along with the current position x of the vessel in the channel and the observer's height Hv. The cross-track factor is classified in Table 1.

Cross_Track Factor

| CTF | Description | Interpretation |
| --- | --- | --- |
| over 75% | Not Acceptable | Range must be improved or it will be unworkable. |
| 50% - 75% | Poor | Decrease the cross-track factor if physically possible. |
| 30% - 50% | Fair | Decrease the cross-track factor only if moderate cost involved. |
| 20% - 30% | Good | Decrease the cross-track factor only if little cost involved. |
| 15% - 20% | Very Good | Do not expend more funds to decrease the cross-track factor. |
| 10% - 15% | Excellent | The cross-track factor should not be less than 10% at the far end of the channel. |

## Geographical Range

The geographical range Rg of a light or a daymark is calculated by the following equation.

Rg=2.03 du∙HhU+HvhU	(28)

where

H: 	Height of the light or lower end of the daymark;

Hv:	Height of the observer on vessel;

hU=1 m:	Unit for height;

dU=1 M=1852 m	Unit for distance (1 nautical mile).

Remark: Written with hidden dimensions, the equation is: Rg=2.03∙H+Hv.

When a required geographical range is given, then the minimum height of a light or a daymark achieving that range can be derived from:

H=Rg2.03∙dU-HvhU2∙hU	(29)

Remark: Written with hidden dimensions, the equation is: H=Rg2.03-Hv2

## Heights

There are several requirements for the height of the front and rear light. Each requirement leads to a different minimum value of the height in the form of Hlight≥Hmin. Each minimum value for the height caused by individual requirements is given an numerical? index to identify it according to its requirement. The height of the lights are significantly controlled by the minimum observer height Hv=Hv,min the leading line is designed for. It is assumed that for observer heights lower than Hv,min the leading line will not work.

### Front Light

#### Safe Height above Water

- To avoid damage from waves or vandals or obstruction by e.g. grass the front light HFL has to be above the safe height above water (Hshaw):

HFL≥HFL,min1=Hshaw.	(30)

- When a daymark with length LFL,DM,sel is used, the lower end of the daymark should be above the safe height and the light above the upper end of the daymark: HFL,lower_end_daymark≥Hshaw.

- In this case the minimum front light height is:

HFL≥HFL,min2=Hshaw+HFL,DM,sel	(31)

#### Geographical Range

The height of the front light above mean high water must be large enough to be seen above the horizon i.e. its geographical range exceeds the distance to the far end of the channel Rg=M+C.

HFL≥HFL,min3=Rg2.03 dU-HvhU2∙hU	(32)

- When a daymark with length HFL is used, the lower end of the daymark must be high enough above mean high water so that its geographical range exceeds the far end of the channel.

- HFL,lower_end_daymark=Rg2.03 dU-HvhU2∙hU
When the light is positioned at the upper end of the daymark the daymark length must be added to the minimum front light height.

HFL≥HFL,min4=Rg2.03 dU-HobshU2+HFL,DM∙hU	(33)

#### Avoid Occlusion by Obstruction

When an obstruction is between the front light and the observer and it seems to be relevant for the visibility of the front light, a calculation is necessary to estimate whether the front light is occluded. The height Hobs and the distance S between the obstruction and front light are needed for this calculation.

Minimum height of front light considering an obstruction

From Figure 11 the minimum height of the front light HFL,min can be derived (ship/observer at  mean low water).

Hobs-Zobs-Hv-MTRx-M+S=HFL,min-ZFL-Hv-MTRx

HFL,min=xx-M+S∙Hobs-Zobs-Hv+MTR+ZFL+Hv-MTR

The dip of horizon for obstruction and front light is:

Zobs=α∙x-M+S2	and	ZFL=α∙x2	where α=6.75∙10-8m-1.

This leads to a minimum height for the front light considering presence of the obstruction and the geographic range of… / dip of horizon for…?:

HFL,min=xx-M+S∙Hobs-Hv+MTR+Hv-MTR-α∙x∙x-M+S+α∙x2

HFL,min=Hv-MTR+xx-M+S∙Hobs-Hv+MTR+α∙x∙M-S

The equation is used for x=M+C (far end) and x=M (near end) for the lowest height of eye Hv,min.

Far end:
HFL,min5=Hv,min-MTR+M+CC+S∙Hobs-Hv+MTR+α∙M+C∙M-S	(34)

Near end:
HFL,min6=Hv,min-MTR+MS∙Hobs-Hv+MTR+α∙M∙M-S	(35)

When the front light has a daymark with length LFL,DM, it is assumed that the lower end of the daymark must not be obstructed and that the light will be at the upper end of the daymark. In this case the selected height of the daymark LFL,DM,sel is added to the minimum front light height.

Far end:
HFL,min7=LFL,DM,sel+Hv,min-MTR+M+CC+S∙Hobs-Hv+MTR+α∙M+C∙M-S	(36)

Near end:
HFL,min8=LFL,DM,sel+Hv,min-MTR+MS∙Hobs-Hv+MTR+α∙M∙M-S	(37)

#### Selected Front Light Height

The recommended minimum height HFL,rec is the maximum value of the heights calculated above. With this value, all requirements stated before in this chapter are fulfilled.

However, a different value for the height may be chosen, which is called HFL,sel.

### Rear Light

The calculation of the rear light height requires that the front light height has already been selected (HFL,sel).

#### Blur

The rear light must be high enough so that the elevation difference in whole useful segment is larger than γmin (6.5), to avoid blur.

γmin=HRL-Hv+MTRx+R-HFL,sel-Hv+MTRx-α∙R

HRL,min=x+R∙γmin+HFL,sel-Hv+MTRx+α∙R+Hv-MTR

The minimum height of the rear light considering blur is calculated for x=M+C (far end) and x=M (near end) for the lowest height of eye Hv.

Far end:
HRL,min1=M+C+R∙γmin+HFL,sel-Hv+MTRM+C+α∙R+Hv-MTR	(38)

Near end:
HRL,min2=M+R∙γmin+HFL,sel-Hv+MTRM+α∙R+Hv-MTR	(39)

#### Geographical Range

From blur calculation, which includes the dip of horizon according to the Earth curvature, it is guaranteed that the rear light always appear above the horizon as well. So there is no need to check the rear light for geographical range. However, for the list of lights and the charts the geographical range of the rear light may be calculated separately.

#### Front Light not obscuring Daymark of Rear Light

At the far end of the channel (x=M+C) the entire daymark area of the rear light must appear above the front light.

Far end:
HRL,min3=LRL,DM,sel+Hv-MTR+M+C+R∙HFL,sel+MTR-HvM+C+α∙M+C+R∙R
	(40)

At the near end of the channel (x=C) it is not necessary that the entire daymark is visible, because it appears much larger to the observer. An accepted compromise is that one half of the rear light daymark is not obscured by the front daymark.

Near end: 
HRL,min4=12∙LRL,DM,sel+Hv-MTR+M+R∙HFL,sel+MTR-HvM+C+α∙M+R∙R	(41)

#### Avoid Occlusion by Obstruction, without Daymark

The equations for the front light (6.9.1.3) are valid for the rear light with the following transitions:

HFL,min		→	HRL,min

M		→	M+R

This leads to:

HRL,min=Hv-MTR+xx-M-R+S∙Hobs-Hv+MTR+α∙x∙M+R-S

Far end:
HRL,min5=Hv-MTR+M+R+CC+S∙Hobs-Hv+MTR+α∙M+R+C∙M+R-S	(42)

Near end:
HRL,min6=Hv-MTR+M+RS∙Hobs-Hv+MTR+α∙M+R∙M+R-S	(43)

#### Avoid occlusion by obstruction, with daymark

At the far end of the channel (C+M+R) the entire daymark area of the rear light must appear above the obstruction. In this case the full height of the daymark is added to the minimum rear light height.

Far end:
HRL,min7=LRL,DM,sel+Hv-MTR+M+R+CC+S∙Hobs-Hv+MTR+α∙M+R+C∙M+R-S	(44)

At the near end of the channel (x=M) it is not necessary that the entire daymark is visible, because it appears much larger to the observer. An accepted compromise is that one half of the rear light daymark is not obscured by the obstruction.

Near end:
HRL,min8=12∙LRL,DM,sel+Hv-MTR+M+R∙HFL,sel+MTR-HvM+C+α∙M+R∙R	(45)

#### Selected Rear Light Height

The recommended minimum height is the maximum value of the heights calculated before. With this value, all requirement stated before in this chapter are fulfilled.

However, a different value for the height may be chosen, which is called HRL,sel.

## Daymark

The size of the daymark depends on the required range: the longer the range, the greater the size of the daymark. When considering geometry only, the dimensions (length LDM or width WDM) of a daymark should be proportional to the required range: LDM~DDM and  WDM~DDM with DDM as the required range of a daymark.

### Guideline 2001 Design

However, the atmosphere will reduce the contrast of the daymark for long ranges according to 'Koschmieder's law'. The loss of contrast will reduce the range of the daymark. This can be compensated by increasing the daymarks size with an exponential function of the daymark range:  LDM~expDDM and  WDM~expDDM.

The US Coast Guard has done some investigations on daymarks [6] and have decided? that for their design the recommended height of the daymark can be calculated with the equation

LDM,rec=hU∙expDDM-aVminbVmin LDM,rec=hU∙eDDM-aVminbVmin 	(46)

where hU=1 m and aVmin, bVmin are the coefficients presented in Table 2 [6].

The inverse function is the range DDM of the specific daymark (US Coast Guard design)

DDM=a(V )+b(V )∙lnLDMhU	(48)

where V as the visibility, LDM the daymark length and hU=1 m.

The recommended daymark width is WDM,rec=23∙LDM,rec. This calculation was introduced in the 2001 IALA Leading Line spreadsheet.

Guideline 2001 Daybord Design

The calculation of the daymark size is based on the distance from the tower to the far end of the channel XFL,far=aM+C (for front light) or XRL,far=R+M+C (for rear light) and the minimum visibility Vmin. The visibility is represented by an integer value of 1 to 10 nautical miles (M), so there are 10 equations to calculate the size.

Daymark length

| Visibility Vmin [M] | Daymark length (vertical) | (49) |
| --- | --- | --- |
| 1 | LDM,rec=1 m∙exp⁡X-651.8 m806.4 m |  |
| 2 | LDM,rec=1 m∙exp⁡X-689.3 m1235.6 m |  |
| 3 | LDM,rec=1 m∙exp⁡X-377.8 m2041.5 m |  |
| 4 | LDM,rec=1 m∙exp⁡X-242.5 m2689.5 m |  |
| 5 | LDM,rec=1 m∙exp⁡X-107.2 m3337.5 m |  |
| 6 | LDM,rec=1 m∙exp⁡X+19.9 m3714.7 m |  |
| 7 | LDM,rec=1 m∙exp⁡X+159 m4199 m |  |
| 8 | LDM,rec=1 m∙exp⁡X+297.5 m4522.8 m |  |
| 9 | LDM,rec=1 m∙exp⁡X+326.4 m4896.2 m |  |
| 10 | LDM,rec=1m∙exp⁡X+489.2 m5386.1 m |  |

where

X=XFL,far for front light and

X=XRL,far for rear light.

### Alternative Size Calculation

The US Coast Guard calculation leads to very large daymark sizes, when long ranges or poor visibility have to be considered. As an atternative way, the size may be based on geometrical calculation only.

This can be done with the tools of IALA guideline 1094 .

The daymark should appear with a minimum vertical subtense angle of 3' (0.873 mrad) at the far end of the channel. For better conspicuity the double value may be chosen (6' = 1.745 mrad), which is near to the recommended value of topmarks for buoys (IALA 1983).

The recommended height for the daymarks are therefore

Required subtense angle 3':		
LDM,rec=0.873∙10-3∙X (minimum IALA requirement)	(50)

Required subtense angle 6':		
LDM,rec=1.745∙10-3∙X (enhanced daymark conspicuity)	(51)

where

X=XFL,far for front light and

X=XRL,far for rear light.

The recommended daymark lengths according to the three different models, including the lengths for ten different visibility values, are shown in Figure 13.

Comparison of recommended daymark lengths

The maximum length of a daymark with guideline 2001 dimension is approximately 12 m. It is assumed that the ratio length :width=3 : 2. Although the ratio 3:2 is used in 2001 IALA guideline, US Coast Guard itself prefers the ratio 2:1.

When the ratio between vertical length and width is increased the length may be above 12 m.

The German authority often uses lighthouses without a dayoard as daymarks. For long range lights the accepted vertical subtense angle at maximum viewing distance is 3' (= 0.873 mrad). The accepted ratio is 3:1.

As a conclusion the 3'-rule is an easy and suitable method for calculating the daymark length, which results in moderate daymark sizes with no need for further investigation on minimum visibility.

Finnish method for consideration

k = d * 0,00052 + v1

l = d * 0,0004 + v2

k – height of the dayboard [m];

l – width of the dayboard [m];

d – viewing distance [m];

v1 ja v2 – constants.

Constants:

- Commercial fairways at sea: v1 = 1,9 ja v2 = 1,4

- Inland and small craft routes: v1 = 1,3 ja v2 = 0,9

On commercial fairways at sea the minimum dimensions are 4.5*3.0 m, at Inland and small craft routes 1.5*1.0 m

### Alternative Daymark Designs

The shape and the colour of a leading line daymark is not restricted by the IALA Maritime Buoyage System (MBS). Competent authorities may chose different design according to their own experience/preference, but rectangular and triangular shapes are recommended by MBS.

In any case, it is advised to choose the vertical length of the daymark with the tools provided in chapter 6.10.1 or 6.10.2.

#### Length Width Ratio

Although the IALA 2001 Design was introduced by US Coast Guard into the IALA guideline 2001, the design of the daymarks are different in the United States [9].

The ratio between the length and the width according to AtoN Manual of US Coast Guard is 2 : 1 and the centre area has a width of LDM/8 only.

U.S. Coast Guard Design

#### Colours

When a series of leading lines are used, there may occur confusion to correlate front and rear daymark of a leading line. In this case it is advised to use different colours for the daymark of each leading line. So the leading line can be identified by its specific colour code used for front and rear daymark.

Alternative daymark colour codes (U.S. Coast Guard)

The colours chosen may also increase the contrast to a specific background. White outer bars often produce a good contrast to forest or rock as background, whereas black may be preferred when the sky or horizon is the background.

#### Shapes

Many nations use non-rectangular shapes for the daymark. The shape of front and rear daymark are mirrored. Some nations use stripes in 'nun-shape' and some nations use equilateral triangles.

Alternative daymark shapes

#### Retroreflective Daymarks

Some nations use retroreflective sheets for the daymark. The retroreflective materials replaces the light for night time use, however, with much shorter ranges. So at night time, the mariner needs to illuminate the front and rear daymarks with a searchlight to use the leading line.

More information is given in IALA Guideline G1145 Application of Retroreflecting Material on AtoN.

Finnish guidance about reflectors:

Reflectors are used as a substitute for lights at small craft routes and other less important fairways. They can be useful for other fairways too in case of damaged lights. The leading line still functions when used with a search light. Small crafts are assumed to have search lights with intensity of ca 10 000 cd and larger vessels search lights with 100 000 cd.

Minimal area of the reflector is found with the formula

Converted for illumination in the observer’s eye

A – area of the reflector, m2

E – threshold of the illumination at the eye, lx

d – viewing distance, m

I – intensity of the search light, cd

R – reflectivity of the retroreflector, lx/m2/cd

In principle the calculations are made as for lights, but as the problem is getting high enough reflection, maximum intensity is not calculated. The reflectors are positioned so that it covers ¾ of the middle stripe so it covers ¼ of the total area of the dayboard. Reflector of the front mark starts from the lower edge of the dayboard and reflector of the rear mark from the upper edge of the dayboard (to avoid blurring of the reflectors degrading usability of the leading line. Reflection diminishes fast with the distance. Even with high light intensities and large reflectors the largest usable distances are ca 4000 m but practical ranges are 0 …. 2500 m. For best possible reflection the reflector is always white. When the areas of the reflectors are determined, the final values of E are calculated for both marks.

Ask Pärtel: Put in here alternative designs and size calculations?

# Design Methodology

The design process is illustrated in Figure 17.  A spreadsheet considering the geometric and photometric layout of leading lines is provided by IALA.

The basic input for all calculations are the dimensions of the channel. The calculation should start with an initial (preliminary) assessment of the tower positions according to chapter 6.1.

From then on, the design methodology is 'trial and error', that means that the designer defines input parameters and looks whether the output gives suitable results. When the results are not suitable the input has to be changed until all results are acceptable (iteration loop). The result is a complete geometric (distances, dimensions) and photometric (light intensities) layout. For identification the inputs are numbered with a prefix "#", the outputs with "%" and the checks with "§".

Design process

When a daytime light is considered the design process will be repeated for daytime.

Besides the considerations described here a complete geometric and photometric layout should also be monitored concerning practical design considerations. This may include optimisation of cost/benefit ratio, maintenance, construction and power requirements. If the calculated layout is not suitable because of practical design considerations, alternative markings or channel sizes should be regarded. This may also lead to a new leading line calculation with a different channel layout.

## Preliminary Assessment of Tower Positions

This calculation on the first worksheet gives a first hint to the tower positions. It should be checked immediately whether these positions are available, before running through the rest of the process. When the positions are not available, nearby positions, which are available can be used for the first step of the iteration loop. The calculations are based on chapter 6.1.

## Iteration Input Parameters

After a preliminary assessment of the tower positions the next step is to input all necessary parameters for a complete calculation on the next worksheet. The input parameters are:

DTL	Use of daytime lights DTL=yes / no	(input #1)

C	Length of channel	(input #2)

W	Width of channel	(input #3)

MTR	Mean tidal range	(input #4)

Er	Background lighting	(input #5)

none: Er=10-6 lx

minor: Er=10-5 lx

substantial: Er=10-4 lx

Hv	Height of the observer’s eye (at vessel)	(input #6)

Vmin	Minimum Visibility	(input #7)

Vdsg	Design Visibility (standard is 10 M)	(input #8)

Vmax	Maximum Visibility (standard is 20 M)	(input #9)

R	Distance between front to rear light	(input #10)

M	Distance between front light to near end of channel M	(input #11)

Hshaw	Safe height above water	(input #12)

DM	Use of daymarks DM=yes / no	(input #13)

S	Optional: Position from near end of channel to obstruction	(input #14)

Hobs	Optional: Height of obstructions	(input #15)

LFL,DM,sel	Optional: Selected daymark length of front light	(input #16)

LRL,DM,sel	Optional: Selected daymark length of rear light	(input #17)

IFL,sel	Selected luminous intensity of front light	(input #18)

IRL,sel	Selected luminous intensity of rear light	(input #19)

HFL,sel	Selected height of front light	(input #20)

HRL,sel	Selected height of rear light	(input #21)

Calculation method for recommended luminous intensity	(input #22)

Calculation method for daymark length	(input #23)

Factor for geographical range	(input #24)

All calculations should be done for minimum and maximum observer height of eye Hv.

## Iteration Results

### Daymark Size (optional)

LFL,DM,rec	Front light, recommended daymark length	(output %1)

WFL,DM,rec	Front light, recommended daymark width	(output %2)

LRL,DM,rec	Rear light, recommended daymark length	(output %3)

WRL,DM,rec 	Rear light, recommended daymark width	(output %4)

Calculation is based on chapter 6.10.

### Minimum Intensity

IFL,min	Minimum intensity of front light	(output %5)

IRL,min	Minimum Intensity rear light	(output %6)

Calculation is based on chapter 6.2.1.

### Recommended Intensity

IFL,rec	Recommended intensity front light	(output %7)

IRL,rec	Recommended Intensity rear light	(output %8)

Calculation is based on chapter 6.2.2.

### Maximum Intensity

IFL,max	Maximum intensity front light	(output %9)

IRL,max	Maximum Intensity rear light	(output %10)

Calculation is based on chapter 6.2.4.

### Intensity Ratio

r	Recommended intensity ratio between front and rear light	(output %11)

IRL,selIFL,sel	Intensity ratio for selected intensities	(output %12)

Calculation is based on chapter 6.2.2.

### Recommended Heights

HFL,rec	Recommended minimum height of front light	(output %13)

HRL,rec	Recommended minimum height of rear light 	(output %14)

Calculation is based on chapter 6.9. The recommended height is the maximum value of all calculated minimum heights, so therefore all requirements are fulfilled.

### Illuminance at the Eye of the Observer

The illuminance at the eye of the observer produced by front and rear light should be calculated at various distances and meteorological visibilities.

EFL,minx	Front light, minimum visibility	(output %15)

EFL,dsgx	Front light, design visibility	(output %16)

EFL,maxx	Front light, maximum visibility	(output %17)

ERL,minx	Rear light, minimum visibility	(output %18)

ERL,dsgx	Rear light, design visibility	(output %19)

ERL,maxx	Rear light, maximum visibility	(output %20)

Calculation is based on chapter 6.2. When the luminous intensities are fixed (IFL,sel and IRL,sel), the illuminance  should be checked all inside the channel.

### Minimum Elevation Difference

γmin	Minimum elevation difference	(output %21)

The minimum elevation difference γmin is connected to the observer height and it should be checked all inside the channel.

Calculation is based on chapter 6.5.

### Cross-Track Factor

CTF	Cross-Track Factor	(output %22)

The cross-track factor CTF should be checked all inside the channel. Calculation is based on chapter 6.7.

The CTF is the final value to benchmark the leading line function.

## Iteration Assessment

The parameters of the leading line during iteration process need to be checked according to the requirements and to decide, when the iteration can be stopped.

For some results, the assessment should be done for many observer's positions inside the channel. Some output is information for the leading line designer only and does not need to be tested.

### Blur Test

The elevation angle should be equal or greater than the minimum elevation angle.

γx≥γmin	(check §1)

Elevation angle see chapter 6.3, minimum elevation angle see chapter 6.5.

### Cross-Track Factor

The cross-track factor should be acceptable for? the mariner and must not be greater than 75%.

CTF ≤75%	(check §2)

### Brightness of Front Light

The illuminance at the eye of the observer produced by the front light EFL,min should be greater than the required illuminance Er.

EFL,min≥Er	(check §3)

It needs to be checked at the far end of the channel only.

### Brightness of Rear Light

The illuminance at the eye of the observer produced by the rear light ERL,min should be greater than the required illuminance Er.

ERL,min≥Er	(check §4)

It needs to be checked at the far end of the channel only.

### Glare Test for Front Light

The illuminance at the eye of the observer produced by the front light EFL,max should be smaller than the maximum illuminance Emax.

EFL,max≤Emax	(check §5)

It needs to be checked at the near end of the channel and for nighttime only.

### Glare Test for Rear Light

The illuminance at the eye of the observer produced by the rear light ERL,max should be smaller than the maximum illuminance Emax.

ERL,max≤Emax	(check §6)

It needs to be checked at the near end of the channel and for nighttime only.

### Check Daymark Size Front Light

The range of the daymark of the front light DDM,FL (see chapter 6.10) should be equal or greater than the distance to the far end of the channel.

DDM,FL≥M+C	(check §7)

### Check Daymark Size Rear Light

The range of the daymark of the rear light DDM,RL (see chapter 6.10) should be equal or greater than the distance to the far end of the channel.

DDM,RL≥R+M+C	(check §8)

### Rear Light above Front Light considering Blur

The rear light should appear above front light with the minimum elevation angle (see 6.9.2.1).	(check §9)

Far end:

HRL,sel≥HRL,min1

Near end:

HRL,sel≥HRL,min2

### Front Light above Safe Height above Water

The light or daymark should be above safe height above water (see 6.9.1.1).

Without daymark:

HFL,sel≥HFL,min1	(check §10)

With daymark:

HFL,sel≥HFL,min2	(check §15)

### Front Light above Horizon

The light or daymark should appear above horizon (see 6.9.1.2).

Without daymark:

HFL,sel≥HFL,min3	(check §11)

With daymark:

HFL,sel≥HFL,min4	(check §14)

### Obstruction not obscuring Front Light

The light should appear above obstruction (see 6.9.1.3).	(check §12)

Far end:

HFL,sel≥HFL,min5

Near end:

HFL,sel≥HFL,min6

### Obstruction not obscuring Rear Light

The rear light should appear above the obstruction (see 6.9.2.4).	(check §13)

Far end:

HRL,sel≥HRL,min5

Near end:

HRL,sel≥HRL,min6

### Obstruction not obscuring Front Light Daymark

The entire daymark should appear above the obstruction (see 6.9.1.3).

Far end:

HFL,sel≥HFL,min7	(check §18 / § 20)

Near end:

HFL,sel≥HFL,min8	(check §18 / §20)

### Front Light not obscuring Daymark of Rear Light

At the far end the entire rear light daymark should appear above front light (see 6.9.2.3).

HRL,sel≥HRL,min3	(check §16)

At the near end the upper half of the rear light daymark should appear above front light (see 6.9.2.3).

HRL,sel≥HRL,min4	(check §17)

### Obstruction not obscuring Rear Light Daymark

At the far end the entire daymark area of the rear light should appear above the obstruction (see 6.9.2.5).

HRL,sel≥HRL,min7	(check §19 / § 21)

At the near end the upper half of the rear light daymark should appear above the obstruction (see 6.9.2.5).

HRL,sel≥HRL,min8	(check §19 / §21)

## Final Leading Line Design

The iteration process may not result in a design that fulfils all requirements. Therefore it may be necessary to start with other initial positions or to look for alternative markings. In any case the design should be checked for the properties below.

### Lateral Sensitivity

#### Cross-Track Factor

The cross-track factor CTF is the most important value to benchmark the leading line function. It is calculated with the sensitivity equation for ΘD (see chapter 8.10)

The  CTF is defined to be a ratio of the lateral distance at which a mariner can detect with certainty that a vessel is not on the channel centerline, divided by half the channel width, and expressed as a percentage.

A  CTFof 25% indicates that a mariner may be as far as 25% of the way towards the edge of the channel, when he can detect, with certainty, that he is off centreline. When using  CTF as an expression of lateral sensitivity, a higher  CTF implies a lower sensitivity and vice versa.

#### Evaluation of Acceptable Cross-Track Factors.

Table 1 in chapter 6.7 provides guidelines on the description and acceptability of various cross-track factors. Instead of setting upper (%) limits on the CTF, the designer should weigh the nautical margins available against the risk that passing vessels will be overly confined (a small  CTF (%) may result in increased risk of collision between passing vessels).

If the CTF at the far end is adequate, chances are good that the CTF at the near end is much smaller. If there are marks at the turning point at the near end, they will allow the mariner to judge the edge of the channel, and the small CTF may be of no concern.

When a small CTF is a problem, the range design can be modified to have an identical CTF at the far end, but the CTF at the near end will not be as small. Figure 18 illustrates the situation where the CTF at the far end is identical, but the CTF at the near end varies with the two designs. By moving the range structures back from the near end of the channel and increasing M and R, the CTF at the near end is increased while keeping the CTF at the far end the same.

#### Range Design Selection

Usually the range shown on the bottom of Figure 18 will give better service because the cross-track factor does not vary as much between the near and far ends, and the illuminance produced by each light on the eye of the user will be more nearly equal along the entire length of the channel. On the other hand this design might require larger dayboards, taller towers, or lights of increased intensity. It is up to the range designer to select the design that is most appropriate for the given situation.

Two range designs having the same CTF at the far end but different CTF at the near end

### Maximum Intensity less than Minimum Intensity

This situation arises when one or both leading lights (usually the FL) are too close to the channel, creating a glare problem. The best solution is to move the leading lights back from the near end of the channel, increasing M, until the situation is resolved. If that is not possible, then choose a light signal that provides the Minimum Intensity, to alleviate the glare situation as much as possible. Another way to reduce glare is to set the focal heights of the lights so as to be significantly different from the primary Hv for vessels using the channel. This may result in the user being out of the primary portion of the beam, resulting in a reduction in intensity as the vessel approaches the near end of the channel.

### Maximum Intensity less than Recommended Intensity

This problem is similar to that described above, except that the Minimum Intensity does not result in glare. It is still generally best to move the leading lights back from the near end of the channel, thereby allowing selection of the Recommended Intensity. If that is not possible, than select an intensity between the Minimum and Maximum Intensities for the constrained leading light. The intensity for the other light should be selected to try and match the recommended ratio of intensities, so as to provide a good balance of illuminances. Note that in some instances this may result in the selected intensity for the RL being less than the Minimum Intensity. The leading line designer must compromise between the intensity requirements and the recommended ratio of illuminance to optimize marking of the channel.

### Compromises during Leading Line Design

There is no single correct design for a given leading line. There are multiple successful combinations of optics, lamps, structure locations, optic heights, flash characteristics, colours, etc. With all the different designs possible for a given leading line, there comes a point where the designer must select which design to use. Selecting the design that optimizes the cost/benefit ratio for marking a channel is a trial and error process that requires practice. The selection criteria may include cost of construction, maintenance, cross-track factor, off-axis distance, tower heights, power requirements, Hv of primary user, and user input. This is just a partial list, but indicates that design of a leading line is more of an art than a science.

# supplemental INFORMATION

## Lights

### Fan versus Pencil Beam Lights

The lanterns of a leading line may consist of fan or pencil beam lights. Fan beam lights (large horizontal beam width) or omnidirectional lights are preferred when the required ranges are small and when the light should be visible far outside the channel centreline. The use of omnidirectional lanterns also precludes the requirement for passing lights on towers located in navigable waters.

However, an omnidirectional light needs much more energy and may cause glare, when vessels are passing. In this case, it is better to choose pencil beam lights

### Beam Width of Pencil Beam Lights

There is often confusion regarding the effect of a narrow spread lens on the sensitivity of a leading line. The beam width of the optic has nothing to do with the lateral sensitivity of a leading line. The beam width is of some concern when a narrow spread lens is used, as care must be taken to ensure that the minimum intensities identified as necessary are provided over the full width of the channel. The angle subtended at the far end of the useful segment ϕ, (see Figure 19) is given by:

ϕ≈WX	(52)

where: 	ϕ	in rad;

X distance from the light to the far end of the channel in metres; and

W the width of the channel in metres.

Remark:

Written in degree, the equation is: ϕ in degree≈57,3 degreeradian∙WX

The equation is used for front light (position XFL,far) and rear light (position XRL,far), which result in two different values ϕFL and ϕRL.

Beam Width of Leading Lights

The need for acquisition of the leading lights prior to turning onto a leading line should also be considered when selecting the beam width of the leading lights. In order to reach the first useful segment of the leading line it will often be necessary to observe at least one of the leading lights in a region to seaward and/or to either side of the channel. This is called the 'acquisition region'. The selected leading lights should have a beam width (and intensity) sufficient to cover the desired acquisition point.

### Additional Lights

If the design does NOT use omnidirectional optics at night, and the structure is located in navigable waters, it may be necessary to add omnidirectional lights to the structure. These additional lights should be mounted where they will not be blocked by the structure.

Additional lights should be mounted at a low enough height to ensure that the lights will be visible to vessels with a low height of eye. As an example, if a leading line has a front tower height of 8 m and a rear tower height of 25 m, the additional light for the front tower can be mounted directly above the front leading light. The additional light should display the same characteristic as the leading light, and should be synchronized with the leading light.

On towers greater than about 12 m, the additional lights may be designated as passing lights and installed at a lower level than the leading light. Installation of a passing light requires two optics, as the structure will partially occlude the output from each lantern. The passing lights should be mounted on opposite corners of the structure, and should be synchronized.

## Daymarks versus Daytime Lights

Traditionally leading lights, particularly those powered by batteries, were switched offduring daylight, with the daytime signal provided by leading marks. Recent efficiency improvements in optics combined with solar power have allowed expanded use of daytime leading lights, even when commercial power is not readily available. The following are some points to consider when deciding on the daytime signal:

Marks are simple. Having no moving parts they require little maintenance and so are more reliable than lights. Smaller marks are also easy to maintain, with no special training required for servicing personnel.

Daytime leading lights provide a superior signal. In marginal conditions they can be seen further than day marks. Furthermore, substituting lights for large marks may result in less costly tower structures and foundations. Daytime lights, however, require more complex lighting and power systems, which will increase hardware costs and the technical demands on the servicing personnel. However, the higher initial equipment costs will likely be more than offset by reduced structural costs.

## Colour and Flashes

In many cases, it is advantageous to use the same colour and synchronized flashes for front and leading light of a leading line. In the presence of rival lights this will highlight that these two lights belong together. When a channel is marked by a series of leading lines in succession, each leading line should have a different flash character than the nearby ones.

Fixed (F) characteristics should be used sparingly, if at all. Lights displaying a fixed characteristic, especially white light signals, can be difficult to identify against even minimal background lighting. Furthermore, lights displaying a characteristic with a three second flash duration provide approximately 96% of the intensity of a fixed light signal, yield longer lamp service intervals, have lower power consumption, and provide greater conspicuity than the fixed light signal.

Russians also say that

If the lights are not synchronized the length of flashes and periods of the front lights should be shorter than these of the rear light.

Duration of the flashes should not be less than 0.5 s.

Length of the flashes of the rear light should be selected based on maximum overlapping of the duration of the flashes of the leading lights. The narrower the channel the more the flashes must be seen simultaneously.

## Tower Placement

Placement of the leading line structures determines the axis of the leading line. Tower placement along the centreline should be within ±3 meters of the desired position, while the lateral error in placement of the towers to either side of the true centreline should be limited to approximately ±0.3 meters. See Figure 20.

Tower Placement

## Lantern Placement

Proper beacon placement is necessary to ensure that blur will not occur during nighttime and that each light or group of lights will be viewed as a single source. All heights are the vertical distances from mean high water to the centreline of the optics. When using day & night leading lights, the lower optic on the front tower and the upper optic on the rear tower should be the nighttime lights. The only exception is if the front tower uses an omnidirectional nighttime light, which should then be mounted above the daytime light(s). Multiple optics for daytime lights should be installed on a horizontal plane, and should typically not exceed three across. Horizontal separation should be kept to a minimum to be sure the lights are viewed as a point source.

## Servicing Considerations

Towers should be designed to ensure that they can be serviced safely. Since many optics are serviced from the front, there should be 0.75 m of deck space available all around the optic, to allow for easy and safe access. Additionally, lanterns should be elevated a minimum of 0.5 m off the deck. When lanterns are installed more than approximately 1.25 m above the deck, a work platform should be built into the structure to allow personnel to comfortably access the lantern. Optic support structures should take into consideration any doors or servicing hatches on the installed lanterns. Railings should be installed, where appropriate, with either careful placement to prevent obstruction of the light or removable safety chains in front of optics. Operation and maintenance guides, if prepared, should be passed to the assigned servicing unit.

## Construction Details

Boat landings should be oriented to allow easy boarding under prevailing current and wind conditions. Boarding ladders should have rail extensions to allow easy transition from the deck to the ladder. For towers in excess of 20 m height, the designer should consider installation of a stairway instead of a ladder. All-weather, hand operated winches, with covers, should be installed on the main deck containing power system equipment and on the lantern deck to facilitate easy handling of hardware. Solar panels should be installed so that access to both sides of the array are possible and shall not be shadowed by railings, antennas, towers, shelters, etc., within an arc of ± 90 degrees of panel orientation.

## Safety

Installations using large batteries should have safety covers on intercell connectors to protect from accidental shorting. Battery rooms should have servicing equipment (hydrometer, tarp for covering solar panel, etc.) and safety equipment (eye wash station, gloves, goggles, etc.) available to servicing personnel in the event they do not bring the equipment with them.

## Daymark Mounting

Daymark mountings on structures should be strong enough to secure the daymark up to the tower’s designed wind load, while allowing servicing personnel easy replacement. Daymark mountings should not exceed the tower strength (daymark mountings should fail before the structure does). Access to the daymark by ladder or platform is necessary to remove/replace fasteners. Use of day/night lights is encouraged on ranges requiring marks in excess of 3 m in length, as these marks are the most hazardous to replace.

## Sensitivity of a Leading Line

The IALA Recommendation of 1998 classified the sensitivity (precision) of a leading line into three categories. However, these categories have never been used strictly. The IALA guideline and the spreadsheet use the "Cross Track Factor" (CTF) instead. The CTF is calculated with the formulae of the second category.

Categories:

- The standard deviation of the bearing difference of the two lights, when the observer has the impression that the two lights are vertically in line, should be calculated using the formulae:

ΘQ=maxΘ1,Θ2	(53)
where
	Θ1=0.05∙10-3rad+0.038∙γ  for γ≤5∙10-3 rad;
	Θ1=0.1∙10-3rad+0.028∙γ  for 5∙10-3<γ≤20∙10-3 rad; and 	
	Θ2=0.07∙γmin.

- The magnitude of the minimum value of the bearing difference of the two lights, when an off-axis deviation in a definite direction is considered as detected with certainty by the observer, should calculated using the formulae:

ΘD=maxΘ'1,Θ'2	(54)
where
	Θ'1=0.16∙10-3rad+0.12∙γ  for γ≤5∙10-3 rad;		Θ'1=0.31∙10-3rad+0.09∙γ  for 5∙10-3<γ≤20∙10-3 rad; and 		Θ'2=0.224∙γmin.

- In the interest of safety, the axis of the leading lights be established in such a manner that for the type of ships and navigation conditions considered, the ship may safely depart from the axis up to a point where the bearing difference as observed by the navigator reaches the following value in radians:

ΘM=maxΘ''1,Θ''2	(55)
where
	Θ''1=0.25∙10-3rad+0.19∙γ  for γ≤5∙10-3 rad;
	Θ''1=0.5∙10-3rad+0.14∙γ  for 5∙10-3<γ≤20∙10-3 rad; and 		Θ''2=0.35∙γmin.

See chapter 6.5 and 6.6.

# references

IALA Recommendation on Leading Lights E-112, May 1998)

IALA Guideline 1023 The Design of Leading Lines, December 2001

IALA Guideline G1148 - Determination of Required Luminous Intensity for Marine Signal Lights

Système International d'Unités (International System of Units) published by Bureau International des Poids et Mesures (International organization established by the Metre Convention), www.bipm.org

Regulations of the IHO for International (INT) Charts and Chart Specifications of the IHO, Edition 4.9.0, 2021

United States of America, US Coast Guard, Instruction M16500.4, Range Design, 1980

Report of the Proceedings of the International Technical Conference on Lighthouses and other Aids to Navigation , Paris 1993, published by Trinity House 1936

IALA Guideline 1094 Daymarks for Aids to Navigation, Edition 2.0, June 2016

United States Coast Guard Aids to Navigation Manual - Technical, U.S. Department of Homeland Security, Apr 06 2010

# Abbreviations and Technical Quantities

## Roman

| Abbreviation | Explanation | Chapter |
| --- | --- | --- |
| a | Function for daymark size calculation | 6.10.1 |
| b | Function for daymark size calculation | 6.10.1 |
| C | Length of useful segment | 5.1 |
| CTF | Cross-track factor | 6.7 |
| Dindex | usually a range when used with an index |  |
| DDM | Range of a daymark | 5.4 |
| DDM,FL | Range of front light daymark | 7.4.7 |
| DDM,RL | Range of rear light daymark | 7.4.8 |
| DTL | indicates the use of daytime lights (yes/no) | 5.4 |
| DM | indicates the use of daymarks (yes/no) | 5.4 |
| dU | unit distance (dU=1852m=1M) | 5 |
| E | illuminance | 5.4 |
| EFL | Illuminance at the eye of the observer by the front light | 5.3 |
| EFL,dsgx | Illuminance from front light, design visibility | 7.3.7 |
| EFL,maxx | Illuminance from front light, maximum visibility | 7.3.7 |
| EFL,minx | Illuminance from front light, minimum visibility | 7.3.7 |
| Emax | Maximum illuminance at the eye of the observer | 5.3 |
| Er | Required illuminance at the eye of the observer | 5.3 |
| ERL,dsgx | Illuminance from rear light, design visibility | 7.3.7 |
| ERL,maxx | Illuminance from rear light, maximum visibility | 7.3.7 |
| ERL,minx | Illuminance from rear light, minimum visibility | 7.3.7 |
| ERL | Illuminance at the eye of the observer by the rear light | 5.3 |
| EU | Unit illuminance (EU=1 lx) | 5 |
| E+ | maximum illuminance at the eye either from FL or RL | 6.5 |
| IFL | Luminous intensity, front light | 5.3 |
| IRL | Luminous intensity, rear light | 5.3 |
| H | Height | 5.4 |
| HFL | Height of front light above MHW or MSL | 5.2 |
| HFL,min1 | Minimum height of/or comma front light (height above safe water) | 6.9.1.1 |
| HFL,min2 | Minimum height front light daymark (height above safe water) | 6.9.1.1 |
| HFL,min3 | Minimum height front light (geographical range) | 6.9.1.2 |
| HFL,min4 | Minimum height front light daymark (geographical range) | 6.9.1.2 |
| HFL,min5 | Minimum height front light (obstruction, far end) | 6.9.1.3 |
| HFL,min6 | Minimum height front light (obstruction, near end) | 6.9.1.3 |
| HFL,min7 | Minimum height front light daymark (obstruction, far end) | 6.9.1.3 |
| HFL,min8 | Minimum height front light daymark (obstruction, near end) | 6.9.1.3 |
| HFL,rec | Recommended height of the front light | 6.9.1.4 |
| HFL,sel | Selected height of the front light | 6.9.1.4 |
| Hobs | Height of an obstruction above MHW or MSL | 6.9.1.3 |
| Hv | Height of observer (on vessel) above sea level | 5.2 |
| HRL | Height of rear light above MHW or MSL | 5.2 |
| HRL,min1 | Minimum height rear light (blur, far end) | 6.9.2.1 |
| HRL,min2 | Minimum height rear light (blur, near end) | 6.9.2.1 |
| HRL,min3 | Minimum height rear light daymark (front light not obscuring, far end) | 6.9.2.3 |
| HRL,min4 | Minimum height rear light daymark (front light not obscuring, near end) | 6.9.2.3 |
| HRL,min5 | Minimum height rear light (obstruction, far end) | 6.9.2.4 |
| HRL,min6 | Minimum height rear light (obstruction, near end) | 6.9.2.4 |
| HRL,min7 | Minimum height rear light daymark (obstruction, far end) | 6.9.2.5 |
| HRL,min8 | Minimum height rear light daymark (obstruction, near end) | 6.9.2.5 |
| HRL,rec | Recommended height of the rear light | 6.9.2.6 |
| HRL,sel | Selected height of the rear light | 6.9.2.6 |
| Hshaw | Safe height above water | 6.9.1.1 |
| hU | Unit height hU=1 m | 5 |
| I | luminous intensity | 5.4 |
| IFL,max | Maximum luminous intensity, front light | 6.2.4 |
| IFL,min | Minimum luminous intensity, front light | 6.2.1 |
| IFL,rec | Recommended luminous intensity, front light | 6.2.2 |
| IFL,sel | Selected luminous intensity, front light | 6.5 |
| IFL,max | Maximum luminous intensity, rear light | 6.2.4 |
| IFL,min | Minimum luminous intensity, rear light | 6.2.1 |
| IFL,rec | Recommended luminous intensity, rear light | 6.2.2 |
| IFL,sel | Selected luminous intensity, rear light | 6.5 |
| L | (vertical) length, usually for the daymark | 5.4 |
| L | Distance to front structure from far end to useful segment | 5.1 |
| LDM,rec | Recommended vertical length of a daymark | 6.10.1 |
| LFL,DM | Vertical length of the front light daymark | 5.2 |
| LFL,DM,rec | Recommended vertical length of the front light daymark | 7.3.1 |
| LFL,DM,sel | Selected vertical length of the front light daymark | 6.9.1.1 |
| LRL,DM | Vertical length of the rear light daymark | 5.2 |
| LRL,DM,sel | Selected vertical length of the rear light daymark | 6.9.1.1 |
| LRL,DM,rec | Recommended vertical length of the rear light daymark | 7.3.1 |
| M | Distance to front structure from near end to useful segment | 5.1 |
| Minitial | Initial value of M for iteration process | 6.1 |
| MHW | Mean high water | 5.2 |
| MLW | Mean low water | 5.2 |
| MSL | Mean sea level | 5.2 |
| MTR | Mean tidal range | 5.2 |
| R | Distance between leading line structures | 5.1 |
| Rinitial | Initial value of R for iteration process | 6.1 |
| Rg | geographical range | 5.4 |
| r | ratio of intensities | 5.4 |
| r1 | ratio of intensities, first value | 6.2.2 |
| r2 | ratio of intensities, second value | 6.2.2 |
| S | distance between an obstruction and front light | 5.4 |
| V | visibility | 5.4 |
| Vdsg | Design Meteorological visibility | 5.3 |
| Vmax | Maximum Meteorological visibility | 5.3 |
| Vmin | Minimum Meteorological visibility | 5.3 |
| W | Channel width | 5.1 |
| Windex | (horizontal) extension or width when used with index |  |
| WDM,rec | Recommend width of a daymark | 6.10.1 |
| WFL,DM,rec | Recommend width of front light daymark | 7.3.1 |
| WRL,DM,rec | Recommend width of rear light daymark | 7.3.1 |
| x | Distance of observer (vessel) from front tower | 5.1 |
| XFL,mid | Distance front light to middle of channel | 6.2.2 |
| XFL,far | Distance front light to far end of channel | 6.2.2 |
| XRL,mid | Distance rear light to middle of channel | 6.2.2 |
| XRL,far | Distance rear light to near end of channel | 6.2.2 |
| y | Distance of observer from centerline of channel | 5.1 |
| YD | Sensitivity type D | 6.7 |
| Z | dip of horizon | 5.4 |
| ZFL | Dip of horizon at front light | 5.2 |
| ZRL | Dip of horizon at rear light | 5.2 |

## Greek

| Abbreviation | Explanation | Chapter |
| --- | --- | --- |
| α | factor for dip calculation (α=6.75*10-8m-1) | 5.4 |
| ϕ | angle used to describe horizontal divergence | 5.4 |
| θ | Bearing difference | 5.1 |
| ΘD | Sensitivity type D (category 2) | 6.6 |
| ΘM | Sensitivity type M (category 3) | 8.10 |
| ΘQ | Sensitivity type Q (category 1) | 8.10 |
| θFL | Bearing front light | 6.4 |
| θRL | Bearing rear light | 6.4 |
| Θ1 | Partial result sensitivity type Q (category 1) | 8.10 |
| Θ2 | Partial result sensitivity type Q (category 1) | 8.10 |
| Θ'1 | Partial result sensitivity type D (category 2) | 6.6 |
| Θ'2 | Partial result sensitivity type D (category 2) | 6.6 |
| Θ''1 | Partial result sensitivity type M (category 3) | 8.10 |
| Θ''2 | Partial result sensitivity type M (category 3) | 8.10 |
| γ | Elevation difference | 5.2 |
| γFL | Elevation front light | 6.3 |
| γmin | Minimum elevation difference | 6.5 |
| γRL | Elevation rear light | 6.3 |

SIMPLIFIED INTENSITY CALCULATION

Introduction

The minimum required luminous intensity Imin of a marine signal light was traditionally calculated with the Allard's law (see IALA Guideline G1148 [1]).

Imin=Emin∙dmax2∙0.05-dmaxVmin	(A1)

where:

Imin	is the minimum required intensity (cd);

Emin	is the minimum illuminance at the eye of the observer (lx);

dmax	is the distance of farthest point of the useable range to the AtoN light (m); and

Vmin	is the minimum meteorological visibility in the area of interest (m).

The minimum illuminance at the eye of the observer was standardized to Emin=2∙10-7lx in 1933 at an international conference [7]. This illuminance value is linked to the physiological threshold of the eye of the observer and leads to very little intensities for short ranges. For example, a light with a useable range of 2 M and a minimum visibility of Vmin=10 M would get an intensity of 5 cd only.

required intensity of Leading Lights

It was commonly accepted that lights of leading lines should be much more intense than the all other lights and therefore the minimum required luminous intensity for leading lights was increased to Eminleading line=10-6lx. This value was introduced in the 1998 Recommendation [1].

With this convention the required minimum intensity for leading lights (black line at Figure 21) becomes five times higher than the other lights (blue line at Figure 21).

Required intensities when calculated with different input for V and E

In addition, to provide for worse weather, it is always possible / it may be prudent? to choose a minimum visibility lower than 10 M, which will increase the required intensity as well. This is done in Figure 21 with a visibility of Vmin=4 M. (red line at Figure 21).

When the IALA Guideline was developed in 2001, it was recognized that all these measures in calculations/methodology? were not enough to represent the existing lights, which have been proven to provide suitable intensities for decades. The calculated minimum required intensities in the ranges about 4 000 m and 7 000 m were still below 1 000 cd, but existing lights for these ranges were found to be about 10 000 cd or even 100 000 cd.

To solve this problem a new value called 'recommended intensity' was introduced in the IALA Guideline 2001. This recommended intensity is ten times the minimum value (Figure 22).

Recommended Intensity for a meteorological visibility Vmin=10 M (red dotted line)

IALA Guideline G1148

An alternative procedure to get suitable values for the required or recommended luminous intensity is available in IALA Guideline G1148 [3], which presents measures when background illumination and rivals lights have to be considered. The guideline includes the minimum required illuminance value 10-5lx for minor and 10-4lx for considerable background lighting. Both were already introduced in the 2001 IALA Guideline on leading lines. Guideline G1148 also presents the option to calculate the factor for / the effect of background illumination based on measurement of it.

The application of Guideline G1148 requires more work on identifying the background illumination and the rival lights. This is necessary, when problems concerning the visibility of marine signal lights have been reported and there is a need of action.

Simplified Method

The German Administration investigated its exiting leading lights and found out that all methods, which still have the Allard's Law as a basis, do not really represent the actual situation that has caused no problems/complaints and seems to work fine (Figure 23).

Luminous intensity of German leading lights

For short range lights (Xfar) the actual intensity is much higher than the ones calculated according to '2001 IALA Guideline', even when the minimum visibility is about 4 M (① in Figure 23). In these cases the lights are very often nearby a  harbour or a city with a lot of background illumination and rival lights.

For long range lights the existing intensities are below the values calculated according to '2001 IALA Guideline' (②in Figure 23). Background illumination and rival lights do not play an import role in these cases, but instead glare at near end of useful segment and high costs are relevant.

Considering the existing lights, a simplified empirical equation for determining a recommended luminous intensity for leading lights was be derived.

Irec,sim=Irec,M∙XFardU3	(A2)

where:

Irec,M=1000 cd	(recommended luminous intensity for XFar=1852 m=1 M); and

dU=1852 m=1 M 	(unit distance).

There is some arbitrariness in this this equation and the coefficients were chosen to keep the equation simple, but it corresponds very well to existing German leading lights.

The calculation is done for the front light and the rear light intensity is harmonized with it using the 'illumination ratio calculation' of the '2001 IALA Guideline'.

The equation does not require to fix a meteorological visibility or to estimate background illumination or rival lights. However, the resulting intensities have been proven acceptable for decades and they include background illumination and rival lights indirectly.

When the simplified intensity calculation does not give suitable intensities and problems have been reported, it is advised to use the methods of IALA Guideline G1148 [3].

Recommended luminous intensity (simple method)