Input paper:   ENG19-3.1.2.5

Input paper for the following Committee(s): 	  				Purpose of paper:

□  ARM	X  ENG		□  PAP					□  Input

□  DTEC	□  VTS							X  Information

Agenda item 			n.n

Technical Domain / Task Number 2	Radionavigation services

Author(s) / Submitter(s)		Younghoon Han, Tae Hyun Fang, Kiyeol Seo (KRISO), Woo Gyoung Park, Seungcheol Lee (MOF)

# Development Result of the Korean R-Mode Test Bed (TRACE)

# Summary

## Purpose of the document

This input contains the research and development results of the R-Mode Test bed Project (TRACE) conducted in Republic of Korea. It is expected that the results of this research will be used as a reference when developing a new guideline for R-Mode implementation in the future.

## Related documents

- ENG12-3.1.34 Introduction to Korean R-Mode testbed project (TRACE)

- ENG14-3.1.3.6 Status update of the Korean R-Mode testbed project (TRACE)

- ENG16-3.1.3.10 Status update on Korean R-Mode test bed project (TRACE)

# Background

The Republic of Korea (R.O.K.) is making efforts to set up a backup navigation system to provide resilient Position, Navigation and Timing (PNT) information to maritime users in case of GNSS emergency and to prepare for GNSS vulnerability. The Ministry of Oceans and Fisheries planned to provide the R-Mode service along with the eLoran service to expand the backup PNT system to all maritime areas in Korea and provide the harbour entrance and port approach requirements for maritime users as required by the IMO. KRISO, the lead organization for the Terrestrial Ranging-Augmented, Complementary and Enhanced (TRACE) project, launched the Korean R-Mode testbed project in April 2020 to develop core technologies and demonstrated performance at a test port by September 2023.

# Discussion

## Korean R-Mode test bed configuration

The TRACE project plans to utilize the medium-frequency (MF) R-Mode, VDES R-Mode signal, and eLoran signal currently in service to achieve the project goals. Therefore, the testbed system will consist of a transmitting station for MF R-Mode signals, the VDES R-Mode signals, and eLoran signals and an integrated navigation receiver that receives and integrates each signal to determine its position. The integrated navigation receiver receives GNSS signals and terrestrial navigation signals. Furthermore, the system consists of a monitoring station for integrity information generation, a differential R-Mode reference station system for generating correction data to improve the positioning accuracy for MF R-Mode users, and an operating system to monitor and control the status of each system.

Korean R-Mode system

Figure 2 shows the testbed configuration. The Daesan Port located in the north-central region of the West Sea, Korea was selected as the testbed area for the actual performance, where eLoran signal reception is possible along with the signals of the MF R-Mode transmitting station. Therefore, the Socheong, Palmi, and Eocheong Island DGNSS marine reference stations and the Chungju DGNSS inland reference station was upgraded to MF R-Mode transmitting stations. The VDES R-Mode transmitting station was installed at Humang AIS base stations, Nanjido, and Budo where it can complement the HDOP. The differential R-Mode reference station and the monitoring station was installed near the Daesan Port as shown in Figure 3.

Figure 7: Korean R-Mode testbed configuration (Left: MF R-Mode, Right: VDES R-Mode)

The differential R-Mode reference station and the monitoring station

The high level system architecture of Korean R-Mode test bed is shown in Figure 4. Differential R-Mode system (DRS) receives MF R-Mode signals and generates corrections for low and high CWs. R-Mode monitoring system (RMS) receives MF R-Mode corrections and feedback to DRS monitoring results. RMS monitors MF and VDES R-Mode signals and generates integrity information. VDES R-Mode Transmitting system (VRT) receives integrity information from RMS and sends it to users. MF R-Mode Transmitting system (MRT) receives MF integrity information, corrections and UDREs from RMS and generates R-Mode RTCM messages to send it to users. R-Mode Operation system (RMOS) monitors and controls each sub system.

Korean R-Mode system architecture

## MF R-Mode transmitting system development

The MF R-Mode transmitting system was upgraded from the DGNSS system operated by the National maritime PNT office in Korea. Since all MF R-Mode transmitting systems are operated in synchronization with the R-Mode Reference Time (RMST), a reference time generation device is required for this. RMST is synchronized with the GPS Time (GPST) and maintains synchronization performance for 15 days based on learning data in case of GPS emergency. The R-Mode Exciter generates an R-Mode signal with CW added to the existing MSK signal in accordance with the RMST and transmits it to the transmitter (amplifier). The R-Mode signal is amplified through the transmitter and transmitted through the Antenna Tuning Unit (ATU). The output R-Mode signal is coupled to the ATU and transmitted to the R-Mode Exciter again through the connected feedback cable. The exciter checks the timing of the signal that has been fed back and controls it so that the signal can be transmitted at the exact timing.

The architecture of a Korean MF R-Mode transmitting system is shown in Figure 5. The red boxes indicate the parts that have been changed or added to the existing DGNSS system. The R-Mode RTCM generator for correction and integrity information transmission for MF R-Mode and the R-Mode time synchronization system have been added. The MSK modulator has been replaced by the Exciter for R-Mode signal generation and the high-power amplifier replaced for R-Mode as well. Additionally, a monitoring and control system for R-Mode transmitting system was added.

Korean MF R-Mode transmitting stations broadcast MF R-Mode signal with R-Mode RTCM message. R-Mode monitoring station sends R-Mode RTCM message parameters to R-Mode RTCM generator every 5 seconds not to exceed 200 bps. R-Mode RTCM generator receives DGNSS and MF R-Mode RTCM messages and send them to transmitter according to the schedule.

MF R-Mode transmitting system architecture

## Differential MF R-Mode system and R-Mode monitoring system development

The configuration of the differential MF R-Mode system (DRS) and R-Mode monitoring system (RMS) is as shown in Figure 6. The DRS and RMS are largely divided into an antenna part, a signal reception part, a signal processing part, a power supply part, and a network part, and although they have the same equipment configuration, each is responsible for a function appropriate to its role in the signal processing part. The antenna part consists of a MF antenna and a GNSS antenna, and the GNSS antenna is used for initial position determination considering that it switches to R-Mode navigation in the event of a GNSS emergency. The signal reception part generates measurements from GNSS signals and MF R-Mode signals. Each signal processing unit consists of a server where the core algorithms and operating SW of the DRS and MRS operate. Figure 7 is the DRS operation monitor, which allows you to check the signal strength, signal-to-noise ratio, and residual values ​​received from each transmitting station, and check the correction information and position error. Figure 8 is the MRS operation monitor, where you can check the quality check results, integrity results, position error, UDRE, etc. when the correction information generated by DRS is applied to the MRS measurements.

Fig. 5. Differential MF R-mode system and monitoring system configuration.

Differential MF R-mode system GUI.

R-mode monitoring system GUI.

## MF R-Mode receiver development

The MF R-Mode receiver was developed by DARBS, and the receiver configuration is shown in Figure 9. There is a GPS receiving module for initial operation, and it consists of a MF band filter and an Analog to Digital Converter (ADC), a module for receiving and processing a total of four signals, a module for logging data and analysing the spectrum, and an oscillator. The receiver meets the ITU-R M.823-3 [1] and RTCM SC-10402.3 [2] standards, and is capable of real-time MF R-Mode signal processing, 4-channel MSK demodulation, 4-channel CW range estimation, and signal quality measurement.

MF R-mode receiver configuration.

## VDES R-Mode development

The main goal of the VDES R-Mode Testbed is to use VDES standard messages for ranging measurements and to evaluate the performance. The concept of VDES R-Mode Ranging is to transmit a ranging sequence in a VDES message and estimate the distance between a transmitting station and a receiving station using a time of arrival (TOA) measurement technique.

VDES R-Mode Ranging Sequence and Navigation data uses VDE-TER standard messages. It was implemented according to the ITU-R M2092-1 [3] and IALA Guideline 1158 [4]. Link ID 17 is applied to navigation data and transmitted every 1 minute. Link ID 37 is applied to ranging sequence and transmitted every 800 ms. The ranging sequence should be appropriately combine two known sequences to satisfy the required performance for a given scenario. For the VDES R-Mode in the TRACE project, only gold sequence was used to generate the ganging sequence (γ=1).

VDES R-MODE standard message format

VDES R-Mode physical layer was implemented in an FPGA-based self-made HW. The VDES R-Mode transmitting system (VRT) and VDES R-Mode receiver (VRR) consisted of a suitable band pass filter, a power amplifier, a low noise amplifier (LNA), and an antenna. Figure 11 and Table 1 show specifications of the manufactured VRT and VRR. VRT was transmitted R-Mode signal using lower frequency to avoid the interference issues.

VDES R-MODE transmitter (Left) and receiver (Right)

Specifications of VRT and VRR

| Parameter | Specification |  | Parameter | Value |
| --- | --- | --- | --- | --- |
| Frequency (MHz) | 157.2375 161.8375 |  | Output power | 12.5 W |
| Band Width (KHz) | 100 |  | Working Voltage | AC 220 V |
| Modulation | π/4-QPSK |  | Size | 270x223.2x270 W x H x D(mm) |
| Symbol Rate | 76.8 Ksps @100 kHz |  |  |  |
| Multiple Access | TDMA |  | Weight | 9.1 kg |

## Integrated navigation receiver development

The integrated navigation receiver developed in the TRACE project was studied for the purpose of feasibility test with a configuration that combines three individual navigation receivers, as shown in Figure 12. The integrated navigation receiver receives measurements of each signal from the previously described MF R-Mode receiver and VDES R-Mode receiver, and estimates the integrated position result in the integrated navigation module capable of processing GNSS and eLoran signals. At this time, in order to reduce the clock error between individual receivers, a common oscillator was used to input to each receiver. In order to calculate the position result in the integrated navigation receiver, three or more signals of one frequency band must be capable of being received.

Integrated navigation receiver (Left) and GUI (Right)

## Test results

### VDES R-Mode test results

In VDES R-Mode, it is very important to install the transmitting antenna and receiving antenna in a high place so that there is no obstacle in the propagation path and to secure line of sight. In the TRACE project, we tried to install the antenna in a high place as much as possible, but line of sight could not be secured smoothly due to environmental constraints. Therefore, proper results could not be obtained in the test bed area due to large ships, islands, mountains, etc. located in the propagation path. Therefore, the installed transmitting system was removed and the test was performed in a short-baseline environment where line of sight was secured, and the performance of about 10 m was confirmed. The test environment and results at that scenario are as shown in Figure 13.

Short baseline VDES R-MODE test environment and result

### MF R-Mode test results

The performance of the MF R-Mode system was evaluated in terms of time synchronization performance between transmitting stations, static positioning performance, and dynamic positioning performance. First, the time synchronization performance between transmitting systems was evaluated by comparing the RMST of each transmitter with the GPST based on the Palmi station, as shown in Figure 14. When analysing data for approximately 26 days in July 2023, a synchronization error of approximately 5 ns (RMS) was observed.

The static positioning performance was evaluated using the R-Mode monitoring station receiver data located at Daesan port. At this time, the base line between the differential R-Mode station and the monitoring station antenna is very close. The true position was obtained in advance using a high precise GNSS receiver. The left side of Figure 15 shows the latitude and longitude positions estimated by the receiver for 24 hours on July 23, 2023. The inner red circle indicates a 10 m error radius compared to the true value, and the outer circle indicates a 50 m error radius. The right figure shows the horizontal position error over time, and the error mainly increases at night between 7:00 PM and 6:00 AM. This is an error that increases when terrestrial signals are reflected to the ionosphere during the night when there is no solar activity due to the sky-wave effect caused by ionospheric reflection, generating multipath signals. The sky-wave effect can cause errors of more than 100 meters, and in this test, the effect was reduced by applying correction data at a short base line. Figure 16 shows the distribution and cumulative distribution of the horizontal position error during the 24 hours. Through cumulative distribution, it was confirmed that the daily horizontal position error in a static environment was approximately 8.85 m (95%).

Time synchronization errors of MF R-mode transmitting stations (Reference: Palmi).

Static: MF R-mode horizontal positioning (left) and positioning error over time (right)

Static: MF R-mode horizontal error distribution

The dynamic positioning performance was confirmed through the results of a sea test using a vessel in Daesan port. The data of the vessel moving for one hour from 9:40 to 10:40 am on July 12, 2023 were used for the analysis. The true value of the dynamic position was used as the TerraStar precise positioning service of the Novatel receiver, and the position results output from the Ublox GNSS receiver and the position results output from the MF R-Mode receiver were checked together. Figure 17 shows the vessel used in the test and the integrated navigation receiver installed on the vessel, and Figure 18 shows the positioning result and horizontal positioning error according to the movement of the vessel. The MF R-Mode positioning result generally showed performance within 20 m (95%) compared to the precision position, and showed performance within 10 m (95%) and performance at the level of 40 m (95%) depending on the section. Notably, the Ublox receiver also showed performance similar to the MF R-Mode, and it was confirmed that the performance of the MF R-Mode was relatively better in some sections.

Test vessel and Integrated navigation receiver

Vessel test results: vessel positioning (Left) and positioning error (Right)

# References

ITU, Recommendation M.823-3, Technical characteristics of differential transmissions for global navigation satellite systems from maritime radio beacons in the frequency band 283.5-315 kHz in Region 1 and 285-325 kHz in Regions 2 and 3, 2006.

RTCM, RTCM SC-10402.3. Recommended Standards for Differential GNSS (Global Navigation Satellite Systems) Service, Version 2.3, 2010.

ITU, Recommendation ITU-R M.2092-1, Technical characteristics for a VHF data exchange system in the VHF maritime mobile band, 2022.

IALA, Guideline G1158-VDES R-Mode, Edition 1, 2020.

# Action requested of the Committee

The Committee is requested to consider the development result of the Korean R-Mode test bed, and take action as appropriate.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain