Input paper:   ENG19-n.n.n

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					□  Input

□  DTEC	□  VTS							X  Information

Agenda item 			n.n

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			Emma RIEU-STÉPHAN – Cerema – France

Replacement of mercury rotary systems of French lighthouses

# Summary

The purpose of this document is to inform ENG committee members on France's initiative to eradicate mercury from lighthouses by 2030, and to introduce the oil bath solution developed by Cerema in this context.

# Background

Developed in 1825 by Augustin Fresnel and perfected in 1889 by Arthur Meurs with the collaboration of the F. Barbier company, the mercury bath has been used on many optic rotation mechanisms.

Mercury is a metal present in liquid form in nature. It is classified as carcinogenic, mutagenic and reprotoxic, and it is considered a public health concern [1]. As a result, various countries are looking for ways to replace these systems. At the present time, there are two different approaches:

- Replacing the original Fresnel Lens with a LED lantern, assuming that the nominal range meets the user’s requirement

- Reengineering the rotating mercury system

# Strategy to eradicate mercury in lighthouses

Historically, mercury has played an important role in the rotating systems of French lighthouses. Some 70 lighthouses are still equipped with mercury, in various sizes of baths and focal length.

Figure 1 : Quantity of lighthouses with mercury according to the focal length

The aim of the new French strategy defined by the national member, DGAMPA, is to eradicate mercury in lighthouses by 2030. An ambitious goal that requires a combination of both approaches.

In France, research has been realised for several years to propose solutions for eliminating mercury by reengineering the rotating system. A new system, based on ball bearings immersed in an oil bath, has been designed. It can now be installed to replace mercury bath with a radius of less than 560 mm.

Work is currently underway to design a similar system for larger diameters, notably 680 mm mercury bath.

With these different sizes of system, around 30 of the remaining 70 lighthouses could potentially be treated.

# Description of the new system

## Generality

French consideration has focused on a solution that preserves the historic lighting system: Fresnel lens and light source.

The solutions studied had to meet the following conditions:

- Preservation of the visual aspect

- Ensuring quality service for users (Fresnel optics are highly efficient, they guarantee high nautical range with low power consumption light source)

- Reliability and long operating life

- Easy and economic maintenance (standard tooling) with long intervals.

- Low friction and low starting torque

- No increase in rotation motor power consumption

A first prototype was designed in 2018 and installed in 2019 in La Réunion. Since its installation, it has given complete satisfaction. Further developments on subsequent prototypes have resulted in a mature solution that can meet a wide range of requirements.

The principle is based on an oversized ball bearing immersed in an oil bath. A cross-section of the prototype and a general view are shown below in Figure 2.

For the moment, two different models have been designed to accommodate different lens sizes:

- 380 mm for optics weighing less than 200 kg depending on their configuration,

- 560 mm for optics weighing less than 350 kg depending on their configuration.

Figure 2 : General view and Cross-section of the prototype

## Replacement process

To date, this new generation of rotating system has been installed on seven lighthouses. The stages in the process are as follows (Figure 3). A complete description of each stage is provided in Appendix 1.

Figure 3 : Replacement process of mercury bath

## Results and prospects

Erreur ! Source du renvoi introuvable. shows the list of lighthouses whose systems have already been replaced along with their optic type.

Table 1 : Lists of lighthouses with new systems

| Name of lighthouse | Date of system replacement | Type of lens / estimated weight |
| --- | --- | --- |
| Phare de Bel Air | October 2019 | Focal length = 0,25 / 110 kg |
| Phare de la Revellatta | May 2022 | Focal length = 0,3 / 140 kg |
| Phare de Contis | October 2022 | Focal length = 0,3 / 200 kg |
| La Hague | June 2023 | Focal length = 0,3 / 150 kg |
| Phare de Grouin du cou | March 2024 | Focal length = 0,25 / 300 kg |
| Phare de la Caravelle | May 2024 | Focal length = 0,25 / 140 kg |
| Phare des sept îles | June 2024 | Focal length = 0,3 / 250 kg |

Energy consumption to power the motors is insignificant compared to that of the light source. It remains within the same orders of values with the new system (between 6 and 12 Watts at 24 Vdc steady-state with Table 1 listing).

The next step is to design a prototype for large ring mercury bath. The reflexion is engaged to renovate the lighthouse of Mahina in Papeete. The lens and the mercury bath have the following characteristics:

- Focal length: 0.5 m

- Lens weight: 840 kg

- Exterior bath diameter: 1 m

The work is scheduled for 2025/2026.

# References

REGULATION (EU) 2017/852 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 17 May 2017 on mercury, and repealing Regulation (EC) No 1102/2008

Process

# Site preparation; engineering and design

- Visit to the lighthouse

- Taking the various measurements

- Installation and testing of the rotating system in the workshop

- Preparing the rotation device and electric motor (Brushless technology)

Figure 4 : Testing of the rotating system in the workshop

# On-site installation; removing the Fresnel lens

Figure 5 : Removing the Fresnel lens

# On-site installation; emptying the mercury, dismantling the old system and storing the mechanism

Figure 6 : Emptying the mercury

Figure 7 : Dismantling of the old system

# On-site installation; transport of the new mechanism from the walkway (helicopter, crane, etc.), installation of the system and oiling

Figure 8 : Installation of the new system

# On-site installation; reassembling the lens

Figure 9 : Reassembling the Fresnel lens

# On-site installation; wiring of controllers and motors

Figure 10 : Installing controllers and motors

# On-site installation; testing and validate

Figure 11 : Testing at “phare de Contis”

Installation examples

# Installation on « Phare de Bel Air »

| Lighthouse photo | Before | Present |
| --- | --- | --- |

# Installation on « Phare de La Revellatta »

| Lighthouse photo | Before | Present |
| --- | --- | --- |

# Installation on « Phare de Contis »

| Lighthouse photo | Before | Present |
| --- | --- | --- |

# Installation on « Phare de La Hague »

| Lighthouse photo | Before | Present |
| --- | --- | --- |

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain