ENG19-9.2.2.9

IALA S-240

Product Specification

Edition 1.2.0 – April 2024

IALA DGNSS Station Almanac Product Specification

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| 11/05/2020 | Various | Updating specification and various edits following review at IALA ENG10 |
| 27/03/2024 | Various | Updating specification and various edits following review from ENG members |

Table of Contents

Contents

1	Overview	6

1.1	Introduction	6

1.2	References	6

1.2.1	Normative References	6

1.2.2	Informative References	6

1.3	Terms, Definitions and abbreviations	7

1.3.1	Terms and definitions	7

1.3.2	Acronyms	7

1.4	Product Specification metadata	8

1.4.1	IALA Product Specification Maintenance	8

2	Specification Scope	9

3	Data Product Identification	10

4	Data Content and structure	11

4.1	Introduction	11

4.2	Application Schema	11

4.3	Feature Catalogue	13

4.3.1	Introduction	13

4.3.2	Feature Types	13

4.4	Units of Measure	15

4.5	Geometry	16

5	Co-ordinate reference systems (CRS)	16

5.1	Introduction	16

5.2	Horizontal Geodetic Datum	16

5.3	Vertical Datum	16

5.4	Units of Measure	16

6	Data Quality	17

6.1	Introduction	17

6.2	Quality measure elements	17

6.3	Test methods	20

6.4	Data quality testing and reporting	20

7	Data Capture and classification	21

7.1	Introduction	21

7.2	Data Encoding and Product Delivery	22

7.2.1	Data Encoding	22

7.2.2	Types of Datasets	22

7.2.3	Content of Update Datasets	22

7.3	Encoding of Latitude and Longitude	23

7.4	Numeric Attribute Encoding	23

7.5	Text Attribute Values	23

7.6	Mandatory Attribute Values	24

7.7	Unknown Attribute Values	24

7.8	Structure of dataset files	24

7.9	Object identifiers	24

7.10	Data coverage	25

7.11	Data overlap	25

7.12	Data extent	25

8	Data Maintenance	25

8.1	Introduction	25

8.2	Production process for base and update datasets	26

8.3	Dataset updates and cancellation	26

8.4	Support file updates	26

8.5	Feature and portrayal catalogues	26

8.6	Feature history, versions, and change tracking	27

8.7	Dataset encryption	27

9	Portrayal	27

10	Data Product format (Encoding)	27

11	Data Product delivery	28

11.1	Data Product Delivery Information	28

11.1.1	Dataset loading	28

11.1.2	New editions	29

11.2	Dataset size	29

11.3	Exchange Set	29

11.4	Support Files	30

11.5	Support File Naming Convention	30

11.6	Dataset Naming Convention	31

11.7	Update dataset naming convention	31

11.8	Catalogue File Naming Convention	32

12	Metadata	32

12.1	Introduction	32

12.2	Dataset Metadata	32

12.3	Update and Cancellation Dataset Metadata	35

12.4	Support file Metadata	36

12.5	Exchange Set Metadata	37

12.6	Catalogue File Metadata.	38

ANNEX A	Data classification and Encoding Guide	40

ANNEX B	Validation Check	45

ANNEX D   S-240 GML Schema	51

ANNEX D   S-240 Feature Catalogue	62

# Overview

## Introduction

The DGNSS Station Almanac is a document published and maintained by IALA, which contains information about the worldwide DGNSS stations. The DGNSS Station Almanac Product Specification provides a common structure for the exchange of information about DGNSS stations. The product contains the station name, identification numbers of reference stations and transmitting station, positions, nominal range, operation status, transmitted message types, frequency, bit rate, remarks related to a DGNSS station.

The Product Specification can be used to exchange DGNSS station information in a consistent form between IALA, Lighthouse Authorities, Hydrographic Offices and other organizations (to include commercial and professional agencies, mariners and manufacturers). This information can also be provided as a document converted from the dataset

## References

### Normative References

The following normative documents contain provisions that, through reference in this text, constitute provisions of this document.

- IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2018).

- ISO 3166-1. 1997. Country Codes. 1997.

- ISO/TS 19103:2005 Geographic Information - Conceptual schema language

- ISO 19109:2005 Geographic Information - Rules for Application Schema

- ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

- ISO 19131:2007 Geographic Information - Data product specifications

- ISO 19136:2007 Geographic Information – Geography Markup Language

- ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### Informative References

The following informative documents provide additional information, including background information, but are not required to develop applications for data conforming to this specification.

- ITU-R M.823-3 Technical characteristics of differential transmissions for global navigation satellite systems from maritime radio beacons in the frequency band 283.5-315 kHz in Region 1 and 285-325 kHz in Region 2 and 3

- RTCM 10402.3 RTCM Recommended Standards for Differential GNSS(Global Navigation Satellite Systems) Service version 2.3.

- IALA Recommendation R-121 On the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5-325 kHz (Ed. 1.1, December 2004)

- IALA 2015. DGNSS Station Almanac.

- IALA 2010. IALA Navguide.

- IALA 2012. International Dictionary of Marine Aids to Navigation.

- ISO/IEC 19757-3, Information technology – Document Schema Definition Languages (DSDL) – Part 3: Rule-based validation – Schematron.

## Terms, Definitions and abbreviations

### Terms and definitions

Terms and definitions have been taken from the normative references cited in clause 1.2.1.

### Acronyms

This product specification adopts the following convention for symbols and abbreviated terms:

| IALA | International Association of Marine Aids to Navigation and Lighthouse Authorities |
| --- | --- |
| CRS | Coordinate Reference System |
| ECDIS | Electronic Chart Display and Information System |
| ENC | Electronic Navigational Chart |
| IHO | International Hydrographic Organization |
| IMO | International Maritime Organization |
| DGNSS | Differential Global Navigation Satellite System |
| WGS84 | World Geodetic System 1984 |
| XML | Extensible Markup Language |
| XSL |  |
| XSD |  |
| UML | Unified Modelling Language |
| GML | Geography Markup Language |
| OCG | Open Geospatial Consortium |
| UTF-8 |  |
| EPSG |  |
| GM |  |
| PNG | Portable Network Graphics |
| SVG | Scalable Vector Graphics |
| TIFF | Tagged Image File Format |

## Product Specification metadata

| Title | DGNSS Station Almanac |
| --- | --- |
| Version | 1.0.0 |
| Identifier | S-240 |
| S-100 version | 4.0.0 |
| Date | 17 May 2020 |
| Language | English |
| Classification: | 001 - unclassified |
| Contact: | IALA-AISM 10, rue des Gaudines 78100 Saint Germain en Laye, France Telephone: +33 1 34 51 70 01 Fax: +33 1 34 51 82 05 |
| URL: | http://iala-aism.org |
| Maintenance: | The product specification is maintained by IALA-AISM and amendments are performed on a needs base, up to maximum one new release per calendar year. |

### IALA Product Specification Maintenance

This chapter is for clarification only on PS Maintenance.

#### Introduction

Changes to a product specification will be released by IALA-AISM as a new edition, revision, or clarification.

#### New Edition

New editions of a product specification introduce significant changes. New editions enable new concepts, such as the ability to support new functions or applications, or the introduction of new constructs or data types.

#### Revisions

Revisions are defined as substantive semantic changes to a product specification. Typically, revisions will change a product specification to correct factual errors; introduce necessary changes that have become evident as a result of practical experience or changing circumstances.  A revision must not be classified as a clarification.  Revisions could have an impact on either existing users or future users of a product specification.  All cumulative clarifications must be included with the release of approved corrections.

Changes in a revision are minor and ensure backward compatibility with the previous versions within the same edition.  Newer revisions, for example, introduce new features and attributes.  Within the same edition, a data product of one version could always be processed with a later version of the feature and portrayal catalogues.

#### Clarification

Clarifications are non-substantive changes to a product specification. Typically, clarifications: remove ambiguity; correct grammatical and spelling errors; amend or update cross references; insert improved graphics, spelling, punctuation and grammar.  A clarification must not cause any substantive semantic change to a product specification.

Changes in a clarification are minor and ensure backward compatibility with the previous versions within the same edition.  Within the same edition, a data product of one clarification version could always be processed with a later version of the feature and portrayal catalogues, and a portrayal catalogue can always rely on earlier versions of the feature catalogues.

#### Version Numbers

The associated version control numbering to identify changes (n) to a product specification must be as follows:

New editions denoted as n.0.0

Revisions denoted as n.n.0

Clarifications denoted as n.n.n

# Specification Scope

This product specification describes one data product and therefore requires only one scope which is described below:

| Scope ID: | DGNSS Station Almanac datasets. |
| --- | --- |
| Hierarchical level: | MD_ScopeCode - 005 |
| Hierarchical level name: | dataset. |
| Level description: | information applies to the dataset |
| Extent: | EX_Extent.description: Global coverage of maritime areas |

# Data Product Identification

This clause describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 13 of this specification. The information identifying the data product may include the following items from S-100 4.0.0 clause 11-6 (adapted from ISO 19115-1).

| Title | DGNSS Station Almanac |
| --- | --- |
| Alternate Title | IALA – DGNSS |
| Abstract | Name, Identification numbers, positions, operation status of DGNSS stations. Additional general information can also be included. |
| Topic Category | Environment, Oceans, Structure and Transportation |
| Geographic Description | Areas specific to marine and inland navigation. |
| Spatial Extent | Global |
| Description | DGSS Station information |
| Spatial Resolution | All scales |
| Purpose | The purpose of a DGNSS Station Almanac dataset is to exchange name, identification numbers and operation status of DGNSS station. Additional general information can also be included. |
| Language | English |
| Spatial Representation Type | Vector (Point) |
| Point of Contact | IALA |
| Use Limitation | None |

# Data Content and structure

## Introduction

A DGNSS Station Almanac product is based on the S-100 General Feature Model. A General Feature Model is a meta-model of feature types. A feature may have properties that may be operations, attributes or associations. Any feature may have a number of attributes.  A feature is not defined in terms of a single geometry, but rather as a conceptually meaningful object within a particular domain, one or more of whose properties may be geometric. The 'Feature' is the fundamental unit of geospatial information, so the Feature Model is the fundamental meta-model used for developing an Application Schema.

The following sub-sections contain the product application schema expressed in UML and an associated feature catalogue. The feature catalogue provides a full description of each feature type including its attributes and attribute values in the data product.

## Application Schema

The UML model shown in Figure 4.1 below illustrates an application schema of DGNSS Station Almanac. It includes a general description of elements used to construct the application schema, and the relationships between them. These elements include feature types, information types, simple attributes, complex attributes, aggregations and associations.

A brief description of these is provided below and the full description is included in the feature catalogue.

Figure 1. Application Schema of DGNSS Station Almanac

A feature is an abstraction of real world phenomena. A certain feature type is the class used to describe all instances of that kind of real world object. In object-oriented modelling, feature types are equivalent to classes and feature instances are equivalent to objects.

An information type is an identifiable object that can be associated with features in order to carry information pertaining to the associated features.

Simple attributes can be enumerations, code lists or simple types (e.g. integer or character string).

Complex attributes are properties of a feature which can be divided into multiple sub attributes and are used where objects have properties that better fit a hierarchical structure. They provide a better construct for encoding list attributes on objects such as light sectors.

An association is a relationship that links instances of one feature or information type with instances of the same or different feature and information types. Each relationship has a name and two roles thus giving a more detailed representation of the real world relationships within the dataset.

## Feature Catalogue

### Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product. The S-240 Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema. A Feature Catalogue in XML format shall be available from IALA with an accompanying XSL Stylesheet.

| Name: | DGNSS Station Almanac Feature Catalogue |
| --- | --- |
| Scope: | Ocean, Coastal, Ports, Harbors and Inland waters |
| Version Number | 1.2.0 |
| Field of application: |  |
| Version Date: | 2024-04-05 |
| Producer: | IALA 10, rue des Gaudines 78100 Saint Germain en Laye, France Telephone: +33 1 34 51 70 01, Fax: +33 1 34 51 82 05 URL : https://www.iala-aism.org |
| Language | English |

### Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

#### Geographic

Geographic (Geo) feature types carries the descriptive characteristics of a real-world entity.

#### Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

#### Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

#### Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

#### Attributes

S-240 defines attribute as either simple or complex. Attributes guideline is shown below:

Numeric Attribute Values

Floating point or integer attribute values must not be padded by non-significant zeroes.

Text Attribute Values

Character strings must be encoded using the character set specified in Unicode Transformation Format-8 (UTF-8).

Mandatory Attribute Values

Where attributes determine the display of a Feature, or are essential to the overall feature type construction they may be mandatory. All mandatory attributes are identified in the feature catalogue.

S-240 uses several types of simple attributes; they are listed in the following table:

Table 1. Simple feature attributes

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| CharacterString | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601. EXAMPLE 19980918 (YYYYMMDD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |
| Truncated date | One or more components of the Date type are omitted. |

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

Figure 2. textContent - a complex attribute

## Units of Measure

The following units of measure is used in DGNSS Station Almanac datasets;

- Orientation is given in decimal degrees

- Radio frequency is given in hertz

- Uncertainty is given in metres

- Distances are given in metres or nautical miles

## Geometry

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses three types of geometries: GM_Point, GM_OrientableCurve, and GM_OrientableSurface.

Figure 3. Geometric primitives

# Co-ordinate reference systems (CRS)

## Introduction

The single coordinate reference system is separated into the horizontal and vertical components.

## Horizontal Geodetic Datum

WGS84 (World Geodetic System of 1984) must be used for the horizontal reference system for spatial data. WGS84 must be used as the reference ellipsoid.

## Vertical Datum

Vertical datum shall be selected from the list in verticalDatum enumeration. If the local datum value is used, additional information must be provided using the comment attribute in the metadata, or information attribute on the feature instance. Positive value is above vertical datum and negative is below vertical datum.

## Units of Measure

Units of measure for S-240 datasets must be as follows,

For height, depth and vertical length it must be metres.

For range it must be nautical miles

For orientation it must be degrees

For signal duration it must be seconds

For audible signal output it must be decibel

For signal frequency it must be Hertz

# Data Quality

## Introduction

Data quality allows users and user systems to assess fitness for use of the provided data. Data quality measures and the associated evaluation are reported as metadata of a data product. This metadata improves interoperability with other data products and provides usage by user groups that the data product was not originally intended for. The secondary users can make assessments of the data product usefulness in their application based on the reported data quality measures.

For S-240 the following data quality elements have been included:

- Conformance to this Product Specification;

- Intended purpose of the data product;

- Completeness of the data product in terms of coverage;

- Logical Consistency;

- Positional Uncertainty and Accuracy;

- Thematic Accuracy;

- Temporal Quality;

- Aggregation measures;

- Elements specifically required for the data product;

- Validation checks or conformance checks including:

  - General tests for dataset integrity;

  - Specific tests for a specific data model;

## Quality measure elements

The recommended data quality measures and their applicability in S-240 are indicated in the table below. NA indicated the measure is not applicable. This table reproduces the first 4 columns of the data quality checklist recommended elements and replaces the final column with descriptions of the scope of the element in the context of S-240 datasets.

| Data quality element and sub element | Definition | DQ measure / description | Evaluation scope | Scope in S-240 |
| --- | --- | --- | --- | --- |
| Completeness / Commission | Excess data present in a dataset, as described by the scope. | numberOfExcessItems / This data quality measure indicates the number of items in the dataset, that should not have been present in the dataset. | dataset/dataset series | All features and info types |
| Completeness / Commission | Excess data present in a dataset, as described by the scope. | numberOfDuplicateFeatureInstances / This data quality measure indicates the total number of exact duplications of feature instances within the data. | dataset/dataset series | All features and info types |
| Completeness / Omission | Data absent from the dataset, as described by the scope. | numberOfMissingItems / This data quality measure is an indicator that shows that a specific item is missing in the data. | dataset/dataset series/spatial object type | All features and info types |
| Logical Consistency / Conceptual Consistency | Adherence to the rules of a conceptual schema. | numberOfInvalidSurfaceOverlaps / This data quality measure is a count of the total number of erroneous overlaps within the data. Which surfaces may overlap and which must not is application dependent. Not all overlapping surfaces are necessarily erroneous. | spatial object / spatial object type | Features with surface geometry; spatial objects of type surface |
| Logical Consistency / Domain Consistency | Adherence of the values to the value domains. | numberOfNonconformantItems / This data quality measure is a count of all items in the dataset that are not in conformance with their value domain. | spatial object / spatial object type | All features and info types |
| Logical Consistency / Format Consistency | Degree to which data is stored in accordance with the physical structure of the data set, as described by the scope | physicalStructureConflictsNumber / This data quality measure is a count of all items in the dataset that are stored in conflict with the physical structure of the dataset. | dataset/dataset series | All features and info types |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | rateOfFaultyPointCurveConnections / This data quality measure indicates the number of faulty link-node connections in relation to the number of supposed link-node connections. This data quality measure gives the erroneous point-curve connections in relation to the total number of point-curve connections. | spatial object / spatial object type | Features with curve geometry; spatial objects of curve types |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | numberOfMissingConnectionsUndershoots / This data quality measure is a count of items in the dataset within the parameter tolerance that are mismatched due to undershoots. | spatial object / spatial object type | Features with curve geometry; spatial objects of curve types |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | numberOfMissingConnectionsOvershoots / This data quality measure is a count of items in the dataset within the parameter tolerance that are mismatched due to overshoots. | spatial object / spatial object type | Features with curve geometry; spatial objects of curve types |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | numberOfInvalidSlivers / This data quality measure is a count of all items in the dataset that are invalid sliver surfaces. A sliver is an unintended area that occurs when adjacent surfaces are not digitized properly. The borders of the adjacent surfaces may unintentionally gap or overlap to cause a topological error. | dataset / dataset series | Features with surface geometry; spatial objects of type surface |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | numberOfInvalidSelfIntersects / This data quality measure is a count of all items in the dataset that illegally intersect with themselves. | spatial object / spatial object type | Features with surface geometry; spatial objects of type surface or curve |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | numberOfInvalidSelfOverlap / This data quality measure is a count of all items in the dataset that illegally self-overlap. | spatial object / spatial object type | Features with surface geometry; spatial objects of type surface or curve |
| Positional Accuracy / Absolute or External Accuracy | Closeness of reported coordinative values to values accepted as or being true. | Root Mean Square Error / Standard deviation, where the true value is not estimated from the observations but known a priori. | spatial object / spatial object type | objects that have coordinative values associated. |
| Positional Accuracy / Vertical Position Accuracy | Closeness of reported coordinative values to values accepted as or being true. | linearMapAccuracy2Sigma / Half length of the interval defined by an upper and lower limit in which the true value lies with probability 95%. | spatial object / spatial object type | objects that have a vertical coordinate values associated. |
| Positional Accuracy / Horizontal Position Accuracy | Closeness of reported coordinative values to values accepted as or being true. | linearMapAccuracy2Sigma / Half length of the interval defined by an upper and lower limit in which the true value lies with probability 95%. | spatial object / spatial object type | objects that have a horizontal coordinate values associated. |
| Positional Accuracy / Gridded Data Position Accuracy | Closeness of reported coordinative values to values accepted as or being true. | Root mean square error of planimetry / Radius of a circle around the given point, in which the true value lies with probability P. | spatial object / spatial object type | NA. S-240 does not have features with gridded geometry |
| Temporal Quality / Temporal Consistency | Consistency with time. | Correctness of ordered events or sequences, if reported. | dataset/dataset series/spatial object type | Features with time intervals, fixed/periodic date ranges, schedules. |
| Thematic Accuracy / ThematicClassificationCorrectness | Comparison of the classes assigned to features or their attributes to a universe of discourse. | miscalculationRate / This data quality measure indicates the number of incorrectly classified features in relation to the number of features that are supposed to be there. [Adapted from ISO 19157] This is a RATE which is a ratio, and is expressed as a REAL number representing the rational fraction corresponding to the numerator and denominator of the ratio. For example, if there are 1 items that are classified incorrectly and there are 100 of the items in the dataset then the ratio is 1/100 and the reported rate = 0.01. | dataset/dataset series/spatial object type | All features and info types |
| Aggregation Measures / AggregationMeasures | In a data product specification, several requirements are set up for a product to conform to the specification. | DataProductSpecificationPassed / This data quality measure is a boolean indicating that all requirements in the referred data product specification are fulfilled. | dataset/dataset series/spatial object type | Dataset as a whole |
| Aggregation Measures / AggregationMeasures | In a data product specification, several requirements are set up for a product to conform to the specification. | DataProductSpecificationFailRate / This data quality measure is a number indicating the number of data product specification requirements that are not fulfilled by the current product/dataset in relation to the total number of data product specification requirements. | dataset/dataset series/spatial object type | Dataset as a whole |

## Test methods

[TBD. Define (or provide a pointer to) testing methods for each element in the table. Some test methods may just reference Annex B (Validation Checks) with the direction to report the number of instances in the dataset which fail the checks for that quality element.]

## Data quality testing and reporting

S-240 products must be tested with the S-240 specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. The checks are listed in Annex B.

Production and certification processes for S-240 data should include a standalone quality report which provides full information on the original results (with evaluation procedures and measures applied).

The dataset or exchange set metadata that is distributed with the exchange set may describe only the aggregated result with a reference to the original results described in the standalone quality report. The aggregated Data Quality result provides an indication if the dataset has passed conformance to the Data Product Specification.

Data Quality Measure Aggregation results should be included to indicate if the dataset/dataset series have passed the Product Specifications. The elements which must be included are described in the table below.

Table 2. Elements of data quality aggregated report (extract from DQWG checklist)

| Data quality element and sub element | Definition | DQ measure / description | Evaluation scope | Applicable to spatial representation types |
| --- | --- | --- | --- | --- |
| Aggregation Measures / AggregationMeasures | In a data product specification, several requirements are set up for a product to conform to the specification. | DataProductSpecificationPassed / This data quality measure is a boolean indicating that all requirements in the referred data product specification are fulfilled. | dataset | All features and information types of the dataset |
| Aggregation Measures / AggregationMeasures | In a data product specification, several requirements are set up for a product to conform to the specification. | DataProductSpecificationFailRate / This data quality measure is a number indicating the number of data product specification requirements that are not fulfilled by the current product/dataset in relation to the total number of data product specification requirements. | dataset | All features and information types of the dataset |

# Data Capture and classification

## Introduction

S-240 products must be based on data sources released by an appropriate DGNSS Station Almanac defining authority. Data source must be described in each data product.

The production process used to generate DGNSS Station Almanac products may be described in the dataset metadata.

Table 3. Data capture information

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| dataSource | Identification of the kinds of data sources usable to product datasets compliant with the considering specification | 0..* | CharacterString |
| productionProcess | Link to a textual description of the production process (including encoding guide) applicable to the datasets compliant with the considering specification | 0..* | CharacterString (URL) |

## Data Encoding and Product Delivery

### Data Encoding

The principal encoding will be the Open Geospatial Consortium (OGC), Geography Markup Language (GML) format. GML is an XML grammar designed to express geographical features. It serves as a modelling language for geographic systems as well as an open interchange format for geographic transactions.

### Types of Datasets

A dataset is a grouping of features, attributes, geometry and metadata which comprises a specific coverage. The following types of S-240 dataset may be produced and contained within an exchange set:

Table 4. DGNSS Station Almanac dataset types

| Dataset | Explanations |
| --- | --- |
| New dataset (base dataset): | Data for an area different (in coverage and/or extent) to existing datasets. |
| New Edition of a dataset: | A re-issue plus new information which has not been previously distributed by Updates. Each New Edition of a dataset must have the same name as the dataset that it replaces and should have the same spatial extents. The edition number in the dataset discovery metadata shall increment up by one from the previous edition. |
| Update dataset | A delta change of the latest edition of a dataset. If there are more than one update dataset, the subsequent update will be a delta of the base dataset + earlier update datasets. |
| Cancellation | Used to cancel dataset. |

### Content of Update Datasets

Update datasets can only contain replacements, deletions and additions of whole feature instances or information instances. This means that when a feature or information instance is updated, the new version must contain all the attributes of the old instance, including any inline spatial attributes (i.e., inline geometry), except those attributes that are being removed.

An association to an instance of a feature or information type is treated as an attribute of the referring instance, and therefore adding or deleting an association means the original referring instance must be replaced with a new version. The instance at the other end of the association needs to be replaced if and only if it contains a reference to the first instance.

Spatial objects that are not inline (i.e., geometry that is encoded as an independent spatial object in the dataset) is treated like any other object, i.e., it needs to be updated if and only if the primitive has changed (e.g., a coordinate is updated).

Feature and information type instances are deleted without replacement by setting the fixedDateRange.dateEnd attribute of the instance to the date of deletion, which will usually be the issue date of the update.

## Encoding of Latitude and Longitude

Values of latitude and longitude must be accurate to 7 decimal places. Coordinates must be encoded as decimals in the format described below. The encoding is indicated by multiplication factor fields defined in the dataset identification record.

Values should be coded as decimal numbers with 7 or fewer digits after the decimal. The normative encoding is in degrees, with an accuracy of 10-7 degrees, i.e., 7 digits after the decimal point.

The decimal point must be indicated by the “.” character.

Trailing zeroes after the decimal point (and the decimal point itself if appropriate) may be omitted at producer discretion, but the accuracy must still be as indicated (e.g., 10-7 degrees for coordinates of default accuracy).

Latitude and longitude multiplication factors held in the Dataset Structure Information field under [coordMultFactorX] and [coordMultFactorY] must be set to a value corresponding to the encoding, i.e., {1} for coordinates encoded in decimal degrees.

EXAMPLE 1	A longitude = 42.0000 is converted into X = longitude * coordMultFactorX = 42.0000 * 1 = 	42.0000000.

## Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

## Text Attribute Values

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

## Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue and summarised in Annex A – Data Classification and Encoding Guide.

## Unknown Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” .

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

## Structure of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

## Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

## Data coverage

All areas of a dataset must be covered by a DataCoverage meta feature.

An update dataset must not change the limit of a Data Coverage feature for the base dataset. Where the limit of a Data Coverage feature for a base dataset is to be changed, this must be done by issuing a new edition of the dataset.

## Data overlap

S-240 datasets shall not overlap other S-240 datasets.

## Data extent

Datasets must not cross the 180° meridian of longitude

# Data Maintenance

## Introduction

Datasets are maintained as needed and must include mechanisms for DGNSS Station Almanac updating. Data updates will be made by new editions. The maintenance and update frequency of DGNSS Station Almanac datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 5. Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

## Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix B.

## Dataset updates and cancellation

The purpose of issue of the dataset is indicated in the “purpose” field of the dataset discovery metadata. In order to terminate a dataset, an update dataset file is created for which the edition number must be set to 0. This convention is only used to cancel a base dataset file.

Where a dataset is cancelled and its name is reused at a later date, the issue date must be greater than the issue date of the cancelled dataset.

When the dataset is cancelled it must be removed from the system.

An exchange set may contain base dataset files and update dataset files for the same datasets. Under these circumstances the update dataset files must follow in the correct sequential order from the last update applied to the base dataset file.

## Support file updates

The purpose of issue is indicated in the “purpose” field of the support file discovery metadata. Support files carrying the “deletion” flag in metadata must be removed from the system. When a feature or information type pointing to a text, picture or application file is deleted or updated so that it no longer references the file, the system software must check to see whether any other feature or information type references the same file, before that file is deleted.

Updates or deletions of a support file may require concurrent updates to feature or information type instance attributes that depend on the file, e.g., pictorialRepresentation, fileReference and fileLocator attributes.

## Feature and portrayal catalogues

For each new version of the S-240 Product Specification a new feature and portrayal catalogue will be released. The system must be able to manage datasets and their catalogues that are created on different versions of the S-240 product specification.

## Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

[Version numbers can be indicated in datasets by adding a new ‘version’ attribute to FeatureType and InformationType’, or as instance metadata – simple enough in GML but likely more trouble in other encodings.]

## Dataset encryption

TBD

# Portrayal

A Portrayal Catalogue defines the display rules and symbology which apply to the Features defined in this specification.

The Portrayal Catalogue contains portrayal functions that map features to symbology. It also contains symbol definitions, colour definitions, portrayal parameters and portrayal management concepts such as viewing groups. Each catalogue should include a reference to a portrayal library that contains a set of portrayal rules and a set of portrayal specifications (Table 9.1). This information should be included in the product specification for each identified scope.

Table 6. Portrayal Information

| Item Name | Description | M/O | Card | Type |
| --- | --- | --- | --- | --- |
| portrayalLibraryCitation | Bibliographic reference to the portrayal library | O | 0..1 | CI_Citation (ISO 19115) |

# Data Product format (Encoding)

The principal encoding will be the Open Geospatial Consortium (OGC), Geography Markup Language (GML) format. GML is an XML grammar designed to express geographical features. It serves as a modelling language for geographic systems as well as an open interchange format for geographic transactions.

Table 7. Data product format

| formatName | S-100 GML |
| --- | --- |
| version | 1.0 |
| characterSet | UTF-8 |
| specification | S-100 GML Profile 1.0 |

# Data Product delivery

## Data Product Delivery Information

This data product specification defines GML as the primary format in which S-240 data products are delivered. The delivery format is described by the following items (from ISO 19131:2005):  format name, version, specification, language, character set.

Table 8. Data product delivery

| Name | ISO 19131 Elements | Value |
| --- | --- | --- |
| Format name | DPS_DeliveryInformation.deliveryFormat > DPS_DeliveryFormat.formatName | GML* |
| Version | DPS_DeliveryInformation.deliveryFormat > DPS_DeliveryFormat.version | 3.2.1 |
| Specification description | DPS_DeliveryInformation.deliveryFormat > DPS_DeliveryFormat.specification | GML* |
| Language | DPS_DeliveryInformation.deliveryFormat > DPS_DeliveryFormat.language | English |
| Character set | DPS_DeliveryInformation.deliveryFormat > DPS_DeliveryFormat.characterSet > MD_CharacterSetCode | 004 – utf8 |

* GML is an XML encoding for the transport and storage of geographic information, including both the geometry and the properties of geographic features, between distributed systems. The XML Schema for the GML application schema is provided in a schema document S240.xsd which imports other schema(s) defining common types. Feature instance shall validate against S240.xsd and conform to all other requirements specified in this data product specification including all constraints not captured in the XML Schema document.

### Dataset loading

Datasets must always be loaded in the order of base dataset first, then update datasets in the corrected sequential order. Systems are not to load updates out of order, for example if update 1-5 is present, then 6 is missing, update 7 must not be loaded.

### New editions

When a new edition of a dataset is received, the system must replace the previous edition, along with any updates with the new edition of the dataset. Loading of subsequent updates follow the same rule as above.

## Dataset size

S-240 datasets shall not exceed 20MB.

Update datasets shall not exceed 500KB.

## Exchange Set

Data which conforms to this product specification must be delivered by means of an exchange set.

An exchange set will consist of one or more S-240 datasets.  An exchange set may also include one or more support files containing supplementary information encoded in separate files. These are linked to the S-240 dataset features, using the attributes described below. Each exchange set will include a single (XML) catalogue file, S-240 exchange set catalogues conform to S-100 4.0.0 Figure 4a-D-2 without modification, containing discovery metadata for each S240 dataset as well as support files. S-240 Exchange set structure conforms to S-100 4.0.0 Figure 4a-D-3 without modification.

Figure 4. Exchange set structure

## Support Files

Support files contain ancillary textual or graphic information in separate (linked in dataset) files. Information should be encoded in a structured format as defined by W3C.  The following formats would be suitable for graphics:

- Portable Network Graphics (PNG) [Edition 2.0]

- Scalable Vector Graphics (SVG) [Edition 1.1]

- Tagged Image File Format (TIFF) [Edition 6.0]

- Joint Photographic Experts Group (JPEG) [Edition 1.02 ]

Note: PNG is an extensible file format designed for lossless, portable storage of raster images. It provides a patent-free replacement for the GIF format and also replicates many common uses of TIFF. The PNG edition 2 format has been adopted as an ISO standard, (ISO/IEC 15948:2003).  SVG is a language for describing two-dimensional graphics in XML [XML10]. SVG allows for three types of graphic objects: vector graphic shapes (e.g., paths consisting of straight lines and curves), images and text. The JPEG standard specifies the codec, which defines how an image is compressed into a stream of bytes and decompressed back into an image, but not the file format used to contain that stream. (The term "JPEG" is an acronym for the Joint Photographic Experts Group, which is the body that created the standard).

## Support File Naming Convention

All support files will have unique world-wide file identifiers. The file identifier of support information should not be used to describe the physical content of the file. The support file metadata that accompanies the file will inform the user of the name and purpose of the file (new, replacement and deletion).

In this encoding the support files are named according to the specifications given below:

CCNNN240XXXXXXXX.YYY

The main part forms an identifier where:

- the first two characters identify the issuing agency

- the third to fifth characters must be NNN to identify that this is GNSS publication information,

- the sixth to eighth characters must be 240 to identify that this is S-240 DGNSS Station Almanac,

- the ninth up to the sixteenth character can be used in any way by the producer to provide a unique file name for the dataset. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- .YYY – support file extension.

## Dataset Naming Convention

All dataset files will have unique world-wide file identifiers. The file identifier of the dataset should not be used to describe the physical content of the file. The dataset file metadata that accompanies the file will inform the user of the name and purpose of the file (new, replacement and deletion).

In this encoding the dataset files are named according to the specifications given below:

CCNNN240XXXXXXXX.GML

The main part forms an identifier where:

- the first two characters identify the issuing agency

- the third to fifth characters must be NNN to identify that this is GNSS publication information,

- the sixth to eighth characters must be 240 to identify that this is S-240 DGNSS Station Almanac,

- the ninth up to the sixteenth character can be used in any way by the producer to provide a unique file name for the dataset. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

## Update dataset naming convention

All update dataset files will have an identical name to the base dataset, aside from the separator and update number sequence.

In this encoding the update dataset files are named according to the specifications given below:

CCNNN240XXXXXXXX_XXX.GML

The main part forms an identifier where:

- the first two characters identify the issuing agency

- the third to fifth characters must be NNNI to identify that this is GNSS publication information,

- the sixth to eighth characters must be 240 to identify that this is DGNSS Station Almanac,

- the ninth up to the sixteenth character can be used in any way by the producer to provide a unique file name for the dataset. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- The seventeenth character shall be an underscore

- The eighteenth to twentieth characters shall be numerical (0-9 characters to indicate the place of the update dataset in the update sequence.

## Catalogue File Naming Convention

The exchange catalogue acts as the table of contents for the exchange set. The catalogue file of the exchange set must be named CATALOG.240.XML. No other file in the exchange set may be named CATALOG.240.XML. The content of the exchange catalogue file is described in Section 14.

# Metadata

## Introduction

The S-240 metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 5. Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0.  All other metadata fields must be blank.

## Dataset Metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 9. Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S-100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name (see 10.11.6) |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString |  |
| dataProtection | 0..1 | {1} or {2} | CharacterString | 1. Encrypted 2. Unencrypted |
| protectionScheme | 0..1 |  | CharacterString | For example S-63 |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_Cla ssificationCode (codelist) | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_ResponsibleParty) | brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 | S-240 version N.n | CharacterString | This must be encoded as S240.N.n |
| producingAgency | 1 |  | CI_ResponsibleParty | Party responsible for generating the dataset. |
| horizontalDatumReference | 1 | WGS84 | CharacterString | EPSG |
| horizontalDatumValue | 1 | WGS84 | Integer | 4326 |
| verticalDatum | 1 |  | S100_VerticalAndSoundingDatum | (ignored by applications, not relevant to S-240) |
| soundingDatum | 1 | nilled | S100_VerticalAndSoundingDatum | (ignored by applications, not relevant to S-240) |
| dataType | 1 | GML | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS, ECS, radar ??? |

## Update and Cancellation Dataset Metadata

Update dataset metadata is intended to describe information about an update dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of an update dataset. Whereas dataset metadata is usually fairly comprehensive, metadata for update datasets only describe the issue date and sequential relation to the base dataset.

Table 10. Update dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name (see 11.7) |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Brief description of the update. |
| dataProtection | 0..1 | {1} or {2} | CharacterString | Value must be same as base dataset. |
| protectionScheme | 0..1 |  | CharacterString | Value must be same as base dataset. |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints - >MD_RestrictionCode <copyright> (ISO 19115) | Value must be same as base dataset. |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_Cla ssificationCode (codelist) | Value must be same as base dataset. |
| purpose | 1 | {3}, {4} | CharacterString | - Update - Cancellation |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_ResponsibleParty) | brief description of the resource and/or resource series usage |
| editionNumber | 1 |  | Integer | Value must be same as base dataset. |
| updateNumber | 1 |  | CharacterString | Update sequence number, must match file name. |
| updateApplicationDate | 1 |  | Date | Date of update |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 | S-240 version N.n | CharacterString | Value must be same as base dataset. |
| producingAgency | 1 |  | CI_ResponsibleParty | Party responsible for generating the dataset. |
| horizontalDatumReference | 1 | WGS84 | CharacterString | EPSG |
| horizontalDatumValue | 1 | WGS84 | Integer | 4326 |
| verticalDatum | 1 |  | S100_VerticalAndSoundingDatum | (ignored by applications, not relevant to S-240) |
| soundingDatum | 1 | nilled | S100_VerticalAndSoundingDatum | (ignored by applications, not relevant to S-240) |
| dataType | 1 | GML | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-240 | CharacterString | Value must be same as base dataset. |

## Support file Metadata

Support file metadata is intended to describe information about a data resource. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a data resource.

Table 11. Support file metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_SupportFiletDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString |  |
| fileLocation | 1 |  | CharacterString | Path relative to the root directory of the exchange set. The location of the file after the exchange set is unpacked into directory <EXCH_ROOT> will be <EXCH_ROOT>/<filePath>/<filename> |
| purpose | 1 |  | S100_SupportFilePurpose | new, replacement, or deletion. |
| editionNumber | 1 |  | CharacterString | When a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. Edition number remains the same for a re-issue |
| issueDate | 1 |  | Date |  |
| supportFileSpecification | 1 |  | S100_SupportFileSpecification |  |
| dataType | 1 |  | S100_SupportFileFormat |  |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 |  | CharacterString |  |
| comment | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString | Reference to the appropriate digital signature algorithm |
| digitalSignatureValue | 0..1 |  | CharacterString |  |

## Exchange Set Metadata

Frequently datasets are packaged and distributed as composite exchange sets by third party vendors. An exchange set could contain many different types of datasets, sourced from different data producers. For example an exchange set may contain numerous dataset files, ancillary data files, discovery metadata files and others.  Exchange set metadata contains metadata about the contents of the exchange set and metadata about the data distributor.

Figure 6. Exchange set metadata

## Catalogue File Metadata.

All S-240 Catalogue metadata files must contain at least the following metadata elements.

Table 12. Catalogue file metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification |
| metadataLanguage | 1..2 | English, French | CharacterString | All datasets conforming to this PS must use English language. The catalogue file must be in English with the optional addition of French. |
| exchangeCatalogueName | 1 | CATALOG.240.XML | CharacterString | Catalogue filename |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| algorithmMethod | 0..1 |  | CharacterString | For example. RAR or ZIP |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |

# ANNEX A	Data classification and Encoding Guide

| Definition: DGNSS STATION REGION Information about a region that each DGNSS station is located |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-240 Information Feature: DgnssStationRegion |  |  |  |  |  |
| Primitives: None |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-240 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Date of issue |  |  |  | (S) DA | 1,1 |
| Date of last update |  |  |  | (S) DA | 1,1 |
| country |  |  |  | (S) TE | 1,* |
| Encoding Guide |  |  |  |  |  |

| Definition: DGNSS STATION ALMANAC Information about each DGNSS station |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-240 Information Feature: DGNSSStationAlmanac |  |  |  |  |  |
| Primitives: None |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-240 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Bit rate |  |  |  | IN | 1,1 |
| Signal Frequency |  |  |  | RE | 1,1 |
| Nominal Range At |  |  |  | IN | 1,1 |
| Nominal Range Km |  |  |  | IN | 1,1 |
| Radiobeacon Health |  | 1 : Radio Beacon Operation Normal 2 : No Integrity Monitor Operating 3 : No Information Available 4 : Don’t Use This Radio Beacon |  | EN | 1,1 |
| Reference Station IDs |  |  |  | (S) TE | 0,2 |
| Station Name |  |  |  | (S) TE | 1,1 |
| Transmitted Messages Types |  | 1 : Differential GPS Corrections 2 : Delta Differential GPS Corrections 3 : GPS Reference Station Parameters 4 : Reference Station Datum 5 : GPS Constellation Health 6 : GPS Null Frame 7 : DGPS Radio beacon Almanac 8 : Pseudolite Almanac 9 : GPS Partial Correction Set 10 : P-Code Differential Corrections 11 : C/A-Code L1L2 Delta Corrections 12 : Pseudolite Station Parameters 13 : Ground Transmitter Parameters 14 : GPS Time of Week 15 : Ionospheric Delay Message 16 : GPS Special Message 17 : GPS Ephemerides 18 : RTK Uncorrected Carrier Phases 19 : RTK Uncorrected Pseudoranges 20 : RTK Carrier Phase Corrections 21 : RTK/Hi-Accuracy Pseudo range Corrections 22 : Extended Reference Station Parameters 23 : Antenna Type Definition Record 24 : Antenna Reference Point(ARP) 27 : Extended Radio beacon Almanac 31 : Differential GLONASS Corrections 32 : Differential GLONASS Reference Station Parameters 33 : GLONASS Constellation Health 34 : GLONASS Partial Differential Correction Set 35 : GLONASS Radio beacon Almanac 36 : GLONASS Special Message 37 : GNSS System Time Off set 59 : Proprietary Message 60 : Multipurpose Usage |  | EN | 1,* |
| Transmitting Station ID |  |  |  | (S) TE | 0,1 |
| Encoding Guide |  |  |  |  |  |

| IHO Definition: RADIO STATION. A place equipped to transmit radio waves. Such a station may be either stationary or mobile, and may also be provided with a radio receiver. In British terminology, also called w/t station. (IHO Dictionary – S-32). |  |  |  |
| --- | --- | --- | --- |
| S-240 Geo Feature: Radio station (RDOSTA) |  |  |  |
| Primitives: Point |  |  |  |
| Real World | Paper Chart Symbol | ECDIS Symbol |  |
| S-101 Attribute | Allowable Encoding Value | Type | Multiplicity |
| Call sign |  | TE | 0,1 |
| Category of radio station | 10 : Differential GNSS | EN | 0,1 |
| Communication channel |  | TE | 0,* |
| Estimated range |  | RE | 0,1 |
| Feature name |  | C | 0,* |
| Display name |  | (S) BO | 0,1 |
| Language | ISO 639-3 | (S) TE | 0,1 |
| Name |  | (S) TE | 1,1 |
| Fixed date range |  | C | 0,1 |
| Date end | ISO 8601:1988 | (S) DA | 0,1 |
| Date start | ISO 8601:1988 | (S) DA | 0,1 |
| Periodic date range |  | C | 0,* |
| Date end | ISO 8601:1988 | (S) DA | 1,1 |
| Date start | ISO 8601:1988 | (S) DA | 1,1 |
| Signal frequency |  | IN | 0,1 |
| Status | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private | EN | 0,* |
| Scale minimum | See clause X.X | IN | 0,1 |
| INT 1 Reference: S 10-16 ### Radio stations (see S-4 – B-480-484) Transmissions from radio stations may provide mariners with a line of position. Most radio position fixing systems require Radio Direction Finding (RDF) equipment to determine the bearing of the transmitting device; such equipment is generally no longer fitted on vessels. The exception is “emergency use only” VHF-based direction finding services (which do not use RDF equipment. Consequently, the following radio position-fixing stations are now obsolete and there is no longer any value in encoding them on ENCs: - Circular (non-directional) (RC), directional (RD) and rotating pattern (RW) marine radiobeacons; - Consol beacons (Consol); - Aeronautical radiobeacons (Aero RC); - Radio direction-finding stations (except VHF-based emergency stations) (RG); - Coast Radio Stations providing ‘QTG’ service (R). The feature “radio station” is used to encode the point of transmission of the signal. If it is required to encode a radio station, it must be done using the feature Radio Station. Remarks: - The Radio Station must only be used to encode the technical equipment itself, independent of the building or structure in which it is installed. If it is required to encode the building or structure (e.g. mast, tower, radar dome), it must be done using an appropriate feature (e.g. Building, Landmark). - Further information (e.g. transmission characteristic) may be encoded using complex attributes information or textual description. - Each VHF-channel should be indicated, using the attribute communication channel - If it is required to encode a DGPS station, it must be done using Radio Station, with attribute category of radio station = 10 (Differential GNSS). - Where required, the attribute signal frequency must be quoted in Hertz, e.g. a signal frequency of 950 MHz must be encoded as 950000000. ### Radio direction-finding stations (see S-4 – B-483) If it is required to encode a radio direction-finding station, it must be done using a Radio Station feature, with attribute category of radio station = 5 (radio direction-finding station). The identification signal may be encoded using the attribute call sign. Remarks: - Direction–finding is now only provided as an emergency service by VHF. Distinction: Radar station; radio calling-in point. |  |  |  |

# ANNEX B	Validation Check

- References

IHO S-58 ENC VALIDATION CHECKS Edition 6.0.0 – 2016

[IHO data quality checklist to be added after it gets a formal document number.]

- Abbreviation

PS – Product Specification

DCEG – Data Capture and Encoding Guide

- Production validation checks for S-240 DGNSS Station Almanac

The following checks are intended for production systems designed to produce S-240 DGNSS Station Almanac datasets. The checks can be administered at any time during the production phase. All checks should be considered as warnings, even though more severe classifications are available, due to the status of the development and lack of experience with system use of S-240 datasets, it is considered premature to classify any checks as error or critical error at this time.

  - Check Classification

| C | Critical Error | An error which would make a S-240 dataset unusable in application system |
| --- | --- | --- |
| E | Error | An error which may degrade the quality of the S-240 dataset through appearance or usability |
| W | Warning | An error which may be duplication or an inconsistency which will not noticeably degrade the usability of an S-240 dataset |

    - Check application

| B | Base | Apply check to new dataset, new edition, and post-update dataset (after updates have been applied to the base). |
| --- | --- | --- |
| U | Update | Apply check to update datasets in isolation. |
| S | Post-update | Apply check only to a post-update dataset, i.e., subsequent to application of all available updates. |

Checks do not apply to dataset terminations or cancellations, except where the check description explicitly states it applies in case of a termination or cancellation.

  - Checks relating to S-240 Product Specification

[This table will be extended with indication of which of the data quality elements applies to each test. The elements are described in Section 9 or the main specification.]

| No | Check description | Check message | Check solution | Conformity to: | Apply to |
| --- | --- | --- | --- | --- | --- |
| 100 | For each feature object where its geometry is not COVERED_BY a DataCoverage | Objects fall outside the coverage object. | Ensure objects are not outside of the limits of the cell. |  | B, S |
| 101 | If the cell file size is greater than 20 Megabytes. | The cell is larger than 20Mb in size. | Ensure that the cell is not larger than 20Mb. |  | B, S |
| 102 | For each feature record where the name is not unique WITHIN the dataset. | Duplicate FOIDs exist within the dataset. | Ensure that no duplicate FOIDs exist. |  | B, U, S |
| 103 | If either QualityOfNonbathymetricData or DataCoverage meta objects do not exist within the data set. | Mandatory feature objects are missing. | Include mandatory meta feature objects QualityOfNonbathymetricData or DataCoverage |  | B, S |
| 104 | If any mandatory attributes are not Present. | Mandatory attributes are not encoded | Populate mandatory attributes (If unknown encode attribute with empty value). |  | B, U |
| 105 | For each feature object with an attribute of type Float or Integer where the value contains zeroes before the first numerical digit or after the last numerical digit. | Values have been padded with non-significant zeroes. | Remove non-significant zeroes. |  | B, U |
| 106 | For each feature object with an attribute value identical to a corresponding attribute of a meta object it is COVERED_BY. | An attribute value of a meta object is duplicated on a geo object. | Remove duplicate value from geo object. | Logical consistency | B, S |
| 107 | For each association between features instances, features instances and information instances, and between information instances that is not defined in the feature catalogue | Wrong association used | Use correct association type | Logical consistency | B, U |
| 108 | For each role name on associations that is not defined in the feature catalogue | Wrong role used | Use correct role name | Logical consistency | B, U |
| 109 | For each association that is not defined in the feature catalogue. | Unknown association is used. | Use association that is defined in the feature catalogue. | Logical consistency | B, U |
| 110 | For each role name that is not defined in the feature catalogue. | Unknown role name is used. | Use role name that is defined in the feature catalogue. | Logical consistency | B, U |
| 111 | For each association ensure associated classes are only those permitted by the feature catalogue. | Class is associated in an illegal association. | Ensure correct association is used between classes. | Logical consistency | B, U, S |
| 112 | For each role name ensure it is only used with permitted associations. | Role name is used on an illegal association. | Ensure correct role names are used on the association. | Logical consistency | B, U, S |
| 113 | Ensure dataset conformance to the GML schema | Dataset does not conform to the GML schema | Ensure conformance to the GML schema |  | B, U |
| 114 | Ensure all text fields are encoded using UTF-8 | Illegal character set used. | Change character encoding to UTF-8 |  | B, U |
| 115 | For each feature instance where more than one featureName is present, and the name subattribute of two or more featureName instances is equal. | Values name sub attribute are identical. | Ensure that name subattributes are populated with the correct values. | Logical consistency | B, U |
| 116 | For each feature instance where textContent attribute is present, and two or more information subAttributes are present, and the text subattributes are equal | Values for text sub attribute are identical. | Ensure that text subattributes are populated with the correct values. | Logical consistency | B, U |
| 117 | For each feature instance where textContent attribute is present, and two or more information subAttributes are present, and the combination of fileReference and fileLocator subattributes are equal | Values for file reference and locator combinations are identical. | Ensure that national language attributes are populated with the correct values. | Logical consistency | B, U |
| 118 | For each featureName subattribute with language not equal to eng, and where featureName subattributes with language equal to eng is not present. | Name is encoded in national language only. | Populate text attribute with English text. | Logical consistency | B, U |
| 119 | For each information subattribute with language not equal to eng, and where information subattribute with language equal to eng is not present. | Text is encoded in national language only | Populate name attribute with English text. | Logical consistency | B, U |
| 120 | If the horizontalDatum reference and value attributes of DataSetDiscoveryMetadata are Not equal to EPSG:4326 (WGS 84). | horizontalDatum reference and value are not EPSG 4326 | Set the horizontalDatum reference and value attributes to EPSG 4326 |  | B, U |
| 121 | If the file names in an exchange set are not in accordance with the Product Specification. | File names are not in accordance with the Product Specification. | Amend file names. |  | B, U |
| 122 | For each feature instance that does not OVERLAP OR is WITHIN an area of dataCoverage | Object outside area of coverage. | Remove object or amend coverage. |  | B, S |
| 123 | For each feature instance, which does not have a valid feature class label/code as defined by the feature catalogue. | Object has invalid feature class code. | Amend object class code. | Logical consistency | B, U |
| 124 | For each attribute, which does not have a valid attribute label/code as defined by the feature catalogue. | Attribute has invalid attribute label/code. | Amend attribute label/code. | Logical consistency | B, U |
| 125 | For each feature object, which contains attributes outside the list of permissible attributes for the feature class (as defined in the feature catalogue). | Attribute not permitted on feature class. | Remove attribute. | Logical consistency | B, U |
| 126 | For each feature instance which is not COVERED_BY the combined coverage of QualityOfNonBathymetricData meta feature instance. | Feature instance not covered by an QualityOfNonBathymetricData instance. | Ensure full coverage of QualityOfNonBathymetricData instance |  | B, S |
| 127 | If the order of the data in a dataset is not correct. | Incorrect data order. | Amend data order. |  | B, U |
| 128 | For each attribute instance where the total number of instances exceed the permitted number of instances | Too many instances of attribute. | Ensure correct attribute encoding. | Logical consistency | B, U |
| 129 | For each feature instance where periodicDateRange subattributes dateEnd and dateStart are notNull AND their values are identical. | Object has identical values of periodicDateRange subattributes dateEnd and dateStart. | Ensure values of periodicDateRange subattributes dateEnd and dateStart are logical. | Logical consistency | B, U |
| 130 | For each feature instance where periodicDateRange subattribute dateStartis notNull AND dateEnd is Null OR not Present. | Object has dateStart without a value of dateEnd. | Populate dateEnd or remove dateStart . | Logical consistency | B, U |
| 131 | For each feature instance where periodicDateRange subattribute is notNull AND dateStart is Null OR not Present. | Object has dateEnd without a value of dateStart. | Populate dateStart or remove dateEnd. | Logical consistency | B, U |
| 132 | For each linear geometry, which contains vertices at a density Greater than 0.3mm at 1:10000. | Vertex density exceeds the allowable tolerance. | Generalise edge(s). |  | B, U |
| 135 | For each instance of a file referenced in the data, and it is not present in the exchange set | File referenced in the dataset is not present in the exchange set | Add file to exchange set or remove reference to file. |  | B, S |
| 136 | For each instance of a dataset, present in the exchange set and that does not have dataset discovery metadata. | Dataset discovery metadata is missing for dataset | Add dataset discovery metadata |  | B, U |
| 137 | For each instance of a support file, present in the exchange set and that does not have support file discovery metadata. | Support file discovery metadata is missing for support file. | Support file discovery metadata |  | B, U |
| 138 | For each file referenced by the catalogue file in the exchange set, and not present in the exchange set | File is missing from exchange set. | Add file to exchange set or remove reference to file. |  | B, U |
| 140 | For each dataset discovery metadata file that does not correspond to the dataset discovery metadata content table. | Dataset discovery metadata file that does not correspond to the dataset discovery metadata content table. | Ensure correct encoding of the discovery metadata file |  | B, U |
| 141 | For each support file discovery metadata file, does not correspond to the support file discovery metadata content table. | Support file discovery metadata file, does not correspond to the support file discovery metadata content table. | Ensure correct encoding of the support file metadata file |  | B, U |
| 142 | For each catalogue file discovery metadata file, does not correspond to the catalogue file discovery metadata content table. | Catalogue file discovery metadata file, does not correspond to the catalogue file discovery metadata content table. | Ensure correct encoding of the catalogue metadata file. |  | B, U |
| 143 | For each fixedDateRange where dateEnd and dateStart are not Null and dateEnd is less than or equal to dateStart. | dateEnd less than dateStart | Amend values of dateEnd or dateStart accordingly. | Logical consistency | B, U |
| 144 | For each fixedDateRange where both dateEnd and dateStart are omitted or Null | FixedDateRange not Populated making the attribute meaningless. | Populate at least one of dateEnd or dateStart | Logical consistency | B, U |
| 145 | For each fixedDateRange where dateEnd or dateStart is not Null but the year component is not specified. | Year is required in fixedDateRange dates. | Populate year component of dateStart and dateEnd | Logical consistency | B, U |
| 146 | For each textContent attribute where both information and onlineResource are missing or null. | TextContent is not Populated with meaningful information. | Populate information or onlineResource. | Logical consistency | B, U |
| 147 | For each information attribute where both fileReference and text subattributes are missing or null | Information is not Populated with meaningful information. | Populate fileReference or text attribute. | Logical consistency | B, U |
| 148 | For each sourceIndication where the sub-attributes are missing or null | SourceIndication is not populated with meaningful information. | Populate source indication | Logical consistency | B, U |
| 167 | If the update dataset file size is greater than 500Kbytes | The update is larger than 500 KB in size. | Ensure that the cell is not larger than 500 Kbytes |  | U |
| 168 | For each update dataset, a base dataset of the same dataset name is not either present on the system, in the same exchange set, or in another exchange set in the collection of exchange sets. | Base dataset not present | Verify that the absence base dataset is not an error. | Logical consistency | U |
| 169 | For each update dataset with updateNumber N > 1, the base dataset is present and updates 1 to N-1 are not present on the system, in the exchange set, or in another exchange set on the system. | Preceding updates not present | Obtain and install preceding updates. | Logical consistency | U |
| 170 | For each update dataset, without a feature, information type, or support file | Empty update | Add the data or support file or correct the metadata. | Logical consistency | U |
| 171 | For each cancellation (termination) of a dataset that does not exist on the system or has already been cancelled | Terminated dataset is not present | Ignore the update | Logical consistency | U |
| 172 | For each cancellation (termination) of a dataset where the update exchange set contains a corresponding dataset file | Cancellations cannot contain data objects | Remove the dataset file from the exchange set or correct the metadata | Logical consistency | U |

# ANNEX C S-240 GML Schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns:S100="http://www.iho.int/s100gml/5.0" xmlns:gml="http://www.opengis.net/gml/3.2" xmlns="http://www.iho.int/test/gml/cs0/1.0" targetNamespace="http://www.iho.int/test/gml/cs0/1.0" elementFormDefault="qualified" version="1.0">

<!-- ========================================================================================== -->

<!-- S100 XML/GML Schema for test                                                               -->

<!-- ========================================================================================== -->

<xs:annotation>

<xs:documentation>Application schema for test dataset</xs:documentation>

</xs:annotation>

<!-- ========================================================================================== -->

<!-- Import XSD for reference (s100gmlbase.xsd, S100_gmlProfile.xsd, S100_gmlProfileLevels.xsd) -->

<!-- ========================================================================================== -->

<xs:import namespace="http://www.opengis.net/gml/3.2" schemaLocation="https://schemas.s100dev.net/schemas/S100/5.0.0/S100GML/20220620/S100_gmlProfile.xsd"/>

<xs:import namespace="http://www.iho.int/s100gml/5.0" schemaLocation="https://schemas.s100dev.net/schemas/S100/5.0.0/S100GML/20220620/s100gmlbase.xsd"/>

<!-- ========================================================================================== -->

<!-- common types - consider defining a common                                                  -->

<!-- ========================================================================================== -->

<xs:complexType name="S100_TruncatedDate">

<xs:annotation>

<xs:documentation>built in date types from W3C XML schema, implementing S-100 truncated date</xs:documentation>

</xs:annotation>

<xs:choice>

<xs:element name="gDay" type="xs:gDay"/>

<xs:element name="gMonth" type="xs:gMonth"/>

<xs:element name="gYear" type="xs:gYear"/>

<xs:element name="gMonthDay" type="xs:gMonthDay"/>

<xs:element name="gYearMonth" type="xs:gYearMonth"/>

<xs:element name="date" type="xs:date"/>

</xs:choice>

</xs:complexType>

<xs:simpleType name="ISO639-3">

<xs:annotation>

<xs:documentation>stub for ISO 639-3 language codes</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:pattern value="\w{3}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="ISO639-2">

<xs:annotation>

<xs:documentation>stub for ISO 639-2 language codes</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:pattern value="\w{3}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="ISO3166-alpha2">

<xs:annotation>

<xs:documentation>stub for ISO 3166 alpha2 country codes</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:pattern value="\w{2}"/>

</xs:restriction>

</xs:simpleType>

<!-- ========================================================================================== -->

<!-- types and elements for the simple attribute types definition                               -->

<!-- ========================================================================================== -->

<!-- ========================================================================================== -->

<!-- Enumeration                                                                                -->

<!-- ========================================================================================== -->

<xs:simpleType name="transmittedMessageTypeType">

<xs:restriction base="xs:string">

<xs:enumeration value="Differential GPS Corrections">

<xs:annotation>

<xs:documentation>Primary message type which provides the differentially corrected pseudorange measurement that should be processed by the User Equipment navigation filter.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Delta Differential GPS Corrections">

<xs:annotation>

<xs:documentation>Message that is provided for situations where the user equipment may not immediately decode new satellite ephemerides in the satellite data.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GPS Reference Station Parameters">

<xs:annotation>

<xs:documentation>Message that contains reference station information. It includes GPS coordinates (Earth-Centered-Earth-Fixed (ECEF)) of the reference station antenna to the nearest centimetre.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Reference Station Datum">

<xs:annotation>

<xs:documentation>Message that contains the reference station datum information. It is designed to inform user equipment of the datum used for the reference station coordinates in Message Type 3 or 32 associated with the corrections being broadcast by the Reference Station.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GPS Constellation Health">

<xs:annotation>

<xs:documentation>Message that provides information that can assist in the operation of differential GPS user equipment. It is a mechanism through which the observations made at the reference station can be automatically used by the user equipment to improve performance without operator intervention.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GPS Null Frame">

<xs:annotation>

<xs:documentation>Message that contains no parameters and will be used as transmission fill, if required. Its purpose is to provide messages when the GPS Reference Station has no other message ready to send, or to synchronize the beginning of a message to come unspecified epoch.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="DGPS Radio Beacon Almanac">

<xs:annotation>

<xs:documentation>Message that sends the radiobeacon almanac, which provides the location, frequency, service range, and health information for a network of marine radiobeacons equipped to transmit differential GPS data. It also provides the identification of the broadcast station.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Pseudolite Almanac">

<xs:annotation>

<xs:documentation>Message that sends the pseudolite almanac. It is similar to the beacon almanac message (Message Type 7).</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GPS Partial Correction Set">

<xs:annotation>

<xs:documentation>Message that services the same purpose as the Type 1 Message, in that it contains the primary differential GPS corrections. However, unlike Type 1&#39;s, this message does not require a complete satellite set.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="P-Code Differential Corrections">

<xs:annotation>

<xs:documentation>This message has been reserved for P-code users, who will want differential corrections for both L1 and L2 frequencies.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="C/A-Code L1 L2 Delta Corrections">

<xs:annotation>

<xs:documentation>This message has been reserved for C/A-code L2 corrections in case future GPS satellites transmit the C/A code on the L2 frequency.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Pseudolite Station Parameters">

<xs:annotation>

<xs:documentation>This message has been reserved for a Pseudolite Station Parameter message which will provide clock offset parameters and the location of the phase center of the pseudolite transmitter antenna.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Ground Transmitter Parameters">

<xs:annotation>

<xs:documentation>Message that identifies the location and estimated range of the data link transmitter transmitting the differential corrections.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GPS Time of Week">

<xs:annotation>

<xs:documentation>Message that is a special time tag message intended to supplement the time information provided by the modified Z-count available in the message header.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Ionospheric Delay Message">

<xs:annotation>

<xs:documentation>Message that is designed to provide ionospheric delay and delay rate of change along the line of sight vector to different satellites.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GPS Special Message">

<xs:annotation>

<xs:documentation>Message that is a special GPS ASCII message that can be displayed on a printer, CRT, LCD or other graphic display.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GPS Ephemerides">

<xs:annotation>

<xs:documentation>Message that contains GPS satellite ephemeris information.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="RTK Uncorrected Carrier Phases">

<xs:annotation>

<xs:documentation>Message that gives Real-Time Kinematic (RTK) measurements for uncorrected carrier phases.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="RTK Uncorrected Pseudo Ranges">

<xs:annotation>

<xs:documentation>Message that gives Real-Time Kinematic (RTK) measurements for uncorrected pseudoranges.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="RTK Carrier Phase Corrections">

<xs:annotation>

<xs:documentation>Message that gives Real-Time Kinematic (RTK) carrier phase corrections.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="RTK/Hi-Accuracy Pseudorange Corrections">

<xs:annotation>

<xs:documentation>Message that gives Real-Time Kinematic (RTK) / High accuracy pseudorange corrections.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Extended Reference Station Parameters">

<xs:annotation>

<xs:documentation>Message that provides a means of achieving sub-millimetre precision for base station coordinates in a kinematic application and base station antenna height above a monument, which enables mobile units to reference measured position to the monument directly in real time.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Antenna Type Definition Record">

<xs:annotation>

<xs:documentation>Message the provides the information on the antenna type used on the reference station.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Antenna Reference Point (ARP)">

<xs:annotation>

<xs:documentation>Message that has been introduced to replace messages 3 and 22 for Real-Time Kinematic (RTK) operations.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Extended Radio Beacon Almanac">

<xs:annotation>

<xs:documentation>This message provides the ID of the reference stations.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Differential GLONASS Corrections">

<xs:annotation>

<xs:documentation>This is the primary message type which provides the pseudorange correction for any user receiver GLONASS measurement time.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Differential GLONASS Reference Station Parameters">

<xs:annotation>

<xs:documentation>This message contains GLONASS reference station information.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GLONASS Constellation Health">

<xs:annotation>

<xs:documentation>This message provides information that can assist in the operation of differential user equipment. It is a mechanism through which the observations made at the reference station can be automatically used by the user equipment to improve performance without operator intervention.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GLONASS Partial Differential Correction Set">

<xs:annotation>

<xs:documentation>Message that services the same purpose as the Type 31 Message, in that it contains the primary differential GLONASS corrections. However, unlike Type 31&#39;s, this message do not require a complete satellite set.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GLONASS Radio Beacon Almanac">

<xs:annotation>

<xs:documentation>Message that sends the radiobeacon almanac, which provides the location, frequency, service range, and health information for a network of marine radiobeacons equipped to transmit differential GLONASS data. It also provides the identification of the broadcast station.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GLONASS Special Message">

<xs:annotation>

<xs:documentation>Message that is a special GLONASS ASCII message that can be displayed on a printer, CRT, LCD or other graphic display.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="GNSS System Time Offset">

<xs:annotation>

<xs:documentation>This message is used to relate the differential corrections from different GNSS&#39;s to each other.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Proprietary Message">

<xs:annotation>

<xs:documentation>This message is reserved for private use by operators who wish to use their data links to communicate proprietary information to their users.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Multipurpose Usage">

<xs:annotation>

<xs:documentation>This message is reserved at least temporarily, for use in exploring the use of multi-purpose data links.</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="radioBeaconHealthType">

<xs:restriction base="xs:string">

<xs:enumeration value="Radio Beacon Operational Normal">

<xs:annotation>

<xs:documentation>The operation of DGNSS station is normal.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="No Integrity Monitor Operating">

<xs:annotation>

<xs:documentation>The integrity monitoring is not operated.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="No Information Available">

<xs:annotation>

<xs:documentation>Information about DGNSS station is not available.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Do Not Use This Radio Beacon">

<xs:annotation>

<xs:documentation>Do not use any information from DGNSS station.</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="statusType">

<xs:restriction base="xs:string">

<xs:enumeration value="Permanent">

<xs:annotation>

<xs:documentation>Intended to last or function indefinitely.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Occasional">

<xs:annotation>

<xs:documentation>Acting on special occasions; happening irregularly.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Not in Use">

<xs:annotation>

<xs:documentation>Use has ceased, but the facility still exists intact; disused.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Periodic/Intermittent">

<xs:annotation>

<xs:documentation>Recurring at intervals.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Temporary">

<xs:annotation>

<xs:documentation>Meant to last only for a time.</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Private">

<xs:annotation>

<xs:documentation>Administered by an individual or corporation, rather than a State or a public body.</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="categoryOfRadioStationType">

<xs:restriction base="xs:string">

<xs:enumeration value="Differential GNSS">

<xs:annotation>

<xs:documentation>Differential GNSS is implemented by placing a GNSS monitor receiver at a precisely known location. Instead of computing a navigation fix, the monitor determines the range error to every GNSS satellite it can track. These ranging errors are then transmitted to local users where they are applied as corrections before computing the navigation result.</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<!-- ========================================================================================== -->

<!-- types and elements for the complex attribute types definition                              -->

<!-- ========================================================================================== -->

<xs:complexType name="featureNameType">

<xs:annotation>

<xs:documentation>Provides the name of an entity, defines the national language of the name, and provides the option to display the name at various system display settings.</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="displayName" type="xs:boolean" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>A statement expressing if a feature name is to be displayed in certain system display settings or not.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="language" type="xs:string" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>The method of human communication, either spoken or written, consisting of the use of words in a structured and conventional way.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="name" type="xs:string" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>The individual name of a feature.</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="fixedDateRangeType">

<xs:annotation>

<xs:documentation>An active period of a single fixed event or occurrence, as the date range between discrete start and end dates.</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="dateEnd" type="S100_TruncatedDate" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>The latest date on which an object (for example a buoy) will be present.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="dateStart" type="S100_TruncatedDate" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>The earliest date on which an object (for example a buoy) will be present.</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="informationType">

<xs:annotation>

<xs:documentation>Textual information about the feature. The information may be provided as a string of text or as a file name of a single external text file that contains the text.</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="fileLocator" type="xs:string" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>The location of a fragment of text or other information in a support file.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="fileReference" type="xs:string" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>The file name of an externally referenced text file.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="headline" type="xs:string" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>Words set at the head of a passage or page to introduce or categorize.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="language" type="xs:string" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>The method of human communication, either spoken or written, consisting of the use of words in a structured and conventional way.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="text" type="xs:string" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>A non-formatted digital text string.</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="periodicDateRangeType">

<xs:annotation>

<xs:documentation>The active period of a recurring event or occurrence.</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="dateEnd" type="S100_TruncatedDate" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>The latest date on which an object (for example a buoy) will be present.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="dateStart" type="S100_TruncatedDate" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>The earliest date on which an object (for example a buoy) will be present.</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="textualDescriptionType">

<xs:annotation>

<xs:documentation>Encodes the file name of a single external text file that contains the text in a defined language, which provides additional textual information that cannot be provided using other allowable attributes for the feature.</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="fileReference" type="xs:string" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>The file name of an externally referenced text file.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="language" type="xs:string" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>The method of human communication, either spoken or written, consisting of the use of words in a structured and conventional way.</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<!-- ========================================================================================== -->

<!-- types and elements for the information types definition                                    -->

<!-- ========================================================================================== -->

<xs:element name="InformationType" type="InformationTypeType" abstract="true"/>

<xs:complexType name="InformationTypeType" abstract="true">

<xs:annotation>

<xs:documentation>Generalized information type which carry all the common attributes</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="S100:AbstractInformationType"/>

</xs:complexContent>

</xs:complexType>

<xs:element name="SupplementaryInformation" type="SupplementaryInformationType"/>

<xs:complexType name="SupplementaryInformationType">

<xs:annotation>

<xs:documentation>Additional information that is useful within the context.</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="InformationTypeType">

<xs:sequence>

<xs:element name="information" type="informationType" minOccurs="1" maxOccurs="unbounded">

<xs:annotation>

<xs:documentation>Textual information about the feature. The information may be provided as a string of text or as a file name of a single external text file that contains the text.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="textualDescription" type="textualDescriptionType" minOccurs="1" maxOccurs="unbounded">

<xs:annotation>

<xs:documentation>Encodes the file name of a single external text file that contains the text in a defined language, which provides additional textual information that cannot be provided using other allowable attributes for the feature.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="pictorialRepresentation" type="xs:string" minOccurs="0" maxOccurs="unbounded">

<xs:annotation>

<xs:documentation>Indicates whether a pictorial representation of the feature is available.</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:element name="DgnssStationAlmanac" type="DgnssStationAlmanacType"/>

<xs:complexType name="DgnssStationAlmanacType">

<xs:annotation>

<xs:documentation>Detailed information about a specific DGNSS station.</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="InformationTypeType">

<xs:sequence>

<xs:element name="bitRate" type="xs:integer" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>Bit rate of Medium frequency from DGNSS station.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="signalFrequency" type="xs:integer" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>The frequency of a signal.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="nominalRangeAt" type="xs:integer" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>Nominal range in microvolt per metre for the Medium frequency signal of DGNSS.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="nominalRangeKM" type="xs:integer" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>Nominal range in Km (kilometre) for the Medium frequency signal of DGNSS.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="radioBeaconHealth" type="radioBeaconHealthType" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>Operational status and integrity monitoring of DGNSS station.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="referenceStationID" type="xs:string" minOccurs="0" maxOccurs="2">

<xs:annotation>

<xs:documentation>Identification of reference station.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="stationName" type="xs:string" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>The name of the reference tide station with reference water level for tidal stream panel observations.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="transmittedMessageType" type="transmittedMessageTypeType" minOccurs="1" maxOccurs="unbounded">

<xs:annotation>

<xs:documentation>Transmitted message type from DGNSS station.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="transmittingStationID" type="xs:string" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>ID of transmitting station.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="stationRegion" type="gml:ReferenceType" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation/>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:element name="DgnssStationRegion" type="DgnssStationRegionType"/>

<xs:complexType name="DgnssStationRegionType">

<xs:annotation>

<xs:documentation>Approximate region where DGNSS station is located.</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="InformationTypeType">

<xs:sequence>

<xs:element name="dateOfIssue" type="xs:dateTime" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>First date item was reported to IALA.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="dateOfLastUpdate" type="xs:date" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation>Date when something was last updated.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="countryName" type="xs:string" minOccurs="1" maxOccurs="unbounded">

<xs:annotation>

<xs:documentation>The name of a nation.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="dgnssStationAlmanac" type="gml:ReferenceType" minOccurs="1" maxOccurs="unbounded">

<xs:annotation>

<xs:documentation/>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:element name="RadioStation" type="RadioStationType"/>

<xs:complexType name="RadioStationType">

<xs:annotation>

<xs:documentation>A place equipped to transmit radio waves. Such a station may be either stationary or mobile, and may also be provided with a radio receiver.</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="S100:AbstractFeatureType">

<xs:sequence>

<xs:element name="callSign" type="xs:string" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>The designated call-sign of a station (radio station, radar station, pilot, ...).</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="categoryOfRadioStation" type="categoryOfRadioStationType" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>Classification of radio services offered by a radio station.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="communicationChannel" type="xs:string" minOccurs="0" maxOccurs="unbounded">

<xs:annotation>

<xs:documentation>A channel number assigned to a specific radio frequency, frequencies or frequency band.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="fixedDateRange" type="fixedDateRangeType" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>An active period of a single fixed event or occurrence, as the date range between discrete start and end dates.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="periodicDateRange" type="periodicDateRangeType" minOccurs="0" maxOccurs="unbounded">

<xs:annotation>

<xs:documentation>The active period of a recurring event or occurrence.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="scaleMinimum" type="xs:integer" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>The minimum scale at which the feature may be used for example for ECDIS presentation.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="status" type="statusType" minOccurs="0" maxOccurs="unbounded">

<xs:annotation>

<xs:documentation>The condition of an object at a given instant in time.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="featureName" type="featureNameType" minOccurs="0" maxOccurs="unbounded">

<xs:annotation>

<xs:documentation>Provides the name of an entity, defines the national language of the name, and provides the option to display the name at various system display settings.</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="stationAlmanac" type="gml:ReferenceType" minOccurs="1" maxOccurs="1">

<xs:annotation>

<xs:documentation/>

</xs:annotation>

</xs:element>

<xs:element name="providedBy" type="gml:ReferenceType" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation/>

</xs:annotation>

</xs:element>

<xs:element name="geometry" maxOccurs="unbounded">

<xs:complexType>

<xs:choice>

<xs:element ref="S100:pointProperty"/>

</xs:choice>

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:group name="MemberObjects">

<xs:annotation>

<xs:documentation>group of informtion, feature and meta objects, all GML members</xs:documentation>

</xs:annotation>

<xs:choice>

<xs:element name="SupplementaryInformation"/>

<xs:element name="DgnssStationAlmanac"/>

<xs:element name="DgnssStationRegion"/>

<xs:element name="RadioStation"/>

</xs:choice>

</xs:group>

<xs:complexType name="ThisDatasetType">

<xs:complexContent>

<xs:extension base="S100:DatasetType">

<xs:sequence>

<xs:element name="members">

<xs:complexType>

<xs:complexContent>

<xs:extension base="gml:AbstractFeatureMemberType">

<xs:sequence>

<xs:group ref="MemberObjects" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:element name="Dataset" type="ThisDatasetType"/>

</xs:schema>

# ANNEX D S-240 Feature Catalogue

<?xml version="1.0" encoding="utf-8"?>

<S100FC:S100_FC_FeatureCatalogue xmlns:S100FC="http://www.iho.int/S100FC/5.0" xmlns:S100Base="http://www.iho.int/S100Base/5.0" xmlns:S100CI="http://www.iho.int/S100CI/5.0" xmlns:xlink="http://www.w3.org/1999/xlink" xmlns:S100CD="http://www.iho.int/S100CD/5.0" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="http://www.iho.int/S100FC/5.0 https://schemas.s100dev.net/schemas/S100/5.0.0/S100FC/20220610/S100FC.xsd">

<S100FC:name>DGNSS Station Almanac</S100FC:name>

<S100FC:scope>-</S100FC:scope>

<S100FC:fieldOfApplication>-</S100FC:fieldOfApplication>

<S100FC:versionNumber>1.2.0</S100FC:versionNumber>

<S100FC:versionDate>2024-03-29</S100FC:versionDate>

<S100FC:productId>S-240</S100FC:productId>

<S100FC:producer>

<S100CI:role S100CI:nilReason="unknown">pointOfContact</S100CI:role>

<S100CI:party>

<S100CI:CI_Organisation>

<S100CI:name>IALA</S100CI:name>

<S100CI:contactInfo>

</S100CI:contactInfo>

</S100CI:CI_Organisation>

</S100CI:party>

</S100FC:producer>

<S100FC:classification>unclassified</S100FC:classification>

<S100FC:S100_FC_DefinitionSources>

<S100FC:FC_DefinitionSource id="IHOREG">

<S100FC:source>

<S100CI:title>IHO GI Registry</S100CI:title>

<S100CI:onlineResource>

<S100CI:linkage>http://registry.iho.int</S100CI:linkage>

</S100CI:onlineResource>

</S100FC:source>

</S100FC:FC_DefinitionSource>

</S100FC:S100_FC_DefinitionSources>

<S100FC:S100_FC_SimpleAttributes>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Text</S100FC:name>

<S100FC:definition>A non-formatted digital text string.</S100FC:definition>

<S100FC:code>text</S100FC:code>

<S100FC:remarks>Should be used, for example, to hold the information that is for short cautionary or explanatory notes. Therefore, text populated in text must not exceed 300 characters. Text may be in English, or in a national language. No formatting of text is possible within text. If formatted text, or text strings exceeding 300 characters, is required, then an alternate concept should be used.</S100FC:remarks>

<S100FC:alias>INFORM</S100FC:alias>

<S100FC:alias>NINFOM</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>176</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Headline</S100FC:name>

<S100FC:definition>Words set at the head of a passage or page to introduce or categorize.</S100FC:definition>

<S100FC:code>headline</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>108</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>File Reference</S100FC:name>

<S100FC:definition>The file name of an externally referenced text file.</S100FC:definition>

<S100FC:code>fileReference</S100FC:code>

<S100FC:alias>TXTDSC</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>102</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>File Locator</S100FC:name>

<S100FC:definition>The location of a fragment of text or other information in a support file.</S100FC:definition>

<S100FC:code>fileLocator</S100FC:code>

<S100FC:remarks>Application schemas must describe how the associated file is identified. The associated file will commonly be named in a file reference co-attribute of the same complex attribute. Each DCEG must specify requirements for the format of the associated file and the semantics of file locator. For example, the value of file locator may be an HTML ID in an HTML file, line number in a text file) or a bookmark in a PDF file.</S100FC:remarks>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>101</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Date Start</S100FC:name>

<S100FC:definition>The earliest date on which an object (for example a buoy) will be present.</S100FC:definition>

<S100FC:code>dateStart</S100FC:code>

<S100FC:remarks>The Date Start should be encoded using 4 digits for the calendar year (YYYY), 2 digits for the month (MM) (for example April = 04) and 2 digits for the day (DD). When no specific month and/or day is required/known, indication of the month and/or day is omitted, and replaced with dashes (-). When no specific year is required (that is, the event or date range ends at the same time each year) the following two cases may be considered:- same day each year: ----MMDD- same month each year: ----MM--This conforms to ISO 8601: 2004. Date Start indicates the earliest date of an event or the start of a date range. It is used to indicate the start of a fixed date range, the start of a periodic date range, or the deployment or implementation of a feature at a specific date in the future.</S100FC:remarks>

<S100FC:alias>DATSTA</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>792</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>S100_TruncatedDate</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Date End</S100FC:name>

<S100FC:definition>The latest date on which an object (for example a buoy) will be present.</S100FC:definition>

<S100FC:code>dateEnd</S100FC:code>

<S100FC:remarks>The Date End should be encoded using 4 digits for the calendar year (YYYY), 2 digits for the month (MM) (for example April = 04) and 2 digits for the day (DD). When no specific month and/or day is required/known, indication of the month and/or day is omitted, and replaced with dashes (-). When no specific year is required (that is, the event or date range ends at the same time each year) the following two cases may be considered:- same day each year: ----MMDD- same month each year: ----MM--This conforms to ISO 8601: 2004. Date End indicates the latest date of an event or the end of a date range. It is used to indicate the end of a fixed date range, the end of a periodic date range, or the removal or cancellation of a feature at a specific date in the future.</S100FC:remarks>

<S100FC:alias>DATEND</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>790</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>S100_TruncatedDate</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Name</S100FC:name>

<S100FC:definition>The individual name of a feature.</S100FC:definition>

<S100FC:code>name</S100FC:code>

<S100FC:alias>OBJNAM</S100FC:alias>

<S100FC:alias>NOBJNM</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>134</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Language</S100FC:name>

<S100FC:definition>The method of human communication, either spoken or written, consisting of the use of words in a structured and conventional way.</S100FC:definition>

<S100FC:code>language</S100FC:code>

<S100FC:remarks>The language is encoded by a 3 character code following ISO 639-2/T.</S100FC:remarks>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>120</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Display Name</S100FC:name>

<S100FC:definition>A statement expressing if a feature name is to be displayed in certain system display settings or not.</S100FC:definition>

<S100FC:code>displayName</S100FC:code>

<S100FC:remarks>Where it is allowable to encode multiple instances of feature name for a single feature instance, only one feature name instance can indicate that the name is to be displayed (display name set to True).</S100FC:remarks>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>94</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>boolean</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Country Name</S100FC:name>

<S100FC:definition>The name of a nation.</S100FC:definition>

<S100FC:code>countryName</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>449</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Date of Last Update</S100FC:name>

<S100FC:definition>Date when something was last updated.</S100FC:definition>

<S100FC:code>dateOfLastUpdate</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>455</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>date</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Date of Issue</S100FC:name>

<S100FC:definition>First date item was reported to IALA.</S100FC:definition>

<S100FC:code>dateOfIssue</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>454</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>dateTime</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Transmitting Station ID</S100FC:name>

<S100FC:definition>ID of transmitting station.</S100FC:definition>

<S100FC:code>transmittingStationID</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>688</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Transmitted Message Type</S100FC:name>

<S100FC:definition>Transmitted message type from DGNSS station.</S100FC:definition>

<S100FC:code>transmittedMessageType</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>686</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>enumeration</S100FC:valueType>

<S100FC:listedValues>

<S100FC:listedValue>

<S100FC:label>Differential GPS Corrections</S100FC:label>

<S100FC:definition>Primary message type which provides the differentially corrected pseudorange measurement that should be processed by the User Equipment navigation filter.</S100FC:definition>

<S100FC:code>1</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2598</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Delta Differential GPS Corrections</S100FC:label>

<S100FC:definition>Message that is provided for situations where the user equipment may not immediately decode new satellite ephemerides in the satellite data.</S100FC:definition>

<S100FC:code>2</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2599</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GPS Reference Station Parameters</S100FC:label>

<S100FC:definition>Message that contains reference station information. It includes GPS coordinates (Earth-Centered-Earth-Fixed (ECEF)) of the reference station antenna to the nearest centimetre.</S100FC:definition>

<S100FC:code>3</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2600</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Reference Station Datum</S100FC:label>

<S100FC:definition>Message that contains the reference station datum information. It is designed to inform user equipment of the datum used for the reference station coordinates in Message Type 3 or 32 associated with the corrections being broadcast by the Reference Station.</S100FC:definition>

<S100FC:code>4</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2601</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GPS Constellation Health</S100FC:label>

<S100FC:definition>Message that provides information that can assist in the operation of differential GPS user equipment. It is a mechanism through which the observations made at the reference station can be automatically used by the user equipment to improve performance without operator intervention.</S100FC:definition>

<S100FC:code>5</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2602</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GPS Null Frame</S100FC:label>

<S100FC:definition>Message that contains no parameters and will be used as transmission fill, if required. Its purpose is to provide messages when the GPS Reference Station has no other message ready to send, or to synchronize the beginning of a message to come unspecified epoch.</S100FC:definition>

<S100FC:code>6</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2603</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>DGPS Radio Beacon Almanac</S100FC:label>

<S100FC:definition>Message that sends the radiobeacon almanac, which provides the location, frequency, service range, and health information for a network of marine radiobeacons equipped to transmit differential GPS data. It also provides the identification of the broadcast station.</S100FC:definition>

<S100FC:code>7</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2604</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Pseudolite Almanac</S100FC:label>

<S100FC:definition>Message that sends the pseudolite almanac. It is similar to the beacon almanac message (Message Type 7).</S100FC:definition>

<S100FC:code>8</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2605</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GPS Partial Correction Set</S100FC:label>

<S100FC:definition>Message that services the same purpose as the Type 1 Message, in that it contains the primary differential GPS corrections. However, unlike Type 1&apos;s, this message does not require a complete satellite set.</S100FC:definition>

<S100FC:code>9</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2606</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>P-Code Differential Corrections</S100FC:label>

<S100FC:definition>This message has been reserved for P-code users, who will want differential corrections for both L1 and L2 frequencies.</S100FC:definition>

<S100FC:code>10</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2607</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>C/A-Code L1 L2 Delta Corrections</S100FC:label>

<S100FC:definition>This message has been reserved for C/A-code L2 corrections in case future GPS satellites transmit the C/A code on the L2 frequency.</S100FC:definition>

<S100FC:code>11</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2608</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Pseudolite Station Parameters</S100FC:label>

<S100FC:definition>This message has been reserved for a Pseudolite Station Parameter message which will provide clock offset parameters and the location of the phase center of the pseudolite transmitter antenna.</S100FC:definition>

<S100FC:code>12</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2609</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Ground Transmitter Parameters</S100FC:label>

<S100FC:definition>Message that identifies the location and estimated range of the data link transmitter transmitting the differential corrections.</S100FC:definition>

<S100FC:code>13</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2610</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GPS Time of Week</S100FC:label>

<S100FC:definition>Message that is a special time tag message intended to supplement the time information provided by the modified Z-count available in the message header.</S100FC:definition>

<S100FC:code>14</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2611</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Ionospheric Delay Message</S100FC:label>

<S100FC:definition>Message that is designed to provide ionospheric delay and delay rate of change along the line of sight vector to different satellites.</S100FC:definition>

<S100FC:code>15</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2612</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GPS Special Message</S100FC:label>

<S100FC:definition>Message that is a special GPS ASCII message that can be displayed on a printer, CRT, LCD or other graphic display.</S100FC:definition>

<S100FC:code>16</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2613</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GPS Ephemerides</S100FC:label>

<S100FC:definition>Message that contains GPS satellite ephemeris information.</S100FC:definition>

<S100FC:code>17</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2614</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>RTK Uncorrected Carrier Phases</S100FC:label>

<S100FC:definition>Message that gives Real-Time Kinematic (RTK) measurements for uncorrected carrier phases.</S100FC:definition>

<S100FC:code>18</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2615</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>RTK Uncorrected Pseudo Ranges</S100FC:label>

<S100FC:definition>Message that gives Real-Time Kinematic (RTK) measurements for uncorrected pseudoranges.</S100FC:definition>

<S100FC:code>19</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2616</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>RTK Carrier Phase Corrections</S100FC:label>

<S100FC:definition>Message that gives Real-Time Kinematic (RTK) carrier phase corrections.</S100FC:definition>

<S100FC:code>20</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2617</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>RTK/Hi-Accuracy Pseudorange Corrections</S100FC:label>

<S100FC:definition>Message that gives Real-Time Kinematic (RTK) / High accuracy pseudorange corrections.</S100FC:definition>

<S100FC:code>21</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2618</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Extended Reference Station Parameters</S100FC:label>

<S100FC:definition>Message that provides a means of achieving sub-millimetre precision for base station coordinates in a kinematic application and base station antenna height above a monument, which enables mobile units to reference measured position to the monument directly in real time.</S100FC:definition>

<S100FC:code>22</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2619</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Antenna Type Definition Record</S100FC:label>

<S100FC:definition>Message the provides the information on the antenna type used on the reference station.</S100FC:definition>

<S100FC:code>23</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2620</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Antenna Reference Point (ARP)</S100FC:label>

<S100FC:definition>Message that has been introduced to replace messages 3 and 22 for Real-Time Kinematic (RTK) operations.</S100FC:definition>

<S100FC:code>24</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2621</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Extended Radio Beacon Almanac</S100FC:label>

<S100FC:definition>This message provides the ID of the reference stations.</S100FC:definition>

<S100FC:code>27</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2622</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Differential GLONASS Corrections</S100FC:label>

<S100FC:definition>This is the primary message type which provides the pseudorange correction for any user receiver GLONASS measurement time.</S100FC:definition>

<S100FC:code>31</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2623</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Differential GLONASS Reference Station Parameters</S100FC:label>

<S100FC:definition>This message contains GLONASS reference station information.</S100FC:definition>

<S100FC:code>32</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2624</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GLONASS Constellation Health</S100FC:label>

<S100FC:definition>This message provides information that can assist in the operation of differential user equipment. It is a mechanism through which the observations made at the reference station can be automatically used by the user equipment to improve performance without operator intervention.</S100FC:definition>

<S100FC:code>33</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2625</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GLONASS Partial Differential Correction Set</S100FC:label>

<S100FC:definition>Message that services the same purpose as the Type 31 Message, in that it contains the primary differential GLONASS corrections. However, unlike Type 31&apos;s, this message do not require a complete satellite set.</S100FC:definition>

<S100FC:code>34</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2626</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GLONASS Radio Beacon Almanac</S100FC:label>

<S100FC:definition>Message that sends the radiobeacon almanac, which provides the location, frequency, service range, and health information for a network of marine radiobeacons equipped to transmit differential GLONASS data. It also provides the identification of the broadcast station.</S100FC:definition>

<S100FC:code>35</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2627</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GLONASS Special Message</S100FC:label>

<S100FC:definition>Message that is a special GLONASS ASCII message that can be displayed on a printer, CRT, LCD or other graphic display.</S100FC:definition>

<S100FC:code>36</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2628</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>GNSS System Time Offset</S100FC:label>

<S100FC:definition>This message is used to relate the differential corrections from different GNSS&apos;s to each other.</S100FC:definition>

<S100FC:code>37</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2629</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Proprietary Message</S100FC:label>

<S100FC:definition>This message is reserved for private use by operators who wish to use their data links to communicate proprietary information to their users.</S100FC:definition>

<S100FC:code>59</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2630</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Multipurpose Usage</S100FC:label>

<S100FC:definition>This message is reserved at least temporarily, for use in exploring the use of multi-purpose data links.</S100FC:definition>

<S100FC:code>60</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2631</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

</S100FC:listedValues>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Station Name</S100FC:name>

<S100FC:definition>The name of the reference tide station with reference water level for tidal stream panel observations.</S100FC:definition>

<S100FC:code>stationName</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>168</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Reference Station ID</S100FC:name>

<S100FC:definition>Identification of reference station.</S100FC:definition>

<S100FC:code>referenceStationID</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>614</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Radio Beacon Health</S100FC:name>

<S100FC:definition>Operational status and integrity monitoring of DGNSS station.</S100FC:definition>

<S100FC:code>radioBeaconHealth</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>610</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>enumeration</S100FC:valueType>

<S100FC:listedValues>

<S100FC:listedValue>

<S100FC:label>Radio Beacon Operational Normal</S100FC:label>

<S100FC:definition>The operation of DGNSS station is normal.</S100FC:definition>

<S100FC:code>1</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2536</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>No Integrity Monitor Operating</S100FC:label>

<S100FC:definition>The integrity monitoring is not operated.</S100FC:definition>

<S100FC:code>2</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2537</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>No Information Available</S100FC:label>

<S100FC:definition>Information about DGNSS station is not available.</S100FC:definition>

<S100FC:code>3</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2538</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Do Not Use This Radio Beacon</S100FC:label>

<S100FC:definition>Do not use any information from DGNSS station.</S100FC:definition>

<S100FC:code>4</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>2539</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

</S100FC:listedValues>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Nominal Range KM</S100FC:name>

<S100FC:definition>Nominal range in Km (kilometre) for the Medium frequency signal of DGNSS.</S100FC:definition>

<S100FC:code>nominalRangeKM</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>950</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>integer</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Nominal Range At</S100FC:name>

<S100FC:definition>Nominal range in microvolt per metre for the Medium frequency signal of DGNSS.</S100FC:definition>

<S100FC:code>nominalRangeAt</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>949</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>integer</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Signal Frequency</S100FC:name>

<S100FC:definition>The frequency of a signal.</S100FC:definition>

<S100FC:code>signalFrequency</S100FC:code>

<S100FC:alias>SIGFRQ</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>1105</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>integer</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Bit Rate</S100FC:name>

<S100FC:definition>Bit rate of Medium frequency from DGNSS station.</S100FC:definition>

<S100FC:code>bitRate</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>916</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>integer</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Pictorial Representation</S100FC:name>

<S100FC:definition>Indicates whether a pictorial representation of the feature is available.</S100FC:definition>

<S100FC:code>pictorialRepresentation</S100FC:code>

<S100FC:remarks>The &apos;pictorial representation&apos; could be a drawing or a photo. The string encodes the file name of an external graphic file (pixel/vector).</S100FC:remarks>

<S100FC:alias>PICREP</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>142</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Status</S100FC:name>

<S100FC:definition>The condition of an object at a given instant in time.</S100FC:definition>

<S100FC:code>status</S100FC:code>

<S100FC:alias>STATUS</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>725</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>enumeration</S100FC:valueType>

<S100FC:listedValues>

<S100FC:listedValue>

<S100FC:label>Permanent</S100FC:label>

<S100FC:definition>Intended to last or function indefinitely.</S100FC:definition>

<S100FC:code>1</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>1052</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Occasional</S100FC:label>

<S100FC:definition>Acting on special occasions; happening irregularly.</S100FC:definition>

<S100FC:code>2</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>1053</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Not in Use</S100FC:label>

<S100FC:definition>Use has ceased, but the facility still exists intact; disused.</S100FC:definition>

<S100FC:code>4</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>1055</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Periodic/Intermittent</S100FC:label>

<S100FC:definition>Recurring at intervals.</S100FC:definition>

<S100FC:code>5</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>1056</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Temporary</S100FC:label>

<S100FC:definition>Meant to last only for a time.</S100FC:definition>

<S100FC:code>7</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>1058</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

<S100FC:listedValue>

<S100FC:label>Private</S100FC:label>

<S100FC:definition>Administered by an individual or corporation, rather than a State or a public body.</S100FC:definition>

<S100FC:code>8</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>1059</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

</S100FC:listedValues>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Scale Minimum</S100FC:name>

<S100FC:definition>The minimum scale at which the feature may be used for example for ECDIS presentation.</S100FC:definition>

<S100FC:code>scaleMinimum</S100FC:code>

<S100FC:remarks>The modulus of the scale is indicated, that is 1:1 250 000 is encoded as 1250000.</S100FC:remarks>

<S100FC:alias>SCAMIN</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>958</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>integer</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Communication Channel</S100FC:name>

<S100FC:definition>A channel number assigned to a specific radio frequency, frequencies or frequency band.</S100FC:definition>

<S100FC:code>communicationChannel</S100FC:code>

<S100FC:remarks>The expected input is the specific VHF-Channel. The attribute &apos;communication channel&apos; encodes the various VHF-channels used for communication.</S100FC:remarks>

<S100FC:alias>COMCHA</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>74</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Category of Radio Station</S100FC:name>

<S100FC:definition>Classification of radio services offered by a radio station.</S100FC:definition>

<S100FC:code>categoryOfRadioStation</S100FC:code>

<S100FC:remarks>A radiobeacon is a radio transmitter which emits a distinctive or characteristic signal on which a bearing may be taken.</S100FC:remarks>

<S100FC:alias>CATROS</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>52</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>enumeration</S100FC:valueType>

<S100FC:listedValues>

<S100FC:listedValue>

<S100FC:label>Differential GNSS</S100FC:label>

<S100FC:definition>Differential GNSS is implemented by placing a GNSS monitor receiver at a precisely known location. Instead of computing a navigation fix, the monitor determines the range error to every GNSS satellite it can track. These ranging errors are then transmitted to local users where they are applied as corrections before computing the navigation result.</S100FC:definition>

<S100FC:code>10</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>385</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:listedValue>

</S100FC:listedValues>

</S100FC:S100_FC_SimpleAttribute>

<S100FC:S100_FC_SimpleAttribute>

<S100FC:name>Call Sign</S100FC:name>

<S100FC:definition>The designated call-sign of a station (radio station, radar station, pilot, ...).</S100FC:definition>

<S100FC:code>callSign</S100FC:code>

<S100FC:alias>CALSGN</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>271</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:valueType>text</S100FC:valueType>

</S100FC:S100_FC_SimpleAttribute>

</S100FC:S100_FC_SimpleAttributes>

<S100FC:S100_FC_ComplexAttributes>

<S100FC:S100_FC_ComplexAttribute>

<S100FC:name>Feature Name</S100FC:name>

<S100FC:definition>Provides the name of an entity, defines the national language of the name, and provides the option to display the name at various system display settings.</S100FC:definition>

<S100FC:code>featureName</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>1129</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:subAttributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="displayName" />

</S100FC:subAttributeBinding>

<S100FC:subAttributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="language" />

</S100FC:subAttributeBinding>

<S100FC:subAttributeBinding sequential="true">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="name" />

</S100FC:subAttributeBinding>

</S100FC:S100_FC_ComplexAttribute>

<S100FC:S100_FC_ComplexAttribute>

<S100FC:name>Fixed Date Range</S100FC:name>

<S100FC:definition>An active period of a single fixed event or occurrence, as the date range between discrete start and end dates.</S100FC:definition>

<S100FC:code>fixedDateRange</S100FC:code>

<S100FC:remarks>Dates must be encoded in the format YYYYMMDD; using 4 digits for the calendar year (YYYY) and, optionally, 2 digits for the month (MM) (for example April = 04) and 2 digits for the day (DD). When no specific month and/or day is required/known, the values are replaced with dashes (-). The date range of a recurring event or occurrence must be encoded using periodicDateRange.</S100FC:remarks>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>798</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:subAttributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="dateEnd" />

</S100FC:subAttributeBinding>

<S100FC:subAttributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="dateStart" />

</S100FC:subAttributeBinding>

</S100FC:S100_FC_ComplexAttribute>

<S100FC:S100_FC_ComplexAttribute>

<S100FC:name>Information</S100FC:name>

<S100FC:definition>Textual information about the feature. The information may be provided as a string of text or as a file name of a single external text file that contains the text.</S100FC:definition>

<S100FC:code>information</S100FC:code>

<S100FC:remarks>At least one of the sub-attributes file reference or text must be populated.The sub-attribute file reference is generally used for long text strings or those that require formatting, however, there is no restriction on the type of text (except for lexical level) that can be held in files referenced by sub-attribute file reference.</S100FC:remarks>

<S100FC:alias>INFORM</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>1130</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:subAttributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="fileLocator" />

</S100FC:subAttributeBinding>

<S100FC:subAttributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="fileReference" />

</S100FC:subAttributeBinding>

<S100FC:subAttributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="headline" />

</S100FC:subAttributeBinding>

<S100FC:subAttributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="language" />

</S100FC:subAttributeBinding>

<S100FC:subAttributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="text" />

</S100FC:subAttributeBinding>

</S100FC:S100_FC_ComplexAttribute>

<S100FC:S100_FC_ComplexAttribute>

<S100FC:name>Periodic Date Range</S100FC:name>

<S100FC:definition>The active period of a recurring event or occurrence.</S100FC:definition>

<S100FC:code>periodicDateRange</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>794</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:subAttributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="dateEnd" />

</S100FC:subAttributeBinding>

<S100FC:subAttributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="dateStart" />

</S100FC:subAttributeBinding>

</S100FC:S100_FC_ComplexAttribute>

<S100FC:S100_FC_ComplexAttribute>

<S100FC:name>Textual Description</S100FC:name>

<S100FC:definition>Encodes the file name of a single external text file that contains the text in a defined language, which provides additional textual information that cannot be provided using other allowable attributes for the feature.</S100FC:definition>

<S100FC:code>textualDescription</S100FC:code>

<S100FC:alias>TXTDSC</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>294</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:subAttributeBinding sequential="true">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="fileReference" />

</S100FC:subAttributeBinding>

<S100FC:subAttributeBinding sequential="true">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="language" />

</S100FC:subAttributeBinding>

</S100FC:S100_FC_ComplexAttribute>

</S100FC:S100_FC_ComplexAttributes>

<S100FC:S100_FC_Roles>

<S100FC:S100_FC_Role>

<S100FC:name>Provided by</S100FC:name>

<S100FC:definition>a pointer to a specific feature(s) for which further information is required.</S100FC:definition>

<S100FC:code>providedBy</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>4</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:S100_FC_Role>

<S100FC:S100_FC_Role>

<S100FC:name>Provides</S100FC:name>

<S100FC:definition>a pointer to a specific feature(s) for which further information is required.</S100FC:definition>

<S100FC:code>provides</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>5</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

</S100FC:S100_FC_Role>

<S100FC:S100_FC_Role>

<S100FC:name>Station Region</S100FC:name>

<S100FC:definition>-</S100FC:definition>

<S100FC:code>stationRegion</S100FC:code>

<S100FC:remarks>-</S100FC:remarks>

</S100FC:S100_FC_Role>

<S100FC:S100_FC_Role>

<S100FC:name>DGNSS Station Almanac</S100FC:name>

<S100FC:definition>-</S100FC:definition>

<S100FC:code>dgnssStationAlmanac</S100FC:code>

<S100FC:remarks>-</S100FC:remarks>

</S100FC:S100_FC_Role>

<S100FC:S100_FC_Role>

<S100FC:name>Station Almanac</S100FC:name>

<S100FC:definition>-</S100FC:definition>

<S100FC:code>stationAlmanac</S100FC:code>

<S100FC:remarks>-</S100FC:remarks>

</S100FC:S100_FC_Role>

<S100FC:S100_FC_Role>

<S100FC:name>Radio Station</S100FC:name>

<S100FC:definition>-</S100FC:definition>

<S100FC:code>radioStation</S100FC:code>

<S100FC:remarks>-</S100FC:remarks>

</S100FC:S100_FC_Role>

</S100FC:S100_FC_Roles>

<S100FC:S100_FC_InformationAssociations>

<S100FC:S100_FC_InformationAssociation isAbstract="false">

<S100FC:name>Group of DGNSS Station</S100FC:name>

<S100FC:definition>-</S100FC:definition>

<S100FC:code>GroupOfDgnssStation</S100FC:code>

<S100FC:remarks>-</S100FC:remarks>

<S100FC:role ref="stationRegion" />

<S100FC:role ref="dgnssStationAlmanac" />

</S100FC:S100_FC_InformationAssociation>

<S100FC:S100_FC_InformationAssociation isAbstract="false">

<S100FC:name>Details of DGNSS Station </S100FC:name>

<S100FC:definition>-</S100FC:definition>

<S100FC:code>DetailsOfDgnssStation</S100FC:code>

<S100FC:remarks>-</S100FC:remarks>

<S100FC:role ref="stationAlmanac" />

<S100FC:role ref="radioStation" />

</S100FC:S100_FC_InformationAssociation>

<S100FC:S100_FC_InformationAssociation isAbstract="false">

<S100FC:name>Supplement Information</S100FC:name>

<S100FC:definition>-</S100FC:definition>

<S100FC:code>SupplementInformation</S100FC:code>

<S100FC:remarks>-</S100FC:remarks>

<S100FC:role ref="provides" />

<S100FC:role ref="providedBy" />

</S100FC:S100_FC_InformationAssociation>

</S100FC:S100_FC_InformationAssociations>

<S100FC:S100_FC_InformationTypes>

<S100FC:S100_FC_InformationType isAbstract="false">

<S100FC:name>Supplementary Information</S100FC:name>

<S100FC:definition>Additional information that is useful within the context.</S100FC:definition>

<S100FC:code>SupplementaryInformation</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>50</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="true" infinite="true" />

</S100FC:multiplicity>

<S100FC:attribute ref="information" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="true" infinite="true" />

</S100FC:multiplicity>

<S100FC:attribute ref="textualDescription" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="true" infinite="true" />

</S100FC:multiplicity>

<S100FC:attribute ref="pictorialRepresentation" />

</S100FC:attributeBinding>

</S100FC:S100_FC_InformationType>

<S100FC:S100_FC_InformationType isAbstract="false">

<S100FC:name>DGNSS Station Almanac</S100FC:name>

<S100FC:definition>Detailed information about a specific DGNSS station.</S100FC:definition>

<S100FC:code>DgnssStationAlmanac</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>39</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="bitRate" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="signalFrequency" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="nominalRangeAt" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="nominalRangeKM" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:permittedValues>

<S100FC:value>1</S100FC:value>

<S100FC:value>2</S100FC:value>

<S100FC:value>3</S100FC:value>

<S100FC:value>4</S100FC:value>

</S100FC:permittedValues>

<S100FC:attribute ref="radioBeaconHealth" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">2</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="referenceStationID" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="stationName" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="true" infinite="true" />

</S100FC:multiplicity>

<S100FC:permittedValues>

<S100FC:value>1</S100FC:value>

<S100FC:value>2</S100FC:value>

<S100FC:value>3</S100FC:value>

<S100FC:value>4</S100FC:value>

<S100FC:value>5</S100FC:value>

<S100FC:value>6</S100FC:value>

<S100FC:value>7</S100FC:value>

<S100FC:value>8</S100FC:value>

<S100FC:value>9</S100FC:value>

<S100FC:value>10</S100FC:value>

<S100FC:value>11</S100FC:value>

<S100FC:value>12</S100FC:value>

<S100FC:value>13</S100FC:value>

<S100FC:value>14</S100FC:value>

<S100FC:value>15</S100FC:value>

<S100FC:value>16</S100FC:value>

<S100FC:value>17</S100FC:value>

<S100FC:value>18</S100FC:value>

<S100FC:value>19</S100FC:value>

<S100FC:value>20</S100FC:value>

<S100FC:value>21</S100FC:value>

<S100FC:value>22</S100FC:value>

<S100FC:value>23</S100FC:value>

<S100FC:value>24</S100FC:value>

<S100FC:value>27</S100FC:value>

<S100FC:value>31</S100FC:value>

<S100FC:value>32</S100FC:value>

<S100FC:value>33</S100FC:value>

<S100FC:value>34</S100FC:value>

<S100FC:value>35</S100FC:value>

<S100FC:value>36</S100FC:value>

<S100FC:value>37</S100FC:value>

<S100FC:value>59</S100FC:value>

<S100FC:value>60</S100FC:value>

</S100FC:permittedValues>

<S100FC:attribute ref="transmittedMessageType" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="transmittingStationID" />

</S100FC:attributeBinding>

<S100FC:informationBinding roleType="association">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:association ref="GroupOfDgnssStation" />

<S100FC:role ref="stationRegion" />

<S100FC:informationType ref="DgnssStationRegion" />

</S100FC:informationBinding>

</S100FC:S100_FC_InformationType>

<S100FC:S100_FC_InformationType isAbstract="false">

<S100FC:name>DGNSS Station Region</S100FC:name>

<S100FC:definition>Approximate region where DGNSS station is located.</S100FC:definition>

<S100FC:code>DgnssStationRegion</S100FC:code>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>40</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="dateOfIssue" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="dateOfLastUpdate" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="true" infinite="true" />

</S100FC:multiplicity>

<S100FC:attribute ref="countryName" />

</S100FC:attributeBinding>

<S100FC:informationBinding roleType="association">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="true" infinite="true" />

</S100FC:multiplicity>

<S100FC:association ref="GroupOfDgnssStation" />

<S100FC:role ref="dgnssStationAlmanac" />

<S100FC:informationType ref="DgnssStationAlmanac" />

</S100FC:informationBinding>

</S100FC:S100_FC_InformationType>

</S100FC:S100_FC_InformationTypes>

<S100FC:S100_FC_FeatureTypes>

<S100FC:S100_FC_FeatureType isAbstract="false">

<S100FC:name>Radio Station</S100FC:name>

<S100FC:definition>A place equipped to transmit radio waves. Such a station may be either stationary or mobile, and may also be provided with a radio receiver.</S100FC:definition>

<S100FC:code>RadioStation</S100FC:code>

<S100FC:remarks>The transmission of a radio station may serve to provide mariners with a line of position (IHO Chart Specifications, M-4). The feature &apos;radio station&apos; is used to encode the point of transmission of the signal.</S100FC:remarks>

<S100FC:alias>RDOSTA</S100FC:alias>

<S100FC:alias>W/T Station</S100FC:alias>

<S100FC:definitionReference>

<S100FC:sourceIdentifier>717</S100FC:sourceIdentifier>

<S100FC:definitionSource ref="IHOREG" />

</S100FC:definitionReference>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="callSign" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:permittedValues>

<S100FC:value>5</S100FC:value>

<S100FC:value>8</S100FC:value>

<S100FC:value>9</S100FC:value>

<S100FC:value>10</S100FC:value>

<S100FC:value>11</S100FC:value>

<S100FC:value>12</S100FC:value>

<S100FC:value>13</S100FC:value>

<S100FC:value>14</S100FC:value>

<S100FC:value>19</S100FC:value>

<S100FC:value>20</S100FC:value>

</S100FC:permittedValues>

<S100FC:attribute ref="categoryOfRadioStation" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="true" infinite="true" />

</S100FC:multiplicity>

<S100FC:attribute ref="communicationChannel" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="fixedDateRange" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="true" infinite="true" />

</S100FC:multiplicity>

<S100FC:attribute ref="periodicDateRange" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:attribute ref="scaleMinimum" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="true" infinite="true" />

</S100FC:multiplicity>

<S100FC:permittedValues>

<S100FC:value>1</S100FC:value>

<S100FC:value>2</S100FC:value>

<S100FC:value>4</S100FC:value>

<S100FC:value>5</S100FC:value>

<S100FC:value>7</S100FC:value>

<S100FC:value>8</S100FC:value>

</S100FC:permittedValues>

<S100FC:attribute ref="status" />

</S100FC:attributeBinding>

<S100FC:attributeBinding sequential="false">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="true" infinite="true" />

</S100FC:multiplicity>

<S100FC:attribute ref="featureName" />

</S100FC:attributeBinding>

<S100FC:informationBinding roleType="association">

<S100FC:multiplicity>

<S100Base:lower>1</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:association ref="DetailsOfDgnssStation" />

<S100FC:role ref="stationAlmanac" />

<S100FC:informationType ref="DgnssStationAlmanac" />

</S100FC:informationBinding>

<S100FC:informationBinding roleType="association">

<S100FC:multiplicity>

<S100Base:lower>0</S100Base:lower>

<S100Base:upper xsi:nil="false" infinite="false">1</S100Base:upper>

</S100FC:multiplicity>

<S100FC:association ref="SupplementInformation" />

<S100FC:role ref="providedBy" />

<S100FC:informationType ref="SupplementaryInformation" />

</S100FC:informationBinding>

<S100FC:featureUseType>geographic</S100FC:featureUseType>

<S100FC:permittedPrimitives>point</S100FC:permittedPrimitives>

</S100FC:S100_FC_FeatureType>

</S100FC:S100_FC_FeatureTypes>

</S100FC:S100_FC_FeatureCatalogue>