Input paper:  ENAV18-11.13

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					√   Input

√ ENAV	□  VTS							□  Information

Agenda item			11

Technical Domain / Task Number 	Communication (VDES) 2.1.2

Author(s) / Submitter(s)			Jens K. Jensen/DMA

Björn Andreason/SMA

# Facilitation of VDES testbeds

# Summary

In order to facilitate a safe transition from use of the frequencies for Analogue voice transmissions in the VHF Coastal Radio Service to digital VDES transmissions and promote VDES testbeds to develop, demonstrate and validate the system – a transition process for the use of the frequencies involved needs to be managed.

All the frequencies considered for VDES are in widespread use by coast or special stations around the world – and only in very isolated instances have authorities begun preparations for a transition towards the introduction of VDES

Based on the needs of a particular test bed project intending to conduct testbeds that span a large number of affected administrations, this paper proposes a process for how IALA could facilitate VDES testbeds as well as promote a safe transition for the introduction of VDES, preventing interference with existing VHF Coastal Radio Services.

## Purpose of the document

The communication WG of the ENAV Committee is invited to consider and possibly progress the proposed actions, and liaise with relevant telecommunication bodies if deemed appropriate.

## Related documents

# Background

## VDES development

Based on the outcome of World Radio Conference 2015, a window of opportunity exists for establishing testbeds to qualify and validate the concept of VDES (VHF Data Exchange System) in the period until January 1st 2019, using frequencies which after this date are allocated for implementing digital transmissions. Depending on ITU region, some of the frequency allocations are already change or will change  January 1st 2017. In many areas for some time beyond 2017, the VDES will need to exist “subject to coordination with affected administrations.”

Many existing VHF Coastal Radio Stations utilize the frequencies under reallocation for VDES, and a transition process needs to be initiated by Maritime and Telecommunication authorities, if VDES is to be introduced.

An initial list of Coast and Special stations in service, utilizing the VDES spectrum are provided in Appendix, based on a search in the ITU MARS database dated March 1st 2016. Note that each station may operate many radio sites – and from experience, the MARS database is not entirely complete. Many other stations may be utilizing the channels in real life.

| Channel | Number of Coast Stations utilizing this channel (Note: One Station may operate this channel on many sites!) |
| --- | --- |
| ASM1 (27) | 121 |
| ASM2 (28) | 111 |
| VDE (24) | 111 |
| VDE (84) | 38 |
| VDE (25) | 106 |
| VDE (85) | 43 |
| VDE (26) | 175 |
| VDE (86) | 29 |

Meanwhile, a need exists to establish testbeds and conduct validation projects, which can prove the VDES concept and provide user and technical experience, to feed back into the standardization processes. This is needed to achieve a harmonized and well-functioning VDES service, before investments already made prevent the user feedback and practical experience from improving the service.

## The STM validation project testbeds

The STM validation project (an EU cofounded project conducted 2015-2017) intends to equip 300 ships and 13 ports in large scale testbeds covering Northern Europe and the Mediterranean, to achieve validation of concepts related to standardized strategic and near real time voyage management functions, as well as collaborative decision making in port environments – and interaction between ships and ports.

One of the aims of the project is to validate a concept for exchange of monitored route segments for the near future between ships, based on previous experience obtained in similar projects, which have revealed a number of shortcomings in utilizing the AIS ASM for route exchange defined in IMO Safety of Navigation Circular 289.

### Route exchange ship-ship

Introducing route exchange ship-ship will give the intentions of other ships. Nothing in the current “navigational process” will be changed and the master is still responsible. The route exchange will solely introduce a new tool which helps the OOW to plan ahead, foresee possible dangerous situations and reduce route detours due to traffic conditions.

The route exchange should be used to avoid collision situations and close quarter situations. When in close quarter situations COLREGs are always in force.

### Test bed usage

Ships participating in the test beds should exchange routes (send and receive route segments) ship to ship via new route message (ASM) developed within the project and display them on ECDIS. Routes should not be displayed unless activated/clicked by other ship. The number of waypoints, updating frequency etc. will be further described in the route message format description. Route exchange should only be tested when it does not interfere with the safe navigation of participating ships which is up to OOW to decide.

# Discussion

Denmark restructured it’s Coastal Radio service during 2015, and executed a channel reallocation exercise as part of the project, clearing the VDES frequencies for the future. Experience from this indicate, that channel reallocation is a challenging task, technically, organizationally and politically. There are costs associated with changing frequencies of coast stations, and thorough planning is needed. It is therefore a time consuming process – and the timing of when allocations of WRC-15 enter into force is dependant on region, but not far away.

In reality, this exposes a significant VHF channel reallocation task, which will affect many coast and special stations in the coming years – and support for coordination & planning is needed.

Therefore actions to kickstart a process of transition from analogue to digital on the VDES channels needs to be prepared.

The intention of the STM validation project is to design a proposal for a better suited ASM for exchange of route intentions between ships, and possibly utilize the ASM frequencies (upper leg of channel 27/28 - or a mix of ASM and AIS frequencies to demonstrate transition with backwards compatibility towards existing AIS) to operationally validate the route exchange function, in order to:

- Conduct a demonstration of this use case for VDES and gain operational experience

- Ensure the transmitted ASM’s do not interfere with ships not taking part in the testbeds, carrying existing ECDIS or other navigational equipment, which have implemented support the IMO SN-289 route exchange ASM received on the AIS frequencies

- The AIS frequencies are not loaded excessively by the testbed activities, and the regulatory challenges that may be associated with transmitting ASM’s on the AIS frequencies in the domains of affected administrations are addressed

As the large scale testbeds will include ships travelling most of maritime Europe and beyond, and cover a large number of affected administrations, the use of the ASM frequencies may pose a significant challenge. The need of the STM validation project is therefore a coordinated application to all affected administrations in Europe – which is a huge task for the project partners themselves.

This challenge is quite similar to any other testbed project that would want to demonstrate or validate technical or operational aspects of introducing VDES in the transition period.

In principle, any project intending to conduct testbeds would need to either obtain permission from every single affected administration for the VDES frequencies to be used – which would indicate a need to restrict tests to be conducted in limited areas - or to implement and maintain some ‘geofencing’ mechanism that can secure that the testbed ASM’s are only transmitted inside confined testbed areas.

This paper proposes the following actions in order to

- Prevent interference of testbed activities to existing usage of the frequencies by active Coastal Radio Stations or other users

- Prevent interference between activities of several testbeds

- Consider transition needs when eventually introducing VDES into an environment, where it may take decades before the end of sporadic or local usage of the VDES frequencies for analogue voice VHF

Proposed actions:

- Create a collection of Coast stations utilising the Appendix 18 VHF channels  (including the VDES channels) and indicating type of service (analogue/digital ), with a visual (map) presentation, similar to the overview of the IALA DGNSS Beacon network.

- Liaise with the ITU secretariat on populating and maintaining the collection from the information contained in the MARS database and receiving updates. The MARS database registrations should be considered the authoritative ‘ground truth’.)

- Create an interface for ‘crowdsource’ maintenance of the collection, allowing indication of stations not in the MARS database, or planned changes to working channels, which will facilitate transition planning. This will provide a good basis for planning of testbeds on the VDES channels, and function as a facilitator of negotiating channel reallocation between neighbouring and affected administrations.

- Comms WG to consider which technical solutions may allow coexistence and avoid interference between analogue and digital stations on the VDES channels

  - Carrier Sense protection – and potential technical drawbacks or interference implications

  - Other options like ‘geofencing’, local channel management - and the potential technical and system integrity/security implications

- Comms WG to (quickly) develop a guideline for good practice when setting up and coordinating the activities of VDES testbeds.

- Comms WG to draft a coordinated application for utilization of the VDES spectrum for testbeds, helping IALA members to formulate an application for conducting testbeds based on the good practices in the testbed guideline.

- Encourage IALA national members to submit the coordinated application to it’s own national telecommunication authority, and provide a national point of contact, where the national member may assist in coordinating testbed activities in the waters of the members national domain under the model testbed permission.  (In particular in Europe, this would assist the STM validation project VDES testbed tremendously)

- Consider whether liaison with CEPT (or ITU WP5B) on the issues above would assist in promoting a sound basis for conducting testbeds, and ultimately a safe transition path for introducing VDES.

Initial list of stations utilizing VDES channels

The following is derived from the ITU MARS database. (http://www.itu.int/online/mms/mars/coast/coast_search.sh)

Note that each Coast Station may utilize the channel on several individual radio sites.

- ASM channels

ASM1 - Channel 27

| Station Name | Symbol | MMSI | Services | Updated |
| --- | --- | --- | --- | --- |
| AASIAAT RADIO | GRL | 003313000 | DSC-WATCH MED-ADVICE NOTICE-NAV CP | 14/04/2011 |
| AL QUNFUDHAH RADIO | ARS |  | DSC-WATCH CP | 17/06/2004 |
| AL WAJH RADIO | ARS |  | DSC-WATCH CP | 17/06/2004 |
| ALEXANDRIA RADIO | EGY | 006221111 | DSC-WATCH NAVINFO CP | 15/04/2011 |
| ALGER RADIO | ALG | 006052110 | DSC-WATCH CP | 14/04/2011 |
| ANTOFAGASTA ZONAL RADIO | CHL | 007250050 | DSC-WATCH MED-ADVICE METEO CP | 14/04/2011 |
| ANTWERPEN RADIO | BEL | 002050485 | DSC-WATCH METEO NOTICE-NAV CP | 26/04/2011 |
| ARGENTINA RADIO | ARG | 007010111 | DSC-WATCH CP | 23/12/2014 |
| ARKHANGELSK RADIO | RUS | 002734414 | DSC-WATCH CP | 15/04/2011 |
| BANGKOK RADIO | THA | 005671000 | DSC-WATCH CP | 14/04/2011 |
| BARI RADIO | I | 002470002 002470121 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| BEJAIA RADIO (ANNABA RADIO ) | ALG | 006053815 | CP | 30/08/2011 |
| BODO RADIO | NOR | 002570700 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| BODRUM RADIO (ISTANBUL TURK RADIO ) | TUR | 002713000 | DSC-WATCH CP | 14/09/2011 |
| CABO LA NAO RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 21/07/2015 |
| CAPE TOWN RADIO | AFS | 006010001 | DSC-WATCH METEO NOTICE-NAV CP | 24/10/2011 |
| CHERCHELL RADIO | ALG | 006052111 | DSC-WATCH CP | 25/11/2010 |
| CHETUMAL, QUINTANA ROO RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003451120 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| CIUDAD DEL CARMEN, CAMPECHE RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003450710 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| CIVITAVECCHIA RADIO | I | 002470001 002470123 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| COATZACOALCOS, VERACRUZ RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003450320 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| CONSTANTA RADIO | ROU | 002640570 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV CP PORTINFO NAVAREA | 15/04/2011 |
| COZUMEL, QUINTANA ROO RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003451110 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| CYPRUS RADIO | CYP | 002091000 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 14/04/2011 |
| DA NANG RADIO | VTN | 005742030 | DSC-WATCH METEO NOTICE-NAV CP | 02/09/2015 |
| DALIAN RADIO | CHN | 004121300 | DSC-WATCH METEO NOTICE-NAV VTS PILOT CP | 30/09/2011 |
| DURBAN RADIO | AFS |  | METEO NOTICE-NAV CP | 25/10/2011 |
| FAISTOS RADIO | GRC |  | METEO NOTICE-NAV | 07/08/2014 |
| FESTOS RADIO (OLYMPIA RADIO ) | GRC |  | CP | 26/01/2011 |
| FLENSBURG RADIO | D |  | CP | 04/01/2010 |
| FLORO RADIO | NOR | 002570500 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 08/08/2014 |
| GENAVEH RADIO | IRN | 004225206 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| GENOVA RADIO | I | 002470001 002470125 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| GRIMSBY RADIO | G |  | MED-ADVICE METEO | 13/04/2011 |
| GUANGZHOU RADIO | CHN | 004123100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV VTS CP | 15/04/2011 |
| GUNSAN RADIO | KOR | 004400201 004400501 004403501 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| HALIFAX COAST GUARD RADIO | CAN | 003160016 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| HAMBURG RADIO | D |  | CP | 04/01/2010 |
| INCHEON RADIO | KOR | 004400003 004400004 004403001 | DSC-WATCH MED-ADVICE CP AIS | 03/08/2015 |
| INEBOLU RADIO (ISTANBUL TURK RADIO ) | TUR | 002712000 | DSC-WATCH CP | 14/09/2011 |
| JACUTINGA | B |  | CP | 18/08/2015 |
| JAKARTA RADIO/PKX | INS | 005250000 | DSC-WATCH NAVINFO METEO NOTICE-NAV UTC CP | 12/05/2011 |
| JEDDAH RADIO | ARS | 004030000 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 14/04/2011 |
| JEJU RADIO | KOR | 004400701 004400702 004403701 | DSC-WATCH MED-ADVICE CP AIS | 09/07/2015 |
| JOINVILLE | B |  | CP | 18/08/2015 |
| JUNCAO | B |  | CP | 08/09/2015 |
| KAMENNYY C YNAO/MKAMENNYYDISPET-MP | RUS |  | CP | 28/11/2014 |
| KANGNUNG RADIO | KOR | 004400602 004400603 004400604 | DSC-WATCH MED-ADVICE CP AIS | 26/04/2011 |
| KEFALLINIA RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| KHORRAMSHAHR RADIO | IRN | 004224101 004225101 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| KIRINSKOE GKM SAKH/KIRINSKOEDISPET-MP | RUS |  | CP | 11/11/2014 |
| KOSSEIR RADIO | EGY | 006221112 | DSC-WATCH NAVINFO CP | 27/04/2011 |
| LAND'S END RADIO | G |  | MED-ADVICE METEO NOTICE-NAV | 26/04/2011 |
| LAS PALMAS RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 23/07/2015 |
| LAVAR RADIO | IRN | 004225204 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| LAZARO CARDENAS, MICHOACAN RADIO (MAZATLAN, SINALOA RADIO ) | MEX | 003451610 | DSC-WATCH METEO NOTICE-NAV CP | 06/03/2012 |
| LES ESCOUMINS COAST GUARD RADIO | CAN | 003160026 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| MACEIO | B |  | CP | 18/08/2015 |
| MACHICHACO RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 24/07/2015 |
| MAGALLANES ZONAL RADIO | CHL | 007250380 | DSC-WATCH MED-ADVICE METEO CP SAR AGENCY | 16/09/2011 |
| MAHDIA RADIO | TUN |  | CP | 14/04/2011 |
| MIDDELKERKE RADIO (OOSTENDE RADIO ) | BEL |  | CP | 11/12/2001 |
| MOBILE, ALABAMA RADIO/WLO | USA | 003660003 | DSC-WATCH MED-ADVICE CP | 12/08/2014 |
| MOKPO RADIO | KOR | 004400304 004400307 004400308 004400309 004400310 004403301 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| MONG CAI RADIO | VTN | 005741010 | DSC-WATCH METEO NOTICE-NAV CP | 26/09/2011 |
| MORRO DA POLICIA | B |  | CP | 18/08/2015 |
| MRCC PAPEETE | OCE | 005461000 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV | 06/09/2011 |
| MSI SWEDEN | S | 002653500 | METEO NOTICE-NAV | 27/08/2015 |
| MUSCAT RADIO | OMA |  | MED-ADVICE CP | 26/04/2011 |
| NAPOLI RADIO | I | 002470002 002470130 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| NESKAUPSTADUR RADIO | ISL | 002510100 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| OOSTENDE RADIO | BEL | 002050480 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 28/04/2011 |
| PALERMO RADIO | I | 002470002 | DSC-WATCH METEO NOTICE-NAV CP | 26/04/2011 |
| PALMA RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 27/07/2015 |
| PARANAPIACABA | B |  | CP | 18/08/2015 |
| PLACENTIA COAST GUARD RADIO | CAN | 003160019 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| POHANG RADIO | KOR | 004400401 004400402 004403401 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| POROS RADIO (OLYMPIA RADIO ) | GRC |  | NOTICE-NAV CP | 26/01/2011 |
| PORT AUX BASQUES COAST GUARD RADIO | CAN | 003160018 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| PORT ELIZABETH RADIO | AFS |  | METEO NOTICE-NAV CP | 25/10/2011 |
| PORT SAID RADIO | EGY | 006221113 | DSC-WATCH CP | 27/04/2011 |
| PORTPATRICK RADIO | G |  | MED-ADVICE METEO NOTICE-NAV | 14/08/2014 |
| PRAIA DO FUTURO | B |  | CP | 18/08/2015 |
| PRESCOTT COAST GUARD RADIO | CAN | 003160029 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| PRINCE RUPERT RADIO | CAN |  | CP | 26/04/2011 |
| PROGRESO, YUCATAN RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003450910 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| PUERTO AYORA RADIO | EQA | 007354757 | DSC-WATCH METEO NOTICE-NAV CP | 07/09/2015 |
| PUERTO MONTT ZONAL RADIO | CHL | 007250230 | DSC-WATCH MED-ADVICE METEO CP SAR AGENCY | 16/09/2011 |
| PUERTO VALLARTA, JALISCO RADIO (MAZATLAN, SINALOA RADIO ) | MEX | 003451210 | DSC-WATCH METEO NOTICE-NAV CP | 09/10/2008 |
| PUSAN RADIO | KOR | 004400101 004400103 004400104 004400105 004400106 004403102 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| RABIGH RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| REYKJAVIK RADIO | ISL | 002510100 | DSC-WATCH METEO NOTICE-NAV CP | 27/04/2011 |
| ROGALAND RADIO | NOR | 002570300 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| SABETTA YANAO/SABETTA RADIO MP | RUS |  | VTS CP PORTINFO | 26/05/2015 |
| SAFI RADIO | MRC |  | MED-ADVICE METEO NOTICE-NAV CP | 30/09/2011 |
| SALALAH RADIO | OMA |  | MED-ADVICE CP | 27/05/2008 |
| SARNIA COAST GUARD RADIO | CAN | 003160030 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| SHETLAND RADIO | G |  | MED-ADVICE METEO | 15/08/2014 |
| SIGLUFJOERDUR RADIO | ISL | 002510100 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| SKIKDA RADIO | ALG | 006053816 | CP | 14/04/2011 |
| STOCKHOLM RADIO | S | 002652000 | CP | 26/08/2015 |
| TALCAHUANO ZONAL RADIO | CHL | 007250170 | DSC-WATCH MED-ADVICE METEO CP SAR AGENCY | 13/01/2012 |
| TALLINN RADIO | EST | 002761000 | DSC-WATCH MED-ADVICE NAVINFO METEO NOTICE-NAV CP | 16/04/2012 |
| TAMPICO, TAMAULIPAS RADIO | MEX | 003450110 | DSC-WATCH CP | 15/04/2011 |
| TENES RADIO | ALG | 006052113 | DSC-WATCH CP | 14/04/2011 |
| TIKSI YAKUT/TIKSI-LORP | RUS |  | CP | 14/07/2015 |
| TJOME RADIO | NOR | 002570100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| TORTOLA RADIO | VRG |  | CP | 12/08/1993 |
| TRIESTE RADIO | I | 002470001 002470135 | DSC-WATCH METEO NOTICE-NAV CP | 18/08/2015 |
| TSOUKALAS RADIO (OLYMPIA RADIO ) | GRC |  | NOTICE-NAV CP | 26/01/2011 |
| ULREUNG RADIO | KOR | 004400403 004400404 | DSC-WATCH MED-ADVICE CP | 26/04/2011 |
| ULSAN RADIO | KOR | 004400102 | DSC-WATCH MED-ADVICE CP | 12/11/2007 |
| UMM LUJJ RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| VALPARAISO PLAYA ANCHA RADIO | CHL | 007251860 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV UTC CP NAVAREA SAR AGENCY | 16/09/2011 |
| VARDO RADIO | NOR | 002570800 | DSC-WATCH MED-ADVICE NAVINFO METEO NOTICE-NAV CP | 26/04/2011 |
| VERACRUZ, VERACRUZ RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003450310 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| VESTMANNAEYJAR RADIO | ISL | 002510100 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| WALVIS BAY RADIO | NMB |  | MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| YEOSU RADIO | KOR | 004400305 004400306 | DSC-WATCH MED-ADVICE CP | 09/07/2015 |
| ZEEBRUGGE RADIO (OOSTENDE RADIO ) | BEL |  | CP | 06/08/2014 |
| ZHANJIANG RADIO | CHN | 004123300 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV VTS CP AIS | 03/10/2011 |
| Number of records found = 121 |  |  |  |  |

ASM2 - Channel 28

| Station Name | Symbol | MMSI | Services | Updated |
| --- | --- | --- | --- | --- |
| AASIAAT RADIO | GRL | 003313000 | DSC-WATCH MED-ADVICE NOTICE-NAV CP | 14/04/2011 |
| ACAPULCO, GUERRERO RADIO (MAZATLAN, SINALOA RADIO ) | MEX | 003451810 | DSC-WATCH METEO NOTICE-NAV CP | 14/04/2011 |
| ACCUMERSIEL RADIO | D |  | CP | 18/12/2009 |
| AGADIR RADIO | MRC |  | MED-ADVICE METEO NOTICE-NAV CP | 30/09/2011 |
| AL BIRK RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| ALEXANDRIA RADIO | EGY | 006221111 | DSC-WATCH NAVINFO CP | 15/04/2011 |
| ALGER RADIO | ALG | 006052110 | DSC-WATCH CP | 14/04/2011 |
| ALLOUIS (FRANCE INTER) | F |  | UTC | 07/08/2014 |
| AMBOINA RADIO/PKE | INS | 005250006 | DSC-WATCH METEO NOTICE-NAV CP | 04/05/2011 |
| AYVALIK RADIO (ISTANBUL TURK RADIO ) | TUR | 002711000 | DSC-WATCH CP | 26/04/2011 |
| BANDIRMA RADIO (ISTANBUL TURK RADIO ) | TUR | 002711000 | DSC-WATCH CP | 14/04/2011 |
| BEJAIA RADIO (ANNABA RADIO ) | ALG | 006053815 | CP | 30/08/2011 |
| BELAWAN RADIO/PKB | INS | 005250003 | DSC-WATCH METEO NOTICE-NAV CP | 20/05/2011 |
| BERMUDA RADIO | BER |  | MED-ADVICE METEO CP | 06/08/2014 |
| BODO RADIO | NOR | 002570700 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| BREMEN RADIO | D |  | CP | 18/12/2009 |
| BROYCHAS RADIO (OLYMPIA RADIO ) | GRC |  | NOTICE-NAV CP | 26/01/2011 |
| CABO LA NAO RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 21/07/2015 |
| CASIMIRO DE ABREU | B |  | CP | 18/08/2015 |
| CASPER, CALIFORNIA RADIO | USA |  | CP | 26/04/2011 |
| CHERCHELL RADIO | ALG | 006052111 | DSC-WATCH CP | 25/11/2010 |
| CILACAP RADIO/PKR3 | INS | 005250030 | DSC-WATCH NOTICE-NAV CP | 20/05/2011 |
| CROMARTY RADIO | G |  | MED-ADVICE METEO | 12/08/2014 |
| DELCAMBRE, LOUISIANA RADIO | USA |  | CP | 26/04/2011 |
| DIKMEN RADIO (ISTANBUL TURK RADIO ) | TUR | 002712000 | DSC-WATCH CP | 14/09/2011 |
| DOHA RADIO | QAT | 004661010 | DSC-WATCH METEO NOTICE-NAV CP | 26/04/2011 |
| DUBROVNIK RADIO | HRV | 002380300 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 13/12/2011 |
| DUMAI RADIO/PKP | INS | 005250004 | DSC-WATCH METEO NOTICE-NAV CP | 27/04/2011 |
| DURBAN RADIO | AFS |  | METEO NOTICE-NAV CP | 25/10/2011 |
| EMIRATES RADIO | UAE | 004700000 | DSC-WATCH CP | 26/04/2011 |
| ENSENADA, BAJA CALIFORNIA RADIO (MAZATLAN, SINALOA RADIO ) | MEX | 003450210 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| ESSAOUIRA RADIO | MRC |  | CP | 26/04/2011 |
| FLORO RADIO | NOR | 002570500 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 08/08/2014 |
| GHAZAOUET RADIO | ALG | 006054119 | DSC-WATCH CP | 14/04/2011 |
| GRESIK RADIO | INS |  | CP | 27/04/2011 |
| GUARUJA | B |  | CP | 18/08/2015 |
| GUAYAQUIL RADIO | EQA | 007354750 | DSC-WATCH NOTICE-NAV CP | 26/04/2011 |
| GUNSAN RADIO | KOR | 004400201 004400501 004403501 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| JAKARTA RADIO/PKX | INS | 005250000 | DSC-WATCH NAVINFO METEO NOTICE-NAV UTC CP | 12/05/2011 |
| JEJU RADIO | KOR | 004400701 004400702 004403701 | DSC-WATCH MED-ADVICE CP AIS | 09/07/2015 |
| JIANGYIN RADIO | CHN | 004126620 | DSC-WATCH NOTICE-NAV PILOT CP PORTINFO | 21/11/2011 |
| JIZAN RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| KAMENNYY C YNAO/MYS KAMENNYY-GPNNP | RUS |  | CP | 27/11/2014 |
| KANGNUNG RADIO | KOR | 004400602 004400603 004400604 | DSC-WATCH MED-ADVICE CP AIS | 26/04/2011 |
| KAZAKIN RADIO (ISTANBUL TURK RADIO ) | TUR | 002713000 | DSC-WATCH CP | 14/09/2011 |
| KEFALLINIA RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| KOSSEIR RADIO | EGY | 006221112 | DSC-WATCH NAVINFO CP | 27/04/2011 |
| KUMAI RADIO | INS | 005251522 | DSC-WATCH CP | 12/06/2011 |
| KUPANG RADIO/PKK | INS | 005250010 | DSC-WATCH NOTICE-NAV CP | 17/01/2012 |
| MAHDIA RADIO | TUN |  | CP | 14/04/2011 |
| MANZANILLO, COLIMA RADIO (MAZATLAN, SINALOA RADIO ) | MEX | 003451410 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| MAZATLAN, SINALOA RADIO | MEX | 003450810 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| MOBILE, ALABAMA RADIO/WLO | USA | 003660003 | DSC-WATCH MED-ADVICE CP | 12/08/2014 |
| MOKPO RADIO | KOR | 004400304 004400307 004400308 004400309 004400310 004403301 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| MORRO DO BRILHANTE | B |  | CP | 18/08/2015 |
| MRCC NOUMEA | NCL | 005401000 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV | 06/09/2011 |
| MSI SWEDEN | S | 002653500 | METEO NOTICE-NAV | 27/08/2015 |
| NITON RADIO | G |  | MED-ADVICE METEO NOTICE-NAV | 14/08/2014 |
| NOUMEA RADIO | NCL |  | CP AIS | 06/09/2011 |
| OREN RADIO (ISTANBUL TURK RADIO ) | TUR | 002713000 | DSC-WATCH CP | 14/09/2011 |
| PALEMBANG RADIO | INS | 005251507 | DSC-WATCH METEO NOTICE-NAV CP | 12/06/2011 |
| PLACENTIA COAST GUARD RADIO | CAN | 003160019 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| POROS RADIO (OLYMPIA RADIO ) | GRC |  | NOTICE-NAV CP | 26/01/2011 |
| PORT AUX BASQUES COAST GUARD RADIO | CAN | 003160018 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| PORT ELIZABETH RADIO | AFS |  | METEO NOTICE-NAV CP | 25/10/2011 |
| PORT SAID RADIO | EGY | 006221113 | DSC-WATCH CP | 27/04/2011 |
| PORTO CERVO RADIO | I | 002470001 002470132 | DSC-WATCH METEO NOTICE-NAV CP | 18/08/2015 |
| PRESCOTT COAST GUARD RADIO | CAN | 003160029 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| PUERTO AYORA RADIO | EQA | 007354757 | DSC-WATCH METEO NOTICE-NAV CP | 07/09/2015 |
| PUSAN RADIO | KOR | 004400101 004400103 004400104 004400105 004400106 004403102 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| QINHUANGDAO RADIO | CHN | 004121200 | DSC-WATCH VTS PORTINFO | 03/10/2011 |
| ROGALAND RADIO | NOR | 002570300 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| ROGERS CITY, MICHIGAN RADIO | USA |  | CP | 15/04/2011 |
| S. DIEGO, CALIFORNIA RADIO | USA |  | MED-ADVICE CP | 26/04/2011 |
| S. THOMAS, VIRGIN ISLANDS RADIO | VIR |  | CP | 15/04/2011 |
| SABANG RADIO/PKA | INS |  | NOTICE-NAV CP | 28/04/2011 |
| SALVADOR PARIPE | B |  | CP | 18/08/2015 |
| SHANNON COAST GUARD RADIO (MRSC VALENTIA RADIO ) | IRL |  | MED-ADVICE RCC(s) METEO NOTICE-NAV | 16/09/2011 |
| SHUAIBA RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| SPLIT RADIO | HRV | 002380100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 14/04/2011 |
| STOCKHOLM RADIO | S | 002652000 | CP | 26/08/2015 |
| SURABAYA RADIO/PKD | INS | 005250001 | DSC-WATCH METEO NOTICE-NAV CP | 27/04/2011 |
| TARIFA RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 23/07/2015 |
| TELUK BAYUR RADIO/PKV | INS | 005250075 | DSC-WATCH CP | 08/02/2012 |
| TURKU RADIO | FIN | 002300230 | DSC-WATCH METEO NOTICE-NAV | 28/04/2011 |
| ULREUNG RADIO | KOR | 004400403 004400404 | DSC-WATCH MED-ADVICE CP | 26/04/2011 |
| ULSAN RADIO | KOR | 004400102 | DSC-WATCH MED-ADVICE CP | 12/11/2007 |
| VARDO RADIO | NOR | 002570800 | DSC-WATCH MED-ADVICE NAVINFO METEO NOTICE-NAV CP | 26/04/2011 |
| VARNA RADIO | BUL | 002070810 | DSC-WATCH METEO NOTICE-NAV CP | 26/04/2011 |
| YEOSU RADIO | KOR | 004400305 004400306 | DSC-WATCH MED-ADVICE CP | 09/07/2015 |
| Number of records found = 90 |  |  |  |  |

- VDE channels

Channel 24

| Station Name | Symbol | MMSI | Services | Updated |
| --- | --- | --- | --- | --- |
| AASIAAT RADIO | GRL | 003313000 | DSC-WATCH MED-ADVICE NOTICE-NAV CP | 14/04/2011 |
| AGADIR RADIO | MRC |  | MED-ADVICE METEO NOTICE-NAV CP | 30/09/2011 |
| AKCABAT RADIO (ISTANBUL TURK RADIO ) | TUR | 002712000 | DSC-WATCH CP | 14/09/2011 |
| AKDAG RADIO (ISTANBUL TURK RADIO ) | TUR | 002711000 | DSC-WATCH CP | 15/04/2011 |
| AL QUNFUDHAH RADIO | ARS |  | DSC-WATCH CP | 17/06/2004 |
| AL WAJH RADIO | ARS |  | DSC-WATCH CP | 17/06/2004 |
| ALEXANDRIA RADIO | EGY | 006221111 | DSC-WATCH NAVINFO CP | 15/04/2011 |
| ALGER RADIO | ALG | 006052110 | DSC-WATCH CP | 14/04/2011 |
| ANDROS RADIO (OLYMPIA RADIO ) | GRC |  | NOTICE-NAV CP | 26/01/2011 |
| ANNABA RADIO | ALG | 006053814 | DSC-WATCH CP | 14/04/2011 |
| ANTWERPEN RADIO | BEL | 002050485 | DSC-WATCH METEO NOTICE-NAV CP | 26/04/2011 |
| ARACAJU | B |  | CP | 17/08/2015 |
| ARACATI | B |  | CP | 18/08/2015 |
| AVALON, CALIFORNIA RADIO | USA |  | CP | 26/04/2011 |
| BANGKOK RADIO | THA | 005671000 | DSC-WATCH CP | 14/04/2011 |
| BAR RADIO | MNE | 002620001 002620002 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV VTS CP CES-CP AIS SAR AGENCY | 14/04/2011 |
| BARI RADIO | I | 002470002 002470121 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| BIZERTE RADIO | TUN |  | CP | 14/04/2011 |
| BODO RADIO | NOR | 002570700 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| CABEDELO | B |  | CP | 18/08/2015 |
| CABO DE GATA RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 24/07/2015 |
| CAPE TOWN RADIO | AFS | 006010001 | DSC-WATCH METEO NOTICE-NAV CP | 24/10/2011 |
| CASABLANCA RADIO | MRC | 002422450 | MED-ADVICE METEO NOTICE-NAV CP | 30/09/2011 |
| CELTIC RADIO | G |  | MED-ADVICE METEO | 13/04/2011 |
| CHABAHAR RADIO | IRN | 004224401 004225400 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| COLLAFIRTH RADIO | G |  | MED-ADVICE METEO | 11/08/2014 |
| CONSTANTA RADIO | ROU | 002640570 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV CP PORTINFO NAVAREA | 15/04/2011 |
| CYPRUS RADIO | CYP | 002091000 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 14/04/2011 |
| DAYER RADIO | IRN | 004225203 | DSC-WATCH CP PORTINFO | 05/08/2011 |
| DEYLAM RADIO | IRN | 004225205 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| DJIBOUTI RADIO | DJI | 006211000 | DSC-WATCH CP | 26/04/2011 |
| DOHA RADIO | QAT | 004661010 | DSC-WATCH METEO NOTICE-NAV CP | 26/04/2011 |
| DURBAN RADIO | AFS |  | METEO NOTICE-NAV CP | 25/10/2011 |
| ELBE-WESER RADIO | D |  | CP | 18/12/2009 |
| ESMERALDAS RADIO | EQA | 007354752 | DSC-WATCH CP | 11/09/2015 |
| ESSAOUIRA RADIO | MRC |  | CP | 26/04/2011 |
| FLORO RADIO | NOR | 002570500 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 08/08/2014 |
| FORTH RADIO | G |  | MED-ADVICE METEO | 13/04/2011 |
| GENT RADIO (ANTWERPEN RADIO ) | BEL |  | CP | 11/12/2001 |
| GHAZAOUET RADIO | ALG | 006054119 | DSC-WATCH CP | 14/04/2011 |
| GLEN HEAD COAST GUARD RADIO (MRSC MALIN HEAD RADIO ) | IRL |  | MED-ADVICE RCC(s) METEO NOTICE-NAV | 16/09/2011 |
| GUAYAQUIL RADIO | EQA | 007354750 | DSC-WATCH NOTICE-NAV CP | 26/04/2011 |
| GUNSAN RADIO | KOR | 004400201 004400501 004403501 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| HALIFAX COAST GUARD RADIO | CAN | 003160016 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| INCHEON RADIO | KOR | 004400003 004400004 004403001 | DSC-WATCH MED-ADVICE CP AIS | 03/08/2015 |
| JEFFERSONVILLE, INDIANA RADIO | USA |  | CP | 15/04/2011 |
| JEJU RADIO | KOR | 004400701 004400702 004403701 | DSC-WATCH MED-ADVICE CP AIS | 09/07/2015 |
| JIJEL RADIO | ALG |  | CP PORTINFO | 25/11/2010 |
| JUBAIL RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| KANGNUNG RADIO | KOR | 004400602 004400603 004400604 | DSC-WATCH MED-ADVICE CP AIS | 26/04/2011 |
| KELTEPE RADIO (ISTANBUL TURK RADIO ) | TUR | 002711000 | DSC-WATCH CP | 27/04/2011 |
| KOSSEIR RADIO | EGY | 006221112 | DSC-WATCH NAVINFO CP | 27/04/2011 |
| LABRADOR COAST GUARD RADIO | CAN | 003160022 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| LAS PALMAS RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 23/07/2015 |
| LES ESCOUMINS COAST GUARD RADIO | CAN | 003160026 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| LUBECK RADIO | D |  | CP | 04/01/2010 |
| MACHICHACO RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 24/07/2015 |
| MAURITIUS RADIO | MAU | 006452700 | DSC-WATCH METEO NOTICE-NAV | 05/09/2013 |
| MOBILE, ALABAMA RADIO/WLO | USA | 003660003 | DSC-WATCH MED-ADVICE CP | 12/08/2014 |
| MOKPO RADIO | KOR | 004400304 004400307 004400308 004400309 004400310 004403301 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| MONACO RADIO | MCO |  | METEO | 30/06/2015 |
| MRCC NOUMEA | NCL | 005401000 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV | 06/09/2011 |
| MRSC VALENTIA RADIO | IRL | 002500200 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV SAR AGENCY | 16/09/2011 |
| MSI SWEDEN | S | 002653500 | METEO NOTICE-NAV | 27/08/2015 |
| NAPOLI RADIO | I | 002470002 002470130 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| NOUMEA RADIO | NCL |  | CP AIS | 06/09/2011 |
| OMAN COAST RADIO/A4A20 (MUSCAT) | OMA | 004610013 | DSC-WATCH MED-ADVICE RCC(s) NAVINFO NOTICE-NAV CP | 19/05/2015 |
| OMAN COAST RADIO/A4A21 (SALALAH) | OMA | 004610014 | DSC-WATCH MED-ADVICE RCC(s) NAVINFO NOTICE-NAV CP | 19/05/2015 |
| OMAN COAST RADIO/A4A22 (KHASAB) | OMA | 004610015 | DSC-WATCH MED-ADVICE RCC(s) NOTICE-NAV CP | 04/06/2015 |
| ORAN RADIO | ALG | 006054117 | DSC-WATCH CP | 14/04/2011 |
| OSORIO | B |  | CP | 18/08/2015 |
| PALMA RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 27/07/2015 |
| PARANAGUA-SEDE | B |  | CP | 18/08/2015 |
| PATMOS RADIO (OLYMPIA RADIO ) | GRC |  | NOTICE-NAV CP | 26/01/2011 |
| PLACENTIA COAST GUARD RADIO | CAN | 003160019 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| POHANG RADIO | KOR | 004400401 004400402 004403401 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| PORT AUX BASQUES COAST GUARD RADIO | CAN | 003160018 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| PORT ELIZABETH RADIO | AFS |  | METEO NOTICE-NAV CP | 25/10/2011 |
| PORT SAID RADIO | EGY | 006221113 | DSC-WATCH CP | 27/04/2011 |
| PRESCOTT COAST GUARD RADIO | CAN | 003160029 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| PUERTO AYORA RADIO | EQA | 007354757 | DSC-WATCH METEO NOTICE-NAV CP | 07/09/2015 |
| PUSAN RADIO | KOR | 004400101 004400103 004400104 004400105 004400106 004403102 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| QUEBEC COAST GUARD RADIO | CAN | 003160027 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| RABIGH RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| RIJEKA RADIO | HRV | 002380200 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 13/12/2011 |
| ROGALAND RADIO | NOR | 002570300 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| S. LORENZO RADIO | EQA |  | CP | 23/09/2009 |
| S. THOMAS, VIRGIN ISLANDS RADIO | VIR |  | CP | 15/04/2011 |
| SAMPIT RADIO | INS | 005251521 | DSC-WATCH CP PORTINFO | 12/06/2011 |
| SAO SEBASTIAO | B |  | CP | 18/08/2015 |
| SARNIA COAST GUARD RADIO | CAN | 003160030 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| SFAX RADIO | TUN |  | CP | 14/04/2011 |
| SFENDAMI RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| SHUAIBA RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| SIGLUFJOERDUR RADIO | ISL | 002510100 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| SKYE RADIO | G |  | MED-ADVICE METEO | 15/08/2014 |
| STOCKHOLM RADIO | S | 002652000 | CP | 26/08/2015 |
| SYDNEY COAST GUARD RADIO | CAN | 003160017 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| TABARKA RADIO | TUN |  | CP | 14/04/2011 |
| TANGER RADIO | MRC |  | MED-ADVICE METEO NOTICE-NAV CP | 30/09/2011 |
| TEIXEIRA DE FREITAS | B |  | CP | 18/08/2015 |
| TJOME RADIO | NOR | 002570100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| TORSHAVN RADIO | FRO | 002311000 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 28/04/2011 |
| TURKU RADIO | FIN | 002300230 | DSC-WATCH METEO NOTICE-NAV | 28/04/2011 |
| ULREUNG RADIO | KOR | 004400403 004400404 | DSC-WATCH MED-ADVICE CP | 26/04/2011 |
| VARDO RADIO | NOR | 002570800 | DSC-WATCH MED-ADVICE NAVINFO METEO NOTICE-NAV CP | 26/04/2011 |
| VARNA RADIO | BUL | 002070810 | DSC-WATCH METEO NOTICE-NAV CP | 26/04/2011 |
| WITOWO RADIO | POL | 002610210 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 27/04/2011 |
| YEOSU RADIO | KOR | 004400305 004400306 | DSC-WATCH MED-ADVICE CP | 09/07/2015 |
| ZARZIS RADIO | TUN |  | CP | 14/04/2011 |
| ZEMMOURI RADIO | ALG |  | CP | 06/08/2014 |
| Number of records found = 111 |  |  |  |  |

Channel 84

| Station Name | Symbol | MMSI | Services | Updated |
| --- | --- | --- | --- | --- |
| AKDAG RADIO (ISTANBUL TURK RADIO ) | TUR | 002711000 | DSC-WATCH CP | 15/04/2011 |
| ALEXANDRIA RADIO | EGY | 006221111 | DSC-WATCH NAVINFO CP | 15/04/2011 |
| ALGER RADIO | ALG | 006052110 | DSC-WATCH CP | 14/04/2011 |
| BODO RADIO | NOR | 002570700 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| BUSHEHR RADIO | IRN | 004224200 004225200 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| CAPE TOWN RADIO | AFS | 006010001 | DSC-WATCH METEO NOTICE-NAV CP | 24/10/2011 |
| COMOX COAST GUARD RADIO | CAN | 003160014 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| CROTONE RADIO | I | 002470002 002470124 | DSC-WATCH METEO NOTICE-NAV CP | 01/09/2015 |
| DELLYS RADIO | ALG | 006052112 | DSC-WATCH CP | 25/11/2010 |
| EL KALA RADIO | ALG |  | CP PORTINFO | 06/08/2014 |
| EMIRATES RADIO | UAE | 004700000 | DSC-WATCH CP | 26/04/2011 |
| FLORO RADIO | NOR | 002570500 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 08/08/2014 |
| KELTEPE RADIO (ISTANBUL TURK RADIO ) | TUR | 002711000 | DSC-WATCH CP | 27/04/2011 |
| KNOSSOS RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| KOSSEIR RADIO | EGY | 006221112 | DSC-WATCH NAVINFO CP | 27/04/2011 |
| LAMPEDUSA RADIO | I | 002470002 002470126 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| LES ESCOUMINS COAST GUARD RADIO | CAN | 003160026 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| LIVRAMENTO | B |  | CP | 18/08/2015 |
| MOBILE, ALABAMA RADIO/WLO | USA | 003660003 | DSC-WATCH MED-ADVICE CP | 12/08/2014 |
| MSI SWEDEN | S | 002653500 | METEO NOTICE-NAV | 27/08/2015 |
| NEW YORK, NEW YORK RADIO | USA |  | CP | 26/04/2011 |
| OREN RADIO (ISTANBUL TURK RADIO ) | TUR | 002713000 | DSC-WATCH CP | 14/09/2011 |
| PALERMO RADIO | I | 002470002 | DSC-WATCH METEO NOTICE-NAV CP | 26/04/2011 |
| PARNITHA RADIO (OLYMPIA RADIO ) | GRC |  | CP | 26/01/2011 |
| PETALIDI RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| PORT SAID RADIO | EGY | 006221113 | DSC-WATCH CP | 27/04/2011 |
| PORTO TORRES RADIO | I | 002470001 002470133 | DSC-WATCH METEO NOTICE-NAV CP | 18/08/2015 |
| PRINCE RUPERT COAST GUARD RADIO | CAN | 003160013 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| QUEBEC COAST GUARD RADIO | CAN | 003160027 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| ROGALAND RADIO | NOR | 002570300 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| ROGERS CITY, MICHIGAN RADIO | USA |  | CP | 15/04/2011 |
| S. THOMAS, VIRGIN ISLANDS RADIO | VIR |  | CP | 15/04/2011 |
| SANTA CRUZ | B |  | CP | 18/08/2015 |
| STOCKHOLM RADIO | S | 002652000 | CP | 26/08/2015 |
| TJOME RADIO | NOR | 002570100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| TURKU RADIO | FIN | 002300230 | DSC-WATCH METEO NOTICE-NAV | 28/04/2011 |
| VARDO RADIO | NOR | 002570800 | DSC-WATCH MED-ADVICE NAVINFO METEO NOTICE-NAV CP | 26/04/2011 |
| VICTORIA COAST GUARD RADIO | CAN | 003160011 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| Number of records found = 38 |  |  |  |  |

Channel 25

| Station Name | Symbol | MMSI | Services | Updated |
| --- | --- | --- | --- | --- |
| AASIAAT RADIO | GRL | 003313000 | DSC-WATCH MED-ADVICE NOTICE-NAV CP | 14/04/2011 |
| ABADAN RADIO | IRN | 004224102 | DSC-WATCH CP PORTINFO | 14/04/2011 |
| ABBAS RADIO | IRN | 004224300 004225300 | DSC-WATCH CP PORTINFO | 14/04/2011 |
| ACAPULCO, GUERRERO RADIO (MAZATLAN, SINALOA RADIO ) | MEX | 003451810 | DSC-WATCH METEO NOTICE-NAV CP | 14/04/2011 |
| AL BIRK RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| ALEXANDRIA RADIO | EGY | 006221111 | DSC-WATCH NAVINFO CP | 15/04/2011 |
| ALGER RADIO | ALG | 006052110 | DSC-WATCH CP | 14/04/2011 |
| ANAMUR RADIO (ISTANBUL TURK RADIO ) | TUR | 002713000 | DSC-WATCH CP | 14/09/2011 |
| ANCONA RADIO | I | 002470001 002470119 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| ANNABA RADIO | ALG | 006053814 | DSC-WATCH CP | 14/04/2011 |
| ARKHANGELSK RADIO | RUS | 002734414 | DSC-WATCH CP | 15/04/2011 |
| ARRECIFE RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 20/07/2015 |
| BANGKOK RADIO | THA | 005671000 | DSC-WATCH CP | 14/04/2011 |
| BELEM MOSQUEIRO | B |  | CP | 18/08/2015 |
| BODO RADIO | NOR | 002570700 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| BOU HAROUN RADIO | ALG |  | CP | 06/08/2014 |
| BREMEN RADIO | D |  | CP | 18/12/2009 |
| BREVES | B |  | CP | 18/08/2015 |
| BUCHAN RADIO | G |  | MED-ADVICE METEO | 11/08/2014 |
| BUSHEHR RADIO | IRN | 004224200 004225200 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| CABO DE GATA RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 24/07/2015 |
| CAPE TOWN RADIO | AFS | 006010001 | DSC-WATCH METEO NOTICE-NAV CP | 24/10/2011 |
| CONSTANTA RADIO | ROU | 002640570 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV CP PORTINFO NAVAREA | 15/04/2011 |
| CUA ONG RADIO | VTN | 005741020 | DSC-WATCH METEO NOTICE-NAV CP | 02/09/2015 |
| CYPRUS RADIO | CYP | 002091000 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 14/04/2011 |
| DAMMAM RADIO | ARS |  | DSC-WATCH METEO CP | 14/04/2011 |
| DILEKTEPE RADIO (ISTANBUL TURK RADIO ) | TUR | 002713000 | DSC-WATCH CP | 14/09/2011 |
| DURBAN RADIO | AFS |  | METEO NOTICE-NAV CP | 25/10/2011 |
| ENSENADA, BAJA CALIFORNIA RADIO (MAZATLAN, SINALOA RADIO ) | MEX | 003450210 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| FLORO RADIO | NOR | 002570500 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 08/08/2014 |
| GENOVA RADIO | I | 002470001 002470125 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| GUANGZHOU RADIO | CHN | 004123100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV VTS CP | 15/04/2011 |
| GUNSAN RADIO | KOR | 004400201 004400501 004403501 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| HAI PHONG RADIO | VTN | 005741040 | DSC-WATCH METEO NOTICE-NAV CP | 23/09/2011 |
| HORNAFJOERDUR RADIO | ISL | 002510100 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| INCHEON RADIO | KOR | 004400003 004400004 004403001 | DSC-WATCH MED-ADVICE CP AIS | 03/08/2015 |
| ISLAY RADIO | G |  | MED-ADVICE METEO | 12/08/2014 |
| ITACOATIARA | B |  | CP | 18/08/2015 |
| JAKARTA RADIO/PKX | INS | 005250000 | DSC-WATCH NAVINFO METEO NOTICE-NAV UTC CP | 12/05/2011 |
| JEDDAH RADIO | ARS | 004030000 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 14/04/2011 |
| JEFFERSONVILLE, INDIANA RADIO | USA |  | CP | 15/04/2011 |
| JEJU RADIO | KOR | 004400701 004400702 004403701 | DSC-WATCH MED-ADVICE CP AIS | 09/07/2015 |
| JERSEY RADIO | G | 002320060 | DSC-WATCH METEO NOTICE-NAV CP | 22/01/2009 |
| JIZAN RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| KAMENNYY C YNAO/MYS KAMENNYY-STGM | RUS |  | VTS CP | 07/10/2014 |
| KANGNUNG RADIO | KOR | 004400602 004400603 004400604 | DSC-WATCH MED-ADVICE CP AIS | 26/04/2011 |
| KHORRAMSHAHR RADIO | IRN | 004224101 004225101 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| KOSSEIR RADIO | EGY | 006221112 | DSC-WATCH NAVINFO CP | 27/04/2011 |
| LAGUNA | B |  | CP | 18/08/2015 |
| LAMPEDUSA RADIO | I | 002470002 002470126 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| LENGEH RADIO | IRN | 004224302 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| LES ESCOUMINS COAST GUARD RADIO | CAN | 003160026 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| MAHYADAG RADIO (ISTANBUL TURK RADIO ) | TUR | 002711000 | DSC-WATCH CP | 27/04/2011 |
| MANZANILLO, COLIMA RADIO (MAZATLAN, SINALOA RADIO ) | MEX | 003451410 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| MARKIZ RADIO (ISTANBUL TURK RADIO ) | TUR | 002713000 | DSC-WATCH CP | 14/09/2011 |
| MAZARA DEL VALLO RADIO | I | 002470002 002470128 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| MAZATLAN, SINALOA RADIO | MEX | 003450810 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| MOBILE, ALABAMA RADIO/WLO | USA | 003660003 | DSC-WATCH MED-ADVICE CP | 12/08/2014 |
| MOKPO RADIO | KOR | 004400304 004400307 004400308 004400309 004400310 004403301 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| MONACO RADIO | MCO |  | METEO | 30/06/2015 |
| MOSSORO | B |  | CP | 18/08/2015 |
| MOSTAGANEM RADIO | ALG | 006054118 | DSC-WATCH CP | 25/11/2010 |
| MRCC NOUMEA | NCL | 005401000 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV | 06/09/2011 |
| MSI SWEDEN | S | 002653500 | METEO NOTICE-NAV | 27/08/2015 |
| NAPOLI RADIO | I | 002470002 002470130 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| NEVELSK RADIO | RUS |  | CP | 15/04/2011 |
| NEW YORK, NEW YORK RADIO | USA |  | CP | 26/04/2011 |
| NORDFRIESLAND RADIO | D |  | CP | 04/01/2010 |
| NOUMEA RADIO | NCL |  | CP AIS | 06/09/2011 |
| PARNIS RADIO | GRC |  | METEO NOTICE-NAV | 07/08/2014 |
| PARNITHA RADIO (OLYMPIA RADIO ) | GRC |  | CP | 26/01/2011 |
| PELOTAS | B |  | CP | 18/08/2015 |
| POHANG RADIO | KOR | 004400401 004400402 004403401 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| PORT ELIZABETH RADIO | AFS |  | METEO NOTICE-NAV CP | 25/10/2011 |
| PORT SAID RADIO | EGY | 006221113 | DSC-WATCH CP | 27/04/2011 |
| PUERTO AYORA RADIO | EQA | 007354757 | DSC-WATCH METEO NOTICE-NAV CP | 07/09/2015 |
| PUSAN RADIO | KOR | 004400101 004400103 004400104 004400105 004400106 004403102 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| QUEBEC COAST GUARD RADIO | CAN | 003160027 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| RIO NOVO DO SUL | B |  | CP | 18/08/2015 |
| ROGALAND RADIO | NOR | 002570300 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| ROMA RADIO | I | 002470001 | DSC-WATCH METEO NOTICE-NAV CP | 26/04/2011 |
| S. THOMAS, VIRGIN ISLANDS RADIO | VIR |  | CP | 15/04/2011 |
| SALVADOR-SEDE | B |  | CP | 18/08/2015 |
| SANTAREM | B |  | CP | 18/08/2015 |
| SAO MATEUS | B |  | CP | 18/08/2015 |
| SHANTOU | CHN |  | RCC(s) PORTINFO | 04/10/2011 |
| STOCKHOLM RADIO | S | 002652000 | CP | 26/08/2015 |
| TENES RADIO | ALG | 006052113 | DSC-WATCH CP | 14/04/2011 |
| THASSOS RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| TIANJIN RADIO | CHN | 004121100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV VTS CP | 15/04/2011 |
| TJOME RADIO | NOR | 002570100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| TORSHAVN RADIO | FRO | 002311000 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 28/04/2011 |
| TRIESTE RADIO | I | 002470001 002470135 | DSC-WATCH METEO NOTICE-NAV CP | 18/08/2015 |
| TUAPSE RADIO | RUS | 002734413 | DSC-WATCH CP | 13/01/2011 |
| TUNIS RADIO | TUN |  | CP | 14/04/2011 |
| TURKU RADIO | FIN | 002300230 | DSC-WATCH METEO NOTICE-NAV | 28/04/2011 |
| ULREUNG RADIO | KOR | 004400403 004400404 | DSC-WATCH MED-ADVICE CP | 26/04/2011 |
| ULSAN RADIO | KOR | 004400102 | DSC-WATCH MED-ADVICE CP | 12/11/2007 |
| UMM LUJJ RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| VARDO RADIO | NOR | 002570800 | DSC-WATCH MED-ADVICE NAVINFO METEO NOTICE-NAV CP | 26/04/2011 |
| VUNG TAU RADIO | VTN | 005743020 | DSC-WATCH METEO NOTICE-NAV CP | 03/02/2012 |
| WHITBY RADIO | G |  | MED-ADVICE METEO | 15/08/2014 |
| WITOWO RADIO | POL | 002610210 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 27/04/2011 |
| XIAMEN RADIO | CHN | 004122700 | DSC-WATCH VTS CP | 15/04/2011 |
| YEOSU RADIO | KOR | 004400305 004400306 | DSC-WATCH MED-ADVICE CP | 09/07/2015 |
| ZHANJIANG RADIO | CHN | 004123300 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV VTS CP AIS | 03/10/2011 |
| Number of records found = 106 |  |  |  |  |

Channel 85

| Station Name | Symbol | MMSI | Services | Updated |
| --- | --- | --- | --- | --- |
| ALEXANDRIA RADIO | EGY | 006221111 | DSC-WATCH NAVINFO CP | 15/04/2011 |
| ANGRA | B |  | CP | 17/08/2015 |
| AUGUSTA RADIO | I | 002470002 002470120 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| BERDYANSK RADIO | UKR | 002723672 | DSC-WATCH RCC(s) | 13/01/2011 |
| BODO RADIO | NOR | 002570700 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| CABO LA NAO RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 21/07/2015 |
| CAPE TOWN RADIO | AFS | 006010001 | DSC-WATCH METEO NOTICE-NAV CP | 24/10/2011 |
| CHIOS RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| DE PANNE RADIO (OOSTENDE RADIO ) | BEL |  | CP | 11/12/2001 |
| DELCAMBRE, LOUISIANA RADIO | USA |  | CP | 26/04/2011 |
| DUBROVNIK RADIO | HRV | 002380300 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 13/12/2011 |
| EMIRATES RADIO | UAE | 004700000 | DSC-WATCH CP | 26/04/2011 |
| FLORO RADIO | NOR | 002570500 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 08/08/2014 |
| GENOVA RADIO | I | 002470001 002470125 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| HMB (AZIZYA) RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| IQALUIT COAST GUARD RADIO | CAN | 003160023 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| KHAFJI RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| KOSSEIR RADIO | EGY | 006221112 | DSC-WATCH NAVINFO CP | 27/04/2011 |
| KYTHIRA RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| LAMPEDUSA RADIO | I | 002470002 002470126 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| LES ESCOUMINS COAST GUARD RADIO | CAN | 003160026 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| MESSINA RADIO | I | 002470002 002470129 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| MIDDELKERKE RADIO (OOSTENDE RADIO ) | BEL |  | CP | 11/12/2001 |
| MOBILE, ALABAMA RADIO/WLO | USA | 003660003 | DSC-WATCH MED-ADVICE CP | 12/08/2014 |
| MRSC MALIN HEAD RADIO | IRL | 002500100 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV SAR AGENCY | 13/09/2011 |
| OMAN COAST RADIO/A4A23 (ASHKARAH) | OMA | 004610016 | DSC-WATCH MED-ADVICE RCC(s) NOTICE-NAV CP | 19/05/2015 |
| OOSTENDE RADIO | BEL | 002050480 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 28/04/2011 |
| PALMA RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 27/07/2015 |
| PATRA RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| PLACENTIA COAST GUARD RADIO | CAN | 003160019 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| PORT SAID RADIO | EGY | 006221113 | DSC-WATCH CP | 27/04/2011 |
| PORTO CERVO RADIO | I | 002470001 002470132 | DSC-WATCH METEO NOTICE-NAV CP | 18/08/2015 |
| PRESCOTT COAST GUARD RADIO | CAN | 003160029 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| QUEBEC COAST GUARD RADIO | CAN | 003160027 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| ROGALAND RADIO | NOR | 002570300 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| S. THOMAS, VIRGIN ISLANDS RADIO | VIR |  | CP | 15/04/2011 |
| SARNIA COAST GUARD RADIO | CAN | 003160030 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| SITIA RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| THASSOS RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| TJOME RADIO | NOR | 002570100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| VARDO RADIO | NOR | 002570800 | DSC-WATCH MED-ADVICE NAVINFO METEO NOTICE-NAV CP | 26/04/2011 |
| YILDIZTEPE RADIO (ISTANBUL TURK RADIO ) | TUR | 002712000 | DSC-WATCH CP | 14/09/2011 |
| ZONGULDAK RADIO (ISTANBUL TURK RADIO ) | TUR | 002712000 | DSC-WATCH CP | 01/02/2012 |
| Number of records found = 43 |  |  |  |  |

Channel 26

| Station Name | Symbol | MMSI | Services | Updated |
| --- | --- | --- | --- | --- |
| AASIAAT RADIO | GRL | 003313000 | DSC-WATCH MED-ADVICE NOTICE-NAV CP | 14/04/2011 |
| AL LITH RADIO | ARS |  | DSC-WATCH CP | 17/06/2004 |
| ALEXANDRIA RADIO | EGY | 006221111 | DSC-WATCH NAVINFO CP | 15/04/2011 |
| ALGER RADIO | ALG | 006052110 | DSC-WATCH CP | 14/04/2011 |
| ALMEIRIM | B |  | CP | 17/08/2015 |
| AMBOINA RADIO/PKE | INS | 005250006 | DSC-WATCH METEO NOTICE-NAV CP | 04/05/2011 |
| ANGLESEY RADIO | G |  | MED-ADVICE METEO | 11/08/2014 |
| ANGRA | B |  | CP | 17/08/2015 |
| ANNABA RADIO | ALG | 006053814 | DSC-WATCH CP | 14/04/2011 |
| ARGENTINA RADIO | ARG | 007010111 | DSC-WATCH CP | 23/12/2014 |
| ARKHANGELSK RADIO | RUS | 002734414 | DSC-WATCH CP | 15/04/2011 |
| ARRECIFE RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 20/07/2015 |
| ASCENSION RADIO | ASC |  | CP | 15/04/2011 |
| AVALON, CALIFORNIA RADIO | USA |  | CP | 26/04/2011 |
| BAHIA RADIO | EQA | 007354753 | DSC-WATCH CP | 12/08/2015 |
| BALIKPAPAN RADIO/PKN | INS | 005250009 | DSC-WATCH NOTICE-NAV CP | 20/05/2011 |
| BANGKOK RADIO | THA | 005671000 | DSC-WATCH CP | 14/04/2011 |
| BANJARMASIN RADIO/PKG | INS | 005251520 | DSC-WATCH NOTICE-NAV CP | 12/06/2011 |
| BARI RADIO | I | 002470002 002470121 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| BATU LICIN RADIO | INS |  | CP | 26/04/2011 |
| BELAWAN RADIO/PKB | INS | 005250003 | DSC-WATCH METEO NOTICE-NAV CP | 20/05/2011 |
| BELEM-SEDE | B |  | CP | 18/08/2015 |
| BERMUDA RADIO | BER |  | MED-ADVICE METEO CP | 06/08/2014 |
| BITUNG RADIO/PKM | INS | 005250005 | DSC-WATCH METEO NOTICE-NAV CP | 20/05/2011 |
| BODO RADIO | NOR | 002570700 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| CABO DE GATA RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 24/07/2015 |
| CAMLICA RADIO (ISTANBUL TURK RADIO ) | TUR | 002711000 | DSC-WATCH CP | 27/04/2011 |
| CAPE TOWN RADIO | AFS | 006010001 | DSC-WATCH METEO NOTICE-NAV CP | 24/10/2011 |
| CHETUMAL, QUINTANA ROO RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003451120 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| CILACAP RADIO/PKR3 | INS | 005250030 | DSC-WATCH NOTICE-NAV CP | 20/05/2011 |
| CIUDAD DEL CARMEN, CAMPECHE RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003450710 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| CLIFDEN COAST GUARD RADIO (MRSC MALIN HEAD RADIO ) | IRL |  | MED-ADVICE RCC(s) METEO NOTICE-NAV | 13/09/2011 |
| CLYDE RADIO | G |  | MED-ADVICE METEO | 13/04/2011 |
| COATZACOALCOS, VERACRUZ RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003450320 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| COMOX COAST GUARD RADIO | CAN | 003160014 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| CONSTANTA RADIO | ROU | 002640570 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV CP PORTINFO NAVAREA | 15/04/2011 |
| CORK COAST GUARD RADIO (MRSC VALENTIA RADIO ) | IRL |  | MED-ADVICE RCC(s) METEO NOTICE-NAV | 16/09/2011 |
| CORUNA RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 24/07/2015 |
| COZUMEL, QUINTANA ROO RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003451110 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| CULLERCOATS RADIO | G |  | MED-ADVICE METEO NOTICE-NAV | 12/08/2014 |
| CYPRUS RADIO | CYP | 002091000 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 14/04/2011 |
| DALIAN RADIO | CHN | 004121300 | DSC-WATCH METEO NOTICE-NAV VTS PILOT CP | 30/09/2011 |
| DJIBOUTI RADIO | DJI | 006211000 | DSC-WATCH CP | 26/04/2011 |
| DOHA RADIO | QAT | 004661010 | DSC-WATCH METEO NOTICE-NAV CP | 26/04/2011 |
| DUBA RADIO | ARS |  | DSC-WATCH CP | 17/06/2004 |
| DUMAI RADIO/PKP | INS | 005250004 | DSC-WATCH METEO NOTICE-NAV CP | 27/04/2011 |
| DURBAN RADIO | AFS |  | METEO NOTICE-NAV CP | 25/10/2011 |
| DUTMEN RADIO (ISTANBUL TURK RADIO ) | TUR | 002712000 | DSC-WATCH CP | 14/09/2011 |
| FERNANDO DE NORONHA | B |  | CP | 18/08/2015 |
| FESTOS RADIO (OLYMPIA RADIO ) | GRC |  | CP | 26/01/2011 |
| FLORIANOPOLIS | B |  | CP | 18/08/2015 |
| FLORO RADIO | NOR | 002570500 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 08/08/2014 |
| GENAVEH RADIO | IRN | 004225206 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| GRESIK RADIO | INS |  | CP | 27/04/2011 |
| GUAYAQUIL RADIO | EQA | 007354750 | DSC-WATCH NOTICE-NAV CP | 26/04/2011 |
| GUNSAN RADIO | KOR | 004400201 004400501 004403501 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| HALIFAX COAST GUARD RADIO | CAN | 003160016 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| HEBRIDES RADIO | G |  | MED-ADVICE METEO | 12/08/2014 |
| HO CHI MINH RADIO | VTN | 005743030 | DSC-WATCH METEO NOTICE-NAV CP | 23/09/2011 |
| HUMBER RADIO | G |  | MED-ADVICE METEO NOTICE-NAV | 12/08/2014 |
| INCHEON RADIO | KOR | 004400003 004400004 004403001 | DSC-WATCH MED-ADVICE CP AIS | 03/08/2015 |
| IQALUIT COAST GUARD RADIO | CAN | 003160023 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| ITAOCA | B |  | CP | 18/08/2015 |
| JAKARTA RADIO/PKX | INS | 005250000 | DSC-WATCH NAVINFO METEO NOTICE-NAV UTC CP | 12/05/2011 |
| JAYAPURA RADIO/PNK | INS | 005250007 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 20/05/2011 |
| JEFFERSONVILLE, INDIANA RADIO | USA |  | CP | 15/04/2011 |
| JEJU RADIO | KOR | 004400701 004400702 004403701 | DSC-WATCH MED-ADVICE CP AIS | 09/07/2015 |
| JUBAIL RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| KANGNUNG RADIO | KOR | 004400602 004400603 004400604 | DSC-WATCH MED-ADVICE CP AIS | 26/04/2011 |
| KARIMUNJAWA RADIO | INS |  | CP | 27/04/2011 |
| KEFALLINIA RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| KHOMEINI RADIO | IRN | 004224100 004225100 | DSC-WATCH CP PORTINFO | 23/07/2015 |
| KOSSEIR RADIO | EGY | 006221112 | DSC-WATCH NAVINFO CP | 27/04/2011 |
| KUMAI RADIO | INS | 005251522 | DSC-WATCH CP | 12/06/2011 |
| KUPANG RADIO/PKK | INS | 005250010 | DSC-WATCH NOTICE-NAV CP | 17/01/2012 |
| LABRADOR COAST GUARD RADIO | CAN | 003160022 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| LAVAR RADIO | IRN | 004225204 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| LAZARO CARDENAS, MICHOACAN RADIO (MAZATLAN, SINALOA RADIO ) | MEX | 003451610 | DSC-WATCH METEO NOTICE-NAV CP | 06/03/2012 |
| LES ESCOUMINS COAST GUARD RADIO | CAN | 003160026 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| LIVORNO RADIO | I | 002470001 002470127 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| MACAPA | B |  | CP | 18/08/2015 |
| MACHICHACO RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 24/07/2015 |
| MAKASSAR RADIO/PKF | INS | 005250002 | DSC-WATCH METEO NOTICE-NAV CP | 27/04/2011 |
| MANAUS-RADIO (PONTA DAS LAJES) (MRCC BRAZIL ) | B | 007100003 | DSC-WATCH CP | 15/09/2015 |
| MANTA RADIO | EQA | 007354754 | DSC-WATCH CP | 26/04/2011 |
| MAZARA DEL VALLO RADIO | I | 002470002 002470128 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| MERAUKE RADIO | INS | 005250021 | DSC-WATCH CP PORTINFO | 27/04/2011 |
| MOBILE, ALABAMA RADIO/WLO | USA | 003660003 | DSC-WATCH MED-ADVICE CP | 12/08/2014 |
| MOKPO RADIO | KOR | 004400304 004400307 004400308 004400309 004400310 004403301 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| MORRO DO MEIO | B |  | CP | 18/08/2015 |
| MRCC NOUMEA | NCL | 005401000 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV | 06/09/2011 |
| MRCC PAPEETE | OCE | 005461000 | DSC-WATCH MED-ADVICE RCC(s) METEO NOTICE-NAV | 06/09/2011 |
| MSI SWEDEN | S | 002653500 | METEO NOTICE-NAV | 27/08/2015 |
| MURMANSK RADIO | RUS | 002733744 | CP | 15/04/2011 |
| MUSCAT RADIO | OMA |  | MED-ADVICE CP | 26/04/2011 |
| NATAL | B |  | CP | 18/08/2015 |
| NEW YORK, NEW YORK RADIO | USA |  | CP | 26/04/2011 |
| NORDFRIESLAND RADIO | D |  | CP | 04/01/2010 |
| NORTH FORELAND RADIO | G |  | MED-ADVICE METEO NOTICE-NAV | 14/08/2014 |
| NOUMEA RADIO | NCL |  | CP AIS | 06/09/2011 |
| OBHUR RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| OLINDA | B |  | CP | 15/09/2015 |
| OMAN COAST RADIO/A4A21 (SALALAH) | OMA | 004610014 | DSC-WATCH MED-ADVICE RCC(s) NAVINFO NOTICE-NAV CP | 19/05/2015 |
| OMAN COAST RADIO/A4A22 (KHASAB) | OMA | 004610015 | DSC-WATCH MED-ADVICE RCC(s) NOTICE-NAV CP | 04/06/2015 |
| ORKNEY RADIO | G |  | MED-ADVICE METEO | 14/08/2014 |
| PALAMUT RADIO (ISTANBUL TURK RADIO ) | TUR | 002713000 | DSC-WATCH CP | 14/09/2011 |
| PALEMBANG RADIO | INS | 005251507 | DSC-WATCH METEO NOTICE-NAV CP | 12/06/2011 |
| PALOPO RADIO | INS | 005250071 | DSC-WATCH CP | 03/06/2011 |
| PANTOLOAN RADIO | INS | 005250018 | DSC-WATCH CP | 27/04/2011 |
| PARINTINS | B |  | CP | 18/08/2015 |
| PLACENTIA COAST GUARD RADIO | CAN | 003160019 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| POHANG RADIO | KOR | 004400401 004400402 004403401 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| PONTIANAK RADIO/PKS | INS | 005250016 | DSC-WATCH NOTICE-NAV CP PORTINFO | 27/04/2011 |
| POROS RADIO (OLYMPIA RADIO ) | GRC |  | NOTICE-NAV CP | 26/01/2011 |
| PORT AUX BASQUES COAST GUARD RADIO | CAN | 003160018 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| PORT ELIZABETH RADIO | AFS |  | METEO NOTICE-NAV CP | 25/10/2011 |
| PORT SAID RADIO | EGY | 006221113 | DSC-WATCH CP | 27/04/2011 |
| PORTO CERVO RADIO | I | 002470001 002470132 | DSC-WATCH METEO NOTICE-NAV CP | 18/08/2015 |
| PORTO TORRES RADIO | I | 002470001 002470133 | DSC-WATCH METEO NOTICE-NAV CP | 18/08/2015 |
| PRESCOTT COAST GUARD RADIO | CAN | 003160029 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| PRINCE RUPERT COAST GUARD RADIO | CAN | 003160013 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| PROGRESO, YUCATAN RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003450910 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| PUERTO BOLIVAR RADIO | EQA | 007354756 | DSC-WATCH CP | 26/04/2011 |
| PUERTO VALLARTA, JALISCO RADIO (MAZATLAN, SINALOA RADIO ) | MEX | 003451210 | DSC-WATCH METEO NOTICE-NAV CP | 09/10/2008 |
| PUSAN RADIO | KOR | 004400101 004400103 004400104 004400105 004400106 004403102 | DSC-WATCH MED-ADVICE CP AIS | 04/09/2013 |
| QINHUANGDAO RADIO | CHN | 004121200 | DSC-WATCH VTS PORTINFO | 03/10/2011 |
| QUEBEC COAST GUARD RADIO | CAN | 003160027 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| REYKJAVIK RADIO | ISL | 002510100 | DSC-WATCH METEO NOTICE-NAV CP | 27/04/2011 |
| ROGALAND RADIO | NOR | 002570300 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| ROGERS CITY, MICHIGAN RADIO | USA |  | CP | 15/04/2011 |
| S. FRANCISCO, CALIFORNIA RADIO/KMH828 | USA |  | MED-ADVICE | 26/04/2011 |
| S. HELENA RADIO | SHN |  | MED-ADVICE CP | 15/04/2011 |
| SABANG RADIO/PKA | INS |  | NOTICE-NAV CP | 28/04/2011 |
| SALALAH RADIO | OMA |  | MED-ADVICE CP | 27/05/2008 |
| SALINAS RADIO | EQA | 007354755 | DSC-WATCH CP | 26/04/2011 |
| SAMPIT RADIO | INS | 005251521 | DSC-WATCH CP PORTINFO | 12/06/2011 |
| SARNIA COAST GUARD RADIO | CAN | 003160030 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| SELAYAR RADIO | INS |  | CP | 27/04/2011 |
| SEMARANG RADIO/PKR | INS | 005250008 | DSC-WATCH NOTICE-NAV CP PORTINFO | 27/04/2011 |
| SHANTOU | CHN |  | RCC(s) PORTINFO | 04/10/2011 |
| SHUQAIQ RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| SIGLUFJOERDUR RADIO | ISL | 002510100 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| SKIKDA RADIO | ALG | 006053816 | CP | 14/04/2011 |
| SORONG RADIO/PKY4 | INS | 005250011 | DSC-WATCH NOTICE-NAV CP PORTINFO | 27/04/2011 |
| START POINT RADIO | G |  | MED-ADVICE METEO | 15/08/2014 |
| STOCKHOLM RADIO | S | 002652000 | CP | 26/08/2015 |
| STONEHAVEN RADIO | G |  | MED-ADVICE METEO NOTICE-NAV | 15/08/2014 |
| SURABAYA RADIO/PKD | INS | 005250001 | DSC-WATCH METEO NOTICE-NAV CP | 27/04/2011 |
| SYDNEY COAST GUARD RADIO | CAN | 003160017 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| TALLINN RADIO | EST | 002761000 | DSC-WATCH MED-ADVICE NAVINFO METEO NOTICE-NAV CP | 16/04/2012 |
| TAMPICO, TAMAULIPAS RADIO | MEX | 003450110 | DSC-WATCH CP | 15/04/2011 |
| TARAKAN RADIO/PKO | INS | 005250017 | DSC-WATCH CP PORTINFO | 20/05/2011 |
| TARIFA RADIO | E |  | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 23/07/2015 |
| TEGAL RADIO/PKR26 | INS | 005251513 | DSC-WATCH CP PORTINFO | 12/06/2011 |
| TELUK BAYUR RADIO/PKV | INS | 005250075 | DSC-WATCH CP | 08/02/2012 |
| TIANJIN RADIO | CHN | 004121100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV VTS CP | 15/04/2011 |
| TJOME RADIO | NOR | 002570100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| TORSHAVN RADIO | FRO | 002311000 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 28/04/2011 |
| TRIESTE RADIO | I | 002470001 002470135 | DSC-WATCH METEO NOTICE-NAV CP | 18/08/2015 |
| TSOUKALAS RADIO (OLYMPIA RADIO ) | GRC |  | NOTICE-NAV CP | 26/01/2011 |
| TUAPSE RADIO | RUS | 002734413 | DSC-WATCH CP | 13/01/2011 |
| TUNIS RADIO | TUN |  | CP | 14/04/2011 |
| TURKU RADIO | FIN | 002300230 | DSC-WATCH METEO NOTICE-NAV | 28/04/2011 |
| ULREUNG RADIO | KOR | 004400403 004400404 | DSC-WATCH MED-ADVICE CP | 26/04/2011 |
| ULSAN RADIO | KOR | 004400102 | DSC-WATCH MED-ADVICE CP | 12/11/2007 |
| VARDO RADIO | NOR | 002570800 | DSC-WATCH MED-ADVICE NAVINFO METEO NOTICE-NAV CP | 26/04/2011 |
| VARNA RADIO | BUL | 002070810 | DSC-WATCH METEO NOTICE-NAV CP | 26/04/2011 |
| VERACRUZ, VERACRUZ RADIO (TAMPICO, TAMAULIPAS RADIO ) | MEX | 003450310 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| VESTMANNAEYJAR RADIO | ISL | 002510100 | DSC-WATCH METEO NOTICE-NAV CP | 15/04/2011 |
| VICTORIA COAST GUARD RADIO | CAN | 003160011 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| WALVIS BAY RADIO | NMB |  | MED-ADVICE METEO NOTICE-NAV CP | 15/04/2011 |
| WITOWO RADIO | POL | 002610210 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 27/04/2011 |
| XIAMEN RADIO | CHN | 004122700 | DSC-WATCH VTS CP | 15/04/2011 |
| YANBU RADIO | ARS |  | DSC-WATCH CP | 28/06/2004 |
| YEOSU RADIO | KOR | 004400305 004400306 | DSC-WATCH MED-ADVICE CP | 09/07/2015 |
| Number of records found = 175 |  |  |  |  |

Channel 86

| Station Name | Symbol | MMSI | Services | Updated |
| --- | --- | --- | --- | --- |
| AL ASHKARA RADIO | OMA |  | MED-ADVICE CP | 12/08/2008 |
| ALEXANDRIA RADIO | EGY | 006221111 | DSC-WATCH NAVINFO CP | 15/04/2011 |
| AMIRABAD RADIO | IRN | 004224601 004225601 | DSC-WATCH CP PORTINFO | 26/04/2011 |
| AUGUSTA RADIO | I | 002470002 002470120 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| BODO RADIO | NOR | 002570700 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| CAPE TOWN RADIO | AFS | 006010001 | DSC-WATCH METEO NOTICE-NAV CP | 24/10/2011 |
| CAVALINHO | B |  | CP | 18/08/2015 |
| EMIRATES RADIO | UAE | 004700000 | DSC-WATCH CP | 26/04/2011 |
| FLORO RADIO | NOR | 002570500 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 08/08/2014 |
| ITAOCA | B |  | CP | 18/08/2015 |
| KOSSEIR RADIO | EGY | 006221112 | DSC-WATCH NAVINFO CP | 27/04/2011 |
| KYTHIRA RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| MAZARA DEL VALLO RADIO | I | 002470002 002470128 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| MOBILE, ALABAMA RADIO/WLO | USA | 003660003 | DSC-WATCH MED-ADVICE CP | 12/08/2014 |
| NAPOLI RADIO | I | 002470002 002470130 | DSC-WATCH METEO NOTICE-NAV CP | 17/08/2015 |
| ODESSA MRCC | UKR | 002723660 | DSC-WATCH RCC(s) | 12/09/2013 |
| PORT SAID RADIO | EGY | 006221113 | DSC-WATCH CP | 27/04/2011 |
| PORTO CERVO RADIO | I | 002470001 002470132 | DSC-WATCH METEO NOTICE-NAV CP | 18/08/2015 |
| PYRGOS RADIO (OLYMPIA RADIO ) | GRC |  | NOTICE-NAV CP | 04/11/2003 |
| ROGALAND RADIO | NOR | 002570300 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| ROGERS CITY, MICHIGAN RADIO | USA |  | CP | 15/04/2011 |
| S. BENEDETTO DEL TRONTO RADIO | I | 002470001 002470134 | DSC-WATCH METEO NOTICE-NAV CP | 18/08/2015 |
| S. DIEGO, CALIFORNIA RADIO | USA |  | MED-ADVICE CP | 26/04/2011 |
| SITIA RADIO (OLYMPIA RADIO ) | GRC |  | METEO NOTICE-NAV CP | 26/01/2011 |
| SKYROS RADIO (OLYMPIA RADIO ) | GRC |  | NOTICE-NAV CP | 26/01/2011 |
| TJOME RADIO | NOR | 002570100 | DSC-WATCH MED-ADVICE METEO NOTICE-NAV CP | 26/04/2011 |
| TORTOLA RADIO | VRG |  | CP | 12/08/1993 |
| TRIESTE RADIO | I | 002470001 002470135 | DSC-WATCH METEO NOTICE-NAV CP | 18/08/2015 |
| VARDO RADIO | NOR | 002570800 | DSC-WATCH MED-ADVICE NAVINFO METEO NOTICE-NAV CP | 26/04/2011 |
| Number of records found = 29 |  |  |  |  |