Input paper:   ENAV19-11.18

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					x  Input

x  ENAV	□  VTS							□  Information

Agenda item 			11

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			N Ward

# Use of AIS Data in Court Cases

# Summary

This paper draws on previous documents to provide a basis for developing guidance on the use of AIS data in court cases. It makes reference to the previous documents and suggests a format for the guidance information.

## Purpose of the document

The Committee is invited to consider using the paper to develop guidance on the use of AIS Data in Court Cases.

## Related documents

Input paper ENAV 15-10.17 and Final Report of Council 62.

# Background

A paper was submitted to ENAV 15 drawing attention to the vulnerability of AIS to interference and spoofing and proposing guidance for Members on preventive measures. This was prompted in part by published reports on the ease of spoofing AIS reports on publicly available websites. The vulnerabilities of AIS were well-known, but some advice to IALA Members was considered appropriate to ensure that precautions were taken against malicious interference.

At the same time the use of AIS data in court cases was becoming more common and so the vulnerabilities were drawn to the attention of the IALA Legal Advisory Panel. Subsequently LAP reported to the IALA Council on the admissibility of AIS evidence and as a result the Council invited LAP and the ENAV Committee to work together with the aim of developing guidance on AIS data when they are to be used as evidence at court cases (C62 Final Report p.16).

# Discussion

The essential information from the ENAV 15 paper is reproduced in the Annex, edited to form the basis for a guidance note.

# References

ENAV 15-10.17

C62 Final Report

# Action requested of the Committee

The Committee is invited to consider the text in the Annex as a basis for developing guidance on the use of AIS Data in Court Cases and forward to LAP17.

- Annex 1

Introduction

The Automatic Identification System (AIS) is an internationally standardised, VHF Broadcast system providing identity, position and status of ships. Carriage of Class A AIS units is required for SOLAS Convention vessels. Class B equipment is now carried voluntarily on many non-SOLAS vessels, including leisure and fishing craft and service vessels. AIS is also used to provide Aids to Navigation (AtoN) on fixed and floating platforms.  The position of vessels and floating AtoN used by AIS is generally provided by GPS, although other electronic position fixing systems (EPFS) could be used.

Robustness of AIS Data

The robustness of AIS Data should be considered if it is to be used in court cases. AIS was originally provided for safety reasons, not for security purposes, so it was never designed to be resistant to malicious interference.

Studies have identified the following potential causes of failure:

•	Incorrect data input to AIS unit

•	Disruption to GNSS (GPS)

•	Failure of AIS unit

•	Degradation of VHF propagation

•	Loss of VHF reception

•	Control system malfunction

The first two of these causes can be brought about by deliberate broadcast of false information, including manipulation of GPS, so-called spoofing - for example a false position or identity can be broadcast. The recent suggestion that false DGNSS corrections could be broadcast introduces another approach. In common with other navigation and communications systems, AIS should not be relied upon as the sole source of information.

Precautions

Precautions can be taken by Administrations providing services via AIS, such as AIS AtoN or DGNSS corrections, or using AIS data for analysis purposes. For example the following measures can be considered:

All AIS information should be verified by some other means, for example radar or VHF DF

Any apparently false or anomalous signals should be investigated by cross-checking with other AIS receiving stations and displays

The integrity of broadcast information, especially AIS AtoN and DGNSS corrections, should be monitored to ensure that identity, position, status and other message contents are correct

The integrity of GPS (or other EPFS) should be monitored, for example by Differential GPS

The validity of any DGNSS corrections provided via Message 17 should be continuously checked against corrections provided by authorised reference stations with integrity monitoring, such as those used with MF beacons

Equipment used to broadcast AIS signals, such as base stations, should be located in secure premises and unauthorised access should not be possible

AIS monitoring provided via the internet should only be relied upon if control and access are provided by secure systems.

Local Navigation Warnings should be issued if false AIS signals are being broadcast

The source of any false AIS signals should be identified and preventive action taken

Conclusions

If the vulnerabilities discussed above are taken into consideration and the advised precautions have been taken, then AIS data may be considered sufficiently reliable for use in court cases.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain