# Report from CAMBRIDGE 2016 Intersessional

# IALA ENAV Working Group 3 – Telecommunication

The IALA eNAV working group 3 telecommunication met at an intersessional meeting in Cambridge, Canada the week of June 20, 2016, hosted by exactEarth.  The agenda called for progress on the following objectives:

- Discuss outcome of WP5B meeting (currently happening in Geneva)

  - Russian input for VDE-SAT

  - Status of 2092

  - Status of 1371

  - Status AI 1.9.2; PDNReport ITU-R M. [VDE-SAT]

  - Status AI 1.9.1 ;PDNReport ITU-R M. [Maritime Autonomous Radio Device]

  - Status AI 1.8; Modernization of GMDSS

- Finalize channel model document

- Finalize and harmonize bulletin board message structure

- Discuss capacity estimation (briefly presented by Krzysztof at eNAV18)

- Discuss with a goal to finalize the base modulation for the VDE channels

- Continue work on PDNReport ITU-R M.[VDE SAT]

- Update Maritime Radio Communication Plan (MRCP)

The input documents can be found on the e-navigation.nl website under Filebrowser then select the directory 20160620 Intersessional Kanada/Input, or alternatively can be accessed on the FTP:

ftp: ftp.e-navigation.nl

username: ftpuser

password: @C0penhagen

Under the Directory:    201606 Intersessional Kanada/Working Input

The output documents can be found on the e-navigation.nl website under Filebrowser then select the directory 20160118 Intersessional Dublin/Output, or alternatively can be accessed on the FTP:

ftp: ftp.e-navigation.nl

username: ftpuser

password: @C0penhagen

Under the Directory:    2016060 Intersessional Kanada /Output

The group began discussions with a report from Christian Rissone on the output of the May ITU WP5B meeting.  During this meeting the Russian delegation provided a paper describing the issue they see and the testing they believe will be required to justify the VDE-SAT channel allocation.  The Russian paper has now been incorporated into our VDE-SAT report.   The group plans to prepare a draft version of the report for submission to ITU WP5B in November. The following actions were recorded in the Change Log:

| Description of Change | Action |
| --- | --- |
| section 3.2: Potential use of the frequency band 160.975-161.475 MHz versus channels 2024/2084/2025/2085/2026/2086 for the satellite downlink | draft section 3 |
| section 4: Technical description of the VDE-SAT | Update Annexes 4, 5, 6 of 2092 and put it into the document section 4 |
| section 8: Testing, demonstrations and measurements | provide input |
| liaison note to WP 5B regarding the 500 kHz gap to be used for sat downlink | describe the options 1) as in 2092; 2) 500 kHz |

The group went on to discuss the current status of 2092 rev 0. ITU has agreed to allow the standard to remain unchanged until 2017 and possibly 2019 if the group believes this would be helpful.  Discussions were held on the pros and cons of keeping the version static until 2019.  In the end, the group has resolved to publish rev1 in 2017, without the satellite frequencies specified, and will prepare a liaison to ITU at eNAV19.

Next on the agenda was the current status of 1371, the ITU has agreed to open this document at the request of China.  China wishes to add the BeiDou GNSS system to the 1371 specification.   The group will prepare additional input for eNAV19 and will submit to ITU WP5B at the November meeting. These changes may include a new message type 28 that would be use to indicate ASM and VDE capabilities from shore and ship.  In addition, this would be an excellent opportunity to eliminate channel management in AIS.  Finally, IEC has provided further clarifications that need to be incorporated.

The discussion around AI 1.9.1 ;PDNReport ITU-R M. [Maritime Autonomous Radio Device]. The proposal is to provide additional frequencies to support this functionality however, this may not satisfy the commercial needs driving these type devices.  The group did not come to a final conclusion except that the group did agree that this is an issue that should be dealt with in a way that allows such innovative applications.   Further discussions will be held at eNAV19.

Similarly the discussion on GMDSS modernization did not lead to any actionable items beyond continuing to monitor the progress of the correspondence group.

The channel model document was discussed in detail and while there is still a lot of work to do, the progress made intersessionally has been significant.   The group intends to send a draft copy of this report to the ITU WP5B meeting in November to inform ITU on all of the work being done to support the development of 2092.  The following actions were recorded in the Change Log:

| Description of Change | Action |
| --- | --- |
| General Edits | change name to VDES radio communications model - and update user case scenarios |
|  | structure of the document: Intro, Cases, Ref- models, default models, construction models …. Appendices |
|  | provide input on Tropo-scatter |
|  | ship to ship; provide information, e.g. theoretical model for path loss |
|  | ship to aircraft; information on low angel satellite, more info needed |
|  | aircraft to ship; aviation industry? |
|  | ship to satellite; provide data |
|  | ship to satellite; analyse data |
|  | provide information on path loss |
|  | provide shadowing data |
|  | Ship height/hull design |
|  | noise and interference on the bridge for a range of ships |
| dynamic ship movement | provide data on dynamic ship movement, e.g. angel, heel and role, antenna height 10" to 60" + 4m (IMO sea area A1) -> send e-mail request to Jorge |
| man made noise | noise and interference on the bridge for a range of ships |
| interference | field measurements; caused by radar |
| antennas | antenna characteristics and imperfection; typical, range |
| multi path channel model |  |

The VDE-TER and VDE-SAT bulletin boards have been harmonized and in fact reduced to a two slot message.   The editorial changes are being integrated into 2092 and will be an official OUTPUT of eNAV19.   We were able to accomplish this by providing pointers to ancillary data such as the Link Configuration Table.   This table is used to define the various modulation and message lengths which also allowed us to combine fields in the packet structure such that the MCS and length is now a single field.  The group has also agreed that we need a default channel configuration, for those cases when a valid bulletin board is not present on the VDL. The group has been tasked with developing use-cases describing when the default would be used.

The group had a lengthy discussion on the risks of both frequency (AIS) and time management of the VDL. We also had a discussion on the separation of the VDE-TER upper legs and the ASM/AIS channels.  There is concern that while filtering is possible, it will degrade the performance of AIS reception.   A sub-group has been tasked with analyzing this possibility and providing possible solutions.  The group also recognized that is we are able to secure the frequencies in the 500kHz gap, the need for frequency management might not be necessary.   In fact, the group isn’t sure that we need to build in the complexity of frequency management if it won’t be required.  We have agreed to keep frequency management in the specification for now but will discuss again at eNAV19.  The group also agreed to prepare a liaison to ITU explaining why moving the satellite component to the 500kHz gap will simplify the entire VDES as sharing between VDE-TER and VDE-SAT would no longer be required.  This liaison will outline the three alternatives, keep the current allocation without change, move only the satellite down component, and lastly move satellite up and down.

The group discussed capacity calculations previously provided by Krzysztof and have concluded that logical channels controlled via the bulletin board will, in fact, provide sufficient capacity to support the expected services provided by the VDE.   The chair will liaise with the User Group to verify that the assumptions used in the calculations are relevant to their use-cases.  In particular we need to understand if the capacity assumed matches user expectations and we also need to understand how often messages will need to be repeated in order to satisfy the user requirement. Further, if we have not provided sufficient separation between the upper legs and the ASM/AIS channels the VDE-TER would have to consider the AIS loading in which case limited number of slots might be available.  An intersessional group has been formed to examine this problem including the extent of the problem.

This led to a discussion on slot selection on the ASM channels it was agreed that for the ship terminals the current slot selection criteria described in 1371 would be used and for the shore FATDMA would be used when Free Slots are not available.

The group went on to discuss a proposal for a harmonized packet structure and have agreed that this is possible and will be integrated into 2092 for output at eNAV19.  With the packet structure defined we can begin defining message content and hope to have this completed by eNAV19 as well.

The group has also agreed to create an ancillary design justification document that will be the repository for data and studies supporting the design decisions within 2092.

The MRCP will be updated to reflect changes from WRC15.  This will be done intersessionally.

# eNAV committee requirements progress summary

* On target, behind target but under control behind target needs action

| Task | Start Session | Planned End Session | Revised End Session | Progress Indicator |  |  | Status Overview |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Green | Yellow | Red |  |
| WG3 Telecommunications |  |  |  |  |  |  |  |
| 1.4.1 Develop VDES Message Structures | 18 | 21 |  | ☒ | ☐ | ☐ | Commence ENAV18 |
| 1.4.2 Assist in the Development of Message Structures for e-Nav | 18 | 21 |  | ☒ | ☐ | ☐ | Commence ENAV18 provided WG1 is ready |
| 2.1.1 Update the Marine Radio Communication Plan | 20 | 21 |  | ☒ | ☐ | ☒ | Commence ENAV20 |
| 2.1.2 Develop Recommendation on VDES | 15 | 21 |  | ☒ | ☐ | ☐ | On-going |
| 2.1.3 Organise a Workshop on VHF Data Exchange System | 17 | 17 |  | ☒ | ☐ | ☐ | Complete |
| 2.2.1 Update IALA Recommendations and Guidelines for AIS and VDES | 15 | 21 |  | ☒ | ☐ | ☐ | On-going |
| 2.3.1 Manage Application Specific Message (ASM) catalogue | 20 | 21 |  | ☒ | ☐ | ☐ | Complete until ENAV20 |
| 3.2.1 Liaise with ARM regarding Virtual AtoN | 17 | 17 |  | ☒ | ☐ | ☐ | Complete |

# Output Papers

The output papers will be referred to IALA council for approval, then submitted to ITU.

| Number |  | Title | Status |
| --- | --- | --- | --- |
| Output 1 |  | Report on Comms Intersessional Cambridge Canada June 2016 | Final |
| Output 2 |  | 20160623 V3_ITU-R-REC-M.2092-+ WD PDNR VDE-SAT changelog | Working Document |

# Attendees

| Name | E-mail address |
| --- | --- |
| Kensuke Arisaka | Arisaka.kensuke@jrc.co.jp |
| Antti Kukkonen | antti.kukkonen@furuno.fi |
| Peggy Browning | peggy.browning@exactearth.com |
| Krzysztof Bronk | K.Bronk@itl.waw.pl |
| Jan Safar | jan.safar@gla-rrnav.org |
| Christian Rissone | christian.rissone@anfr.fr |
| Ross Norsworthy | Ross_Norsworthy@msn.com |
| Hans-Christian Haugli | hans.christian.haugli@spacenorway.no |
| Werner Hauptfleisch | Werner.h@imisglobal.com |
| Jorge Arroyo | jorge.arroyo@uscg.mil |
| Ernie Batty | Ernie.b@imisglobal.com |
| Johnny Schultz | johnnyschultz@odysseyconsult.com |
| Yoshio Miyadera | Miyadera.yoshio@jrc.co.jp |
| Lars Loge | lars.loge@statsat.no |
| Masayuki Takahashi | Takahashi.masayuki@jrc.co.jp |
| Peter Kito | peterkito@jrcamerica.com |
| Timothy Dyson | Tdyson01@harris.com |
| Cato Eliassen | Cato.eliassen@kongsberg.com |
| Tom Meloche | Tom.meloche@l-3com.com |
| Francisco Lazaro | Francisco.lazaroblasco@dlr.de |
| Pieter Winter | pieter.winter@stonethree.com |
| Donna Kocak | dkocak@harris.com |
| Hiroo Naruse | Naruse.hiroo@jrc.co.jp |
| Stefan Bober | stefan.bober@wsv.bund.de |
| Arunas Macikunas | arunas.macikunas@exactearth.com |
| Nader Alagha | nader.alagha@esa.int |
| Hiroyasu Nakagawa | Hiroyasu.nakagawa@furuno.jp |
| Takamasa Yauchi | Yauchi427@oki.com |
| Jean-Francois Coutu | Jean-francois.coutu@dfo-mpo.gc.ca |
| Ronald Raulefs | Ronald.raulefs@dlr.de |