MSPs fully within IALA’s domain

- MSP 1 – VTS Information Service (IS)

| Service definition owner | Contributors (other stakeholders in service definition) | Users (Service provider and consumer) | Associated S-100 Product Specification(s) | Body responsible for defining technical specifications of service | GAP / overlap |
| --- | --- | --- | --- | --- | --- |
| IALA VTS Committee | Other IALA Committees | VTS, Navigator, Ports, Allied services | S-210 (Inter-VTS Exchange format) S-123 (Radio Services) S-124 (Nav warnings) S-111 (plus other met-services) | TBD | Unclear at this point which of the S-100 Prod Specs apply |

- MSP 2 – Navigational Assistance Service (NAS)

| Service definition owner | Contributors (other stakeholders in service definition) | Users (Service provider and consumer) | Associated S-100 Product Specification(s) | Body responsible for defining technical specifications of service | GAP / overlap |
| --- | --- | --- | --- | --- | --- |
| IALA VTS Committee | Other IALA Committees | VTS, Navigator | S-125 (Navigational Services) |  | (Dependency on MSP 1) Unclear at this point which of the S-100 Prod Specs apply |

- MSP 3 – Traffic Organization Service (TOS)

| Service definition owner | Contributors (other stakeholders in service definition) | Users (Service provider and consumer) | Associated S-100 Product Specification(s) | Body responsible for defining technical specifications of service | GAP / overlap |
| --- | --- | --- | --- | --- | --- |
| IALA VTS Committee | Other IALA Committees | VTS, Navigator | S-127 (Traffic Management) |  | (Dependency on MSP 1) Unclear at this point which of the S-100 Prod Specs apply |

MSPs where IALA is not the service definition owner:

- MSP 8 – Vessel Shore Reporting

| Service definition owner | Contributors (other stakeholders in service definition) | Users (Service provider and consumer) | Associated S-100 Product Specification(s) | Body responsible for defining technical specifications of service | GAP / overlap |
| --- | --- | --- | --- | --- | --- |
| IMO | IALA, National authorities, Ports, WHO, ILO, Customs, WCO, etc. | National authorities, Master, Shipowner/operator agent | ??? |  | Unclear at this point which of the S-100 Prod Specs apply |