IALA Guideline

????

Maritime Service Portfolios

Edition 1.0

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

1.	INTRODUCTION	6

1.1.	General Description	6

1.2.	Background	6

1.3.	Scalability	6

1.4.	Scope	6

1.5.	Use	6

2.	SERVICE PROVIDERS & STAKEHOLDERS	6

2.1.	Definition	6

2.2.	Responsible service providers	6

3.	Defined sea areas for information services	7

4.	OVERVIEW (Example Heading level 1)	8

4.1.	MS 1 VTS Information Service (IS)	8

4.1.1.	Area of operation	8

4.1.2.	Definition	8

4.1.3.	Scope	8

4.1.4.	Objective	8

4.1.5.	User requirements	8

4.1.6.	Portrayal and SQA	8

4.1.7.	Examples	9

4.2.	MS 2 Navigational Assistance Service (NAS)	9

4.2.1.	Definition	9

4.2.2.	Scope	9

4.2.3.	Objective	9

4.2.4.	User requirements	9

4.3.	MS3 Traffic Organization Service (TOS)	9

4.3.1.	Definition	9

4.3.2.	Scope	10

4.3.3.	Objective	10

4.3.4.	User requirements	10

4.4.	MS4 Local Port Service (LPS)	10

4.4.1.	Definition	10

4.4.2.	Scope	10

4.4.3.	Objective	10

4.4.4.	User requirements	11

4.5.	MS 5 Maritime Safety Information service (MSI)	11

4.5.1.	Definition	11

4.5.2.	Scope	11

4.5.3.	Objective	11

4.5.4.	User requirements	11

4.6.	MS6 Pilotage service	11

4.6.1.	Definition	11

4.6.2.	Scope	11

4.6.3.	Objective	11

4.6.4.	User requirements	11

4.7.	MS7 Tugs service	11

4.7.1.	Definition	12

4.7.2.	Scope	12

4.7.3.	Objective	12

4.7.4.	User requirements	12

4.8.	MS8 Vessel shore reporting	12

4.8.1.	Definition	12

4.8.2.	Scope	13

4.8.3.	Objective	13

4.8.4.	User requirements	13

4.9.	MS9 Telemedical Assistance Service (TMAS)	13

4.9.1.	Definition	13

4.9.2.	Scope	14

4.9.3.	Objective	14

4.9.4.	User requirements	14

4.10.	MSP10 Maritime Assistance Service (MAS)	14

4.10.1.	Definition	14

4.10.2.	Scope	14

4.10.3.	Objective	14

4.10.4.	User requirements	14

4.11.	MS 11 Nautical Chart Service	14

4.11.1.	Definition	14

4.11.2.	Scope	15

4.11.3.	Objective	15

4.11.4.	User requirements	15

4.12.	MS 12 Nautical publications service	15

4.12.1.	Scope	15

4.12.2.	Objective	15

4.12.3.	User requirements	15

4.13.	MSP13 Ice navigation service	15

4.13.1.	Definition	15

4.13.2.	Scope	15

4.13.3.	Objective	16

4.13.4.	User requirements	16

4.14.	MSP14 Meteorological information service	16

4.14.1.	Definition	16

4.14.2.	Scope	16

4.14.3.	Objective	16

4.14.4.	User requirements	16

4.15.	MSP15 Real-time hydrographic and environmental information services	16

4.15.1.	Definition	16

4.15.2.	Scope	17

4.15.3.	Objective	17

4.15.4.	User requirement	17

4.16.	MSP16 Search and Rescue (SAR) Service	17

4.16.1.	Definition	17

4.16.2.	Scope	17

4.16.3.	Objective	17

4.16.4.	User requirements	17

4.17.	MS 17 Aids to Navigation services (AtoN)	17

4.18.	MS 18 Communication services	17

4.19.	MS 19 PNT and augmentation services	17

4.20.	[MS 20 Anti-piracy information]	17

5.	ASSESSMENT OF SUITABLE SERVICES	18

5.1.	Services	18

6.	RELEVANT ASSOCIATED IMO GUIDELINES	18

6.1.	Guidelines on SQA and HCD	18

6.2.	Guidelines on Display of nav info from comms	18

6.3.	Guidelines on test beds reporting	18

7.	LIST OF PUBLICATIONS THAT CAN BE DIGITAL	18

8.	ACRONYMS	18

ANNEX A	ANNEX	19

APPENDIX 1	APPENDIX TITLE	20

List of Tables

Table 1	Responsible Authorities	7

# INTRODUCTION

## General Description

MSPs have been identified in the e-navigation SIP (NCSR 1/28 annex 7) as the framework for the electronic provision of information related to maritime services in a harmonized way between shore and ships.  The list of maritime services available from a port, region or sea area is known as a Maritime Service Portfolio.  The information services in the portfolio will be different depending on the port or region facilities.  As an example, a small port might only offer a few services where a large port might contain many.

The purpose of the guideline is to give those offering such information services, guidance on their structure and how they fit into a Portfolio.

These guidelines also aim to harmonise globally the format and structure used to exchange that information, so that a ship can successfully use these services anywhere.

Note that the portrayal of MSP information is not covered by this guideline. Display on board ship is covered by Guidelines being developed by IMO on the display of navigational information received from communications equipment. IALA Guideline 1105 provides information for display for VTS .

## Background

A lack of coordination in the provision of information related to maritime services and among organizations responsible for the provision of services for use in MSPs may lead to the duplication of efforts, development of regional solutions, use of different communication systems, non-standardized displays on board, and the provision of superfluous or non-interoperable information.  IMO has already approved work on developing Guidelines for harmonized display of navigation information (MSC Circ.xxx) received by communications equipment.  Note that the  IHO S-100 framework has been chosen as a baseline for the Common Maritime Data Structure (CMDS) for e-Navigation.

However, the consistent provision of this information in the delivery phase preceding display still lacks harmonization.

## Scope

The purpose of this guideline is to bring the necessary coordination by defining and harmonizing the format, structure and provide guidance on communication channels for the electronic exchange of information between shore and ship. Examples of MSP are shown both for each service and overall which are not limiting.

## Concept of operation

“”How it works and an overview of a communication and information service (eg maritime cloud) in connection with MSPs. Also CMDS and S100 (T Christensen presentation)””

## Scalability

This guideline is prepared to ensure that MSPs are suitable for any type and size of vessel, not just IMO SOLAS vessels.  This is known as scalability.

## Use

This guideline focusses on the practical and operational use of MSPs. The Guideline aims to support the planning, implementation and support of those who are involved in providing services to the Mariner

# SERVICE PROVIDERS & STAKEHOLDERS

## Definition

Service providers are…Stakeholders are…users……….;

## Responsible service providers

In each country there will be service providers responsible for each service.  The table below identifies the authority responsible in each case, which can be different between countries.

Responsible Authorities

| Service No | Identified Services | Identified Responsible Service Provider |
| --- | --- | --- |
| 1 | VTS Information Service (IS) | VTS Authority |
| 2 | Navigational Assistance Service (NAS) | National Competent VTS Authority/Coastal or Port Authority |
| 3 | Traffic Organisation Service (TOS) | National Competent VTS Authority/Coastal or Port authority |
| 4 | Local port Service (LPS) | Local Port/Harbour operator |
| 5 | Maritime Safety Information Service (MSIS) | National Competent Authority |
| 6 | Pilotage service | Pilot Authority/Pilot Organization |
| 7 | Tug Service | Tug Authority |
| 8 | Vessel Shore Reporting | National Competent Authority/Shipowner/ Operator/Master/Agent |
| 9 | Telemedical Assistance Service (TMAS) | National health organization / dedicated health organization |
| 10 | Maritime Assistance Service (MAS) | Coastal/Port Authority / Organization |
| 11 | Nautical Chart Service | National Hydrographic Authority / Organization |
| 12 | Nautical Publications service | National Hydrographic Authority / Organization |
| 13 | Ice navigation Service | National Competent Authority Organization |
| 14 | Meteorological information service | National Meteorological Authority Public Institutions |
| 15 | Real time hydrographic and environmental information service | National Hydrographic and Meteorological Authorities |
| 16 | Search and Rescue Service | SAR Authorities |

# Defined sea areas for information services

The following six areas have been identified for the delivery of MSPs:

port areas and approaches;

coastal waters and confined or restricted areas;

open sea and open areas;

areas with offshore and/or infrastructure developments;

polar areas; and

other remote areas.

This list does not prevent relevant service providers from establishing a portfolio of services in a discreet area not defined above.

# MARITIME SERVICES

## MS 1 VTS Information Service (IS)

### Objectives

The 'VTS IS' is defined by IMO as “a service to ensure that essential information becomes available in time for on-board navigational decision-making” (Res. A857(20)).

### Description

IS is provided by broadcasting information at fixed times and intervals or when deemed necessary by the VTS or at the request of a vessel.

An Information Service involves maintaining a traffic image and allows interaction with traffic and response to developing traffic situations. An Information Service should provide essential and timely information to assist the onboard decision-making process.

The VTS IS is designed to improve the safety and efficiency of vessel traffic and to protect the environment.  Other such services include those such as: Routing, Channel information, Security level, Berthing, Anchorage, Time slot, Traffic monitoring and assessment, Waterway conditions, Weather, Navigational hazards, any other factors that may influence the vessel's transit, Reports on the position, Identity and intentions of other traffic.

### Operating Nodes

VTS centre or other shore based centres and vessels entering leaving and transiting the area of operation

### Users

VTS operator, OOW and Master

### Stakeholders

Port authorities, Ships agent, shipowner,

### Area of operation

VTS areas

### Operational elements

Information may/should/shall include but is not limited to local information relating to:

the position, identity, intention and destination of vessels;

amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points;

the mandatory reporting of vessel traffic movements;

meteorological and hydrological conditions, notices to mariners, status of aids to navigation;

manoeuvrability limitations of vessels in the VTS area that may impose restrictions on the navigation of other vessels, or any other potential hindrances; or

any information concerning the safe navigation of the vessel.

### Relationship with other MSPs

There may be overlapping information with the following MSPs: MS 2, MS 3, MS 4, MS 5 MS 14 MS 15 and possibly others

### Examples

## MS 2 Navigational Assistance Service (NAS)

### Definition

The NAS is defined by IMO as “a service to assist on-board navigational decision-making and to monitor its effects” (IMO Res.A857(20)).

NAS may be provided on request by a vessel in circumstances such as equipment failure or navigational unfamiliarity. Specific examples of developing situations where NAS may be provided by the VTS include:

Risk of grounding;

Vessel deviating from the recommended track or sailing plan;

Vessel unsure of its position or unable to determine its position;

Vessel unsure of the route to its destination;

Assistance to a vessel to an anchoring position;

Vessel navigational or manoeuvring equipment casualty;

Inclement conditions (e.g. low visibility, high winds);

Potential collision between vessels;

Potential collision with a fixed object or hazard;

Assistance to a vessel to support the unexpected incapacity of a key member of the bridge team,

At the request of the Master.

### Scope

### Objective

### User requirements

All information related to this service should be displayed in real time.

Only information about dangerous ships nominated by the mariner should be displayed to prevent screen clutter.

When dangerous ships nominated by mariner do not keep their route in the case of route plan exchange, an alert should be emitted.

## MS3 Traffic Organization Service (TOS)

### Definition

The TOS is defined by IMO as “a service to prevent the development of dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the VTS area” (IMO Res.A857(20)).

The purpose of the TOS is to prevent hazardous situations from developing and to ensure safe and efficient navigation through the VTS area.

TOS should be provided when the VTS is authorized to provide services, such as when:

vessel movements need to be planned or prioritized to prevent congestion or dangerous situations;

special transports or vessels with hazardous or polluting cargo may affect the flow of other traffic and need to be organized;

an operating system of traffic clearances or sailing plans, or both, has been established;

the allocation of space needs to be organized;

mandatory reporting of movements in the VTS area has been established;

special routes should be followed;

speed limits should be observed;

the VTS observes a developing situation and deems it necessary to interact and coordinate vessel traffic;

nautical activities (e.g. sailing regattas) or marine works in-progress (such as dredging or submarine cable-laying) may interfere with the flow of vessel movement.

### Scope

### Objective

### User requirements

## MS4 Local Port Service (LPS)

### Definition

LPS is applicable to those ports where it has been assessed that a VTS, as described above, is excessive or inappropriate.

The main difference arising from the provision of LPS is that it does not interact with traffic, nor is it required to have the ability and/or the resources to respond to developing traffic situations and there is no requirement for a vessel traffic image to be maintained.

Provision of LPS is designed to improve port safety and co-ordination of port services within the port community by dissemination of port information to vessels and berth or terminal operators.  It is mainly concerned with the management of the port, by the supply of information on berth and port conditions.  Provision of LPS can also act as a medium for liaison between vessels and allied services, as well as providing a basis for implementing port emergency plans.  Examples of LPS may include:

berthing information;

availability of port services;

shipping schedules;

meteorological and hydrological data.

### Scope

### Objective

### User requirements

## MS 5 Maritime Safety Information service (MSI)

### Definition

The Global Maritime Distress and Safety System (GMDSS) as described in SOLAS Chapter IV defines the seventh functional requirement as: 'Every ship, while at sea, shall be capable of transmitting and receiving maritime safety information'.

The MSI service is an internationally co-ordinated network of broadcasts of Maritime Safety Information from official information providers, such as:

National Hydrographic Offices, for navigational warnings and chart correction data;

National Meteorological Offices, for weather warnings and forecasts;

Rescue Co-ordination Centres (RCCs), for shore-to-ship distress alerts;

The International Ice Patrol, for Oceanic ice hazards.

### Scope

### Objective

### User requirements

## MS6 Pilotage service

### Definition

The aim of the pilotage service is to safeguard traffic at sea and protect the environment by ensuring that vessels operating in pilotage area have navigators with adequate qualifications for safe navigation.  Each pilotage area needs highly specialized experience and local knowledge on the part of the pilot.

Efficient pilotage depends, among other things, upon the effectiveness of the communications and information exchanges between the pilot, the master and the bridge personnel and upon the mutual understanding, each has for the functions and duties of the other.

Establishment of effective co-ordination between the pilot, the master and the bridge personnel, taking due account of the ship's systems and equipment available to the pilot, will aid a safe and expeditious passage.

### Scope

Contact info?

### Objective

### User requirements

## MS7 Tugs service

### Definition

Efficient tug operations depend on, among other things, the effectiveness of the communications and information exchanges between relevant stakeholders.  The aim of the tugs services is to safeguard traffic at sea and protect the environment by conducting operations such as:

transportation (personnel and staff from port to anchorage) operations;

ship assistance (ex: mooring) operations;

salvage (grounded ships or structures) operations;

shore operations;

towage (harbour/ocean) operations;

escort operations;

oil spill response operations.

### Scope

### Objective

### User requirements

## MS8 Vessel shore reporting

### Definition

The aim of vessel shore reporting is to safeguard traffic at sea, ensure personnel safety and security, ensure environmental protection and increase the efficiency of maritime operations.

The aim of vessel shore reporting is to safeguard traffic at sea, ensure personnel safety and security, ensure environmental protection and increase the efficiency of maritime operations.

Automated ship reporting is one of the most important solutions to reduce the Mariners workload (amount of time spent on preparing and submitting reports to shore-based authorities).  To achieve this, reports should be automatically generated as much as possible from on-board systems. Some of the ways the administrative burden of vessel shore reporting can be reduced are:

single-entry of reporting information into ICT collection tools that store it in a repository and ICT tools that assists with the generation all required reports from this repository;

automated collection of information from ship-board systems that is required for reporting (for example Ballast Management System, Emissions Control System, Waste Management System, Navigation System, etc., etc.);

ICT tools that allow mariners to delegate to shore-based personnel (at the discretion of the ship’s owner/operator) the tasks of information collection, generation and submittal of required reports;

reduce the administrative burden by encouraging all national reporting requirements to use standardized digital reporting formats based on the S-200 Product Specification of the Common Maritime Data Structure;

automated or semi-automated digital distribution/communication of required reports via available networks.

### Scope

Submission and distribution of all reports required by all shore-based authorities in the required format and in the required timeframe.

### Objective

Reduce the burden of submittal and distribution of required reports

### User requirements

Provide ICT tools for shipboard and shore-based personnel to streamline the processes and procedures associated with submittal, generation and distribution of required reports, including retrieval of information from other ship systems (Ballast Management, Waste Management System, Emission Control System, Navigation System, etc., etc.) and from shore-based sources (cargo and passenger booking offices, crewing agents, stevedores, etc., etc.).

Such tools should alert the user what information is missing in the repository that prevents generation of the required reports for an upcoming port call, which reports will need to be submitted when, to whom in what format and via which communications network.

The repository structure shall comply with the latest version of the S-200 Product Specification for the Common Maritime Data Structure.

The reports shall fulfil the exact requirements of each and every shore-based authority.  This means adhering to the requirements for report format (hard copy, fax, MS Word, PDF, RTF, XML, Excel, CSV, etc.), its graphical layout, it’s language(s), the specification of its fields, its units of measure, allowed abbreviations, its deadline (relative to the arrival at the next port), how it is authenticated, how it is to be submitted, who it should be addressed to, etc., etc.

The reports should be created in the proper time and time period to report before her arrival at ports or sea area automatically and authorised by master before submission.

The information related to ship operation should not be revised intentionally by mariner and should be collected directly from ship’s automatic monitoring system.

To fulfil the above user requirements an eco-system shall be established in which developers of such ICT Tools can thrive and provide shipping lines with a number of alternative solutions.

This, in turn, requires building and maintaining a library of required reports that are uniquely identified and characterized by their requirements for format, deadline, content, etc., etc. (FONASBA, which is an association of shipping agents that has 'Observer' status at IMO may be enticed to build and maintain the report library).  The eco-system also requires developing and maintaining an S-200 Product Specification for CMDS that can be used to generate all required reports in the library.  Lastly it requires that ships’ systems that generate reporting information be certified to be compliant with an international machine-to-machine interface standard or ship network standards such as IEC 61162 series.  A prime candidate for such standards are those developed by the Open Connectivity Foundation for the Internet of Things (IoT).

## MS9 Telemedical Assistance Service (TMAS)

### Definition

According to the IMO/ILO resolution 164 the TMAS centre should provide medical advice for seafarers 24 h/day, 365 days/year. TMAS should be permanently staffed by physicians qualified in conducting remote consultations and who are well versed in the particular nature of treatment onboard ship.

Within the maritime medicine the prevailing view has for a long time been that a standardization of the TMAS services is both necessary and wanted. This would firstly enhance the quality of the medical practice, and secondly, a standardization of reporting and registering of medical events will make a much better basis for advancement.

### Scope

### Objective

### User requirements

## MSP10 Maritime Assistance Service (MAS)

### Definition

The primary mission of MAS is to handle communication between the coastal State, ship's officers requiring assistance, and other players in maritime community.  These can be fleet owners, salvage companies, port authorities, brokers, etc.

The MAS is on 24-hour alert to deploy rapid assistance and professional support for ships in connection with:

Combating pollution, fire and explosions on board, collision, grounding, maritime security, terror mitigation, etc.

The Ship Security Alert System enables a vessel to send a distress call if it is attacked by pirates, etc.  On receiving such a call, the MAS is responsible for alerting the relevant authorities responsible for a response.

The MAS is responsible only for receiving and transmitting communications and monitoring the situation.  It serves as a point of contact between the master and the coastal State concerned if the ship's situation requires exchanges of information between the ship and the coastal State.

Situations where the MAS apply are as follow:

A ship involve in an incident (loss of cargo, accidental discharge of oil, etc.) that does impair its seakeeping ability but nevertheless has to be reported;

a ship in need of assistance according to the master's assessment, but not in distress situation that requires the rescue of personnel on board;

a ship in distress when those on board have already been rescued, with the possible exception of those who have remained aboard or have been placed on board to attempt to deal with the ship's situation.

The MAS entails the implementation of procedures and instructions enabling the forwarding of any given information to the competent organization and requiring the organizations concerned to go through the MAS in order to make contact with the ship.

### Scope

### Objective

### User requirements

## MS 11 Nautical Chart Service

### Definition

The aim of the nautical chart service is to safeguard navigation at sea by providing information such as nature and form of the coast, water depth, tides table, obstructions and other dangers to navigation, location and type of aids to navigation.

The Nautical Chart service also ensure the distribution, update and licensing of electronic chart to vessels and other maritime parties.

### Scope

### Objective

### User requirements

## MS 12 Nautical publications service

4.12.1	Definition

The aim of the nautical publication service is to promote navigation awareness and safe navigation of ships.  The nature of waterways described by any given nautical publication changes regularly, and a mariner navigating by use of an old or uncorrected publication is courting disaster.  Nautical publications include:

Tidal currents, aids to navigation system, buoys and fog signals, radio aids to marine navigation, chart symbols, terms and abbreviations, sailing directions.

A Chart and Publication Correction Record Card system can be used to ensure that every publication is properly corrected prior use by mariners.

### Scope

### Objective

### User requirements

## MSP13 Ice navigation service

### Definition

The ice navigation service is critical to safeguard the ship navigation in ice-conditions, given how quickly the ice maps become outdated in the rapid changing conditions of the ice-covered navigational regions.  Such services include:

ice condition information and operational recommendations/advice;

ice condition around a vessel;

vessel routing;

vessel escort and ice breaking;

ice drift load and momentum;

ice patrol.

### Scope

### Objective

### User requirements

## MSP14 Meteorological information service

### Definition

The meteorological service is essential to safeguard the traffic at sea by providing real-time weather conditions, forecasts, warnings, and weather routeing to mariners who will use these types of information to support their decision-making.

The meteorological service is essential to safeguard the traffic at sea by providing weather, climate digital forecasts and related information to mariners who will use these types of information to support their decision making.  Such information includes:

weather routing, solar radiation, precipitation;

cold/hot periods, warnings;

air temperature, wind speed &direction;

cloud cover, barometric pressure.

### Scope

### Objective

### User requirements

## MSP15 Real-time hydrographic and environmental information services

### Definition

The real time and forecast hydrographic and environmental information services are essential to safeguard navigation at sea and protect the environment.

The real time hydrographic and environmental information service is essential to safeguard navigation at sea and protect the environment.  The service provided include:

current speed and direction;

wave height;

marine habitat and bathymetry;

Sailing Directions (or pilots): detailed descriptions of areas of the sea, shipping routes, harbours, aids to navigation, regulations, etc.;

Lists of lights: descriptions of lighthouses and lightbouys;

tide surge prediction tables and tidal stream atlases;

ephemerides and nautical almanacs for celestial navigation;

Notice to Mariners: periodical (often weekly) updates and corrections for nautical charts and publications.

### Scope

### Objective

### User requirement

## MSP16 Search and Rescue (SAR) Service

### Definition

The SAR service is responsible for assisting, coordinating search and rescue operations at sea. In maintaining a state of full readiness, the Services may assist the following search and rescue functions:

The crew and passengers of vessels in distress;

Victims of maritime and aircraft accidents or incidents.

The SAR services must also coordinate the evacuation of seriously injured or ill person from a vessel at sea when the person requires medical treatment sooner than the vessel would be able to get him or her to a suitable medical facility.

The Services may also be pro-actively involved in activities such as:

Information collection, distribution, and coordination;

Monitoring towing operations;

Monitors and evaluates levels of risk from Maritime Safety Information (MSI) broadcasts to ensure an immediate response in case of life threatening situations developing;

Monitoring vessels not under command;

Pollution reports and vessels aground.

### Scope

### Objective

### User requirements

## MS 17 Aids to Navigation services (AtoN)

## MS 18 Communication services

## MS 19 PNT and augmentation services

## [MS 20 Anti-piracy information]

# ASSESSMENT OF SUITABLE SERVICES

## Services

# RELEVANT ASSOCIATED IMO and other GUIDELINES/PerformancE STANDARDS

## Guidelines on SQA and HCD

## Guidelines on Display of navigation information from communications

## Guidelines on test beds reporting

# LIST OF PUBLICATIONS THAT CAN BE DIGITAL

# ACRONYMS

AtoN	Aid(s) to Navigation

Circ.	Circular (IMO)

CMDS	Common Maritime Data Structure

COMSAR	Former Sub Committee on Communications and Search and Rescue (IMO)

CSV	Comma Separated Variable(s)

fax	Facsimile

FONSABA	Federation of National Associations of Ship Brokers and Agents

GMDSS	Global Maritime Distress and Safety System

HCD	Human Centred Design

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities

ICT	Information and Communications Technology

IEC	International Electrotechnical Commission

ILO	International Labour Organization (UN)

IoT	Internet of Things

IS	Information Service, as part of Vessel Traffic Services

IMO	International Maritime Organization (UN)

LPS	Local Port Service(s)

MAS	Maritime Assistance Service

MSC	Maritime Safety Committee (IMO)

MSIS	Maritime Safety Information Service

MSP	Maritime Service Portfolio(s)

NAS	Navigational Assistance Service, as part of Vessel Traffic Services

NAV	Former Sub Committee on Navigation(IMO)

NCSR	Sub Committee on Navigation, Communications and Search and Rescue (formerly COMSAR and NAV) (IMO)

PDF	Portable Document Format

PNT	Position, Navigation and Timing

RCC	Rescue Co-ordination Centre(s)

Res.	Resolution

RTF	Rich Text Format

SAR	Search and Rescue

SIP	IMO e-Navigation Strategy Implementation Plan (NCSR1/28, Annex 7; as adopted by MSC94, Nov. 2014)

SOLAS	International Convention for the Safety of Life at Sea, 1974 (as amended)

SQA	Software Quality Assurance

S-100	Geospatial Information Registry (IHO)

S-200	IALA domain for S-100 Product Specifications

TMAS	Telemedical Assistance Service

TOS	Traffic Organisation Service, as part of Vessel Traffic Services

VTS	Vessel Traffic Service(s)

XML	eXtensible Markup Language

ANNEX

Guidelines should have lettered Annexes.  Numbered Appendices are attached to Annexes.

ANNEXES B onward 'number' automatically.  Each has its own heading styles in the style pane.

ANNEX A HEAD1

Body Text

Annex A Heading 2

Body text

Annex A Heading 3

Body Text

Annex A Heading 4

Body Text

APPENDIX TITLE

APPENDIX 1 HEADING 1

Body Text

Appendix Heading 2

Body Text

Appendix Heading 3

Body Text

Appendix heading 4

Body text