Input paper:   ENAV19-13.13

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		□  ENG		□  PAP					  Input

ENAV	□  VTS							□  Information

Agenda item 			13

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)		Manuel López (GSA), Jorge Morán (ESSP), Juan Vázquez (ESSP),

Pedro Pintor (ESSP), María Mota (ESSP)

# Relevant architectures for the transmission of SBAS corrections over existing maritime AtoN - two cases studies: IALA beacon and AIS

# Summary

The present document provides a description of the most relevant potential architectures that could be used to broadcast DGNSS corrections generated from the EGNOS (SBAS) message (obtained from SIS and/or EDAS) over IALA beacons or AIS. The document includes also a description on how SBAS messages in RTCA format could be mapped into RTCM 2.x format.

## Purpose of the document

The purpose of the document is to provide this technical description for the Committee review in order to obtain comments and to be considered in the development of the associated IALA Guidance material.

# References

MOPS DO-229D Minimum Operational Performance Standards for Global Positioning System/Wide Area Augmentation System Airborne Equipment, 12/13/2006

ENAV18-13.16 Transmission of SBAS corrections over AIS

ENAV18-13.20 Transmission of SBAS corrections over IALA beacons

IMO Resolution A.1046 (27), November 2011.

RTCM 10402.3 Recommended Standards For Differential GNSS services, August 20, 2001

IALA Guideline 1112, Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz, Edition 1, May 2015

IALA Recommendation A-124 Appendix 16 – DGNSS Broadcasts from an AIS Service

IALA Guidelines On Ship-Borne Automatic Identification System (AIS) Volume I Part Ii: Technical Aspects Of AIS, Edition 1.1, December 2002

# Action requested of the Committee

The Committee is invited to consider the information provided in the Annex.

- ANNEX 1: Relevant architectures for the transmission of SBAS corrections over existing maritime AtoN: IALA beacon or AIS

The present document provides a description of the different architectures which have been identified as the most suitable for the transmission of SBAS messages over existing AtoN: IALA beacons or AIS.

This information is structured as follows:

- Section 1: Description of the most relevant architectures for the transmission of SBAS corrections over IALA beacons.

- Section 2: Description of the most relevant architectures for the transmission of SBAS corrections over AIS.

- Section 3: this section provides a description on how SBAS messages in RTCA format could be mapped into to RTCM 2.x format.

EGNOS OVER IALA BEACONS: DESCRIPTION OF THE MOST RELEVANT ARCHITECTURES

A detailed assessment of several EGNOS based architectures has been performed. The objective of this analysis was to identify the most promising EGNOS based architecture(s) to be considered for deployment in an operational environment.

The starting point for this analysis has been the definition of two high-level architectures for the set-up of an EGNOS based DGNSS service, with either equipment all sited at the broadcast locations (classic approach), or some of the infrastructure is centralized with only the transmitting equipment at the broadcast site (network approach).

Then, a lower level detail analysis of several EGNOS based architectures has been performed, comparing each architecture to the traditional DGNSS architecture (see section 1.1). In order to identify the most promising EGNOS based architectures, a trade off analysis has been done, considering the following key features (see section 1.5):

- Infrastructure at site.

- Legacy receivers compatibility.

- Integrity monitoring.

- Local effects impact corrections.

- Independence of corrections generation vs integrity check.

- Communication lines.

- MF link monitoring.

- Redundancy.

- Jamming and Spoofing Resiliency.

- Corrections generation separated from the transmission technology.

Among the criteria listed above, the following are considered to be mandatory or highly recommended for an EGNOS based architecture to be implemented as an alternative to the current DGNSS solutions:

- The “Legacy receivers compatibility” (LEG) and “Integrity Monitoring” (IM) functionalities.

- IALA/IMO recommendations:

  - Independence of corrections generation  vs integrity check

  - MF link monitoring

  - Corrections generation separated from the transmission technology

- Redundant solutions to meet 99.8% availability requirement.

Taking this into account, among the solutions analysed, the ones meeting the mandatory features and recommendations mentioned above are:

- Hybrid Decentralised Architecture: Traditional DGNSS + SISNeT Based (see section 1.2)

- Hybrid Centralised Architecture: Traditional DGNSS + EGNOS Based VRS (see section 1.3)

- Redundant Fully EGNOS Based Solution (see section 1.4)

The particular details of these three architectures are described in the sections below.

In any case, it should be noted that the assessment of the most convenient architecture for each particular scenario shall be based on a case by case analysis. This analysis shall take into account the topology of the existing IALA DGNSS infrastructure (if any) - network or classic approach, the availability of communication lines connecting the different elements comprising the current architecture or the type of GNSS receivers available (EGNOS enabled or not), among other technical aspects and also any relevant operational/service requirement provided by the target maritime authority.

  - Baseline IALA DGNSS service architecture

For a better understanding of the added value of the solutions analysed and its potential drawbacks/limitations, a comparison with respect to a traditional baseline architecture has been performed. This baseline architecture has been defined as a DGNSS service based on the classic approach [6], the IALA recommended architecture of an IALA DGNSS station, including the Reference Station (RS) and the Integrity Monitoring (IM) modules. This baseline has been selected under the understanding that it is the typical (or one of the most common) set-up used today to provide an IALA DGNSS service.

As depicted in Figure 1, the Reference Station component computes the psedurorange corrections based on the GNSS raw data (observations and ephemeris) collected by a GNSS receiver (including the GNSS antenna) and the surveyed position of the station. These corrections are then encoded in RTCM format and MSK modulated, to be transmitted to the users in the LF/MF band (285 - 325 kHz).

The integrity of the corrections transmitted to the users, is checked by the Integrity Monitor module, which processes the GNSS raw data and the DGNSS corrections collected by a DGNSS receiver. It is to be noted that the “DGNSS receiver” box depicted in the figure includes: GNSS antenna, MF antenna and MSK demodulator (MF receiver).

Therefore, the integrity approach of this architecture corresponds to the Post-Broadcast Monitoring concept. This monitoring concept, not only monitors the integrity of the data but also the availability of the radio link and the quality of the signal transmitted. This is one of the major differences with respect to the Pre-Broadcast concept which includes the integrity monitoring functionality, but does not check the availability of the MF radio link.

Baseline/current IALA Architecture

In terms of equipment, and excluding the transmission part which is the same for all the solutions presented in this document, this architecture consists of:

- Two GNSS Antennas.

- One MF Receiver (MF Antenna + MSK Demodulator).

- One DGNSS Receiver (without including the MF receiver)

- One GNSS Receiver

- One Reference Station

- One Integrity Monitoring

- The RTCM encoder.

It is also noted that this architecture is commonly doubled in order to meet the availability requirements defined by IMO [4].

  - Hybrid Decentralised Architecture: Traditional DGNSS + SISNeT Based

This is a redundant decentralised solution, combining the traditional DGNSS configuration and the SISNeT based architecture which counts on two RS and two IM locally onsite.

The two IM are identical and in line with the current architecture while for the second RS, as depicted in the following figure, the GNSS receiver and the standard reference station are replaced by a SISNeT Client and a RTCA to RTCM converter. Please note also that all the components highlighted in the following figure (SISNeT Client, RTCA to RTCM converter, Integrity Monitoring and RTCM encoder) could be integrated in a single SW module. This module will be responsible for:

- Getting the SBAS corrections and GPS ephemeris from the EDAS SISNeT server.

- Converting the SBAS messages into DGPS corrections in RTCM format.

- Checking the integrity of these corrections (pre and/or post broadcast) based on the information provided by the DGNSS receiver

- Encoding the differential corrections together with the integrity data in RTCM format.

Hybrid Decentralised Architecture: Traditional DGNSS + SISNeT Based

Please note also that in order to reduce the infrastructure needed at each beacon site (maintaining redundant Integrity Monitoring architecture), the raw data collected by the GNSS receiver and/or the DGNSS receiver of the traditional architecture (left chain in the figure) could be used by the integrity monitoring module of the EGNOS based solution (right chain).

Another option for this architecture is to replace the SISNeT client by a GNSS antenna and an SBAS enabled receiver. In that case, internet connectivity will not be needed in the reference station, but more equipment will be required (GNSS antenna and SBAS receiver) at each beacon site.

  - Hybrid Centralised Architecture: Traditional DGNSS + EGNOS Based VRS

This solution combines a traditional DGNSS station deployed at each beacon site with a centralised EGNOS based VRS solution.

Both the RS and the IM stations are centralised in the “Central Facility”, and therefore, the only infrastructure needed at each beacon site is the communication lines and the transmission equipment (apart from the Ntrip client to retrieve the differential corrections).

The EGNOS based centralised solution is based on the pre-broadcast monitoring approach, using the raw data collected by a network of GNSS receivers to check the integrity of the corrections generated and a network of MF receivers, to monitor the radio link availability and the quality of the signal transmitted.

It is to be noted that the DGNSS and GNSS receivers in the traditional DGNSS architecture (left chain in figure below) could be also used to monitor the signal and corrections transmitted by the EGNOS based solution (right chain in the figure below).

Finally, it is important to remark that apart from the reduction of HW/SW components of the EGNOS based centralised solution at each beacon site, another advantage of this solution is that it is resilient in front of jamming attacks in the vicinity of the reference station. In that situation, the traditional reference station will not be able to collect GNSS raw data and therefore not able to generate differential corrections. However, the EGNOS based VRS solution will not be affected by this attack.

Hybrid Centralised Architecture: Traditional DGNSS + EGNOS Based VRS

  - Redundant Fully EGNOS Based Solution

This is a fully EGNOS based solution, combining the decentralised (left chain) and centralised (right chain) approaches.

The centralised solution corresponds with the architecture presented in the previous section (right chain), where the RS and the IM stations are centralised in the “Central Facility”. For the decentralised solution (left chain), it is noted that the SBAS corrections can be obtained from a SISNeT client or from an SBAS enabled receiver. The raw data collected by the GNSS receiver and/or the DGNSS receiver of the decentralised architecture (left chain in the figure) could be used by the integrity monitoring module of the EGNOS based centralised solution (right chain).

In order to increase the system redundancy, it is recommended to use two independent RTCA to RTCM SW solutions in the decentralised architecture (left chain) and the centralised architecture (right chain).

Fully EGNOS Based Solution

  - Trade-Off Analysis

As mentioned before, a trade-off analysis of potential EGNOS-based architectures has been done, taking into account the key features described in the table below, derived from the baseline architecture and considering the following colour code:

- Green colour: Feature improved w.r.t. the baseline.

- White colour: Same as the baseline (no improvement/degradation).

- Red colour: Feature degraded w.r.t. the baseline.

It should be noted that, for some features, only two of the above assessments will be possible. As an example, for the MF Link Monitoring capability, a ‘green’ assessment is not possible (no improvement) since:

- Architecture provides MF link monitoring: same situation as baseline architecture (white colour).

- Architecture does not provide MF link monitoring: degraded situation with respect to baseline architecture (red colour).

- Key features (legend) used for the assessment

| Key feature ID | Key feature | Baseline (Current) Architecture | Assessment |  |  |
| --- | --- | --- | --- | --- | --- |
| INF | Infrastructure at site | 2 RS + 2 IM (with MF Rx) | Reduced infrastructure | Similar infrastructure | Additional infrastructure |
| LEG | Legacy receivers compatibility | Yes (corrections provided in RTCM format) | N/A | Yes | No |
| IM | Integrity monitoring | Yes | Additional Integrity check | Yes | No |
| LOC | Local effects impact corrections | Yes | No | Yes | N/A |
| IND (*) | Independence of corrections generation vs integrity check | Yes | N/A | Yes | No |
| COM | Communication lines | Standard communication lines (to connect the RS & IM to the remote CS) | No communication lines | Standard communication lines | High availability communication lines |
| MF (*) | MF link monitoring | Yes | N/A | Yes | No |
| RED | Redundancy | Yes (RS only) | Increased redundancy | Same (2 RS & 2IM) | Decreased redundancy |
| JSR | Jamming and Spoofing Resiliency | No | Yes | No | N/A |
| SEP (*) | Corrections generation separated from the transmission technology | No | Yes | No | N/A |

Among the identified criteria, the following are considered to be mandatory or highly recommended for an EGNOS based architecture to be presented to the maritime authorities as an alternative to their current solutions with a reasonable probability of success:

- The “Legacy receivers compatibility” (LEG) and “Integrity Monitoring” (IM) functionalities are considered mandatory (highlighted in red in the table). Therefore, EGNOS based DGNSS service architectures requiring new user equipment are proposed to be discarded.

- The following IALA/IMO recommendations that should be considered when designing an EGNOS based DGNSS architecture (highlighted with an asterisk (*) in the tables):

  - Independence of corrections generation  vs integrity check

  - MF link monitoring

  - Corrections generation separated from the transmission technology

Moreover, in order to meet the availability requirement (99.8%) for general maritime navigation, redundant solutions should be considered.

The table below presents the trade-off analysis performed for the three architectures identified as the most promising ones:

- Trade-Off Analysis - EGNOS over IALA beacons

| Title | INF | LEG | IM | LOC |  | IND(*) | COM | MF(*) | RED | JSR |  | SEP(*) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Current Architecture (assuming redundancy) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hybrid Decentralised |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hybrid Centralised |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Redundant fully EGNOS |  |  |  |  |  |  |  |  |  |  |  |  |  |

Note that there are several architectures for which the assessment of the key features depends on the data used for the reference station or the integrity monitoring. For instance, for the “Hybrid Centralised” solution, the assessment of the “Local effects impact corrections” depends on which corrections are finally transmitted to the users: the corrections generated by the decentralised DGNSS solution or by the centralised EGNOS based architecture. In that case, the corresponding cell has been split to account for the two possible options.

EGNOS OVER AIS: DESCRIPTION OF THE MOST RELEVANT ARCHITECTURES

A detailed assessment of several architectures for the transmission of EGNOS correction over AIS has been performed. The objective of this analysis was to identify those EGNOS based architectures with the largest potential to be introduced in an operational environment.

Two general architectures for the provision of EGNOS corrections via AIS have been analysed:

- EGNOS based DGNSS solution over decentralised AIS service, where the corrections are generated by an external component to each base station, that receives the EGNOS messages and GPS ephemeris from EDAS or the EGNOS SIS, and converts these corrections into RTCM format.

- EGNOS based DGNSS solution over centralised AIS service, where the corrections are generated in a central facility (ASM) and distribute it to the different AIS base stations.

Then, a lower level detail analysis of several EGNOS based architectures has been done, comparing each architecture with respect to the baseline one (see section 2.1). In this trade-off analysis, the following key features have been considered:

- Infrastructure at site.

- Local effects impact corrections.

- Independence of corrections generation vs integrity check.

- Communication lines.

- VDL Monitoring.

- Redundancy.

- Jamming and Spoofing Resiliency.

- Corrections generation separated from the transmission means.

- Customized corrections for each AIS Base Station.

- Access to internal data (GNSS raw data or signal parameters) from the base or mobile station needed.

Among the key features used for the analysis, two of them are considered to be mandatory or highly recommended to be met by any EGNOS based architecture to be proposed for deployment in an operational environment:

- The “independence of corrections generation vs integrity check” feature comes from an IALA recommendation[6] stating that the data used to check the data integrity shall be independent from the data used to generate the pseudorange corrections.

- The access to the data collected by the internal GNSS receiver. According to the consulted information for already existing AIS infrastructures (commercial AIS base stations), access to the data collected by the internal receiver (GNSS raw data or signal parameters) is not possible without changing the interfaces of the AIS Base or Mobile Stations. On those scenarios, where the AIS infrastructure is already deployed, it is recommended to use an external receiver to collect the data needed to compute the pseudorange corrections and check the integrity. Given this limitation, the architectures for those cases where already existing infrastructures cannot be (partially) reused are to be discarded.

Taking this into account, among the solutions analysed, the ones meeting the mandatory features and recommendations mentioned above are:

- EGNOS based DGNSS solution over decentralised AIS service.

- EGNOS based DGNSS solution over centralised AIS service.

The best alternative to be selected for each case will depend on the type of service provided and on the operational scenario under analysis. Hence, specific analysis would be required to conclude on the best EGNOS based AIS service architecture for each specific scenario (infrastructure deployed, operational requirements, service performance requirements, topology of the deployed AIS service, etc)-

In case the AIS shore infrastructure is developed from scratch, the base stations can be customised and equipped with additional functionalities (technically, cost impact unknown at this stage).

  - Baseline IALA AIS architecture

For a better understanding of the added value of the SBAS based solutions considered and its potential drawbacks/limitations, a comparison with respect to an arbitrary baseline architecture, where SBAS is not used, is provided. This choice (discarding non SBAS based architectures for the trade-off) has been made under the assumption that the ultimate goal of the study was to analyse the added value that SBAS could provide for the provision of DGNSS corrections over AIS (not to identify improvements to the current baseline solutions not including the use of SBAS).

The arbitrary baseline architecture for the provision of DGNSS corrections over AIS has been defined as an AIS service, getting the pseudorange corrections in RTCM format (including integrity) from a traditional DGNSS reference station. It is also assumed that standard communication lines are available at the base station. These baseline has been selected under the understanding that it is the typical (or one of the most common) set-up used today to provide DGNSS corrections over AIS.

Functional block diagram of an AIS base station [8]

  - EGNOS based DGNSS solution over decentralised AIS service

In this case, the source for the generation of the DGPS corrections to be broadcast by the AIS station is the EGNOS message (either obtained from the EGNOS SIS or from the EDAS SISNeT service).

As depicted in the figure below, the DGNSS corrections are provided as input (via a dedicated port) to the AIS Base Station, therefore, whether these corrections are received from a traditional DGNSS stations or generated based on EGNOS, is completely transparent for the AIS Base Station. Taking this into account, it is not necessary to do any change on the AIS Base Station but only on the external Reference Station and Integrity Monitoring (RS & IM). The external RS shall be replaced by a RS software to produce the differential GPS correction taking the SBAS messages as input. This component would basically consist of an RTCA to RTCM converter (see section 3 for details on this conversion). A pre-broadcast integrity monitoring concept could be implemented to check the integrity of the differential corrections generated by the EGNOS based RS.

A block diagram of the resulting RS & IM, including both HW and SW components is included hereafter.  It is to be noted that the SBAS message and the GPS ephemeris can be obtained from an SBAS enabled receiver or from the EDAS SISNeT service over the internet. Regarding the GNSS raw data needed to check integrity of the corrections, this data could be obtained from a dedicated GNSS receiver or from one of the different networks of receivers available (via internet/Ntrip).

Also, in case of using an SBAS enabled receiver to obtain the EGNOS message (instead of the EDAS SISNeT service), it could be considered to use the GNSS observations collected by this receiver to check the integrity of the data (note that the observations are not used to generate the differential corrections).

EGNOS based AIS station: RS & IM block diagram

The corrections generated by the RTCA to RTCM converter will be provided to the AIS Controller Unit in RTCM format (via a dedicated input port). Therefore, there will be no change with respect to the current interface, being the Controller Unit in charge of converting the RTCM message into VDL Message Type 17 format. As detailed above, it is important to remark that this solution is completely transparent for the base station itself, since it receives the corrections in RTCM format regardless they are generated by a traditional reference station or converted from the EGNOS message to RTCM format.

Finally, it is important to remark that the provision of DGNSS corrections is not a core functionality of the AIS system, just an optional message that could be transmitted through the MT17. Therefore, the VDL channel monitoring does not depend on the transmission or not of this message. For that reason, for the design of the high level architectures presented in this paper (decentralised and centralised), the monitoring of the VDL channel has not been taken into account.

  - EGNOS based DGNSS solution over Centralised AIS service

Although it is possible to generate the Message Type 17 in an “isolated” base station, the most common solution for the provision of DGNSS corrections via AIS is to generate the Message Type 17 in a central facility (ASM) and distribute it to the different AIS base stations.

As depicted in the following figure, the DGNSS correction data from the reference station(s) is encapsulated in an IEC 61162 VDM sentence (discarding the preamble and parity fields) by the AIS Logical Shore Station (AIS-LSS) for processing by the AIS PSS Controlling Unit (AIS-PCU). The Message Type 17 generated by the Logical Shore Station is then provided to each base station and finally transmitted to the users through the VDL channel.

Traditional DGNSS over AIS Centralised solution

Considering the short coverage of the AIS base stations (within LoS range), a set of base stations is normally distributed alongside rivers, canals, coast and ports to cover the whole service area. On the other hand, taking into account that the range of a DGNSS station is in the order of 200 NM, the corrections generated by a reference station are normally used to feed multiple AIS base stations. This means that the same DGNSS corrections are transmitted by several stations. This architecture is depicted in the following figure.

DGNSS over AIS Network Configuration

The EGNOS corrections in RTCA format can be customized and converted into RTCM format for any location placed within the EGNOS service area. Therefore, it is possible to generate RTCM data streams customised for each AIS base station and therefore provide DGNSS corrections for short baseline lengths.

At very high level, the architecture of this solution would consist on:

- Central Facility (CF), responsible for the generation of the PRC corrections (including integrity).

- Monitoring Network, providing GNSS data for the integrity monitoring check.

- AIS Service Manager (ASM), which retrieves the EGNOS based DGNSS corrections in RTCM format and converts them in an IEC 61162 VDM sentence (discarding the preamble and parity fields) to be then distributed to the final users by the AIS base stations using the VDL channel.

EGNOS based DGNSS corrections over AIS architecture

A more detailed description of each of these components is provided below:

    - Central Facility

The Central Facility is the main component of a centralised EGNOS based DGNSS service. The primary function of the Central Facility is to compute the Pseudorange Corrections for all the satellites above the elevation mask. PRCs and ancillary information (e.g. antenna location) are encoded into RTCM 10402.3 and transmitted to each beacon transmitter site.

The source for the generation of the DGPS corrections to be broadcast by the transmitter could be the EGNOS Signal in Space or the EGNOS messages received from EDAS.

Also, in order to check the integrity of the corrections computed based on EGNOS, the Central Facility processes the GPS raw data received from a network of GNSS receivers. This network could be a dedicated/proprietary one (set of receivers specifically deployed near the AIS transmitters) or take advantage of the GNSS networks available.

    - Monitoring Network

As detailed above, the Central Facility needs to have access to GPS measurements collected from a receiver located within the validity area of each set of DGNSS corrections.

One alternative is to have a dedicated network of GNSS receivers. These receivers shall be capable of transmitting (via internet) the raw data collected to the Central Facility.

Another option is to get the GNSS raw data (used for the integrity monitoring) from one of the different networks of receivers available. The main disadvantage of this solution is that the Aton provider needs to rely on an external entity, so it could be necessary (service provider decision) to establish a Service Level Agreement (SLA) to guarantee the reception of the data with the quality and availability required.

    - AIS Service Manager (ASM)

The RTCM corrections generated by the central facility are transmitted to the AIS Service Manager which converts them in an IEC 61162 VDM sentence (discarding the preamble and parity fields) to be then distributed to the final users by the AIS base stations using the VDL channel.

Internally to the AIS service (ASM or LSS) each corrections set will be routed to the target AIS Base Station (AIS-PCU).

It is important to remark that this component does not need to be modified with respect to a traditional DGNSS solution. All the inputs/outputs are the same and in the same format, therefore, no change is required. This means that the fact that the RTCM corrections are generated based on the EGNOS message or by a traditional DGNSS reference station is completely transparent for the ASM.

  - Trade-Off Analysis

This assessment has been done taking into account the key features (described in the table below) derived from the baseline architecture and considering the following colour code:

- Green colour: Feature improved w.r.t. the baseline.

- White colour: Same as the baseline (no improvement/degradation).

- Red colour: Feature degraded w.r.t. the baseline.

It should be noted that all the architectures analysed, provide integrity monitoring. For that reason, the “integrity monitoring” capability has not been included in in the trade-off analysis (see section below), since there is no difference among the architectures addressed.

- Key features (legend) used for the assessment

| Key feature ID | Key feature | Baseline (Current) Architecture | Assessment |  |  |
| --- | --- | --- | --- | --- | --- |
| INF | Infrastructure at site | 1 RS + 1 IM (external to the AIS Base Station) | Reduced infrastructure | Similar infrastructure | Additional infrastructure |
| LOC | Local effects impact corrections | Yes | No | Yes | N/A |
| IND | Independence of corrections generation vs integrity check | Yes | N/A | Yes | No |
| COM | Communication lines | Standard communication lines | No communication lines | Standard communication lines | High availability communication lines |
| VDL | VDL Monitoring | No | Yes | No | N/A |
| RED | Redundancy | No | Increased redundancy | No | N/A |
| JSR | Jamming and Spoofing Resiliency | No | Yes | No | N/A |
| SEP | Corrections generation separated from the transmission means | No | Yes | No | N/A |
| CUS | Customized corrections for each AIS Base Station | No | Yes | No | N/A |
| INT | Access to internal data (GNSS raw data or signal parameters) from the base or mobile station needed | No | N/A | No | Yes |

Table 0-1: Trade-Off Legend

It is highlighted that for some features, only two of the above assessments will be possible. As an example, for the VDL Monitoring capability, a ‘red’ assessment is not possible (degradation) since:

- Architecture provides VDL monitoring: improved situation with respect to baseline architecture (green colour).

- Architecture does not provide VDL monitoring: same situation as baseline architecture (white colour).

On the other hand, for the EGNOS based decentralised solution the assessment of the ‘COM’ and ‘JSR’ features depends on the data used for the reference station (SIS or EDAS SISNeT service). For instance, in case of getting the SBAS data from the SIS, high availability communication lines are not needed (‘COM’) but the solution is not robust in front of jamming attacks. For that reason, the corresponding cells have been split to account for these two different options.

The table below presents the trade-off analysis performed for the indicated architectures:

- Trade-Off Analysis - EGNOS over AIS

| Title | INF | LOC | IND | COM |  | VDL | RED | JSR |  | CUS | INT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline |  |  |  |  |  |  |  |  |  |  |  |
| EGNOS based decentralised |  |  |  |  |  |  |  |  |  |  |  |
| EGNOS based centralised |  |  |  |  |  |  |  |  |  |  |  |

Mapping of RTCA SBAS messageS to RTCM 2.x

IALA DGNSS employs the principle that the main sources of error in satellite navigation (i.e. satellite clock errors, satellite ephemeris errors, tropospheric and ionospheric delay estimation errors) are highly correlated for two users located relatively close to each other. Differential GNSS corrections are computed by placing a reference station with a GNSS receiver at a known location, determining corrections to the satellite ranging signals, and broadcasting these corrections to users. This removes the bias errors common to the reference station and user receivers, and improves the positional accuracy. The accuracy is then limited by user receiver noise, inter-channel biases, user local effects and differential station location uncertainty.

EGNOS, as any SBAS system, is also providing corrections to the same errors (i.e. satellite clock and ephemeris errors and ionospheric delay estimation errors), with one exception: the troposphere. For this error source, SBAS systems do not provide corrections; users are expected to apply a model to reduce the error in the position due to this effect.

Taking this into account, the EGNOS messages can be used for the generation of Virtual Reference Stations (VRS). The principle behind this solution would be to convert the wide area EGNOS corrections in RTCA format, into local area corrections in RTCM SC-104 format  (EGNOS-VRS) for the locations of interest for maritime users (e.g. beacon locations).

For the sake of clarity, the Pseudorange corrections provided by EGNOS will be called from now on EGNOS PRC, the Pseudorange corrections calculated by DGNSS reference stations will be called DGNSS PRC and the Pseudorange corrections calculated mapping EGNOS differential corrections into RTCM format will be called E-DGNSS PRC.

  - Pseudorange Rate Corrections for RTCM 2.X MT1/MT9

RTCM corrections can be reconstructed from SBAS provided data using the following equations:

PRCtz-count=PRCtof+RRCtof×tz-count-tof+δ∆ttz-count×c+εtz-count+ICitz-count+TCitz-count

Where:

| PRC(t z-count) | is the RTCM MT1 PRC broadcast to the user |
| --- | --- |
| PRCtof | Is the most recent EGNOS fast corrections MT2-5 and 25 |
| RRCtof | Is the range rate corrections calculated according to equation A-17 in MOPS Appendix A using the most recent fast correction and a previous one. |
| tof | Time of applicability of the most recent EGNOS fast correction |
| tz-count | Reference time for the RTCM MT1 parameters |
| δ∆ttz-count | EGNOS clock correction from MT25 calculated according to MOPS Appendix A. |
| c | Speed of light 2.99792458x108 m/s |
| εtz-count | EGNOS ephemeris correction from MT25 projected to the radial direction user-satellite (line of sight). This radial ephemeris correction is inaccurate as it is kept the same value for the whole period of use of the E-DGNSS PRC. For a pure SBAS user applying MOPS, the radial ephemeris correction is embedded in the satellite coordinate vector and therefore in the pseudorange calculation. |
| ICitz-count | Is the ionospheric delay calculated according to equation A-41 in MOPS Appendix A. Note this is always a negative value. |
| TCitz-count | Is the tropospheric delay calculated according to section A.4.2.4 in MOPS Appendix A. Note this is always a negative value. |

The  εtz-count maximum error induced should be estimated. Furthermore, this error contribution is present in the RRC computation below, so it will be somewhat compensated by the PRC estimation mechanism which uses this RRC. It is to be confirmed that the residual effect might then be dependent on radial acceleration only.

The provision of the εtz-count implies the need of the reference station to have track to that specific satellite. However, according to [5], DGNSS must send corrections to all satellites in view of the reference station. The satellites monitored by EGNOS might be a different set of satellites than those in view from the DGNSS reference station. The corrections then shall be sent to the subset of GPS satellites monitored by EGNOS and also in view from the station. For the GPS satellites in view by the reference station and not-monitored by EGNOS, the E-DGNSS PRC shall be set to 1000 0000 0000 0000 and the E-DGNSS RRC be set to 1000 0000, preventing the rover to use these satellites.

Upon the transmission of a new clock and ephemeris data from GPS, EGNOS continues to broadcast corrections to the old long term clock and ephemeris data for a period of 2 to 4 minutes so all the users can acquire the new GPS data. The corrections processor should take this EGNOS characteristic into account

  - Range Rate Corrections for RTCM 2.X MT1/MT9

DGNSS Range Rate Correction (RRC) is an attempt to "extend the life" of the pseudorange correction as it "grows old" [5]. A possible E-DGNSS RRC can be calculated using the difference between the last calculation of EGNOS PRC and a previous one available at the corrections processor. The approach proposed from [1] is basic and a first or second order filter would be possibly more appropriate, especially for the compensation of the residual ephemeris error discussed above εtz-count.

RRCtz-count=PRCtcurrent-PRCtprevious∆t

Where:

| RRC(t z-count) | is the RTCM MT1 RRC broadcast to the user |
| --- | --- |
| PRCtcurrent | Is the most recent E-DGNSS PRC calculated using the last set of EGNOS fast corrections available and its correspondent EGNOS slow corrections and, ionospheric corrections. |
| PRCtprevious | Is a previous E-DGNSS PRC calculated using a previous set of EGNOS fast corrections available and its correspondent EGNOS slow corrections and , ionospheric corrections. |
| tz-count | Reference time for the RTCM MT1 parameters |
| ∆t | tcurrent-tprevious |

The calculation proposed is in line with MOPS229D. However, other approaches may yield better RRC performance. DGNSS receiver typically use a proprietary RRC algorithms based on a second order filter.

As the update rate of EGNOS fast corrections (typically six seconds) could be higher than the DGNSS MT1 update rate, note that the most recent E-DGNSS and a previous E-DGNSS PRC not necessarily are the last two E-DGNSS broadcast in MT1.  The E-DGNSS PRC is a “snapshot” of the whole EGNOS corrections in the reference station at a specific epoch (slow, fast, ionosphere and troposphere), therefore the update rate might be selected by the reference station and there is no constraint from EGNOS messages update rate.

  - E-DGNSS UDRE

According to [5], DGNSS UDRE does not support integrity information (it is only used to weight the measurements to obtain a more accurate solution), DGNSS UDRE is a one-sigma estimate of the uncertainty in the pseudorange correction as estimated by the reference station, and combines the estimated effects of multipath, signal-to-noise ratio, and other effects.

In the other hand, EGNOS UDRE provides integrity information for the EGNOS user and it only bounds the combined fast and long term corrections. The E-DGNSS UDRE concept should be in line with DGNSS UDRE concept (and not the EGNOS UDRE). The calculation can be done following MOPS Appendix J.

σi2=σi,flt2+σi,UIRE2 + σi,TROPO2

Where

| σi2 | E-DGNSS UDRE (squared) for an specific satellite at t z-count |
| --- | --- |
| σi,flt2 | Model variance for the long term, fast and range rate corrections as defined in MOPS Appendix A and J |
| σi,UIRE2 | Model variance for the slant range ionospheric error as defined in MOPS Appendix A and J. |
| σi,TROPO2 | Model variance for the tropospheric error as defined in MOPS section A.4.2.5. |

Sigma value obtained will be coded according to Table 4-6 and scaled according to Table 4-2 in [5]. Sigma values could vary depending on the location and time but typical ranges could be around 1 m and 5m. DGNSS UDRE scale factors such as 0.3, 0.5 or 0.75 might be the most used.

  - E-DGNSS integrity alerts

EGNOS sends integrity flags for GPS satellites. EGNOS effective time to alert is 6 seconds. E-DGNSS must take advantage of the alert information broadcast by EGNOS. E-DGNSS might set a satellite as “Do not Use” (DU) immediately using RTCM MT1 or MT9, if EGNOS sets that satellite DU through EGNOS MT6. For the specific satellite, the DGNSS MT1/9 PRC field shall be set to binary 1000 0000 0000 0000 [5] and MT1/9 RRC shall be set to binary 1000 0000 which indicates a problem and the User Equipment should immediately stop using this satellite.

Other possible situations when E-DGNSS must send an alert for a satellite are:

- when either the E-DGNSS PRC or the RRC is higher than the maximum range for PRC (±10485.44m) or RRC (±4.064m/s) allowed by RTCM MT1.

- when EGNOS ionosphere corrections are not available for a  satellite monitored by EGNOS.

  - Summary E-DGNSS parameters

The parameters that must be included for each satellite in RTCM MT1/MT9 for E-DGNSS are:

- Derivation of E-DGNSS

| E-DGNSS Parameter for DGNSS MT1 | Derived in |
| --- | --- |
| UDRE | σi2 |
| PRC | PRCtz-count |
| RRC | RRCtz-count |
| ISSUE OF DATA | Issue of data of the GPS navigation data being used. Also included in MT25 of MOPS Appendix A [1] |

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain