IALA Guideline

1???

The Technical Approach to Establishing a Maritime eLoran Service

Edition 1.0 (Draft)

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

1.	INTRODUCTION	5

1.1.	Performance requirements	5

1.1.1.	Requirements for a Backup Navigation System	5

1.2.	An Overview of eLoran	6

1.3.	eLoran signal structure	8

1.4.	Recommended reading	11

1.5.	The development of eLoran	12

1.5.1.	Positioning Mode and eLoran Receivers	12

1.5.2.	The Loran Data Channel	13

1.5.3.	Additional Secondary Factors (ASF)	13

1.5.4.	DLoran	16

1.5.5.	Time and Frequency Equipment	17

1.5.6.	Benefits of eLoran	17

1.6.	Standards Process	17

1.6.1.	International Regulatory Bodies	17

1.6.2.	The Status of Loran Documentation	18

2.	THE PROCESS OF ESTABLISHING AN eLORAN SERVICE	18

2.1.	System Coverage Prediction and Analysis	20

2.2.	Identify Locations for Transmitters	20

2.3.	Select Group Repetition Interval(s)	21

2.4.	Install Transmitters	21

2.5.	Establish Applications - Voyage Phase Requirements	21

2.5.1.	Port Approach Voyage Phase	21

2.5.2.	Coastal Voyage Phase	23

3.	OPERATIONAL ASPECTS	24

3.1.	Concept of Operations	24

3.2.	Reference Datum	24

3.3.	Monitoring	24

3.4.	Publication of information	25

3.5.	Performance Verification	26

3.5.1.	Coverage Verification	26

3.5.2.	Availability Verification	26

3.5.3.	Continuity Verification	27

3.5.4.	Integrity Verification	27

3.5.5.	Accuracy Verification	29

4.	ACRONYMS	29

5.	REFERENCES	31

List of Tables

Table 1	Requirements for GNSS systems [based on IMO A.1046 (27) & IMO A.915(22)	5

Table 2	Suggested minimum maritime user requirements for a backup system for general navigation	6

Table 3	eLoran required reading for service providers	11

Table 4	Definitions of different generations of Loran	12

Table 5	Information on maintenance and outages to be promulgated to users	25

List of Figures

Figure 1	The service architecture for eLoran and the components required. After ‘eLoran Definition Document’, V1.0, October 2007, International Loran Association	8

Figure 2	eLoran signal structure and frequency spectrum	9

Figure 3	A model of the power spectrum of an eLoran signal	10

Figure 4	Zoomed in view of the power spectrum shown in Figure 3	10

Figure 5	Real-life, off-air signal measurements	11

Figure 6	Example of all-in-view positioning using Anthorn, Lessay and Sylt. With all-in-view mode signals need not belong to the same GRI and more than three signals are employed in the positioning solution in order to provide integrity	14

Figure 7	ASF due to Lessay results in a position offset if it is not taken into account.	15

Figure 8	An example of an ASF Measurement System	16

List of Equations

Equation 1	Geographical range	4

Equation 2	Theory of Special Relativity	4

# INTRODUCTION

Service providers are opting to provide eLoran services as part of a robust PNT solution, as eLoran is dissimilar and complementary to GNSS services.  The aim of this Guideline is to enable service providers to deliver, monitor and assess the performance of eLoran services in a common manner.

System performance is based on the assumptions that the system provider conforms to these Guidelines and that the user equipment meets the design and installation standards as specified in the referenced documentation.

## Performance requirements

IMO Resolution A.1046 (27) [1] details the requirements for World-Wide Radio Navigation Systems (WWRNS) considering vessels operating in the Ocean and harbour entrances, harbour approaches and coastal waters.  The requirements are described by accuracy, integrity, availability, and continuity [2].  Table 1 summarizes the requirements specified in IMO Resolution A.1046 (27), whereby the requirement on availability is given as signal availability describing the availability of radio navigation signals in the specific coverage area [3].

Requirements for GNSS systems [based on IMO A.1046 (27) & IMO A.915(22)

|  | System Level |  |  |  | Service Level |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Absolute Horizontal Accuracy (95%) | Integrity |  |  | Signal Availability (2 years) | Continuity (over 15 minutes) |
|  |  | Alert Limit | Time to Alarm | Integrity Risk |  |  |
| Area | m | m | s | % | % | % |
| Ocean | < 100 | N/A | N/A | N/A | > 99.8 | N/A |
| Harbour entrances, harbour approaches and coastal waters | < 10 | 25 | 101 | 1 x 10-5 | > 99.8 | 99.97 |
| 1 Generation of integrity warnings in cases of system malfunctions, non-availability or discontinuities. |  |  |  |  |  |  |

Receiver equipment for GNSS systems intended for navigational purposes on ships with maximum speeds not exceeding 70 knots shall meet the minimum performance requirements outlined in Table 1.

### Requirements for a Backup Navigation System

As stated in [4]:

‘Where the risk assessment concludes that a backup system (i.e. a system ensuring continued operation, but not necessarily with the full functionality of the primary system) is necessary, suggested minimum maritime user requirements (derived from IMO Resolution A.915(22)) for such a system are listed at Appendix A.  It may however be impractical to expect backup systems to achieve some of these standards, such as global coverage in the ocean phase of navigation or metre level accuracy in the port phase.  In these cases, it might be necessary to navigate the ocean phase by dead-reckoning, or delay port manoeuvres until the primary navigation system is restored.  The argument for a backup system may be dependent on the perceived threat to the primary system and the likely duration of primary system outages.’

For convenience Table 2 illustrates the user requirements for a backup system as outlined in Appendix A of [4].

Suggested minimum maritime user requirements for a backup system for general navigation

There are two main voyage phases of interest to the eLoran user community; Port Approach Phase, and Coastal Phase.

Port Approach Phase requires the highest achievable accuracy performance from eLoran, ~10 m (95%).  This can only be achieved through bespoke Additional Secondary Factor (ASF) measurements and the installation and use of a Differential-Loran (DLoran) reference station at each port.

Coastal Voyage Phase requires 100 m (95%) accuracy.  This can be achieved by eLoran without the use of differential-Loran reference stations and by computer modelling ASF data over a wide area and then calibrating the data with a sparse set of measured ASF data, which may most conveniently be collected automatically by vessels going about their everyday business – we refer to these vessels as ‘vessels of convenience’.  Coastal ASF maps have been shown to be able to provide positioning accuracy of the order of 30 to 50 m.

These two statements, made without explanation of the key technical terms at this stage, cover the foundation of maritime eLoran service provision and the rest of this document describes the motivation, background and the methodology required to achieve this performance with eLoran.

## An Overview of eLoran

Enhanced Loran (eLoran) is a low-frequency, long range Terrestrial Radionavigation System, capable of providing positioning, navigation and timing (PNT) service for use by many modes of transport, including maritime.  eLoran transmits pulsed groundwave signals with a central frequency of 100kHz.  This low frequency gives the signals their LOng RAnge Navigation capability from widely spaced transmitters.  The receiver’s position is determined by the measurement of the times of arrival (TOA) (or pseudorange) of these pulses.

Pseudoranges from at least three transmitters are required to be measured in order to determine a position solution by trilateration.  Measuring more than three transmissions (preferably five) provides the user with RAIM (Receiver Autonomous Integrity Monitoring) capability in addition to positioning accuracy.

An eLoran system includes the following elements:

Several eLoran transmitters broadcasting a UTC synchronised and standardised eLoran signal [5];

The signal incorporates a data message channel (the Loran Data Channel), which may take several forms [6], [7];

An identified service area, in which the signal propagation characteristic, represented by Additional Secondary Factors (ASFs), have been measured or modelled through software with the resulting modelled data calibrated using a much smaller set of measurements than would otherwise be required without such modelling [8];

Where accuracy is required to support the Port Approach Voyage Phase, Differential-Loran (DLoran) Reference Stations should be installed;

These calculate differential corrections, which are sent to the mariner via the Loran Data Channel for reception using the same eLoran receiver used for positioning [9], [10];

An infrastructure based integrity monitoring system that takes two main forms:

Alarms and alerts concerning the health and status of eLoran transmitters and their associated transmissions and the health and status of DLoran reference stations and their transmitted differential-corrections;

The capability to monitor the effects of solar weather is required to be installed in locations that are particularly prone to such effects (geomagnetic storms, proton events, coronal mass ejections etc.).

The effects on eLoran vary with the geomagnetic latitude of the user and the distance between the user and the transmitters. Integrity monitors of this kind remotely monitor the quality of the received signals and are able to interface to the Loran Data Channel in order to issue timely integrity warnings to the mariner.  Integrity monitors may be co-located with DLoran Reference Stations where appropriate, however, separate processing hardware may be required because of the more complicated mathematical analysis of the signals required to be performed by the monitoring receiver/computer setup.

A Control and Monitoring Centre, which provides a remote human/machine interface to the set of transmitters and/or DLoran reference stations deployed by the service provider. Its role is to control the system(s) and monitor alarm conditions;

Control Centre personnel should have the power to deploy engineers to remedy any issues with equipment. It may be that the organisation that is responsible for the set of transmitters is not the same organisation that is responsible for the DLoran reference stations, in which case the eLoran transmitter Control/Monitor Centre may be separate from the DLoran Reference Station Control/Monitoring Centre.

A data communications backbone is required for the various components of the system to communicate with their respective Control Centres, and to link DLoran reference stations with transmitters.

Such data communications include items such as control messages, differential correction information, integrity alerts, UTC time dissemination etc.  The data backbone will usually be implemented using Internet Protocol, and may run on a private network, or via the public Internet.  In the latter case it is appropriate to implement one or more Virtual Private Networks (VPN) for data security purposes.

Figure 1 illustrates an overview of the eLoran system for maritime applications.

The service architecture for eLoran and the components required. After ‘eLoran Definition Document’, V1.0, October 2007, International Loran Association

## eLoran signal structure

Figure 2 shows the signal structure and frequency spectrum of the eLoran signal [5].  The figure is divided into three sections. In the top left of the diagram is shown the individual Loran pulse.  It has a specially designed shape (given by the equation underneath the plot) with a centre frequency of 100 kHz.  At such a low frequency the main mode of propagation is by a very stable radio frequency groundwave.  The front edge of the pulse is carefully designed to provide high signal strength at the tracking point within a receiver, while the tracking point is early enough in the pulse to avoid early arriving skywave (an unwanted reflection of the groundwave pulse off the ionosphere). The pulse duration is approximately 250 µs.  The tail of the pulse is not defined; however, the transmitter suppresses it sufficiently to maintain 99% of the energy of the eLoran signal within the band 90 kHz to 110kHz as shown in the frequency plot in the top right of the figure.

A transmitter broadcasts a group of 8 of these ‘navigation’ pulses, transmitted at 1 millisecond spacing.  The additional 9th pulse (actually in 10th pulse position!) shown in the Master pulse group in the diagram is now obsolete for eLoran, being a legacy of Loran-C.  However, 9th and 10th pulses may be added in order to implement the Loran Data Channel.

eLoran signal structure and frequency spectrum

All eLoran transmitters transmit at the same frequency, so they cannot all transmit at the same time.  Instead one transmitter, designated the Master, transmits a group of pulses followed a set time later by one of several successive Secondary transmitters.  The time delay between the Master transmission and a Secondary is called the Emission Delay (ED) of that Secondary.  The time interval between successive Master station transmissions within the same group is called the Group Repetition Interval (GRI), also sometimes referred to as the ‘rate’.

A Secondary transmitter’s Emission Delay includes the signal propagation delay between the Master and the given Secondary, and a Coding Delay (CD) intended to position the Secondary transmissions within the GRI such that nowhere in the coverage area do the transmissions overlap.  Careful design of the GRI and selection of Coding Delays are a vital part of system design.

The pulses of each GRI have an alternating Phase Coding according to the diagram shown in the lower right of Figure 2, and the PC term in the equation.  A Phase Code Interval (PCI) is made of up two GRIs (A and B) the phase coding of GRI A is different to that of GRI B.  A ‘+’ sign indicates a shift in phase of the pulse by 180, while a ‘-‘ indicates a 0 phase shift.  Phase coding is provided in order to mitigate the effects of long-delay, multi-hop skywave by employing correlators in the user’s receiver.

Figure 3 shows the ‘total’ spectrum of the eLoran signal (provided through computer modelling). The spectrum of the pulse shown in Figure 2 is convoluted with the spectrum of the rate structure shown at the bottom of that figure. The effect of this rate structure can be seen in Figure 4, which shows that the eLoran spectrum is made up of a large number of finely separated spectral lines each separated by: 1/(2×GRI).

A model of the power spectrum of an eLoran signal

Zoomed in view of the power spectrum shown in Figure 3

Finally, Figure 5 shows real-life measurements of the eLoran spectrum made using an eLoran receiver’s Fast Fourier Transform (FFT) algorithm.  The peaks seen in the spectrum are continuous wave interfering (CWI) signals appearing in and around the frequency band; the green bands are notch filters implemented by the receiver to mitigate this CWI.

Real-life, off-air signal measurements

## Recommended reading

Table 3 contains a list of standards and other documentation that should be considered to be ‘required reading’ by a service provider intending to provide eLoran services, and as background to the understanding of the implications of the rest of this IALA Guideline.  Other references and relevant background reading can be found in Section 5 of this document.

eLoran required reading for service providers

| Topic | Document(s) |
| --- | --- |
| eLoran evolution and recommendations | ‘Loran’s Capability to Mitigate the Impact of a GPS Outage on GPS Position, Navigation, and Time Applications’, Prepared for the Federal Aviation Administration Vice President for Technical Operations Navigation Services Directorate, March 2004 |
| System Definition | Enhanced Loran (eLoran) Definition Document, International Loran Association, 2007 |
| eLoran Signal Specification | Enhanced Loran (eLoran) LORIPP/LORAPP Draft Specification of the eLoran System, Rev. 4.0 |
| Receiver Minimum Performance Specifications | Minimum Performance Standards for Marine Loran Receiving Equipment, RTCM Special Committee 127 |
| Loran Data Channel | Tri-State (e.g. Eurofix) - ITU-R Recommendation M.589-2*, Technical characteristics of methods of data transmission and interference protection for radionavigation services in the frequency bands between 70 and 130 kHz 9th Pulse - ‘Enhanced Loran for Maritime Harbor Entrance and Approach’, Benjamin Peterson et al, Institute of Navigation National Technical Meeting 2004. |

## The development of eLoran

eLoran is an enhanced version of the original Loran-C system.

Any discussion of the difference between Loran-C and eLoran is complicated because Loran-C has been modernised at different times and to different extents in different areas of the World.  The definitions in Table 4 are used to assist the comparison.

Definitions of different generations of Loran

| USCG Loran-C | The original version of Loran-C (c. 1960s) based on tube transmitters, Service Area Monitor (SAM) control, ASF look-up tables and hyperbolic navigation, requiring large numbers of people on site. Typical accuracy: 460 m (95%). |
| --- | --- |
| Modernised Loran-C | The original version of the Northwest European Loran System (NELS) (c. 1990s) based on solid-state transmitters, time-of-emission control, ASF software modelling, hyperbolic or rho-rho navigation, and requiring very few people on site. Typical accuracy: 100 m (95%). |
| Prototype eLoran1 | The UK General Lighthouse Authority’s system (c. 2008) based on modernised Loran-C together with (i) Eurofix as the LDC, (ii) all-in-view navigation, (iii) precise ASF surveys, and (iii) differential-Loran reference stations for maritime use. Typical accuracy: 10-20 m (95%). |
| eLoran | This is based on (i) updated station equipment to improve timing stability, (ii) mitigation of vulnerabilities to ensure high availability, (iii) LDC at all stations, and (iv) modernised control/monitoring capability at a Control Centre. Typical accuracy: 10-20 m (95%). |

### Positioning Mode and eLoran Receivers

Loran-C was a hyperbolic system and the user measured the Time Differences (TDs) between the arrival times of Loran-C signals from pairs of transmitters (typically the master and one of the secondary stations) within the same group. Measuring all points on the earth where the same TD is measured and plotting those points on a navigation chart it would be seen that the points all lie along a hyperbolic curve defined by the location of the pair of transmitters.  Measuring another TD, using the same master but now a different secondary, derives another hyperbolic curve – where the two hyperbolic curves cross is the observer’s position.  With hyperbolic navigation each station’s transmission does not have to be synchronized to a precise clock – as long as the TDs measured between the pairs of stations is constant at a given location the system is considered to be stable.

Control of this synchronisation was performed by installing a Service Area Monitor (SAM) somewhere in the coverage area.  The SAM would be configured with a nominal set of TDs determined at the SAM’s location.  The Master station of each pair was not synchronised to any external source of time and was allowed to drift, however the secondary stations were adjusted by measurements made at the SAM to maintain the constant TD at that location.  This has the implication that the stability of the hyperbolic lines was only precisely maintained at the location of the SAM, other locations within the coverage area may witness accuracy degradation because of the distance from the SAM calibration point.

Although eLoran can still be used in hyperbolic mode, modern eLoran receivers operate in all-in-view mode – this works just like GPS.  In all-in-view, looking at all point s on the earth with the same propagation time from a transmitter and plotting those points on a navigation chart it would be seen that all those points lie on a circle with the transmitter at the centre.  Taking measurements from 3 such transmitters, thus forming 3 circles, allows a receiver to compute its position.  In order to measure the propagation time, each transmission needs to be synchronized to a precise clock, common to all transmitters.

This ensures that Loran System Time is maintained, and is based on the assumption that all Master transmitters began transmitting at midnight on 1st January 1958 (the Loran Epoch).  Each transmitter’s broadcast is therefore synchronised to UTC.  This method of control is referred to as Time of Emission (or Transmission) (TOE, or TOT) control.  This means that there is now no bias in positioning accuracy as there was when employing SAM control.  In addition, UTC messages can be broadcast over the Loran Data Channel (LDC), which include a figure for the current number of leap-seconds between Loran System Time and UT1 (mean solar time).  This implies that eLoran can be used to provide timing users with a precise source of time independent of satellite systems.

In order to use eLoran, users should operate a receiver capable of calculating an eLoran position in accordance to the technical specifications set out in [11], [12] and incorporating an up-to-date ASF correction database.

Earlier generation Loran-C receivers will be able to utilise any core Loran signals received, but will not be able to benefit from ASF mapping or the eLoran LDC messages.

### The Loran Data Channel

The principal difference between Loran-C and eLoran is the addition of an LDC by pulse position modulation (PPM) of the eLoran signal.  There are two main LDC techniques in development.  One is Eurofix, based on a standardised, balanced tri-state PPM [6].  This is the modulation of the last 6 pulses in a station’s 8 pulse group transmission, so the pulses used for Position, Navigation and Timing can also be used to transmit data.  The other LDC concept is 9th Pulse [5], [7], which is again a pulse position modulation scheme, using an extra pulse added to the end of the transmitted 8-pulse groups.  It is also possible to add a 10th and even an 11th pulse to the broadcast as long as there is space in the GRI.  For maritime services the use of tri-state PPM and Nth Pulse is not mutually exclusive.

The data channel conveys corrections, warnings, and signal integrity information to the user’s receiver via the Loran transmission.  The data transmitted may not be needed for all applications but will include at a minimum:

The identity of the station;

Almanacs of Loran transmitting and differential monitor sites - making it possible to change systems without rendering previous eLoran receivers obsolete;

Absolute time based on the Co-ordinated Universal Time (UTC) scale;

Leap-second offsets between eLoran system time and UTC;

Warnings of anomalous radio propagation conditions including early skywaves;

Warnings of signal failures, aimed at maximizing the integrity of the system;

Messages that allow users to authenticate the eLoran transmissions;

Official-use only messages;

Differential Loran corrections, to maximize accuracy for maritime and timing users;

Differential GNSS corrections may also be broadcast as an option depending on the available bandwidth of the LDC.

### Additional Secondary Factors (ASF)

eLoran receivers calculate their position by measuring how long it takes the system’s 100 kHz groundwave radio signals to reach them over the surface of the earth from the transmitters.  The measured propagation times are called ‘pseudoranges’.  eLoran receivers measure the pseudoranges of a number of signals from transmitters placed around the coverage area.

Example of all-in-view positioning using Anthorn, Lessay and Sylt. With all-in-view mode signals need not belong to the same GRI and more than three signals are employed in the positioning solution in order to provide integrity

Figure 6 shows an example of some lines of position (circles) with the transmitters at Sylt (Germany), Anthorn (United Kingdom) and Lessay (France) at the centres.  The reception of signals from three stations is sufficient to compute a position but more signals are typically employed in the positioning solution in order to provide integrity.  Signal received need NOT belong to the same GRI.

The receiver measures these time delays and, by assuming a value for the propagation speed of the signal, the position relative to the transmitters can be computed.  The speed of a ground-wave radio signal depends on the electrical conductivity of the surface over which it travels.  For example, it travels slowest over ice, deserts and mountains, a little more quickly over good farm land and quickest of all over sea water.

eLoran receivers compute their position in two stages.  Firstly, they assume that the entire earth’s surface is covered in sea-water and they therefore employ a sea-water propagation model for the speed of propagation [11].  This model is based on a set of standard parameters adopted by the United States Coast Guard and models propagation over sea-water in earth’s atmosphere very accurately.

In the second stage, the delays due to land paths are taken into account by adding them to the pseudorange measurements.  These delays are called Additional Secondary Factor (ASF) delays.  Their values are expressed as microseconds of propagation delay, and are typically supplied to users as a database built into their receivers.  ASF are the dominant propagation phenomenon affecting the accuracy of positioning and navigation with eLoran.

If ASF are not taken into account they appear as a bias, or offset, in the measured pseudorange of an eLoran signal.  Figure 7 illustrates this; the position of the vessel determined by the eLoran receiver is offset from its true position because of the land in the propagation path from the eLoran transmitter at Lessay.

ASF due to Lessay results in a position offset if it is not taken into account.

ASF data will be required to be provided to the mariner.  There are two main voyage phases that require to be supported:

Port Approach Voyage Phase – data will be measured through bespoke measurement campaigns at each port approach as discussed above

Coastal Voyage Phase – data will be modelled to the best of the service provider’s ability.  The modelled data will be calibrated using a small amount of ASF measurements gathered through the operation of ‘vessels of convenience’.

eLoran receivers may be integrated with GNSS, and such ‘Resilient PNT’ receivers are available.  These receivers can be employed not just for navigation purposes but for automatically measuring and logging ASF data, since they already have a GNSS receiver ‘built in’.

#### Determining ASF Values

The eLoran service provider is responsible for producing maps of Additional Secondary Factors for its operating region. There are several ways to produce ASF data:

- Measure ASFs directly using an ASF measurement system.

Model ASFs using computer software.

Combine modelled data with a set of measurements measured using vessels.

#### Measuring ASFs

The simplest, and most accurate, way to map ASF is by measuring them using special equipment aboard a survey vessel.  The true position (the ground-truth) is typically determined using differential GNSS, which has a horizontal accuracy of approximately 1 m (95%).  If the ground-truth position is compared to the position given by an eLoran receiver with a sea-water only propagation model, the position offset would be due to ASF.  However, ASF measurement is expensive in terms of ship time, and so this method is reserved for bespoke surveys for areas requiring the greatest positioning accuracy available from eLoran – Harbour Approaches.

Figure 8 shows an example of an ASF measurement system.  The unit consists of an eLoran receiver; a GPS receiver to provide ‘ground-truth’ position and a precise time-tag against which to measure the eLoran signals’ times of arrival; custom electronics, and a PC in one convenient 19inch rack mountable unit.  ASF surveying software runs on this system and is used to process and validate the ASF data collected [8].  During bespoke surveys the equipment is typically monitored by a human operator and the success of the ASF measurement process, and the quality of the resulting data can be determined in real-time lending the possibility of re-surveying areas should a problem be found.

An example of an ASF Measurement System

#### Modelling ASFs

Measurement is only practical for small regions such as harbour approaches where methods of surveying have evolved that take into account the physics of eLoran signal propagation and the characteristics of ASF as they build up over land along the signal propagation path from the transmitter to the mariner’s vessel.  For larger areas it is more appropriate to model the ASF data using computer software.

There are sophisticated methods available for modelling eLoran propagation and producing software generated ASF maps.  These techniques take into account the electrical ground conductivity of the surface of the earth (land or sea), terrain elevation and coastline, using a set of databases.  ASF maps are produced in grid format and at required grid densities for a given coverage area.  In this way large ASF maps, one for each eLoran transmitter within the coverage area can be produced; as has been done for the waters around the United Kingdom and Ireland [10].  Such maps, following calibration with measured ASF data, are suitable for use during the Coastal Voyage Phase, where the mariner may be far away from a DLoran Reference Station. Interpolation is used to determine the ASF for a vessel’s location when between grid points [11].

### DLoran

In order to get the highest accuracy from eLoran, a service called DLoran will need to be provided in those regions requiring 10 m (95%) accuracy – for example during a harbour approach.  DLoran works in a similar way to Differential GNSS, in that a DLoran Reference Station is set up to cover a specific service area.  Radio propagation speed over sea-water is well known and easily modelled [11].  The land along the propagation path needs to be taken into account because the signal propagates slower over land and therefore is subjected to additional propagation delay above the delay due to sea-water propagation.  This extra delay is compensated for by providing a table of ASF, within the receiver, for each transmitter.

During the establishment of the service area precise ASF measurements would be made along the harbour approach and the data processed ahead of eventual publication.  However, ASF measurements are only valid for the day on which they are made because the electrical characteristics of the land change with rainfall and season of the year.

DLoran Reference Stations continuously measure the propagation time of the eLoran signals to the service area and compute the differences between the nominal measured ASF stored within the reference station, and the current value of the ASF measured by the eLoran receiver at the reference station.  This correction is broadcast to the user via the LDC; and the corrections used to obtain the best positioning accuracy possible.  Variations in TOA of the signals also include components due to transmitter timing variations and atmospheric effects; these are automatically lumped together with the ASF variation in the computed differential correction.

### Time and Frequency Equipment

New eLoran transmitters contain up to three caesium clocks.  The increased timing stability and integrity of the eLoran signal means that the signal can be used as a precise source of time and frequency for telecommunications applications, power grid synchronization, financial transactions etc.  UTC can be recovered to an accuracy of 50ns or better from an eLoran transmission, and the frequency stability is in the order of 10-12 or better.

eLoran signals should be synchronised to Universal Co-ordinated Time (UTC).

The Signal Performance Standard [5] requires:

The 10 second exponential moving average of all navigation pulses (not modulated) shall be within 25 ns of UTC;

The 1 second exponential moving average of all navigation pulses (not modulated) shall be within 100 ns of UTC;

The peak to peak variation of the 5 second exponential moving average of all navigation pulses (not modulated) shall be less than 10 ns within a 20-minute period.

Time of Emission adjustment is performed by frequency steering of the driving Caesium oscillators, rather than making integer time adjustment steps [13].

### Benefits of eLoran

eLoran is an aid to navigation that is complementary to GNSS, providing continuation of Position, Navigation and Timing (PNT) service provision when GNSS becomes unavailable for any particular reason. eLoran receivers may be integrated with GNSS receivers providing a seamless backup.

eLoran meets the position accuracy requirements for non-precision approach for aviation; the harbour entrance and approach requirements for the maritime sector; and the timing and frequency requirements of communications system providers. It is a system that has no failure modes in common with GNSS.  It is complementary in the following way:

eLoran is Low Frequency while GNSS is Ultra High Frequency;

eLoran has transmitters on the earth while GNSS has space-based transmitters;

eLoran is high power while GNSS is very low power.

## Standards Process

This section sets out the roles of the various international bodies involved in standardisation of Position, Navigation and Timing equipment and systems.

### International Regulatory Bodies

The International Maritime Organisation (IMO) is the United Nations specialized agency with responsibility for the safety and security of shipping and the prevention of marine pollution by ships.  IMO sets the performance standards for systems and onboard equipment, based on established user requirements.

The International Telecommunication Union (ITU) is the UN specialized agency for information and communications technologies.  ITU determines the technical characteristics and radio spectrum requirements for radio-determination systems.

The International Electrotechnical Commission (IEC) is the international standards and conformity assessment body for all fields of electrotechnology.  IEC provides the requirements and test specification for the onboard user equipment, based on the output from IMO and ITU.

The Radio Technical Commission for Maritime Services (RTCM) is an international non-profit scientific, professional and educational organization.  RTCM prepares recommendations for maritime radio equipment and services, which may be used in IEC specifications.

### The Status of Loran Documentation

There are existing IMO, ITU, RTCM and IEC standards and recommendations for Loran, but these are for Loran-C and relate to outdated technology.

#### ILA & RTCM

The International Loran Association (ILA – now defunct) produced documentation for eLoran. In 2007 the ILA published an eLoran Definition Document and this is the starting point for preparing performance standards and an interface control document.

RTCM established Special Committee 127 (SC-127) for eLoran and a new RTCM performance standard is being developed, including a final signal specification for DLoran correction characteristics.  The draft receiver standard has been adopted by SC-127 and may provide input to an IEC document.

RTCM also set up Special Committee 131 (SC-131) on Multi-system Shipborne Navigation Receivers.  New draft Performance Standards for Multi-system Navigation Receivers were finalised and approved by IMO in June 2015 for implementation in 2017.  SC-131 has also prepared a specification to be provided to IEC as a draft Requirements and Testing standard.

#### ITU – Technical Characteristics & Spectrum Requirements

Recommendation M.589-3 (08/01) already provides technical characteristics for tri-state data transmission and interference protection.  It requires revision and the addition of an annex giving more detail of the modulation system to be adopted.

#### IMO – Performance Standards & WWRNS Recognition

IMO Resolution A.818(19) contains performance standards for Loran-C, based on long out-of-date technology.  A new resolution is required before a World Wide Radio Navigation System (WWRNS) submission could be made.

The IMO Performance Standards for Multi-system Receivers contain a reference to PNT Guidelines, which mainly refer to onboard systems and have yet to be fully developed.

#### IEC – Equipment Test Standards

IEC document 61075 Ed. 1.0 (1991-07) “Loran-C receivers for ships – Minimum performance standards, methods of testing and required test results” is based on an RTCM standard from the 1980s and is not relevant to modern Loran.

#### IALA – Operational Standards for Service Providers, Database of stations & GRIs

The database of transmitting stations and GRIs, maintained by IALA, should be updated as an S-200 series Product Specification as new data is produced or changes are made to almanacs.

IALA is likely to be involved in developing guidelines for PNT infrastructure that could include terrestrial systems as backups to GNSS.

# THE PROCESS OF ESTABLISHING AN eLORAN SERVICE

This section provides an overview on how a maritime service provider could establish an eLoran service. Essential background to this process can be found in the essential reading list of Table 3, other required reading material is also identified in order to orientate the service provider or its contractor to the technology.

Consideration must be given to the following aspects:

transmitters – location, number, GRI, size of antennas and Effective Radiated Power (ERP). Also the physical security of the sites; and repair / maintenance duties;

Control Centre(s) – location, duties, Time of Emission (TOE) control;

Differential-Loran (DLoran) Reference Stations – location, number and LDC data bandwidth requirements;

data communications backbone for transmitters and DLoran reference stations – architecture, design and security;

infrastructure based integrity monitoring – ‘early skywave’ effects, transmitter and reference station health and status monitoring and reporting;

additional Secondary Factor (ASF) surveys for Port Approach Voyage Phase;

additional Secondary Factor data for Coastal Voyage Phase;

ASF database publication and updates;

transmitter and DLoran reference station almanac publication;

LDC - including message types, structure, encryption and data bandwidth requirements;

development of standards (both National and International) including equipage requirements for vessels operating in Service Provider’s national waters.

As a guide for further investigation each of the steps involved is outlined below:

- System coverage prediction and analysis:

Obtain/develop software or consultancy to perform coverage prediction.

Determine the ERP required for the transmitters to support the intended application.

Analyse the resulting predictions of accuracy, integrity, availability and continuity and determine their adequacy.

Consider additional or alternate transmitter configurations and iterate the process from point b as required.

Use coverage predictions to identify candidate locations for the establishment of new eLoran transmitters:

Ensure that proposed transmitter sites are able to support the size of antenna structure required to provide the desired ERP.

For the set of transmitters, select an operating GRI and a set of transmitter designators:

eLoran stations may in future operate only on a single GRI, thus reducing Cross-Rate Interference (CRI).

Establish the Emission Delays of each of the secondary transmitters.

Note that for one or two additional transmitters there may be space available in the GRI of an existing set of transmitters covering the region.

Consider the interoperability of neighbouring GRI to minimise the effect of CRI.

Install transmitters:

Establish contracts with site providers.

Procure transmitters and maintenance contracts.

Install transmitters.

Establish an eLoran Control/Monitoring Centre if required or make arrangements for transmitters to be integrated into an existing regional control/monitoring centre.

Establish Applications - Voyage Phase Requirements:

Identify the Voyage Phase required for specific regions around the service provider’s territory; Port Approach Voyage Phase or Coastal Voyage Phase.

For Port Approach Voyage Phase (requiring ~10 m accuracy):

Identify Ports to be served.

Identify locations for the installation of differential Loran reference stations.

Plan an ASF measurement survey based on the extent of the port’s VTS (Vessel Traffic Service) coverage area, or vessel confluence zone.

Establish the DLoran reference station(s) and incorporate the generated differential corrections into the LDC.

Perform ASF measurement survey ensuring all transmitters are on air.

Process the ASF data.

Perform validation of the ASF data, by assessing the performance of DLoran for each port approach.

Publish data to receiver manufacturers.

For Coastal Voyage Phase (requiring not better than 10 m accuracy):

Identify coverage region.

Produce ASF data using modelling software.

Establish a programme of automatic eLoran shipboard monitoring using service providers’ own vessels, or vessels of convenience owned by other ship operators that operate within the coverage area.

Gather sufficient sparsely measured ASF data with which to calibrate the modelled ASF data.

Perform validation of the calibrated modelled data using further data measured by the shipboard eLoran monitoring receivers.

Publish the ASF data for the Coastal Voyage Phase region to receiver manufacturers.

The following sections discuss the above in further detail.

## System Coverage Prediction and Analysis

The first step in identifying a system configuration is to perform computer software based coverage modelling.  Service providers are recommended to obtain or develop software for this purpose, or obtain consultancy from experts in this area.  Coverage predictions should allow the estimation of the geographical area over which the eLoran service will meet the application requirements outlined in Table 1 and Table 2.  The provider should start with estimates for the number and locations of required transmitters.  As a starting point, certain heuristics can be applied; for example, assuming a 250 kW ERP transmitter has an approximate range of 1000km, and that at least 3 transmitters are required to allow positioning, with 5 required to be received to provide full RAIM-type integrity within a user’s receiver.  Coverage prediction should be made for accuracy, integrity, availability and continuity.

## Identify Locations for Transmitters

The analysis of coverage estimates will allow the provider to estimate the ERP required for each transmitter.  Knowing the required ERP will allow the provider to determine the type and size of antenna needed to be installed at a proposed transmitter site, and that will determine the location and size of the site itself.  The ERP of the transmitter may be constrained by the size of site available, in which case further coverage analysis will be required in order to confirm service performance resulting from the use of the proposed site.

## Select Group Repetition Interval(s)

One or more GRI will need to be assigned to the transmitters.  Techniques are available to achieve this, which may be computerised.  Emission Delays should also be selected for each of the transmitters at this time.

Where the provider is adding one or more new transmitters to an existing set of transmitters it is appropriate to consider assigning the new transmitters to the already existing GRI(s) by an appropriate choice of Emission Delay(s); this will avoid creating any additional CRI.

## Install Transmitters

It is recommended that intended eLoran service providers liaise with their nation’s radio service providers who may have a capability to provide a location and assist in the installation of the eLoran transmitter. eLoran service provision typically involves international cooperation and coordination to run the system, for example as with NELS and FERNS.  In these scenarios there will invariably be a requirement for a Control Centre, whose day to day duties include monitoring the eLoran signal and transmitter performance, coordinating maintenance activities and providing notices to users of authorised and unauthorised transmitter off-air events.  New eLoran transmitters that are to be integrated to an existing set will need to also be integrated into Control Centre operations.  The Control Centre should be tasked with maintaining synchronisation of the eLoran transmissions to UTC, and controlling the access to the LDC of customers requiring access to the data channel without navigation services.

To transmit an eLoran signal, a mast of 200m is recommended, with 24 top loading elements and an appropriate earth mat that may extend up to a 300 m radius around the mast.  The transmitter output power shall be in the region of 200kW-1MW.  For the Loran Data Channel, and control and monitoring the local infrastructure must provide reliable power and broadband Internet connectivity with the appropriate security. It may be a requirement to provide separate Internet access for each of the LDC and monitoring and control. In either case redundant data communications systems should be provided, with spatial diversity of the redundant data links given a high priority.

For redundancy it is normal practice to duplicate the transmitter, timing and control units and power supply.  The use of Uninterrupted Power Supplies to prevent outages in the event of short term loss of power supplies is strongly recommended.

The performance of the LF transmitter and its antenna can be affected by weather conditions and an automatic Antenna Tuning Unit (ATU) should be used to minimise such effects.  Lightning protection techniques are highly recommended.

Service providers are recommended to contact eLoran transmitter vendors for more information and installation options.

Information on the format of eLoran transmitter almanac data may be obtained from RTCM SC-127 [11]. Transmitter almanac information should be published in IHO S-200 series format; see section 3.4.

## Establish Applications - Voyage Phase Requirements

The service provider should identify performance requirements around the provider’s coastal territory.  Providing ports and harbours with Port Approach Voyage phase capability will require the installation of one or more differential-Loran Reference Stations and ASF surveys.  Coastal Voyage phase will require the production of ASF data through modelling and calibration with measured ASF data.

### Port Approach Voyage Phase

A DLoran Reference Station will be required to be installed for each port or harbour to be provided with 10 m (95%) positioning accuracy capability.  The typical geographical range of validity of the differential-corrections computed by a DLoran reference station is limited by an effect called ‘spatial decorrelation’, the effect of which is dependent on the difference in propagation paths between the eLoran transmitter and the user, and the eLoran transmitter and the reference station.  Numerous DLoran reference stations will be required in order to cover a large region.

#### Reference Station Installation

A suitable building will require to be found to house the reference station, with power, Internet connection, roof space for the eLoran antenna and access available for maintenance activities.  Legal and financial compensation agreements will need to be made with port authorities and licences or leases may be required.

When surveying the likely location of the reference station, it is recommended that the service provider perform a radio frequency ‘noise survey’ of candidate buildings to ensure the quality of reception of the eLoran signals at the location.  Certain electrical equipment can affect the signal-to-noise ratio of the eLoran signals depending on proximity. For example, power supplies to LED lighting, elevator motor equipment, air-conditioning plant, switch mode power supplies of radar, radio and satellite receiving equipment and computer monitor screens can all have a detrimental effect on eLoran signal quality if such equipment is too close (1 metre or so) to an eLoran receiver antenna.  For each port, the DLoran reference station should be installed and put into operation before the ASF measurement survey of the port is performed, as the reference station will need to be operating in order to take account of any signal propagation changes that might occur during the ASF survey.

The reference station should be provided with a unique identification number for automatic use by receivers.  Information on the format of DLoran Reference Station almanac data may be obtained from RTCM SC-127 documentation.  DLoran Reference Station almanac information should also be published in IHO S-200 series format; see Section 3.4.

#### Planning ASF measurement Surveys

An ASF database will be required for each port or harbour approach.  Since the area covered by a port approach is limited, it is appropriate to obtain the required data by measurement rather than modelling.  The resulting data will be the highest quality.

Using processing techniques and knowledge about the physics of eLoran signal propagation and ASF build-up over land it is not necessary to perform grid type surveying, as used to establish bathymetry.  Rather ASF measurements are made around the edges of the area to be covered and the data is then post-processed using interpolation and extrapolation to cover those areas not directly measured.

#### Perform the ASF Survey

The service provider should ensure that the local DLoran Reference Station is operating correctly before and during the survey.  In addition, the provider should ensure that all eLoran transmitters intended to be used for position fixing in the area by users, are operating correctly and that there is no transmitter maintenance planned during the survey that will either take the transmitter off-air, or affect the performance of the transmitted signal.

Vessels should traverse around the edges of the VTS service area and along heavily-trafficked approach channels.  Depending on the extent of the VTS area, it may be appropriate to traverse other lines within the area.  The aim is to gather as much high quality data in the most efficient manner possible in order to save ship time, personnel time and fuel. It is recommended that, for Port Approach ASF, data is gathered and monitored in real-time with an operator sat at the measurement equipment.  The software should be able to provide information on the quality and amount of data gathered, and decision can then be made whether to re-survey areas that are of poorer quality than others.  ASF measurement equipment may be purchased from an appropriate vendor.

#### ASF Data Processing

Once the survey has been completed the data will need to be processed to interpolate and extrapolate data to those regions where data was not measured.  The data should be output in a grid format, most conveniently a rectangular grid of individual ASF cells for each transmitter intended to be used for position fixing in the area.  The ASF data cells should be spaced no more than 500 m apart. Interpolation and filtering techniques are available that take into account the physics of ASF as it increases along the signal propagation path as the eLoran signal propagates over land and recovers slightly (and eventually flattens out) over sea water.

At the same time as producing the ASF data grids an equivalently sized grid of ASF measurement errors, one error value per ASF grid cell, should be produced.  This should be published alongside the ASF data for use within a user’s receiver.  The ASF error data will be used in RAIM type integrity equations as one component of a Horizontal Protection Level (HPL) computation.

#### ASF Data Validation

Once the final output ASF data has been produced it is recommended that the service provider validate eLoran positioning in the area of interest.  This validation should employ the ASF map and the local DLoran Reference Station.

#### ASF Data Publication

The ASF (and ASF error) data grids should be converted into RTCM SC-127 format ready for use by receiver manufacturers.  It should also be handed to IALA for conversion to IHO S-245 format; see section 3.4.

### Coastal Voyage Phase

It is not anticipated that DLoran will be available for the Coastal Voyage Phase.  This phase of navigation is assumed to take place outside of harbour approach areas at greater distance from a DLoran reference station where the differential-corrections are not valid.  The Coastal Voyage phase does not require as high accuracy positioning as the Port Approach Phase (see Table 2).  eLoran using ASFs alone has been demonstrated to be capable of providing 30 m to 50 m positioning accuracy at the 95% probability level.

#### Identify the Coverage Region

The service provider should identify the coverage region within which Coastal Voyage Phase performance from eLoran is required.  The service provider may need to consult coverage maps in order to verify that coverage within this region is feasible.

#### Produce ASF Data Using Modelling Software

The coastal voyage phase by definition covers a much greater geographical area than the harbour approach. ASF data is most efficiently produced through software modelling and subsequent calibration using a much smaller set of measured data.  Sophisticated ASF modelling software is available, which accounts for the electrical conductivity of the earth’s surface, vector coastline data and digital terrain elevation data.

#### eLoran Shipboard Monitoring

Measured ASF data may be gathered by ‘vessels of convenience’, or crowd sourcing.  Once vessels have been fitted with multi-source receivers containing one or more GNSS and an eLoran receiver then this can be a source of ongoing measurements against which to calibrate and monitor the performance of the service provider’s ASF data.  The shipboard monitoring equipment may also serve to provide the service provider with validation data on the performance of its eLoran service.

#### ASF Data Processing and Calibration

Measured ASF data, or the data that can be used to derive measured ASF, should be collected from the service provider’s shipboard eLoran monitoring vessels regularly, and the modelled data re-calibrated with it and republished.  In this way the service provider ensures that the mariner is provided with the latest data.  The re-publication of data may be achieved using an e-Navigation service providing data through the IHO S-245 format.  This could be an automated process in order to ensure timeliness of updates.

#### ASF Data Validation

Once the final output ASF data has been produced it is recommended that the service provider validate eLoran positioning in the area of interest.  This validation should employ the calibrated ASF map and ‘vessels of convenience’, or crowd sourcing of an agreed set of validating users.

#### ASF Data Publication

The ASF (and ASF error) data grids should be converted into RTCM SC-127 format ready for use by receiver manufacturers. It should also be handed to IALA for conversion to IHO S-245 format; see section 3.4.

# OPERATIONAL ASPECTS

Aspects of service providers’ operations are presented here, with some detail found in [13].

## Concept of Operations

eLoran stations are unmanned and fully automated;

each station contains an Uninterruptible Power Supply (UPS) to minimise outages due to power failure.

An eLoran service provider should:

continuously monitor the service and manage any disruptions;

inform users of important properties of the service and communicate warnings about service disruptions to the user;

manage any maintenance work or changes to the service in such a way where service disruption is minimized and the users are provided with advance warning;

verify the service is performing according to specifications and provide such information to users.

## Reference Datum

There is a need to ensure the interoperability of ASF data measured within different service providers’ territories.  The eLoran datum employed in a region will ultimately be derived from the position reference used to survey ASF data (related to the ground-truth system employed, for example WGS84 for DGNSS).  The datum used should be stated in each of the service provider’s publications, however, it is recommended that WGS84 should be employed directly, or ASF and almanac data converted to WGS84 prior to publication.

The use of the incorrect datum could result in positioning errors.

## Monitoring

On-site monitors should be provided at each transmitting station, to check the transmitted signal and the data content.

Additional signal monitoring is recommended using receivers placed at sites within the coverage area, to validate broadcast site RF and signal performance.  Communication lines to a central control and monitoring centre may link integrity monitors.  Data may either be logged at the integrity or reference station and downloaded periodically or passed directly to the central control and monitoring centre.  It is recommended that this data be archived for a period sufficient to meet local litigation requirements.

Information on each DLoran reference station or integrity monitor shall be provided to a control and monitoring centre.  The information will include:

identity (ID) of the Reference Station;

location of the Reference Station;

nominal ASF Values at the Reference Station;

ECD values measured by the reference station;

applicable maritime areas monitored by the reference station;

acceptable eLoran transmitters tracked by the reference station and recommended for use in positioning in the area;

the format is repeated for each reference station and eLoran transmitter combination in each operational area;

integrity alert and type of alert as deemed necessary.

## Publication of information

Individual Service Providers are encouraged to publish service descriptions, including coverage predictions and system performance statistics.

In addition to the information contained in the standard LDC message types notice of current or planned signal unavailability should be provided to users through the appropriate service (e.g. coastal radio station, VTS, Navtex, Safetynet etc.).

Wherever practicable, information on scheduled and unscheduled off-air periods should be promulgated to users as shown in Table 5:

Information on maintenance and outages to be promulgated to users

| Event Type | Date and Time Period | Provision of Notice to Users |
| --- | --- | --- |
| Scheduled maintenance | Date and expected downtime | One month in advance is recommended, but at least 1 week in advance is mandatory |
| Unscheduled outages | Expected downtime | As soon as practicable and not more than 1 hour after the occurrence |

IALA Guideline 1016 on Bilateral Agreements and Inter-Agency MoU on the Provision of DGNSS services in the frequency band 283.5 kHz– 325 kHz contains examples of guidance on the information that should be exchanged between co-operating agencies and, where appropriate, the circumstances and timing for it to be exchanged. Those guidelines can be extended to eLoran by analogy.

IALA will maintain a master list of eLoran transmitters and reference stations on the Internet. Input to the master list will be prepared by each Service Provider.  IALA will provide an electronic template that should be used for initial entry of station data and assembly of the complete Service Provider submission.  After initial submission, the Service Provider will be responsible for updating their own sections of the IALA master list.  The process to incorporate changes will require each Service Provider to provide a complete, updated section of the list for all the eLoran sites that they operate.  Each complete submission will appear exactly as submitted.

The service provider is recommended to publish that they follow IMO Resolutions and IALA Recommendations for the provision of eLoran, giving emphasis to the provision of integrity information.

The service provider is required to tell the user at which ports, or within which areas, they can expect to receive eLoran signals and where they can expect to meet the harbour requirements.

eLoran transmitter and almanac information shall also be made available in IHO S-200 format, which will then afford the data’s provision by e-Navigation service.  The current S-200 series designations for eLoran are:

S-245: ASF (Additional Secondary Factor) data;

S-246: eLoran transmitter almanac data;

S-247: Differential-Loran Reference Station almanac data.

## Performance Verification

It is recommended that the service provider measures the performance components continuously in order both to detect service disruptions and to determine if the performance requirements are being met over an extended period of time.

Performance should take account of the declared eLoran service area. In order to maximize the combined performance service providers should:

co-ordinate the scheduling of maintenance work so that the effect on coverage is reduced;

exchange information about service disruptions;

exchange information about achieved performance.

### Coverage Verification

Published coverage diagrams are normally based on software modelling predictions and should be verified by measurements.  Measurements can be gathered automatically using ‘vessels of convenience’, which may be the service providers own vessels or vessels belonging to other regional partners.

Coverage modelling should include predictions for accuracy, integrity, availability and continuity.

The modelling process can be quite complex and difficult, especially over mixed land/sea paths.  Advice regarding modelling can be sought through IALA, and background technical information is available as described in Section 2; providing guidance and references in order to accomplish a capability in coverage prediction.

### Availability Verification

The availability standard adopted for an eLoran service is related to the techniques used in planning and implementing the service.

Availability is defined in IMO Resolution A.915 (22) [15] as:

‘The percentage of time that an aid, or system of aids, is performing a required function under stated conditions. The non-availability can be caused by scheduled and/or unscheduled interruptions.’

Availability should include the ability to provide positioning accuracy with ‘green-light’ integrity; in order for the system to be available, the system’s integrity monitor (IM) must declare that the system is usable.

For an ‘all-in-view’ system, calculation of system availability is extremely complex, will vary with the number of stations available to a receiver and their effective ranges, and hence differ between specific geographic areas.  The effective range of a transmitter will be influenced by SNR, which is a very local phenomenon.

For the service provider, a much simpler approach is to calculate the availability that each transmitter station is on-air and in tolerance.

Signal availability is defined as the availability of a radio signal in a specified coverage area.  Mathematically this can be written as:

Availability A*=MTBOMTBO+MTSR

Where:

MTBO is the mean time between outages; based on a 30-day averaging period

MTSR is the mean time to service restoration; based on a 30-day averaging period

This accounts for scheduled and unscheduled service interruptions, i.e. preventative and corrective maintenance.

- * Alternatively expressed as UP TIME/TOTAL TIME where TOTAL TIME = 30 days

### Continuity Verification

Inherent in a radionavigation service is the capability to provide accurate position fixing and integrity information without interruption during a specified period (normally short term).  Interruptions to eLoran deny vital information to the users and, if frequent, erode user confidence in the ability of the service to provide that information.  The frequency of unusable events, not the length of the usable periods, determines continuity performance.

In the event that a healthy and monitored eLoran transmitter or reference station begins to experience intermittent failures (i.e. failures separated in time by a period less than one continuity time interval (CTI)), the period of intermittent operation would be counted as a single failure event for continuity purposes.

Continuity in the coverage area can be measured (1) at the broadcast site, (2) by the use of a far field monitor, or (3) by using a combination of these methods.  Continuity is based upon the mean time between failures as measured over a two-year period and a 15 minute CTI.

Continuity Failures: All unscheduled non-momentary unusable events described in the availability section are considered failures.  Unlike availability, continuity does not count scheduled maintenance events as failures.  Since public notices are provided for all scheduled maintenance events, users should be aware of such planned outages and plan the voyage accordingly.

The equation below refers to an availability type calculation, i.e. the average over the CTI that the service is available or the probability that the service is available at any instant during the CTI.  It is more appropriate to calculate the probability that the service is available throughout the CTI.

Assuming the service is functioning at the beginning of the operation, then the probability that it is still functioning at a time (t) later is:

C = exp (-t/MTBF)

This is the standard expression for reliability and excludes scheduled outages (i.e. uses MTBF) assuming that planned outages will be notified and the operation will not take place.

The probability that the service will be available after a time CTI, that is (the continuity) is then:

C = exp (-CTI/MTBF)

If MTBF is very much greater than CTI, this can be approximated to:

C = 1 – (CTI/MTBF)

Where:

MTBF is the mean time between failures; based on a 2 year averaging period, and

CTI is the Continuity Time Interval; in the case of maritime continuity equal to 15 minutes

### Integrity Verification

There are two forms of integrity that the service provider should be aware of:

- Infrastructure (External) based integrity monitoring.

User Receiver (Internal) based integrity monitoring.

#### Infrastructure Based Integrity Monitoring

System status information will be vital to maintaining knowledge of integrity and positioning accuracy. eLoran integrity alerts shall be generated by an infrastructure (external) based integrity monitoring system, mostly to mitigate and alert the user to rare ‘early skywave’ events [13].  Early skywave is an effect whereby a copy of the ‘wanted’ groundwave signal pulse arrives by skywave reflection when the ionosphere is particularly low in altitude owing to strong solar activity.  The magnitude of the effect depends on the distance between a user and the respective eLoran transmitter, and the user’s geomagnetic latitude. Where necessary ‘early skywave’ monitors should be installed at suitable locations [17] [18].

On the rare occasions when strong sky wave interference is likely, or there is likely to be a transmitter or differential correction data outage, users should be notified within a particular time-to-alarm period (typically 10 seconds according to IMO requirements).

eLoran integrity information is disseminated by the Loran Data Channel (LDC).  Under certain conditions of transmission restoration from outage, eLoran transmitters themselves may enter ‘blink mode’ [11], and the LDC has messages defined for transmitter and DLoran reference station health messages.

The IMO [3] define Time to Alarm as:

‘The time elapsed between the occurrence of a failure in the system and its presentation on the bridge.’

The budget for Time-To-Alarm is typically made up of the following latencies and delays:

- Detection of fault at integrity monitor site.

Transmission of alarm via Internet VPN to transmitter for broadcast over LDC.

Message scheduling delay at eLoran transmitter, e.g. 2 seconds for Anthorn on 6731Y.

LDC message transmission time at LDC bit rates.

LDC message reception and decode time at the user’s receiver.

Time-to-alarms are difficult to verify by the service provider since they do not have direct access to users’ receivers.  However, DLoran reference stations and ‘early skywave’ monitors should be able to log historic data, and monitoring and control centres should be able to log historic alarms.  Integrity verification for the DLoran service may be accomplished by post-processing and analysis of the logged data identifying those alarms and conditions under which the TTA was not met.  This would provide the service provider with the opportunity to improve the system should it be found to be operating incorrectly or inefficiently.

#### User Receiver Based Integrity Monitoring

In a multi-source receiver containing both eLoran and GNSS based receivers each should independently verify their own Integrity at the levels set down by the IMO for electronic position-fixing at sea [1] [3]. Receiver integrity algorithms are available for eLoran [2].

Tight multi-system integration at the level of pseudorange measurements has several benefits, including greater accuracy; improved availability of solution; the ability to calibrate cross-system biases; and increased coverage or better HDOP by combining signals from several systems.  However, for safety-of-life navigation where a high level of integrity is required any multi-system integration or calibration process risks introducing faults from one system into another.  Likewise, the use of Kalman Filtering or any other type of multi-epoch smoothing propagates Integrity Risk from one epoch to the next – both techniques risk ‘poisoning the well’ if severe Integrity hazards such as GNSS Jamming present themselves and go undetected.

As a general rule, for high-Integrity applications cross-system calibration and cascaded filtering should be avoided whenever possible [16] to maintain the level of system-separation needed to ‘quarantine’ the Integrity Risk.

#### Horizontal Alert Limit (HAL) and Horizontal Protection Level (HPL)

All errors contributing to the system error are expressed as uncertainties.  These uncertainties are summed in quadrature and transformed into the positioning domain.  The resulting two-dimensional position error may be represented as an error ellipse.  The error ellipse is linearly scaled to a positioning probability level corresponding to the desired integrity risk level; for example 10-5 integrity risk, which indicates that there is a 1 in 100000 chance that a fault occurs and it is not detected by the system.

Traditionally the Horizontal Protection Level (HPL) is a computation performed by system designers to ensure the proper functioning of the system, but the same computation may be made available aboard ship in order to offer the mariner some reassurance that his navigation system is performing within the requirements of particular applications.

For integrity purposes it is recommended that receiver manufacturers compute a HPL, for any navigation system, given knowledge and assumptions of the uncertainties in the components of the positioning solution.  Guidance is available on how to do [2].

Associated with each set of requirements and voyage phase is a Horizontal Alert Limit (HAL) (see Table 2), which is the limit (in metres) of positioning error tolerable by the given application.  Should the computed HPL breach the HAL an alarm should be generated within the receiver and a Multi-Source Receiver can then switch sources of PNT to one that is demonstrating an HPL that is lower than the HAL.  An example can be seen in Table 2 where for the Coastal voyage phase the Alert Limit is 250 m.

The magnitude of the HPL is dependent on the predicted error bounds to positioning accuracy; and is built up from an error budget that is dependent on the PNT system currently in use by the vessel. As such, the HPL computation takes into account such parameters as noise on the pseudorange measurements, geometry to satellites or transmitters (Horizontal Dilution Of Precision), errors in augmentation data (ASF, differential corrections etc.), and the current integrity risk of the current application; for example, for (Table 2) Port Approach requires 10-4 integrity risk over 3 hours. (Table 2).

### Accuracy Verification

Where differential eLoran corrections are provided e.g. in Port and Port Approach areas, the absolute horizontal accuracy should be better than 10 m at the 95% probability level within the published coverage area; for Coastal Voyage phase areas the accuracy should be better than 100 m at the 95% probability level.  Future eLoran receivers will form one component of a multi-source receiver unit. GNSS will also be provided within the same unit.  Under normal operations this allows the possibility to measure the performance of eLoran against one or more GNSS (including DGNSS).

# ACRONYMS

ASF	Additional secondary factor

ATU	Antenna Tuning Unit

CD	Coding Delay

CRI	Cross-Rate Interference

CTI	Continuity Time Interval

CWI	Continuous Wave Interfering

DGNSS	Differential Global Navigation Satellite System

DLoran	Differential Loran

ECD	3.3

ED	Emission Delay

ERP	Effective Radiated Power

FERNS	Far East Radio Navigation Service

FFT	Fast Fourier Transform

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

GRI	Group Repetition Interval

HAL	Horizontal Alert Limit

HDOP	Horizontal Dilution of Precision

HPL	Horizontal Protection Level

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities

ID	Identity

IEC	International Electrotechnical Commission

IHO	International Hydrographic Organization

ILA	International Loran Association

IMO	International Maritime Organisation

ITU-R	International Telecommunication Union-Radiocommunication Sector

kHz	kilohertz

km	kilometre

kW	kilowatt

LDC	Loran Data Channel

LED	Light-Emitting Diode

LORAN	LOng Range NAVigation

LORRAP	Table 3

LORRIP	Table 3

m	metre

MoU	Memorandum of Understanding

MTBF	Mean time between failures

MTBO	Mean time between outages

MTSR	Mean time to service restoration

MW	megawatt

N/A	Not applicable

NELS	Northwest European Loran System

PC	Personal Computer

PC	Phase Code

PCI	Phase Code Interval

PNT	Position, Navigation & Timing

PPM	Pulse Position Modulation

RAIM	Receiver Autonomous Integrity Monitoring

rho-rho	Table 4 (Does this refer to the Rho-Rho Navigation System?)

RTCM	Radio Technical Commission for Maritime Services

SAM	Service Area Monitor

SC	Special Committee (RTCM)

TD	Time Difference(s)

TOA	Time of arrival

TOE	Time of emission

TOT	Time of transmission

UPS	Uninterruptible Power Supply

UTC	Universal Time Co-ordinated

UT1	Mean solar time

VPN	Virtual Private Network

VTS	Vessel Traffic Service(s)

WGS84	World Geodetic System 1984 (Reference co-ordinate system used by GPS)

WWRNS	World-Wide Radio Navigation Systems

µs	microsecond

# REFERENCES

IMO Resolution A.1046 (27) – ‘Worldwide Radionavigation System’

‘Minimum Performance Standards for Marine eLoran Receiving Equipment’, V2.07, RTCM SC-127, June 2015

IMO Resolution A-915 (22) ‘Maritime Policy for a Future Global Navigation Satellite System (GNSS)’, Publications Section, International Maritime Organization

‘IALA Recommendation R-129 ‘On GNSS Vulnerability and Mitigation Measures. Edition 2 December 2008

‘Enhanced Loran (eLoran) LORIPP/LORAPP Draft Specification of the eLoran System’, Rev. 4.0.

ITU-R Recommendation M.589-2, ‘Technical characteristics of methods of data transmission and interference protection for radionavigation services in the frequency bands between 70 and 130 kHz’.

‘Enhanced Loran for Maritime Harbor Entrance and Approach’, Benjamin Peterson et al, Institute of Navigation National Technical Meeting 2004.

‘ASF Surveying and Quality Assurance’, Chris Hargreaves and Paul Williams, ION PLANS, April 2012.

‘Enhanced Loran for Maritime Harbor Entrance and Approach’, Benjamin Peterson and Kenneth Dykstra, Peterson Integrated Geopositioning, LLC, Peter F. Swaszek, University of Rhode Island, John Macaluso, Kevin M. Carroll, and Anthony Hawes, U. S. Coast Guard Loran Support Unit, Gordon K. Weeks Jr., U. S. Coast Guard Command and Control Center, Institute of Navigation National Technical Meeting 2004.

‘The GLA’ eLoran Initial Operational Capability’, Paul Williams and Chris Hargreaves, The General Lighthouse Authorities of the United Kingdom and Ireland, European Navigation Conference, Gdansk, Poland, April 2012.

Minimum Performance Standards for Marine Loran Receiving Equipment, Revision 2.7 DRAFT, June 23 2015, RTCM Special Committee 127.

IEC 61108-1 – ‘Maritime navigation and radiocommunication equipment and systems –Global navigation satellite systems (GNSS)’

‘Loran's Capability to Mitigate the Impact of a GPS Outage on GPS Position, Navigation, and Time Applications’, Prepared for the Federal Aviation Administration, Vice President for Technical Operations, Navigation Services Directorate; dated 31 March 2004

‘IALA List of radionavigation services - Loran-C/Chayka’ Collated and published by the International Association of Lighthouse Authorities, First Edition - 1st July 1996

[IMO Resolution A.915(22):2002, ‘Revised Maritime Policy And Requirements For A Future Global Navigation Satellite System (GNSS)’

‘Principles of GNSS, Inertial, and Multisensor Integrated Navigation Systems’, Paul D. Groves, Artech House, 2008.

‘Conditions Leading to Anomalously Early Skywave’, Peter B. Morris, Northrop Grumman Information Technology, Presented to The International Loran Association 32nd Annual Convention and Technical Symposium Boulder, Colorado, 03 – 07 November 2003.

‘Analysis and Modeling of Skywave Behavior’, Sherman Lo, Stanford University, Robert Wenzel, Booz Allen Hamilton, Per Enge, Stanford University, International Loran Association, 2009.

It should be noted that since publication of this reference, Continuity is calculated over 15 minutes and not 3 hours.

All generations of Loran support Stratum 1 frequency for telecommunications.  Prototype eLoran and eLoran support UTC time of day.  eLoran will support sub-50ns precise timing.