Reference: ENAV19-14.1.6

# Purpose

This document should be used to submit proposals for additions, amendments or deletions of terms in the International Dictionary of Marine Aids to Navigation (Dictionary). Proposals will be considered by the Dictionary Working Group (DWG) of the IALA Policy Advisory Panel (PAP) and the Dictionary will be amended if the proposal(s) is approved.

# Proposed amendment of the Dictionary

Please use the following table to submit proposals for additions, amendments or deletions of terms in the Dictionary.

| Term | Dictionary Number | Source (meeting/ document/ person) | Old definition | Proposed definition English | Proposed definition French | Proposed definition Spanish | Reason for change | Date | Accept Reject |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service Oriented Architecture (SOA) | - | IALA Rec e-NAV140 (Ed. 2), Recommends No. 1 | - | A SOA is a design pattern in which application components provide services to other components via a communications protocol typically over a network. |  |  | New term | May 2015 | IALA Council approved |
| Encapsulation | - | IALA Guideline 1113, para 2.2, page 6 | - | Encapsulation hides the technology’s sophistication of the system as a whole and thus reduces complexity, i.e. the details of the system architectures, namely their respective technologies and functions, are not shown. |  |  | New term | May 2015 | IALA Council approved |
| Functional link | - | IALA Guideline 1113, para 2.5 | - | The functional link is the functional connection for data exchange between the shore-based system and the shipboard equipment and thus is the abstract statement in regard to requirement analysis for the data exchange of the application. |  |  | New term | May 2015 | IALA Council approved |
| Physical link | - | IALA Guideline 1113, para 2.5 | - | The physical link is the part of the physical path for data exchange between the shore-based technical system and the shipboard equipment, and employs one or more appropriate mediums such as radio waves or light. |  |  | New term | May 2015 | IALA Council approved |
| Common Maritime Data Structure (CMDS) | - | IALA Guideline 1113, para 2.7 | - | The Common Maritime Data Structure (CMDS) is an abstract data representation of those parts of the maritime domain defined in it at any given time. Specifically, it represents the entities and relationships among the entities that exist in the maritime domain as meta-level data descriptions but does not represent processes. |  |  | New term | May 2015 | IALA Council approved |
| Traceability | - | IALA Guideline 1113, para 3.1 | - | Traceability allows to link user or management requirements with system engineering requirements. |  |  | New term | May 2015 | IALA Council approved |
| Layering | - | IALA Guideline 1113, para 3.2 | - | Layering is a structuring methodology, which permits the relevant aspects of the desired information/data, services, and systems to be viewed as logically composed of a hierarchy of layers, each wrapping the lower layers and separating them from the higher layers. |  |  | New term | May 2015 | IALA Council approved |
| Common Shore-based System (CSS) | - | IALA Guideline 1113, chapter 4, intro, and IALA Guideline 1114, Chapter 1 | - | Common Shore-based System (CSS) is a generic name for the shore-based technical system, which is designed and built in accordance with the principles of the Common Shore-based System Architecture (CSSA). ‘Common’ here is used in the meaning of several shore authorities employing a similar system architecture when designing and implementing their shore-based infrastructure; ‘common’ is not meant to mean that they are sharing the very same shore-based infrastructure. |  |  | New term | May 2015 | IALA Council approved |
| Common Shore-based System Architecture (CSSA) | - | IALA Guideline 1113, chapter 4, intro, in conjunction with para 2.2 | - | Common Shore-based System Architecture (CSSA) is a generic name for the architecture of a shore-based infrastructure, which achieves minimum common service levels for the shore-provided services and a smooth transition for the shipboard electronic environment when passing through adjacent areas of provision of the same technical service provided by different shore-based systems. To that end, the CSSA achieves and exploits commonalities of the data objects and their encoding formats exchanged between the shore-based technical systems interacting with the electronic environments of transiting vessels. By ‘common’ it is not implied to say ‘shared’ which is the other major meaning of the word. |  |  | New term | May 2015 | IALA Council approved |
| Data Collection and Data Transfer Services (DCT) |  | IALA Guideline 1114, chapter 2 |  | In the context of the Common Shore-based System Architecture (CSSA), the Data Collection and Data Transfer Services (DCT) are a group of technical services interfacing the shore-based system via the physical links to traffic objects’ electronic systems, to the waterways and to the natural environment. |  |  | New term | May 2015 | IALA Council approved |
| Value Added Data Processing Services (VAD) |  | IALA Guideline 1114, chapter 2 | - | In the context of the Common Shore-based System Architecture (CSSA), the Value Added Data Processing Services (VAD) are a group of individual technical services. Their main task is to add value to (raw) data by processing, combination, comparison etc., store data and information and provide it upon request to other technical services. |  |  | New term | May 2015 | IALA Council approved |
| User Interaction Service (UIA) |  | IALA Guideline 1114, chapter 2 |  | In the context of the Common Shore-based System Architecture (CSSA), the User Interaction Service (UIA) is specialised to provide the Human-Machine-Interface (HMI) to the primary users of the Common Shore-based System (CSS), i.e. such users as are supported directly by the system via dedicated displays, keyboards and other human interaction devices. |  |  | New term | May 2015 | IALA Council approved |
| Gateway Service (GWY) |  | IALA Guideline 1114, chapter 2 |  | In the context of the Common Shore-based System Architecture (CSSA), the Gateway Service (GWY) specialises in data exchange shore-to-shore. It interfaces mainly to external systems of third parties, which provide data upon request by own system and/or receive access to relevant data from own system, upon legitimate request. The Gateway Service can also interface different shore-based systems locally, regionally, and globally. |  |  | New term | May 2015 | IALA Council approved |
| AIS Service (AIS) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the AIS Service (AIS) is the shore-based infrastructure of a shore-based authority, which allows reception of messages of the Automatic Identification System (AIS) from traffic objects and transmission of messages to them. In addition, certain features of the AIS Service allow the Common Shore-based System (CSS) of a competent authority to perform certain management functions for the Automatic Identification System intended only for competent authorities. |  |  | New term | May 2015 | IALA Council approved |
| Radar Services (RAD) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Radar Service (RAD) delivers to the Common Shore-based System (CSS) all radar-derived data, i.e. radar images, radar plots, and radar tracks, each of which are provided with radar-specific relevant quality parameters. |  |  | New term | May 2015 | IALA Council approved |
| Direction Finding Service (DFS) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Direction Finding Service (DFS) determines the current position of a mobile VHF voice/data transmission device operating on one of the several frequency channels monitored. Thus, a DF track together with relevant quality parameters (e.g. accuracy estimation) is provided to the Common Shore-based System (CSS). |  |  | New term | May 2015 | IALA Council approved |
| DGNSS Augmentation Service (DGN) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the DGNSS Augmentation Service (DGN) delivers to the CSS all relevant measurements regarding the current ‘on-air’ status of selected components of World Wide Radio Navigation System (WWRNS), such as particular GNSS, terrestrial augmentation and terrestrial backup-systems of other authorities. Also, based on these run-time measurements, the DGNSS Augmentation Service provides own DGNSS augmentation data to the own CSS for transmission to traffic objects e.g. via the AIS Service or via the Medium Frequency Broadcast Service. |  |  | New term | May 2015 | IALA Council approved |
| Medium Frequency Broadcast Service (MFB) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Medium Frequency Broadcast Service (MFB) transmits RTCM 104 standard encoded data using medium frequency bands around 300 kHz (radiobeacon band). |  |  | New term | May 2015 | IALA Council approved |
| D-Mode |  | IALA Guideline 1114, chapter 3 |  | The D-Mode designates the transmission of DGNSS augmentation data by a shore-based technical service. |  |  | New term | May 2015 | IALA Council approved |
| R-Mode |  | IALA Guideline 1114, chapter 3 |  | The R-Mode designates the transmission by a shore-based technical service that include a precise timing signal that would allow appropriately equipped shipboard receivers to determine the range to the transmission sites for ranging which in turn would be a contribution to create a position fix based on the terrestrial Signals-of-Opportunities (SoOP) approach. |  |  | New term | May 2015 | IALA Council approved |
| T-Mode |  | IALA Guideline 1114, chapter 3 |  | The T-Mode designates the transparent transmission of relevant data other than DGNSS augmentation data, see D-Mode, and precise timing for ranging, see R-Mode, by a shore-based technical service. |  |  | New term | May 2015 | IALA Council approved |
| VHF Communication Service |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the VHF Communication Service (VHF) allows for shore-based bidirectional voice communication with any mobile radio station operating in the VHF maritime mobile service band. |  |  | New term | May 2015 | IALA Council approved |
| GMDSS VHF DSC Service (DSC) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the GMDSS VHF DSC Service (DSC) allows the CSS to participate in the digital GMDSS DSC-Channel 70 in the VHF maritime mobile service band. |  |  | New term | May 2015 | IALA Council approved |
| HF Communication Service (HFC) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the HF Communication Service (HFC) provides the shore-based bidirectional voice and data communication with any mobile station operating on the HF frequencies (short wave bands) assigned to maritime use. |  |  | New term | May 2015 | IALA Council approved |
| HF GMDSS Service (HFG) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the HF GMDSS Service (HFG) allows for the bi-directional communication in the HF channels dedicated to the GMDSS. |  |  | New term | May 2015 | IALA Council approved |
| Navtex Service (NTX) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the shore-based technical service transmitting NAVTEX data is the Navtex Service (NTX), i.e. it transmits navigation and meteorological warnings and forecasts as well as urgent safety information over MF radio to automated Navtex receivers/printers on-board ships. |  |  | New term | May 2015 | IALA Council approved |
| Aviation Communication Service (AVC) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), The Aviation Communication Service (AVC) allows for shore-based bidirectional voice and/or possibly data communication with any aircraft or helicopter, thus supporting operational applications such as Search and Rescue (SAR) and pollution response. |  |  | New term | May 2015 | IALA Council approved |
| Fixed Visual Aids Service (FXA) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Fixed Visual Aids Service (FXA) provides fixed visual Aids-toNavigation to the mariner. On site racons and / or on site AtoN AIS stations would conceptually be a part of the FXA. |  |  | New term | May 2015 | IALA Council approved |
| Floating Visual Aids Service (FLA) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Floating Visual Aids Service (FLA) provides floating visual Aids-toNavigation to the mariner. On site racons and / or on site AtoN AIS stations would conceptually be a part of the FLA. |  |  | New term | May 2015 | IALA Council approved |
| CCTV Video Service (VID) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the CCTV Video Service (VID) provides the CSS with the capability to visually observe the areas of waterways covered by video camera observation. |  |  | New term | May 2015 | IALA Council approved |
| Local Public Address Service (LPA) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Local Public Address Service (LPA) allows for acoustically addressing areas of a waterway, for acoustic voice communications and for the dissemination of other acoustic signals to mariners. Hence, the acoustic fog signals would belong to that service. |  |  | New term | May 2015 | IALA Council approved |
| Environmental Sensor Service (ENS) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Environmental Sensor Service (ENS) provides the Common Shore-based System (CSS) with measurements of relevant parameters of the maritime environment. Such relevant sensor data could be hydrological, meteorological or of other physical nature. |  |  | New term | May 2015 | IALA Council approved |
| Position Determination Service (POS) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Position Determination Service (POS) processes all real-time data regarding traffic objects (vessels, aircraft, floating objects etc.) from all available relevant sensory sources (e.g. AIS, radar, direction finding). Also, fusion algorithms dealing with the different sensory information may be processed here, thus providing the Common Shore-based System (CSS) with a multi-sensor validated track of every traffic object under surveillance by the CSS, together with relevant quality parameters such as accuracy estimations, tracking status and reliability etc. |  |  | New term | May 2015 | IALA Council approved |
| Ship Data Consistency Analysis Service (SDA) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Ship Data Consistency Analysis Service (SDA) collects, evaluates, stores and provides on demand non-real-time data regarding traffic objects, in particular their static and voyage related data. To that end this technical service draws on a variety of available sources of information for that kind of data, such as AIS reports, historical data from past voyages of the same traffic object, manually acquired data, and third party data bases. This service also provides pseudo tracking identifiers for the Position Determination Service (POS), for correlation of real-time tracking data with static and voyage related data of the same traffic objects. Amongst other things, this service may handle data as defined by IMO FAL (Convention). |  |  | New term | May 2015 | IALA Council approved |
| Environmental Data Evaluation Service (ENE) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Environmental Data Evaluation Service (ENE) collects, evaluates, stores and provides on demand all data of the physical environment of the waterways relevant to safety and efficiency of shipping and to the protection of the environment. The Environmental Data Evaluation Service also creates added-value information on the physical environment by evaluating the available raw data from the various sources by appropriate algorithms. For instance, tidal windows and under-keel clearances (UKC) may be calculated here. |  |  | New term | May 2015 | IALA Council approved |
| Vector Chart Service (VEC) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Vector Chart Service (VEC) collects, stores, and provides on demand all data pertaining to vectorized ENCs, both drawing on official sources for ENCs but also drawing on the system’s own sources, i.e. on manual input. The Vector Chart Service maintains the ENCs from external sources as they are, but provides overlay layers which contain the additional information from the system’s own sources, such as coverage areas of operational and technical services or route topology modelling data. |  |  | New term | May 2015 | IALA Council approved |
| Shipping Industry Database Service (SID) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Shipping Industry Database Service (SID) collects, stores, and provides on demand all data pertaining to the shipping industry, i.e. data provided by or learned about relevant stakeholders. |  |  | New term | May 2015 | IALA Council approved |
| Maritime Portfolio Registry Service (MPR) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Maritime Portfolio Registry Service (MPR) collects, stores and provides on demand all meta-level data pertaining to service portfolios in and for the maritime domain, in particular the Maritime Service Portfolios (MSPs), as defined by IMO/IALA. In the context of the ‘Maritime Cloud,’ a local copy of the Maritime Identity Registry (MIR), as appropriate for the Common Shore-based System (CSS), and parts of the MSP Almanac could be located here too. The Maritime Portfolio Registry Service would thus support the interaction of the CSS with the ‘Maritime Cloud.’ |  |  | New term | May 2015 | IALA Council approved |
| Maritime Messaging Service (MMS) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Maritime Messaging Service (MMS) is the technical service of the Common Shore-based System (CSS) specialized in composing messages encoded in technology- or link-specific formats for telecommunication services such as AIS Application Specific Messages (ASM),for Navtex, short message (for GSM), and ‘T-Mode’ in general, from high-level abstractly encoded data sentences received from requesting services. Also, the MMS could enable the CSS to interact with the Maritime Cloud. |  |  | New term | May 2015 | IALA Council approved |
| Data Mining Service (DMS) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Data Mining Service (DMS) allows for complex queries on present and historical data available from all other technical services of the system. It creates stores and provides reports to the requesting service. |  |  | New term | May 2015 | IALA Council approved |
| Shore-Based Wide Area Network Service (SBN) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Shore-Based Wide Area Network Service (SBN) is a utility service, which support any other requesting service of the CSS. It provides the feeder link and the backbone connectivity for the individual services of the Common Shore-based System (CSS). |  |  | New term | May 2015 | IALA Council approved |
| On-Site Infrastructure (INF) |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the On-Site Infrastructure (INF) provides resource building blocks needed to support the service components on their sites of installation, hence On-Site Infrastructure. Main topics include housing and other structures, traditional utility provision such as power, water, sewer and roads, precise timing, LAN, independent fault detection and alert management, HMIs for Technical Operation Personnel and Technical Development Personnel. |  |  | New term | May 2015 | IALA Council approved |
| Operational Presentation Surface |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Operational Presentation Surface (OPS) is the functional interface of the User Interaction Service as a HMI to its users. It is generally subdivided into individual functional windows, each with their own specified user interaction. |  |  | New term | May 2015 | IALA Council approved |
| Basic Service |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), each of the technical services except for the User Interaction Service provides a functional (M2M) interface to any other of the technical services of their own Common Shore-based System (CSS), i.e. on their ‘domestic’ side. This interface provides the respective functions of a technical service as a set of Basic Services specific to that technical service. |  |  | New term | May 2015 | IALA Council approved |
| Configuration Type |  | IALA Guideline 1114, chapter 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the concept of Configuration Type encapsulates the ability to customise presentation to different shore-based users of the Common Shore-based System (CSS). For instance, ‘traffic image display’ will be required by several operational users, although the configuration settings for each of the operational users may be different (e.g. tactical traffic display vs. strategic traffic display). |  |  | New term | May 2015 | IALA Council approved |
| Technical Operating Personnel |  | IALA Guideline 1114, chapters 2 and 3 |  | In the context of the Common Shore-based System Architecture (CSSA), the Technical Operating Personnel operates and maintains technical equipment, but has no licence to change structure and/or functionality. |  |  | New term | May 2015 | IALA Council approved |
| Technical Development Personnel |  | IALA Guideline 1114, chapters 2 and 3 |  | n the context of the Common Shore-based System Architecture (CSSA), the technical Development Personnel plans, deploys and further develops technical equipment and has licence to change structure and/or functionality. |  |  | New term | May 2015 | IALA Council approved |