Input paper:   ENAV19-5.3.1

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM			□  ENG		□  PAP				□  Input

□  ENAV		□  VTS						  Information

Agenda item 			5

Technical Domain / Task Number 2	COMMUNICATIONS

Author(s) / Submitter(s)			Capt. Jean-Charles CORNILLOU

# Report from rapporteur on GMDSS

# Summary

The present report provides the outcomes on the modernization of GMDSS after 12th IMO/ITU EG from 11th to 15th July 2016 hold at IMO headquarter and chaired by Christian Rissone (France).

## Purpose of the document

This document report the latest update on the modernization of GMDSS discussion at IMO.

## Related documents

NCSR 1/28 annex 10: outcomes of the high level review of GMDSS (adopted by MSC)

NCSR 1/28 annex 11: revised planned of work for the GMDSS review and modernization project (endorsed by MSC95)

NCSR 3/29 annex 7: outcome of the detailed review of GMDSS (adopted by MSC)

IMO/ITU EG 12/4/2: remarks on the review of GMDSS taking into account the outcome of NCSR 3 (enclosed)

# Background

# 2.1 	Report of the twelve meeting oF THE IMO/ITU Experts groups

The Correspondence Group on the Modernization of the GMDSS (CG), under the coordination of the USA, submit an interim report, containing the preliminary draft of the Modernization Plan of the GMDSS, to the Joint IMO/ITU Experts Group for its consideration. Taking into account the outcome of discussions at the 12th IMO/ITU Experts Group, the CG will submit a report, including the document containing a preliminary draft of the Modernization Plan of the GMDSS, for consideration at NCSR 4 in March 2017.

2.1.1	 Annexes to the Modernization Plan:

The Group agreed that the following should be annexed to the Modernization Plan:

.1 Plan of work for the revision and development of legal instruments, performance standards and guidance material;

.2 list of IMO instruments which should be reviewed;

.3 information, including demonstration and documentation, for the proposed planned outputs, in accordance with MSC-MEPC.1/Circ.4/Rev.4;

.4 outcome of the High level Review (NCSR 1/29, annex 10); and

.5 outcome of the Detailed Review (NCSR 3/28, annex 7).

2.1.2	Proposed amendments to SOLAS Chapter IV

The IMO/ITU EG considered a proposal by France, Portugal and Romania (IMO/ITU EG 12/4/2) on amendments to SOLAS Chapter IV. In the view of the co-sponsors, the design of the GMDSS, as developed in the general review, needed to be simplified so that it remained straightforward, logical and clear to users. The revision of SOLAS would be discussed after approval of the Modernization Plan and the approval of a new agenda item for this purpose. Therefore, it was expected that this matter would be discussed from 2018 onwards. At that time, the revision should be considered on the basis of the outcome of the review of the GMDSS, taking into account new developments.

2.1.3	Involvement of the HTW Sub-Committee

It was noted that the Human Element would be embodied in the process to ensure that the implementation of the modernised GMDSS and the technology were fit for purpose for both, aboard and ashore. In this context, NCSR 4 should be invited to send the draft Modernization Plan to the HTW Sub-Committee for their consideration and to provide comments and advice, as appropriate.

2.1.4	Non SOLAS vessels

A discussion took place on the participation of non-SOLAS vessels in the GMDSS. It should be ensured that these vessels would not be excluded from participating in the GMDSS when implementing the Modernization Plan.

2.1.5	The relation with e-navigation

The IMO/ITU EG noted the view that it was important to identify the commonalities between GMDSS and the e-navigation concept. Other views were expressed that some caution should be taken, since it was difficult to foresee the consequences. In this context, it was noted that MSC 96 had approved a new output for the NCSR Sub-Committee on e-navigation to define and harmonize the format and structure of Maritime Service Portfolios (MSPs) and to provide guidance on the appropriate communication channels used for the electronic exchange of information between shore and ship, including any necessary coordination mechanisms and transitional arrangements that may be required.

2.1.6	 Additional communication satellite providers in the GMDSS

1- Provision of GMDSS satellite services, including redefinition of Sea Area A3

The IMO/ITU EG noted that the MSC 96 had included an agenda item in the provisional agenda for NCSR 4 to revise SOLAS Chapter IV to include additional mobile satellite systems recognized for use in the GMDSS. It was noted that some of the work currently identified in the Modernization Plan might be undertaken under this agenda item already at NCSR 4. However, it was agreed to keep these items in the Modernization Plan, to safeguard consistency when considering the further revision of SOLAS as a result of the Modernization of the GMDSS.

2- Compatibility of multiple GMDSS satellite services with respect to the delivery of MSI

The IMO/ITU EG had an extensive discussion on the matter related to the compatibility of multiple GMDSS satellite services with respect to the delivery of MSI and noted that he following issues were of relevance:

.1 the formatting of Enhanced Group Calling (EGC) MSI message content;

.2 the routeing of the message from the originators (MSI providers and SAR operators) to satellite service providers for broadcasting; and

.3 the requirement for MSI providers and SAR operators to monitor the broadcast for every provider.

3- Formatting of Enhanced Group Calling (EGC) MSI message content

The IMO/ITUC EG noted that the Joint IMO/IHO/WMO MSI Manual was providing guidance on standardisation and harmonization of the format of Enhanced Group Calling (EGC) MSI messages. It was further noted that the IAMSAR Manual, Volume II, was providing guidance to SAR operators for formatting SAR related EGC, referring to the International SafetyNET Manual explaining that this Manual included examples and coding which must be followed for preparing SafetyNET broadcasts, including SAR broadcasts.

4- Possible solution for the routeing of the message through a single point of distribution

The IMO/ITU EG  further recalled a possible solution, suggested by the ICAO/IMO JWG on SAR (IMO/ITU EG 11/4/2), for transmitting EGC by having one single point of distribution where message originators (MSI providers and SAR operators) would deliver their messages, which would be forwarded to satellite service providers for broadcasting through their respective network.

In this context, it was noted that remaining questions to be answered, among others, where who would operate, maintain and finance such a single point of distribution. A view was noted that this concept could have similarities with the LRIT system. Another view was noted that the “maritime cloud” under development in the field of e-navigation, might play a role in this regard in future. But whatever the solution, the question of the management of such a system is pending.

5- Monitoring of broadcasts

Noting the requirement for the originator (MSI providers and SAR operators) to monitor the broadcast of their messages by every satellite service provider, the originators would experience increased costs if separate receivers were needed for this purpose. It was noted that this issue needed more consideration in the Correspondence Group.

6- The need to minimize the costs, or at least cost increases

NCSR Sub-Committee focused the need to minimize the costs, or at least cost increases, for MSI providers and SAR operators to provide and monitor MSI broadcasts over multiple GMDSS satellite service providers. It was noted that the issue of interoperability was discussed by the satellite Industry. One of the concerns to provide the service of broadcasting MSI free of charge to the originator was the risk for a much larger amount of messages which would have its effect on the capacity of the satellites. In this context, it was further noted that Inmarsat was currently broadcasting an average of 1,000 messages a day.

7- Revision of resolution A.707(17)

Resolution A.707(17) on charges for distress, urgency and safety messages through the Inmarsat system should be revised to make it generic for all satellite services recognised for use in the GMDSS. It was noted that the cost element with multiple GMDSS satellite service providers should be reviewed to minimize the costs, or at least cost increases, for MSI providers and SAR operators to provide and monitor MSI broadcasts over multiple GMDSS satellite service providers. In this context, Recommendation ITU-T D.90 on Charging, billing, international accounting and settlement in the maritime services is of interest to the GMDSS satellite service providers, but is not affecting the requirements set out by IMO in resolution A.707(17).

Considering all related items listed above related to the question of additional communication satellite providers to GMDSS, the Correspondence Group is invited to further consider issues related to multiple GMDSS satellite services with respect to the delivery of MSI, and update the preliminary draft of the Modernization Plan accordingly.

2.1.7	VHF Data Exchange System (VDES)

The IMO/ITU EG noted that paragraph 24 on VDES should not be placed under the heading ’Provision of GMDSS satellite services, including redefinition of Sea Area A3’, and, consequently, it was placed in a separate section.

2.1.8	Routeing of distress alerts and related information

The Group invited the Correspondence Group to further consider the issue of the routeing of distress alerts and related information, taking also into account document NCSR 3/14/1 (United States) on the possible use of the Cospas-Sarsat system for distribution of GMDSS digital distress alerts in addition to the current 406 MHz beacon alerts (NCSR 3/29, paragraphs 14.10 to 14.13). In this context, it was particularly noted that not only the routeing of distress alerts received by satellite systems be considered, but for all distress alerts.

2.1.9	Cospas-Sarsat MEOSAR System and Second-Generation 406 MHz Distress Beacons

The United States (IMO/ITU EG 12/4/1) provided information  on the International Cospas-Sarsat Programme's introduction of the Medium-altitude Earth Orbiting Satellite Search and Rescue (MEOSAR) System and Second-Generation 406 MHz Distress Beacons (SGB), as well as the consequential need to review and consider updating of IMO resolution A.810(19) on the 406 MHz EPIRB Performance Standard. In this context, the revision of this performance standard was already an agenda item for the NCSR Sub-Committee. The NCSR Sub-Committee should further consider the possibility to allow for the addition of an AIS technology locating device to the EPIRB.

2.1.10	Carriage requirements for SART in lifeboats and liferafts

A decision needed to be taken as to whether all lifeboats, and whether some or all inflatable liferafts should be equipped with installed Search and Rescue Transmitters (SART), and how that requirement should be introduced. This point should be further considered when revising SOLAS, with the involvement of the SSE Sub-Committee.

2.1.11	NAVDAT

NAVDAT had almost not been taken into account by IMO up to now. It was further noted that there would be a need for a new output for the incorporation of NAVDAT in the GMDSS. It was also noted that tests have been conducted and were still taking place and that results of those tests should be brought to the attention of the Organization. The name NAVDAT should not be changed, since it was already widely used elsewhere, including in ITU documentation.

Interference and other issues, including a coordination scheme needed to be further considered. In this context, it was noted that it might not be possible to copy the existing NAVTEX scheme.

2.1.12	HF communications

The IMO/ITU EG noted the need for an updated list of HF stations in the GMDSS Master Plan, including information on coast stations capable of receiving and responding to test messages.

2.1.13	Shore-based personnel training

In addition to seafarer training, shore-based personnel training and operational requirements would be affected and that amendments to the Radio Regulations, IAMSAR Manuel, COMSAR/Circ.33 on the GMDSS Coast Station Operator’s Certificate (CSOC) Model course might be required.

# 2.2 	conclusion oF THE 12TH IMO/ITU Experts groups

It was, in particular, noted that new outputs/agenda items were needed to continue the work. After consideration, the IMO/ITU EG agreed that the following outputs would be needed as a minimum:

.1 revision of the relevant regulations of SOLAS Chapters III and IV, and all necessary related and consequential amendments to other existing instruments, including non-mandatory instruments;

.2 revision of resolution A.1001(25) and MSC.1/Circ.1414; and

.3 development of performance standards for NAVDAT.

Having noted the information provided by the coordinator of the Correspondence Group, that this Group was already aiming to deliver a complete draft of the Modernization Plan for consideration at NCSR 4, the IMO/ITU EG agreed that the aim should be to have the Modernization Plan endorsed by NCSR 4 with the view to approval by MSC 98 in June 2017. This would allow MSC 97 to take the need for the new outputs into consideration when identifying the products to be included as planned outputs in the High-level Action Plan for the 2018-2019 biennium.

# References

Report to related documents in paragraph 2

# Action requested of the Committee

The ENAV Committee is requested to take note of the above information and take action accordingly, in particular:

- to consider GMDSS as a framework for the development of e-navigation, and

- to consider shore-to-shore communication in completion of the GMDSS modernization carried out by IMO.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain