Input paper:   ENAV19-6.6

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					□  Input

X  ENAV	□  VTS							X  Information

Agenda item 			6

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)		Mikael Lind for the STM validation project

Viktoria Swedish ICT (Sweden) 
Swedish Maritime Administration (Sweden) 
Valenciaport Foundation (Spain)

# Port Call Message Standard, version 0.5

# Background

Ship-to-port, port-to-ship, port-to-port, as well as port actor-to-port actor communication needs to be standardized and precise in order to contribute to efficient coordination of port calls. In this information paper the port call message standard is presented. This has been developed based on 1) multiple instances of Port Collaborative Decision Making (PortCDM), as a sub concept of Sea Traffic Management (STM) validated within the STM validation project, in different ports, and on 2) joint efforts with contemporary developments (such as the emerging time stamp standard, route exchange format, EPCIS according to GS1, and standards adopted by port community systems and National Single Windows) using time stamp standards.  The port call message standard builds upon the idea takes as its point of departure, the business logic, i.e. the understanding of how to conceive a port call, in order to derive time stamps relevant to share information about.

# The Port Call Process conceived

A port call process is conceived as divided into three process steps; Arrival, Port Visit, and Departure, and two contextual process steps; pre-arrival and post-departure. The Port Visit reflects the purpose of call. Each process step is to be conceived as existing of different sub processes and each sub process covers a number of events (see figure 1 below). Dependent of the purpose of call there will be different combinations of sub processes and events. This hierarchical structure provides a capability to position a state as part of an event, an event as part of a sub process, and a sub process as part of a process step, providing basis for deriving situational awareness for enhanced co-ordination. Sub processes and events could be conducted sequentially and in parallel. During a port call there will always be an arrival and departure process step and most likely a port visit. Combination of sub processes and events are dependent of the purpose of call.

Figure 1: The conception of a port call process

A port call is thus a process composed of process steps for arrival, port visit, and departure putting the operations associated with the turn-around process of the vessel at the core. The arrival process consists of port arrival sub process. The port visit is composed by series of port manoeuvring, berth visit, and anchoring sub processes. As indicated in figure 1 the port visit can be constituted by multiple port manoeuvring, berth visit, and anchoring events. In this way berth shifting can be captured. The departure process step captures the port departure as a sub process. From a port perspective, the arrival step is preceded by pre-arrival capturing sub processes related to actions related to the vessel’s journey stretching back to the departure from previous port, and the departure step is succeeded by actions related to the vessel’s journey stretching to the arrival to the next port.

In figure 2, a metro map is used as the metaphor for illustrating the complexity, and the need for collaboration between multiple actors, in staging a port call. The metro map expresses a positioned state catalogue in relation to the port call process and its actors on a generic level. The metro map is also a source for inspiration for different dashboards being used, adapted to each actor’s needs, for providing situational awareness enabling informed decisions on collaborative foundations to be made by each actor. In figure 2, each metro line represents an actor and each metro station represents a state — such as a location state or service state — that is of importance for the coordination of the port call process. The metaphor illustrates a flow of states having a coordinating function in a port call, from the arrival of a vessel (left part of figure 2) to its departure (right part of figure 2). By including states related departure previous port and arrival next port the port visit at the focused port can be associated to a chain of port visits and thus enabling port-to-port collaboration.

As depicted in figure 2, there are several states that indicate a high degree of coordination and synchronized performances from several actors. The states that require two or more actors to be synchronized in time and/or space are referred to as coordination points. States prior to coordination points are as important for an optimal realization of a port calls but are more actor specific in their nature and are defined as actor specific milestones.

Figure 2: States and coordination points in the port call process 
- a generic state chart used for port specific adaptation of the PortCDM concept

The composition of states related to a particular event represent stages of progress for an event, as e.g. the result of the event towage_requested is an indication of the progress of the event towage.

# The constituents of time stamps

To a particular state there are different communicative functions that are associated. The time of when a particular state should be reached could be recommended to someone(s) for its occurrence (as e.g. a recommendation of when the vessel should initiate the port visit). The time of when a particular state could be targeted of when it is aimed to occur as well as estimated of when it is expected to be occur. When a state has occurred its status becomes actual. The intentions of reaching a particular state could also be cancelled.

These different time types, i.e. recommended, targeted, estimated, actual, and cancelled are different possible time types that could enable enhanced coordination among involved actors. Associated states to an event represents the progress of the event, is initiated by the progress of other states (within the same or in other events), and could trigger other states (within the same or in other events).

A state could concern the certain time a physical object has arrived at, or departed from a particular geographical spot (location state), such as the vessel is at berth (all fast), and the certain time a particular service is to be commenced or completed (service state), such as cargo operations are commenced. Commencing and completing a service is normally preceded by sequences of communicative acts such as requested, request received, denied, and confirmed regulated in the time sequence of the state (see section 3.2) below.

A unique combination of time type and state type constitute the time stamp (figure 3), as e.g. Estimated Time of Arrival Vessel at Pilot Boarding Area which is the specification of what is often referred to as ETA. This suggested level of granularity specifying what it concerns (reference object, see section 2.2.1 on location states) and to which location, decreasing the probability for misunderstanding.

Figure 3: The constituent of a time stamp

A full list of, so far, identified states (including abbreviations and synonyms) associated to a port call is captured in appendix II. This list of location and service states are to be seen as generic. In future versions of the port call message standard different types of states associated with different types of port calls, such as cruises, container traffic, wet and dry bulk, ferries etc. will be pinpointed.

## Location states

A location state captures the location (such as traffic area, berth, tug zone, pilot boarding area, etc.) to which a reference object (such as vessel, pilot, tug, etc.) arrive to or depart from (figure 4). As for example, based on the nomenclature for a location state an arrival for a vessel to traffic area is expressed as Arrival_Vessel_Traffic_Area. A location state is thus defined by a unique combination of time sequence, reference object, and from_location/to_location.

Figure 4:The constituents of a location state

In other words, a reference object is the object that changes location. The following reference objects have been identified so far (extensible list for future versions of the port call message standard):

| - Vessel | - Pilot | - Ice Breaker |
| --- | --- | --- |
| - Tug | - Escort tug | - Passenger |
| - Agent | - Arrival Moorer | - Security |
| - Departure Moorer | - Pontoons & Fenders | - Skilled worker |
| - Gangway | - Pilot Boat |  |

For an arrival state the to_location is mandatory and the from_location is optional and for a departure state the from_location is mandatory and the to_location is optional. The following locations have been identified so far (extensible list for future versions of the port call message standard):

| - Anchoring Area | - Berth | - Etug zone |
| --- | --- | --- |
| - Variable locations | - Pilot Boarding Area | - Rendezvous Area |
| - Traffic Area | - Tug Zone | - Vessel |

## Service states

A service state expresses services (such as towage, pilotage etc.) requested by someone and to be supplied to someone and whether the message describes commencement or completion (c.f. figure 5). Agreements between actors are patterns of interaction constructed of requests, reception of requests, denials and confirmations between actors, as for example, pilot requested and pilot confirmed. The time sequence characteristics possible to express associated to service states are; requested, request received, denied, confirmed, commenced, and completed. In combination with the time type, the first four states will most likely be an actual time, as e.g. actual time of pilotage requested addressing which time the service is requested, request received for, denied, and confirmed. A service state is thus defined by a unique combination of service object and time sequence.

The location for where the service to be supplied is possible to specify, but optional. If the service implies a movement from one location to another, as in the case of pilotage and towage, it is possible to specify from which location (“from location”) and to which location (“to location”) the service is conducted. In the case of that it is relevant to specify where the service is to be/are being performed (not involving any movement) as in the case of arrival mooring operations and departure mooring operations, the “at location” is possible to use for specifying where the service is realized.

Figure 5: The constituents of a service state

The following service objects have so far been identified (extensible list for future versions of the port call message standard):

| - Anchoring | - Gangway | - Security |
| --- | --- | --- |
| - Arrival Mooring Operation | - Departure Moorers | - Arrival Moorers |
| - Berth Shifting | - Arrival Anchoring Operation | - Arrival Berth |
| - Departure Anchoring Operation | - Arrival PortArea | - VTSAreaEntry |
| - Departure PortArea | - Bunkering Operation | - Cargo Operation |
| - Departure Mooring Operation | - Departure Berth | - Garbage Operation |
| - Pilotage | - VTSAreaDepart | - Escort Towage |
| - Pilotboat | - IceBreaking Operation | - LubeOil Operation |
| - PostCargo Survey | - PreCargo Survey | - Pontoons & Fenders |
| - Towage | - Provision Operation | - Skilled worker |
| - Slop operation | - Sludge Operation | - Tours |
| - Forklift | - Water Operation | - Embarking |

These different services are options dependent on the purpose of the call, the characteristics of the cargo, and particular port characteristics. As e.g. pre/port cargo operations survey is not applicable for all types of cargo. The list of possible service objects is thus dependent of the characteristics of the port call such as, type of vessel, and port characteristics.

## Different time types

As indicated in figure 3, five time types are covered in the port call message standard. The five basic time types are defined as follows:

- TT – Targeted Time: The time when a particular actor committed to arrive at, or depart from, a certain location (location state) or initiate/complete a particular service (service state).

- RT – Recommended Time: The time recommended to another actor to arrive at/depart from a certain location (location state) or initiate/complete a particular service (service state). Recommended times is used as a basis for changing targeted times.

- CT – Cancelled Time: The cancellation of a time stamp, as e.g. cancellation of an earlier targeted, recommended, or estimated time to arrive at/depart from a certain location (location state) or a particular service (service state).

- ET - Estimated Time: The time for when a particular actor estimates to (possibly based on calculations) arrive at, or depart from, a certain location (location state) or initiate/complete a particular service (service state). An estimated time is often made based on assumptions on forecasted circumstances (such as weather forecasts) and insights on other actors’ estimations.

- AT – Actual Time: The time when an actor arrived at/departed from a certain location (location state) or initiated/completed a particular service (service state). The actual time is used for evaluation based on the actual occurrence related to planned and/or estimated times. Actual times can also be used for billing, logbooks and/or statements of facts.

# States associated with the different events

Each event is built upon the different states that are reached by different actions performed by involved actors. The actions are associated with different events. A composition of states is thus used to define a particular event. In the figure 6, different states associated with typical events are captured.

In order to support the coordination, states in different events are to be related to each other, as e.g. the state arrival_mooringOp_completed in the event arrival berth is a pre-condition for the state cargoOp_commenced in the event cargo operations.

Figure 6: States associated with events

# Summary – the constituents of the port call process

To summarize, the port call process is a complex sequence of activities, some optional and some mandatory, and some sets of sequences possibly repeated. However, the more information about states, the better possibility to coordinate for the purpose of optimization and synchronization. Thus, it needs to be captured on different levels of granularity to provide complete operational support. A layered model has therefore informed the composition of a port call message standard. The basic unit of analysis is the time stamp. A combination of states characterizes an event and a sub-process is compound of one or several events. In figure 7, these layers (including instances) are captured.

Figure 7: Layered model constituting the port call process

To be noted is that there might be several different events (service objects) applicable for different purposes of the call. Examples of such events would be bunkering, repairs, waste/garbage disposal, supplies/provision, sloop operations, sludge operations, water supply, as well as formalities (customs, immigration, health inspections etc.). All of these are not yet covered, i.e. there is room for extension of the standard. The port call message standard allows for bringing in new service object adapted to different purposes of calls and to different port needs. Such service objects would then follow patterns of requested-request received-denied-confirmed-commenced-completed.

# Information model and XML-schema

## Information Model

PortCall is the central object in the port call information model (c.f. figure 8). A PortCall encapsulates the data related to one Vessel’s visit to one Port as part of one inbound Voyage and one (optional) outbound voyage dependent on if the voyage is concluded in the port call. This means that a voyage could refer any number of port calls, i.e. two or more dependent if the same voyage includes one or several port calls, and a port call could refer to one or two voyages. At the Destination PortCall of a Voyage, this Voyage is the inbound Voyage of the PortCall. This PortCall will become the departing PortCall of a new outbound Voyage.

The Vessel, Port and Voyage entities are assumed to be part of, and defined in, the sea traffic management (STM) information domain (indicated by the <<STM>> stereotype). The PortCall entity might also be part of the STM information domain, but shall at least have a unique STM identifier—port call messages, carrying data about a port call must provide the port call identifier as means of identifying the port call for which the data applies. It is further assumed that Location is part of the STM information domain, keeping a registry of all valid locations, including ports, and the relevant locations within a port (e.g. rendezvous point, anchoring areas, and quays).

Port call data are encapsulated in a number of abstractions. The top level abstraction is the ProcessStep, which represents overarching port call phases (referred to as process steps in section 3 above) currently envisioned to be: Pre-Arrival, Arrival, Port visit, Departure, and Post-departure. A ProcessStep is further subdivided into SubProcess entities, where a SubProcess represents a set of related events. An example of a SubProcess instance is port manoeuvring, i.e. taking the vessel from an anchoring zone to berth, or between quays. A SubProcess entity is subdivided into Event instances, where an Event represents a small and coherent unit of work. Examples of typical events are pilotage, towage, cargo operations, arrival berth, departure berth, arrival port area, and departure port area. An Event, in turn, comprises State instances. A State represents a progress (or state) of a specific operation. The progress of an operation is chosen carefully such that it carries important properties for synchronizing and evaluating overall port call progress, and for planning future operations. Typical examples of State instances are: arrival vessel to traffic area (Arrival_Vessel_TrafficArea) (i.e. to the port area), cargo operations completed (CargoOp_Completed), and towage commenced (Towage_Commenced). Following the nomenclature discussed in section 3, concrete instances of a State can be either a LocationState or a ServiceState is depicted in the model.

A LocationState represents a reference object’s arrival to, or departure from (controlled by the type LocationTimeSequence), a specific location.

A ServiceState represents the progress of a service object (controlled by the type ServiceTimeSequence), such as cargo operations commenced or towage completed. The abstract ServiceState is specialized into either NauticalServiceState or StationaryServiceState depending on whether the service concerns movement (from one location to another, e.g. towage) or is performed at a specified location (e.g bunkering or cargo operations). Performing actor is also captured for service states.

Finally, a State may contain any number of Statement instances. Together with LocationState and ServiceState, Statement dictates content and structure of the Port Call Message format. A Statement represents a reported data point, and corresponds directly to exactly one message in the Port Call Messaging format. A Statement comprises a stated time (timeStatement) and a specification of the meaning of that time (controlled by the type TimeType). For instance, providing an estimated time for the State ‘arrival_vessel_trafficarea’ with the TimeType ESTIMATED would mean an estimation of when the vessel will arrive to the port. In addition, a Statement comprises statement meta-data (which Actor reported the data, and at which time) as well as a free text comment.

In addition to the information model described above, the StateDefinition entity represents a valid PortCDM state. While combining elements of the Port Call Message format may produce any number of possible States, only a subset of these is considered valid within PortCDM (an initial set is shown in the port call metro map metaphor). It is envisioned that a standardized State Catalogue defines the set of valid PortCDM states in terms of StateDefinition instances. A StateDefinition shall comprise at least an identifier, a name/description together with the Port Call Message (PCM) format elements that define it. Example StateDefinition instances may be defined as:

ID: 		Arrival_Vessel_TrafficArea

Name:		Vessel arrival to traffic area

PCM definition:	Type: LocationState,

ReferenceObject=VESSEL, TimeSequence=ARRIVAL_TO, Location=TRAFFICAREA

ID: 		CargoOp_Commenced

Name:		Cargo operations Commenced

PCM definition:	Type: StationaryServiceState,

ServiceObject=CARGO_OPERATION, TimeSequence=COMMENCED

Figure 8: PortCDM Information Model (version 0.0.14 2016-09-02).

## XML Schema and schema definition

This section describes the technical implementation of a port call message. While port call messages are represented by the Statement class in the port call information model, parts of the message elements are held by the State class to which the Statement belongs; for instance: ReferenceObject, TimeSequence and Location. Such elements determine to which StateDefinition the implied Statement belongs. Thus, in relation to the Port Call information model, a port call message carry in addition to the reported time and related meta-data, certain information regarding which State the reported time concerns, and thereby implicitly also the type of event. Identification of the more abstract entities in the information model (as well as the individual State and Event instances) will require algorithmic solutions.

The technical implementation for Port Call Message Format is described as an XML Schema (see figure 9 and appendix II for schema definition) to facilitate an unambiguous representation of the format that allows implementation in a range of different software environments in a consistent manner. A few additions and adaptations stemming from the nature of XML schemas and simple convenience of usage that are worth noting:

- LocationState has no explicit element holding the time sequence. Instead, this is expressed as a choice between two similar elements, arrivalLocation and departureLocation. Any LocationState must contain exactly one element, either an arrivalLocation or a departureLocation. Designing it this way lets us impose a specific rule that makes ”from” optional for an arrivalLocation and ”to” optional for a departureLocation

- messageId is based on UUID in URN format, in accordance with RFC 4122. This allows us to assign messageIds in a distributed manner with no centralised id management and still be guaranteed that all messageIds are globally unique. As a result, any message that is sent has an Id

- groupWith is a messageId pointing to another, previously sent message that holds information that refers to a related data point. This facilitates good algorithmical identification of related messages, pertaining to, for instance, the same berth shifting event. The field is optional.

Location, the type that holds information about the actual locations for LocationStates have three identifying fields:

- name, which is a logical location, in the sense that it identifies a location as being a quay, pilot station or anchoring area. This is the only mandatory field

- identity points to a specific instance of a logical location. This can be a quay number, the name of a specific anchoring area or similar.

- Position is the geographical position of the location. Mainly intended for locations that are not geographically fixed over time, such as a specific rendezvous point between vessel and pilot.

Figure 9: XML-schema (ver 0.0.15)

# Acknowledgement

This version 0.5 of the Port Call Message Standard is a further refinement of the Port Call Message Standard 0.42 issued in the spring of 2016. The refinement has been informed by experiences from putting the standard into use in multiple instances. The refinement to this version has been a collaborative effort between Mikael Lind, Sandra Haraldson, Almir Zerem, Mathias Karlsson, Niklas Mellegård, Eddie Olsson, Robert Rylander (Viktoria Swedish ICT), Mikael Hägg (Swedish Maritime Administration), Sean Deehan, Gabriel Ferrús Clari, and José Andrés Giménez Maldonado (ValenciaPort Foundation).

# Action requested of the Committee

The Committee is requested to note the information and take appropriate action.

# Appendix I: State cataLogue for location and service states

Figure 10: Definition of location states

Figure 11: Definition of service states -  part 1

Figure 12: Definition of service states -  part 2

Figure 12: Definition of service states -  part 3

# Appendix II: Schema definition

<?xml version="1.0" encoding="utf-8" ?>

<!-- changelog

Version: 0.0.15

Revision date: 2016-09-01

- new message type: MessageOperation. For handling operations on previously submitted Port Call Messages

- enumeration updates

+ LocationReferenceObject

- AGENT

- MOORER

+ ServiceObject

- ARRIVAL_MOORING_OPERATION

- DEPARTURE_MOORING_OPERATION

+ ServiceTimeSequence

- CANCELLED removed (into TimeType)

+ TimeType

- CANCELLED added

+LogicalLocation

- NEXT_PORT and PREVIOUS_PORT removed

- length of comment now set to 200 characters

- time moved into LocationState and ServiceState

- timeType moved into LocationState and ServiceState

Version: 0.0.14

Revision date: 2016-07-07

- enumeration updates

+ ServiceObject

- GANGWAY

- EMBARKING

- PILOT_BOAT

- PONTOONS_AND_FENDERS

- SECURITY

- TOURS

- FORKLIFT

+ LocationReferenceObject

- SKILLED_WORKER

+ ServiceTimeSequence

- CANCELLED

- update MRN type to allow for upper case letters

Version: 0.0.13

Revision date: 2016-05-23

- made performingActor in ServiceState optional

- changed performingActor in ServiceState to a simple MRN, to allow for any type of MRN based identifier

- changed schema namespace to URN

- added schema location to appInfo

- fixed inconsistent formatting of changelog

Version: 0.0.12

Revision date: 2016-05-06

- changed regex for PortCallIdentifier to allow upper case characters in URN

Version: 0.0.11

Revision date: 2016-04-26

- enumeration updates

+ ServiceTimeSequence

- REQUEST -> REQUESTED

- CONFIRM -> CONFIRMED

- new REQUEST_RECEIVED

- new DENIED

+ServiceObject

- ARRIVAL_ANCHORAGEAREA -> ARRIVAL_ANCHORING_OPERATION

- DEPARTURE_ANCHORAGEAREA -> DEPARTURE_ANCHORING_OPERATION

- new ANCHORING

- new ARRIVAL_VTSAREA

- new BERTH_SHIFTING

- new DEPARTURE_VTSAREA

+ LocationReferenceObject

- ICE_BREAKER -> ICEBREAKER

+LogicalLocation

- ANCHORAGE_AREA -> ANCHORING_AREA

- PILOT_STATION -> PILOT_BOARDING_AREA

- TUG_STATION -> TUG_ZONE

- new ETUG_ZONE

- new LOC

- new NEXT_PORT

- new PREVIOUS_PORT

- new RENDEZV_AREA

- new VESSEL

Version: 0.0.9

Revision date: 2016-04-19

- renamed ServiceReferenceObject to follow concept standard

- updated ServiceObject with new list of constants

Version: 0.0.8

Revision date: 2016-04-15

- field length for PortCallMessageIdentifier changed from 67 to 66, to match the actual pattern

Version: 0.0.7

Revision date: 2016-04-06

- Made reportedBy and reportedAt optional, since they should be determined be the receiving service

Version: 0.0.6

Revision date: 2016-04-01

- Added  PORT to LogicalLocation

Version: 0.0.5

Revision date: 2016-03-31

- Moved REQUEST and CONFIRM from TimeType to ServiceTimeSequence

Version: 0.0.4

Revision date: 2016-03-16

- Renamed LatLong type Position

- Added coordinate system to documentation for Position

- Added type MRN

- Added MRN based identifiers for port call and message

Version: 0.0.3

Revision date:2016-02-19

- Made groupWith optional

Version: 0.0.2

Revision date:2016-02-19

- Added missing element portCallId to root node

- Added performingActor to type ServiceState

- Added messageId

- Added groupWith

-->

<xs:schema attributeFormDefault="unqualified"

elementFormDefault="qualified"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:pcm="urn:x-mrn:stm:schema:port-call-message:0.0.15"

targetNamespace="urn:x-mrn:stm:schema:port-call-message:0.0.15">

<xs:annotation>

<xs:appinfo>

version = 0.0.15

revisionDate = 2016-09-01

schemaLocation = http://specification.portcdm.eu/pcm/v0.0.15/port-call-message.xsd

</xs:appinfo>

<xs:documentation>

Schema for PortCDM PortCall Message Format

The root element is a Port Call Message, containing some metadata about when and by whom this message was

sent, followed by a time type and either a location state or a service state.

<dl>

<dt>messageId</dt>

<dd>URN formatted and UUID-based so that no central identity management is necessary</dd>

<dt>groupWith</dt>

<dd>messageId of another message that this message should be grouped with. This is to facilitate

repeated events like berth shifting by improving the ability to group data</dd>

<dt>portCallId/localJobId</dt>

<dd>portCallId is the global port call id allocated by PortCDM. localJobId is the job or port call id used in the connecting system. For connectors, one or zero

of these can be used. Port Call Messages going into Message Brokering must have portCallId set appropriately</dd>

</dl>

There is also an optional comment element. All time values are in ISO-8601 format as per the dateTime type, in UTC, as denoted by the Z suffix

</xs:documentation>

</xs:annotation>

<xs:element name="portCallMessage">

<xs:complexType>

<xs:sequence>

<xs:choice minOccurs="0">

<xs:element name="portCallId" type="pcm:PortCallIdentifier"/>

<xs:element name="localJobId" type="pcm:LocalJobIdentifier"/>

</xs:choice>

<xs:element name="vesselId" type="pcm:VesselIdentifier" minOccurs="0"/>

<xs:element name="messageId" type="pcm:PortCallMessageIdentifier"/>

<xs:element name="groupWith" type="pcm:PortCallMessageIdentifier" minOccurs="0"/>

<xs:element name="reportedAt" type="xs:dateTime" minOccurs="0"/>

<xs:element name="reportedBy" type="xs:string" minOccurs="0"/>

<xs:element name="comment" minOccurs="0">

<xs:annotation>

<xs:documentation>

Comment is limited to a maximum of 200 characters.

Please note that since the encoding is UTF-8, this means a maximum of 800 bytes in this

field

</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:maxLength value="200"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:choice>

<xs:element name="messageOperation" type="pcm:MessageOperation"/>

<xs:element name="locationState" type="pcm:LocationState" />

<xs:element name="serviceState" type="pcm:ServiceState" />

</xs:choice>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:complexType name="LocationState">

<xs:annotation>

<xs:documentation>

LocationState is a representation for LOCATION STATE in the specification.

It describes a time related data point for an arrival to or departure from a specified location.

Time sequence is expressed implicitly by the choice of element arrivalLocation or departureLocation.

This is to avoid the redundancy of having to add a fixed element in each of the respective elements

(ARRIVAL for arrivalLocation and DEPARTURE for departureLocation. Using this design allows us to dictate

in the messaging format itself that "to" is mandatory, and "from" optional if time sequence is ARRIVAL

and the reversed for time sequence DEPARTURE

</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="referenceObject" type="pcm:LocationReferenceObject"/>

<xs:element name="time" type="xs:dateTime"/>

<xs:element name="timeType" type="pcm:TimeType"/>

<xs:choice>

<xs:element name="arrivalLocation">

<xs:complexType>

<xs:sequence>

<xs:element type="pcm:Location" name="from" minOccurs="0"/>

<xs:element type="pcm:Location" name="to"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="departureLocation">

<xs:complexType>

<xs:sequence>

<xs:element type="pcm:Location" name="from"/>

<xs:element type="pcm:Location" name="to" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:choice>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ServiceState">

<xs:annotation>

<xs:documentation>

ServiceState is a representation for SERVICE STATE in the specification.

It describes a time related data point for a service. The service can be either stationary

(sludge, bunkering, cargo operations ) in which case the "at" elements hold its location or navigational

(towage, escort towage or piloting) which requires to and from locations.

performingActor is a MRN, which allows for any type of identifier that can be expressed in that format.

This can be an indirect reference to an actor, like the IMO or MMSI based identifier for a tug boat, which

in itself is not an actor, but a resource associated with an actor.

</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="serviceObject" type="pcm:ServiceObject"/>

<xs:element name="performingActor" type="pcm:MRN" minOccurs="0"/>

<xs:element name="timeSequence" type="pcm:ServiceTimeSequence"/>

<xs:element name="time" type="xs:dateTime"/>

<xs:element name="timeType" type="pcm:TimeType"/>

<xs:choice>

<xs:element name="at" type="pcm:Location"/>

<xs:element name="between">

<xs:complexType>

<xs:sequence>

<xs:element name="to" type="pcm:Location"/>

<xs:element name="from" type="pcm:Location"/>

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:choice>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="ServiceObject">

<xs:restriction base="xs:string">

<xs:enumeration value="ANCHORING"/>

<xs:enumeration value="ARRIVAL_ANCHORING_OPERATION"/>

<xs:enumeration value="ARRIVAL_BERTH"/>

<xs:enumeration value="ARRIVAL_PORTAREA"/>

<xs:enumeration value="ARRIVAL_VTSAREA"/>

<xs:enumeration value="BERTH_SHIFTING"/>

<xs:enumeration value="BUNKERING_OPERATION"/>

<xs:enumeration value="CARGO_OPERATION"/>

<xs:enumeration value="DEPARTURE_ANCHORING_OPERATION"/>

<xs:enumeration value="DEPARTURE_BERTH"/>

<xs:enumeration value="DEPARTURE_PORTAREA"/>

<xs:enumeration value="DEPARTURE_VTSAREA"/>

<xs:enumeration value="ESCORT_TOWAGE"/>

<xs:enumeration value="GARBAGE_OPERATION"/>

<xs:enumeration value="ICEBREAKING_OPERATION"/>

<xs:enumeration value="LUBEOIL_OPERATION"/>

<xs:enumeration value="ARRIVAL_MOORING_OPERATION"/>

<xs:enumeration value="DEPARTURE_MOORING_OPERATION"/>

<xs:enumeration value="PILOTAGE"/>

<xs:enumeration value="POSTCARGOSURVEY"/>

<xs:enumeration value="PRECARGOSURVEY"/>

<xs:enumeration value="PROVISION_OPERATION"/>

<xs:enumeration value="SLOP_OPERATION"/>

<xs:enumeration value="SLUDGE_OPERATION"/>

<xs:enumeration value="TOWAGE"/>

<xs:enumeration value="WATER_OPERATION"/>

<xs:enumeration value="GANGWAY"/>

<xs:enumeration value="EMBARKING"/>

<xs:enumeration value="PILOT_BOAT"/>

<xs:enumeration value="PONTOONS_AND_FENDERS"/>

<xs:enumeration value="SECURITY"/>

<xs:enumeration value="TOURS"/>

<xs:enumeration value="FORKLIFT"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="TimeType">

<xs:restriction base="xs:string">

<xs:enumeration value="ESTIMATED"/>

<xs:enumeration value="ACTUAL"/>

<xs:enumeration value="TARGET"/>

<xs:enumeration value="RECOMMENDED"/>

<xs:enumeration value="CANCELLED"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="Location">

<xs:sequence>

<xs:element name="locationType" type="pcm:LogicalLocation"/>

<!-- Geographical position, for dynamically defined locations like TUG_STATION -->

<xs:element name="position" type="pcm:Position" minOccurs="0"/>

<!-- Location name such as specific quay number or anchoring area name -->

<xs:element name="name" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="ServiceTimeSequence">

<xs:restriction base="xs:string">

<xs:enumeration value="COMMENCED"/>

<xs:enumeration value="COMPLETED"/>

<xs:enumeration value="CONFIRMED"/>

<xs:enumeration value="DENIED"/>

<xs:enumeration value="REQUESTED"/>

<xs:enumeration value="REQUEST_RECEIVED"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="LocationReferenceObject">

<xs:restriction base="xs:string">

<xs:enumeration value="VESSEL"/>

<xs:enumeration value="TUG"/>

<xs:enumeration value="ESCORT_TUG"/>

<xs:enumeration value="PILOT"/>

<xs:enumeration value="PILOT_BOAT"/>

<xs:enumeration value="ICEBREAKER"/>

<xs:enumeration value="AGENT"/>

<xs:enumeration value="ARRIVAL_MOORER"/>

<xs:enumeration value="DEPARTURE_MOORER"/>

<xs:enumeration value="PASSENGER"/>

<xs:enumeration value="SECURITY"/>

<xs:enumeration value="PONTOONS_AND_FENDERS"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="LogicalLocation">

<xs:restriction base="xs:string">

<xs:enumeration value="ANCHORING_AREA"/>

<xs:enumeration value="BERTH"/>

<xs:enumeration value="ETUG_ZONE"/>

<xs:enumeration value="LOC"/>

<xs:enumeration value="PILOT_BOARDING_AREA"/>

<xs:enumeration value="RENDEZV_AREA"/>

<xs:enumeration value="TRAFFIC_AREA"/>

<xs:enumeration value="TUG_ZONE"/>

<xs:enumeration value="VESSEL"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="Position">

<xs:annotation>

<xs:documentation>

To allow us to include physical, geographical position for locations that aren't predefined at a fixed

geographical position i.e. the rendezvous point for vessel and tug boat. Coordinate system is WGS84 Decimal Degrees. Format is decimal degrees notation as described in ISO 6709.

</xs:documentation>

</xs:annotation>

<xs:all>

<xs:element name="latitude">

<xs:annotation>

<xs:documentation>

Latitude in decimal degrees notation as described in ISO 6709. Northern latitudes are denoted by

positive numbers, southern latitudes by negative.

</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:double">

<xs:minInclusive value="-90"/>

<xs:maxInclusive value="90"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="longitude">

<xs:annotation>

<xs:documentation>

Longitude in decimal degrees notation as described in ISO 6709. Eastern longitudes are denoted

by positive numbers, western longitudes by negative

</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:double">

<xs:minInclusive value="-180"/>

<xs:maxInclusive value="180"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:all>

</xs:complexType>

<xs:complexType name="Actor">

<xs:sequence>

<xs:element name="id" type="xs:string"/>

<xs:element name="name" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="UUID">

<xs:restriction base="xs:string">

<xs:length value="36" fixed="true" />

<xs:pattern value="[0-9a-fA-F]{8}-[0-9a-fA-F]{4}-4[0-9a-fA-F]{3}-[8-9a-bA-B][0-9a-fA-F]{3}-[0-9a-fA-F]{12}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="MRN">

<xs:annotation>

<xs:documentation>Marine Resource Name identifier, based on URN. Note that the NID, including the mrn: prefix can be no more than 31 characters long</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:pattern value="urn:mrn:[A-Za-z0-9][A-Za-z0-9-]{0,27}:[A-Za-z0-9()+,\-.:=@;$_!*'%/?#]+"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PortCallIdentifier">

<xs:annotation>

<xs:documentation>Port call identifier, based on MRN. First element of the NSS should be the UN/Locode of the port</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:maxLength value="120"/>

<xs:pattern value="urn:x-mrn:stm:portcdm:port_call:[A-Za-z]{5}:[A-Za-z0-9()+,\-.:=@;$_!*'%/?#]+"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="LocalJobIdentifier">

<xs:annotation>

<xs:documentation>Port call identifier, based on MRN. First element of the NSS should be the UN/Locode of

the port

</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:maxLength value="120"/>

<xs:pattern value="urn:x-mrn:stm:portcdm:local_job:[A-Za-z0-9_]+:[A-Za-z0-9_]+"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PortCallMessageIdentifier">

<xs:annotation>

<xs:documentation>Port call message identifier, based on MRN and UUID. </xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:length value="66" fixed="true" />

<xs:pattern value="urn:x-mrn:stm:portcdm:message:[0-9a-fA-F]{8}-[0-9a-fA-F]{4}-4[0-9a-fA-F]{3}-[8-9a-bA-B][0-9a-fA-F]{3}-[0-9a-fA-F]{12}"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="MessageOperation">

<xs:annotation>

<xs:documentation>Administrative operations on messages sent before</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="operation">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="WITHDRAW"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="messageId" type="pcm:PortCallMessageIdentifier"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="VesselIdentifier">

<xs:annotation>

<xs:documentation>Vessel identifier, based on MRN. Can be either IMO or MMSI based</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:pattern value="urn:x-mrn:stm:vessel:(IMO|MMSI):[0-9]{7}"/>

</xs:restriction>

</xs:simpleType>

</xs:schema>

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain

Lind, M., et al., Overcoming the inability to predict - a PortCDM future, in 10th International Harbor Masters' Association Congress – Global Port & Marine Operations. 2016: Vancouver, Canada.

Within the ECDIS community (for navigational purposes), “planned time” is used for expressing the targeted time.

Within the ECDIS community (for navigational purposes), “planned time” (PTA) for arriving at a particular location (flowpoint) is used for expressing the targeted time. This means that PTA and TTA is used with the same meaning, but the TTA is used within the PortCDM community.

Example showing two LocationState elements, one for TimeSequence ARRIVAL and one for TimeSequence DEPARTURE. In both cases the optional element to and from, respectively, are included and commented as being optional

<pcm:locationState>

<pcm:arrivalLocation>

<!--Optional:-->

<pcm:from>

<pcm:name>BERTH</pcm:name>

</pcm:from>

<pcm:to>

<pcm:name>ANCHORAGE_AREA</pcm:name>

</pcm:to>

</pcm:arrivalLocation>

</pcm:locationState>

<pcm:locationState>

<pcm:departureLocation>

<pcm:from>

<pcm:name>ANCHORAGE_AREA</pcm:name>

</pcm:from>

<!--Optional:-->

<pcm:to>

<pcm:name>TUG_STATION</pcm:name>

</pcm:to>

</pcm:departureLocation>

</pcm:locationState>

www.ietf.org/rfc/rfc4122.txt

Lind M., Haraldson S. et al (2016) Port Call Message Standard, version 0.42, STM Validation Project