Input paper:   ENAV19-9.11

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					√  Input

√  ENAV	□  VTS							□ Information

Agenda item 			9

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			Mr. Janusz Hajdel, Mr. Krzysztof Mendalka, Mr. Michał Burka. SPRINT S.A. Poland

# Proposal for standard for data exchange for the MS8 vessel shore reporting

# Summary

Sprint S.A. as an IALA Industrial Member, proposes to consider whether the IALA should recommend standard proposed by EMSA (ISO-28005) as leading to "MS8 SHORE VESSEL REPORTING" and proposes to undertake their own research under the aegis of IALA on the possibility of adapting of the ISO28005-2 to the S-100 within the domain S-2XX.

## Purpose of the document

The Committee ENAV19  is invited to give opinions on proposals  for recommend standard proposed by EMSA (expanded ISO-28005), leading to "MS8 SHORE VESSEL REPORTING".

The Committee ENAV19  is invited to give opinions on that Sprint S.A to undertake their own research under the aegis of IALA on the possibility of adapting of the ISO28005-2 to the S-100 within the domain S-2XX

The Committee ENAV 19 forward this information paper to Work Group of ENAV, provide comments and determine the next step in develop, analysis and research.

## Related documents

IALA Guideline on MSPS, section 4.8. ”MS8 SHORE VESSEL REPORTING”

IMO FAL 40th session - http://www.imo.org/en/MediaCentre/MeetingSummaries/FAL/Pages/FAL-40th-session.aspx.

National Single Window for ship reporting – Prototype EMSA.  2015 [http://www.emsa.europa.eu/nsw.html] 

System interface documentation_20151001 [ http://www.emsa.europa.eu/nsw/download/3675/1934/23.html ]

- Master File-NSW prototype v133-20151103 [ http://www.emsa.europa.eu/nsw/download/3994/1934/23.html ]

Enav18 Automated ship reporting.  Originator: Fred Pot. [ http://www.iala-aism.org/media-lib/13401/enav18_12_12_automated_ship_reporting_v1_1.docx ]

ENAV18-12.6 MSP8 Vessel Shore Reporting . Originator: Ulrich Tagne. [ http://www.iala-aism.org/media-lib/13449/enav18_12_6_msp8_vessel_shore_reporting.xlsx ]

Integrating IEC and ISO information models into the S-100 Common Maritime Data Structure. Ørnulf Jan Rødseth. Norsk Marinteknisk Forskningsinstitutt AS Norwegian Marine Technology Research Institute.

# Background

After the IALA workshop in Lisbon (2016-05-23 - 2016-05-26) Sprint S.A. as an IALA Industrial Member wishes to draw attention to the problem of standard for data exchange for the MS8 vessel shore reporting.

This aspect was also noted by the IMO FAL 40th session [1] (http://www.imo.org/en/MediaCentre/MeetingSummaries/FAL/Pages/FAL-40th-session.aspx).

We wish to note at this point that many of the existing vessel shore reporting systems and maritime single window systems uses an ISO-28005 standard. EMSA also elected ISO28005 as the standard for data exchange for the "Pilot National Single Window for ship reporting - Prototype"[2].

The participants of the project are: Bulgaria, Greece, Italy, Malta, Romania and Norway. In the course of the work EMSA expanded ISO28005-2 standard for new positions, which allowed complete all the requirements of Directive 2010/65 / EU, including:

• 72h pre-arrival notice as required under article 9 of directive 2009/16 / EC on Port State Control;

• Border checks on persons (Regulation (EC) No 562/2006);

• Notification of dangerous or polluting goods Carried on board (Directive 2002/59 / EC);

• Notification of waste and residues (Directive 2000/59 / EC);

• Notification of security information (Regulation (EC) No 725/2004);

• Entry Summary Declaration (Regulation (EEC) 2913/92 and Regulation (EC) 450/2008);

• FAL forms 1 to 7 (Maritime Declaration of Health)

• The arrival and departure notifications (ATA and ATD) as required under article 24 of directive 2009/16 / EC.

Poland also decided to implement extended ISO28005-2 standard in the current Polish Single Window System.

Database structure(UML) based on ISO-28005. NSW Polish Single Window System[6]

IALA established using of the S-100 standard in all Maritime Services, but to date there is no S-100 standard for MS8 Vessel Shore reporting.

Sprint S.A. proposes to consider whether the IALA should recommend standard proposed by EMSA (ISO-28005) as leading to "MS8 SHORE VESSEL REPORTING".

Sprint S.A also proposes to undertake their own research under the aegis of IALA on the possibility of adapting of the ISO28005-2 to the S-100 within the domain S-2XX. Our work would depend on the analysis of the possibilities of using ISO-28005 and existing EMSA recommendations as the basis for the standard S-XXX for "MS8 SHORE VESSEL REPORTING." We will base on existing work  [3] [4] and future work (in Oct 2015 TC80 established a new independent workgroup WG17 Common Maritime Data Structure (CMDS) [5]).

Some study already exits  [3] [4] , but we want provide the practical approach to these issues.

# References

IMO FAL 40th session - http://www.imo.org/en/MediaCentre/MeetingSummaries/FAL/Pages/FAL-40th-session.aspx.

National Single Window for ship reporting – Prototype EMSA.  2015 [http://www.emsa.europa.eu/nsw.html] 

System interface documentation_20151001 [ http://www.emsa.europa.eu/nsw/download/3675/1934/23.html ]

Master File-NSW prototype v133-20151103 [ http://www.emsa.europa.eu/nsw/download/3994/1934/23.html ]

- EMSA NSW Data Mapping

ENAV18-12.6 MSP8 Vessel Shore Reporting . Originator: Ulrich Tagne. [ http://www.iala-aism.org/media-lib/13449/enav18_12_6_msp8_vessel_shore_reporting.xlsx ]

Integrating IEC and ISO information models into the S-100 Common Maritime Data Structure. Ørnulf Jan Rødseth. Norsk Marinteknisk Forskningsinstitutt AS Norwegian Marine Technology Research Institute

https://www.iho.int/mtg_docs/com_wg/HSSC/HSSC7/HSSC7-07.3A_IEC_Activities_affecting_HSSC.pdf

Structure of database Polish NSW

# Action requested of the Committee

The Committee is requested to:

- give opinions on proposals  for recommend standard proposed by EMSA (expanded ISO-28005) as leading to "MS8 SHORE VESSEL REPORTING"

give opinions on that Sprint S.A to undertake their own research under the aegis of IALA on the possibility of adapting of the ISO28005-2 to the S-100 within the domain S-2XX

Scope of data in EMSA NSW[2]

| Group/ Element Name | Definition | Occ | Code (if relevant) |
| --- | --- | --- | --- |
| Ship identification |  | 1 |  |
| Ship name | Given name of the ship in the ship registry | 0-1 | Upon SOLAS, chapter I, part B, regulation 15 "Form Certificates", "the particulars inserted in the certificates shall be in Roman characters and Arabic figures". (From “A” to ”Z” and from 0 to 9). Additional characters allowed are dots “.”, dashes “-“ and single apostrophe “ ’ ”. |
| Call sign | Call sign for the ship. Sequence of letters and numbers, unique to each ship by which ships can be identified usually in radio communications. | 0-1 |  |
| IMO number | Unique ship identification number assigned by Lloyd's Register – Fairplay in accordance with IMO resolution A.600(15). | 0-1 | IMO number – IMO Res A.600 (15) |
| MMSI number | Identifier used by maritime digital selective calling (DSC), automatic identification systems (AIS) and certain other equipment to uniquely identify a ship or a coast radio station. | 0-1 | MID according to the ITU regulation. Length of the MMSI number should always be 9 |
| Comment | Any other information related to ship identity | 0-1 | Possibility to provide information (e.g. registration number) on vessels without IMO or MMSI (e.g. pleasure craft, fishing vessels…) |
| Ship particulars |  | 0-1 |  |
| Flag state of ship | The ISO code for the country subdivision in which the means of transport is registered | 0-1 | The Alpha-2 code (two-digits flag code) in accordance with the standard ISO 3166-1. |
| Certificate of registry - Port | Port where the certificate of registry was issued | 0-1 |  |
| Certificate of registry - Date | Date of issue of certificate of registry | 0-1 |  |
| Certificate of registry - Number | Number of the certification of registry | 0-1 |  |
| Inmarsat call number | Number indicating the location of the ship by satellite services of Inmarsat | 0-5 |  |
| Gross tonnage | The measure of the overall size of a ship determined in accordance with the provisions of the International Convention on Tonnage Measurement of Ships, 1969 | 0-1 |  |
| Net tonnage | The measure of the useful capacity of a ship determined in accordance with the provisions of the International Convention on Tonnage Measurement of Ships, 1969 | 0-1 |  |
| Ship type | Code specifying the type of means of transport. | 0-1 | codes the ship type according to UNECE R28. The actual codes shall be taken from and constructed according to the above reference. The code is a two- or three-digit number without any inserted space. |
| Name of company | Name of ship's operating company, as defined in the ISM code | 0-1 |  |
| IMO company identification number | IMO company identification number | 0-1 |  |
| Additional Ship particulars |  | 0-1 |  |
| Year of built | The year ship was constructed | 0-1 |  |
| Dead weight | Ship's dead weight, measured in metric tonnes | 0-1 |  |
| Length overall | Ship's length overall, measured in metres | 0-1 |  |
| Summer draught | Ship's summer draught in metres | 0-1 |  |
| Beam | Ship's beam, measured in metres | 0-1 |  |
| Company contact details |  | 0-1 |  |
| Phone |  | 0-1 |  |
| Fax |  | 0-1 |  |
| Email |  | 0-1 |  |
| Port call |  | 1 |  |
| Port of call | When referring to a voyage leg, this is the port at the end of that leg. This is normally the case when this element is used in pre-arrival notifications. When referring to a port stay, this is the port where the ship is. In case when this element is used in departure notifications, this is the port that the ship leaves. | 1 | The “port of call” attribute must only be the LOCODE of the specific port of call or its dependent port’s LOCODE |
| ETA port of call | Estimated time and date of arrival at the port of call | 0-1 |  |
| ETD port of call | Estimated time and date of departure from the port of call | 0-1 |  |
| Position in port of call | Position of the ship in the port (berth or station) | 0-1 | Any 50 character free text value will be accepted. However M.S are encouraged to utilise a 15 characters (subsidiary) LOCODE identifying the position of a subsidiary location within the port or port approaches |
| Port facility | Facility as defined in ISPS | 0-1 | The port facility's code in the IMO GISIS maritime security database |
| Name of agent | Name of the organisation representing the ship in the context of the call in the port. This may be the Company or an agent, dependent on circumstances | 0-1 |  |
| Contact details of agent | Contact details of agent at port of call | 0-1 |  |
| Phone |  | 0-1 | with an international prefix code |
| Fax |  | 0-1 | with an international prefix code |
| Email |  | 0-1 |  |
| Purpose of call | Primary purpose of the call | 0-9 | EDIFACT codes (8025) |
| Brief description of onboard cargo | This is a short text giving an overview of what cargo the ship carries. This shall also contain brief details of any harmful substances and gases that could endanger persons or the environment. | 0-1 | Type of cargo |
| Additional port call data |  |  |  |
| Fore draught | Ship's fore draught, in metres | 0-1 |  |
| Mid-ship draught | Ship's mid-ship draught, in metres | 0-1 |  |
| Aft draught | Ship's aft draught, in metres | 0-1 |  |
| Air draught | Ship's air draught, in metres | 0-1 |  |
| Pre-arrival 72 hrs notification |  | 0-1 |  |
| Possible anchorage | Whether the call is expected to call at an anchorage | 0-1 | Yes/ No status code |
| Planned operations | Planned operations at the port of call or anchorage of destination (loading, unloading, other) | 0-1 |  |
| Planned works | Planned statutory survey inspection and substantial maintenance and repair work to be carried out whilst in the port of destination | 0-1 |  |
| Tanker hull configuration | Configuration: single hull, single hull with SBT, double hull | 0-1 | Possible values: SHT - indicating a single hull tanker; SHT-SBT - indicating a single hull with segregated ballast tanks (SBT); DHT - indicating a double hull tanker |
| Volume and nature of cargo | volume and nature of the cargo on board the tanker | 0-1 |  |
| Condition of cargo and ballast tanks | Condition of the cargo and ballast tanks: full, empty, inerted | 0-1 |  |
| Arrival |  | 0-1 |  |
| ATA port of call | Actual time that ship arrived at port of call or anchorage | 1 |  |
| Anchorage | Whether the call is at an anchorage | 0-1 | Yes/ No status code |
| Departure |  | 0-1 |  |
| ATD port of call | Actual time that ship departed from port of call or anchorage | 1 |  |
| Voyage |  | 0-1 |  |
| Voyage Number | This is a carrier-assigned reference code for a voyage | 0-1 |  |
| Next port | When referring to a specific leg of a voyage, the next port is the next port to be visited after the leg's arrival port. When referring to a port stay, the next port of call is the arrival port for the next leg leading from this port. | 0-1 | The port is identified by its 5-digit LOCODE |
| ETA to next port | Estimated time and date of arrival at the next port | 0-1 |  |
| Last port | When referring to a leg of a voyage, this is the departure port for that leg. When referring to a port stay, it is the departure port of the leg that led to this port. | 0-1 | The port is identified by its 5-digit LOCODE |
| ETD from last port | Estimated time and date of departure from the from Last port | 0-1 |  |
| Itinerary of cruise ship | List of ports where the ship is expected to call from the initial departure port to the final arrival port of the cruise, with the corresponding dates and times of expected arrivals. | 0-n |  |
| Port |  | 1 |  |
| Expected date and time of arrival |  | 1 |  |
| Last calls at port facilities | Last calls at port facilities | 0-n |  |
| Port | Port of call | 1 | The port is identified by its 5-digit LOCODE |
| Port facility | Port facility identified by its IMO port facility number as in GISIS database | 0-1 | Port facility's code in the IMO GISIS maritime security database |
| Date of arrival | Actual date of arrival | 0-1 |  |
| Date of departure | Actual date of departure | 0-1 |  |
| Dangerous and polluting goods | DPG related information regarding the port call | 0-1 |  |
| INF ship class | Class of the ship needed for INF cargoes as defined in Regulation VII/14.2 / Code for the license of the vessel ship according to the INF Code | 0-1 | Code for the license of the vessel according to the INF Code. Possible values: "INF1", "INF2" and "INF3" |
| Confirm DPG list on board | Confirmation that a list or manifest or appropriate loading plan giving details of the dangerous or polluting goods carried and of their location on the ship is on board. This is a yes/no data element. To be included in NSW | 1 | Yes/ No status code |
| DPG contact details | Address from which detailed information on the polluting and dangerous cargo may be obtained | 0-1 |  |
| Given name |  | 0-1 |  |
| Family name |  | 0-1 |  |
| Location |  | 0-1 | LOCODE |
| Phone |  | 1 | with an international prefix code |
| Fax |  | 0-1 | with an international prefix code |
| Email |  | 0-1 |  |
| Cargo declaration |  | 0-n |  |
| LRN |  | 0-1 |  |
| MRN | Unique reference number for a ENS-declaration issued by the customs. The MRN is unique per ENS declaration and is a 18-digit number that is automatically allocated by the customs office that receives the entry summary declaration (ENS) | 0-1 |  |
| Reporting party (EORI) | Reporting party who has reported the consignment for the specific type of declaration | 0-1 |  |
| First Port of Arrival in EU |  | 0-1 | The port is identified by its 5-digit LOCODE |
| ETA_ENS | Estimated date time of arrival at first place of arrival in Customs territory. ENS - Not to be updated | 0-1 |  |
| Country of routing | Countries of routing reported in ENS declaration | 0-n |  |
| Code |  | 1 | Alpha-2 code in accordance with ISO 3166-1. |
| Consignment | Cargo consignment within a cargo declaration | 0-n |  |
| Port of loading | Identify of the port where the cargo was loaded on board the ship | 0-1 | The port is identified by its 5-digit LOCODE |
| Port of discharge | Identity of the port where the cargo will be discharged from the ship | 0-1 | The port is identified by its 5-digit LOCODE |
| Transport document ID | Transport document, e.g., Bill of Lading, identity code | 0-1 |  |
| Consignment - Cargo details | Cargo related details of a consignment | 0-n |  |
| Number of items |  | 0-1 |  |
| UCR | Unique Consignment reference number assigned by the person concerned to the consignment in question. It may take the form of WCO (ISO 15459) This information should only be provided as an alternative to data element "shipping marks" | 0-1 | ISO15495 |
| PlaceWhenceConsigned | Definition (WCO): Name of the place in country from which the goods were initially despatched to the importing country. Remark: The place of loading may be different from the port of loading | 0-1 |  |
| GoodsReceiptPlace | Definition (WCO): Name of the place at which the goods have been or are to be received by the consignee | 0-1 |  |
| Carrier | Party undertaking or arranging transport of goods between named points | 0-1 |  |
| Name |  | 0-1 |  |
| Street and number |  | 0-1 |  |
| Postal code |  | 0-1 |  |
| City |  | 0-1 |  |
| Country code |  | 0-1 | Alpha-2 code in accordance with ISO 3166-1. |
| Consignor | The party consigning goods as stipulated in the transport contract by the party ordering transport | 0-1 |  |
| Name |  | 0-1 |  |
| Street and number |  | 0-1 |  |
| Postal code |  | 0-1 |  |
| City |  | 0-1 |  |
| Country code |  | 0-1 | Alpha-2 code in accordance with ISO 3166-1. |
| Consignee | Party to which goods are consigned | 0-1 |  |
| Name |  | 0-1 |  |
| Street and number |  | 0-1 |  |
| Postal code |  | 0-1 |  |
| City |  | 0-1 |  |
| Country code |  | 0-1 | Alpha-2 code in accordance with ISO 3166-1. |
| NotifyParty | Party to be notified | 0-1 |  |
| Name |  | 0-1 |  |
| Street and number |  | 0-1 |  |
| Postal code |  | 0-1 |  |
| City |  | 0-1 |  |
| Country code |  | 0-1 | Alpha-2 code in accordance with ISO 3166-1. |
| FreightMethodOfPayment | Code specifying the payment method for transport charges | 0-1 | EDIFACT codes (4215) |
| Number of the authorisation | Number of the authorisation for the status of the authorised person lodging the manifest for the proof of Community status | 0-1 |  |
| AdditionalInformation | Information regarding consignment | 0-1 |  |
| Cargo item | Cargo item within a consignment | 0-1 |  |
| SequenceNumber | Unique identifier for goods item within consignment which is needed as reference for updates and deletions | 1 |  |
| Number of Packages | This is the total number of packages on all cargo units covered by this cargo item | 0-1 |  |
| Package type | This is a description of the outer package of the cargo item | 0-1 | Two-letter alphabetic code of annex V or VI??? of UNECE R21 EDIFACT codes (7065) |
| Gross quantity | The weight (mass) or volume of the dangerous goods contained in a package including the weight (mass) or volume of any packaging material for each DG item | 0-1 | Units of measurement to be considered: KGM, TNE, M3 |
| Net quantity | Net quantity of cargo item: value and unit | 0-1 | Units of measurement to be considered: KGM, TNE, M3 |
| Stowage position | The position or place where goods are stored on board the vessel. This is used in case of cargo which is not in a container or on a trailer. | 1 |  |
| Transport Unit ID | Identification of the transport equipment. For containers, this shall be the identification code as defined in ISO 6346 (limited to goods under IMDG code) | 0-1 | UN/ECE Recommendation 15 |
| Cargo item - Cargo details | Cargo related details of a cargo item | 0-n |  |
| ShippingMarks |  | 0-1 | UN/ECE Recommendation 15 |
| Description of goods | Textual description of the goods | 0-1 |  |
| HS Code | Harmonized Commodity Description and Coding System developed and maintained by the World Customs Organization | 0-1 | Use period (.) between the four first digits (heading) and the next two digits (sub-heading). Additional code digits may be added after the sub-heading after a new period (.). The sub-heading and associated period is optional. |
| Measurement | The physical size of the cargo item, where applicable. This should be omitted, e.g., for bulk cargo. | 0-1 |  |
| Seal number | Seal which is used for transport equipment | 0-1 |  |
| Reference to the proof of Community status by non-authorised consignor | The proof of Community status MRN shall be provided where the the person lodging the manifest is not an authorised consignor | 0-1 |  |
| Customs Status | Status of goods as identified by customs for regulation purposes or as a result of an action by the reporting party or government agency | 0-1 | EDIFACT codes (4405) |
| Cargo item - DPG details | DPG related details of a cargo item | 0-1 |  |
| Textual reference | This is the proper shipping name, completed with the technical name where appropriate, for goods under IMDG Code, or the product name for goods under IBC Code and IGC Code, or the bulk cargo shipping name for goods under IMSBC Code, or the name of oil for goods under Annex I to the MARPOL Convention. | 1 |  |
| DG classification | Indication of the classification used (IMDG, IGC, IBC, IMSBC, MARPOL) | 1 | Values: “IMDG”, “IGC”, “IBC”, “MARPOL_ANNEX1”, “IMSBC" |
| IMO hazard class | UN Hazard Code for this cargo item as defined in IMDG. It specifies the hazard code for the actual substance. Subsidiary hazard codes may be added where applicable in the "Subsidiary Risks" data items. | 0-1 | IMO Hazard class (IMDG-IBC-IGC codes) of DPG |
| UN number | United Nations Dangerous Goods Identifier (UNDG), unique serial number assigned within the United Nations to substances and articles contained in a list of the dangerous goods most commonly carried. | 0-1 | UN number of DPG. Please use ‘NONE’ if UN number not existing. |
| Packing group | Where applicable: Packing danger group code as appropriate and as defined in IMDG. The packing group for a chemical indicates the degree of hazard associated with its transportation. The highest group is Group I (great danger); Group II is next (medium danger), while Group III chemicals present the lowest hazard (minor danger). Packing groups are often shown on MSDS data sheets for chemicals under the heading "Transport Information". | 0-1 | Code as appropriate and as defined in IMDG: "I", "II", "III" and "None" |
| Subsidiary risks | Any risks in addition to the class to which dangerous goods are assigned; and which is determined by a requirement to have a subsidiary risk. | 0-5 |  |
| Flashpoint | The temperature in degrees Celsius at which a liquid will give off enough flammable vapour to be ignited. according IMDG Code DG Class 3 | 0-1 |  |
| MARPOL pollution code | This code applies to noxious liquid substances as defined in MARPOL, Annex II (e.g. X, Y, Z, OS) | 0-1 | The code values are those defined in MARPOL, Annex II: "X", "Y", "Z" and "OS" |
| EmS | Emergency response procedures for ships carrying dangerous goods number | 0-2 |  |
| Additional information | Any additional information regarding dangrous and polluting goods on board | 0-1 |  |
| Ship's Stores |  | 0-n |  |
| Name of article | Description of the dutiable store item that the ship carries | 0-1 |  |
| Quantity | Quantity of the ship's store item: amount and unit | 0-1 |  |
| Location on board | This is the on board location of the ship's stores item | 0-1 |  |
| Official use | Description indicates the purpose of official usage of each article of the ship's store item. | 0-1 |  |
| Waste |  | 0-1 |  |
| Last port delivered | Last port where ship-generated waste was delivered | 0-1 | The port is identified by its 5-digit LOCODE |
| Last port delivered date | Last date when ship-generated waste was delivered | 0-1 |  |
| Waste delivery type | If ship delivers all, some or none of its waste in the port it reports to | 1 | Possible values: "All", "Some" or "None" |
| Accurate and correct details | Confirmation that the above details are accurate and correct. This is a yes/no data element. | 1 | Yes/ No status code |
| Sufficient onboard capacity | Confirmation that there is sufficient dedicated onboard capacity. This is a yes/no data element. | 1 | Yes/ No status code |
| Waste disposal information |  | 0-n |  |
| Waste type | Type of waste | 1 |  |
| Waste type code | Code of waste type | 1 | Code as defined in Waste BR |
| Waste type description | Free text description of waste type | 0-1 |  |
| Amount to be delivered | Quantity in cubic metres (m3) of waste to be delivered in port. This can be an estimate. | 1 |  |
| Maximum dedicated storage capacity | Maximum dedicated storage capacity for this type of waste in cubic metres (m3). | 0-1 |  |
| Amount retained on board | Quantity in cubic metres (m3) of waste to be retained on board | 0-1 |  |
| Port of delivery of remaining waste | Name of port where remaining waste will be disposed of | 0-1 | The port is identified by its 5-digit LOCODE |
| Estimated amount of waste to be generated | Estimate amount of waste to be generated between notification and next port of call in cubic metres (m3). | 0-1 |  |
| Waste delivery receipt | Waste delivery receipt | 0-1 |  |
| Location or terminal name | Location or terminal name | 1 |  |
| Reception facility provider | Reception facility provider(s) | 1 |  |
| Treatment facility provider | Treatment facility provider(s) if different from above | 0-1 |  |
| Waste discharge from | Waste discharge data and time - from | 0-1 |  |
| Waste discharge to | Waste discharge data and time - to | 0-1 |  |
| Waste received | Type and amount of waste received | 0-n |  |
| Waste type | Type of waste received | 1 |  |
| Waste type code | Code of waste type | 1 | Code as defined in Waste BR |
| Waste type description | Free text description of waste type | 0-1 |  |
| Quantity received | Quantity of waste received in cubic metres (m3) | 1 |  |
| Security |  | 0-1 |  |
| CSO name | Full name of the Company Security Officer | 1 |  |
| Given name |  | 0-1 |  |
| Family name |  | 1 |  |
| CSO contact details | Company Security Officer's 24 hour contact details: phone, fax, e-mail | 1 |  |
| Phone |  | 0-1 | with an international prefix code |
| Fax |  | 0-1 | with an international prefix code |
| Email |  | 0-1 |  |
| Valid ISSC | Does the ship have a valid International Ship Security Certificate (ISSC)? This is a yes/no data element. | 1 | Yes/ No status code |
| Reason for no valid ISSC | The reasons why the ship has no valid ISSC | 0-1 |  |
| ISSC |  | 0-1 |  |
| ISSC Type | To indicate if the ship is provided with an International Ship Security Certificate or an Interim International Ship Security Certificate | 1 | Possible values: "Full" or "Interim" |
| ISSC issuer type | Type of organization (Administration or Recognized Security Organization) | 0-1 | Possible values: "GVT" or "RSO" |
| ISSC issuer | Name of the issuing body. | 1 |  |
| ISSC expiry date | Date indicating when the ISSC will be expired | 1 |  |
| Approved security plan on board | This is a yes/no data element. Indicates if the ship has an approved security plan on board | 1 | Yes/ No status code |
| Current ship security level | Ship's current security level according to the ISPS provisions | 1 | The security levels according to the ISPS code: "SL0", "SL1", "SL2" and "SL3" |
| Last calls at port facilities | Security related information regarding last calls at port facility (to be appended to list in voyage group) | 0-n |  |
| Security level | Port's security level according to ISPS provisions | 0-1 | The security levels according to the ISPS code: "SL1", "SL2" and "SL3" |
| Special or additional security measures | Special or additional security measures taken by the ship during call | 0-1 |  |
| Ship-to-ship activities | Ship-to-ship activities, in chronological order (most recent first), which were carried out during the last 10 calls at port facilities | 0-n |  |
| Location | Location as a port, a geographic position and/or a named location | 1 |  |
| Locode |  | 0-1 | LOCODE |
| Latitude |  | 0-1 | minInclusive value="-90" and maxInclusive value="90" |
| Longitude |  | 0-1 | minInclusive value="-180" and maxInclusive value="180" |
| Location name |  | 0-1 |  |
| Date from |  | 1 |  |
| Date to |  | 1 |  |
| Activity | Description of ship-to-ship activity performed | 1 | EDIFACT codes (8025) |
| Security measures | Security measures applied in lieu | 0-1 |  |
| Security related matter to report | Security related matter to report, if any | 0-1 |  |
| Number of persons on board |  | 0-1 |  |
| Number of persons on board | Total number of persons on board the ship | 1 | The value 0 (Zero) is not allowed. Note that the type “INT” prohibits the use of dots and commas |
| Number of passengers | Total number of passengers on board the ship | 0-1 |  |
| Number of crew | Total number of crew on board the ship | 0-1 |  |
| Stowaways | Have any stowaways been found on board? This is yes/no data element | 1 | Yes/ No status code |
| Passengers |  | 0-n |  |
| Number | Sequence number of passenger | 1 |  |
| Family name | Family name or surname as in passport or valid ID Document | 1 |  |
| Given name | Given name or first name as in passport or valid ID Document | 1 |  |
| Gender | Gender | 0-1 |  |
| Nationality | The stated or factual country of citizenship shown on the crew's passenger’s identification document. | 1 | The Alpha-2 code (two-digits flag code) in accordance with the standard ISO 3166-1. |
| Date of birth | Date of birth | 1 |  |
| Place of birth | Place of birth, city name or similar | 1 |  |
| Country of birth | Country of birth | 1 | The Alpha-2 code (two-digits flag code) in accordance with the standard ISO 3166-1. |
| Nature of identity document | The types of documents are muster book, passport or other legal identity card with picture. If none of this is available, other shall be used. | 1 | ?? Issue of coding: no match between ISO28005 and WCO |
| Number of identity document | Registration number of the specified document | 1 |  |
| Port of embarkation | Port where the passenger embarked the ship | 1 | The port is identified by its 5-digit LOCODE |
| Embarkation date | Date when the passenger embarked the ship | 0-1 |  |
| Port of disembarkation | Port where the passenger disembarked the ship | 1 | The port is identified by its 5-digit LOCODE |
| Transit | If the passenger is a transit passenger in this port call, i.e., if the passenger enters the ship again before departure. This is a yes/no data element | 1 | Yes/ No status code |
| Visa/Residence Permit number | Visa number or Residence Permit number, as applicable | 0-1 |  |
| Crew |  | 0-n |  |
| Number | Sequence number of crew | 1 |  |
| Family name | Family name, name | 1 |  |
| Given name | Given name, first name, surname | 1 |  |
| Gender | Gender | 0-1 |  |
| Duty of crew | Crew duty or rank | 1 | ?? Issue of coding: no standard list |
| Nationality | The stated or factual country of citizenship shown on the crew’s identification document. | 1 | The Alpha-2 code (two-digits flag code) in accordance with the standard ISO 3166-1. |
| Date of birth | Date of birth | 1 |  |
| Place of birth | Place of birth, city name or similar | 1 |  |
| Country of birth | Country of birth | 1 | The Alpha-2 code (two-digits flag code) in accordance with the standard ISO 3166-1. |
| Nature of identity document | The types of documents are muster book, passport or other legal identity card with picture. If none of this is available, other shall be used. | 1 | ?? Issue of coding: no match between ISO28005 and WCO |
| Number of identity document | Registration number of the specified document | 1 |  |
| Visa/Residence Permit number | Visa number or Residence Permit number, as applicable | 0-1 |  |
| Embarkation date | Date when the crew member embarked the ship | 0-1 |  |
| Crew's Effects |  | 0-n |  |
| Number | Sequence number of crew | 1 |  |
| Crew’s effects | Effects ineligible for relief from customs duties and taxes or subject to prohibit ions or restrictions (e.g. wines, spirits, cigarettes, tobacco, etc.). | 0-1 |  |
| Health |  | 0-1 |  |
| Valid Sanitation Control Exemption or Control Certificate | Valid Sanitation Control Exemption or Control Certificate carried on board? It is either "No", or "Sanitary Control Exemption" or "Sanitary Control Certificate" | 0-1 | Yes/ No status code |
| Place of issue | Location where the Sanitation Control Exemption or Control Certificate was issued. | 0-1 |  |
| Date of Issue | Date when the Sanitation Control Exemption or Control Certificate was issued. | 0-1 |  |
| Re-inspection required | Whether re-inspection is required. It is a yes/no data element | 0-1 | Yes/ No status code |
| Visited infected area | Has ship/vessel visited an infected area identified by the World Health Organization? It is a yes/no data element | 0-1 | Yes/ No status code |
| Port of call in infected area | If ship visited an infected area identified by WHO, this is the port of call in that infected area | 0-1 | If No to previous question, provide details The port is identified by its 5-digit LOCODE |
| Date of call in infected area | If ship visited an infected area identified by WHO, this is the date of the call in that infected area | 0-1 |  |
| Any person died | Has any person died on board during the voyage otherwise than as a result of accident? This is a yes/no data element | 0-1 | Yes/ No status code |
| Number of deaths | Total number of deaths | 0-1 | If Yes to previous question, provide number |
| Disease on board | Is there on board or has there been during the international voyage any case of disease which you suspect to be of an infectious nature? This is a yes/no data element | 0-1 | Yes/ No status code |
| Ill persons greater than expected | Has the total number of ill passengers during the voyage been greater than normal/expected? This is a yes/no data element | 0-1 | Yes/ No status code |
| Number of ill persons | Number of ill persons during the voyage | 0-1 |  |
| Ill persons now | Is there any ill person on board now? This is a yes/no data element | 0-1 |  |
| Medical consulted | Was a medical practitioner consulted? This is a yes/no data element | 0-1 | Yes/ No status code |
| Infection condition on board | Are you aware of any condition on board which may lead to infection or spread of disease? This is a yes/no data element | 0-1 | Yes/ No status code |
| Sanitary measure applied | Has any sanitary measure (e.g. quarantine, isolation, disinfection or decontamination) been applied on board? This is a yes/no data element | 0-1 | Yes/ No status code |
| Sanitary measure | Sanitary measure | 0-n |  |
| Type of sanitary measure | Type of sanitary measure | 0-1 |  |
| Place of sanitary measure | Place of sanitary measure | 0-1 |  |
| Date of sanitary measure | Date of sanitary measure | 0-1 |  |
| Location stowaways joined ship | Location where the stowaways is assumed to have joined the ship, if any. | 0-1 |  |
| Sick animal | Is there a sick animal or pet on board? This is yes/no data element | 0-1 | Yes/ No status code |
| Health - MDH Attachment |  | 0-n |  |
| Number | Sequence number of crew or passenger | 1 |  |
| Crew or passenger | Whether the person is a crew or a passenger | 1 |  |
| Illness | Nature of illness | 0-1 |  |
| Symptoms date | Date of onset of symptoms | 0-1 |  |
| Reported to port medical | Reported to a port medical officer? This is a yes/no data element | 0-1 |  |
| State | Whether the person recovered, is still ill or died | 0-1 |  |
| Case Disposal | Whether the person is still on board, was evacuated, or was buried at sea | 0-1 |  |
| Location of evacuation | Name of the port or airport where person was evacuated | 0-1 |  |
| Treatment | Drugs, medicines or other treatment given to patient | 0-1 |  |
| Comments | Comments on the specific case in the MDH attachment | 0-1 |  |
| Remarks |  | 0-1 |  |
| General remarks | This is a human-readable general remark to be optionally made by the master should the latter consider it appropriate to do so. | 0-1 |  |
| Bunkers remaining onboard |  |  |  |
| Quantity of heavy fuel oil | Quantity of heavy fuel oil, in metric tonnes | 0-1 |  |
| Quantity of gas oil | Quantity of gas oil, in metric tonnes | 0-1 |  |
| Quantity of marine gas oil | Quantity of marine gas oil, in metric tonnes | 0-1 |  |
| Quantity of marine diesel oil | Quantity of marine diesel oil, in metric tonnes | 0-1 |  |
| Quantity of any other type of oil | Quantity of any other type of oil, in metric tonnes | 0-1 |  |
| Bunker delivery receipt | Bunker delivery receipt notes of the above quantities is attached | 0-1 |  |
| Civil Liability Certificate for Oil Pollution Damage | Certificate of insurance or other financial security in respect of civil liability for oil pollution damage, issued in accordance with the provisions of article VII of the International Convention on Civil Liability for Oil Pollution Damage/1992 |  |  |
| Status | Certificate status. Use "Other" if ship has other financial security or insurance | 0-1 |  |
| Expiry date |  | 0-1 |  |
| Comment | If other financial security or insurance, specify here | 0-1 |  |
| Civil Liability Certificate for Bunker Oil Pollution Damage | Certificate of insurance or other financial security in respect of civil liability for bunker oil pollution damage, issued in accordance with the provisions of article 7 of the International Convention on Civil Liability for Bunker Oil Pollution Damage/2001 |  |  |
| Status | Certificate status. Use "Other" if ship has other financial security or insurance | 0-1 |  |
| Expiry date |  | 0-1 |  |
| Comment | If other financial security or insurance, specify here | 0-1 |  |
| Ship defects | Problems that could cause significantly reduced manoeuvrability |  |  |
| Hull integrity | Any problems with hull or superstructure that may cause operational problems | 0-1 |  |
| Manoeuvrability | Defects in hull, steering or propulsion systems that affect manoeuvrability | 0-1 |  |
| Mooring | Anchor or mooring systems problems | 0-1 |  |
| Cargo handling | Problems that may impact loading or unloading performance | 0-1 |  |
| Communication | Problems in communication systems | 0-1 |  |
| Navigation | Problems with bridge and/or navigation systems | 0-1 |  |
| Other | Other systems | 0-1 |  |

QUOTATION: IALA WORKSHOP

QUOTATION: IALA WORKSHOP ON SHORE-BASED MARITIME SERVICES FOR THEORY TO PRACTICAL SERVICES Working  - Group 1 statement of work – file: SBMS1-2.5 WG1 statement of work

1.1	MS8 Vessel shore reporting

1.1.1	Definition

“...    	3.8	MS8 Vessel shore reporting

3.8.1	Definition

The aim of vessel shore reporting is to safeguard traffic at sea, ensure personnel safety and security, ensure environmental protection and increase the efficiency of maritime operations.

The aim of vessel shore reporting is to safeguard traffic at sea, ensure personnel safety and security, ensure environmental protection and increase the efficiency of maritime operations.

Automated ship reporting is one of the most important solutions to reduce the Mariners workload (amount of time spent on preparing and submitting reports to shore-based authorities). To achieve this, reports should be automatically generated as much as possible from on-board systems. Some of the ways the administrative burden of vessel shore reporting can be reduced are:

•	Single-entry of reporting information into  ICT collection tools that store it in a repository and ICT tools that assists with the generation all required reports from this repository.

•	Automated collection of information from ship-board systems that is required for reporting (for example Ballast Management System, Emissions Control System, Waste Management System, Navigation  System, etc., etc.)

•	ICT tools that allow mariners to delegate to shore-based personnel (at the discretion of the ship’s owner/operator) the tasks of information collection, generation and submittal of required reports

•	Reduce the administrative burden by encouraging all national reporting requirements to use standardized digital reporting formats based on the S-200 Product Specification of the Common Maritime Data Structure

•	Automated or semi-automated digital distribution/communication of required reports

…”

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain