Input paper:  ENAV19-n.n.n

Input paper for the following Committee(s):					Purpose of paper:

□	ARM	□  ENG		□  PAP					  Input

- ENAV	□  VTS							□  Information

Agenda item			3.1

Technical Domain / Task Number	WG4 / 5.1.9 MSP8 Vessel Shore Reporting

Author(s) / Submitter(s)			Prof. Yu Yung-Ho, Fred Pot

Proposal for Attributes of a Ship Reporting Data Model

Summary

This paper describes a proposal for the list of attributes of a Data Model that will enable automation of Ship Reporting (“Vessel Shore Reporting”). Attributes are also known as the Data Directory or the list of database fields of the Data Model. The other elements that are required to complete the design of a Data Model are 1) identification of the entities of the model and 2) their inter-relationships. Although these are inferred in the proposed list of attributes they have yet to be formally identified in an S-2XX Product Specification.

The proposed list expands on the list of attributes that was proposed for MS8 Vessel Shore Reporting in the draft Guideline for Maritime Service Portfolios in IALA Document ENAV18-12.15 Draft IALA Guidelines on MSPs. That list limited report generation to IMO FAL Convention and National Single Window reports.

The proposed list of attributes (See ANNEX 1) was developed by Prof. Yu Yung-Ho at the request of the Chair of WG4 of the ENAV Committee to modify the initial list to allow generation of not only the IMO FAL Convention and the National Single Window reports but also all other reports required by shore-based authorities. Please note that the proposed list of attributes adds “Sub-Category” and “Data Type” classification columns and that the properties (category name, attribute name, attribute category, etc.) of some of the attributes were re-classified and/or clarified.

Purpose of the document

The ENAV Committee is invited to consider adoption of the proposed list of attributes for the Ship Reporting Data Model as an IALA Standard.

Related documents

ENAV18-12.12 Automated Ship Reporting v1.1

ENAV18-12.15 Draft IALA Guidelines on MSPs

ENAV18-12.6 MSP8 Vessel Shore Reporting

Background

IMO’s Sub-Committee on Navigation, Communications and Search and Rescue (NCSR) 1st session in March 2014 adopted “S2: means for standardized and automated reporting” as a prioritized solution. Task T9 of the e-navigation Strategic Implementation Plan (SIP) specifically lists “Investigate the best way to automate the collection of internal ship data for reporting including static and dynamic information” in 2016.

Discussion

Automated ship reporting systems are expected to significantly reduce the administrative burden on the bridge team to submit reports to shore-based authorities. ICT solutions that will automate ship reporting, i.e. data collection and report generation, will use a database.

An international standard (S-2XX Product Specification) for the structure of the ship reporting database will need to be adopted to create an “Eco System” that allows vendors of data collection and report generation solutions to thrive. If each vendor were to adopt a proprietary database structure to support their solutions then the market for such solutions will be fragmented and interoperability between systems will be non-existent, yet, interoperability is a crucial for adoption of automated ship reporting systems by the maritime industry because it involves stakeholders with a variety of requirements for data sources, language, formatting, business rules and work flows.

Examples of reports that ships are required to submit to shore-based authorities are provided in ANNEXES 2, 3 and 4. All of these can be accommodated with customized ICT solutions (“Apps”) that use a single international standard for the structure of the ship reporting database (i.e. Data Model).

Adopting an international standard for the structure of the ship reporting database will allow stakeholders, including shipping lines, to mix-and-match data collection and report generation ICT solutions from a variety of vendors and it will, eventually, allow shore-based authorities to, with proper authorization, query a single/distributed, cloud-based, database to download relevant information into their own system (see “Scenario 3” described in ENAV18-12.12 Automated Ship Reporting v1.1) without having to deal with a plethora of database structures (i.e. attributes, entities and their inter-relationships).

The proposed list of attributes supports generation of all reports that ships are required to submit to shore-based authorities, whatever their form, format, language, etc.

The proposed list of attributes is offered as a possible standard to be used in the development of an S-2XX Data Model for Automated Ship Reporting.

References

e-Navigation Strategy Implementation Plan (SIP) NCSR1/28, Annex 7

Strategy for the development and implementation of e-navigation MSC85/26 Annex 20

Action requested of the Committee

The ENAV Committee is invited to consider adoption and maintenance of the automated ship reporting database structure as an IALA standard.

- ANNEX 1

- Ship Reporting Data Model Attribute Specification

- ANNEX 2

- Sample Reports

- ANNEX 3

- Tokyo Arrival DECLARATION

ANNEX 4

- BALLAST WATER OPERATIONS LOG