Input paper:    DTEC2-n.n.n

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					□  Input

X  DTEC	□  VTS							☑   Information

Agenda item 			n.n

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			……China MSA……………

Updated and Supplement Information for the Review of Ships’ Air Draft Remote Measurement Technology (SADRMT)

Summary

The document ENAV30-5.1.2.4 has provided prototype testing information on the ships’ air draft remote measurement technology. This paper additionally provides progress information on the technology application and give some responses to those technical review questions.

It is worth noting that SADRMT is a combined utilization technology concept of existing equipment, and its core computer control programs and algorithms cannot be effectively presented in this document. This concept would potentially be used in fields such us VTS operation, bridge management, etc. This paper presents the application case of Zhoudai Bridge in China.

Purpose of the document

Provide some supplementary and updated information for the document ENAV30-5.1.2.4 and its technical review table.

Related documents

ENAV30-5.1.2.4 Ships’ Air Draft Remote Measurement Technology Review Using Guideline G1153 Template

Background

SADRMT is an integrated technology that combines existing equipment and performs remote measurement and monitoring of ships’ air draft based on a certain computer algorithm. A prototype system test was conducted on Zhoudai Bridge in November 2021. Due to system complexity and fund requirements, its system design and construction scheme did not receive preliminary acceptance and application permission until December 2023. Recently, the installation and deployment of system has been completed and trial operation has begun.

Discussion

3.1 Technology development, suppliers and marketization situation

In the test of the SADRMT prototype system in November 2021, Shanghai Maritime University demonstrated the performance of the system it developed (already had been presented in the document ENAV30-5.1.2.4). At the present time, since this technology does not produce any actual products and the basic principles are relatively simple, many organizations in China claim to have the capability to implement such technology based on the same principles, including Wuhan University of Technology, Shanghai Huahai Navigation Technology Co., Ltd., China Transportation Planning and Design Institute Co., Ltd., etc.

The equipment supplier of the technology application project of Zhoudai Bridge is Hikvision, the software and control system supplier is Zhejiang Future Transportation Industry Development Co., Ltd., and the project implementation plan design is provided by Zhejiang communication construction Group Co., Ltd.

Hikvision, with its mature equipment production and technical strength, has occupied a large share of the Chinese market in the promotion and application of this technology.

3.2 Application project of Zhoudai Bridge

Due to the complexity of the system and the funding requirements for project implementation, the system design and deployment plan for Zhoudai Bridge did not receive preliminary acceptance and installation permission until December 2023. The system equipment has recently completed installation and debugging, and trial operation has begun. Sufficient trial operation data has not yet been generated to evaluate the overall operating performance of the system.

The system trial operation data shows that within a range of 6 kilometres, the measurement error is less than or equal to 1 meter, and within a range of 3 kilometres, the measurement error is less than 0.5 meters. Some trial operation data recorded on January 20th, 2024 are shown in Table 1.

Table 1 Trial operation data

The local VTS authorities have not yet accessed the system data, however, in view of the potential, they will access in the near future and use it as one of the information bases to provide traffic organization services in bridge waters.

Accosrding to the characteristics of the system and the size of vessels in the water area, the area between 5000m and 3000m on both sides of the bridge centreline was set as monitoring area. The system will quickly capture and identify the status of ships entering this water area, and dynamically tracks ships that will enter the bridge water. The area between 3000m and 1500m on both sides of the bridge centreline was set as warning area level III. The area between 1500m and 1000m on both sides of the bridge centreline was set as warning area level II. An emergency alarm area is located within 1000m on both sides of the bridge centreline. The schematic diagram is shown in Figure 1.

Figure 1 Front-end deployment diagram

In view of the scalability of the system, in the field application of Zhoudai Bridge, the bridge manager also customized various types of optical sensors, meteorological sensors, loudspeakers etc. The installation of bridge on-site equipment is shown in Figures 2, 3, 4, and 5, and the user interface of the system is shown in Figures 6 and 7. It is worth noting that the user interface is still being optimized based on trial feedback.

Figure 2 Radar and AIS terminal                 Figure 3 Electro-optical sensor on the bridge

Figure 4 Electro-optical sensor on the bottom         Figure 5 Oriented optical sensor and loudspeaker

Figure 6 User interface

Figure 7 Target recognition and capture

3.3 Legal and compliance issues related to technology applications

Up to now, this technology has not been widely used in practical applications and is still in the stage of exploration and trial application. No legal issues have been found in its application on Zhoudai Bridge, and it has not yet been subject to quality supervision and certification by a qualified third party. However, it is undeniable that considering the different legal and regulatory systems of different countries, the application of emerging technologies should be specifically analyzed and evaluated based on the local legal and regulatory environment.

References

- N/A

Action requested of the Committee

The committee is kindly invited to note the above information, and take actions as appropriate.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain