Input paper:   DTEC2-5.2.3.6

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					□  Input

X  DTEC	□  VTS							x  Information

Agenda item 			n.n

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			Maritime and Port Authority of Singapore

Agency for Science, Technology and Research - Institute for    Infocomm Research - (A*STAR-I2R)

# VDES Development in Singapore

# Summary

International shipping is the life blood of the global economy and is responsible for transporting about 90% of world trade. To ply the shipping routes safely and efficiently, shipping needs reliable and secure ship-ship and ship-shore communications, especially in open seas. Very High Frequency (VHF) Data Exchange System (VDES[1]) is a new maritime digital data exchange system that is globally interoperable and available for two-way ship-ship and ship-shore communications. VDES has a satellite component that is able to provide world-wide coverage in support of the IMO’s e-Navigation strategy.

The Maritime and Port Authority of Singapore (MPA), ST Engineering, and A*STAR-I2R of Singapore have invested in the development of the Automatic Identification System (AIS), Application Specific Message (ASM) and VDES modules over the past few years. Since 2016, A*STAR-I2R has been working on the development of a VDES-capable device that supports AIS, ASM and VDES standards for terrestrial and satellite requirements. An initial prototype VDES terrestrial payload, codenamed VDES-1, was conceived and has undergone testing for AIS, ASM and VDES for terrestrial requirements. As part of VDES satellite capability build up, A*STAR-I2R has also developed a VDES satellite payload, codenamed L4-PAYL, and launched it into the 10o inclination Equatorial Orbital Satellite LUMELITE-4 (L4). The L4 was successfully launched to its intended orbit on 22 April 2023. The VDES-1 and L4-PAYL are currently undergoing testing and evaluation of VDE, AIS, and ASM capabilities through sea trials and In-Orbit Trials.

This paper presents the details about the design, current capabilities and reports of the initial test trials done for the VDES-1 and L4-PAYL. The presentation of the details is structured as follows:

- Background

- VDES R&D Roadmap

- Tests, Trials and Results for VDES-1 and L4-PAYL

- Conclusions

- References

- Action Requested of the Committee

## Purpose of the document

Information Paper

## Related documents

Nil.

# Background

The International Association of Lighthouse Authorities (IALA) has defined the specifications for the development of the VDES in the VHF frequency band 156-162 MHz. VDES addresses the increased congestion of the AIS and supports extended capabilities for more applications usage scenarios with the inclusion of a satellite component. The MPA, along with research and industry partners in Singapore, has embarked on the journey to develop and demonstrate the VDES terrestrial and satellite capabilities.

VDES consists of various components, in particular the VDE Terrestrial (VDE-TER) and VDES Satellite (VDE-SAT) components, that need to be developed in order to support the usage scenarios defined. VDE-TER and VDE-SAT systems developed by A*STAR-I2R have incorporated AIS, ASM, VDE channels in both the upper and lower frequencies as part of the system design.

A*STAR-I2R’s VDE-TER and VDE-SAT Systems Implemented Design

The VDE-TER system, in its current iteration of VDES-1, is a 2nd generation miniaturized VDES multi-channel transceiver. Several sea trials have been conducted between July 2021 and February 2022 and the results are presented in this paper. The sea trial results include the receiver sensitivity performance as well as the packet error rate (PER) versus the signal-to-noise ratio (SNR) performance for the VDE, ASM and AIS channels. These performances were measured under single-channel transmission and multiple-channel transmission scenarios. The results measured previously using the single-channel transceiver were used as a benchmark for comparison.

VDE-TER Implementation Support

The VDE-SAT system, in its current iteration of L4-PAYL, is a VDES multi-channel transceiver supporting VDE Link IDs 20, 21, 32 and 33. A standalone function of AIS also is supported and currently undergoing testing and evaluation. The L4-PAYL was successfully launched at a 10o inclination equatorial orbital on 22 April 2023. Mission trials were conducted successfully to transmit VDE messages from Ground to Satellite and the L4-PAYL was able to establish link and receive messages using Link IDs 20 and 21. The VDES-1 was used for terrestrial ground transmission. Link performance evaluation of VDE Link IDs 20 and 21 is currently still in progress. Frequency filing and ITU coordination efforts are currently in progress for the support of downlink transmission with Link IDs 32 and 33. The mission trial results and next steps in the evolution of the satellite payload system are presented. These can form the baseline for the future evaluation of other link IDs.

L4-PAYL Implementation Support

# VDES R&D ROADMAP

Singapore VDES R&D Roadmap

Figure 4 shows the proposed roadmap for the development and implementation of the VDES system in Singapore, which has started since 2016 with the development of the VDE-TER ship terminal, i.e. VDES-1. The terrestrial testing of the VDES-1 prototype was successfully completed in 2021. The VDES satellite terminal and 12U satellite bus development started in 2018 and was completed and launched in April 2023. Currently, in-orbit testing of the various satellite link IDs is underway and is expected to be completed by mid-2024. There are also plans to develop and launch a 1U VDES satellite terminal. This will be followed by the lab demonstration of the identified VDES use cases, such as ship reporting, search & rescue, intelligent routing, etc. At the same time, interoperability testing with international partners will commence and is expected to last till 2026. The next phase could involve the integration of VDES with other communication systems such as Satcom, 4G/5G, LTE, etc. together with message forwarding which might require the development of a marine mesh network. All these efforts will enable Singapore to be ready for VDES commercialisation when the VDES standards are expected to be finalized in 2028.

# TESTS, Trials and Results for VDES-1 and L4-PAYL

The VDE-TER and VDE-SAT designs are based on a common FPGA-based physical layer architecture that integrates the functions of the VDE, ASM, AIS in the VHF maritime mobile band (156.025-162.025 MHz). The transceiver design can support VDE, AIS1, AIS2, ASM1 and ASM2 bands. VDES MAC protocol based on 20200320_Revisions_to_G1139 Rev_3.docx is supported including the IEC61162-450 Presentation Interface Layer specification. This section presents the design and usage scenarios those that can be demonstrated or in development using the VDES-1 and L4-PAYL.

Various VDE usage scenarios were conceived and the corresponding trials were conducted to evaluate the performance of the VDES-1 and L4-PAYL.

VDE-TER and VDE-SAT Usage Scenarios

## VDE-TER System and Sea Trials

The VDE-TER transceiver supports AIS1, AIS2, ASM1, ASM2 and VDE channels 11 to 19. Several sea trials were conducted with the objective of evaluating the link performance for PER and SNR.

### VDE-TER Sea Trial

The functionality of the VDE, ASM and AIS channels were tested with single channel transceiver subsystem for the baseline reference of the performance. After the baseline tests and the corresponding performances were satisfactory the RF subsystems were integrated, and the complete VDES multi-channel transceiver was again tested and measured.

VDE-TER Sea Trial Setup

Antenna Mounted on the Ship

Location Map for the VDE-TER Sea Trials

A VDES-1 transmitter was installed on Ship A, where the signals were transmitted at 6W. Concurrently, two VDES-1 receivers were installed on Ship B. One of the receivers was configured with the Automatic Gain Control (AGC) locked while the other receiver was configured with the AGC unlocked. The setup of the transmitter and receivers can be seen in the photos below.

VDES-1 Transmitter Setup on Ship A

VDES-1 Receiver Setup on Ship B

### VDE-TER Receiver Sensitivity

The sensitivity of the VDES multi-channel receiver, which indicates the received power level of the signal required to achieve a certain level of PER, was measured for the VDE, ASM and AIS channels. The results for the VDE channels are shown below for Link IDs 11-19, respectively. The sensitivity of the multi-channel receiver is largely better than that of the single-channel receiver.

With reference to [1] Section B 2.2 Table 17, the measured results from ship-to-ship sea trials have shown that the achieved sensitivity has exceeded the defined specifications.

Receiver Sensitivity for VDE Link IDs

### PER vs SNR Results for VDE-TER

The PER vs. SNR performance of the multi-channel transceiver (MCH) were measured, first without the RF subsystem, and then subsequently integrated with the RF subsystem. The results were compared against the previous single-channel transceiver (SCH). The results of the PER vs. SNR performance for VDE channels were presented in Figure 12 for Link IDs 11-19, respectively. The performances for single-channel transmission (VDE-only) as well as for multiple channel transmission (VDE+ASM, VDE+AIS, VDE+ASM+AIS) were measured and compared. Note that multiple channel transmissions will be invoked only when there are no empty slots available for selection and occupied slots must be selected instead (adhering to certain rules as stipulated in the VDES Standards). This implies that the traffic is extremely heavy. Under such extreme heavy traffic conditions, it is highly unlikely that 16QAM of the VDE channel will be used. Hence, the measurements under multiple channel transmission for the 16QAM was not conducted.

Reference [1] Section B Table 17 refers the Receiver sensitivity and characteristics to 20% PER @-107dBm signal power. Below results from Ship-to-Ship sea trials represent the Sensitivity confirms better from the defined specifications.

PER vs SNR Performance for VDE Link IDs

It can be observed that the performance of the VDES multiple-channel transceiver is relatively consistent, regardless of the number of channels invoked. In other words, the performance with only the transmission of the VDE signal is like that when there are multiple channel transmissions, for example VDE + ASM signals and so on. Furthermore, the performance of the multi-channel transceiver is also like that of the previous single-channel transceiver. Hence there is no performance loss despite upgrading to the more versatile multi-channel transceiver.

### VDE-TER Sea Trial – Shore to Ship

Bedok Lighthouse as in the map shown was chosen for the transmitter location. A MPA vessel transiting from the lighthouse to a distance of 20km until Eastern Buoy and back was chosen for the receiver.

Shore to Ship Test Methodology

Shore Station Setup at Lighthouse in Bedok

Location Map for the Trial

### PER vs Distance and PER vs SNR Results for VDE-TER

As part of test methodology, the measurements were done for the PER and SNR with the ship moving in Singapore waters within a range of up to 20km between Singapore Bedok Lighthouse and Eastern Buoy. The results presented below is PER vs Distance and PER vs SNR for the VDE-TER trials.

Reference [1] Section B 2.2 Table 17 refers the Receiver sensitivity and characteristics to 20% PER @-107dBm signal power.

Performance Plots for VDE-TER Link IDs 11-19

## VDE-SAT System

VDE-SAT [1] [2] is a platform consisting of:

- A satellite payload system with AIS, VDE transceiver

- RF synthesizer onboard payload with antenna

- VDE terrestrial mobile station system with AIS, ASM, VDE transceiver with RF system.

The L4 satellite payload implementations supported are uplink Link IDs 20 and 21, and downlink Link IDs 32 and 33. The AIS receiver is also integrated. Besides satellite payload, on-ground mobile station VDE-TER is also implemented in FPGA and used for mission trials.

VDE-SAT System

## VDE-SAT Test Trials and Results

L4-PAYL was qualified in the lab with the support for AIS and VDE Link IDs 20, 21, 32, 33 and subsequently launched into orbit. Several mission trials are in progress now.

### VDE-SAT Lab Trials

Comprehensive in-lab software and hardware integration testing was done with satellite payload and VDE-TER mobile stations, with the setup shown in Figure 18.

VDE Lab Test System

The detailed test results are shown in Figure 19 and 20 which includes uplink Link IDs 20 and 21 and downlink Link ID 33. For uplink ID20 and ID21, the results showed that the receiver on the satellite payload produced  10% PER at low received power of -130dbm and -115.5dbm respectively. For downlink ID33, the results revealed that the VDES receiver payload produced 10% PER at low received power of -106.5dbm. Hence, the VDES receiver payload of L4-PAYL is capable of achieving PER of 10% at low signal power.

VDES Satellite Uplink Test Results

VDES Satellite Downlink Test Result

Frequency filing was done for the uplink/downlink at three different frequency bands and the spectrum utilization was recorded as below.

With reference to [1] Section A.1.2.11, the measurement results showed that the VDE-1 and VDE-SAT devices can conform to the spectrum utilization specifications.

Approved Frequencies for the Trial

| Centre Freq (MHz) | Bandwidth (kHz) | Channel | Ground to Space / Space to Ground |
| --- | --- | --- | --- |
| 161.9750 | 25 | AIS1 | Ground to Space |
| 162.0250 | 25 | AIS2 | Ground to Space |
| 161.8625 | 50 | VDE 2025, 2085 | Ground to Space |
| 161.8625 | 50 | VDE 2025, 2085 | Space to Ground |

Spectrum Utilization Plot for VDE at 161.8625MHz, AIS1 at 161.975MHz and AIS2 at 162.025MHz

### VDE-SAT In-Orbit Testing

The L4 satellite was successfully deployed on 22 April 2023, 1710 hours and ground contact was subsequently established during the first pass at 1842 hours. L4-PAYL VDES antennas were deployed successfully for the VDES trials.

The L4-PAYL missions trial system components at different locations were setup to send and receive the mission configuration data, report the data via telemetry, tracking and control (TT&C) and run the missions while the ground VDE-TER terminal is transmitting VDE data. The scenarios are shown below.

- L4 ground control system infrastructure, with S-band and UHF antennas for TT&C.

- L4 mission control software and applications.

- VDE-TER mobile terminal and applications with antennas.

- Ships in the sea with commercial AIS1/AIS2 transmitters.

VDE Mission Trial & Operations Setup

AIS1/AIS2 SAT Receive Mission Trial Setup

VDE Ground Station Setup

VDE-SAT System Test Components

| Location | Purpose | Comm Type | Activity Details |
| --- | --- | --- | --- |
| Ground Station & L4 Mission Control Applications | TT&C Uplink | S-band | Mission Data Uplink, Ground Pass 1 Data Rate: up to 128Kbps |
| VDE Ground Control Station | VDE-TER Transmit | VDES Link IDs 20/21 TX | Setup VDE Ground Terminal VDE Antenna: CX4-3 Omnidirectional Dipole, 1.26m Optional VDE Antenna: CP type, Height ~2m, TX Power: 6dBW |
| L4 SAT Payload | VDES Payload Mission | VDES Link IDs 20/21 RX | VDES Antenna RX CF: 161.8625MHz, BW: 50KHz Orientation: 1m long, Circular Polarization Runs Mission <= 5mins, stores VDES report Data after mission complete @OBC, TT&C Downlink in subsequent Ground Pass |
| Ground Station & L4 Mission Control Applications | TT&C Downlink | S-band | Mission Reports VDES Data Downlink Data Rate: up to 128Kbps |

L4 TT&C applications successfully turned on the VDE-SAT payload (L4-PAYL) and missions are being carried out as of today. VDE application trials with Link IDs 20 and 21 and AIS missions are also in progress.

# Conclusions

### 5.1	VDE-TER

VDE-TER sea trials have shown that the VDES-1 device was able to conform to the VDES reference specifications. Ship-to-Ship and Shore-to-Ship trials and the associated PER and SNR results conclude that the 2nd generation VDES-1 in the current iteration is ready to undergo more extensive interoperability testing and studies.

- Multi-channel capabilities were tested and the results conform to the reference specifications.

- PER vs SNR and PER vs Distance of all VDE channels in both the sea trials shows good result conforming to specifications.

## VDE-SAT

Several VDE-SAT mission trials are currently in progress with some success using the L4-PAYL device. The lab tests as referenced in Section 4.3.1 demonstrate :

- The VDES receiver payload of L4-PAYL is capable of achieving 10% PER at low signal power.

- Transmit power spectrum utilization and received power measurements conform to the reference specifications.

Several test missions are in progress at this moment to further validate the VDES and AIS performance of the L4-PAYL. ITU coordination and frequency filing for the downlink is ongoing. Link ID 20 for several missions were successful with VDE-TER ground terminal transmitting at power of 6dBW using the omnidirectional antenna.

# References

G1139 The Technical Specification of VDES, Edition 3, June 2019.

Technical Studies on the Satellite Component of the VHF Data Exchange System (ITU-R M.2435-0).

Technical Characteristics for a VHF Data Exchange System in the VHF Maritime Mobile Band (ITU-R M.2092-1, 02/2022).

# Action requested of the Committee

The Committee is requested to consider the information provided in this document and take actions as appropriate.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain