AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper     -     78100     Saint Germain en Laye        -       France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05 Telex 695499 ialaism f

E-mail          :         iala-aism@wanadoo.fr      Internet      :      hjttp://iala-aism.org

DRAFT

IALA Recommendation

On

Operational Procedures for Vessel Traffic Services

Edition 1

June 2004-02-06

DRAFT IALA Recommendation on Operational Procedures for Vessel Traffic Services 
February 2004

[IALA Secretariat to review and amend as required]

THE COUNCIL

NOTING the function of the Council with respect to safety of navigation, the efficiency of maritime traffic and the protection of the environment,

NOTING ALSO the International Maritime Organization Resolution A.857(20) on

Guidelines for Vessel Traffic Services (VTS), in particular paragraph 2.5.3,

NOTING FURTHER:

.1 the work program of the VTS Committee, in particular the item Generic Operational Procedure for VTS, including the use of Standard Marine Communication Phrases,

.2 the IALA VTS Manual, in particular paragraph 6.1

HAVING CONSIDERED the proposals made by the VTS Committee.

ADOPTS the Recommendation on Operational Procedures as set out in the annex

to this recommendation; and

RECOMMENDS:

.1 National Members and other appropriate Authorities providing, or intending to provide, vessel traffic services adopt this recommendation.

.2 this recommendation be promulgated to IALA Members and subsequently incorporated in the IALA VTS Manual at it’s next update.

Annex

Operational Procedures for Vessel Traffic Services

The purpose of a VTS is to improve maritime safety, efficiency of navigation, safety of life at sea and the protection of the marine environment and/or the adjacent shore area, work sites and offshore installations from possible adverse effects of marine traffic in a given area.  The VTS may also have a role to play in maritime security.

VTS Authorities are responsible for ensuring that the objectives of a VTS are met and the standards set by the Competent Authority for levels of service and operator qualifications continue to be met.  In developing procedures for use within a VTS the levels of delegated authority for VTS staff should be taken into account.  Adoption of operational procedures as part of a quality control programme approved by the Competent Authority can ensure that standards are maintained and services are consistently delivered.

Appendixes 1 and 2 have been prepared as recommendations to assist VTS Authorities in identifying key aspects that should be considered when developing operational procedures for a VTS Centre.  The list is neither mandatory nor exhaustive and should be adapted to suit individual needs.  In preparing these guidelines it is recognised that:

- The nature of the tasks and activities to be performed will depend on the capability of the VTS, the area of its jurisdiction and the services to be provided. In general these tasks and activities all involve the collecting, processing and dissemination of information. The collecting and dissemination of this information will involve both internal and external communications, while the processing of the information will be carried out within the VTS Centre itself. The level of decision-making that can be taken within the VTS Centre should be identified.

- The successful pursuit of operational objectives is dependent upon co-operation and trust among service users, VTS personnel and allied services. This can only be achieved through the reliability of the VTS information, and it is based on the assured availability, continuity and quality of the declared type and levels of service provided to all stakeholders.

The attached procedures make a clear distinction between internal and external procedures.

Internal procedures address operating instruments, interactions among the staff and the internal routing and distribution of data.

External procedures govern the interaction with users, including allied services.

A further distinction is made between routine procedures and those related to incidents such as search and rescue and environmental protection within both the Internal and External procedures.

Operating procedures, particularly those relating to external communications will assist the user in understanding information or instructions given by a VTS.

Finally, it is recommended that all operational procedures be laid down in handbooks or manuals, both printed and electronic.  These procedures should be an integral part of regular training exercises and adherence to procedures should be monitored.

* - * - * - *

Appendix 1

Overview of Internal VTS Procedures

VTS Authorities need to develop procedures for a number of routine operations internal to the VTS.  The following topics may be considered for inclusion.  The list is neither mandatory nor exhaustive and should be adapted to suit individual needs.

# Routine Procedures:

## Gathering and Recording of information

Information gathered by a VTS Authority should be stored for a period of time such that data can be retrieved for a reasonable period after an incident/accident to comply with national requirements.  This may include information obtained through:

- Communications (internal and/or external)

- Sensor data – (i.e. data used to generate the traffic image such as radar, CCTV, AIS)

- Management information data (i.e. vessel/cargo information and their movements)

- Meteorological and hydrological support

- Other sources

Adequate provision should be made for the retrieval and presentation of this information.

## Operational staff

The number of operators at any time should be based upon safe and efficient operations in the VTS area to meet the operational needs and be reflected in the manpower planning, this should include staff rotation and rest periods within any given shift.

## Equipment operation, maintenance, calibration and updating

All manuals/handbooks for equipment operation, maintenance (preventative and remedial), calibration and updating should be readily available to the appropriate personnel.

Key considerations include:

- Equipment operation, to cover all normal and emergency procedures.

- Determination of acceptable availability criteria for equipment

- Categorization and prioritization of maintenance and defects

- Calibration of all sensors within specified tolerance level.

- Updating of equipment (hardware and software) and their associated manuals/handbooks.

## nteraction with allied services

Verification of internal procedures for communications with, inter alia

- Pilots, Tugs, Agents

- Governments Agencies

- Private Organisations

## Public Relations

Procedure and policy for dealing with the media and public.  Issues that may wish to be considered include:

Lead agency for public relations

Protection of sensitive information

## Security

Procedures for security of the data transmission and storage, personnel, building and VTS structures, performed in line with local and national requirements.

## raining

Consideration should be given to procedures for regular assessments and application of appropriate measures for compliance in accordance with IALA V-103.

## Watch handover

Consideration should be given to procedures for formalising the watch handover.  This may include:

- Present traffic situation

- Expected / developing traffic situations

- Incidents and special operations (e.g. SAR or military operations)

- Environmental conditions

- Equipment performance / availability

- Status of Allied Services

- Personnel availability

- Appropriate times for watch handover

- Documenting the handover

Further guidance on procedures for watch handover can be sought from IMO Resolution on Watchkeeping (RES A.285(VIII) Recommendation on Basic Principles and Operational Guidance Relating to Navigational Watchkeeping).

## Vessel handover

Consideration should be given to procedures for vessel handover between adjacent sectors.  This may include:

- Mutual understanding of the handover

- Appropriate information communicated between operators (e.g. communication channel, vessel identification, etc)

- Documenting the handover

## Maintenance of marine publications

The VTS Authority should ensure that procedures are in place for maintaining, updating and disseminating charts (paper and electronic) and associated marine publications.

# Emergency Procedures

A VTS Authority should have contingency plan to ensure safety of VTS personnel and continuity of operations in the event of an emergency.

VTS Authorities should consider procedures to address events such as the following:

- System Failure

  - Loss of external communications

  - Loss of internal communications

  - Loss of functionality of surveillance equipment

  - Loss of port information management system

- Internal emergencies, for example fire and flood

- Forced evacuation of VTS centre

- Personnel medical emergencies

- Hostile activities

These may include:

- Remedial Action

- Callout Procedures

- Fall-back options

- Recording

- Evaluation

Appendix 2

Overview of External VTS Procedures

VTS Authorities need to develop procedures for a number of routine operations external to the VTS.  External information exchange should be standardised as much as possible, such as the use of SMCP (Standard Marine Communication Phrases).

A VTS Authority should develop procedures governing the interaction with vessels and allied services within the limits and delegation of authority and the category/categories of service being provided.

The following topics may be considered for inclusion.  The list is neither mandatory nor exhaustive and should be adapted to suit local needs.

## Pre-Arrival Information

Pre-arrival information is basic information regarding the vessel and its intent to enter the VTS area.  This may include:

- Consider the pre-arrival information required, its format and its timing.  This may include a Route Plan, ETA, Draught, Hazardous cargo, ISPS security level, etc.

- Identify communications required with approaching vessels.

- Ensure advanced information has been obtained

- Establish procedures for non-compliance

- Exchange of information with allied services

## Vessels Entering VTS Area

When a vessel enters the VTS Area procedures for the following actions should be considered:

- Establish communications and verify the identity and position of the vessel

- Identify requirements for initial information exchange.  These may include:

- Confirm Reporting Requirements

- Provide relevant traffic information

- Provide navigational / fairway information

- Establish compliance with IMO requirements (charts and publications, passage plan, mechanical defects, personnel shortfalls)

- Update information with allied services

## Vessels Transiting VTS Area

When a vessel is transiting the VTS Area procedures for the following actions should be considered:

Mandatory Participation

- Identify reporting requirements for the category/categories of service provided

- Provision of relevant information to participating vessels at regular intervals or on demand or deemed necessary by the VTSO

  - Environmental conditions

  - Traffic situation

  - Navigational conditions

  - Traffic separations/overtaking restrictions

  - Warnings and restrictions concerning the movement of traffic in the area

- Special provisions for vessels carrying hazardous cargo

- Ensure compliance with pilotage directions and any special requirements for a pilot vessel being off station

- Establish procedures for non-compliance

- Exchange/Update information on allied services

Voluntary Participation

- Consider any particular requirements to track/monitor and communicate with vessels not required to participate in the VTS

- Exchange/Update information on allied services

## Vessels at Anchor

When a vessel is at anchor within the VTS Area procedures for the following should be considered.  These may depend on the VTS capability to monitor the vessel position under prevailing conditions:

- Anchorage assignment

- Communication requirements

- Reporting requirement for vessels prior to leaving the anchorage.

- Non-compliance

- Exchange/Update information with respect to allied services

## Vessels at Berth

When a vessel is at berth within the VTS Area procedures for the following should be considered.  These may depend on the VTS capability to monitor the vessel position under prevailing conditions:

- Reporting requirements for vessels on arrival at berth

- Security requirements and/or level

- Special requirements to maintain communications watch

- Need for restrictions for other vessels passing the berth

- Reporting requirement for vessels prior to leaving the berth.

- Non-compliance

- Exchange/Update information with respect to allied services

## Vessels Departing the VTS Area

When a vessel is departing the VTS Area procedures for the following should be considered.  :

- Reporting requirement for vessels prior to departing the area

- Non-compliance

- Handover with adjacent or next VTS

## Transition between Adjacent VTS Areas

When a vessel is transiting between adjacent areas procedures for the following should be considered:

- Identify procedures for handover, vessel identification, communication procedures and vessel monitoring.

## Adverse environmental conditions

In situations of adverse environmental conditions within the VTS Area such as poor visibility, strong currents, high winds, etc procedures may be required for special provisions and restrictions that may include:

- Restriction or prohibition on movement

- Additional reporting requirements

- Separation between vessels

Special consideration may need to be given depending on vessel characteristics and local geography.

## Emergencies In the VTS Area

The services of the VTS centre should be maintained during any emergency response.

### Collision, Capsize, Sinking, Grounding, Fire On Vessel, Man Overboard

In the event of Collision, Capsize, Sinking, Grounding, Fire On Vessel, Man Overboard, procedures for the following actions may need to be considered:

- Alert MRCC

- Inform relevant regulatory authority/ies

- Inform relevant emergency services

- Act on local call-out procedures, including VTS manager

- Need for back-up VTS personnel

- Promulgate information concerning incident to vessels in VTS area

- Restrict traffic in the area

- Activate tugs and other support units if appropriate

- Ensure all recording equipment is operating correctly

### Pollution

In the event of a pollution incident procedures for the following actions may need to be considered:

- Inform relevant regulatory authority/ies

- Alert relevant Environmental Authority

- Assess scale of incident and call in specialist support as appropriate

- Promulgate information concerning incident to vessels in VTS area

- Restrict traffic in the area

### Places of Refuge

Procedures may need to be developed to meet IMO Resolution X/X/X[IALA Secretariat to insert correct resolution number before submitting to Council] and national requirements.

### Medical Emergency

In the event of a medical emergency, procedures for the following actions may need to be considered:

- Inform MRCC where medical assistance has been requested

- Inform coastal radio station for possible MEDICALL

- Consider special manouvering requirements

### Vessel Not Under Command (NUC)

In the event of a vessel NUC, procedures for the following may need to be considered:

- Promulgate information concerning incident to vessels in the VTS area

- Obtain detailed information about on board situation

- Maintain communication with vessel

- Assess vessels proximity to danger (danger to vessel itself and other traffic)

- Activate tugs and other support units if appropriate

### Terrorist Action

In the event of terrorist action, procedures may need to be considered to include alerting the responsible authority and complying with instructions provided.

### Protest Action

In the event of protest action against any vessel transiting the VTS area procedures may need to be considered to:

- Alert responsible authority

- Act on local call-out procedures, including VTS manager

- Promulgate information concerning incident to vessels in the VTS area

- Safety of ships and protestors is paramount

### Natural Disaster

In the event of natural disasters such as earthquake, tidal wave, fire, exceptional weather conditions etc, procedures may need to be considered to:

- Promulgate information accordingly to vessels in the VTS area

- Act on local call-out procedures, including VTS manager

- Inform MRCC as necessary