Draft recommendation on performance requirements for VTS radar services

(Recommendation [leave number blank, to be filled by IALA Secretariat])

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING that [to be completed by Secretariat - for example - IMO has concluded that AIS will improve the safety of navigation and the protection of the environment];

RECOGNISING ALSO that [to be completed by Secretariat - for example - there is a mandatory carriage requirement for AIS equipment on SOLAS Convention vessels that entered into force on 1 July 2002 and will be complete by [2008]];

RECOGNISING FURTHER that [to be completed by Secretariat - for example - documentation of IMO, ITU and IEC refer to the provision of an AIS shore infrastructure as part of the overall operational system];

NOTING the [as above];

NOTING ALSO that;

NOTING FURTHER that;

CONSIDERING that to be completed by Secretariat - for example.

ADOPTS the [name of document] in the annex of this recommendation; and,

RECOMMENDS that National Members and other appropriate Authorities providing marine aids to navigation services [action to be taken].

*    *    *

Annex

Performance requirements for VTS radar services

List of contents

1.	Introduction	4

1.1	Scope	4

1.2	Definitions and Clarifications	4

1.2.1	Software tools	5

1.2.2	Abbreviations	5

1.3	References	7

2.	General Provisions	8

2.1	Implementation of radar services	8

2.2	Radar Parameters	9

2.2.1	Radar frequency	10

2.2.2	Antenna parameters	10

2.2.3	Transmitter characteristics	11

2.2.4	Receiver characteristics	11

2.2.5	System losses	12

2.2.6	Signal processing, extraction and tracking	13

2.3	Radar Target Characteristics	14

2.3.1	Target properties	14

2.3.2	Radar cross-section	14

2.3.3	Characteristics of wind farms	16

2.4	Radar Range performance	18

2.4.1	Antenna height	18

2.4.2	Propagation conditions	18

2.4.3	Lobing	20

2.4.4	Shadow effect	20

2.4.5	Tide	21

2.5	Noise and clutter affecting radar performance	21

2.5.1	Thermal noise	21

2.5.2	Surface and volume clutter	22

2.5.3	Ice	22

2.5.4	Precipitation	23

2.5.5	Man-made noise	23

2.5.6	Multiple reflections (ghost images)	23

2.6	Target discrimination and position accuracy	24

2.6.1	Range resolution	24

2.6.2	Azimuth resolution	24

2.6.3	Range accuracy	25

2.6.4	Azimuth accuracy	25

2.7	Security implications	25

3.	Recommendation on performance standards for VTS radar services	26

3.1	General requirements	26

3.2	Objects to be detected	27

3.3	General features and characteristics	27

3.3.1	Clutter and noise reduction facilities	28

3.3.2	Side lobe suppression	28

3.3.3	Dynamic characteristics	28

3.3.4	Built-in test features	29

3.4	Radar coverage and detection performance	29

3.4.1	Detection range and conditions	29

3.5	Target Discrimination	33

3.6	Plot Extraction and Tracking	35

3.6.1	Plot Extraction	35

3.6.2	Track Initiation	35

3.6.3	Maintaining Track	35

3.6.4	Track Termination	35

3.6.5	Track Data Output	35

3.6.6	Plot extraction and tracking performance	37

3.7	Reliability and availability	39

3.8	Redundancy	39

3.9	Equipment standards and specification levels	40

4.	Performance verification methods	41

# Introduction

Performance requirements for VTS radars are generally different to the requirements for marine navigational radars.  VTS radars normally need to operate simultaneously on short and long range and this leads to dynamic requirements that far exceed those required onboard a ship.

Furthermore, weather related phenomena such as ducting will influence VTS radars more than ships’ radars.  This can have a significant influence on the performance – either positive or negative.

Good clutter suppression is needed for sea clutter and, for most parts of the world, for rain clutter as well.  In addition, the need to see small targets in rough weather conditions is essential, especially if objectives include detection of targets for security purposes.

The introduction of AIS further develops VTS into a modern information system, and the presentation of radar information needs to follow the trend, putting new demands on the radar performance.  Antenna side lobes and ghost targets (multiple reflections) may lead to false and dangerous results when radar returns and AIS plots are associated.  High precision is therefore required to allow for unambiguous correlation of position obtained from two information sources.

## Scope

The aim of this recommendation is to provide comprehensive guidance on the subjects relevant for the VTS application based on the VTS functional model illustrated in Figure 2.1, set generic performance standards for Radar Services in VTS systems and to provide guidelines for their implementation.

## Definitions and Clarifications

For general terms used throughout this document refer to:

IEEE Std 686-1997	IEEE Standard Radar Definitions.

Document specific terms are defined as follows:

Availability is the probability that a system will perform its specified function when required.

Normal weather and propagation conditions are the conditions persisting 99% - 99.9 % of the time as defined by the individual VTS authority.  The rest of the time is considered having adverse weather and propagation conditions.

The polarisation of a radar signal is determined by the orientation of the electrical field. In the case of circular polarisation the field rotates left or right.

The radar PD is the probability of detection at the output of a radar, subsequent to signal processing and plot extraction, but prior to tracking, and presentation.

Reliability is the probability that a system, when it is available performs a specified function without failure under given conditions for a given period of time.

Sea characteristics include wave/swell height, direction and speed of waves/swell and distance between waves/swell.

Squint are the angular difference between the axis of antenna rotation and a selected geometrical axis.

Track swap is the transfer of a track identity (track label) to another track.

### Software tools

| CARPET | Computer Aided Radar Performance Tool TNO Physics and Electronics Laboratory, P.O.Box 96864, 2509 JG The Hague, Netherlands, http://www.tno.nl |
| --- | --- |
| AREPS | Advanced Refractive Effects Prediction System Space and Naval Wafare Systems Center, San Diego, http://sunspot.spawar.navy.mil. |

### Abbreviations

AIS	Automatic Identification System

AREPS	Advanced Refractive Effects Prediction System

ASL	Above Sea Level

CARPET	Computer Aided Radar Performance Evaluation Tool

CW	Continuous Wave

dB	Decibel

dBi	Decibel isotrope

dBm	Decibel milliwatt

DF	Direction Finder

FTC	Fast Time Constant

GHz	GigaHertz

GIT	Georgia Institute of Technology

GPS	Global Positioning System

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities

ICAO	International Civil Aviation Organization

IEC	International Electro-Technical Commission

IEEE	The Institute of Electrical and Electronic Engineers

IMO	International Maritime Organisation

ITU	International Telecommunication Union

kHz	kiloHertz

Ku-band	12.0 – 18.0 GHz

kW	kiloWatt

LNFE	Low Noise Front End

m	metre

m2	square metre

MDS	Minimum Detectable Signal

MHz	MegaHertz

mm/h	millimetre per hour

m/s	metre/second

MSC	Maritime Safety Committee

MTBF	Mean Time Between Failure

MTTR	Mean Time to Repair

N/A	Not applicable

NM	Nautical Mile (also nmi)

PD	Probability of Detection

PFA	Probability of False Alarm

PRF	Pulse Repetition Frequency

PW	Pulse Width

R	Range

RCS 	Radar Cross Section

RF 	Radio Frequency

S-band	2.0 – 4.0 GHz

TNO	Toegepast Natuurkundig Onderzoek

UTC	Universal Time Co-ordinated

UTM	Universal Transverse Mercator

VTS	Vessel Traffic Services

X-band	8.0 – 12.0 GHz

µs	microsecond

º	Degree

>	Greater than

≥	Greater than or equal to

≤	Less than or equal to

## References

| [1] | IEEE Std 686-1997 | IEEE Standard Radar Definitions |
| --- | --- | --- |
| [2] | Merrill I Skolnik | Introduction to Radar Systems, McGraw-HILL Higher Education, ISBN 0-07-290980-3 |
| [3] | P.D.L. Williams, H.D, Cramp and Kay Curtis, | Experimental study of the radar cross section of maritime targets, ELECTRONIC CIRCUITS AND SYSTEMS, July 1978. Vol 2. No 4. |
| [4] | Ingo Harre | RCS in Radar Range Calculations for Maritime Targets. http://www.mar-it.de/Radar/RCS/RCS_18.pdf |
| [5] | IMO MSC Recommendation/x | Revision of Performance Standard for radar reflectors |
| [6] | International Telecommunications Union (ITU) | Radio Regulations |

# General Provisions

The provision of guidance on aspects such as implementation of radar services, general radar parameters, radar target characteristics, radar range performance, the effects of noise and clutter on radar performance and target discrimination and position accuracy are key to the provision of  effective VTS radars.

## Implementation of radar services

Implementation of radar services for VTS requires a sound knowledge of:

- VTS operational requirements;

- Target characteristics and detection range;

- Accuracy and resolution;

- Coverage analysis;

- General physical capabilities and constraints applicable to radar services;

- Assessment of interference risks;

- The influence of weather, including propagation;

- System design, including external interface and program planning.

Furthermore, requirements of radar services may have a high impact on acquisition and life-cycle costs of a VTS, therefore the performance recommendations are divided into three levels, where:

- Standard recommendations are applicable to all three levels of VTS as identified by IMO – Information Service, Navigational Assistance Service and Traffic Organisational Service – for areas with medium to high traffic density and/or without specific navigational hazards.

- Relaxed recommendations are applicable to VTS, Information Service and/or Navigational Assistance Service for areas with low to medium traffic density and without specific navigational hazards.

- Extended recommendations are applicable to VTS areas with high traffic density and/or specific navigational hazards.

The three levels of recommendations may be used as applicable within a VTS, e.g. part of a VTS area may call for standard recommendations and another part may call for relaxed recommendations.

The individual VTS authority should decide on the level of specification and define requirements related to climate and topography as identified in this document.  Consideration of other local conditions may dictate individual performance requirements that are independent of the level of recommendation.

Requirements to detect very small vessels for safety or security reasons may further extend requirements, beyond those identified here for extended recommendations.

Figure 2.1 VTS functional model.

## Radar Parameters

Radar Parameters refer to the limiting factors with respect to radar coverage and include:

- radar frequency band;

- radar location;

- antenna height;

- local weather;

- transmitted power;

- antenna characteristics;

- receiver sensitivity;

- receiver dynamic characteristics;

- processing capabilities;

- system losses.

These aspects combine to determine the minimum magnitude of radar returns that can be detected.

The parameters are often dependent on each other, and different vendors may choose different methods to solve the same issue.  Therefore, it is recommended that overall radar performance requirements are specified, taking the local weather into consideration, rather than specifying radar parameters.

However, in order to understand how the parameters affect radar performance, the influence from some of the parameters are described as follows.

### Radar frequency

Radar frequencies selected for VTS lie typically within the S-band and X-band frequencies, however, higher frequencies such as Ku band may also be utilised.

The major operational effects of different frequencies are:

- For a given physical antenna size, azimuth resolution will increase with higher frequency;

- Antenna gain and, thereby, overall system sensitivity increases with higher frequency for a given physical antenna size, however, system losses also increase with frequency and may be severe at very high frequency, such as Ku band;

- Radar cross section of targets in general will increase with frequency, however, specific types of targets may give particularly strong returns in a given frequency band;

- Disturbance by precipitation increases with frequency.

The majority of VTS services use X-band radars as a best compromise, especially since technologies for rain clutter suppression have matured.

Cost is also a factor and, because of the production volume, X-band radars are the least expensive.

The second most used frequency is S-band, basically because of better weather penetration in case of heavy rainfall.

Frequency band allocation is granted by the ITU, whereas permissions to transmit on given frequencies are granted on a national basis.

Modern transmitter and processing technologies allow several radars in close vicinity to transmit on the same frequency without running the risk of disturbing each other.  The same techniques provide protection against interference from other radars, e.g. radars onboard ships.

### Antenna parameters

The antenna gain is one of the most important factors determining the radar range.  The gain is mainly determined by the operating frequency and the horizontal and vertical beam widths of the antenna array.

Narrowing the horizontal as well as vertical beam widths will result in reduction of the volume clutter and thereby improved performance in precipitation.  The horizontal beam width is also the governing parameter in respect to azimuth resolution and it has significant influence on the amount of surface clutter received.  The vertical beam characteristics are the governing factor in respect to coverage at short range.

Use of circular polarization will reduce unwanted returns from precipitation while maintaining good returns from targets of interest to the VTS operator.  However, some radar reflectors are specifically designed for linear polarisation and their visibility may therefore be reduced by the use of circular polarisation.

If small targets of interest, such as rubber boats, are identified by the VTS as a target of concern, then the selection of antenna polarization is even more complex and should be studied carefully.

Use of high antenna rotation speed increases the scan data rate, but the hits per scan (echo returns per scan from a particular target) is decreased correspondingly.  In some cases this may mask a weak target that would otherwise be detected by using a lower antenna speed.

#### Side lobes

A small part of the radio frequency energy from each transmitted pulse is radiated outside the main antenna beam, producing side lobe patterns whose axis in the horizontal plane have various angles with respect to the main beam.

Side lobes have no effect in case of distant or small surface targets, but the echo from a large target at short range may produce an arc similar to a part of a range ring or they may appear as a series of echoes forming a dotted arc.

Side lobes may also occur far from the main beam producing targets that are very difficult to discriminate from real targets.

The side lobe levels are entirely dependent on the antenna design. Far side lobe characteristics are very important, as side lobes far from the main beam may result in false targets, appearing to the operator with the same characteristics as a valid target.

### Transmitter characteristics

For traditional pulsed radars the average power and peak pulse power has an influence on the detection range.  However, its importance is often overestimated.  High peak power often results in higher losses in the transmission lines of the radar and it will always result in a larger amount of unwanted information (e.g. sea clutter).

Shorter pulse lengths provide better range resolution, a reduction of returns from rain and sea clutter and better overall performance.  The use of relatively short pulses (50 to 300 nanoseconds) is often beneficial for VTS applications.

### Receiver characteristics

The receiver sensitivity (often expressed by the receiver noise factor) is one of the main factors with respect to detection of small distant targets in clear weather.  The noise figure may be expressed in different terms such as ‘Low Noise Front End (LNFE) noise figure’ and ‘overall noise figure’, making it difficult to compare radars from different vendors.

A better expression for receiver sensitivity is the Minimum Detectable Signal (MDS); however, this is not ideal either as it may be difficult to measure this figure in a reliable manner.

The dynamic characteristics are very important, especially if operational conditions require:

- Simultaneous detection of targets on short and long range;

- Supply of signals for more than one display with different range settings;

- Simultaneous supply of signals for different operational roles;

- Supply of signals for automatic video processing and automatic tracking.

Minimum detectable signals at the receiver input and processed onwards lie typically at -90 to -105 dBm in VTS applications, or may be as low as -115 dBm for security applications.

Very large targets at close range are, however, not fully within the beam at short range, limiting the input at the receiver to about + 10 dBm for the typical case.

Consequently, the overall dynamic range needed for the individual radar for a VTS application is typically between 100 and 125 dB.

#### Pulse compression

For pulse compression radars the pulse compression circuit in the receiver will generate so called ‘time side lobes’ which may appear as weaker, false signals in front of and behind large targets.

Special attention should be paid to this effect.

#### Continuous wave

For continuous wave radars similar effects as for pulse compression radars can be observed due to the receiver processing.

In addition spill-over of energy between transmitting and receiving antennas may limit the useful dynamic range.

The use of small antennas allowed by the technology may lead to reduction of azimuth resolution and result in higher susceptibility to rain and sea clutter.

### System losses

System losses reduce performance, especially the ability to detect small distant targets.  Common reasons for system loss are:

- Long waveguide runs;

- Moisture in the waveguide runs;

- Flexible and especially flexible/twistable waveguide parts. (Not to be confused with elliptical waveguides which, in general, have small losses);

- Teflon or plastic gaskets used instead of proper pressure windows, especially if mounted between plain and choke flanges;

- Excessive number of joints in the waveguide system;

- Poor rotary joints;

- Poor workmanship in the manufacture of waveguide parts - it is a skilled persons job;

- Matching losses in receivers and distribution of signals both with inadequate bandwidth / speed;

- Digitalisation of signals using too few bits;

- Losses in detection and tracking.

Furthermore, discontinuities in waveguide installations can result in reflections causing the centre of the radar picture to be totally saturated.  Bad cases can even disturb the transmitted pulse, giving reduced angular resolution as well as reduced performance with respect to discrimination of targets from clutter.

### Signal processing, extraction and tracking

As mentioned above the radar return signal down-converted to video level contains wanted signals from objects of interest, unwanted signals (clutter) – for example sea clutter – and some basic system noise – for example, thermal noise – all mixed together.  The purpose of the signal processing is to enhance the signals from the objects of interest relative to the clutter and noise.  Different techniques are used.  Effective methods include:

- Sweep-to-sweep correlation: Normally, the pulse repetition rate of a radar is so high that a target is hit several times while the antenna beam turns past it.  In contrast, noise will pop up randomly.  Therefore, by correlation of radar return signals from consecutive transmissions, noise can be suppressed efficiently, relative to targets (and clutter).

- Signal differentiation: Extended areas of clutter (e.g. from rain showers) can be reduced by signal differentiation (in radars this is called Fast Time Constant (FTC)).

- Signal integration: The multiple hits on a target as described above can be added together.  This will enhance the returns from the objects of interest relative to noise and, to a lesser extent, relative to clutter.  Further, it will reduce the fluctuations of target echoes.

- Frequency Diversity: Frequency diversity radars employ two transmission frequencies.  The radar returns on the two frequencies from a target will normally be similar, while radar returns from clutter will be different.  If an antenna with frequency dependent squint such as a slotted waveguide antenna is used, there will also be a temporal difference between the returns on the two frequencies.  This means that the target will be present in both radar returns, while it is very likely that the clutter will be present in only one of the returns.  The total effect of using frequency diversity will be an enhancement of target radar returns, relative to clutter radar returns.

- Scan-to-scan correlation: By comparing radar returns from consecutive rotations of the antenna, fast disappearing echoes (e.g. sea waves) can be filtered away while persisting echoes (e.g. ships) can be retained for further processing.  Again, the overall effect is to enhance target radar returns, relative to clutter radar returns.

The output of the signal processing is the processed radar data in a polar grid, i.e. a circular area divided into radar cells, with each cell being characterized by a range value, a range depth, an azimuth value, an azimuth width and a radar return intensity value.  Plot extraction is the process of identifying groups of neighbouring radar cells containing radar return intensity values above a defined threshold value.  Such a group of radar cells is called a plot, and is intended to correspond to a physical target.

Plots are updated on each rotation of the radar.  Plots which persist from rotation to rotation, and which move within reasonable physical limits of speed and acceleration, are intended to correspond to physical targets.  A series of plots associated in this way is called a track.  Erratically appearing plots are rejected as representatives for physical objects and, therefore, as candidates for tracks.

Tracking is the process of associating plots from consecutive rotations of the radar to existing tracks and to initiate new, tentative tracks on plots not associated to existing tracks.

## Radar Target Characteristics

To understand the behaviour of radar targets it is essential to determine expected performance from radar services.

The strength of the echo returned from a target is largely governed by the size of the target, its aspect and its height above the radar horizon.  However, these are not the only factors and a small target of good reflecting properties may well give a stronger echo than a much larger target of poor reflecting properties.

### Target properties

Aspect angle (i.e. the angle at which the radar pulse “hits” the target) is an important factor affecting echo strength.  Certain types of objects that might be expected to give a good echo prove to be very poor radar targets because they reflect most of the energy that strikes them in a direction other than straight back to the radar.  The worst of these are gently curved smooth surfaces, such as sandy beaches, sandbanks, mud banks, and gently sloping hills that are regular in shape and not covered with trees or thick undergrowth.  For the same reason lighthouses that are conical in shape do not give as good a response as might be expected.

Vertical, comparatively smooth surfaces, such as cliffs and isolated buildings, give strong reflection provided they face the radar.

Complex structures such as metal ships with several corners also give strong reflections, whereas ships made of other materials typically give weaker reflections.

### Radar cross-section

The characteristics of a target are defined by its radar cross-section (RCS) and the fluctuations in RCS.

Formally, the RCS is defined as the ratio between the power [in W] scattered by the target back towards the radar receiver and the power density [in W/m2] hitting the target.  Thus RCS is measured in m2 and has the dimension of an area.  There is, however, no simple relation between the physical area of the target and the RCS, as the reflected power depends on the angle of incidence as well as target properties such as material (conducting/non-conducting), coating, physical shape (smooth/sharp corners) etc.

The total reflection from a target is made up from the ‘elementary’ reflections from different parts of the target.  These elementary reflections will interfere constructively or destructively to form the total reflection.  Therefore, small changes in attitude of the target may result in large changes of the amount of reflected power.  The consequence of this is that, in radar performance evaluations, the RCS of a target is best described by a statistical distribution function.  Different kinds of wanted targets (ships, buoys, etc.) and unwanted targets (sea clutter) have different distribution functions.  These differences can be exploited in the radar signal processing to enhance target visibility.

Scientific references giving information about the RCS of objects relevant for VTS can be found in 'Introduction to Radar Systems' [2] and in 'The experimental study of the radar cross-section of maritime targets' [3].  However, since the date of the study, ship design has resulted in larger variation in RCS and experience is that the modified values in Table 2.1 give appropriate values of RCS for various targets.

Comprehensive RCS data for other frequency bands were not available to be included in this document; however, different tests of radar reflectors have yielded differences between X-band and S-band of up to 11 dB. [5]

From evaluation of figure 2.17 in [2], [4] and in the absence of more detailed investigations, a given X-band RCS should be reduced by 40% for S-band range calculations.

Excluded from this are returns from birds and flocks of birds where the RCS of the individual species often results in a higher RCS on S-band than on X-band.

Furthermore it should be noted that RCS is a figure with highly statistical properties.  Facing this, the lower values in should be used for range calculations.  The upper values are relevant for determination of dynamic requirements to the radar services.

### Characteristics of wind farms

The main influence from wind farms will typically be reflections from the towers.  The symmetrical lay out of the wind farms may result in unwanted ghost echoes.

VTS radars will normally not be dependent on Doppler shift and they are therefore not affected by the rotation of wind turbines.

Table 2.1  Typical target characteristics.

| Target types |  | Typical characteristics at X-band |  |  |
| --- | --- | --- | --- | --- |
|  |  | RCS | Height | Fluctuations etc. |
| 1 | Buoys and beacons without radar reflector | Up to 1 m2 | 1 - 4 m ASL | Rapidly fluctuating, highly dependent on sea characteristics. |
| 2 | Buoys and beacons with radar reflector | 10 – 100 m2 |  | Rapidly fluctuating, wind and currents may tilt to blind angles and lobing may cause reflectors to be in blind spots |
| 3 | Small open boat, fibreglass, wood or rubber with outboard motor and at least 2 persons onboard, small speedboat, small fishing vessels or small sailing boats. | 0.5 – 5 m2 | 0.5 to 1 m ASL | Rapidly fluctuating may be hidden behind waves up to 50 % of the time. Slow moving targets tend to lie lower in the water than fast moving ones and therefore RCS visible to the radar tends to increase with speed. |
| 4 | Inshore fishing vessels, sailing boats and speedboats, equipped with radar reflector of good quality. | 3 – 10 m2 | 1 – 2 m ASL | Rapidly fluctuating |
| 5 | Small metal ships, fishing vessels, patrol vessels and the like | 10 – 100 m2 | 2 - 4 m ASL |  |
| 6 | Coasters and the like | 100 – 1000 m2 | 6 - 10 m ASL | RCS is highly dependent on aspect angle of the individual vessel. Rate of fluctuations is typically moderate |
| 7 | Large coasters, Bulk carriers, cargo ships and the like | 1000 – 10000 m2 | 10 - 25 m ASL |  |
| 8 | Container carriers, tankers etc. | 10000 – 1000000 m2 | 15 - 40 m ASL |  |
| 9 | Buildings, cranes. Stacks of containers and other large structures | Up to 1000000 m2 | Depend on site | Insignificant |
| 10 | Floating items, oil drums and the like. Birds, floating or flying. | Up to 1 m2 | 0 to 0.5 m ASL | Rapidly fluctuating, highly dependent on sea characteristics. |
| 11 | Flocks of birds. | Up to 3 m2 | Sea level and up | Rapidly fluctuating, flight paths tend to be characteristic of given species in given areas of interest |

## Radar Range performance

The range performance of a radar system is the result of many effects.  In calm weather and under normal atmospheric conditions it depends mainly on the radar specifications and the antenna height, but sea and weather conditions affect the radar range as described in the following.

### Antenna height

The figures below illustrate how the height of the antenna above the water line affects the maximum and minimum detection range performance.

Figure 2.2  Target range and visibility.

### Propagation conditions

In some areas of the world, especially those with hot, dry climates, propagation effects cause radar performance to vary drastically over time – sometimes from hour to hour.

The density of the atmosphere is not uniform with altitude.  The gradient of density results in a bending of the radar waves in a manner similar to the bending of light waves by an optical prism.  Water vapour is the atmospheric component chiefly responsible for the bending of radar waves in the lower atmosphere as illustrated in   E.  Temperature also has an influence.

The temperature and humidity gradients in the air above the sea surface cause the radar beam to be bent up or down - in most cases down - and to follow the earth’s curvature to some extent, thereby elongating the range for potential target detection. This condition is called super-refraction, or ducting.

The air in contact with the ocean’s surface is saturated with water vapour.  A few metres above the surface the air is not usually saturated, so there is a decrease of water vapour pressure from the surface to some value well above the surface.  Such a change in the moisture distribution, without an accompanying temperature change, forms a so-called evaporation duct, always leading to extended radar range if present.

The atmosphere may also form a trapping layer and generate a wave-guide like duct, a surface-based duct or an elevated duct.

Under trapping layer conditions a radar antenna situated within the duct may receive extremely long-range returns.  The result includes a considerable increase in the amount of clutter and false echoes generated by objects beyond the radar ambiguity range (2nd and multiple-time around echoes).

Above a strong duct (a trapping layer) there is often a layer of low refraction (sub refraction).  An antenna situated above a trapping layer or, even worse, above more than one trapping layer will radiate most of its energy into the sub-refractive layer, which will not direct the energy down towards targets on the sea or on the ground.  Therefore, the radar will detect only very short-range targets.  This may limit usable antenna height in dry hot regions of the world, especially in Middle Eastern countries.

Severe atmospheric conditions, such as thunderstorms or the acute build-up of static electricity during pre-storm situations also have the affect of distorting the radar owing to the contorting effect of the electrical disturbances in the atmosphere.

Figure 2.3  Effects of propagation conditions.

The radar range is limited by the earth’s curvature and is a function of antenna elevation and target height.  A first estimation of its value, in nautical miles (NM), can be derived from the equation:

R (NM) = 2.2 (hA + hT)

This assumes standard atmospheric conditions as defined by the International Civil Aviation Organization, (ICAO).  Antenna height hA and target height hT are measured in metres.

However, it should be noted that detection of large targets can only be expected to the theoretical radar horizon.  Complex calculations such as that illustrated in Figure 2.2 give a better impression of what may be expected.  A software tool such as CARPET can be used for complex calculations.

### Lobing

The surface of the earth or the sea not only restricts the line of sight, but it may also have an effect on the radar coverage of targets within the line of sight.  Two waves arrive at the target via two separate paths:  One travels along the direct path from radar to target; the other path includes a reflection from the water surface.  The two waves can interfere destructively or constructively according to their difference in phase.  This effect is known as lobing.

One of the effects of lobing is that point targets, such as buoys, may become invisible to a radar at certain ranges and in certain conditions as illustrated by Figure 2.4.

Figure 2.4  Coverage diagram for a typical VTS radar.  Detection probability PD is given as a percentage as shown in the lower bar in the figure.

### Shadow effect

For determination of shadow effect, the propagation of radio waves at radar frequencies can be considered as for optical visibility in clear weather.

### Tide

Tide may have a substantial influence on coastlines, river banks etc. and the antenna height above sea level is also affected.

## Noise and clutter affecting radar performance

Noise is the part of the radar signal which is uncorrelated to the transmitter pulse, whereas clutter is unwanted radar return signals from e.g. sea waves, precipitation or land areas.

Targets, as well as various sources of noise or clutter, behave statistically in different ways.  Knowledge of how to limit the effect of noise is essential to obtain adequate performance of the radar system and the ability to detect target echoes depends not only on the signal power, but also on the competing noise and clutter power present in the receiver.

The sources of this noise/clutter lie both within the radar and in the outside environment as illustrated in Figure 2.5.

Figure 2.5  Example of noise sources.

### Thermal noise

Historically, the sensitivity (or MDS) of radar systems has been limited by the thermal noise of the receiver, expressed by the noise figure.  However, processing techniques that became common during the 1990’s can remove the white noise and improve the radar system sensitivity (or MDS).

### Surface and volume clutter

A radar set that is sensitive enough to detect small targets, like buoys and small boats, will also be sensitive enough to detect returns (clutter) from land, sea waves, raindrops, snowflakes and hail.  Clutter is therefore often the limiting factor for detection of targets of interest.

#### Sea Clutter

Sea return is caused by the reflection of the radar signal from water surface irregularities and sea return is therefore composed of a very large number of echoes, originated by the random wave reflection from scan to scan.

Sea and wind relations vary depending on the geographical location.  Ocean areas differs from other waters, and the same seems to be the case for the definition of sea states, which have been seen to be defined by the Beaufort scale (wind), the GIT (Georgia Institute of Technology) model (used by the CARPET radar performance evaluation program) or the Douglas scale.  The GIT model has been used throughout this document.

#### Sea breezes

The large temperature variations between day and night in desert areas, and the associated pressure differences between land and sea, tend to cause strong sea breezes during summertime.  This, combined with layered distribution of air, generates eddies over the sea, often forming distinctive sea clutter patterns.

#### Land clutter

Clutter caused by land will not normally affect the VTS radar service.

### Ice

The echoes obtained from ice depend largely on the form and shape the ice encountered.

The typical characteristics of the various types of ice floes are as follows:

- Smooth flat ice: Most of the radar energy is reflected at its angle of incidence, providing little or no return signal.

It is sometimes an advantage to set up the radar controls to obtain sea clutter right up to the edge of the ice.  If there is sufficient wind to disturb the surface of the water, patches of water in a smooth ice field are often revealed by sea clutter return.

- Pack ice: Strong multiple echoes are usually obtained from this condition producing a pattern on the display not unlike excessive sea clutter.  The ice left in the wake of a vessel passing through an ice field can be distinguished clearly.

- Ice walls: These objects will give strong echoes depending on the angle that they are to the sea surface to scatter the reflected energy.

- Icebergs: The sharp angles of the faces of icebergs usually give a poor return signal.  Their smooth surface will often scatter the reflected signal at acute angles.

- Growlers: The detection of growlers by radar is uncertain due to their small surface area above water.

Detection of icebergs and growlers may therefore require sophisticated techniques, similar to those for detection of small targets performing illegal activities.

### Precipitation

Precipitation in the form of rain, snow, hail and other hydro-meteorological particles affects radar range and tends to give unwanted returns or clutter.

#### Rain clutter and penetration

The radar signal’s ability to penetrate the atmosphere depends on the amount of precipitation and the amounts of precipitation (or rain) clutter returned are dependent on:

- The resolution cell size – influence is reduced with decreasing antenna beam width and  with decreased transmitter pulse width;

- The antenna polarization, where circular polarization is preferred over vertical or horizontal polarization;

- The frequency, where lower frequencies penetrate precipitation better than higher frequencies and give less clutter return.  However, lower frequencies are in most cases associated with severe drawbacks in respect to resolution and cost.  Therefore, the use of higher frequencies and circular polarized antennas in combination with processing techniques tend to be preferred for VTS, except for long-range installations in areas with extreme tropical rainfall.

#### Hail and snow

Hail and snow produce effects similar to that of rain clutter.  Dense snow has a greater effect than that of light flurries which, owing to its small reflecting surface, has minimal effect.

### Man-made noise

Man-made noise consists primarily of signals from other nearby radars (interference).  However, applying noise reduction techniques allows for several radars to operate on the same frequency in close proximity without interference.

### Multiple reflections (ghost images)

Multiple reflections can result in several types of ghost images.  Sometimes these have the appearance of normal targets, but in most cases they are intermittent and poorly defined.  Ghost images maintain a fixed relationship with respect to the true image and have more of an arc-like appearance than the true image, as well as a tendency to smear.  This is especially the case when the radar is located in the vicinity of large structures.

One type of ‘ghost’ is caused by multiple reflections from objects that are close together.  For example, two vessels that are passing within a few hundred metres of each other may, because of reflections of the electrical impulse between the vessels, produce a smaller third, and fourth image at a longer range.  These images will usually appear only momentarily beyond the vessel.

Another type of ‘ghost’ image is caused by secondary reflection from bridges, buildings and other structures.  Such reflections are especially common when the antenna is mounted close to such structures.

Careful notice should be taken of any false image encountered, in order to determine the probable reason for its creation in order to assist in future recognition.  For safe operation, if there is ever any question as to whether a particular image is true or a ‘ghost’, always assume it to be a true image corresponding to an actual object.

## Target discrimination and position accuracy

The ability to discriminate targets depends on the radar range and azimuth resolution as shown in Figure 2.6.

### Range resolution

For traditional pulse radars, the range resolution depends on transmitter Pulse Width (PW), rise time, fall time, allowance for jitter and stretch in the receiver.

In a pulse compression or continuous wave radar, the effective pulse width can be made very short providing high resolution.  The equivalent peak power may be very big.  The drawback is added complexity and unwanted effects such as time side lobes.

Figure 2.6  Range and Azimuth resolution.

### Azimuth resolution

The azimuth characteristics of a radar return from a point target are almost entirely determined by the characteristics of the antenna.  The azimuth beam width is inversely proportional to the antenna length that, in turn, directly affects the radar's angular resolution (and range performance, as antenna gain increases with length).

The angular width of small target echoes (at the - 3 dB) is equal to the horizontal beam width of the antenna (defined at the - 3 dB points).  However, this is a highly theoretical definition of azimuth resolution that is mainly used for comparison of equipments.

Most targets, especially at short or medium ranges, are detected when they are also below the - 3 dB points and, as a result, most echoes appears with an angular width in excess of the antenna beam width.

### Range accuracy

Determination of the range from an antenna to a target (the slant range) is normally dependent on precise, reliable clocks.  The range accuracy is accordingly precise in most modern radar systems.

### Azimuth accuracy

The azimuth accuracy depends mainly on the turning gear, the resolution and accuracy of its associated bearing transmitter.

## Security implications

Using a VTS for security applications is outside the IALA mandate and this document only discusses the implications briefly.

A VTS authority asked to provide information for security purposes must realise that radar is the only VTS tool that can potentially detect small, non-cooperative targets at long distances and in all weather conditions.  Detection and identification of such objects is a difficult task likely to impose requirements to:

- Substantially increase sensitivity of the radar services, also dictating increased dynamic range, additional requirements to suppression of noise and clutter, increased resolution requirements and increased data rates;

- Identify revised functional requirements for extraction, tracking, data handling and display presentation that are likely to be so different from normal VTS operations that they should be separated between the normal VTS operation and operation specific for security.

The requirements for security applications are therefore in general additional to the extended set of requirements outlined in this document.

# Recommendation on performance standards for VTS radar services

## General requirements

The output from a radar service should include radar image and track data.  The radar facilities provided for VTS should comply with the minimum performance standards defined by this document.

The individual VTS authority should ensure that the selected system architecture, the number of equipments, the network capacity, etc. is capable of meeting the performance standards.

Radar services functions should furthermore be designed and implemented to optimise performance and minimize operator workload to the level practical.  Ideally, only basic functions such as start and stop should be controllable by operators and it is recommended to make adaptation to changing weather conditions, etc. automatic.

## Objects to be detected

The radar service in a VTS should be capable of detecting and tracking all types of surface objects defined by the VTS authority in weather conditions normal for the individual site.  Table 3.1 recommends the level of radar service necessary to detect various categories of targets.

Table 3.2  Types of targets (surface objects) to be detected and corresponding (point) target model.

| Target |  | Recommendation level |  |  | Corresponding target model |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Relaxed | Standard | Extended | RCS |  | Height |
|  |  |  |  |  | S-band | X-band |  |
| 1 | Buoys and beacons without radar reflector. Small open boat, fibreglass, wood or rubber with outboard motor and at least 2 persons onboard, small speedboat, small fishing vessels, small sailing boats and the like. |  |  | X |  | 1 m2 | 1 m ASL |
| 2 | Inshore fishing vessels, sailing boats, speedboats and the like. |  | X | X |  | 3 m2 | 2 m ASL |
| 3 | Buoys and beacons with radar reflector | X | X | X | 4 m2 | 10 m2 | 3 m ASL |
| 4 | Small metal ships, fishing vessels, patrol vessels and the like | X | X | X | 40 m2 | 100 m2 | 5 m ASL |
| 5 | Coasters and the like | X | X | X | 400 m2 | 1.000 m2 | 8 m ASL |
| 6 | Large coasters, Bulk carriers, cargo ships and the like. | X | X | X | 4.000 m2 | 10.000 m2 | 12 m ASL |
| 7 | Container carriers, tankers etc. | X | X | X | 40.000 m2 | 100.000 m2 | 18 m ASL |

Any special object of interest should be specified separately.

In addition, the radar service should be available to the maximum extent possible in adverse weather conditions.

The individual VTS authority may find it desirable to discontinue radar services temporarily in the event of adverse conditions, such as extreme wind or icing conditions - or define special requirements and operational procedures in such conditions.

## General features and characteristics

The VTS authority shall ensure that general features and characteristics of the radar services are sufficient to meet detection requirements and ensure ease of operation.  This will traditionally require clutter and noise reduction facilities, sufficient antenna side lobe suppression, sufficient dynamic characteristics and built-in test features.

RF frequencies shall comply with ITU Radio Regulations and national regulations.  Otherwise it is suggested that the supplier decides on the most economical solution to meet the individual performance requirement, especially focussed on resolution, coverage and weather penetration.

### Clutter and noise reduction facilities

Appropriate, clutter reduction facilities should be available to meet the performance criteria as defined per section 3.4.1.4.

This will typically include:

- White noise suppression;

- Sea clutter suppression;

- Rain penetration and volume clutter suppression; and in some cases;

- Adaptation to varying propagation conditions.

The features should preferably be automatic for systems requiring standard or extended specifications.

### Side lobe suppression

Antenna side lobes should be sufficiently low to avoid false targets, especially false targets far from the antenna main lobe.

Table 3.3 provides recommendations about characteristics suitable for the three Recommendation levels.

Table 3.3  Side lobe suppression.

| Antenna side lobe suppression | Recommendation level |  |  |
| --- | --- | --- | --- |
|  | Relaxed | Standard | Extended |
| Minimum first side lobe level suppression | 26 dB | 27 dB | 28 dB |
| Increasing to, at angles +/- 10 or more outside the main lobe. | 33 dB | 34 dB | 35 dB |

In the case of pulse compression or continuous wave (CW) radars, time side lobes should also be evaluated.

### Dynamic characteristics

The dynamic range of the radar service should, in normal weather and propagation conditions, detect and process the surface objects specified in Table 3.2 within the VTS area.  This should be done, avoiding distortion such as:

- Pulse stretch making any significant disturbance of the radar image or the tracking performance;

- Masking of small targets by larger targets (except if shaded);

- Masking of small targets by the effects of time side lobes (in the case of pulse compression or CW radars).

In addition maximum target sizes and target fluctuations for targets of interest, shall be considered.  Refer to Table 2.1.

### Built-in test features

Built-in test features should include monitoring of functions and performance.  It is recommended that results are made accessible for remote monitoring, especially in the case of radars that are installed at locations difficult to access.

## Radar coverage and detection performance

The requirements for radar coverage and range performance should be determined by the VTS authority.

### Detection range and conditions

The operational requirement should be defined under the weather and propagation conditions normal for the individual site.

In normal weather and propagation conditions the surface objects specified in Table 3.2 and within the VTS area should be:

- Clearly displayed from a defined minimum horizontal range (e.g. 50m) from the antenna position to the maximum detection range determined in accordance with section 3.4.1.4;

- Tracked stably from a defined minimum horizontal range (e.g.100 metres) from the antenna position to the maximum detection range determined in accordance with section 3.4.1.4.

The equipment should give a clear indication and tracking of targets at such specified ranges.

Note that excessive antenna heights may increase the above minimum values or require dedicated vertical radiation patterns (inverse cosecant square) to be used for the antenna.  Displacing the radar could also avoid such lack of coverage.

#### Poor Visibility

The radar detection range (or radar visibility) will normally not be affected by poor visual visibility caused by haze, fog or smog and performance requirements in such conditions shall be based on the clear weather values stated in Table 3.4.

#### Performance in sea clutter

The requirements to detect targets in higher sea states should be defined individually depending on site conditions.  Typical values are stated in Table 3.4.

#### Rain penetration and volume clutter

The ability to detect targets in precipitation should be defined by the VTS authority on the basis of statistical information about local weather conditions including:

- the frequency of precipitation;

- the density of precipitation;

- the size of rain cells etc.

Such data will normally be available from local meteorological services.

For precipitation rates normal to the individual site it may be acceptable to reduce detection range (i.e. by 25 %).  This assumes that volume clutter (rain clutter) has been suppressed to obtain acceptable false alarm rates.

In many cases precipitation is not uniform and it may be desirable to specify performance in precipitation showers typical for the individual site.

Alternatively, a simpler method of averaging the precipitation may be used - assuming the precipitation to be uniform.  Typical values for this method have been used in Table 3.4.

#### Determination of range performance

The recommended method for determination of radar coverage and range performance is a combination of site inspections and radar system performance calculations, made by experts with a sound operational and technical knowledge about the subject.

Calculation of performance should be focussed on the smallest targets of interest in poor weather conditions.

Calculations are made on the basis of a simplified model of the targets and the environment based on statistical information and it is important to understand the limitations and tolerances this entails.

Concerns in respect of the largest targets are normally their influence on small targets.  Furthermore, the radar service shall have sufficient dynamic range to handle the amplitude of signals.

Table 3.4 give examples of calculated range performance typical for the three recommendation levels.  The CARPET programme from TNO was used to perform the calculations.

Note that the rain intensity and sea states are increased with increased specification level in the table.

Table 3.4  Typical range performance, X-band.

| Antenna elevation | Target type | Modelled as fluctuating point target |  | Detection and tracking ranges for standard atmosphere and rain/sea state as indicated |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RCS | Height | Relaxed recommendation |  |  | Standard recommendation |  | Extended recommendation |  |
|  |  |  |  | Clear | 2 mm/h rain |  | Clear | 4 mm/h rain | Clear | 10 mm/h rain |
| 20 m ASL | 1 | 1 m2 | 1 m ASL | N/A |  |  | N/A |  | 5 NM | NIL |
|  |  |  |  |  |  |  |  |  | Up to sea state 4 |  |
|  | 2 | 3 m2 | 2 m ASL | N/A |  |  | 7 NM | 4NM | 7 NM | 6 NM |
|  |  |  |  |  |  |  | Up to sea state 3 |  | Up to sea state 5 |  |
|  | 3 | 10 m2 | 3 m ASL | 7 NM |  | 4 NM | 8 NM | 5NM | 9 NM | 7 NM |
|  |  |  |  | Up to sea state 3 |  |  | Up to sea state 4 |  | Up to sea state 6 |  |
|  | 4 | 100 m2 | 5 m ASL | 9 NM |  | 8 NM | 11 NM | 9NM | 12 NM | 10 NM |
|  |  |  |  | Up to sea state 4 |  |  | Up to sea state 5 |  | Up to sea state 7 |  |
|  | 5 | 1000 m2 | 8 m ASL | 12 NM |  | 10 NM | 13 NM | 11 NM | 14 NM | 13 NM |
|  |  |  |  | Up to sea state 5 |  |  | Up to sea state 6 |  | Up to sea state 8 |  |
| 50 m ASL | 1 | 1 m2 | 1 m ASL | N/A |  |  | N/A |  | 10 NM | NIL |
|  |  |  |  |  |  |  |  |  | Up to sea state 4 |  |
|  | 2 | 3 m2 | 2 m ASL | N/A |  |  | 10 NM | 7 NM | 12 NM | 9 NM |
|  |  |  |  |  |  |  | Up to sea state 3 |  | Up to sea state 5 |  |
|  | 3 | 10 m2 | 3 m ASL | 10 NM |  | 6 NM | 12 NM | 8 NM | 14 NM | 12 NM |
|  |  |  |  | Up to sea state 3 |  |  | Up to sea state 4 |  | Up to sea state 7 |  |
|  | 4 | 100 m2 | 5 m ASL | 13 NM |  | 12 NM | 15 NM | 13 NM | 17 NM | 15 NM |
|  |  |  |  | Up to sea state 4 |  |  | Up to sea state 5 |  | Up to sea state 7 |  |
|  | 5 | 1000 m2 | 8 m ASL | 16 NM |  | 15 NM | 18 NM | 17 NM | 20 NM | 18 NM |
|  |  |  |  | Up to sea state 5 |  |  | Up to sea state 6 |  | Up to sea state 8 |  |
| 100 m ASL | 1 | 1 m2 | 1 m ASL | N/A |  |  | N/A |  | 12 NM | NIL |
|  |  |  |  |  |  |  |  |  | Up to sea state 4 |  |
|  | 2 | 3 m2 | 2 m ASL |  |  |  | 13 NM | 5 NM | 16 NM | 10 NM |
|  |  |  |  |  |  |  | Up to sea state 3 |  | Up to sea state 5 |  |
|  | 3 | 10 m2 | 3 m ASL |  |  |  | 17 NM | 10 NM | 18 NM | 16 NM |
|  |  |  |  |  |  |  | Up to sea state 4 |  | Up to sea state 7 |  |
|  | 4 | 100 m2 | 5 m ASL |  |  |  | 20 NM | 19 NM | 22 NM | 20 NM |
|  |  |  |  |  |  |  | Up to sea state 5 |  | Up to sea state 7 |  |
|  | 5 | 1000 m2 | 8 m ASL |  |  |  | 23 NM | 22 NM | 25 NM | 23 NM |
|  |  |  |  |  |  |  | Up to sea state 6 |  | Up to sea state 8 |  |

Table 3.5  Typical range performance, S-band.

| Antenna elevation | Target type | Modelled as fluctuating point target |  | Detection and tracking ranges for standard atmosphere and rain/sea state as indicated |  |
| --- | --- | --- | --- | --- | --- |
|  |  | RCS | Height | Standard recommendation |  |
|  |  |  |  | Clear | 16 mm/h rain |
| 20 m ASL | 3 | 4 m2 | 3 m ASL | 4 NM | 3 NM |
|  |  |  |  | Up to sea state 4 |  |
|  | 4 | 40 m2 | 5 m ASL | 7 NM | 5 NM |
|  |  |  |  | Up to sea state 5 |  |
|  | 5 | 400 m2 | 8 m ASL | 10 NM | 8 NM |
|  |  |  |  | Up to sea state 6 |  |
| 50 m ASL | 3 | 4 m2 | 3 m ASL | 7 NM | 4 NM |
|  |  |  |  | Up to sea state 4 |  |
|  | 4 | 40 m2 | 5 m ASL | 11 NM | 8 NM |
|  |  |  |  | Up to sea state 5 |  |
|  | 5 | 400 m2 | 8 m ASL | 14 NM | 13 NM |
|  |  |  |  | Up to sea state 6 |  |
| 100 m ASL | 3 | 4 m2 | 3 m ASL | 10 NM | NIL |
|  |  |  |  | Up to sea state 4 |  |
|  | 4 | 40 m2 | 5 m ASL | 14 NM | 12 NM |
|  |  |  |  | Up to sea state 5 |  |
|  | 5 | 400 m2 | 8 m ASL | 18 NM | 19 NM |
|  |  |  |  | Up to sea state 6 |  |

#### Propagation conditions

Performance should, in all cases, be evaluated assuming standard atmospheric conditions.

Ducting may occur almost anywhere, and all systems should be designed to eliminate adverse negative effects from this.

For most parts of the world, evaporation ducting tends to persist most of the time, giving extended range, especially for low mounted antennas.  The effect will give average improvement in detection performance and may therefore be very useful in respect to security applications, if required.  The effect is usually not stable enough to be calculated in safety applications.

For areas of the world having:

- Tropical climate;

- Dry and hot climate;

the influence from adverse propagation effects should be analysed in detail.

## Target Discrimination

In normal weather and propagation conditions, surface objects within the VTS area should be separated in presentation, and individually tracked without track swap at any applicable target speed when they are positioned apart and with distances as defined by the individual VTS authority.

Table 3.6. provides examples of characteristics suitable for the recommendation levels.

For larger (non-point) targets the definition of separation is highly dependant on aspect angles, pulse stretch etc.  Proper transmitter and receiver characteristics, rather than specification of absolute separation, will ensure proper separation of such targets.

Table 3.6  Target separation.

| Physical separation between small point targets for discrimination in display and tracking |  |  | Recommendation level |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relaxed |  |  | Standard |  |  |  | Extended |  |  |  |
|  |  |  | Display | Tracking |  |  | Display | Tracking |  |  | Display |  | Tracking |
| In range (targets aligned in azimuth) |  | Short range applications (< 5 NM coverage - Inland waterways, harbours etc) | 25 m | 40 m |  |  | 20 m | 30 m |  |  | 15 m |  | 25 m |
|  |  | Long range applications (up to 20 NM coverage - littoral waters, off-shore etc.) | 75 m | 100 m |  |  | 60 m | 75 m |  |  | 50 m |  | 60 m |
|  |  | Very long range applications (> 20 NM coverage) | N/A |  |  |  | 100 m | 125 m |  |  | 80 m |  | 100 m |
| In azimuth (targets aligned in range) | X-band | Angle between targets as seen from the radar | 1.2 |  | 1,3  |  | 0.7 | 0.8 |  |  | 0.55 |  | 0.6 |
|  |  | Or distance in metres, whichever is the greater | 25 m |  | 40 m |  | 20 m | 30 m |  |  | 15 m |  | 25 m |
|  |  | Corresponding - 3 dB antenna horizontal beam width | 0.6 - 0.7 |  |  |  | 0.4 - 0.45 |  |  |  | 0.35 - 0.4 |  |  |
|  | S-band | Angle between targets as seen from the radar | N/A |  |  |  | 3.5 |  | 4 |  | 1.8 | 2 |  |
|  |  | Or distance in metres, whichever is the greater | N/A |  |  |  | 20 m |  | 30 m |  | 15 m | 25 m |  |
|  |  | Corresponding - 3 dB antenna horizontal beam width | N/A |  |  |  | 1.8 - 2 |  |  |  | 1-1.25 |  |  |

## Plot Extraction and Tracking

### Plot Extraction

Plot extraction (see section 2.2.6) shall be automatic.  The plot extraction process shall be able to handle a minimum of plots per rotation (see Table 3.7).

### Track Initiation

Track initiation should be automatic, automatic except in selected areas, automatic in selected areas, or manual depending on the concept of operations.

In automatic track initiation modes, all plots in a scan shall be considered potential targets.  Some of the plots will be associated with previously established tracks, while the remaining plots shall be considered as candidates for new tracks, i.e. tentative tracks.

Tentative tracks will become confirmed tracks if plots from consecutive scans “fit into the picture” within reasonable physical manoeuvrability limits, otherwise the tentative tracks are discarded.

The tracking system shall be able to handle at least a certain number of tentative tracks and to initiate tracks and eventually to confirm tracks under certain conditions of PD  and PFA.

It shall also be possible to initiate a track manually.  In manual track initiation a plot on the radar display is selected by the operator using a graphical tool.  When selected this plot shall form the starting point for a tentative track which eventually shall be confirmed or discarded, as in the automatic case described above.

### Maintaining Track

If automatically or manually created tentative tracks persist over a certain length of time the tracks shall be promoted to confirmed tracks.  Confirmed tracks shall be shown on the display.  The tracking system shall be able to handle at least a certain number of confirmed tracks (Table 3.7) and to maintain tracks under certain conditions of PD (Table 3.8) and PFA ≤ 0.01.

### Track Termination

If a confirmed track either moves outside a user defined maximum range, into a user defined non-tracking area, or if the quality of the track falls below a predefined minimum, or if the track cannot be updated with new plots over a certain length of time, then the track shall be terminated.  In certain cases the operator shall receive a warning as defined by the VTS Authority.

### Track Data Output

Track data output is contained in track records.  A track record shall contain at least the following data:

- Track identification: an automatically generated integer or alphanumerical label;

- Current track date and time: UTC or GPS date and time and/or local date and time;

- Current target position: Numerous formats: geographical (Latitude, Longitude), UTM (Northing, Easting), local (Range, Bearing);

- Current target speed;

- Current target course: in degrees relative to North.

The track record shall be extendable with user-defined information.  It shall be possible to save track record history lists.

### Plot extraction and tracking performance

The requirements in respect of plot extraction and tracking shall be defined by the individual VTS authority, on the basis of local conditions, number of radar sensors in a system etc.  Table 3.7 suggests values for each individual radar sensor in a system.

Table 3.7  Single radar tracking performance parameters.

| Plot Extraction and Tracking Performance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter |  | Recommendation level |  |  |
|  |  | Relaxed | Standard | Extended |
| Number of plots per antenna rotation |  |  1000 |  2500 |  5000 |
| Number of confirmed tracks |  |  100 |  200 |  300 |
| Time for confirmation of tentative track |  |  1 minutes |  |  |
| Time from track confirmation to achievement of specified accuracy |  |  2 minutes |  |  |
| Time from data loss to automatic track termination |  |  1 minutes |  |  |
| Speed of tracked objects |  |  50 knots |  | 70 knots |
| Turn rate of tracked objects |  |  10°/second |  |  20°/second |
| Accuracy in track position | Range |  0.75 % of range covered by the individual radar or 10m + selected pulse length, whichever is the greater |  |  0.5 % of range covered or 5m + pulse length |
|  | Bearing a |  1, X-band  2, S-band |  |  0.5, X-band  1, S-band |
| Accuracy of track data | Speed a |  2 knots |  1 knot |  1 knot |
|  | Course a |  5° |  2° |  2° |

#### Track initiation and track maintenance

The radar PD should be adapted to the role of the VTS.  The automatic track initiation and track maintenance is optimised accordingly.

Recommendation for the minimum radar PD for track initiation is given in Table 3.8.  For track maintenance a lower minimum radar PD can apply, depending on the tracking principles used by the manufacturers.

Table 3.8  Track initiation.

| Minimum radar PD for track initiation |  |  |  |
| --- | --- | --- | --- |
| Priority of the VTS | Recommendation level |  |  |
|  | Relaxed | Standard | Extended |
| Surveillance and/or traffic monitoring | 0.9 | 0.8 | 0.7 |
| Safety | 0.9 |  |  |

#### False tracks

False tracks may appear as a result of noise, clutter (including wakes) and ghost echoes.  However, the number should not be significant if the recommended values given in Table 3.7 and Table 3.9 are respected.

The maximum number of false tracks allowed is dependant on role of the VTS.  False tracks should be avoided in safety critical areas and occasionally accepted in areas where surveillance and traffic monitoring is the priority.

There is a trade-off between the time for confirmation of tentative track and the number of false tracks.  A longer confirmation time implies less false tracks and it shall be possible to balance this trade-off in the setup of the VTS.

#### Track loss

Track loss may occur as a result of PD < 1 in combination with targets manoeuvring, especially in the vicinity of obstructions such as bridges.

A level generally accepted is that each operator shall correct up to one track loss per hour in all areas where the recommended values given in Table 3.7 and Table 3.8 are respected.

The VTS authority shall address critical areas, such as the vicinity of bridges, and explain expectations to tracking to allow VTS suppliers to make solutions accordingly.

#### Track swap

Swapping of track identity may occur as a result of targets moving close together or even merging for a period of time, especially if targets are overtaking with small difference in speed and course.

A simple method of manual correction shall be employed.

In the case of AIS information being available for the radar track(s) in question, automatic correction should be performed.

The problem may also be addressed by implementing operational procedures to separate targets or to prevent overtaking in critical areas.

## Reliability and availability

The VTS authority shall define the requirements for availability of radar services for any given area in a VTS and thereby the requirements for reliability.

The availability is given by the equation:

Availability = (service time - out of service time) / service time

The out of service time includes the time to repair as well as access time.

Refer to the IALA aids to navigation guide, chapter 11 for methods of measurement of availability and further definitions of terms.

## Redundancy

Table 3.9 specifies the recommended availability for individual radar sites and suggests the redundancy typically employed.

Table 3.9  Redundancy.

| Availability and redundancy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Recommendation level |  |  |  |  |  |
|  |  | Relaxed |  | Standard |  | Extended |  |
| Recommended availability for the individual radar site |  | 99 % |  | 99.7% |  | 99.9% |  |
| Accessibility to site |  | Easy | Difficult | Easy | Difficult | Easy | Difficult |
| Redundancy typically required | Yes |  |  |  | X | X | X |
|  | No | X | X | X |  |  |  |

Overlap in coverage may reduce the need for equipment redundancies at the individual radar site.

## Equipment standards and specification levels

International, and in some cases national standards, apply for radar and associated equipment.  Common standards and suggested specification levels are listed in Table 3.9.

Table 3.10  Equipment standards and specification levels.

| Standard | Subject | Condition | Suggested limits versus equipment location |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Conditioned indoor, office or equipment room | Outdoor |  |  |  |  |
|  |  |  |  | Hot dessert climate | Tropical climate | Subtropical climate | Temperate climate | Arctic climate |
| IEC 68-2 | Dry Heat | Function | 40C | 60C | 55C | 50C |  |  |
|  | Cold | Function | 10C | 0C |  | - 25C |  | -40C |
|  |  | Storage | - 25C |  |  |  |  | -40C |
| IEC 529 | Protection | Storage and function | IP 20 | IP 54 |  |  |  |  |
| IEC 950, UL, CSA | Safety | Storage and function | Refer to standards |  |  |  |  |  |
| ITU-R SM.1541 | Unwanted emissions in the out-of-band domain | Function | Refer to standards, same values apply worldwide independent on climate |  |  |  |  |  |
| ITU-R SM.329-9 | Spurious emissions | Function |  |  |  |  |  |  |

# Performance verification methods

The dynamic characteristics and dependence on propagation conditions adds uncertainty to tests on installed radar systems and it can be difficult to reproduce tests as conditions may vary substantially from day to day.  In addition the complexity of radar systems call for a stepwise approach.  Therefore the combination of tests and calculations listed in Table 4.11 is recommended.

Table 4.11  Performance verification methods.

|  | Design / verification method |  |  |  |
| --- | --- | --- | --- | --- |
|  | During system design | Factory Acceptance Test | Site Acceptance Test | In Operation Test |
| Overall functionality | Analysis | Functional test | Functional test |  |
| Coverage | Analysis including weather influence and propagation |  | Functional test |  |
| Detection performance and dynamic requirements | Simulation including weather influence and propagation | Measurement of antenna and transmitter-receiver parameters, losses etc. | Functional evaluation, compared to weather condition | Analysis of statistical data from the system |
| Target separation | Analysis | Measurement of individual antenna and transceiver parameters | Check using known structures or controlled targets. |  |
| Tracking performance | Analysis or Simulation |  | Functional test Analysis of statistical data from the system | Analysis of statistical data from the system |
| Environmental and Safety issues | Analysis and type tests | Check of workmanship and measurement of parameters | Check of Installation |  |
| Availability | Analysis |  |  | Record and analysis of events |

Within one standard deviation (Gaussian distribution) when sailing on a straight course.  Note that verification may require simulated tracks or other methods due to the fact that it may be impossible to direct a test target to sail with sufficient accuracy.

Air conditioned, ventilated or heated to temperatures between 15C and 30C and with less than 90 % humidity.  If these levels cannot be met, requirements, including a reasonable margin, should change accordingly.

May in addition require special considerations in respect to sun radiation, dust (sand storms) and severely corrosive atmosphere.