Recommendation ITU-R M.2092-0+ (03/2017)

Technical characteristics for a VHF data exchange system in the VHF maritime mobile band ##### WORKING VERSION – BE WARNED !!! CHEERS, IALA WG3

M Series Mobile, radiodetermination, amateur and related satellite services

REVISON FILE NAME: 20170310_WD_WG3_Item_112_RossNoresworthy _REC-M.2092.docx

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2015

ITU 2015

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  M.2092-0,

Technical characteristics for a VHF data exchange system 
in the VHF maritime mobile band

(2015)

Scope

This Recommendation provides the technical characteristics of a VHF data exchange system (VDES) which integrates the functions of VHF data exchange (VDE), application specific messages (ASM) and the automatic identification system (AIS) in the VHF maritime mobile band (156.025-162.025 MHz).

Keywords

Maritime, VHF, VDES, ASM, data, exchange

Abbreviations/Glossary

3GPP	Third generation partnership project

ACK	Acknowledgement

ADDC	Assigned data transfer

ACPR 	Adjacent channel power ratio

AIS	Automatic identification system

AOS	Acquisition-of-signal

APSK	Amplitude phase shift keying

ARQ	Automatic repeat request

ARSC	Announcement response channel

ASC	Announcement signalling channel

ASM	Application-specific messages

AWGN	Additive white Gaussian noise

BBSC 	Bulletin board signalling channel

BCH	Bose Chaudhuri Hocquenghem, an error-correcting-code

BER	Bit error rate

BPSK	Binary phase shift keying

BT	Bandwidth-time

CEPT	European conference of postal and telecommunications administrations

CDMA	Code division multiple access

CG 	Coding gain

CIR	Carrier to interference ratio

C/M 	Carrier to multipath

CNR	Carrier to noise ratio

COMSTATE	Communication state

Control Station	Coast base station, satellite or other VDES station that is transmitting bulletin board and provides routing services to the maritime cloud

CPM	Continuous phase modulation

CQI	Channel quality indicator

CR	Code rate

CRC	Cyclic redundancy check

CRL	Configuration revision level

CS	Carrier sense

CIRM	Comité international radio maritime

CSTDMA 	Carrier sense time division multiple access

CW	Continuous wave

DA	Doherty amplifier

DLS	Data link service

DPD	Digital pre-distortion

EDN 	End delivery notification

EDF	End delivery failure

EIRP	Equivalent isotropic radiated power (e.i.r.p.)

ERP	Effective radiated power (e.r.p.)

ET	Envelope tracking

FATDMA	Fixed access time-division multiple access

FCS 	Frame check sequence

FEC 	Forward error correction

FIFO	First-in first-out

GMSK	Gaussian-filtered minimum shift keying

GNSS	Global navigation satellite system

HS	Hexslots

IALA	International association of marine aids to navigation and lighthouse authorities

ICAO	International civil aviation organization

ID	Identification

IEC	International electrotechnical commission

IMO	International maritime organization

IP	Internet protocol

ITDMA	Incremental time division multiple access

LC	Logical channels

LEO	Low-earth orbiting

LFSR	Linear feedback shift register

LME	Link management entity

LNA	Low noise amplifier

LOS	Loss-of-signal

LSB	Least significant bit

MEO	Medium-earth orbiting

MAC	Media access control

MCS 	Modulation and coding scheme

MDC 	Multicast data channel

MMSI	Maritime mobile service identity

MSB	Most significant bit

NF	Noise figure

NM	Nautical mile

NRZI	Non-return to zero inversion

OFDM 	Orthogonal frequency division multiplexing

OSI	Open systems interconnection

PAPR 	Peak to average power ratio

PC	Physical channels

PL	Physical layer

PFD	Power flux-density

ppm	parts per million

PSK	Phase shift keying

QAM	Quadrature amplitude modulation

QPSK	Quadrature phase shift keying

RADC	Random access short messaging channel

RATDMA 	Random access time-division multiple access

RAC 	Random access channel

RF 	Radio frequency

RSC	Recursive systematic convolutional

RQSC	Random access resource request

RR	Radio regulations

RSSI	Received signal strength indication

SCTDMA 	Slot carrier sense time division multiple access

SFTP	Secure file transfer protocol

SI	Selection interval

SMTP	Simple mail transfer protocol

SNMP	Simple network management protocol

SNR	Signal to noise ratio

SOLAS	Safety of life at sea convention

SOTDMA	Self-organized time division multiple access

SS	Spreading sequences

Sym	Symbol

SYNC	Synchronization

TBB	Terrestrial bulletin board

TBBSC 	Terrestrial bulletin board signalling channel

TDMA	Time division multiple access

UDC	Unicast data channel

UDP	User data protocol

UTC	Coordinated universal time

VDE	VHF data exchange

VDES	VHF data exchange system

VDE-SAT	VHF data exchange-satellite

VDL	VHF data link

VHF	Very high frequency

References

{RD-1}	ETSI EN 302 583 (V1.2.1) – Digital Video Broadcasting (DVB); Framing Structure, channel coding and modulation for Satellite Services to Handheld devices (SH) below 3 GHz.

{RD-2}	TM Synchronization and Channel Coding. Recommendation for Space Data System Standards, CCSDS 131.0-B-2. Blue Book. Issue 2. Washington, D.C.: CCSDS, August 2011.

{RD-3}	R. Mueller, On Random CDMA with Constant Envelope, ISIT 2011.

{RD-4}	Recommendation ITU-R P.372 – Radio Noise.

{RD-5}	Recommendation ITU-T V.42 (03/2002) – Series V: Data Communication over the Telephone Network – Error control – Error-correcting procedures for DCEs using asynchronous-synchronous conversions.

The ITU Radiocommunication Assembly,

considering

a)	that the International Maritime Organization (IMO) has a continuing requirement for a universal shipborne automatic identification system (AIS);

b)	that the use of a universal shipborne AIS allows efficient exchange of navigational data between ships and between ships and shore stations, thereby improving safety of navigation;

c)	that the VHF data exchange system (VDES) should use appropriate access schemes that ensure the protection of AIS while making efficient use of the spectrum and accommodate all users;

d)	that while AIS is used primarily for surveillance and safety of navigation purposes in ship-to-ship use, ship reporting and vessel traffic services applications, a growing need for other maritime safety related communications has developed;

e)	that the VDES shall give priority to AIS, and also accommodate future expansion in the number of users and diversification of data communications applications, including vessels which are not subject to IMO AIS carriage requirements, aids to navigation and search and rescue;

f)	that the VDES has data communications capacity and technical characteristics that support the harmonized collection, integration, exchange, presentation and analysis of marine information on board and ashore by electronic means to enhance berth to berth navigation and related services for safety and security at sea and protection of the marine environment,

recognizing

that the implementation of VDES must ensure that the functions of digital selective calling, AIS and voice distress, safety and calling communication (Channel 16), are not impaired,

noting

that Report ITU-R M.2371 describes the use cases and requirement for VDES,

recommends

1	that VDES should be designed in accordance with the operational characteristics given in Annex 1 and the technical characteristics and examples given in Annexes 2 to 7;

2	that applications of the VDES which make use of application specific messages (ASM) designed for AIS, as defined in Recommendation ITU-R M.1371 should also take into account the international application identifier branch, as specified in IMO SN.1/Circ. 289, maintained and published by IMO;

3	that the design and installation of VDES should also consider relevant technical requirements, recommendations and guidelines published by IMO, IEC and IALA.

Annex 1

Operational characteristics of a VHF data exchange system 
in the VHF maritime mobile band

# 1	General

1.1	The system should give its highest priority to the automatic identification system (AIS) position reporting and safety related information.

1.2	The system installation should be capable of receiving and processing the digital messages and interrogating calls specified by this Recommendation.

1.3	The system should be capable of transmitting additional safety information on request.

1.4	The system installation should be able to operate continuously while under way, moored or at anchor.

1.5	The system should use for the terrestrial links time-division multiple access (TDMA) techniques, access schemes and data transmission methods in a synchronized manner as specified in the Annexes.

1.6	The system should be capable of various modes of operation, including the autonomous, assigned and polled modes.

1.7	The system should provide flexibility for the users in order to prioritize some applications and, consequently, adapt some parameters of the transmission (robustness or capacity) while minimizing system complexity.

1.8	The system should address the use cases identified in Report ITU-R M.2371.

# 2	General description of the VDES

# <TBD item112: ROSS to provide chapter >

# The VDES provides a variety of means for the exchange of data between maritime stations, ship-to-ship, ship-to-shore, shore-to ship, ship-to-satellite and satellite-to-ship. The VDES functionally includes the AIS, either by integration, by interface connection or by radio frequency connection.

# 3	VHF data exchange system functions and frequency usage

<TBD item112: ROSS to provide chapter>

The VDES functions and frequency useage are illustrated pictorially in Figure A1-1.

Figure A1-1

VHF data exchange system functions and frequency usage

NOTE – SAT Up is receive-only by satellite.

## 3.1	VHF data exchange system channel usage in accordance with RR Appendix 18

### 3.1.1	VHF data exchange system: data exchange between terrestrial stations

–	AIS 1 (channel 2087) and AIS 2 (channel 2088) are AIS channels, in accordance with Recommendation ITU-R M.1371

–	ASM 1 (channel 2027) and ASM 2 (channel 2028) are the channels used for application specific messages (ASM)

–	VDE1-A lower legs (channels 1024, 1084, 1025, 1085) are ship-to-shore VDE

–	VDE1-B upper legs (channels 2024, 2084, 2025, 2085) are shore-to-ship and ship-to-ship VDE.

### 3.1.2	VHF data exchange system: data exchange between satellites and terrestrial stations

–	AIS 1 (channel 2087) and AIS 2 (channel 2088) are terrestrial AIS channels that are also used as uplinks for receiving AIS messages by satellite

–	Long Range AIS using channel 75 and channel 76 are specified channels to be used as uplinks for receiving AIS messages by satellite. SAT Up1 (channel 2027) and SAT Up 2 (channel 2028) are used for receiving ASM by satellite

–	SAT Up3 (channels 1024, 1084, 1025, 1085, 1026 and 1086) are used for ship-to-satellite VDE uplinks

–	SAT Downlink (channels 2024, 2084, 2025, 2085, 2026 and 2086) are used for satellite-to-ship VDE downlinks.

## 3.2	Identification <TODO item 112: ROSS to incorporate this moved chapter>

Identification and location of all active maritime stations is provided automatically by means of the AIS. All VDES stations should be uniquely identified. For the purpose of identification, the appropriate numerical identifier, for example maritime mobile service identity (MMSI), should be used, as defined in the latest version of Recommendation ITU-R M.585. Recommendation ITU-R M.1080 should not be applied with respect to the 10th digit (least significant digit).

## 3.3	Presentation interface protocol <TODO item 112: ROSS to incorporate this moved chapter>

For VDES transceivers:

–	data may be input via the presentation interface to be transmitted by the VDES station;

–	data received by the VDES station should be output through the presentation interface.

### 3.4	Technical characteristics

#### 3.4.1	Shipborne VHF data exchange system receivers are protected

As in AIS, shipborne VDES receivers are on the upper legs of RR Appendix 18, 4.6 MHz above the lower legs, which facilitates protection by filtering from receiver blocking by ships VHF radios.

#### 3.4.2	SAT Downlink

The satellite downlink complies with the power flux-density (PFD) mask described in Table A4-1 to minimize interference to terrestrial services and to maximize reception by ship VDES stations.

#### 3.4.3	VDE1 uses both legs of the duplex channels

Channel capacity is utilized for the duplex channels in VDE1 by using the lower legs (VDE1-A) for ship-to-shore and the upper legs (VDE1-B) for shore-to-ship and ship-to-ship digital messaging.

Table A1-1 describes the RR Appendix 18 channels used for the various applications of VDES.

TABLE A1-1

RR Appendix 18 channels for VHF data exchange systems applications: Automatic identification system, application specific messages, VHF data exchange

| RR Appendix 18 channel number |  | Transmitting frequencies (MHz) |  |
| --- | --- | --- | --- |
|  |  | Ship stations (ship-to-shore) (long range AIS) Ship stations (ship-to-satellite) | Coast stations Ship stations (ship-to-ship) Satellite-to-ship |
| AIS 1 |  | 161.975 | 161.975 |
| AIS 2 |  | 162.025 | 162.025 |
| 75 (long range AIS) |  | 156.775 (ships are Tx only) | N/A |
| 76 (long range AIS) |  | 156.825 (ships are Tx only) | N/A |
| 2027 (ASM 1) |  | 161.950 (2027) | 161.950 (2027) |
| 2028 (ASM 2) |  | 162.000 (2028) | 162.000 (2028) |
| 24/84/25/85 (VDE 1) 24 84 25 85 | 24/84/25/85/26/86 (Ship-to-satellite, satellite-to-ship) 24 84 25 85 26 86 | 100/150 kHz channel (24/84/25/85, lower legs (VDE1-A) merged) Ship-to-shore (24/84/25/85/26/86) Ship-to-satellite | 100/150 kHz channel (24/84/25/85, upper legs (VDE1-B) merged) Ship-to-ship, Shore-to-ship (24/84/25/85/26/86) Satellite-to-ship |
|  |  | 157.200 (1024) | 161.800 (2024) |
|  |  | 157.225 (1084) | 161.825 (2084) |
|  |  | 157.250 (1025) | 161.850 (2025) |
|  |  | 157.275 (1085) | 161.875 (2085) |
|  |  | 157.300 (1026) | 161.900 (2026) |
|  |  | 157.325 (1086) | 161.925 (2086) |

3.4.4	VHF data exchange system functions and frequency usage engineer’s perspective

The VDES functions and frequency useage from an engineer’s perspective are illustrated pictorially in Figure A1-2.

|  |  | - | - |
| --- | --- | --- | --- |

Figure A1-2

VHF data exchange system functions and frequency usage engineer’s perspective

# 4	Functions of the VDES

The priority and timing of transmissions shall be in accordance with the following service priorities:

Highest 	Priority 1: AIS transmissions on AIS channels

Priority 2: specified and approved ASM transmissions on ASM channels

Priority 3: all other data exchange on VDE channels

The VDES receivers shall always be active. It is understood that transmissions by own equipment will impair reception by own equipment on own ship.

The VDES should support the following:

## 4.1	Automatic Identification System

The AIS will operate as defined by Recommendation ITU-R M.1371.

## 4.2	Application-Specific Messages – Annex 2

Annex 2 describes the characteristics of the ASM channel that will support applications specific messages in order to improve the efficiency of application-specific message transmissions and to protect the original function of the AIS.

## 4.3	VDE terrestrial – Annex 3

Annex 3 describes the characteristics of the VDE terrestrial channels providing an efficient terrestrial data transfer link enabling a wide variety of applications for the maritime community.

## 4.4	VDE satellite – Annex 4

Annex 4 describes the characteristics of a satellite downlink that will support multi-cast multi-package data transfers and shore originated unicast multi-package data transfers via satellite.

## 4.6	VDES sharing options – Annex 6

Annex 6 describes the characteristics necessary for each component of the VDES to share the available spectrum such that impact between services is minimized and AIS is respected.

## 4.7	VDES original design considerations – Annex 7

Annex 7 is an informative annex that provides additional information on the technical consideration of the VDES. It identifies aspects of both terrestrial and satellite VDE components, including access scheme options, antenna designs, and system sharing.

# 5	Common technical elements of the VDES

# This section describes those elements of the VDES that may be common across the ASM and VDE channels.

## 5.1	Protocol layer overview

The VDES architecture should utilize the open systems interconnection layers 1 to 4 (physical layer, link layer, network layer, transport layer) as illustrated in Fig. A1-3.

Figure A1-3

Seven layer OSI model

Application layer

Presentation layer

Session layer

Transport layer

Network layer

Link layer

Physical layer

Responsibilities of the OSI layers for preparing VDES data for transmission:

### 5.2	Physical layer

This layer provides transmission and reception of raw bit streams over a physical medium including signal modulation, filtering/shaping upon transmission, and amplification, filtering, time and frequency synchronization, demodulation, and decoding upon reception.

#### 5.2.1	Transmission accuracy figures

#### 5.2.1.1	Symbol timing accuracy (at the output)

The timing accuracy of the transmit signal should be better than 5 ppm.

#### 5.2.1.2	Transmitter timing jitter

The timing jitter should be better than 5% of the symbol interval (peak value).

#### 5.2.1.3	 Slot transmission accuracy at the output

The slot transmission accuracy should be better than 100 µs peak relative to UTC time reference for ship stations.

## 5.2.2	Frame Structure

The system uses the Recommendation ITU-R M.1371 concept of a frame. A frame equals one (1) minute and is divided into 2 250 slots. Access to the data link is, by default, given at the start of a slot.  The VDES frame structure is identical and synchronized in time to UTC (as in AIS). The general slot format is shown in Fig. A1-4. Note that “Sync 2” and “Link Configuration Id” are not used by the SAT link. The slot structure for each service is shown in Fig. A1-5. Each element is described in the subsequent sections. Table A1-2 shows the resulting net bitrates (Data bits of the slot structure).

TABLE A1-2

Resulting net bitrates (Data)

| NET user data bitrate per channel [kbps] |  |  |  |
| --- | --- | --- | --- |
|  | Bandwidth |  |  |
|  | 25 kHz | 50 kHz | 100 kHz |
| VDE MCS-0 | 13 | n/a | n/a |
| VDE MCS-1 | 15 | 32 | 66 |
| VDE MCS-3 | 35 | 74 | 150 |
| VDE MCS-5 | 47 | 100 | 200 |

Figure A1-4

Figure A1-5

Slot structure

Training sequence

27 sym

Ramp up

4 symbols

Link CFG

ID

16 sym

Data with CRC-32

192, 448, 704, 960 symbols

Ramp Down

4 symbols

Guard

13   symbols

ASM

Bandwidth 16 kHz

9.6 ksym/s

26.67 ms

256 symbols

Training sequence

27 sym

Ramp up

16 symbols

Link CFG

ID

16 sym

Data with CRC-32

896 symbols

Ramp Down

16 symbols

Guard

53 symbols

VDE

Bandwidth 50 kHz

38.4 ksym/s

1024 symbols

26.67 ms

Training sequence

27 sym

Ramp up

32 symbols

Link CFG

ID

16 sym

Data with CRC-32

1792 symbols

Ramp Down

32 symbols

Guard

[149] symbols

VDE

Bandwidth 100 kHz

76.8 ksym/s

2048 symbols

26.67 ms

Training sequence

27 sym

Ramp up

8 symbols

Link CFG

ID

16 sym

Data with CRC-32

432 symbols

Ramp Down

8 symbols

Guard

21 symbols

VDE

Bandwidth 25 kHz

19.2 ksym/s

512 symbols

26.67 ms

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

27 sym

Ramp up

4 symbols

Link CFG

ID

16 sym

Data with CRC-32

192, 448, 704, 960 symbols

Ramp Down

4 symbols

Guard

13   symbols

ASM

Bandwidth 16 kHz

9.6 ksym/s

26.67 ms

256 symbols

Training sequence

27 sym

Ramp up

16 symbols

Link CFG

ID

16 sym

Data with CRC-32

896 symbols

Ramp Down

16 symbols

Guard

53 symbols

VDE

Bandwidth 50 kHz

38.4 ksym/s

1024 symbols

26.67 ms

Training sequence

27 sym

Ramp up

32 symbols

Link CFG

ID

16 sym

Data with CRC-32

1792 symbols

Ramp Down

32 symbols

Guard

[149] symbols

VDE

Bandwidth 100 kHz

76.8 ksym/s

2048 symbols

26.67 ms

Training sequence

27 sym

Ramp up

8 symbols

Link CFG

ID

16 sym

Data with CRC-32

432 symbols

Ramp Down

8 symbols

Guard

21 symbols

VDE

Bandwidth 25 kHz

19.2 ksym/s

512 symbols

26.67 ms

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

### 5.2.2.1	Frame hierarchy definition

The frame hierarchy is shown in Fig. A1-6. The frame hierarchy definition is independent of the assigned bandwidth to the VDE channel

Figure A1-6

Frame Hierarchy for shared frequency

#### 5.2.2.1.1	Slot

The slot is a time interval of approximately 26.667 ms (60 000 / 2 250 = 80/3 ≈ 26.667).

The slot should be numbered by a slot number. The slot should be cyclically numbered from 0 to 5.

#### 5.2.2.1.2	Hexslot

Six slots should form a Hexslot (HS). The HS has duration of 160 ms.

The HS should be numbered cyclically from 0 to 4. The HS should be incremented after every 6 slots.

#### 5.2.2.1.4	Uberslot

Five Hexslots should form a Uberslot (US). The US should have duration of 800 ms.

The US should be numbered by a US Number. The US should be cyclically numbered from 0 to 14. The US should be incremented whenever the Hexslot returns to 0.

#### 5.2.2.1.5	Sub frame

Fifteen US should form a sub frame. The sub frame should have a duration of 12 seconds. The sub frame should be numbered by a sub frame number. The PL-frame should be cyclically numbered from 0 to 4. The sub frame should be incremented whenever the US returns to 0.

### 5.2.3	Burst transmission structure

### 5.2.3.1	Ramp up

The ramp up time from −50 dBc to −1.5 dBc of the power shall controlled rise time and occur in 416 µs. A gradual ramp-up period provides important spectral shaping to reduce energy spread outside the desired signal modulation bandwidth, and reduces interference to other users of the current and adjacent channel.

### 5.2.3.2	Training sequence

The training sequence is separated in to two sections with the SAT link only using the first 14 symbols and is defined as 111111001101010000011001010.

#### 5.2.3.3	Bit mapping for training sequence and signal information

For training and signal information, following mapping applies:

–	1 maps to π/4 QPSK symbol 3 (1, 1) (see Figure A1-9)

–	0 maps to π/4 QPSK symbol 0 (0, 0).

For π/4 QPSK bit mapping, see § 5.2.8.

### 5.2.3.4	Link Configuration Id

The Link Configuration Id defines the channel configurations.  The Link Configuration Id is used to index the table of channel configurations, see Table [Bulletin Board channel configs. “ENAV19-Bulletingboard-June2016_v5.3” Table 8].

The Link Configuration Id follows the training sequence for transmissions, see Figure A1-5.

The Link Configuration Id consists of 6 bits (D0, D1, D2, D3, D4, D5) encoded into a sequence of 32 bits using biorthogonal (32,6) code.

The link configuration id is not used by the SAT link.

### 5.2.3.5	Data with CRC-32

The data payload with its appended CRC-32 is interleaved (refer to Table A1-3), encoded (refer to § 5.2.4.1) and then scrambled (refer § 5.2.6) and bit mapped.

Unused payload data is zero-filled.

Needs to be VDES specific – further editing

### 5.2.3.6	Bit scrambling

Scrambling of the user data is required to avoid the power spectral density to be concentrated in the narrow band. Refer to § 5.2.6 for the detailed definition of the scrambler sequence.

### 5.2.3.7	Guard time

The guard time consists of the ramp down time from full power to −50 dBc of less than or equal to 416 µs. The remaining time is for delay and jitter.

## 5.2.4	Forward Error Correction

### 5.2.4.1	Encoder Structure

This paragraph defines the general structure of the forward error correction encoder to be used on the satellite and the terrestrial component of the VDES. The overall structure follows the specification in the ETSI EN 302 583 standard {RD-1}.

The general encoder structure is depicted in Fig. A1-7. The encoder consists of two recursive systematic convolutional (RSC) encoders concatenated in parallel. Each encoder produces 3 output bits per input bit. The first RSC encoder produces the bits X, Y0 and Y1, while the second encoder produces the bits X’, Y’0 and Y’1.

The first encoder gets as input a word u of k bits, with k, as specified in § 5.2.4.3. The second encoder input is denoted by u’ and it is a permuted version of the vector u. The permutation is performed according to the definition provided in § 3.5.3 below.

Figure A1-7

Turbo encoder structure (high-level)

### 5.2.4.2	Constituent codes

The constituent codes are specified by the transfer function

GD=1n0(D)d(D)n1(D)d(D)

where

n0D=1+D+D3

n1D=1+D+D2+D3

dD=1+D2+D3.

The constituted encoder definition is provided in Fig. A1-8. For the first k clocks the switch is in position (a), i.e. information is fed into the encoder. For the subsequent 6 clocks, the switch is moved to position (b) to handle the RSC trellis termination. In the first 3 termination clocks, only the RSC 1 (upper branch) is output, while in the subsequent 3 termination clocks, only the output of RSC 2 (lower branch) is provided. The termination is thus given by the sequence of 6 termination bits (X, Y0, Y1, X’, Y’0, Y’1) with X output first.

Figure A1-8

RSC code encoder

### 5.2.4.3	Interleaver definition

The interleaver specification follows {RD-2}.

First factorize k = k1 k2, where the parameters k1 and k2 depend on the choice of the respective code, where k is the information block length. The values are given in Table A1-3.

TABLE A1-3

Interleaver parameters for different information lengths/code rates

| Nominal code rate | Information length | k1¦k2 | Annex | MCS | BW [kHz] |
| --- | --- | --- | --- | --- | --- |
| 1/4 | 23552 | 8¦2944 | 4,5 | TBD | TBD |
| 1/3 | 128 | 2¦64 | 4,5 | TBD | TBD |
| 1/2 | 1920 | 4¦480 | 4,5 | TBD | TBD |
| 1/2 | 23056 | 8¦2882 | 4,5 | TBD | TBD |
| 3/4 | 136 | 2¦68 | 4,5 | TBD | TBD |
| 3/4 | 296 | 2¦148 | 4,5 | TBD | TBD |
| 3/4 | 32800 | 10¦3280 | 4,5 | TBD | TBD |
| ½ | 432 | 2¦216 | 3 | MCS-1 | 25 |
| ½ | 896 | 2¦448 | 3 | MCS-1 | 50 |
| ½ | 1792 | 4¦448 | 3 | MCS-1 | 100 |
| ¾ | 972 | 2¦486 | 3 | MCS-3 | 25 |
| ¾ | 1296 | 2¦648 | 3 | MCS-5 | 25 |
| ¾ | 2016 | 4¦504 | 3 | MCS-3 | 50 |
| ¾ | 2688 | 4¦672 | 3 | MCS-5 | 50 |
| ¾ | 4032 | 4¦1008 | 3 | MCS-3 | 100 |
| ¾ | 5376 | 8¦672 | 3 | MCS-5 | 100 |

This Table will be extended as different information block lengths are defined.

This FEC will be calculated by first choosing prime numbers pq, q  (1, ..., 8)

p1 = 31

p2 = 37

p3 = 43

p4 = 47

p5 = 53

p6 = 59

p7 = 61

p8 = 67

The following operations shall be performed for s  (1, ..., k) to obtain the permutation numbers π(s):

m=s-1 mod 2

i=floor (s-1 /(2k2))

j=floor (s-1 /2))-ik2

t=19i+1 mod (k1/2)

q=t mod 8+1

c=pqj+21m mod k2

πs=2t+ck1/2+1-m

The permutation numbers shall be interpreted such that the sth bit read out after interleaving is the π(s)th bit of the input information block.

### 5.2.4.4	Rate Adaptation

Rate adaptation is obtained by puncturing the encoder output as in § 5.3.1 of {RD-1}, as recalled in Table A1-4 for the first k clocks, and as in {RD-1}.

The puncturing table for the termination part is given in Table A1-7. The last row of the Table is note part of {RD-1}.

Table A1-4 is for the terminations. The last row with ID 8 is introduced in this fdocument to obtain higher rates and is not part of {RD-1}.

TABLE A1-4

Puncturing patterns for data bit periods

| Punc. Pattern ID | Code Rate | Punc. Pattern (X; Y0; Y1; X’; Y’0; Y’1 ¦ X; Y0; Y1; X’; Y’0; Y’1 ¦ …) |
| --- | --- | --- |
| 0 | 1/5 | 1;1;1;0;1;1 |
| 1 | 2/9 | 1;0;1;0;1;1 ¦ 1;1;1;0;1;1 ¦ 1;1;1;0;0;1 ¦ 1;1;1;0;1;1 |
| 2 | 1/4 | 1;1;1;0;0;1 ¦ 1;1;0;0;1;1 |
| 3 | 2/7 | 1;0;1;0;0;1 ¦ 1;0;1;0;1;1 ¦ 1;0;1;0;0;1 ¦ 1;1;1;0;0;1 |
| 4 | 1/3 | 1;1;0;0;1;0 |
| 5 | 2/5 | 1;0;0;0;0;0 ¦ 1;0;1;0;0;1 ¦ 0;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 0;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 0;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 0;0;1;0;0;1 |
| 6 | 1/2 | 1;1;0;0;0;0 ¦ 1;0;0;0;1;0 |
| 7 | 2/3 | 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;1;0;0;1 |
| 8 | 3/4 | 1;0;1;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;1 |
| For each rate, the puncturing table shall be read first from left to right and then from top to bottom. |  |  |

Within a puncturing pattern, a ‘0’ means that the symbol shall be deleted and a ‘1’ means that a symbol shall be passed. A ‘2’ or a ‘3’ means that two or three copies of the symbol shall be passed. This is relevant for the termination periods. In particular

–	For the rate 1/5 turbo code (Punct_Pat_ID=0), the tail output symbols for each of the first three tail bit periods shall be XXXY0Y1, and the tail output symbols for each of the last three tail bit periods shall be X’X’X’Y’0Y’1.

–	For the rate 2/9 turbo code (Punct_Pat_ID=1), the tail output symbols for the first and the second output period shall be XXXY0Y1, for the third output period XXY0Y1, for the fourth and fifth output period X’X’Y’0Y’1, and for the sixth (last) output period X’X’X’Y’0Y’1.

–	For the rate 1/4 turbo code (Punct_Pat_ID=2), the tail output symbols for each of the first three tail bit periods shall be XXY0Y1, and the tail output symbols for each of the last three tail bit periods shall be X’X’ Y’0Y’1.

All other code rates shall be processed similar to the given examples above with the exact puncturing patterns to be derived from {RD-1}.

The puncturing table for the termination part is given in Table A1-5. The last row of the table is introduced in this document to obtain higher rates and is not part of {RD-1}.

TABLE A1-5

Puncturing and repetition patterns for tail bit periods (last 6 clocks)

| Punct. Pattern ID | Code Rate | Punct. / Rep. Pattern (X; Y0; Y1; X’; Y’0; Y’1 ¦ X; Y0; Y1; X’; Y’0; Y’1 ¦ …) |
| --- | --- | --- |
| 0 | 1/5 | 3;1;1;0;0;0 ¦ 3;1;1;0;0;0 ¦ 3;1;1;0;0;0 ¦ 0;0;0;3;1;1 ¦ 0;0;0;3;1;1 ¦ 0;0;0;3;1;1 |
| 1 | 2/9 | 3;1;1;0;0;0 ¦ 3;1;1;0;0;0 ¦ 2;1;1;0;0;0 ¦ 0;0;0;2;1;1 ¦ 0;0;0;2;1;1 ¦ 0;0;0;3;1;1 |
| 2 | 1/4 | 2;1;1;0;0;0 ¦ 2;1;1;0;0;0 ¦ 2;1;1;0;0;0 ¦ 0;0;0;2;1;1 ¦ 0;0;0;2;1;1 ¦ 0;0;0;2;1;1 |
| 3 | 2/7 | 1;1;1;0;0;0 ¦ 2;1;1;0;0;0 ¦ 2;1;1;0;0;0 ¦ 0;0;0;2;1;1 ¦ 0;0;0;1;1;1 ¦ 0;0;0;1;1;1 |
| 4 | 1/3 | 2;1;0;0;0;0 ¦ 2;1;0;0;0;0 ¦ 2;1;0;0;0;0 ¦ 0;0;0;2;1;0 ¦ 0;0;0;2;1;0 ¦ 0;0;0;2;1;0 |
| 5 | 2/5 | 1;1;1;0;0;0 ¦ 1;1;1;0;0;0 ¦ 1;0;1;0;0;0 ¦ 0;0;0;1;1;1 ¦ 0;0;0;1;1;1 ¦ 0;0;0;1;0;1 |
| 6 | 1/2 | 1;1;0;0;0;0 ¦ 1;1;0;0;0;0 ¦ 1;1;0;0;0;0 ¦ 0;0;0;1;1;0 ¦ 0;0;0;1;1;0 ¦ 0;0;0;1;1;0 |
| 7 | 2/3 | 1;0;0;0;0;0 ¦ 1;0;1;0;0;0 ¦ 1;0;1;0;0;0 ¦ 0;0;0;1;0;0 ¦ 0;0;0;1;0;1 ¦ 0;0;0;1;0;1 |
| 8 | 3/4 | 1;0;1;0;0;0 ¦ 1;0;1;0;0;0 ¦ 1;0;1;0;0;0 ¦ 0;0;0;1;0;1 ¦ 0;0;0;1;0;1 ¦ 0;0;0;1;0;1 |
| For each rate, the puncturing table shall be read first from left to right and then from top to bottom. |  |  |

## 5.2.5	CRC

The 32 bit ITU-T V.42 {RD-5} polynomial 0x04C11DB7 CRC is appended to the last segment of the datagram. The CRC is calculated over all fragments of the datagram.

F(x) = x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1

Initial state: 0xFFFFFFFF

## 5.2.6	Bit scrambling

The bit scrambler shown Figure A1-9 uses the polynom:

F(x)=1 + x-14 + x-15

and the initialization sequence as indicated in the top of the figure. For each new encoding block, the bit scrambler is re-initialized.

FIGURE A1-9

Bit scrambling

## 5.2.7	Modulation Coding Schemes

The modulation and coding options and raw channel throughput rates are provided for a range of bandwidths and modulation and coding schemes (MCS).

TABLE A1-6

Modulation and coding schemes

| Annex |  |  | 2 ASM | 3 VDE-TER | 3 VDE-TER | 3 VDE-TER |
| --- | --- | --- | --- | --- | --- | --- |
| Channel Bandwidth (kHz) |  |  | 25 | 25 | 50 | 100 |
| Symbol Rate |  |  | 9.6 | 19.2 | 38.4 | 76.8 |
| Rolloff factor |  |  | 0.35 | 0.3 | 0.3 | 0.3 |
| Modulation and coding scheme | Signal info | CQI | Total throughput kbit/s |  |  |  |
| MCS-0 (π/4 QPSK, no coding) | 0, 0, 0, 0 | 0 | 19.2 | future | future | future |
| MCS-1 (π/4 QPSK, CR = 1/2) | 0, 0, 0, 1 | 1 | TBD | 38.4 | 76.8 | 153.6 |
| MCS-3 (8PSK, CR = 3/4) | 0, 0, 1, 1 | 3 | Future | 57.6 | 115.2 | 230.4 |
| MCS-5 (16QAM, CR = 3/4) | 0, 1, 0, 1 | 5 | Future | 76.8 | 153.6 | 307.2 |

The unused MCS are reserved for future use. A maximum of 16 different MCS can be defined for each physical layer format.

## 5.2.8 	Bit mapping

The bit mappings used throughout the Annexes are shown in figures Figs. A1-7 to A1-10.

Figure A1-7

Bit Mapping for π/4 QPSK

NOTE – Odd symbols in a packet shall be selected from the constellation defined by points ±1±j/2 (shown in green); even symbols shall be selected from the constellation defined by points ±1, ±j (shown in purple).

Figure A1-8

8PSK symbol to bit mapping

Figure A1-9

Bit Mapping for 16QAM

Figure A1-10

16APSK bit to symbol mapping

The 16 APSK modulation constellation is composed of two concentric rings of uniformly spaced 4 and 12 PSK points, respectively in the inner ring of radius R1 and outer ring of radius R2.

The ratio of the outer circle radius to the inner circle radius (γ =R2/R1) shall be equal to 3. R1 shall be set to 1/√7, R2 shall be set to 3/√7 in order to have the average signal energy equal to 1.

Similar to AIS, when data is output on the VHF data link it should be grouped in bytes of 8 bits from top to bottom of the table associated with each message in accordance with ISO/IEC 13239:2002. Each byte should be output with least significant bit first.

## 5.2.9	Antenna gain for VDES ship stations

Existing ship antennas may be used for VDES. The maximum antenna gain for these antennas ranges from 2 dBi to 10 dBi. Representative antenna patterns are shown in Fig. A1-11.

A ship antenna with a minimum gain at 0 degrees elevation of 3 dBi at the receiver input is required.

Figure A1-11

Ship antenna gain vs. elevation angle

### 5.2.10	Noise and interference level

The noise floor is a function of many sources such as vessel electronics, other radio equipment, power supplies, etc., and sensitivity is also reduced by RF cabling losses and the LNA noise figure. Table A1-10 presents representative values for the receiver noise figure.

TABLE A1-10

Ship receiver noise figure calculations

| Antenna noise temperature* | 245.0 | K |
| --- | --- | --- |
| LNA noise figure | 6.0 | dB |
| LNA noise temperature | 813.8 | K |
| Feed loss noise temp at LNA | 0.0 | K |
| Antenna noise temp at LNA | 245.0 | K |
| System noise temp at LNA | 1058.8 | K |
| System noise temp at LNA | 30.2 | dBK |
| * The galactic background antenna noise temperature is 245 K at 160 MHz {RD-4}. |  |  |

## 5.2.11	Transmitter requirements for VDES

### 5.2.11.1	Transmitter power

Except for Annex 2, Table A1-12 defines the requirements for the VDES ship station transmitters.

TABLE A1-12

Transmitter parameters

| Transmitter parameters | Requirements | Condition |
| --- | --- | --- |
| Frequency error | 3 ppm | normal |
| Maximum transmit power capability | For ship stations: Transmit average power should be at least 1 watt and not exceed 25 watts at the transmitter output. For shore stations: Transmit average power should be at least 12.5 watt and not exceed 50 watts at the base of the antenna. ±1.5 dB normal, +2/−6 dB extreme | Conducted |
| Maximum adjacent power levels for 25 kHz channel | 0 dBc −25 dBc −60 dBc | Δfc < ±12.5 kHz ±12.5 kHz < Δfc < ±25 kHz ±25 kHz < Δfc < ±75 kHz |
| Maximum adjacent power levels for 50 kHz channel | 0 dBc −25 dBc −60 dBc | Δfc < ±25 kHz ±25 kHz < Δfc < ±50 kHz ±50 kHz< Δfc < ±100 kHz |
| Maximum adjacent power levels for 100 kHz channel | 0 dBc −25 dBc −60 dBc | Δfc < ±50 kHz ±50 kHz < Δfc < ±100 kHz ±100 kHz < Δfc < ±150 kHz |
| Spurious emissions | −36 dBm −30 dBm | 9 kHz to 1 GHz 1 GHz to 4 GHz |

## 5.2.11.2	Ship e.i.r.p. vs. elevation angle

The minimum ship e.i.r.p. vs elevation angle is shown in Table A1-13. There are no minimum e.i.r.p. requirements above 80 degrees elevation. Table A1-13 is based on a linear transmitter meeting the maximum Adjacent Channel Interference levels defined in Table A1-12. For saturated operation the e.i.r.p. shall be 3 dB higher.

TABLE A1-13

Minimum ship e.i.r.p. vs. elevation angle

| Ship elevation angle | Ship antenna gain | Minimum ship e.i.r.p. with 6 W transmitter |
| --- | --- | --- |
| degrees | dBi | dBW |
| 0 | 3 | 10.8 |
| 10 | 3 | 10.8 |
| 20 | 2.5 | 10.3 |
| 30 | 1 | 8.8 |
| 40 | 0 | 7.8 |
| 50 | −1.5 | 6.3 |
| 60 | −3 | 4.8 |
| 70 | −4 | 3.8 |
| 80 | −10 | −2.2 |
| 90 | −20 | −12.2 |

## 5.2.11.3	Shutdown procedure

An automatic transmitter hardware shutdown procedure and indication should be provided in case a transmitter continues to transmit for more than 2 s. This shutdown procedure should be independent of software control.

## 5.2.11.4	Safety precautions

### The VDES installation, when operating, should not be damaged by the effects of open circuited or short circuited antenna terminals.

### 5.3	Link layer

This layer ensures reliable transmission of data frames between ships, ship and shore, and ship and satellite. The link layer is divided into three sub-layers with the following tasks:

#### 5.3.1	Link management entity

Assemble unique word, format header, Physical Layer Frame (PL-Frame) headers, pilot tones (satellite) and VDES message bits into packets.

#### 5.3.2	Data link services

Calculates and adds CRC check sum and completes the PL-Frame/packet.

#### 5.3.3	Media Access Control

Provides methods for granting data transfer access.

#### 5.3.4	Data encapsulation

The data segments of each PL-Frame contain multiple variable length encapsulated datagrams. Each datagram contains the following encapsulation fields:

–	Datagram type (1 byte)

–	Datagram size (3 bytes)

–	Ship ID (4 bytes)

–	Transaction ID (4 bytes, optional)

–	Datagram sequence number (2 bytes, for multisegment datagrams)

–	Source ID (8 bytes, optional)

–	Datagram payload (variable)

–	Data padding (variable, less than 8 bits)

–	CRC (4 bytes).

#### 5.3.5	Cyclic redundancy check

Refer to § 3.6.

#### 5.3.6	Automatic repeat request (ARQ)

Datagrams may or may not use ARQ, this is defined for each datagram type. An ARQ will request selective retransmission of a specific lost datagram segment.

#### 5.3.7	Acknowledgement (ACK)

All datagrams without CRC errors are acknowledged over the satellite link.

#### 5.3.8	End delivery notification (EDN)

All datagrams successfully delivered to the destination will be notified to the source.

#### 5.3.9	End delivery failure (EDF)

All datagrams not successfully delivered within the timeout or retry limit will be notified to the source.

#### 5.3.10	Physical and logical channels

[Content to be provided by Krzyzstof and Hans]

##### 5.3.10.1	Physical channels

[Content to be provided by Krzyzstof and Hans]

##### 5.3.10.2	Logical channels

[Content to be provided by Krzyzstof and Hans]

###### 5.3.10.2.1	Signalling logical channels

[Content to be provided by Krzyzstof and Hans]

###### 5.3.10.2.2	Bulletin board signalling channel

[Content to be provided by Krzyzstof and Hans]

###### 5.3.10.2.3	Announcement signalling channel (ASC)

[Content to be provided by Krzyzstof and Hans]

###### 5.3.10.2.4	Multicast data channel (MDC)

[Content to be provided by Krzyzstof and Hans]

###### 5.3.10.2.5	Unicast data channel (UDC)

[Content to be provided by Krzyzstof and Hans]

### 5.4	Network layer

This layer is responsible for the management of priority assignments of messages, distribution of transmission packets between channels and data link congestion resolution.

#### 5.4.1	Data transfer protocols

The following downlink protocols shall be supported:

- Control Station-to-ship; Bulletin board transmission

- Control Station-to-ship; Multicast

- Control Station-to-ship; Unicast

- Ship-to-ship; Multicast

- Ship-to-ship; Unicast

- Ship-to-Control Station single packet data transfer; Unicast

- Ship-to-Control Station multi-packet data transfer; Unicast

Figure A1-10

Bulletin board with network version change

Figure A1-11

Multicast protocol (one-way)

Figure A1-12

Shore originated unicast (file transfer) protocol

Figure A1-13

Shore originated poll protocol

Figure A1-14

Ship originated single packet data transfer

Figure A1-15

Ship originated multi-packet data transfer

### 5.5	Transport layer

This layer ensures reliable transmission of the data segments between ships, ship and shore, and ship and satellite, including segmentation, acknowledgement and multiplexing.

## 5.5.1	End to end protocols

Existing Internet protocols such as UDP, SNMP, secure file transfer protocol (SFTP), simple mail transfer protocol (SMTP) as shown in Figs. A4-10 to A4-15 are used.

Terrestrial IP protocols are assumed to be terminated at the satellite gateway.

## 5.5.2	Ship, gateway and device physical addressing

Most commercial ships use a 7-digit IMO number of which the last is a checksum, thus the IMO system can address 1 million ships. The 4 byte VDES physical addressing field has 4.3  109 unique IDs.

The number of networked devices on ships is growing fast and there is a need to directly address local gateways and devices.

In addition to the 4 byte address field, a 2 byte sub addressing field has been added.

The ship, local gateway and device addressing are shown in Table A1-12. Unlike MMSI, there will be no dedicated field or segmentation in this addressing scheme.

TABLE A1-12

Ship, Gateway and Device addressing

| Addressing field | Usage | Range |
| --- | --- | --- |
| 32 bit physical address (all messages) | Ship Terminal ID | 4.3 Billion |
| 16 bit sub addressing | To address local gateways and transducers | Flexible, e.g. 16 gateways each with 4 096 transducers |

## 5.5.3	Shore addressing of ships, gateways and devices

VDES will be accessed from shore using Internet, and it is desirable to use standard protocols such as email.

A database at the gateway will allow shore users to define their own meaningful ship, gateway and device names.

Training sequence

27 sym

Ramp up

4 symbols

Link CFG

ID

16 sym

Data with CRC-32

192, 448, 704, 960 symbols

Ramp Down

4 symbols

Guard

13   symbols

ASM

Bandwidth 16 kHz

9.6 ksym/s

26.67 ms

256 symbols

Training sequence

27 sym

Ramp up

16 symbols

Link CFG

ID

16 sym

Data with CRC-32

896 symbols

Ramp Down

16 symbols

Guard

53 symbols

VDE

Bandwidth 50 kHz

38.4 ksym/s

1024 symbols

26.67 ms

Training sequence

27 sym

Ramp up

32 symbols

Link CFG

ID

16 sym

Data with CRC-32

1792 symbols

Ramp Down

32 symbols

Guard

[149] symbols

VDE

Bandwidth 100 kHz

76.8 ksym/s

2048 symbols

26.67 ms

Training sequence

27 sym

Ramp up

8 symbols

Link CFG

ID

16 sym

Data with CRC-32

432 symbols

Ramp Down

8 symbols

Guard

21 symbols

VDE

Bandwidth 25 kHz

19.2 ksym/s

512 symbols

26.67 ms

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

27 sym

Ramp up

4 symbols

Link CFG

ID

16 sym

Data with CRC-32

192, 448, 704, 960 symbols

Ramp Down

4 symbols

Guard

13   symbols

ASM

Bandwidth 16 kHz

9.6 ksym/s

26.67 ms

256 symbols

Training sequence

27 sym

Ramp up

16 symbols

Link CFG

ID

16 sym

Data with CRC-32

896 symbols

Ramp Down

16 symbols

Guard

53 symbols

VDE

Bandwidth 50 kHz

38.4 ksym/s

1024 symbols

26.67 ms

Training sequence

27 sym

Ramp up

32 symbols

Link CFG

ID

16 sym

Data with CRC-32

1792 symbols

Ramp Down

32 symbols

Guard

[149] symbols

VDE

Bandwidth 100 kHz

76.8 ksym/s

2048 symbols

26.67 ms

Training sequence

27 sym

Ramp up

8 symbols

Link CFG

ID

16 sym

Data with CRC-32

432 symbols

Ramp Down

8 symbols

Guard

21 symbols

VDE

Bandwidth 25 kHz

19.2 ksym/s

512 symbols

26.67 ms

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

GD=1n0(D)d(D)n1(D)d(D)

n0D=1+D+D3

n1D=1+D+D2+D3

dD=1+D2+D3.

m=s-1 mod 2

i=floor (s-1 /(2k2))

j=floor (s-1 /2))-ik2

t=19i+1 mod (k1/2)

q=t mod 8+1

c=pqj+21m mod k2

πs=2t+ck1/2+1-m

Annex 2

Technical characteristics of the Application Specific Message (ASM) channels for the VDES in the VHF maritime mobile band

# This section describes those elements of the ASM that are unique to ASM operation.  For those elements that are common, the cross reference into Annex 1 is provided.

# 1	Structure of the application specific messages

This Annex describes the characteristics of the TDMA access schemes which include random access TDMA (RATDMA), incremental TDMA (ITDMA), fixed access TDMA (FATDMA), and slot carrier sense TDMA (SCTDMA) techniques.

For application specific messages refer to Recommendation ITU-R M.1371 in general.

## 1.1	Specific responsibilities of the OSI layers as defined in Annex 1 for preparing ASM data for transmission

### 1.1.1	Physical layer

Convert digital transmission packet to π/4 Quadrature Phase-Shift Keying (QPSK) signal to modulate transmitter.

### 1.1.2	Link layer

The link layer is divided into three sub-layers with the following tasks.

#### 1.1.2.1	Link management entity

This sub layer has the following functions:

–	Assemble ASM message bits

–	Order the ASM message bits into 8-bit byte for assembly of transmission packet.

#### 1.1.2.2	Data link services

This sub layer has the following functions:

–	Calculate frame check sequence (FCS) of the ASM message bits (see § 3.2.2.3).

–	Append FCS to ASM message to complete creation of transmission packet contents.

–	Complete assembly of transmission packet.

#### 1.1.2.3	Media access control

Media access control provides a method for granting access to the data transfer to the VHF data link (VDL). The method used is a TDMA scheme using a common time reference.

### 1.1.3	Network layer

The network layer is responsible for the management of priority assignments of messages, distribution of transmission packets between channels, and data link congestion resolution.

### 1.1.4	Transport layer

The transport layer is responsible for converting data into transmission packets of correct size and sequencing of data packets.

# 2	Physical layer

## 2.1	Parameters

### 2.1.1	General

The physical layer is responsible for the transfer of a bit-stream from an originator, out on to the data link. The performance requirements for the physical layer are summarized in Tables A2-1 to A2-3.

The low setting and the high setting for each parameter is independent of the other parameters.

TABLE A2-1

Minimum required time division multiple access transmitter characteristics

| Parameter name | Units | Low setting | High setting |
| --- | --- | --- | --- |
| Channel spacing (encoded according to RR Appendix 18 with footnotes)(1) | kHz | 25 | 25 |
| ASM 1 (2027)(1) | MHz | 161.950 | 161.950 |
| ASM 2 (2028)(1) | MHz | 162.000 | 162.000 |
| Transmit output power | W | 1 | 12.5 |
| (1) See Recommendation ITU-R M.1084, Annex 4. |  |  |  |

### 2.1.2	Transmission media

Data transmissions are made in the VHF maritime mobile band. Data transmissions should use ASM 1 and/or ASM 2 channels.

### 2.1.3	Multi-channel operation

The ASM should be capable of receiving on two parallel channels and transmitting on two independent channels. Two separate TDMA receiving processes should be used to simultaneously receive on two independent frequency channels. One TDMA transmitter may be used to enable TDMA transmissions on one or two independent frequency channels.

ASM transmission should be sent on one of the ASM channels. Each new ASM transmission  should switch to the ASM channel not used in the previous transmission.

ITDMA linked transmissions should be on the same channel.

## 2.2	Transceiver characteristics

The transceiver should perform in accordance with the characteristics set forth herein.

TABLE A2-2

Minimum required time division multiple access transmitter characteristics

| Transmitter parameters | Requirements |
| --- | --- |
| Carrier power error | ±1.5 dB |
| Carrier frequency error | ± 500 Hz |
| Slotted modulation mask | ∆fc < ±8 kHz: 0 dBc ±8 kHz < ∆fc < ±16 kHz: below the straight line between −25 dBc at ±8 kHz and –60 dBc at ±16 kHz ±16 kHz < ∆fc < ±25 kHz: below the straight line between −60 dBc at ±16 kHz and –70 dBc at ±25 kHz ±25 kHz < ∆fc < ±62.5 kHz: −70 dBc ∆fc > ±62.5 kHz: −80 dBc |
| Spurious emissions | −36 dBm: 9 kHz ... 1 GHz −30 dBm: 1 GHz ... 4 GHz |

TABLE A2-3

Minimum required time division multiple access receiver characteristics

| Receiver parameters | Requirements |
| --- | --- |
| Sensitivity | 20% PER @ −107 dBm |
| Error behaviour at high input levels | 1% PER @ −77 dBm 1% PER @ −7 dBm |
| Adjacent channel selectivity | 20% PER @ 70 dB |
| Spurious response rejection | 20% PER @ 70 dB |
| Intermodulation response rejection | 20% PER @ 74 dB |
| Spurious emissions | −57 dBm (9 kHz to 1 GHz) −47 dBm (1 GHz to 4 GHz) |
| Blocking | 20% PER @ 86 dB |

## 2.3	Modulation scheme

The base modulation is defined by the Link Configuration Id, see Table [x].

For the modulation bit mapping, see Annex 1.

### 2.4	Frequency stability

The frequency stability of the VHF radio transmitter/receiver should be ±500 Hz or better.

## 2.5	Data transmission bit rate

The transmission bit rate should be 19.2 kbit/s ±10 ppm for π/4 QPSK.

## 2.6	Frame structure

## For the generic definition of the frame structure, see Annex 1.

## 2.7	Signal information

Signal information selects the modulation scheme and coding according to the Link Configuration Id defined in Table [x].

## 2.8	Data encoding

Data coding is not used.

## 2.9	Forward error correction

When forward error correction is used, it will be used as defined in Annex 1. Interleaving and bit scrambling are used, as defined by the FEC designated in the signal information. In the event of no FEC, bit scrambling according to Annex 1 shall be implemented.

## 2.10	Transmitter transient response

The time taken to switch from transmit to receive conditions, and receive to transmit conditions, should not exceed the transmit transmit ramp up and ramp down , see Annex 1; 5.2.3.1. It should be possible to receive a message from the slot directly after or before own transmission.

The equipment should not be able to transmit during channel switching operation.

## 2.11	Transmitter power

The power level is determined by the link management entity (LME) of the link layer.

2.11.1	Provision should be made for two levels of nominal power (high power, low power) as required by some applications. The default operation of the ASM station should be on the high nominal power level.

2.11.2	The nominal levels for the two power settings should be 1 W and 12.5 W (average power); tolerance should be within ±1.5 dB.

# 3	Link layer

The link layer specifies how data is packaged in order to apply error detection and correction to the data transfer. The link layer is divided into three sub-layers.

## 3.1	Sub-layer 1 – medium access control

The medium access control (MAC) sub layer provides a method for granting access to the data transfer medium, i.e. the VHF data link. The access scheme is TDMA using a common time reference.

### 3.1.1	TDMA synchronization

TDMA synchronization is achieved using an algorithm based on a synchronization state as described below. The sync state flag within the ITDMA communication state indicates the synchronization state of a station.

The TDMA receiving process should not depend on slot boundaries.

Synchronization other than UTC direct may be provided by the AIS system.

#### 3.1.1.1	Coordinated universal time direct

A station, which has direct access to coordinated universal time (UTC) timing, with the required accuracy, should indicate this by setting its synchronization state to UTC direct.

#### 3.1.1.2	Coordinated universal time indirect

A station, which is unable to get direct access to UTC, but has access to the AIS system, may get its synchronization from the AIS system. It should then change its synchronization state to indicate the type of synchronization which is being provided by the AIS system.

### 3.1.2	Time division

The slot and frame are as defined in Annex 1. Access to the data link is, by default, given at the start of a slot. The frame start and stop coincide with the UTC minute, when UTC is unavailable the AIS system may provide the frame synchronization.

### 3.1.3	Slot phase and frame synchronization

Slot phase synchronization and frame synchronization is done by using information from UTC or from the AIS system.

#### 3.1.3.1	Slot phase synchronization

Slot phase synchronization is the method whereby the slot boundary is synchronized with a high level of synchronization stability, thereby ensuring no message boundary overlapping or corruption of messages.

#### 3.1.3.2	Frame synchronization

Frame synchronization is the method whereby the current slot number for the frame is known.

### 3.1.4	Slot identification

Each slot is identified by its index (0-2249). Slot zero (0) should be defined as the start of the frame.

### 3.1.5	Slot access

The transmitter should begin transmission by turning on the RF power at slot start.

The transmitter should be turned off after the last bit of the transmission packet has left the transmitting unit. This event must occur within the slots allocated for own transmission. The slot access is performed as described in Annex 1  § -5.2.2.

### 3.1.6	Slot state

Each slot on an ASM channel can be in one of the following states:

–	Free: meaning that the slot is unused on the channel within the receiving range of the own station

–	Internal allocation: meaning that the slot is allocated by own station for the purpose of ASM transmission

–	[available] External allocation by a mobile station: meaning that the slot is allocated for the purpose of ASM transmission by another station.

-	[unavailable: external allocation by a base station, TBD]

## 3.2	Sub layer 2 – data link service

The data link service (DLS) sub layer provides methods for:

–	data link activation and release;

–	data transfer; or

–	error detection, correction and control.

### 3.2.1	Data link activation and release

Based on the MAC sub layer the DLS will listen, activate or release the data link. A slot, marked as free or externally allocated, indicates that own equipment should be in receive mode and listen for other data link users.

### 3.2.2	Data transfer

Data transfer should use a bit-oriented protocol and should be in accordance with this standard.

#### 3.2.2.1	Packet format

Data is transferred using the generic transmission packet as defined in Annex 1 Fig. A1-4:

The packet should be sent from left to right. The training sequence is be used to synchronize the VDES receiver.

#### 3.2.2.2	Summary of the transmission packet

The data packet is defined in Table A2-4.

The ASM channel configurations are defined by Link Configuration Id table, see Table [x].

TABLE A2-4

Packet symbol structure for π/4 QPSK modulation scheme

| Ramp up | 4 symbols |  |  |
| --- | --- | --- | --- |
| Training sequence | 27 symbols | Necessary for synchronization |  |
| Link configuration id | 16 symbols | Decoded from (32,6) biorthogonal code ASM channel configurations as defined in Link Configuration Id table |  |
| Data | 1 slot: 176 2 slot: 432 3 slot: 688 symbols | The symbol count and the information bits varies according to coding rate as defined by the link configuration id field |  |
| CRC | 16 symbols | The CRC only includes the data field |  |
| Ramp Down | 4 symbols | Distance delay and jitter |
| Guard Time | 13 symbols | Distance delay and jitter |  |
| Total | 1 slot: 256 2 slot: 512 3 slot: 768 symbols |  |  |

#### 3.2.2.3	Transmission timing

There should be no modulation during the ramp down period.

#### 3.2.2.4	Long transmission packets

A station may occupy a maximum of 3 consecutive slots for one (1) continuous transmission. Only a single application of the overhead (ramp up, training sequence, flags, FCS, guard time) is required for a long transmission packet.

### 3.2.3	Error detection and control

Error detection is accomplished using a CRC polynomial as described in Annex 1.

### 3.2.4	Forward Error correction

Forward error correction should be handled as described by the Link Configuration Id.

## 3.3	Sub layer 3 – link management entity

The LME controls the operation of the DLS, MAC and the physical layer.

### 3.3.1	Access to the data link

There should be different access schemes for controlling access to the data transfer medium. The application and mode of operation determine the access scheme to be used.

The access schemes are ITDMA, RATDMA, SCTDMA and FATDMA, [MITDMA].

#### 3.3.1.1	Cooperation on the data link

The access schemes operate continuously, and in parallel, on the same physical data link. They all conform to the rules set up by the TDMA. The ASM system must give priority to the AIS system when accessing the physical data link.

#### 3.3.1.2	Candidate slots

Slots, used for transmission, are selected from candidate slots in the selection interval (SI) which is defined as 150 slots.

The selection process uses received data from AIS and ASM.

There should be, at minimum, a set of four candidate slots to choose from.

The candidate slots are primarily selected from slots that are free on AIS and ASM.

The available slots are as defined in Recommendation ITU-R M.1371 and must only be taken from the most distant station(s) within the SI.

If the candidate slot set contains less than four slots, additional candidate slots can be obtained by using the following rules and order (rule 1 followed by rule 2):

Rule 1:	available slot on one AIS channel and free on the other AIS and  free on the ASM [+ VDE] channels

Rule 2:	available slot on both AIS channels (AIS1 and AIS2) and free on all ASM [+ VDE] channels.

[Rule 3: available on AIS channels and available on ASM channels]

When selecting candidates for messages longer than one (1) slot, a candidate slot should be the first slot in a consecutive block of slots that conform to the selection criteria stated above.

If the station cannot find sufficient number of candidate slots, the station should not transmit and should re-schedule the transmission.

The candidate slot selection process also has to consider time periods reserved for the reception of the bulletin board.

The purpose of maintaining a minimum of four candidate slots within the same probability of being used for transmission is to provide high probability of access to the link.

[AIS channels need only to be considered when ASM and AIS are using the same antenna].

### 3.3.2	Modes of operation

There should be two modes of operation. The default mode should be autonomous and may be switched to/from other modes.

#### 3.3.2.1	Autonomous

A station operating autonomously should determine its own schedule for transmission. The station should automatically resolve scheduling conflicts with other stations.

#### 3.3.2.2	Assigned

A station operating in the assigned mode takes into account the transmission schedule of the assigning message when determining when it should transmit.

### 3.3.3	Initialization

At power on, a station should monitor the TDMA channels for one (1) minute to determine channel activity, other participating member IDs, current slot assignments, and possible existence of shore stations. During this time period, a dynamic directory of all stations operating in the system should be established. A frame map should be constructed, which reflects TDMA channel activity. After one (1) minute has elapsed, the station may be available to transmit ASM messages according to its own schedule.

### 3.3.4	Channel access schemes

The access schemes, as defined below, should coexist and operate simultaneously on the TDMA channel. The access schemes ITDMA, RATDMA and FATDMA are as defined in Recommendation ITU-R M.1371.[and new MITDMA]

#### 3.3.4.1	Slot carrier sense time division multiple access (SCTDMA)

SCTDMA may be used for satellite-uplink transmissions.

#### 3.3.4.2	Slot carrier sense time division multiple access algorithm

Access to the data link should be achieved with reference to frame start using UTC direct.

VDES stations using SCTDMA should detect if the slot is used by examining the CS detection window of 11 symbols [1 146 µs] starting at 4 symbols [417 µs] and ending at 15 symbols [1 979 µs] after the start of the slot intended for transmission (T0). Signals within the first 4 symbols [417 µs] of the time period are excluded from the decision to allow for propagation delays and ramp down periods of other units.

VDES stations using the SCTDMA access scheme should not transmit on any slot in which, during the CS detection window, a signal level greater than the “CS detection threshold” is detected.

The transmission of a SCTDMA packet should commence at symbol 28 [2501 µs] after the nominal start of the time period (see Fig. A2-3).

Figure A2-1

Carrier sense timing

#### 3.3.4.3	Carrier sense detection threshold

The carrier sense (CS) detection threshold should be determined over a rolling 60 s interval on each Rx channel separately. The threshold should be determined by measuring the minimum energy level (representing the background noise) plus an offset of 10 dB. The minimum CS detection threshold should be −107 dBm and background noise should be tracked for a range of at least 30 dB (which results in a maximum threshold level of −77 dBm).

#### 3.3.4.4	Network access and entry of a new data stream

For ITDMA and RATDMA at power on, a station should monitor the TDMA channel for one (1) min interval to determine channel activity, other participating member IDs, current slot assignments and reported positions of other users, and possible existence of base stations, as shown in Fig. A2-4. During this time period, a dynamic directory of all members operating in the system should be established. A frame map should be constructed, which reflects TDMA channel activity.

figure A2-2

Network Access for ITDMA and RATDMA

#### 3.3.4.5	Priority of transmissions

There are 3(three) levels of message priority:

[Priority 1 (highest): Critical link management messages]

Priority 2: Safety related messages

Priority 3 (lowest): All other messages.

### 3.3.5	Message structure

The messages should have the following structure shown in Fig. A2-5 inside the data portion of a data packet.

Figure A2-3

Message Structure

Training sequence

MSG

ID

Link CFG

ID

Data

CRC

Training sequence

MSG

ID

Link CFG

ID

Data

CRC

Each message is described using a table with parameter fields listed from top to bottom. Each parameter field is defined with the most significant bit first.

Parameter fields containing sub-fields (e.g. communication state) are defined in separate tables with sub-fields listed top to bottom, with the most significant bit first within each sub-field.

When data is output on the VHF data link it should be grouped in bytes of 8 bits from top to bottom of the table associated with each message. Each byte should be outputted with least significant bit first.

Shall be delete when change log 117 is done: A generic example for a message table is provided in Table A2-5.

TABLE A2-5

Generic Message Table

| Parameter | Symbol | Number of bits | Description |
| --- | --- | --- | --- |
| P1 | T | 6 | Parameter 1 |
| P2 | D | 1 | Parameter 2 |
| P3 | I | 1 | Parameter 3 |
| P4 | M | 27 | Parameter 4 |
| P5 | N | 2 | Parameter 5 |
| Unused | 0 | 3 | Unused bits |

The logical view of data is provided in Table A2-6.

TABLE A2-6

Logical view of data

| Bit order | M----L-- | M------- | -------- | -------- | --LML000 |
| --- | --- | --- | --- | --- | --- |
| Symbol | TTTTTTDI | MMMMMMMM | MMMMMMMM | MMMMMMMM | MMMNN000 |
| Byte order | 1 | 2 | 3 | 4 | 5 |

The output order to VHF data link is provided in Table A2-7.

TABLE A2-7

Output order to VHF data link

| Bit order | --L----M | -------M | -------- | -------- | 000LML-- |
| --- | --- | --- | --- | --- | --- |
| Symbol | IDTTTTTT | MMMMMMMM | MMMMMMMM | MMMMMMMM | 000NNMMM |
| Byte order | 1 | 2 | 3 | 4 | 5 |

#### 3.3.5.1	Message identification (see change log 133)

The message ID should be 6 bits long and should respect the current definitions of message IDs as defined for AIS in Recommendation ITU-R M.1371.

#### 3.3.5.2	Incremental time division multiple access message structure

The ITDMA message structure supplies the necessary information in order to operate in accordance with Recommendation ITU-R M.1371. The message structure is shown in Fig. A2-6.

figure A2-4

ITDMA Message Structure

Training sequence

MSG

ID

Link CFG

ID

Data

CRC

User

ID

Communication State

Training sequence

MSG

ID

Link CFG

ID

Data

CRC

User

ID

Communication State

##### 3.3.5.2.1	User identification

The user ID should be a unique identifier and is 30 bits long.

##### 3.3.5.2.2	Incremental time division multiple access communication state

The communication state provides the following functions:

–	it contains information used by the slot allocation algorithm in the ITDMA concept;

–	it also indicates the synchronization state.

The ITDMA communication state is structured as defined in Recommendation ITU-R M.1371. The ITDMA communication state should apply only to the slot in the channel where the relevant transmission occurs. ASM 1 and ASM 2 are independent channels.

# 4	Network layer

The network layer should be used for:

–	Establishing and maintaining channel connections

–	Management and priority assignments of messages

–	Distribution of transmission packets between channels

–	Data link congestion resolution.

# 5	Transport layer

The transport layer is responsible for:

–	converting data into transmission packets of correct size;

–	sequencing of data packets;

–	interfacing protocol to upper layers.

The interface between the transport layer and higher layers should be performed by the presentation interface.

## 5.1	Definition of transmission packet

A transmission packet is an internal representation of some information which can ultimately be communicated to external systems. The transmission packet is dimensioned so that it conforms to the rules of data transfer.

## 5.2	Transmission packets

### 5.2.1	Addressed Messages

Addressed messages are point to point communications between VDES stations. Addressed messages may require an acknowledgment. When an acknowledgement is required and not received, the VDES stations may retransmit the message.

### 5.2.2	Broadcast messages

A broadcast message lacks a destination identifier ID. Therefore receiving stations should not acknowledge a broadcast message.

### 5.2.3	Conversion to presentation interface messages

Each received transmission packet should be converted to a corresponding presentation interface message and presented in the order they were received regardless of message category. Applications utilizing the presentation interface should be responsible for their own sequencing numbering scheme, as required. For a mobile station, addressed messages should not be output to the presentation interface, if destination User ID (destination MMSI) is different to the ID of own station (own MMSI).

### 5.2.4	Conversion of data into transmission packets

The transport layer should convert data, received from the presentation interface into transmission packets.

## 5.3	Presentation interface protocol

Data, which is to be transmitted by the station, should be input via the presentation interface; data, which is received by the station, should be output through the presentation interface. The formats and protocol used for this data stream are defined by IEC 61162 series.

# 6	Satellite uplink message

Satellite uplink may be provided by VDES equipment. It also may provide by dedicated equipment using slot carrier sense TDMA (SCTDMA) access scheme to consolidate AIS communications and terrestrial ASM communications.

## 6.1	Packet bit structure for satellite uplink message

The data packet for ITDMA, RATDMA and FATDMA is defined in Table A2-8.

The data packet for SCTDMA is as defined in Table A2-9.

TABLE A2-8

Modified packet bit structure for satellite uplink

| Slot composition | Bits | Notes |
| --- | --- | --- |
| Ramp up | 8 | Standard |
| Training sequence | 54 | Standard |
| Link Configuration Id | 32 | Decoded from [Hamming (16,6)] ASM channel configurations as defined in Link Configuration Id table |
| Data field | 232 | Without encoding: 232 bits With encoding: varies according to coding rate defined in the Signal Information field |
| CRC | 32 | Without encoding: 32 bits; With encoding: varies according to coding rate defined in the Signal Information field; Only the data field is included in the CRC |
| Guard Time | 154 | Synch jitter (mobile station) = 6 bits Synch jitter (mobile/satellite) = 2 bits Propagation time delay difference = 144 bits Spare = 2 bits |
| Total | 512 | 512 bits for 19.2 kbits/s π/4 QPSK |

TABLE A2-9

Modified packet bit structure for satellite uplink ASM message with SCTDMA

| Slot composition | Bits | Notes |
| --- | --- | --- |
| Carrier sense period | 56 | Not transmitting (2 917 µs, equivalent to 56 bits) |
| Ramp up | 8 | Standard |
| Training sequence | 54 | Standard |
| Link Configuration Id | 32 | Decoded from [Hamming (16,6)] ASM channel configurations as defined in Link Configuration Id table |
| Data field | 176 | Without encoding:176 bits With encoding: varies according to coding rate defined in the Signal Information field |
| CRC | 32 | Without encoding: 32 bits; With encoding: varies according to coding rate defined in the Signal Information field; Only the data length and data field are included in the CRC |
| Guard Time | 154 | Synch jitter (mobile station) = 6 bits Synch jitter (mobile/satellite) = 2 bits Propagation time delay difference = 144 bits Spare = 2 bits |
| Total | 512 | 512 bits for 19.2 kbits/s π/4 QPSK |

## 6.2	Transmitting the satellite uplink broadcast message

The satellite uplink ASM broadcast message should be transmitted only on ASM channels and not on the following channels: 75, 76, AIS 1, AIS 2 or regional channels.

Annex 3

Technical characteristics of VDE-terrestrial 
in the maritime mobile band

# 1	Introduction

This section describes those elements of the VDE-TER that are unique to VDE-TER operation.  For those elements that are common, the cross reference into Annex 1 is provided. It contains a description of the different protocols according to the OSI layer model and recommends implementation details for each layer.

Data transmission is made in the VHF maritime mobile band. Data transmissions are made within the spectrum allocated for the VDE1-A and VDE1- B. The spectrum may be used as 25 kHz, 50 kHz or 100 kHz channels.

The system should use TDMA techniques in a synchronized manner.

# 2	OSI layer

Refer to Annex 1.

# 3	Physical layer

## 3.1	Range

The communication range of terrestrial VDE is typically 20−50 NM.

## 3.2	Transmitter Parameter settings

Refer to Annex 1 for transmitter parameter settings for mobile stations.

## 3.3	Antenna

Terrestrial VDE may share the same antenna(s) with the other subsystems AIS, ASM, VDE-SAT.

Refer to Annex 1.

## 3.4	Modulation

### 3.4.1	Waveforms

## The waveforms are defined in Annex 1, Modulation and Coding Schemes.

### 3.4.1	Bit Mapping

### For bit mappings, see Annex 1.

## 3.5	Sensitivity and Interference

VDE uses adaptive modulation and coding to maximise spectral efficiency and throughput. Sensitivity and interference levels for the supported modulation methods are given in Table A3-1.

TABLE A3-1

Sensitivity and Carrier to Interference Ratios

| Modulation Coding Scheme | 25 kHz |  | 50 kHz |  | 100 kHz |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Sensitivity (dBm) | CIR (dB) | Sensitivity (dBm) | CIR (dB) | Sensitivity (dBm) | CIR (dB) |
| MCS-1* | −110 | 8 | −107 | 8 | −104 | 8 |
| MCS-3* | −104 | 14 | −101 | 14 | −98 | 14 |
| MCS-5* | −102 | 16 | −99 | 16 | −96 | 16 |
| * Modulation Coding Schemes, see Table A1-6. This table assumes a BER of 1e-6. |  |  |  |  |  |  |

## TBD: add missing receiver parameters (look at Table A2-1).

## 3.6	Symbol timing accuracy

See Annex 1.

## 3.7	Transmitter timing jitter

See Annex 1.

## 3.8	Slot transmission accuracy at the output

See Annex 1.

## 3.9	Frame structure

See Annex 1.

# 4	Link layer

## 4.1	Access Schemes

The VDE terrestrial system should support the following TDMA access schemes:

–	FATDMA;

–	RATDMA;

–	ITDMA.

## 4.2	Data encapsulation

Refer to Annex 1.

## 4.3	Cyclic redundancy check

Refer to Annex 1.

## 4.4	Automatic repeat request (ARQ)

Refer to Annex 1.

## 4.5	Acknowledgement (ACK)

Refer to Annex 1.

## 4.6	End delivery notification (EDN)

Refer to Annex 1.

## 4.7	End delivery failure (EDF)

Refer to Annex 1.

## 4.8	Frame hierarchy

Refer to Annex 1.

## 4.9	Channel resource and media access control for VDE terrestrial

VDE spectrum (and logical channel) resources are allocated by the reception of shore (or satellite-based) media access control messaging (for example, shore stations perform this operation using the bulletin board service). In the event of vessel reception of, both satellite and shore-based stations, the shore-based resource allocations should take precedence.

Duplexing modes are a function of the resource allocation above, as well as the operational mode.

### 4.9.1	Simplex modes

–	Ship-to-ship

Same channel resource is used for both directions of communications.

### 4.9.2	Duplex modes

–	Shore-to-ship

–	Ship-to-shore

Upper and lower spectrum are used for ship to shore or shore to ship respectively, only one side transmits at one time.

Refer to Annex 1 for details of frequency allocations.

## 4.10	Physical and logical channels

VDES uses several channels to carry data. These channels are separated into Physical and Logical channels. A bulletin board may prescribe this channel usage.

Lacking bulletin board information, ship borne stations should employ a default channel configuration of 100 kHz channels on the terrestrial VDES (channel 2024, 2084, 2025 and 2085 combined) operating in a simplex ad-hoc access scheme. The simplex ad-hoc access scheme for ship-to-ship communications should be ITDMA (when possible) or RATDMA.

## 4.11	Physical channels

The physical channels (PC) are determined by the centre frequency and bandwidth.

## 4.12	Logical channels

The logical channels (LC) are divided into signalling and traffic channels. These are described below. Logical channel definitions can be defined based on the physical channel and message time information (frame hierarchy, start time, etc.).

Signalling channels:

–	Terrestrial Bulletin Board (TBB), see § 4.12.1

–	Announcement, see § 4.12.2

–	Random access, see § 4.12.5.

All signalling channels use the most robust modulation and coding scheme.

Traffic channels:

–	Multicast, see § 4.12.3

–	Unicast, see § 4.12.4

–	Random access, see § 4.12.5.

Traffic channels may use a combination of robust and higher bitrate modulation and coding schemes.

### 4.12.1	Terrestrial bulletin board (TBB) signalling channel

Each VDE shore station should employ a fixed logical channel for the TBB. All TBB logical channels will be based on one of a number of predefined structures of the frame hierarchy 50 kHz shore to ship physical channel (2024 and 2084 combined). These are defined to occupy only a portion of the frame (60 seconds, 2 250 slots) to permit possible spectrum and temporal sharing with satellites, see Annex 6.

The TBB defines the network configuration parameters such as signalling channels (control channels) and data channel(s), protocol versions and future network configuration. The TBB takes precedence in the allocation of spectrum (logical channel) resources. This may be co-ordinated with the satellite bulletin board signalling channel to facilitate sharing of mutual spectrum resources.

The logical channels are normally repeated based on the VDES frame hierarchy.

The VDE terrestrial channel usage for the service area of VDE shore station is defined by the TBB, see Annex 1.

The TBB information includes the area of applicability. The TBB does not change often and should be transmitted in regular intervals.

### 4.12.2	Announcement signalling channel (ASC)

This channel(s) will normally carry announcements, MAC information, VDE forward and return resource allocation, CQIs, ARQs, and ACKs. Announcements also include the co-ordination of uni-cast and multi-cast (broadcast) datagrams.

The MAC information includes changes to network version, congestion control (randomization interval (hold-off) and minimum priority level). Some of these parameters will be reflected in the TBB on periodic basis.

The ASC logical channels will be assigned in the TBB and consist of a number of defined structures of the frame hierarchy 50 kHz shore to ship physical channel (2024 and 2084 combined). These are defined to occupy only a portion of the frame (60 seconds, 2 250 slots) to permit possible spectrum and temporal sharing with satellites, see Annex 6.

The ASC defines the physical channel usage (logical channel, i.e. frequency and slot) to an individual ship following a resource request. The VDE shore station uses CQI information from the ship terminal to select the highest throughput format with adequate link margin.

### 4.12.3	Multicast data channel (MDC)

This traffic channel(s) is utilized to send messages to be received by a large number of ships. By default multicast messages are addressed to all stations (i.e. broadcast).

### 4.12.4	Unicast data channel (UDC)

This traffic channel is allocated a specific ship for the duration of a unicast datagram.

This channel is set up after a ship responds to an announcement, and the response includes received channel quality information (CQI) allowing the shore station to maximise throughput.

### 4.12.5	Random access channel (RAC)

This channel has the characteristics of a slotted Aloha channel, uses a random access scheme and will be selected from a predefined list of logical channels.

#### 4.12.5.1	For ship-to-shore, and shore-to-ship communications

A ship station uses this channel to access the network or send a short message.

#### 4.12.5.2	For ship-to-ship when ships are within control area of a VDE shore station

A ship station uses this channel to communicate directly with other ships. This logical channel is allocated by shore station via TBB or ASC.

#### 4.12.5.3	For ship-to-ship when outside the control area of a shore VDE station

A ship station uses these channels to communicate with other ship stations directly via short message, and will also use these random access channels to co-ordinate communication with other ships for larger messages. These logical channels will be based on a number of predefined structures of the frame hierarchy of the ship-to-ship physical channels (2024, 2084, 2025 and 2085 combined). Ship-to-ship random access channels should have fixed physical channel assignments and use the most robust modulation and coding scheme. These logic channels are distinct from the TBB logical channels.

# 5	Network layer

# Refer to Annex 1, assuming “Control Station” refers to a shore station.

# 6	Transport layer

Refer to Annex 1.

Annex 4

Technical characteristics of VDE-SAT service in the VHF maritime mobile band

# 1	Introduction

This section describes those elements of the VDE-SAT that are unique to VDE-SAT operation.  For those elements that are common, the cross reference into Annex 1 is provided. In this context, the following types of functionality are envisaged:

–	VDE-SAT allows two-way communications and transmit-only:

–	Shore initiated polling of information from ships (satellite-ship-satellite)

–	Ship initiated enquiry for information from shore (ship-satellite-ship)

–	Ship initiated data transfer to shore (ship-satellite)

–	Collection of information from transmit-only VDES terminals (ship-satellite). This could be either event driven or periodic. The slot and frequency band for this service should be assigned by the bulletin board and announcement signalling channels.

- Downlink multicast multi-packet data transfer (satellite-ship)

- Shore originated unicast multi-packet data transfer via satellite (satellite ship)

In this annex low earth orbit (LEO) satellites with 600 km altitude are considered to present typical examples of VDE satellite solutions. It should be noted that other orbital selections are also possible according to the overall system design consideration.

The focus of this annex is to describe the physical layer of the OSI model as defined in Annex 1. The overall description of the link, network and the transport layers is provided in Annex 1.

# 2	VDE-SAT physical layer

## 2.1	VDE-SAT uplink key parameters

This section outlines assumptions regarding the VDE-SAT system parameters that are used as representative examples in this annex.

### 2.1.1	Satellite to surface distance range

The orbit height determines the satellite range variations. For example, for a 600 km LEO the maximum range is 2 830 km. For timing purposes a maximum range of 3 000 km will be used.

The minimum range is equal to the orbit height. For a LEO satellite at 600 km altitude the minimum range will be 600 km. This value is used to determine the minimum propagation delay time. Considering these exemplary values for the minimum and maximum ranges, the path delay will vary from 2 ms to 10 ms, a variation of 8 ms as shown in Figure A4-1 and Figure A4-2.

For the VDE-SAT downlink, in addition to the relative delays between signal receptions at a vessel from different satellites, there could be absolute delay due to other sources such as signal processing delay. The satellite service provider should pre-compensate for absolute delay.

Figure A4-1

VDE-SAT downlink timing

Figure A4-2

VDE-SAT Uplink timing

### 2.1.2	Satellite transmission carrier frequency error

The satellite transmission carrier frequency error is the sum of the satellite transmission frequency error and Doppler, and the frequency uncertainty at the receiver. The transmit frequency error at the satellite shall be less than 1 ppm, i.e. ±160 Hz.

A LEO satellite will move at a speed of about 8 km/s and this will cause a maximum Doppler of ±4 kHz at VHF.

### 2.1.3	Ship station transmitter requirements

For ship station transmitter requirements, see Annex 1.

### 2.1.4	Ship station antenna gain

For ship station antenna gain, see Annex 1.

### 2.1.5	Ship station noise plus interference level

For noise plus interference level of ship station, see Annex 1.

### 2.2	Link budget analysis

The link C/N0 is determined by the transmitted e.i.r.p., path losses, propagation losses, receiver sensitivity/figure of merit and local interference levels.

### 2.2.1	VDE-SAT downlink link budget

Examples of link budgets for the VDE-SAT downlink are provided in the following sections.

### 2.2.1.1	Satellite downlink e.i.r.p.

The e.i.r.p. can be derived from PFD mask given in Table A4-1.

TABLE A4-1

Proposed power spectral and PFD mask

θ°=earth-satellite elevation angle

PFDθ° (dBW/(m2*4 kHz))=-149+0.16*θ°                        0°≤θ<45°;-142+0.53*θ°-45°      45°≤θ<60°;-134+0.1*θ°-60°         60°≤θ≤90°.

Table A4-2 shows the theoretical maximum satellite e.i.r.p. as a function of elevation angles for this mask.

TABLE A4-2

Satellite maximum e.i.r.p. vs. elevation angle

| Ship Elevation angle | Powerflux density on ground | Satellite range | Maximum downlink satellite e.i.r.p. |
| --- | --- | --- | --- |
| (degrees) | (dBW/m2/4 kHz) | (km) | (dBW in 25 kHz) |
| 0 | −149.0 | 2 831 | −1.0 |
| 10 | −147.4 | 1 932 | −2.7 |
| 20 | −145.8 | 1 392 | −4.0 |
| 30 | −144.2 | 1 075 | −4.6 |
| 40 | −142.6 | 882 | −4.7 |
| 50 | −139.4 | 761 | −2.8 |
| 60 | −134.0 | 683 | 1.6 |
| 70 | −133.0 | 635 | 2.0 |
| 80 | −132.0 | 608 | 2.6 |
| 90 | −131.0 | 600 | 3.5 |

### 2.2.1.2	Satellite e.i.r.p. vs. elevation

Most of the satellite coverage area and visibility time will be at low elevation angles, and high elevation angle coverage may be sacrificed without significant system capacity loss.

The following two satellite antennas have been analysed and are acceptable.

1)	Yagi Antenna: For this antenna the link budget is optimised for 0 degrees ship elevation angle using a three element Yagi antenna with the satellite pointed at the horizon is given in Table A4-3. Assuming a peak antenna gain of 8 dBi, a transmit RF power of −12.4 dBW in 25 kHz will ensure compliance with the PFD limit. Satellite e.i.r.p. vs. ship elevation is shown in Table A4-3.

TABLE A4-3

Satellite e.i.r.p. vs. elevation using a Yagi antenna

| Ship elevation angle | Nadir offset angle | Boresight offset | Satellite antenna gain | Satellite e.i.r.p. in circular polarization | Satellite range | PFD | Table A4-1 PFD limit | PFD margin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| degrees | degrees | degrees | dBi | dBW | km | dBW/m2/4 kHz | dBW/m2/ 4 kHz | dB |
| 0 | 66.1 | 0 | 8 | −4.4 | 2 830 | −152.4 | −149.0 | 3.4 |
| 10 | 64.2 | 1.9 | 8 | −4.4 | 1 932 | −149.1 | −147.4 | 1.7 |
| 20 | 59.2 | 6.9 | 8 | −4.4 | 1 392 | −146.2 | −145.8 | 0.4 |
| 30 | 52.3 | 13.8 | 7.8 | −4.6 | 1 075 | −144.2 | −144.2 | 0.0 |
| 40 | 44.4 | 21.7 | 6.9 | −5.5 | 882 | −143.4 | −142.6 | 0.8 |
| 50 | 36 | 30.1 | 5.5 | −6.9 | 761 | −143.5 | −139.4 | 4.1 |
| 60 | 27.2 | 38.9 | 3.6 | −8.8 | 683 | −144.5 | −134.0 | 10.5 |
| 70 | 18.2 | 47.9 | 0.7 | −11.7 | 635 | −146.7 | −133.0 | 13.7 |
| 80 | 9.1 | 57 | −2.2 | −14.6 | 608 | −149.2 | −132.0 | 17.2 |
| 90 | 0 | 66.1 | −5.5 | −17.9 | 600 | −152.4 | −131.0 | 21.4 |

2)	Isoflux antenna: This antenna is designed to point at the nadir direction providing a symmetric radiation pattern around the pointing direction. Assuming a peak antenna gain of 2 dBi, a transmit RF power of −5 dBW in 25 kHz will ensure compliance with the PFD limit. Satellite e.i.r.p. vs. ship elevation is shown in Table A4-4.

TABLE A4-4

Satellite e.i.r.p vs. elevation using an isoflux antenna

| Ship elevation angle | Nadir offset angle | Boresight offset | Satellite antenna gain | Satellite e.i.r.p. in circular polarization | Satellite range | PFD | Table A4-1 PFD limit | PFD margin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| degrees | degrees | degrees | dBi | dBW | km | dBW/m2/4 kHz | dBW/m2/4 kHz | dB |
| 0 | 66.1 | 0 | 2 | −3.0 | 2 830 | −151.0 | −149.0 | 2.0 |
| 10 | 64.2 | 1.9 | 1.5 | −3.5 | 1 932 | −148.2 | −147.4 | 0.8 |
| 20 | 59.2 | 6.9 | 1 | −4.0 | 1 392 | −145.8 | −145.8 | 0.0 |
| 30 | 52.3 | 13.8 | −0.5 | −5.5 | 1 075 | −145.1 | −144.2 | 0.9 |
| 40 | 44.4 | 21.7 | −2 | −7.0 | 882 | −144.9 | −142.6 | 2.3 |
| 50 | 36 | 30.1 | −4 | −9.0 | 761 | −145.6 | −139.4 | 6.2 |
| 60 | 27.2 | 38.9 | −5 | −10.0 | 683 | −145.7 | −134.0 | 11.7 |
| 70 | 18.2 | 47.9 | −7 | −12.0 | 635 | −147.0 | −133.0 | 14.0 |
| 80 | 9.1 | 57 | −8 | −13.0 | 608 | −147.6 | −132.0 | 15.6 |
| 90 | 0 | 66.1 | −8.5 | −13.5 | 600 | −148.0 | −131.0 | 17.0 |

### 2.2.1.3	Link C/(N0+I0)

The nominal signal level and C/(N0+I0) vs. elevation for a 25 kHz channel are provided in Table A4-3 and Table A4-4 for Yagi and Isoflux on-board antennas. The assumed ship antenna gain is 3 dBi and the system noise temperature is 30.2 dBK as shown in Table A1-10 of Annex 1.

Because the downlink is PFD limited, increasing the channel bandwidth to 50 kHz or 100 kHz will increase the signal level and C/(N0+I0) by 3 and 6 dB respectively. Limiting the service area to ship elevation angles between 10 and 55 degrees also improves the link margin by 3 dB.

The Isoflux antenna improves the link budget at low elevation angles and provides a wider symmetrical coverage area, but requires a 5 times larger transmitter power on the satellite.

The link budget results with a satellite Yagi antenna is shown in Table A4-6. Isoflux antenna is shown in Table A4-7.

It should be noted that the analyses based on single satellite visibility.

TABLE A4-6

Link budget with satellite Yagi antenna (transmit RF power = −12.4 dBW/25 kHz)

| Ship elevation angle | Satellite EIRP in circular polarization | Satellite range | Path loss | Polarization loss | Ship antenna gain | Antenna signal level | C/N0 | Noise level in 25 kHz BW | C/(N0+I0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (degrees) | (dBW) | (km) | (dB) | (dB) | (dBi) | (dBm) | (dBHz) | (dBm) | (dBHz) |
| 0 | −4.4 | 2 830 | 145.6 | 3 | 3 | −120.0 | 48.4 | −116 | 40.0 |
| 10 | −4.4 | 1 932 | 142.2 | 3 | 3 | −116.7 | 51.7 | −116 | 43.3 |
| 20 | −4.4 | 1 392 | 139.4 | 3 | 2.5 | −114.3 | 54.1 | −116 | 45.7 |
| 30 | −4.6 | 1 075 | 137.2 | 3 | 1 | −113.8 | 54.6 | −116 | 46.2 |
| 40 | −5.5 | 882 | 135.4 | 3 | 0 | −114.0 | 54.4 | −116 | 46.0 |
| 50 | −6.9 | 761 | 134.2 | 3 | −1.5 | −115.6 | 52.8 | −116 | 44.4 |
| 60 | −8.8 | 683 | 133.2 | 3 | −3 | −118.0 | 50.4 | −116 | 41.9 |
| 70 | −11.7 | 635 | 132.6 | 3 | −4 | −121.3 | 47.1 | −116 | 38.7 |
| 80 | −14.6 | 608 | 132.2 | 3 | −10 | −129.8 | 38.6 | −116 | 30.2 |
| 90 | −17.9 | 600 | 132.1 | 3 | −20 | −143.0 | 25.4 | −116 | 17.0 |

TABLE A4-7

Link budget using Isoflux antenna (transmit RF power = −5.0 dBW/25 kHz)

| Ship elevation angle | Sat. EIRP | Path loss | Pol. loss | Ship antenna gain | Ship G/T | C/N0 no interference | Antenna level | Noise level in 25 kHz | C/(N0+I0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| deg | dBW | dB | dB | dBi | dB/K | dBHz | dBm | dBm | dBHz |
| 0 | −3.0 | 145.6 | 3 | 3 | −27.2 | 49.8 | −118.6 | −116 | 41.4 |
| 10 | −3.5 | 142.2 | 3 | 3 | −27.2 | 52.7 | −115.7 | −116 | 44.2 |
| 20 | −4.0 | 139.4 | 3 | 2.5 | −27.7 | 54.5 | −113.9 | −116 | 46.1 |
| 30 | −5.5 | 137.2 | 3 | 1 | −29.2 | 53.7 | −114.7 | −116 | 45.3 |
| 40 | −7.0 | 135.4 | 3 | 0 | −30.2 | 53.0 | −115.4 | −116 | 44.5 |
| 50 | −9.0 | 134.2 | 3 | −1.5 | −31.7 | 50.7 | −117.7 | −116 | 42.3 |
| 60 | −10.0 | 133.2 | 3 | −3 | −33.2 | 49.2 | −119.2 | −116 | 40.8 |
| 70 | −12.0 | 132.6 | 3 | −4 | −34.2 | 46.8 | −121.6 | −116 | 38.4 |
| 80 | −13.0 | 132.2 | 3 | −10 | −40.2 | 40.2 | −128.2 | −116 | 31.8 |
| 90 | −13.5 | 132.1 | 3 | −20 | −50.2 | 29.8 | −138.6 | −116 | 21.4 |

### 2.2.2	VDE-SAT uplink link budget

Examples of link budgets for the VDE-SAT uplink are provided in the following sections.

### 2.2.2.1	Ship station e.i.r.p. vs. elevation angle

For ship station e.i.r.p vs elevation angle, see Annex 1.

### 2.2.2.2	Satellite antenna gain

Table A4-8 presents the gain of a 3-element Yagi satellite antenna with a peak gain of 8 dBi as a functon of elevation angle.

TABLE A4-8

Satellite antenna gain vs. ship elevation angle

| Ship elevation angle | Nadir offset angle | Boresight offset angle | Satellite antenna gain |
| --- | --- | --- | --- |
| deg. | deg. | deg. | dBi |
| 0 | 66.1 | 0 | 8 |
| 10 | 64.2 | 1.9 | 8 |
| 20 | 59.2 | 6.9 | 8 |
| 30 | 52.3 | 13.8 | 7.8 |
| 40 | 44.4 | 21.7 | 6.9 |
| 50 | 36 | 30.1 | 5.5 |
| 60 | 27.2 | 38.9 | 3.6 |
| 70 | 18.2 | 47.9 | 0.7 |
| 80 | 9.1 | 57 | −2.2 |
| 90 | 0 | 66.1 | −5.5 |

### 2.2.2.3	Satellite system noise temperature

The satellite noise level at the receiver input is shown in Table A4-9. Without external interference the system noise temperature is 25.7 dBK.

TABLE A4-9

Satellite receiver system noise temperature

| Antenna noise temperature | 200.0 | K |
| --- | --- | --- |
| Feed losses | 1.0 | dB |
| LNA noise figure | 2.0 | dB |
| LNA noise temperature | 159.7 | K |
| Feedloss noise temp. at LNA | 56.1 | K |
| Antenna noise temp. at LNA | 158.9 | K |
| System noise temp. at LNA | 374.7 | K |
| System noise temp. at LNA | 25.7 | dBK |

### 2.2.2.4	Uplink C/N0

The baseline uplink link budget is given in Table A4-10. It is optimised for 0 degree ship elevation angles.

It can be seen from Table A4-10 that the C/N0 is better than 74 dBHz for ship elevation angles between 0 and 65 degrees.

TABLE A4-10

VDE-SAT Uplink link budget, 6 W ship transmit power

| Ship elevation angle |  | Ship antenna gain | Ship e.i.r.p. | Polarization loss | Range | Path loss | Satellite antenna gain | Satellite G/T | C/N0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| deg |  | dBi | dBW | dB | km | dB | dBi | dB/K | dBHz |
| 0 | 3 |  | 10.8 | 3 | 2 830 | 145.56 | 8 | −17.6 | 73.2 |
| 10 | 3 |  | 10.8 | 3 | 1 932 | 142.25 | 8 | −17.6 | 76.5 |
| 20 | 2.5 |  | 10.3 | 3 | 1 392 | 139.40 | 8 | −17.6 | 78.9 |
| 30 | 1 |  | 8.8 | 3 | 1 075 | 137.16 | 7.8 | −17.8 | 79.4 |
| 40 | 0 |  | 7.8 | 3 | 882 | 135.44 | 6.9 | −18.7 | 79.2 |
| 50 | −1.5 |  | 6.3 | 3 | 761 | 134.16 | 5.5 | −20.1 | 77.6 |
| 60 | −3 |  | 4.8 | 3 | 683 | 133.22 | 3.6 | −22 | 75.2 |
| 70 | −4 |  | 3.8 | 3 | 635 | 132.58 | 0.7 | −24.9 | 71.9 |
| 80 | −10 |  | −2.2 | 3 | 608 | 132.21 | -2.2 | −27.8 | 63.4 |
| 90 | −20 |  | −12.2 | 3 | 600 | 132.09 | -5.5 | −31.1 | 50.2 |

### 2.2.3	Propagation effects

The received signal level on-board a ship will vary due to a number of factors as shown in Table A4-11. A Rice distribution with a carrier to multipath (C/M) ratio of 10 dB and fading bandwidth of 3 Hz is assumed (see Figure A4-3), however the system shall be adaptable to handle significantly worse and better propagation conditions. Mid-latitude fade depths due to ionospheric scintillation are shown in Table A4-12.

TABLE A4-11

Ionospheric effects for elevation angles of about 30° one-way traversal 
(derived from Recommendation ITU-R P.531)

| Effect | Frequency dependence | 0.1 GHz | 0.25 GHz | 1 GHz |
| --- | --- | --- | --- | --- |
| Faraday rotation | 1/f 2 | 30 rotations | 4.8 rotations | 108 |
| Propagation delay | 1/f 2 | 25 µs | 4 µs | 0.25 µs |
| Refraction | 1/f 2 | 1 | 0.16 | 0.6 |
| Variation in the direction of arrival (r.m.s.) | 1/f 2 | 20 | 3.2 | 12 |
| Absorption (auroral and/or polar cap) | ≈1/f 2 | 5 dB | 0.8 dB | 0.05 dB |
| Absorption (mid-latitude) | 1/f 2 | 1 dB | 0.16 dB | 0.01 dB |
| Dispersion | 1/f 3 | 0.4 ps/Hz | 0.026 ps/Hz | 0.0004 ps/Hz |
| Scintillation (1) | See Rec. ITU-R.P.531 | See Rec. ITU-R P.531 | See Rec. ITU-R P.531 | 20 dB peak-to-peak |
| * This estimate is based on a TEC of 1 018 electrons/m2, which is a high value of TEC encountered at low latitudes in day-time with high solar activity. (1) Values observed near the geomagnetic equator during the early night-time hours (local time) at equinox under conditions of high sunspot number. |  |  |  |  |

TABLE A4-12

Mid-latitude fade depths due to ionospheric scintillation (dB)

| Percentage of time (%) | Frequency (GHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0.1 | 0.2 | 0.5 | 1 |
| 1.0 | 5.9 | 1.5 | 0.2 | 0.1 |
| 0.5 | 9.3 | 2.3 | 0.4 | 0.1 |
| 0.2 | 16.6 | 4.2 | 0.7 | 0.2 |
| 0.1 | 25.0 | 6.2 | 1.0 | 0.3 |

Figure A4-3

Ricean fade depth probability

### 2.3	Protection of the radio astronomy service in the 150.05-153 MHz band from harmful interference from the VDE-SAT downlink

An appropriate protection limit for Radio Astronomy service in the 150.05-153.0 MHz band would be −238 dBW/m2 in a 2.95 MHz bandwidth centred around 152 MHz. Accordingly the maximum VDE-SAT downlink emission in the 150.05-153 MHz band should be below values shown in Table A4-13.

TABLE A4-13

Maximum satellite unwanted emissions in the 150.05-153 MHz band

| Ship elevation angle (deg) | RAS limit (W/m2/ 2.95 MHz) | Range (km) | Sat. max. interference e.i.r.p. |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | (W) | (dBW) | (dBW/Hz) |
| 0 | 1.58E-24 | 2830 | 1.60E-10 | −97.97 | −162.67 |
| 10 | 1.58E-24 | 1932 | 7.43E-11 | −101.29 | −165.99 |
| 20 | 1.58E-24 | 1392 | 3.86E-11 | −104.14 | −168.83 |
| 30 | 1.58E-24 | 1075 | 2.30E-11 | −106.38 | −171.08 |
| 40 | 1.58E-24 | 882 | 1.55E-11 | −108.10 | −172.80 |
| 50 | 1.58E-24 | 761 | 1.15E-11 | −109.38 | −174.08 |
| 60 | 1.58E-24 | 683 | 9.29E-12 | −110.32 | −175.02 |
| 70 | 1.58E-24 | 635 | 8.03E-12 | −110.95 | −175.65 |
| 80 | 1.58E-24 | 608 | 7.36E-12 | −111.33 | −176.03 |
| 90 | 1.58E-24 | 600 | 7.17E-12 | −111.44 | −176.14 |

## 2.4	Bit mapping

For bit mappings, see Annex 1.

### 2.5	Spread spectrum with constant envelope

Direct sequence spreading with constant envelope can be implemented according to the spreading strategy {RD-3}. This provides a way to generate constant envelope signals whilst allowing the use of linear modulations (i.e. BPSK, or QPSK for data modulation). In this approach the CPM spreading sequences are selected such that the spread symbols maintain quasi continuous phase even at the transition from one symbol to the next. The CPM spreading principle is provided in Fig. A4-4.

Figure A1-4

CPM Spreading Principle

In order to avoid phase discontinuity at the data symbol transitions, the proposed solution is to adapt the spreading sequence to the modulation data. In other words, the CPM spreading sequence at the edge of each symbol is adapted according to the new input modulation symbol value to avoid any phase discontinuity. Such a solution produces a small loss at the receiver as the receiver does not know the edge symbol part of the used CPM spreading sequence. For a spreading factor of 16 or higher, the resulting correlation loss experienced by the receiver due to this issue is less than 0.25 dB. Performance losses with respect to conventional spreading is thus quite negligible provided that SF = 16 or larger is used.

The CPM spreading sequences are computed and optimized off-line and then stored in the memory of the terminals and receivers. A single spreading code is sufficient for all the users in the system. There is thus no need for storing multiple spreading sequences but just a single spreading sequence.

The stored spreading sequence is then applied starting from the preamble and continuing in the data part (as shown in Fig. A4-5). It should be noted that the actual spreading sequence is actually partly dependent on the modulation symbols in order to ensure continuity of the signal phase when the modulation symbol changes (Fig. A4-4). The spread samples are computed on the basis of the current modulation symbol and previous modulation symbol. For QPSK modulation there are 4 possible values for the phase difference of these two symbols. An index from 0 to 3 can point to the possible phase differences and is used to address which of 4 possible spreading sequences is actually used for computing the output signal. Figure A4-6 illustrates the power spectral properties of the proposed modulation scheme (with spreading factor 16). Due to its constant envelope properties, this modulation scheme can operate with a transmit power amplifier operating close to saturation while maintaining a low power leakage to adjacent channels.

Figure A4-5

Proposed Spreading in the CPM

Figure A4-6

Power spectral properties of spread spectrum with constant envelope

## 2.6	Baseband shaping and quadrature modulation

For baseband shaping of symbols, see Annex 1.

## 2.7	Transmission timing accuracy

For transmission accuracy figures, see Annex 1.

## 2.8	Half duplex and full duplex satellites

The system can be configured for both half and full duplex satellites as shown in Fig. A4-6.

Figure A4-6

Half-duplex and full duplex satellite operation

## 2.9	Frame structure

For frame and transmission burst structure, see Annex 1.

## 2.10	Forward Error Correction and interleaving

For forward error correction and interleaving, see Annex 1.

## 2.11	VDE-SAT link configuration formats

For link configurations available for the VDE-SAT uplink and downlink, see Annex 1.

## 3	VDE-SAT link layer

For description of the link layer, see Annex 1.

## 4	Network layer

For description of the network layer, see Annex 1.

## 5	Transport layer

For description of the transport layer, see Annex 1.

Annex 6

Resource sharing method for VDES Terrestrial and Satellite Services

# 1	Introduction

This Annex describes how resource sharing (i.e. in time and frequency) for utilizing the VHF spectrum available between different VDES services and stations should be accomplished.

A ship may be within range of multiple controlling shore stations.  This annex describes a method for coordinating time and frequency resources between multiple controlling shore stations, particularly the use of bulletin boards and announcement signalling channels, as defined in Annex 1 and 3.

The VDE-SAT is an effective means to extend the VDES to areas outside of coastal VHF coverage. However, due to the large footprint of satellite, the VDE-SAT downlink signal may interfere with VDE-TER in the coastal areas when satellite is in visibility. Similarly, the terrestrial ship-to-shore VDE signals can interfere with the satellite reception of VDE-SAT uplink when a VDE Satellite is in the field of view. The method described in this Annex for resource sharing is derived based on the characteristics of VDE-TER and VDE-SAT, particularly the use of bulletin board and announcement signalling channels, as defined in Annexes 1, 3 and 4.

The channels 24, 84, 25, 85, 26 and 86 are allocated for VDE, with the lower leg frequencies used for ship-to-shore and the upper leg frequencies used for shore-to-ship and ship-to-ship. The channels 2027 (ASM 1) and 2028 (ASM 2) are allocated for ASM. Currently, 2 alternative frequency utilization plans for VDES are under consideration. They describe how resources are allocated and shared between VDE-TER, VDE-SAT and ASM. These 2 alternative frequency utilization plans are illustrated in Fig. A6-1, and described further below.

Figure A6-1

RR APPENDIX 18 and VDES frequency utilization plans

## Frequency plan alternative 1

Frequency plan alternative 1 allow for utilization of the channels 24, 84, 25, 85, 26 and 86 in a shared manner between VDE-TER and VDE-SAT.

–	The four channels 1024, 1084, 1025 and 1085 are shared between ship-to-shore and ship-to-satellite (VDE-SAT uplink) services

–	The two channels 1026 and 1086 are exclusively reserved for ship-to-satellite (VDE-SAT uplink) services

–	The four channels 2024, 2084, 2025 and 2085 are shared among shore-to-ship, ship-to-ship and satellite-to-ship (VDE-SAT downlink) services

–	The two channels 2026 and 2086 are exclusively reserved for satellite-to-ship (VDE-SAT downlink) services.

–	Two channels 2027(ASM 1) and 2028 (ASM 2) are shared between ship-to-shore, ship-to-ship, shore-to-ship and ship-to-satellite services

## Frequency plan alternative 2

Frequency plan alternative 2 allow for utilization of channels 24, 84, 25 and 85 primarily for VDE-TER, while channels 26 and 86 exclusively reserved for VDE-SAT uplink. VDE-SAT uplink is also possible in channels 24, 84, 25 and 85, but the VDE-SAT uplink in these channels do not impose constraints on VDE-TER. Frequencies are exclusively reserved for VDE-SAT downlink within the frequency range 160.9625 MHz to 161.4875 MHz, which is not channelized in RR Appendix 18.

–	The four channels 1024, 1084, 1025 and 1085 are reserved for ship-to-shore services, but ship-to-satellite (VDE-SAT uplink) services are possible without imposing constraints on ship-to-shore services

–	The four channels 2024, 2084, 2025 and 2085 are reserved for shore-to-ship and ship-to-ship services, but ship-to-satellite (VDE-SAT uplink) services are possible without imposing constraints on shore-to-ship and ship-to-ship services

–	The four channels 1026, 1086, 2026 and 2086 are exclusively reserved for ship-to-satellite (VDE-SAT uplink) services.

–	Frequencies are exclusively reserved for satellite-to-ship (VDE-SAT downlink) services within the frequency range 160.9625 MHz to 161.4875 MHz, which is not channelized in RR Appendix 18

–	Two channels 2027(ASM 1) and 2028 (ASM 2) are shared between ship-to-shore, ship-to-ship, shore-to-ship and ship-to-satellite services

# 2	VDES resource sharing principles

## 2.1	AIS priority

Understanding that when transmissions occurs a VDES mobile station with a single antenna will suffer decreased receiver sensitivity, care must be taken to respect the AIS transmission and reception as the highest priority.

## 2.2	Coordination between ASM and VDE

VDE ship transmissions should be coordinated with transmissions on the ASM channels to ensure that ASM messages with new safety and navigational related information can be received.

## 2.3	Shore station VDES control area

The VDES resource assignments in the proximity of a shore station is monitored and controlled by a shore station. Shore stations utilize terrestrial bulletin board (TBB) and announcement signalling channels (ASC) to coordinate the resource assignment within the control area. The shore station may incorporate information regarding VDE satellite communications within the TBB and ASC. The shore station may acquire the VDE satellite information directly from the VDE-Satellite downlink (the satellite bulletin board and ASC) or in coordination with the satellite service providers.

There are dedicated slots and frequency bands for TBB and ASC that are reserved to communicate the required information to each vessel in the control area of a shore station. The default (or initial) assignment are described in § 4 of this Annex.

## 2.4	VDE-SAT resource assignment

Each satellite should use bulletin board and announcement channels (as defined in Annex 1 and 4) to communicate the VDE-SAT resource assignments (both downlink and uplink) to vessels in the coverage area.

There are dedicated slots and frequency bands for the satellite bulletin board and announcement channels that are reserved to communicate the required information to each vessel in the field of view of a satellite.

Since the satellite coverage may include several shore station control areas, the VDE-SAT resource assignment should respect all requirements of shore control areas that are within the field of view at any given time. Within each satellite orbit the information regarding the resource assignment should be updated according to the shore station control areas in the satellite field of view.

A default (or initial) VDE-SAT resource allocation is defined in § 4 below to serve as the starting point for the resource sharing.

# 3	VDE-TER Resource sharing between multiple controlling shore stations

The allocation of frequency and time slots used for the bulletin board announcement must be coordinated between controlling stations.  Other resource assignments are managed based on the content of the bulletin board and announcement signalling channels. The assignment may change dynamically (according to temporal demands).

There are dedicated resources in channel 2024 and 2084 that are assigned to the terrestrial bulletin board and announcement channels, as described in Annex 1 and 3.

Channels 2024, 2084, 2025 and 2085 are shared between multiple controlling stations. The resource sharing must be coordinated between shore station operators. This coordination can be done either directly between the operators or rely on the bulletin board and announcement channels of the shore stations, depending on the shore control areas, the resource assignment may vary. As an initial configuration for resource sharing, the controlling shore stations should adopt a static assignment in time and frequency.

# 4	VDE-TER and VDE-SAT downlink resource sharing

## 4.1	Resource sharing with frequency plan alternative 1

With frequency plan alternative 1, the channels 2026 and 2086 are dedicated to VDE-SAT downlink. Within these exclusive VDE-SAT channels, there are dedicated time slots that are assigned to the satellite bulletin board and announcement signalling channels as described in Annex 1 and 4. Other slot assignments in the exclusive VDE-SAT frequency bands are managed based on the content of the bulletin board and announcement signalling channels. The assignment may change dynamically (according to the satellite coverage or temporal demands).

Channels 2024, 2084, 2025 and 2085 are shared between VDE-SAT Downlink and VDE-TER. Depending on the satellite coverage area and the shore control areas, the resource assignment may vary.

There are dedicated time slots in channel 2024 and 2084 that are assigned to the terrestrial signalling channel and terrestrial bulletin board, as described in Annex 1 and 3. These slots should not be used by VDE-SAT downlink when a VDE shore station is within the satellite coverage area.

A shore station may assign the full resources of channels 2024, 2084, 2025 and 2085 for terrestrial services when there is no transmitting VDE satellite in the field of view.

When a transmitting VDE satellite is in the field of view the resource sharing between VDE-SAT downlink and VDE shore-to-ship and ship-to-ship must be coordinated between the shore operator and the satellite operator. This coordination can be done either directly between the operators or rely on the bulletin board and announcement channels of the satellite and shore stations. As an initial configuration for resource sharing, a static assignment in time and frequency should be adopted by the terrestrial and satellite entities.

–	Channels 2024 and 2084 are exclusively used for terrestrial VDE, maintaining the original signalling assignment that was described above

–	Channels 2026 and 2086 are exclusively used for VDE-SAT downlink, maintaining the original signalling assignment that was described above

–	Channels 2025 and 2085 are time-shared between VDE-SAT downlink and VDE terrestrial services. The time sharing is based on time intervals of 2.4 s (90 slots) that are assigned periodically to VDE-SAT and VDE terrestrial services

As the starting point of VDES resource sharing or in the absence of coordination between the shore and satellite operation, this resource sharing method should be used.

Coordination of resource sharing between VDE ship-to-ship and VDE-SAT downlink for areas not controlled by a VDE shore station is managed by the VDE-SAT bulletin board, as described in Annex 4. As a starting point for this resource sharing or in the absence of any VDE-SAT bulletin board, the resource sharing method described above should be used.

## 4.2	Resource sharing with frequency plan alternative 2

With frequency plan alternative 2, the frequency band from 160.9625 MHz to 161.4875 MHz is dedicated to VDE-SAT downlink. The frequencies in this band are not channelized in RR APPENDIX 18. Within this exclusive VDE-SAT band, there are dedicated channels and time slots that are assigned to the satellite bulletin board and announcement signalling channels as described in Annex 1 and 4. Other slot assignments in this exclusive VDE-SAT frequency band are managed based on the content of the bulletin board and announcement signalling channels. The assignment may change dynamically (according to the satellite coverage or temporal demands).

# 5	Sharing between different VDE satellite systems

The sharing between two or more satellite systems is coordinated between the satellite operators and organized through the bulletin board, delivered by satellites in VDE-SAT downlink band, as described in Annex 1 and 4. Ships use the satellite bulletin boards for channel and resource configuration.

The waveform used for the bulletin board should allow for detection of overlapping signals received from multiple satellites. The use of direct sequence spreading as defined in Annex 4 allows for detection of up to 8 overlapping satellite signals.

# 6	VDE-TER and VDE-SAT uplink resource sharing

## 6.1	Resource sharing with frequency plan alternative 1

With frequency plan alternative 1, the lower frequency bands, channel 1026 and 1086 are dedicated to VDE-SAT uplink while channels 1024, 1084, 1025 and 1085 are shared between VDE-TER and VDE-SAT.

The exclusive VDE-SAT uplink channels may be used for dedicated (demand assigned) or random access to satellite. Since there is no VDE terrestrial interference on these two channels, these channels should be used for higher priority message (safety, distress, acknowledgement, etc.).

Through the bulletin board, a shore station may assign the full resources of channels 1024, 1084, 1025 and 1085 for terrestrial services when there is no receiving VDE satellite in the field of view.

When a transmitting VDE satellite is in the field of view the resource sharing between VDE-SAT uplink and VDE-TER ship-to-shore must be coordinated between the shore operator and the satellite operator. This coordination can be done either directly between the operators or rely on the bulletin board and announcement channels of the satellite and shore stations. As an initial configuration for resource sharing, a static assignment in time and frequency should be adopted by the terrestrial and satellite entities.

–	Channels 1024 and 1084 are exclusively used for VDE-TER ship-to-shore

–	Channels 1026 and 1086 are exclusively used for VDE-SAT uplink (ship-to-satellite)

–	Channels 1025 and 1085 are time-shared between the VDE-SAT uplink and VDE-TER services. The time-sharing is based on time intervals of 1 hexslot (6 slots) that are assigned alternately to VDE-SAT and VDE-TER services

As the starting point of VDES resource sharing or in the absence of coordination between the shore and satellite operation, this resource sharing method should be used.

## 6.2	Resource sharing with frequency plan alternative 2

With frequency plan alternative 2, the utilization of channels 24, 84, 25 and 85 is primarily for VDE-TER. VDE-SAT uplink is also possible in channels 24, 84, 25 and 85, but the VDE-SAT uplink in these channels do not impose constraints on VDE-TER and should only use resources not reserved by VDE-TER.

Channels 26 and 86 exclusively reserved for VDE-SAT uplink. Therefore, on these channels no resources are shared and no sharing scheme is required.

θ°=earth-satellite elevation angle

PFDθ° (dBW/(m2*4 kHz))=-149+0.16*θ°                        0°≤θ<45°;-142+0.53*θ°-45°      45°≤θ<60°;-134+0.1*θ°-60°         60°≤θ≤90°.

-

|  |  |  |  |  |  |  | - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  |  |  |  | - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  |  |  |  | - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

-

-

-

-

-

Annex 7

Original design considerations to validate the VDES concept

# 1	Introduction

This annex provides additional information on the technical considerations of the VDES. It identifies aspects of both terrestrial and satellite VDE components, including access scheme options, antenna design and system sharing.

The annex reflects all original materials that were used to develop Annexes 2 through 6.

# 2	Summary of operational capability and performance

This Annex demonstrates the following operational capability and performance:

–	Protection of GMDSS and AIS, i.e. recognizing that the implementation of VDES must ensure that the function of digital selective calling, AIS and voice distress, safety and calling communication (channel 16), are not impaired

–	Relief of AIS VDL congestion

–	Raw ASM data transfer at 28.8 kbits/s

–	Raw VDE data transfer ship-to-ship, ship-to-shore and shore-to-ship at 307.2 kbits/s

–	Raw VDE satellite data transfer up to 240 kbits/s

–	VDE satellite downlink that satisfies the PFD mask requirements

–	VDE shore-to-ship and ship-to-shore service to 85 km (46 NM)

–	Channel access and sharing schemes that organize the links and mitigate conflicts

–	Full VDES satellite and terrestrial functionality from a single shipborne antenna.

# 3	Technical considerations for VHF data exchange system access schemes

This section provides technical considerations in designing access schemes for VDE terrestrial, VDE Satellite and the interaction between these VDES components.

From Figure A1-1, the satellite downlink shares the spectrum with the terrestrial ship-to-ship and shore-to-ship links, and the satellite uplink shares the spectrum with the terrestrial ship-to-shore link. Thus, access schemes should be considered to mitigate potential conflicts between the links.

# 4	Time division multiple access scheme for the VHF date exchange terrestrial service

The VDES terrestrial service is comprised of ASM, VDE ship-to-shore, VDE shore-to-ship and VDE ship-to-ship. An example of a shipborne VDES transceiver implementation is illustrated in Fig. A7-1. Note that in this implementation example all receivers, including the AIS receivers, are protected from blocking from the shipborne VHF radio by the band-pass filter that attenuates signals from the lower side of RR Appendix 18 band. The AIS receiver blocking issue, along with the fact that the AIS can share the same antenna with the other VDES functions, is incentive for manufacturers to consider this implementation for their VDES system designs.

Figure A7-1

Example VHF data exchange system transceiver implementation

## 4.1	Time division multiple access schemes

### 4.1.1	Time division multiple access scheme for the VHF data exchange system application specific message channels

Note that Recommendation ITU-R M.1371 specifies the access schemes for the AIS messages, including ITDMA, on the AIS channels. It also specifies the structure for ASM and the content options for these messages. VDES provides dedicated ASM channels to relieve congestion on the AIS channels. Under VDES, the access scheme for using the ASM channels could initially be Carrier-Sense TDMA (CSTDMA) for the first transmission in a frame, followed by ITDMA for subsequent transmissions in that frame. This scheme mitigates simultaneous transmissions by ships and/or shore stations on the ASM channels. An ASM transmission should not exceed five contiguous slots.

### 4.1.2	Time division multiple access scheme for the VHF data exchange system ship-to-shore link

The TDMA access scheme for the VDE1-A, ship-to-shore link, could be by reservation through ITDMA from an ASM on either one of the ASM channels, as described in § 4.1.1. A VDE1-A ship-to-shore transmission should not exceed five contiguous slots.

### 4.1.3	Time division multiple access scheme for the VHF data exchange system ship-to-ship link

The TDMA access scheme for the VDE1-B, ship-to-ship link, could be the same as for the ASM channels, i.e. initially by CSTDMA for the first transmission in a frame, followed by ITDMA for subsequent transmissions in that frame. This scheme mitigates simultaneous ship-to-ship transmissions. A VDE1-B ship-to-ship transmission should not exceed five contiguous slots.

### 4.1.4	Time division multiple access scheme for the VHF data exchange system shore-to-ship link

The TDMA access scheme for the VDE1-B, shore-to-ship link, could be the same as for the VDE1 ship-to-shore link, i.e. by reservation through ITDMA from an ASM on either one of the ASM channels. This is necessary because the shore station usually has a very wide coverage area compared to ships, and it needs to have priority access to the VDE1 channel in its coverage area. A VDE1-B shore-to-ship transmission should not exceed five contiguous slots.

## 4.2	Sharing options for the VHF data exchange terrestrial and VHF data exchange satellite services

### 4.2.1	VHF data exchange terrestrial links on the upper legs (VDE1-B) and VHF data exchange satellite downlink

Table A4-1 provides the PFD at the Earth’s surface from the satellite downlink at various elevation angles from 0 to 90. Although the PFD mask is selected to minimize interference to the land mobile service and to maximize reception by ship VDES stations, there is a potential effect of raising the noise floor for reception of the terrestrial VDES links during satellite VDE downlink transmissions when the satellite is the field of view.

Issues to be considered for the sharing the VDE1-B frequencies and the VDE-SAT Downlink are:

–	When shipborne VDES transceivers are simplex they cannot receive while transmitting

–	VDE-SAT downlink transmission levels, by raising the noise floor, will potentially have an impact on reception of ship-to-ship and shore-to-ship VDES

–	Ship-to-ship and shore-to-ship VDES transmissions, depending on the distance, by co-channel interference, will potentially interfere with reception of the VDE-SAT downlink.

#### 4.2.1.1	Frequency division multiple access

Frequency division multiple access (FDMA) is accomplished by using only the upper 50 kHz for the VDE-SAT downlink, i.e. the two channels 2026 and 2086. The FDMA would mitigate the last two issues stated above. Compared to other techniques proposed below, the FDMA would be the most straightforward to implement. However it would result in a reduction of the bandwidth to 1/3, and cause the VDE-SAT downlink transmissions to last three times longer for the same payload, and it would not mitigate the first issue stated above.

#### 4.2.1.2	Time division multiple access

TDMA approach for shore-to-ship/ship-to-ship and VDE-SAT downlink services would allow the full use of the spectrum assigned to each service in a time sharing manner. Time sharing can mitigate all the three of the issues stated in § 4.2.1 above. However, it would impose some design challenges for the VDE-SAT components and compromise the throughput of the VDE-SAT downlink.

The AIS-based TDMA slot structure (2 250 slots/minute/frame) and access schemes (ITDMA, CSTDMA and FATDMA) that are used for VDES are defined in Recommendation ITU-R M.1371. This TDMA organization scheme protects the integrity of the AIS and is used to organize and synchronize the ASM and VDE transmissions.

#### 4.2.1.3	Full frequency reuse (simultaneous transmission)

In this approach, the terrestrial and satellite components are allowed to simultaneously use channels 2024, 2084, 2025 and 2085. The VDE-SAT downlink will additionally use channels 2026 and 2086. The VDE-SAT downlink could continuously broadcast to maximize the data dissemination to a large number of ships in its field of view. This would allow for more efficient implementation of the VDE-SAT receivers. The interference caused by the VDE-SAT downlink on the VDE terrestrial could, in principle, be compensated for by the use of more protected coding scheme in the terrestrial link, only during the satellite passage.

For a most likely scenario of Low Earth Orbit satellites with a polar orbit, the impact of satellite interference could be limited to only less than 15 minutes per day per satellite for geographical locations with latitudes within ±50 degrees, as shown in Fig. A7-2.

Figure A7-2

Time duration where signal level exceeds −117 dBm as a function of geographical position

### 4.2.2	VHF data exchange terrestrial (VDE1-A) and VHF data exchange satellite uplink

Due to the large field of view, a passing satellite would receive a number of colliding messages from different VDE-terrestrial links (ship-to-shore) simultaneously that would interfere with ship-to-satellite links (channels 1024, 1084, 1025 and 1085). The following multiple access schemes can be envisaged to mitigate/minimize the impact of VDE terrestrial link on VDE satellite uplink.

#### 4.2.2.1	Frequency division multiple access

The frequency division multi-access scheme separates the satellite channels into two groups: Channels 1024, 1084, 1025 and 1085 that are subject to terrestrial interference are considered as a single or multi-carrier satellite uplink channel(s). Highly robust waveforms would be selected for these channels to allow for interference mitigations caused by VDE terrestrial.

The second group of carriers are considered to occupy channels 1026 and 1086 where no VDE terrestrial transmission is present.

#### 4.2.2.2	Time division multiple access

VDE-SAT uplink follows the same frame structure as VDE terrestrial occupying VDE1-A channels. There are pre-assigned slots dedicated to satellite transmission preventing interference from any VDE terrestrial link.

Recommendation ITU-R M.1371 specifies the access schemes for the AIS Messages, including ITDMA, on the AIS channels, and it specifies the structure for ASM with various contents. VDES provides dedicated ASM channels to relieve congestion on the AIS channels. Under VDES, the access scheme for using the ASM channels could be initially by CSTDMA (Carrier-Sense TDMA) for the first transmission in a frame, followed by ITDMA for subsequent transmissions in that frame. This scheme mitigates simultaneous transmissions by ships and/or shore stations on the ASM channels.

#### 4.2.2.3	Full frequency reuse

The terrestrial and satellite components are allowed to simultaneously use channels 1024, 1084, 1025 and 1085. The VDE-SAT uplink would use properly designed waveforms occupying the VDE-SAT uplink channels to minimize the impact of interference caused by the VDE terrestrial transmissions.

# 5	VHF data exchange – terrestrial

## 5.1	Waveforms for VHF data exchange

### 5.1.1	Transmission waveforms for VHF data exchange terrestrial links

ITU-approved waveforms for spectrum-efficient data transmission in the VHF maritime band are described in Recommendation ITU-R M.1842. These waveforms have been demonstrated in the land-mobile service and in maritime trials, to provide robust data service and to mitigate multipath degradation at extended propagation ranges in intense electromagnetic environments. Table A7-1 below provides a comparison of performance between the current AIS standard, Recommendation ITU-R M.1371, and the new applications introduced for the terrestrial VDES links, ASM and VDE. Note that the spectrum efficiency for the AIS is much lower than for VDES, but the AIS modulation has superior co-channel rejection which provides better range discrimination and improved safety of navigation for ships. Each modulation type is intended to best fit its designated application (AIS, ASM and VDE).

Propagation range predictions for the terrestrial links are provided in Annex 3 in accordance with the ITU propagation standard Recommendation ITU-R P.1546-5.

TABLE A7-1

ITU-standard transmission waveforms for automatic identification system, application specific message and VHF data exchange terrestrial links

|  | 25 kHz Channels for AIS | 25 kHz Channels for ASM | 100 kHz Channels for VDE |
| --- | --- | --- | --- |
| ITU standard | ITU-R M.1371 | ITU-R M.1842-1 Annex 1 | ITU-R M.1842-1 Annex 4*** |
| Digital modulation | GMSK, single carrier | π/4 DQPSK, single carrier | 16-QAM, 32 multi-carriers, 2.7 kHz spacing |
| Data rate (raw)* | 9.6 kbits/s (1X) | 28.8 kbits/s (3X) | 307.2 kbits/s (32X) |
| Sensitivity** | −107 dBm (min) −112 dBm (typical) | −107 dBm (min) −112 dBm (typical) | −98 dBm (ships) −103 dBm (base stations) |
| Co-channel rejection (CCR)** | 10 dB | 19 dB | 19 dB |
| AIS message types | 1, 2, 3, 5, 18, 19, 27 … | 6, 7, 8, 12, 13, 14, 25, 26 and ASM | VDE messages |
| Rationale | Optimum choice (better CCR) for position reports in a ship-to-ship navigation safety environment. | Provides higher (3X) data transmission than AIS. Inferior CCR (+9 dB) and range discrimination compared to AIS. | Provides much higher (32X) data transmission than AIS. Inferior CCR (+9 dB) and range discrimination compared to AIS. |
| * These figures are raw, over the air, bit transmission rates. The data rates are less, subject to coding, packet structure and forward error correction (FEC). ** These figures are based on published standards. For AIS, the standard is IEC 61993-2 and for VDE the standard is ETSI EN 300 392-2 version 3.4.1, which refers to a land mobile application TETRA. *** For greater robustness where needed, ITU-R M.1842-1 Annex 1 may be used. |  |  |  |

### 5.1.2	Transmission waveform for the 25 kHz application specific message channels

Transmission of ASM on 25 kHz channels should be by π/4 DQPSK single-carrier modulation as described in Annex 1 of Recommendation ITU-R M.1842-1. FEC is applied due to the fact that the ASM messages are not repeated as are AIS position reports (which do not have FEC). The waveform is recommended because it has high sensitivity, 70 dB adjacent channel power ratio (ACPR) and 28.8 kbits/s data rate.

–	It is generated by phase modulation with an inter-symbol rotation of π/4 radians. This produces an amplitude envelope with very moderate peak to average power ratio (PAPR);

–	It has excellent characteristics for detection by satellites as required by the channel plan.

### 5.1.3	Transmission waveform for the 100 kHz VHF data exchange channels

Transmission of VDE on 100 kHz channels should be by 16-QAM, 32 multi-carriers, with 2.7 kHz spacing and 307.2 kbits/s data rate as described in Recommendation ITU-R M.1842-1 Annex 4. This multi-carrier scheme is not OFDM (orthogonal frequency division multiple access) since the carrier spacing is 2.7 kHz which provides more inter-carrier margin than OFDM which would require 2.4 kHz spacing. This waveform is comprised of 32 multi-carriers. Each carrier is modulated by 16-QAM to generate 4-bit symbols at 2 400 symbols/s (2 400 symbols/s/carrier  4 bits/symbol = 9 600 bits/s/carrier).

The long symbol duration (2 400 symbols/s = 416.7 µs/symbol) is designed to mitigate multi-path inter-symbol interference, since (ref: Report ITU-R M.2317) reflections in a 100 kHz maritime channel environment have been found to be contained primarily within the first 10.4 µs. It is noted that further reflections were beyond this, some as far as 50 µs. By comparison, note that AIS uses GMSK to generate 2-bit symbols at 4 800 symbols/s (9 600 bits/s) and that its excellent propagation characteristics have been proven in practice.

The modulation, coding and scrambling techniques described in EN 300 392-2 v.3.4.1 are combined to reduce the amplitude envelope PAPR (PAPR ≤ 10dB) to mitigate the RF power transmitter design difficulty. Both analogue, e.g. Doherty amplifier (DA), and digital, e.g. envelope tracking (ET) and digital pre-distortion (DPD), design techniques for RF power amplifiers are available to provide better than 50% efficiency with this waveform. By comparison, the AIS power amplifiers used by ships and base stations are also approximately 50% efficient. A technical report describing these techniques and others for modern high efficiency power amplifiers with actual test results can be found at: http://www.microwavejournal.com/articles/21965-modern-high-efficiency-amplifier-design-envelope-tracking-doherty-and-outphasing.

Note that the analogue design approach using Doherty amplifiers provides efficiency over 50% and the original patent for this technology has expired. Solid state Doherty amplifiers are currently in service in cellular terrestrial infrastructures which produce the range of power levels needed for shipborne VDES transceivers (12.5 W) and VDES base stations (50 W).

## 5.2	Antenna options for VHF data exchange system terrestrial stations

Commercially available antenna options for the VDES terrestrial stations are characterized in Fig. A7-3 below. Since the shipborne antenna is required to receive the VDES satellite downlink at high elevation angles, the 0 dBd (2.1 dBi) option is selected. To achieve optimum satellite reception, this antenna should be mounted as high as possible, preferably on an extension pole, on the ship to minimize obstructions to the antenna’s view of the horizon. For the terrestrial VDES base station, the 6 dBd (8 dBi) option is selected. These two antennas are used in the propagation range predictions in Annex 2.

Figure A7-4 presents a mask for the receiving antenna gain as a function of elevation that would allow the received signal from satellite to be at constant power level at the receiver input for a wide range of elevation angles, taking into account the PFD constraints imposed on the VDE-SAT downlink (ref. Table A4-1 of Annex 1). Although this mask may not represent the antenna pattern associated with a commercially available antenna, it could serve as a guide for designing an antenna to enhance the satellite reception. The same mask is also applicable to the design of shipborne antenna for VDE terrestrial link due its high directivity in the horizontal direction. Annex 3 provides further rationale for the selection of this mask.

Figure A7-3

Antenna options for shipborne VHF data exchange system stations

Figure A7-4

Mask for “Ideal” antenna

Antenna gain dBi

Elevation in degrees

Gain in dBi

Antenna gain in dBi

Antenna gain dBi

Elevation in degrees

Gain in dBi

Antenna gain in dBi

# 6	VHF data exchange by satellite

VHF data exchange by satellite should use the channels designated for satellite in Table A1-1 of Annex 1 and should be in accordance with this Recommendation. This is further described below.

## 6.1	General

### 6.1.1	VHF data exchange system satellite component

The VHF data exchange VDE satellite component is an effective means to extend the VDES to areas outside of coastal VHF coverage. Hereafter, the satellite component is referred to as the VDE-SAT.

Satellite communications is able to deliver information in a broadcast, multicast or unicast mode to a large number of ships, i.e. efficiently addressing many ships using only minimal radio spectrum resources.

The VDE-SAT provides a communication channel that is complementary to the terrestrial components of the VDES system (i.e. coordinated with terrestrial VDE, ASM and AIS functionalities and their supporting systems).

### 6.1.2	Applications

Continuous exchanges with the maritime community will provide further insight into the priorities, quality of service, security, integrity and other requirements of future VDES services.

There is a large population of smaller size ships which have no satellite communication equipment on board, but do have regular VHF/AIS reception equipment that could benefit from the services mentioned above. This would be of particular benefit for vessel populations in areas with limited shore based infrastructure.

Using low-cost satellite reception technology, VDE-SAT can address a large population of ships and offer services for small vessels, fishing vessels and recreational vessels.

## 6.2	Overall architecture, operational characteristics and assumptions

### 6.2.1	Architecture

The VHF data exchange system architecture is shown in Fig. A7-5 below. The VDE-SAT is composed of one or more satellites transmitting and receiving in the maritime VHF bands this is the space segment.

Due to the frequencies used, it is likely that VDE-SAT will consist of low-earth orbiting (LEO) or medium-earth orbiting (MEO) satellites. VDE-SAT could also consist of hosted payloads on spacecraft in such orbits.

The VDE-SAT user terminals may be integrated in ship-borne VDES equipment. This is called the user segment. These terminals could be integrated in the terrestrial VDE equipment along with ASM and AIS functionalities. Also VDE-SAT receive-only terminals can be considered: these would provide a very cost-effective means to disseminate maritime information to smaller ships outside terrestrial VHF coverage, for example in areas with limited shore based infrastructure.

There will be a ground segment which consists of one of more ground stations that will send and receive maritime information to/from ships for further processing or dissemination, via the space segment. Communication between the coastal VDE station, maritime information provider, 
VDE-SAT ground station and feeder link is not part of the VDES architecture.

Figure A7-5

VHF data exchange-satellite component architecture

### 6.2.2	Operational characteristics

The VDE-SAT should complement the VDE terrestrial in areas in which no terrestrial VDE coverage is available, i.e. at the high-seas.

The VDE-SAT should provide a downlink capability (i.e. allow to send information from a ground station to one or more ships). Note that VDE-SAT will likely use its specific unicast, multicast or broadcast capability which is inherent in a satellite downlink.

The VDE-SAT should provide an uplink capability (i.e. allow a ship to send information to the satellite, for further relaying to a ground station).

As VDE-SAT will be based on LEO or MEO satellite(s), provisions will need to be taken for the discontinuous contact that ships will have with individual satellites. Furthermore, if there are multiple VDE-SAT satellites or payload footprints that overlap, some coordination between them may be required.

It is proposed that VDE-SAT supports priority, pre-emption and precedence for different services; this could be mapped into different downlinks.

## 6.3	Technical characteristics

### 6.3.1	VHF data exchange-satellite channels and spectrum

The VDE-SAT downlink should be used for data downlink from the satellite to vessels in a broadcast, multicast or unicast manner. The VDE-SAT should also provide data uplink from vessels to satellites using one or several multiple-access schemes. The VHF data exchange system via satellite uses the channel allocation shown in Fig. A7-6.

Figure A7-6

VHF data exchange system channel allocation

#### 6.3.1.1	SAT downlink

The satellite downlink frequency spectrum consists of six 25 kHz channels (2024 to 2086). These channels may be bundled into one 150 kHz channel to reduce the guard band (needed due to the frequency Doppler shift of incoming signals from LEO satellites), increase the throughput, and more importantly, improve the power efficiency of the satellite power amplifier (avoiding multi-carrier transmission which typically requires a larger output back-off) (refer to § 5.1.3).

Due to the PFD limit imposed on the VDE-SAT downlink (as part of sharing the frequencies with land mobile), a certain level of redundancy (in the form of frame repetition, forward error correction or higher layer redundancy) is implemented in the VDE-SAT protocol in order to mitigate the error and enhance the data detection probability.

The VDE-SAT downlink signal also includes repeated known symbols (e.g. pilots, preamble, post-amble) to facilitate signal detection and synchronization as well as possible interference mitigation and channel estimation. In order to avoid unwanted in-band spectral lines, the data symbols are scrambled with a known sequence. The example in § 6.4.2.19 concludes that a downlink data rate of 240 kbits/s is possible.

The signal level generated by the satellite should be kept below the PFD mask limit (referred to the Earth’s surface) specified in Table A7-2 below. Note that this is based on coordination with terrestrial VHF services and that the PFD level refers to the vertical component of radiation normal to the Earth’s.

TABLE A7-2

Power flux-density mask

θ°=earth-satellite elevation angle

PFDθ° (dBW/(m2*4 kHz))=-149+0.16*θ°                        0°≤θ<45°;-142+0.53*θ°-45°      45°≤θ<60°;-134+0.1*θ°-60°         60°≤θ≤90°.

This PFD mask is to ensure that there is no harmful interference caused by the satellite downlink on non-maritime terrestrial services sharing the same frequency (ensuring in-band carrier-to-interference requirements of terrestrial service receivers).

#### 6.3.1.2	SAT 3 uplink

The frequency spectrum corresponding to 6 lower VDE channels (starting from Channel 1024) are used for satellite data uplink. Compared to the AIS channels, and long range AIS, these 6 channels provide a significant data uplink capability via satellite.

The access scheme protocol for data uplink via satellite is designed to take into account the entire satellite field of view and to maximize the probability of message detections by avoiding message collisions.

### 6.3.2	Rationale of channel allocation for VHF data exchange-satellite

The frequency plan for the entire VDES, as depicted in Fig. A7-6 above, facilitates a realistic implementation of the proposed system in co-existence with, and complementing, the current AIS. The following points regarding the frequency plan are highlighted:

−	The requirements for VDES concentrate the reception frequencies on board of the ship to a limited range of 250 kHz at the upper maritime VHF band. This provides an efficient implementation of VDES on-board receivers by narrowing the input filter bandwidth, reducing potential impairments due to other activities within the maritime VHF band;

−	The VDE-SAT downlink shares the same frequency range as the terrestrial VDE and AIS. This allows sharing the same antenna as well as the receiver front-end design;

−	Satellite and shore reception frequencies of shipborne VDE signals occupy the lower end of the VHF maritime band. This allows for a complementary service close to the shore and at the high sea while sharing the same spectrum. The frequency separation between the upper and lower spectra (with 4.6 MHz separation) provides an acceptable level of isolation between VDES receiving chain and the VDE ship-borne transmitters;

−	The frequency separation between the uplink and downlink allows hosting VDE-SAT transmitter and receiver on the same satellite which allows for a more cost-effective satellite mission concepts (i.e. reduce number of satellites, improved efficiency and possible interactivity).

## 6.4	Example VHF data exchange system satellite implementation

The following example VDES satellite implementation fits the PFD angular mask and supports the requirements of this Recommendation.

### 6.4.1	Determine the VHF data exchange system satellite orbital characteristics

The following VDES satellite implementation is considered. The satellite orbital characteristics that are needed to support this application are determined as follows.

#### 6.4.1.1	Determine the satellite’s orbit

The example VDES satellite employs a polar orbit at a height of 550 km above the surface of the Earth. The velocity, acceleration and orbital period of the satellite are determined, given: Mearth = 5.98  1024 kg, Rearth = 6.37  106 m.

The satellite’s orbit and the known and unknown parameters are shown in Fig. A7-7 below.

Figure A7-7

Satellite orbital characteristics

| Known: R = Rearth + height = 6.92 106 m Mearth = 5.98 1024 kg G = 6.673 10−11 N m2/kg2; Unknown: v a T. |  |
| --- | --- |

The radius of a satellite’s orbit can be determined from the Earth’s radius and the height of the satellite above the Earth. As shown in Fig. A7-7, the radius of orbit for a satellite is equal to the sum of the Earth’s radius and the height above the Earth. These two quantities are added to yield the orbital radius. The 550 km altitude is first converted to 0.550  106 m and then added to the radius of the Earth.

Determine the velocity of the satellite,

v = SQRT ((G  Mсentral ) / R)

v = SQRT ((6.673  10−11 N m2/kg2)  (5.98  1024 kg) / (6.92  106 m))

v = 7.594  103 m/s.

Determine the acceleration of the satellite,

a = (G  Mcentral)/R2

a = (6.673  10-11 N m2/kg2)  (5.98  1024 kg) / (6.92  106 m)2

a = 8.333 m/s2.

Determine the orbital period of the satellite,

T = SQRT ((4  π2  R3) / (G  Mcentral))

T = SQRT ((4  (3.1415)2  (6.92  106 m)3) / (6.673  10−11 N m2/kg2)  (5.98  1024 kg))

T = 5725.7 s = 1.59 h.

### 6.4.2	VHF data exchange system satellite antenna and downlink characteristics

A directional vertically polarized Yagi-Uda antenna is used for communicating with ships’ vertical antennas and also for conformance with the PFD angular mask.

#### 6.4.2.1	Determine the Earth’s rotation at the equator between each satellite orbit

The period of the Earth Te is approximately 24 hours (86.4  103 s), the radius of the Earth Re is 6.37  106 m and the circumference of the Earth (distance around the equator) is Cearth = 2  (3.1415)  (6.37  106 m) = 40.0239  106 m. Therefore, in each pass of the satellite, the Earth will have rotated at the equator by ROTequator = Cearth  T / Te = 40.0239  106 m  5725.7 s /86.4  103 s = 2.6524  106 m = 2652.4 km.

#### 6.4.2.2	Determine the slant distance to the Earth’s horizon

The slant distance Ds from the satellite to the Earth’s horizon is Ds = SQRT (R2 – Re2) = SQRT ((6.92  106 m)2 – (6.37  106 m)2) = 2.7036  106 m = 2 703.6 km.

#### 6.4.2.3	Determine the slant downward tilt angle to the Earth’s horizon

The satellite’s downward tilt angle to the Earth’s horizon is:

θd = 90 − sin−1 (Re / R) = 90 − sin−1(6.37  106 m / 6.92  106 m) = 90 − 67 = 23 degrees.

#### 6.4.2.4	Determine the width of the antenna coverage path

The example VDES satellite antenna pattern is shown in Fig. A7-8 below. The beam width (±3 dB) of the antenna is 80 degrees. The width of the satellite antenna’s coverage path is:

Wc = 2 (Ds cos (90 − θa/2))

Wc = 2  2.7036  106 m  cos (90 − 80 / 2) = 3.4757  106 m = 3 475.6 km.

NOTE: From 4.2.1 that since ROTequator = 2 652.4 km, this antenna beamwidth (θa = 80) is sufficiently wide for contiguous earth coverage by one satellite every 24 hours. This vertically-polarized Yagi-Uda antenna is pointed in the forward direction with an optimized downward tilt angle to provide the vertical component of radiation for reception by ships’ vertical dipole antennas.

Figure A7-8

Example VDES satellite antenna pattern

#### 6.4.2.5	Determine the maximum Doppler frequency shift (fd) between the satellite and ships in the satellite’s antenna coverage area

The maximum Doppler frequency shift (fd) between the satellite and a ship will occur when the relative velocity between them is a maximum, i.e. when the ship is situated on the satellite’s earth horizon. Note that the coverage for this satellite is only in the forward direction and that the satellite’s antenna pattern will cover ships in the range of 23 degrees (earth’s horizon) downward from the satellite’s velocity vector. Therefore, the maximum Doppler shift is fd (max) = fVDES (v/c)  cos θd = 162  106 (7.594  103)/(3  108)  cos 23 = 3 775 Hz. The satellite transmitter frequency should be reduced by half of fd (max) to provide a range of ±1 887.5 Hz in the coverage area.

Determine the optimum downward tilt angle for the satellite VDES antenna for coverage of ships in the forward direction

From the VDES satellite antenna characteristics in Figure A7-8 above, note that the response is flat to approximately 12. This supports an additional downward tilt of 12 below the horizon of 23 for an optimized total downward tilt angle of 35 degrees below the line that is tangent to the satellite’s orbital path. This provides a sufficient vertical radiation component for ships in the coverage area.

#### 6.4.2.6	Consideration of the angular power flux-density mask limits for transmission by the VHF data exchange system satellite

The PFD angular mask (the maximum allowable PFD in dB(W/(m2  4 kHz)) as a function of the elevation angle from the Earth), is shown in Table A7-2 of § 6.3.1. Note that the PFD mask at 0 (horizon) is −149 dB(W/(m2  4 kHz)), at 45 elevation is −142 dB(W/(m2  4 kHz)), at 60 elevation is −134 dB(W/(m2  4 kHz)) and at 90 (overhead) is −131 dB(W/(m2  4 kHz)).

Note also that since the PFD mask level refers to the vertical component of radiation normal to the Earth’s surface, the polarization loss (≈ 3 dB @ 45 elevation angle) based on the angular relationship between the vertical axis of the satellite antenna and the Earth’s surface should be considered in the determination of the satellite VDES transmitter power.

#### 6.4.2.7	Determine the power flux density levels at elevations of 0, 10, 30, 60 and 90 when the power flux density level at 45 elevation is set to −142 dB(W/(m2  4 kHz))

This section confirms that the elevation angle of 45 is the CPA (closest point of approach) between the PFD mask and the actual radiated VDES space-earth downlink signal.

Calculations of the slant ranges and elevation angles note from the previous calculations that the slant range from the satellite earth horizon is 2 703.6 km. The results of these calculations are shown in Table A7-3 below. Note that the “orbital angle” (the angle of rotation of the satellite’s orbit above the Earth) is used as a reference for geometric calculations (angles and distances) and for time-keeping (elapsed time from the horizon to the point of rotation).

The slant ranges from the satellite to an earth station are determined from the law of cosines (c = SQRT (a2 + b2 + c2 − 2ab cos (C)), where c = slant range, a = Re + h, b = Re and C = the satellite orbital angle. The calculations start with C = 23 (the angle to the horizon) and proceed to C = 0 (the directly above/below position), shown in Table A7-3.

To find the elevation angles, reference angles are determined from the inverse law of cosines (C = cos 1 ((a2 + b2 + c2)/(2ab)) where C = the reference angle between the slant range (line of observation) and the Earth radius (line from the Earth station to the centre of the Earth), a = slant range, b = earth radius and c = Re + h. The elevation angles for the Earth stations are determined by subtracting 90 from the reference angles, also shown in Table A7-3 below.

#### 6.4.2.8	Determine reference levels based on the 45 elevation angle

From Table A7-2, the slant range to the satellite at 45 elevation is 748.3 km and the PFD at 45 elevation is set to the mask limit of −142 dB(W/(m2  4 kHz)). Since the relative angle of the satellite antenna (down-tilted by 35) in that direction is approximately (45 − 35) = 10, the gain of the satellite antenna in that direction, from Fig. A7-19, is 8 dB. These values were used as the set point values (the 0 dB reference levels) to calculate the PFD levels for the other elevation angles.

#### 6.4.2.9	Determine the power flux density level for the elevation angle of 0

The slant range at 0 (horizon) is 2 703.6 km, the satellite relative angle to the horizon is −23, the satellite antenna relative angle with a 35 down-tilt is (35 − 23) = 12 and the gain, from Figure A7-19, is 8 dB. Since the relative range loss is (20 log (748.3/2 703.6)) = −11.2 dB, the PFD at 0 is 11.2 dB below the 45 level (−142 − 11.2) = −153.2 dB(W/(m2  4 kHz)) which is (−149 − (−153.2)) = 4.2 dB below the 0 mask limit.

#### 6.4.2.10	Determine the power flux density level for the elevation angle of 10

The slant range at 10 elevation is 1 818.4 km, the satellite relative angle to the horizon is −23, the satellite antenna relative angle with a 35 down-tilt is (35 − 23− 10) = 2 the gain, from Fig. A7-19, is 8 dB (the same as the reference), the relative range loss is 20 log (748.3/1 818.4) = −7.7 dB and thus the PFD at 10 is (−142 − 7.7) = −149.7 dB(W/(m2  4 kHz)) which is 2.3 dB below the 10 mask limit of −147.4 dB(W/(m2  4 kHz)).

#### 6.4.2.11	Determine the power flux density level for the elevation angle of 30

The slant range at 30 elevation is 993.5 km, the satellite relative angle to the horizon is −23, the satellite antenna relative angle with a 35 down-tilt is (35 − 30) = 5 the gain, from Fig. A7-19, is 8 dB (the same as the reference), the relative range loss is 20 log (748.3/993.5) = −2.5 dB and thus the PFD at 30 is (−142 – 2.5) = −144.5 dB(W/(m2  4 kHz)) which is 0.3 dB below the 10 mask limit of −144.2 dB(W/( m2  4 kHz)).

#### 6.4.2.12	Determine the power flux density level for the elevation angle of 60

The slant range at 60 elevation is 632.7 km, the satellite relative angle to the horizon is −23, the satellite antenna relative angle with a 35 down-tilt is (35 − 60) = −18 the gain, from Fig. A7-19, is 7.5 dB (0.5 dB below the reference), the relative range is 20 log (748.3/632.7) = +1.5 dB (1.5 dB above the reference) and thus the PFD at 60 is (−142 − 0.5 + 1.5) = −141.0 dB(W/(m2  4 kHz)) which is 7.0 dB below the 60 mask limit of −134.0 dB(W/(m2  4 kHz)).

#### 6.4.2.13	Determine the power flux density level for the elevation angle of 90

The slant range at 90 (overhead) is the satellite altitude of 550 km, the gain of the satellite antenna in that direction, from Fig. A7-19, with a down-tilt of 35 degrees is the gain at (35 − 90) = −55 degrees is 2 dB (6 dB below the reference), the relative range factor is 20 log (748.3/550) = +2.7 dB (2.7 dB above the reference) and thus the PFD at 90 is (−142 − 6 + 2.7) = −145.3 dB(W/(m2  4 kHz)) which is 14.3 dB below the 90 mask limit of −131 dB(W/(m2  4 Hz)).

The PFD values for elevation angles from 0 to 90 are shown in Table A7-3 below.

TABLE A7-3

Power flux density for various elevation angles

| Orbital angle (degrees) | Elapsed time from horizon (seconds) | Slant range (km) | Reference angle (degrees) | Elevation angle (degrees) | PFD (actual/mask/margin, in dB(W/(m2 4 kHz))) |
| --- | --- | --- | --- | --- | --- |
| 23 | 0 | 2 703.6 | 90 | 0 | −153.2/−149/4.2 |
| 22 | 15.9 | 2 592.7 | 90.5 | 0.5 | −152.8/−148.9/3.9 |
| 21 | 31.8 | 2 481.6 | 91.0 | 1.0 | −152.4/−148.8/3.6 |
| 20 | 47.7 | 2 370.5 | 93.2 | 3.2 | −152/−148.5/3.5 |
| 19 | 63.6 | 2 259.6 | 94.4 | 4.4 | −151.6/−148.3/3.3 |
| 18 | 79.5 | 2 148.8 | 95.6 | 5.6 | −151.2/−148.1/3.1 |
| 17 | 95.4 | 2 038.3 | 97.0 | 7.0 | −150.7/−147.9/2.8 |
| 16 | 111.3 | 1 928.1 | 98.4 | 8.4 | −150.2/−147.7/2.5 |
| 15 | 127.2 | 1 818.4 | 100.0 | 10.0 | −149.7/−147.4/2.3 |
| 14 | 143.1 | 1 709.2 | 101.6 | 11.6 | −149.2/−147.1/2.1 |
| 13 | 159.0 | 1 600.6 | 103.5 | 13.5 | −148.6/−146.8/1.8 |
| 12 | 175.0 | 1 493.0 | 105.5 | 15.5 | −148/−146.5/1.5 |
| 11 | 190.9 | 1 386.5 | 107.8 | 17.8 | −147.4/−146.1/1.3 |
| 10 | 206.8 | 1 281.4 | 110.3 | 20.3 | −146.7/−145.8/0.9 |
| 9 | 222.7 | 1 178.1 | 113.2 | 23.2 | −145.9/−145.3/0.6 |

TABLE A7-3 (end)

| Orbital angle (degrees) | Elapsed time from horizon (seconds) | Slant range (km) | Reference angle (degrees) | Elevation angle (degrees) | PFD (actual/mask/margin, in dB(W/(m2 4 kHz))) |
| --- | --- | --- | --- | --- | --- |
| 8 | 238.6 | 1 077.3 | 116.6 | 26.6 | −145.2/−144.7/0.5 |
| 7.145 | 252.2 | 993.5 | 120.0 | 30.0 | −144.5/−144.2/0.3 |
| 7 | 254.5 | 979.6 | 120.6 | 30.6 | −144.3/−144.1/0.2 |
| 6 | 270.4 | 886.3 | 125.3 | 35.3 | −143.5/143.35/0.15 |
| 5 | 286.3 | 798.7 | 131.0 | 41.0 | −142.5/−142.4/0.1 |
| 4.38 | 296.1 | 748.3 | 135.0 | 45.0 | −142/−142/0 (reference) |
| 4 | 302.2 | 719.2 | 137.8 | 47.8 | −141.7/−140.5/1.2 |
| 3 | 318.1 | 650.6 | 146.2 | 56.2 | −141.5/−136.1/5.4 |
| 2.7 | 322.9 | 632.7 | 150.0 | 60.0 | −141/−134/7 |
| 2 | 334.0 | 596.8 | 156.1 | 66.1 | −141.8/−133.4/8.4 |
| 1 | 349.9 | 562.1 | 167.6 | 77.6 | −143.1/−132.2/10.9 |
| 0 | 365.8 | 550.0 | 180 | 90 | −145.3/−131/14.3 |

Notes to Table A7-3:

1.	When the PFD level is set to the mask limit of −142 dB (W/(m2  4 kHz)) at 45 elevation angle, the PFD levels at all other elevation angles are below the mask.

2.	The maximum PFD level is −141 dB (W/(m2  4 kHz)) at 60 elevation angle, which is 7 dB below the mask limit level of −134 dB(W/(m2  4 kHz)).

#### 6.4.2.14	Consider the shipborne VHF data exchange system antenna and receiver characteristics

The shipborne antenna and receiver characteristics are considered, along with the satellite radiated PFD levels, to determine the performance of the example VDES satellite downlink.

#### 6.4.2.15	Specify the shipborne VHF data exchange system antenna characteristics

The available shipborne antenna options are comprised of stacked vertical dipole elements of various lengths and gain values, were previously shown in Fig. A7-3 in § 5.3. This analysis considers the 0 dBd antenna because it has the best performance for the elevation angles required for satellite detection.

#### 6.4.2.16	Determine the shipborne VHF data exchange system receiver characteristics

The shipborne VDES receiver characteristics and the coordination levels for the terrestrial service are considered, and the set of metrics in Table A7-4 below are used to determine a reference value of C/N (carrier-to-noise ratio) for the example shipborne VDES receiver.

TABLE A7-4

Metrics for considering ITU-R coordination levels and calculating C/N in a VDES receiver

Power received (referred to the Rx antenna) by a shipboard VHF receiver (reference 25 kHz channel):

Power received (linear formula): Pr = GE2c2/480π2f2, where

G = gain of a half-wavelength (λ/2) dipole antenna = 1.64 E = field strength = 4 10−6 V/m (4 µV/m = +12 dBµ) c = speed of light in free space = 3 108 m/s f = VDES downlink frequency = 161.9 106 (161.9 MHz) λ = 1.852 m (at 161.9 MHz) Pr = 19.02 10−15 W = −137.2 dBW = −107.2 dBm

The logarithmic formula can also be used to calculate Pr (dBm): Pr (dBm) = 42.8 − 20 log F + 20 log E + G, where

G = antenna gain in dBi = 2.1 dBi (2.1 dB over isotropic) F = frequency in MHz = 161.9 Pr (dBm) = 42.8 – 44.1 – 108 + 2.1 = −107.2 dBm (−137.2 dBW) PFD = dB(E) −153.72 = 12−153.72 = −141.72 dB(W/(m2 4 kHz)) from a vertically-polarized source Ae = effective area for a dipole antenna = 0.13λ2 = 0.446 m2 Pr (25 kHz channel) = PFD + 10 log Ae + 10 log (25/4) = −141.7 − 3.5 +8 = −137.2 dBW = −107.2 dBm Power received by a shipboard VDES receiver (reference 150 kHz channel): Noise floor in a 150 kHz bandwidth: kTB = 10 log ((1.38 10−23) (290) (150 103))= −152.2 dBW Rx carrier power (reference) in a 150 kHz bandwidth: C = 10 log ((19.02 10−15) (150/25))= −129.4 dBW Applying adjustments for cable loss (2dB) and Rx noise figure (4 dB), the C/N calculation follows: C/N (150 kHz bandwidth): C/Nref = (−129.4 − 2) − (−152.2 + 4) = 16.8 dB (Rx 0 dBd antenna, 0 elevation) NOTE: These calculations serve to confirm the applicability of the metrics and reference levels.

#### 6.4.2.17	Determine the values of carrier to noise vs. elevation angle for the shipborne VHF data exchange system receiver

Based on the C/N reference level (C/Nref) from Table A7-4, determine the C/N for the PFD values and elevation angles in Table A7-3, taking into account the shipborne antenna angular gain values for the 0 dBd antenna in Fig. A7-3. For this antenna, Ga = 2.1 dBi at 0 elevation angle.

C/N = C/Nref − (−142 − PFD − (2.1 − Ga)), where Ga = shipborne antenna gain at the elevation angle.

–	At 0 elevation, C/N = 16.8 − (−142 − (−153.2) − (2.1 − 2.1)) = 5.6 dB

–	At 10 elevation, C/N = 16.8 − (−142 − (−149.7) − (2.1 − 1.9)) = 8.9 dB

–	At 30 elevation, C/N = 16.8 − (−142 − (−144.5) − (2.1 − (−0.3)) = 11.9 dB

–	At 45 elevation, C/N = 16.8 − (−142 − (−142) − (2.1 − (−3.5)) = 11.2 d.

–	At 60 elevation, C/N = 16.8 − (−142 − (−141) − (2.1 − (−7.6)) = 8.1 dB

–	At 90 elevation, C/N = 16.8 − (−142 − (−145.3) − (2.1 − (−11.6)) = −0.2 dB.

The C/N values for elevation angles from 0 to 90 are shown in Table A7-5 below.

TABLE A7-5

Carrier to noise and power flux density for various elevation angles

| Orbital angle (degrees) | Elapsed time from horizon (seconds) | Slant range (km) | Elevation angle (degrees) | PFD (actual/mask/margin, in dB(W/(m2 4 kHz))) | C/N ship receiver (dB) |
| --- | --- | --- | --- | --- | --- |
| 23 | 0 | 2 703.6 | 0 | −153.2/−149/4.2 | 5.6 |
| 22 | 15.9 | 2 592.7 | 0.5 | −152.8/−148.9/3.9 | 6 |
| 21 | 31.8 | 2 481.6 | 1.0 | −152.4/−148.8/3.6 | 6.4 |
| 20 | 47.7 | 2 370.5 | 3.2 | −152/−148.5/3.5 | 6.8 |
| 19 | 63.6 | 2 259.6 | 4.4 | −151.6/−148.3/3.3 | 7.2 |
| 18 | 79.5 | 2 148.8 | 5.6 | −151.2/−148.1/3.1 | 7.6 |
| 17 | 95.4 | 2 038.3 | 7.0 | −150.7/−147.9/2.8 | 8 |
| 16 | 111.3 | 1 928.1 | 8.4 | −150.2/−147.7/2.5 | 8.5 |
| 15 | 127.2 | 1 818.4 | 10.0 | −149.7/−147.4/2.3 | 8.9 |
| 14 | 143.1 | 1 709.2 | 11.6 | −149.2/−147.1/2.1 | 9.4 |
| 13 | 159.0 | 1 600.6 | 13.5 | −148.6/−146.8/1.8 | 9.7 |
| 12 | 175.0 | 1 493.0 | 15.5 | −148/−146.5/1.5 | 10.2 |
| 11 | 190.9 | 1 386.5 | 17.8 | −147.4/−146.1/1.3 | 10.8 |
| 10 | 206.8 | 1 281.4 | 20.3 | −146.7/−145.8/0.9 | 10.9 |
| 9 | 222.7 | 1 178.1 | 23.2 | −145.9/−145.3/0.6 | 11.5 |
| 8 | 238.6 | 1 077.3 | 26.6 | −145.2/−144.7/0.5 | 11.8 |
| 7.145 | 252.2 | 993.5 | 30.0 | −144.5/−144.2/0.3 | 11.9 |
| 7 | 254.5 | 979.6 | 30.6 | −144.3/−144.1/0.2 | 11.9 |
| 6 | 270.4 | 886.3 | 35.3 | −143.5/143.35/0.15 | 11.9 |
| 5 | 286.3 | 798.7 | 41.0 | −142.5/−142.4/0.1 | 11.7 |
| 4.38 | 296.1 | 748.3 | 45.0 | −142/−142/0 (reference) | 11.2 |
| 4 | 302.2 | 719.2 | 47.8 | −141.7/−140.5/1.2 | 11.0 |
| 3 | 318.1 | 650.6 | 56.2 | −141.5/−136.1/5.4 | 8.6 |
| 2.7 | 322.9 | 632.7 | 60.0 | −141/−134/7 | 8.1 |
| 2 | 334.0 | 596.8 | 66.1 | −141.8/−133.4/8.4 | 4.4 |
| 1 | 349.9 | 562.1 | 77.6 | −143.1/−132.2/10.9 | 2.4 |
| 0 | 365.8 | 550.0 | 90 | −145.3/−131/14.3 | −0.2 |

#### 6.4.2.18	Determine the data rate for elevation angles 00 to 600 using the digital video broadcast by satellite standards

The digital video broadcast standards, by satellite (DVB-S), are designed to provide the maximum utilization of the available bandwidth in a low-to-moderate C/N ratio. The spectral efficiencies for DVB-S2X and DVB-S2 are shown in Fig. A7-9 below.

DVB-S2X is based on the well-established DVB-S2 specification. It uses the proven and powerful LDPC FEC scheme in combination with BCH FEC as outer code and introduces the following additional elements:

–	Smaller roll-off options of 5% and 10% (plus 20%, 25% and 35% in DVB-S2)

–	A finer gradation and extension of number of modulation and coding modes

–	New constellation options for linear and non-linear channels

–	Additional scrambling options for critical co-channel interference situations

–	Channel bonding of up to 3 channels

–	Very Low SNR operation support down to −10 dB SNR; and

–	Super-frame option.

Figure A7-9

Performance of DVB-S2X and DVB-S2

#### 6.4.2.19	Performance conclusion

From Fig. A7-9 above, it is concluded that the DVB-S2X standard transmission applied to the VDES satellite downlink provides spectral efficiency of 1.6 bps/Hz and a date-rate of 240 kbits/s in a 150 kHz bandwidth for C/N ≥ 5 dB, which, from Table A7-5, includes elevation angles from 0 to 60.

# 7	Propagation range predictions for VHF data exchange system terrestrial links

## 7.1	Introduction

This is an informative annex. The excellent propagation characteristics of AIS are well established and appreciated. It is expected that the ASM will have similar performance to AIS. The propagation range predictions for the 100 kHz VDE ship-to-shore and shore-to-ship links follow below.

### 7.1.1	Ship-to-shore application

#### 7.1.1.1	Basis for the coverage assessment

This coverage assessment is based on Recommendation ITU-R P.1546-5 (assuming no ducting), taking into account the antenna height and the seawater propagation path:

Height of antenna (Base Station):	75 m (see graph for various heights)

Transmitter power for ship:		12.5 W

Tx ship antenna gain:			2 dBi (0 dBd)

Rx shore antenna gain:		8 dBi (6 dBd)

Pr:							−103 dBm (VDE shore station sensitivity)

#### 7.1.1.2	Purpose for use of the Recommendation ITU-R P.1546-5 propagation curve

Recommendation ITU-R P.1546-5 prescribes the use of the propagation curves (§3 from Annex 5 and Fig. 4 from Annex 2 of ITU-R P.1546-5), see below Figs. A7-10 and A7-11 of this Annex, assuming no ducting and a smooth earth/sea surface. This analysis may be used as a reference point for field test measurements that usually include some ducting, depending on weather, atmospheric conditions, and other factors.

#### 7.1.1.3	Determination of transmitting/base antenna height, h1

Recommendation ITU-R P.1546-5 specifies (§ 3 of Annex 5) the transmitting/base antenna height, h1, to be used in calculation depending on the type and length of the path. For sea paths h1 is the height of the antenna above mean sea level; for land paths h1 is the height above average terrain.

NOTE: The reference antenna height for the ship’s stations h2 is 10 m.

#### 7.1.1.4	Determination of the minimum field strength (sensitivity threshold) at the VHF data exchange base receiving site

For ship-to-shore:

Power received (linear formula): Pr = GrEr2c2/480π2f 2

Rearranged: Er = √ (480π2f 2Pr/Grc2), where

Er = field strength in V/m

Gr = gain of receiving antenna = 6.3 = 8 dBi

c = speed of light in free space = 3  108 m/s

f = VDE ship-to-shore frequency = 1.57  108 (157 MHz)

Pr = 5  10−14 watts = −133 dBW = −103 dBm

Thus,

Er = 3.21  10−6 = 3.21 µV/m = +10.1 dB µV/m

The logarithmic formula can also be used to calculate Pr (dBm):

Pr (dBm) = 42.8 − 20 log F + 20 log E + G, where

G = antenna gain in dBi = 8 dBi

F = frequency in MHz = 157

Pr (dBm) = 42.8 − 43.9 − 109.9 + 8 = −103 dBm (−133 dBW)

#### 7.1.1.5	Determine the range to the +10.1 dBµ (−103 dBm) coverage limit for a seawater propagation path

Calculate the effective radiated power:

Ps = Pt + G

Pt = 10 log 12.5 − 30 = −19 dBkW (19 dB below 1 kW)

G = 2 dBi = +0 dBd (0 dB over a dipole)

Thus Ps = −19 +0 = −19 dBkW e.r.p.

Fe = F – Ps (vertical scale reference for the propagation graph in Fig. A7-10)

F = +10.1 dBµ

Ps = −19 dBkW

Thus Fe = 10.1 − (−19) = +29.1 dB

#### 7.1.1.6	Determine the seaward ship-to-shore coverage range from Fig. A7-10

The +10.1 dBµ (−103 dBm) range is 85 km, which is 46 NM (use h1 = 75 m).

#### 7.1.1.7	Determine the received signal strength indication values for various other ranges

The reference point received signal strength indication (RSSI) = −103 dBm at a range of 85 km (46 NM) is determined above. For other ranges, the RSSI value is determined from the propagation curve (Fig. A7-10) for the assumed antenna height of 75 m. RSSI values in 10 dB increments above the sensitivity threshold are shown in Table A7-6 below.

TABLE A7-6

VHF data exchange base station received signal strength indication 
value vs. distance ship-to-shore

| −103 dBm | 85 km (46 NM) |
| --- | --- |
| −93 dBm | 60 km |
| −83 dBm | 40 km |
| −73 dBm | 25 km |
| −63 dBm | 15 km |
| −53 dBm | 8 km |
| −43 dBm | 4.5 km |

Figure A7-10

+29.1 dB reference

−103 dBm

+29.1 dB reference

−103 dBm

85 km (46 NM) ship-to-shore Coverage range for −103 dBm shore antenna height = 75 m

85 km (46 NM) ship-to-shore Coverage range for −103 dBm shore antenna height = 75 m

### 7.1.2	Shore-to-ship application

#### 7.1.2.1	Basis for the coverage assessment

Referring to § 2 above, we consider the reverse direction, shore-to-ship, signal levels at the ship receiving site, the shore transmitter power of 50 W and the shore-to-ship frequency of 162 MHz:

Height of antenna (VDES Base Station):	75 m (see graph for various heights)

Transmitter power of VDES on shore:	50 W (at base of shore antenna)

Tx shore antenna gain:			8 dBi (6 dBd)

Rx ships antenna gain:			2 dBi (0 dBd)

Pr:								−98 dBm (VDE ship station sensitivity)

#### 7.1.2.2	Determination of minimum field strength (sensitivity threshold) at the VHF data exchange ship receiving site

For shore-to-ship:

Power received (linear formula): Pr = GrEr2c2/480π2f 2

Rearranged: Er = √(480π2f 2Pr/Grc2), where

Er = field strength in V/m

Gr = gain of receiving antenna = 1.62 = 2.1 dBi

c = speed of light in free space = 3  108 m/s

f = VDE shore-to-ship frequency = 1.62  108 (162 MHz)

Pr = 1.58  10−13 W = −128 dBW = −98 dBm

Thus, Er = 11.61  10−6 V/m = 11.61 µV/m = +21.3 dB µV/m

The logarithmic formula can also be used to calculate Pr (dBm)

Pr (dBm) = 42.8 − 20 log F + 20 log E + G, where

G = antenna gain in dBi = 2.1 dBi

F = frequency in MHz = 162

Pr (dBm) = 42.8 − 44.1 − 98.7 + 2.1 = −98 dBm (−128 dBW).

#### 7.1.2.3	Determine the range to the +21.3 dBµ (−98 dBm) coverage limit for a seawater propagation path

Calculate the effective radiated power:

Ps = Pt + G

Pt = 10 log 50 − 30 = −13 dBkW (13 dB below 1 kW)

G = 8 dBi = +6 dBd (6 dB over a dipole)

Thus Ps = −13 + 6 = −7 dBkW e.r.p.

Fe = F – Ps (vertical scale reference for the propagation graph in Fig. A7-11)

F = +21.3 dBµ

Ps = −7 dBkW

Thus Fe = 21.3 − (−7) = +28.3 dB.

NOTE: that since this value of Fe is within 1 dB of the value calculated in § 7.1.1.5 because the reduced sensitivity of the ship station is compensated by the higher power and antenna gain of the shore base station.

#### 7.1.2.4	Determine the seaward shore-to-ship coverage range from Fig. A7-11

The +28.3 dBµ (−98 dBm) range is 85 km, which is 46 NM (use h1 = 75 m). This is the same as the ship-to-shore coverage range, an ideal balanced two-way coverage, which confirms the proposed choices of antennas and transmitter power values for the shipborne and shore VDES stations.

#### 7.1.2.5	Determine the received signal strength indication values for various other ranges

The reference point: RSSI = −98 dBm at a range of 85 km (46 NM) is determined in § 7.1.1.6 above. For other ranges, the RSSI value is determined from the propagation curve (Fig. A7-11) for the assumed antenna height of 75 m. RSSI values in 10 dB steps above and below the −98 dBm threshold sensitivity for the shipborne VDE receiver are shown in Table A7-7 below.

TABLE A7-7

VHF data exchange ship station received signal strength indication 
value vs. distance shore-to-ship

| −118 dBm | 170 km |
| --- | --- |
| −108 dBm | 130 km |
| −98 dBm | 85 km (46 NM) |
| −88 dBm | 60 km |
| −78 dBm | 40 km |

Figure A7-11

85 km (46 NM) shore-to-ship Coverage range for −98 dBm shore antenna height = 75 m

85 km (46 NM) shore-to-ship Coverage range for −98 dBm shore antenna height = 75 m

+28.3 dB reference −107 dBm

+28.3 dB reference −107 dBm

# 8	Example of VHF data exchange satellite downlink implementation and analysis

## 8.1	Introduction

This is an informative annex providing an example of implementing the VDE-SAT downlink component and presenting performance results.

## 8.2	VHF data exchange satellite orbital characteristics

The spacecraft flies in a circular orbit of 600 km and 68° inclination compliant with orbital debris regulations and safe de-orbiting of the spacecraft after its lifetime. The satellite counts with attitude control mechanisms to guarantee a stable antenna pointing in the nadir direction (i.e. satellite-to-Earth).

Under these assumptions Fig. A7-12 shows the elevation (left axis) of the spacecraft as a function of time as seen by a ground terminal during an overhead pass. The right axis of the same figure depicts the signal delay.

Figure A7-12

Satellite elevation and delay for the selected orbit as a function of time

Elevation in degrees

Elevation in degrees

Delay in seconds

Delay in seconds

Time

Time

Figure A7-12 shows that the satellite is just over 4 minutes above 30° elevation, thus 9 minutes under 30° elevation from acquisition-of-signal (AOS) to loss-of-signal (LOS) for a pass duration of about 13 minutes. The roundtrip delay varies from 19 ms at AOS down to 4 ms at zenith (i.e. 90° elevation). During that pass the Doppler shift varies from −3.73 kHz to +3.73 kHz and the Doppler rate reaches 47 Hz/s at Zenith.

Figure A7-13

Pass elevation scheme for selected orbit over 24 hours

Figure A7-13 illustrates the satellite elevation as a function of time, as seen by a ground terminal at a fixed location in a 24 hour period. As shown the contact periods are short and low. Depending on the latitude, the duration and the number of contact periods will vary (distance is provided in km).

Figure A7-14

Satellite field of view

Figure A7-14 presents the satellite field of view. A wide geographical area is covered by the satellite field of view at any given point of the orbit. For this area, the average instantaneous ship count is 22 000 respectively as shown in Fig. A7-15. The ship count is based on combined received terrestrial and satellite data for AIS class A.

Figure A7-15

Field of view case for ship instantaneous number

Figure A7-15 is indicative of the AIS received by terrestrial stations is displayed in blue while AIS received by satellite is displayed in red.

### 8.2.1	VHF data exchange satellite downlink characteristics

The power flux density mask to be respected is (as also presented in Table A4-1 of Annex 4).

PFDθ° (dBW/(m2*4 kHz))=-149+0.16*θ°                         0°≤θ<45°;-142+0.53*θ°-45°      45°≤θ<60°;-134+0.1*θ°-60°         60°≤θ≤90°.

where θ is the angle of arrival of the incident wave above the horizontal plane, in degrees.

Which is tabulated as follows:

TABLE A7-8

Tabulation of pfd mask

|  | dBW | Δ = −36.0 dB | Δ = +14.0 dB |
| --- | --- | --- | --- |
| Theta | Flux/4 kHz | Flux/1 Hz | Flux 100 kHz |
| 0 | −149.00 | −185.00 | −135.00 |
| 5 | −148.20 | −184.20 | −134.20 |
| 10 | −147.40 | −183.40 | −133.40 |
| 15 | −146.60 | −182.60 | −132.60 |
| 20 | −145.80 | −181.80 | −131.80 |
| 25 | −145.00 | −181.00 | −131.00 |
| 30 | −144.20 | −180.20 | −130.20 |
| 35 | −143.40 | −179.40 | −129.40 |
| 40 | −142.60 | −178.60 | −128.60 |
| 45 | −142.00 | −178.00 | −128.00 |

TABLE A7-8 (end)

|  | dBW | Δ = −36.0 dB | Δ = +14.0 dB |
| --- | --- | --- | --- |
| 50 | −139.35 | −175.35 | −125.35 |
| 55 | −136.70 | −172.70 | −122.70 |
| 60 | −134.00 | −170.00 | −120.00 |
| 65 | −133.50 | −169.50 | −119.50 |
| 70 | −133.00 | −169.00 | −119.00 |
| 75 | −132.50 | −168.50 | −118.50 |
| 80 | −132.00 | −168.00 | −118.00 |
| 85 | −131.50 | −167.50 | −117.50 |
| 90 | −131.00 | −167.00 | −117.00 |

Figure A7-16

Elevation in degrees

Elevation in degrees

Power flux density mask

Power in dBm

Power in dBm

Figure A7-16 depicts the PFD mask in dBm as a function of elevation in a reference bandwidth of 4 kHz and in 100 kHz bandwidth.

The corresponding e.i.r.p. mask seen by the satellite corresponds to a transformed version of the PFD mask dictated by the Earth-satellite geometry.

Figure A7-17 shows the e.i.r.p. mask which is symmetric around the nadir direction (90° angle in the figure).

Figure A7-17

Satellite equivalent isotropic radiated power mask

Elevation in degrees

Elevation in degrees

Power in dB

Power in dB

Assuming a circularly polarized downlink signal from the satellite meeting the e.i.r.p. mask in Fig. A7-17, then the PFD in 100 kHz seen in an overhead pass by a ground terminal is shown as a violet curve in Fig. A7-18. In this figure the signal power of a nearby ship (shown in yellow) is also presented as a benchmark reference. The green line represents the realization of an antenna on the satellite compliant with the e.i.r.p. mask.

Figure A7-18

Receiver carrier input for a 0 dB gain antenna.
Isoflux and compensated satellite transmitter antenna + nearby ship

### 8.2.2	VHF data exchange satellite receiver characteristics

On the receiver side, the ship’s system temperature is considered to be between 630 K (noise figure of 3 dB and 2 dB of cable loss) and 1 500 K. Variations can occur, but it is not expected that the system temperature falls below roughly 900 K in a standard installation. The system temperature accounts for the noise source integrated in the antenna patterns. Some on board ‘industrial’ noise is yet to be added, but will be ignored for the remainder of the document.

### 8.2.3	“Ideal” receiving antenna

For the sake of completeness, the receiver antenna mask that would allow the received signal to be at constant power level at the receiver input is calculated and shown as a function of elevation angle in Fig. A7-19.

Figure A7-19

“Ideal” receiver antenna mask, zenith is 90°

Antenna gain dBi

Elevation in degrees

Gain in dBi

Antenna gain in dBi

Antenna gain dBi

Elevation in degrees

Gain in dBi

Antenna gain in dBi

Figure A7-20

Received carrier power for a receiver with an “ideal” antenna

Elevation in degrees

Elevation in degrees

Carrier at Rx input (with antenna gain)

Antenna gain dBi

Carrier at Rx input (with antenna gain)

Antenna gain dBi

Gain in dBi

Gain in dBi

Signal level in dBm

Signal level in dBm

Figure A7-20 shows the received signal power in dBm at the input of a receiver with the “ideal” receiving antenna as a function of elevation. The link analysis is computed using professional commercial software tools for satellite communications that account for the signal propagation impairments.

The software tool however, does not account for possible loss of power strength at very low elevation (< 1°). The power loss could be as high as 6 dB due to reflecting surface of seawater, mainly in circular or horizontal polarizations. It is worth noting that the signal power at the receiver input is around −101 dBm, and this is 3 dB lower than the Recommendation ITU-R M.1842 recommended sensitivity for 16-QAM for ship stations.

FIgure A7-21

Eb/N0 compensated patterns for “ideal” antenna

Eb/N0 in dB

Eb/N0 in dB

Figure A7-21 shows the corresponding Eb/N0 observed for the 100 kHz carrier in an overhead pass for the “ideal” antenna.

### 8.2.4	Realistic receiving antenna

Four different antennas are considered:

−	The 0 dBd point in the Recommendation ITU-R F.1336 antenna pattern and vertical polarization (antenna 1)

−	A 1.25 λ vertical antenna (commercially available antenna, computed pattern when mounted on the top of the bridge a 200 m long tanker), vertical polarization (antenna 2)

−	A satellite dedicated Turnstile antenna, with right hand circular polarization (RHCP) (antenna 3)

−	A hemispherical 0 dBi gain antenna, vertical polarization (antenna 4).

Using professional software tools for satellite communications, simulations have been carried out to determine the carrier power level at the receiver input and to determine the Eb/N0 in the following cases:

−	Overhead pass

−	Side pass

−	Very low pass.

Results corresponding to each scenario are reported in the following sections.

#### 8.2.4.1	Overhead satellite pass

Figure A7-22

Overhead satellite pass, carrier level at receiver input

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

FIgure A7-23

Overhead satellite pass, Eb/N0 at demodulator input

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

#### 8.2.4.2	Side satellite pass

Consider a 16° elevation pass, the signal power and corresponding signal quality measured in Eb/N0 are presented in the following figures. Due to the variation of the signal strength at the receiver over time (due to the change of elevation and distance), the signal may fall below the detection threshold.

The use of highly robust waveform (as a combination of modulation, coding and frame structure) can potentially improve the performance at the expense of reduced throughput.

Figure A7-24

Carrier level at receiver input, side satellite pass

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Figure A7-25

Eb/N0 at demodulator input, side satellite pass

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

#### 8.2.4.3	Very low side satellite pass

Results for a very low side pass (below 5° elevation) are presented in Figures below.

Figure A7-26

Carrier input at receiver input, very low side satellite pass

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Figure A7-27

Eb/N0 at demodulator input, very low elevation side satellite pass.

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

### 8.2.5	Waveform choice

As shown in previous sections, for a realistic antenna the signal to noise ratio at the input of the receiver can vary considerably as a function of elevation angle. The choice of the waveform modulation, coding and frame structure has a significant impact on the link throughput and its availability.

The decision on continuous versus intermittent transmission of the signal will impact the acquisition, tracking and the overall performance (bit rate, probability of error, etc.) of the VDE satellite broadcasting. At the system level, a slot-based transmission (time division) may increase the complexity of the satellite-terrestrial system interactions and reduce the overall efficiency. However, the coexistence of VDE broadcasting and terrestrial shore-to-ship or ship-to-ship may also impact the detection performance of the terrestrial signal.

The choice of modulation scheme has an impact on the efficiency of the power amplifier on board of the satellite. The use of (quasi-) constant envelope reduces the peak to average power ratio and allows the transmitter to operate at a more power efficient mode with less signal distortion.

In order to facilitate synchronization and signal detection at the receiver, the use of known symbols (as pilot or preamble) is essential as part of the air interface definition.

The use of data sequence randomization (scrambling) facilitates the synchronization and mitigates spectral abnormality.

A system capability to allow more than one coding rate (and modulation scheme) may provide more flexibility in the system dimensioning and service availability.

There are a number of existing open standards with air interface specifications, such as Digital Video Broadcasting via satellite DVB-S2x , DVB-SH and DVB-RCS2, that offer mature technical solutions as a starting point for such design trade-offs. The performance characteristics of DVB-RCS2 waveforms are reported in Table A7-8. Figure A7-28 presents the spectral efficiency (information bits/symbol) as a function of Es/N0 for these waveforms.

Note: DVB-RCS2 reference: ETSI TS 101 545-1 V1.2.1 (2014-04) available at: http://www.etsi.org/deliver/etsi_ts/101500_101599/10154501/01.02.01_60/ts_10154501v010201p.pdf.

TABLE A7-9

Waveform efficiency in additive white Gaussian noise channel

| Frame Size (symbols) | Guard (symbols) | Payload (bits) | Efficiency (bits/symbol) | Es/N0 @ PER=10-5 |
| --- | --- | --- | --- | --- |
| 266 | 4 | 408 | 1.51 | 7.3 |
| 266 | 4 | 440 | 1.63 | 8.71 |
| 266 | 4 | 496 | 1.84 | 10.04 |
| 266 | 4 | 552 | 2.04 | 11.59 |
| 266 | 4 | 672 | 2.49 | 11.73 |
| 266 | 4 | 744 | 2.76 | 13.18 |
| 536 | 4 | 304 | 0.56 | 0.22 |
| 536 | 4 | 472 | 0.87 | 2.34 |
| 536 | 4 | 680 | 1.26 | 4.29 |
| 536 | 4 | 768 | 1.42 | 5.36 |
| 536 | 4 | 864 | 1.60 | 6.68 |
| 536 | 4 | 920 | 1.70 | 8.08 |
| 536 | 4 | 1 040 | 1.93 | 9.31 |
| 536 | 4 | 1 152 | 2.13 | 10.85 |
| 536 | 4 | 1 400 | 2.59 | 11.17 |
| 536 | 4 | 1 552 | 2.87 | 12.56 |
| 1 616 | 4 | 984 | 0.61 | −0.51 |
| 1 616 | 4 | 1 504 | 0.93 | 1.71 |
| 1 616 | 4 | 2 112 | 1.30 | 3.69 |
| 1 616 | 4 | 2 384 | 1.47 | 4.73 |
| 1 616 | 4 | 2 664 | 1.64 | 5.94 |
| 1 616 | 4 | 2 840 | 1.75 | 7.49 |
| 1 616 | 4 | 3 200 | 1.98 | 8.77 |
| 1 616 | 4 | 3 552 | 2.19 | 10.23 |
| 1 616 | 4 | 4 312 | 2.66 | 10.72 |
| 1 616 | 4 | 4 792 | 2.96 | 12.04 |
| 3 236 | 4 | 984 | 0.30 | −3.52 |
| 3 236 | 4 | 1 504 | 0.46 | −1.3 |

Figure A7-28

Spectral efficiency of DVB-RCS2 waveform

Es/N0 (dB)

Es/N0 (dB)

The use of some frequencies in the band 156-164 MHz, contained in this Recommendation, do not comply with the RR currently in force. This Recommendation therefore should not be seen as prejudging the decisions of WRC-15. ITU-R Study Group 5 is invited to review this Recommendation taking into account the decisions made by WRC-15.

Note by the BR Secretariat – The figures in this Recommendation are available in English only. The other languages will be prepared in due course.

Amount of aggregate user data which can be carried by the MSC scheme on a single channel assuming single slot packets including coding and other message encapsultion according to the MSC definition.

The baseband shall employ a root raised cosine filter.

The following example is compliant with the requirement:

Sample the RF signal strength at a rate > 1 kHz, average the samples over a sliding 20 ms period and over a 4 s interval determine the minimum period value. Maintain a history of 15 such intervals. The minimum of all 15 intervals is the background level. Add a fixed 10 dB offset to give the CS detection threshold.