Input paper:   ARM20-6.2.6

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

X  ARM	□  ENG		□  PAP					X  Input

□  ENAV	□  VTS							□  Information

Agenda item 			6.2

Author(s) / Submitter(s)			Namkyun Im, Do Hyun Oh/Mokpo National Maritime University

# Proposal for the Integration of a Marine Traffic Model into Risk Management Framework

# Summary

Effective risk management in maritime navigation requires a structured approach to analyzing vessel movement patterns and assessing potential hazards. IALA's G1018 defines various risk management tools, including PAWSA, IWRAP, and SIRA, as part of its Risk Management Toolbox. Each assessment tool has its own strengths and limitations, making it suitable for different purposes. Among them, PAWSA and SIRA are qualitative assessment tools that incorporate expert experience and opinions. Quantitative risk assessment tools include IWRAP and navigational simulation. In particular, the IWRAP method is an excellent tool for quantitatively analyzing the probability of collisions, groundings, and contact accidents based on historical AIS data and waterway characteristics.

However, the existing quantitative analysis tools within the IALA toolbox are primarily based on general historical vessel traffic volume to determine traffic density and may not fully reflect traffic flow, including traffic direction and speed. Additionally, there has been a demand for risk assessment tools that utilize real-time traffic information.

This study proposes the integration of a traffic flow model as a risk assessment tool to support more precise and data-driven maritime safety evaluations. This model has the advantage of representing traffic flow in a specific area by simultaneously visualizing speed, direction, and density using an arrow-based approach. Furthermore, to address the growing demand for real-time traffic-based risk assessment tools, this study introduces the Marine Traffic Hazard Index model as a future research direction. By utilizing real-time AIS data, this approach enables the visualization of real-time navigational risk levels in specific areas through both 2D and 3D representations.

- Traffic Density Model: This model divides the waterway into detailed grid cells and records vessel passages through each cell, generating a heatmap for a quantitative assessment of traffic density and visual identification of congestion zones.

- Traffic Flow Model: This model builds on density data, analyzing vessel movement direction and speed within each grid. By dividing each grid into 10-degree sectors, the model determines the most frequently used direction and calculates the average speed and heading of vessels in that direction. The results are visualized using arrows, where arrow length represents speed, direction indicates movement trends, and thickness represents density, providing a more intuitive understanding of maritime traffic flow.

- Marine Traffic Hazard Index model: This study proposes a method for real-time risk assessment using AIS data. By utilizing AIS data, the model conducts real-time vessel collision risk evaluations and identifies potential hazardous ship domains. This approach can serve as a real-time risk assessment tool for specific areas.

By applying these models, risk assessments can go beyond simple traffic volume metrics to incorporate traffic flow information that indicates speed and direction, along with real-time traffic data. This approach enhances risk assessment by integrating traffic flow insights, such as direction and speed, with a real-time risk visualization model, ultimately supporting safer route planning, congestion monitoring, and collision avoidance strategies.

As maritime traffic continues to evolve, particularly with the growing implementation of Maritime Autonomous Surface Ships (MASS), this model provides a valuable tool for structured risk management, improving overall navigational safety and decision-making in high-traffic areas.

# Background

A research project supported by the Korean government is currently being conducted in the Republic of Korea to develop hydrographic information standards for smart ships. The objective of this project is to develop and refine new hydrographic information models aligned with the adoption of Maritime Autonomous Surface Ships (MASS). As part of this initiative, a maritime traffic model analyzing traffic density and flow based on AIS data is under development.

Currently, risk assessment methodologies primarily rely on color-coded maps based on vessel traffic density, with high-density areas being used as a proxy for navigational risk. However, this approach does not sufficiently account for historical accident occurrences or specific navigational challenges in certain areas. A more structured and data-based methodology is required to enhance the accuracy of risk assessments.

This study develops a traffic density and flow model using AIS data to analyze vessel movement patterns more systematically. Additionally, the model incorporates historical maritime safety information (MSI) to emphasize accident-prone areas. By integrating MSI data, the model extends beyond traffic density calculations to highlight areas where past collisions and groundings have occurred, providing a more refined understanding of navigational risk.

Currently, the model is based on historical AIS data; however, it has the potential to incorporate real-time traffic information, allowing for dynamic updates to risk assessments. Real-time data integration would enhance situational awareness, enabling vessels to make proactive navigational decisions to avoid high-risk areas.

The proposed model offers a more structured and quantitative approach to risk assessment, supporting collision and grounding risk evaluations, route optimization, and hazard avoidance strategies. As MASS operations expand, integrating real-time traffic and historical accident data into risk models will become increasingly critical for ensuring safe and efficient navigation.

# Discussion

This study develops a traffic density and flow model using AIS data to systematically analyze vessel movement patterns in specific areas and identify regions with a high likelihood of accidents. The research consists of a traffic density model and a traffic flow model, each designed to support a more data-driven and quantitative approach to maritime traffic risk assessment. This study develops a traffic density and flow model using AIS data to systematically analyze vessel movement patterns in specific areas and identify regions with a high likelihood of traffic congestion, incorporating both directional and speed information. The research consists of a traffic density model and a traffic flow model, each designed to support a more data-driven and quantitative approach to maritime traffic risk assessment, with an additional explanation of real-time risk assessment tools.

## TRAFFIC DENSITY MODEL

The traffic density model quantitatively analyzes vessel traffic volume in specific maritime areas and visualizes the results. The model is developed by dividing the waterway into a structured grid system and accumulating the number of vessels passing through each grid cell to calculate density. AIS data is used to analyze vessel movement over a given period, allowing for a comparative evaluation of high-density and low-density regions and supporting risk assessment.

The grid system for analysis is set at three-second intervals, corresponding to approximately 91.3 meters in latitude and longitude. The time interval of AIS data has been optimized to improve the accuracy of density analysis, enabling the identification of high-density and low-density areas based on vessel movement data. The density data is visualized using a 15-level color classification in a heatmap format, allowing for a more intuitive analysis of maritime traffic patterns. Figure 1 shows the color classification on map with three-second grid.

Figure 1. 15-Level Density Color Classification

Additionally, the traffic density model could incorporate Maritime Safety Information (MSI), which is also on-going work within Korea’s research project, to emphasize regions with high accident occurrence. Instead of simply classifying high-traffic areas as risk zones, the model highlights areas with historical accident records, providing a more realistic risk assessment.

To validate the proposed model, it was applied to Mokpo Port in the Republic of Korea for practical analysis. AIS data from 2022 to 2024 was used to analyze traffic density and flow patterns in the study area. Figure 2 illustrates the designated study area, while data collected from this region was used to identify high-density zones and major traffic routes. This analysis quantitatively assesses high-density areas, contributing to the evaluation of potential risk zones and supporting future maritime traffic management strategies. Figures 3 provide a visualization of traffic density in a specific curved section of the waterway, where vessel traffic is highly concentrated.

| Traffic density model Nij N=Number of ships passing, ij=Grid index | (1) |
| --- | --- |

Figure 2. Study Area (Mokpo)

Figure 3. Visualization of Traffic Density

## TRAFFIC FLOW MODEL

The traffic flow model analyzes vessel movement direction and speed based on traffic density data and represents the results visually. AIS data is utilized to systematically assess directional movement trends and speed variations in specific areas, ensuring a more detailed evaluation of maritime traffic flow.

This model employs an arrow-based visualization technique to illustrate maritime traffic flow. Arrows are used to represent movement, where the direction of the arrow indicates vessel movement direction, its length represents vessel speed, and its thickness corresponds to vessel density. This method enables a clear, intuitive representation of vessel movement, particularly in congested areas and major shipping routes.

The traffic flow model builds upon the existing traffic density model and averages vessel speed and heading within each grid. To achieve this, each grid is divided into 10-degree segments across 360 degrees, and the most frequently traveled direction within each grid is selected. The average speed and heading for vessels traveling in this direction are then calculated, resulting in a representative movement vector for each grid. Figure 4 shows an example of a selected segment within 360 degrees divided grid.

Figure 4. Traffic flow speed and heading selection

First, density is derived from the traffic density model, using accumulated vessel passage data over a given period. Speed and direction are calculated by segmenting each grid into 10-degree intervals and analyzing the number of vessels passing through each segment. The most frequently traveled direction is selected as the representative direction, and the average speed and heading of vessels in that direction are determined.

The processed data is then visualized using the arrow-based technique:

- Arrow direction: Representative vessel movement direction

- Arrow length: Representative vessel speed

- Arrow thickness: Vessel density in the respective direction

| Traffic flow model i=0nViN i=0nHiM V=Ship's speed, H=Ship's heading (direction), N=Total number of speed data, M=Total number of heading data | (2) |
| --- | --- |

Figure 5 illustrates the arrows used for visualizing traffic flow, and Figure 6 is the visualization of traffic flow. Figure 7 and 8 shows depicts the overall and detailed traffic density and traffic flow within the study area.

Figure 5. Traffic flow visualization using arrows

Figure 6. Visualization of Traffic Flow

Figure 7. Visualization of Traffic Density and Traffic Flow (Study Area)

Figure 8. Detailed Visualization of Traffic Density and Flow

This method reflects the most dominant movement patterns in each region, providing an intuitive representation of major traffic routes. Additionally, integrating this model with MSI allows for the identification of potential risks by analyzing movement patterns in accident-prone areas. By incorporating traffic flow into risk assessment, this model contributes to improved maritime safety measures and supports Maritime Autonomous Surface Ships (MASS) in planning optimal routes and evasive maneuvers.

## INTRODUCTION OF MARINE TRAFFIC HAZARD INDEX MODEL

To apply the traffic density and flow model in real time, AIS data is used to dynamically assess and transmit maritime traffic risk levels. AIS data is continuously updated, and by processing it in real time, it becomes possible to immediately identify high-risk areas based on the current maritime traffic situation.

Real-time data is collected through AIS transmission systems, which periodically provide information on vessel position, speed, and heading. The collected data is processed by analyzing vessel data above to calculate risk levels and dynamically assess the overall risk status within a given area.

The results of the risk assessment are visualized on a map, where high-risk areas are represented with color variations, and the visualization is continuously updated to reflect changes in traffic conditions. This allows for a more precise risk analysis that not only accounts for high-density areas but also incorporates actual vessel movement patterns and potential collision risks. The process of conducting real-time risk assessments for specific areas using real-time AIS data is detailed in references [6]-[7]. Figure 9 shows an example of visualization of marine traffic risk on grid map. Figure 10 and 11 illustrates marine traffic risk map using real-time AIS data in 2D and 3D.

Figure 9. Example of Visualization of Marine Traffic Risk

Figure 10. Visualization of Marine Traffic Risk in 2D

Figure 11. Visualization of Marine Traffic Risk in 3D

Processing real-time data enables quicker decision-making for route adjustments and collision avoidance. The risk assessment results are provided as map-based visual information, allowing users to intuitively recognize risk levels in specific maritime areas. By integrating real-time data with the traffic density and flow model, it becomes a highly effective tool for maritime traffic risk assessment and management.

## APPLICATION AND UTILIZATION OF THE MODEL

The current traffic density and flow model is developed using historical AIS data, with additional studies exploring the feasibility of real-time data integration. Incorporating real-time traffic flow analysis would enable dynamic risk assessment, allowing for immediate detection of traffic pattern changes and emerging risks.

Future research will explore the inclusion of various maritime environmental factors in the model. While the current study primarily focuses on AIS data for traffic analysis, future work may incorporate weather data, currents, and tides to establish a more comprehensive maritime risk assessment framework.

Furthermore, the potential integration of this model with MASS navigation planning is under consideration. The traffic density and flow model could support MASS in optimizing navigation routes and avoiding high-risk areas. Additionally, the feasibility of linking the model with collision avoidance algorithms to mitigate risks in high-traffic zones is being examined.

This study aims to enhance existing traffic analysis methods with a structured, quantitative approach to maritime risk assessment. Future work will focus on expanding the model by integrating real-time data and additional maritime environmental factors to further improve its effectiveness.

# References

IALA, Guideline G1018, Risk Management, Edition 4.0, June 2022.

IALA Guideline 1123, The Use of IALA Waterway Risk Assessment Programme (IWRAP MK II), June 2022.

IALA Guideline 1124, The Use of Ports and Waterways Safety Assessment (PAWSA) MK II Tool, June 2022.

IALA Guideline 1138, The Use of the Simplified IALA Risk Assessment Method (SIRA), December 2022.

IALA, Vessel Traffic Services (VTS) Manual, Edition 8.4, June 2024.

Marine Traffic Hazard Index for Real-time Risk Assessment in Port Areas, Ocean Engineering, November 2021.

Potential risk ship domain as a danger criterion for real-time ship collision risk evaluation, Ocean Engineering, 2019

# PROPOSAL

This study proposes a revision of THE IALA TOOLBOX in G1018 to incorporate a traffic flow model as a new traffic assessment model. If necessary, a follow-up review should be conducted to develop new guidelines detailing the specifics of this traffic flow model.

Furthermore, a review on the development of a real-time risk assessment toolbox is encouraged. Additionally, the approach proposed in this study should be considered for inclusion as one of the real-time risk assessment tools in the future IALA Risk Assessment Toolbox

# Action requested of the Committee

The Committee is invited to consider the proposed revision of the guidelines for G1018, review the development of a real-time risk assessment toolbox, and take appropriate action as necessary.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain