Input paper:   ARM20-9.1.1

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

X  ARM	□  ENG		□  PAP					□  Input

□  ENAV	□  VTS							X  Information

Agenda item 			9.1

Technical Domain / Task Number 2 	1.4.11

Author(s) / Submitter(s)			Eng. Raúl S. Escalante / Eng. Mariano L. Marpegan

# Availability determination in Argentine waterway

# Summary

The evaluation of the efficiency of an Aid to Navigation (AtoN) or a System of Aids to Navigation as a set of AtoNs aims to establish whether the Level of Service (LOS) offered is compatible with that required for a particular Waterway; otherwise, the corresponding technical and operational improvements should be developed.

One of the most important navigable waterways in the world, the Paraguay—Paraná Waterway, with 3.400 kilometres of waterway in restricted waters, develops 1.680 kilometres along Argentina, which has responsibility for 1.150 Aids to Navigation, becoming the Main Navigable Waterway in the region.

The evaluation criteria of the Level of Service (LOS) of the Aids to Navigation System implemented for the control of the buoyage system and the application of penalties to the Contractor in charge of the maintenance of the system does not follow IALA´s recommendations but are a weighted percentage of buoyage failures with a maximum of 3%, which consequently represents a percentage of normality or minimum instantly Level of Service of 97%, which represents approximately a 99,5 % according to IALA´s methodology for the calculation of the Level of Service. This methodology for the calculation of availability has been used since 1995.

## Purpose of the document

This document is an information paper.

## Related documents

In 2004, IALA developed the documents Guideline G1035 and Recommendation O130, with the criteria and methodology of Availability calculation, which allows the comparison of the Level of Service of similar Aids to Navigation Systems provided by different Organizations.

Main Navigable Waterway in Argentina

Black = Main Navigable Waterway in Argentina

Red + Black = Parana – Paraguay Waterway

# CALCULATION METHODOLOGY

Figure 1 represents the Parana – Paraguay Waterway, and in black it                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                          is indicated the Argentine´s Main Navigable Waterway, which is divided into eight (8) subsections, with the following Aids to Navigation distribution:

Main Navigable Waterway

| Subsections | Length (Km) | AtoN Quantity |
| --- | --- | --- |
| I.0 | 30 | 105 |
| I.1 – Rio de la Plata | 236 | 131 |
| I.2 | 481 | 377 |
| I.3 | 181 | 147 |
| II.0 | 77 | 41 |
| II.1 | 51 | 30 |
| II.2 | 269 | 137 |
| II.3 | 355 | 180 |
| Total | 1.680 | 1.148 |

The calculation of the weighted percentage of failures is carried out for the set of AtoNs included in each subsection into which the waterway is divided as follows:

The following equation expresses the Weighted Percentage:

100×ηCi×fi3×nCi
Where

Ci = Score according to the function of the AtoN.

fi = Score by type of fault, defect or breakdown.

Ƞ = Number of AtoNs in the subsection with faults.

n = Total number of AtoNs in the subsection.

Each AtoN is not categorized as per the recommendation of IALA, and its importance depends on its installed equipment.

Each AtoN, depending on its function, is classified with different scores. For example, isolated danger and cardinal AtoNs have a more significant influence on the calculation than other AtoNs, such as lateral or special AtoNs.

Score according to the AtoN functions

| Function | Ci |
| --- | --- |
| Isolated Danger | 0,75 |
| Leading lights | 0,75 |
| Change of heading | 0,65 |
| Channel entry | 0,65 |
| Cardinal | 0,65 |
| Modified lateral | 0,65 |
| Special | 0,50 |
| Safe Water | 0,40 |
| Lateral | 0,25 |

The weighted percentage will vary depending on the AtoN function and the fault generated. An increase is applied when the fault occurs in consecutive and/or adjacent AtoN.

Example: if an AtoN present a fault (light off) it will have an influence on the calculation according to the function, but if in addition the following AtoN, or the one next to it, is also light off, both AtoNs will have a greater weight in the calculation, and therefore the weighted percentage will be higher.

Depending on the fault recorded in the AtoN, it will have a different score.

Scores according to the AtoN fault

| Fault | Fi |
| --- | --- |
| AtoN missing | 3 |
| Light off | 2 |
| AtoN out of position in progressive | 1 |
| AtoN out of position in towards bank | 2,5 |
| Weak light | 1,5 |
| Light altered of lateral AtoN | 1 |
| Light altered of safe water AtoN | 1,5 |
| Light altered of isolated danger or cardinal AtoN | 3 |
| Light altered of modified lateral AtoN | 2 |
| Discoloured AtoN | 1,5 |
| Radar wave reflector missing | 2 |
| Tide lantern missing | 1,5 |
| Tide lantern out of service | 1,5 |
| Racon missing | 2 |
| Racon out of service | 2 |
| Top mark missing | 1,5 |
| Legend missing, progressive, arrow, etc | 1 |
| Heeled AtoN | 2 |
| Semi-submerged AtoN | 2 |
| Tower missing | 2,5 |
| Lantern missing | 2 |
| Aton hidden by vegetation | 2 |

Clearly, applying these criteria to characterize failures exceeds what IALA recommends regarding the Level of Service (LOS).

However, the variable that most differentiates this criterion from that established by IALA is that the AtoN and, therefore, the Service are evaluated based on a photograph taken of the system's state and do not contemplate a calculation period equivalent to that of IALA (mobile period of three (3) years of evaluation).

## Local example

In the Main Navigable Waterway, the Rio de la Plata subsection shown in Figure 2 represents a system of Aids to Navigation where AtoNs with different functions and types coexist; therefore, it has been selected as a good example.

Rio de la Plata subsection

The following equipment is installed in different signals:

Medium range lantern – 5 NM

Real, Virtual and Synthetic AIS-AtoN

Radar wave reflector

Racon

Tide lantern

Tide gauge stations

Meteorological stations

- With the distribution of the 131 AtoNs presented in Table 4

Distribution of AtoNs at subsection Rio de la Plata

| Function | Quantity |
| --- | --- |
| Safe water | 14 |
| Cardinal | 3 |
| Special | 8 |
| Change of heading | 18 |
| Lateral | 83 |
| Isolated danger | 5 |
|  | 131 |

### Comparison of calculation methodologies – LOS Argentina vs LOS IALA

For subsection I.1—Rio de la Plata, the following failures were simulated starting on 02/17/2025.

Failure scenario

| Function | Km at the waterway | Failure | Function | Score by type of fault | Observation |
| --- | --- | --- | --- | --- | --- |
| Lateral | 66 | AtoN missing | 0,25 | 3 |  |
| Safe water | 81 | Racon out of service | 0,40 | 2 |  |
| Lateral | 143,9 | AtoN missing | 0,25 | 3 |  |
| Isolated danger | 180 | AtoN missing | 0,75 | 3 |  |
| Lateral | 197,4 | Light off | 0,65 | 2 | The score is increased from 0.25 to 0.65 because they are adjacent signals |
| Lateral | 197,4 | Light off | 0,65 | 2 |  |

In the simulated scenario after the Buoyage inspection by the Client, it was detected that 6 AtoNs of Rio de la Plata subsection (5% of AtoN quantity) presented some faults. Currently, the Weighted Percentage of this subsection is 3,44 % = 96,36 % of Availability (a situation of penalty) lower than 97 %. Considering the available 3-year history of the subsection and making the calculation following the IALA recommendation, the percentage regarding the Level of Service (LOS) is 97,9 % of Availability. It results in the criterion of taking an instantaneous value being much more demanding than considering a three-year average.

The client detects system malfunctions through weekly inspections, and the Weighted Percentage is calculated daily. Penalties are very heavy, and to avoid them, the 97 % level must be restored in less than 5 days. This value is dynamic because while some faults are corrected, new ones appear.

### Other examples

- Some examples are provided to show some disadvantages of this way of LOS calculation.

Example 1: A cardinal or isolated danger AtoN with its light altered is considered to have a failure level of 100%, a value equivalent to that same AtoN being missing (because de Ci = 3 in both cases). It seems highly demanding to assess it as if it were missing, a situation in which it would effectively not provide any service, either during the day or at night.

Example 2: An AtoN with a missing radar wave reflector is valued the same as a Light off AtoN. In this case, the possibility arises that the Aton is perfectly positioned, identifiable during the day and at night because its light works correctly; however, its reduced radar response compared to the requirement would indicate a service level equal to that of a Light off Aton.

Example 3: An AtoN that is Light off means 66% of failure, and therefore 34% Availability, which represents a much higher requirement than the criteria of thinking that during the day the AtoN performs its function correctly (50% of the time). At night, it does not perform its function (the other 50%).

For a system composed of 30 AtoNs, if it is verified that one AtoN is missing 1 day in a three-years period with the instantaneous calculation, it represents a level of service below 97%, while considering a continuous evaluation period as established by IALA it corresponds to a service level of:

Availability=1-1365*3*100 = 99,90 %.

# References

G 1035

R 0130

# Action requested of the Committee

The Committee is requested to: (Body text)

- To take notice of the Input Paper

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain