|  | 80/MT7/2426 ###### TC80/MT7 2024-12-18 |
| --- | --- |

##### INTERNATIONAL ELECTROTECHNICAL COMMISSION

TECHNICAL COMMITTEE 80: Maritime navigation and radiocommunication equipment and systems / MT7 ECDIS

Subject: Planned IALA Service specification for route plan exchange between shore and ships

IEC TC80MT7 is responsible for maintenance of IEC 61174 ECDIS standard.  Currently MT7 is working on the new edition 5 of the IEC 61174 for compliance with IMO MSC.530(106)/Rev.1 Performance Standards for ECDIS.

The rev.1 of the IMO Performance Standards has added bi-directional route plan exchange between ship and shore-based service providers based on IEC 63173-1 Route plan format (S-421) and IEC 63173-2 Secure communication (SECOM). The new 5th edition of the IEC 61174 will include how at the ECDIS side to check compliance with IMO MSC.530(106)/Rev.1. The meeting 10th – 12th Dec 2024 of MT7 recognized that sustainable testing of the compliance requires knowledge on how the shore side intend to implement their part of the bi-directional route plan exchange. Further the meeting heard that IALA is working on a related service specification for the shore side.

1. Assumptions of IEC TC80/MT7

MT7 has recognized that the IEC 63173-1 standard for the format of the route plan:

- Is based on IHO S-100 baseline standard

- That S-100 Part 17 specifies as delivery alternative “dataset” or “exchange set”.

- The “dataset” is just content of the route plan together with metadata related to specification of “dataset” in S-100

- The “exchange set” can contain multiple files together with metadata. For S-421 route plans an “exchange set” would consist of: “S-421 dataset”, “CATALOG.XML” (which includes digital signature for S-421 dataset) and “CATALOG.SIGN” (which includes digital signature for CATALOG.XML)

- The IEC 63173-1 leaves open if a practical application is using “dataset” or “exchange set” as delivery method

MT7 has recognized that the IEC 63173-2 standard for secure communication (SECOM):

- Is payload agnostic, i.e. any kind of payload is possible

- The digital signature for the authentication could be part of the payload, i.e. the payload for S-421 Route Plans is an “exchange set”

- The digital signature for the authentication could be external to the payload, i.e. the payload for S-421 Route Plans is a “dataset”. In this case SECOM API can facilitate provision of the digital signature calculated over the payload

MT7 is aware that the new installations onboard ships are subject to IACS cyber security rules IACS UR E26 and IACS UR E27:

- IEC TC80 has worked together with IACS to create IEC 61162-460 type approval standard for compliance with IACS UR E26 and E27

- Based on IEC 61162-460 an onboard ECDIS cannot directly connect to Internet. The onboard ECDIS and its local area network (LAN) is protected by a gateway (460-Gateway). Within the protected LAN it is possible exchange information without authentication, but all exchange of information with things outside of this protected LAN is subject to authentication

- For example, based on above an ECDIS can exchange S-421 route plans with another ECDIS in the same protected LAN using “dataset”

- Based on above an ECDIS receiving an S-421 route plan from shore is required to authenticate the received route and reject use of it if the authentication fails.  MT7 has assumed that the use of “exchange set” would be the appropriate method to facilitate the authentication requirement

2. Timeline of IEC TC80/MT7

IMO MSC.530(106)/Rev.1 requires compliance for all new ECDIS installation from 1st Jan 2029. To facilitate this IEC needs to publish 5th Ed of IEC 61174, European Union (EU) needs time to complete the legislative part of type approval and manufacturers need time to complete type approval.

In IEC, the work of a workgroup (WG) or maintenance team (MT) is completed when the WG or MT has completed the CDV (Committee Draft for Vote). Further steps (FDIS and IS) are part of formal acceptance process by the National Committees (NC) participating the Technical Committee (TC). WG or MT use Committee Draft (CD) to request review comments from NCs for further drafting towards CDV.

The plan of MT7 is to create a CD based on draft IALA guidelines for the shore side before completing the CDV within the year 2025.

## 3. Action requested from IALA

IALA is requested to consider the information provided by this liaison note and to inform IEC TC80/MT7

- Which method, “dataset”, “exchange set” or something else, IALA consider specifying in their planned specification for shore-based route exchange services

- When preliminary information of the specified method would be available for IEC TC80/MT7

- When IALA forecast official publishing of their planned specification for shore-based route exchange services