AIS SYMBOLOGY - A DISCUSSION PAPER

Introduction

- The PLA, as a member of IHMA, has been actively involved in the integration of AIS within a Vessel Traffic Display System and has recently been considering the presentation of AIS data as part of the specification for their VTS Display System upgrade.  The following thoughts are forwarded for consideration at VTS 21.  They do not attempt to form a comprehensive view for all AIS symbology but highlight particular instances where the greater functionality available within a VTS display system can be utilised to provide greater choice.

Baseline Symbology (Non-AIS Targets)

- In preparation for the introduction of AIS symbology, we have made adjustments to the presentation of non-AIS targets.  The key points are as follows (examples are shown at Annex A):

- We are offering operators two chart options both based on S57 data with a limited number of data layers.  One based on the traditional VTS background (Land in yellow, Sea in various shades of blue – the greater the depth of water, the darker the hue) and the other the basic onboard S52 full daylight chart.  Other night/dusk S52 presentation formats are incorporated within the system but will not be used.  Night/dusk formats were assessed to be unnecessary as night vision is not a factor.

- New unidentified tracks that have not been associated in the VTS MIS are circular with a COG vector but no name.

- Fishing vessels are circular with an alpha symbol in the middle.

- Identified vessels change to a square.

- Letter codes are used in the centre of the square to identify its pilotage status (pilot onboard, PEC or sub-pilotage)

- The colour of the square is white on blue (VTS) or black on white (S52) for standard targets.  This can be varied to indicate hazardous or tracking status.  We have selected orange for a “specified vessel” and red for a lost target.

- The vessel name field can be separately colour coded as required.  We use white or blue (VTS) and black on white (S52) for standard targets and use a variation of light or dark green to indicate final destination when a ship is due to pass through our area into an adjacent port.

AIS Symbology

- Having established the non-VTS symbology, this is adapted for AIS tracks as follows (examples are shown at Annex B):

- As for the onboard display, isosceles triangles and ship shapes are used (except that the ship shape is given a pointed bow).  The presentation of triangle or ship shape is automatic and dependent on the zoom (scale) in use.  The transition scale is set to a default value but can be varied by the operator if required.

- The standard non-AIS classification symbols (para 2 above) are shown at the ship’s position and this symbol is within the triangle and set 1/3rd from the base of the triangle when the scale dictates that a triangle should be shown.  The triangle is aligned to heading, if available, or COG if heading is not available.

- The standard non-AIS classification symbols (para 2 above) are shown at the ship’s position and this symbol is located within the ship shape and at the transponder or reference position when the ship shape is displayed.  Ship shape can only be displayed if heading information is available.  COG will emanate from the ship position symbol but heading will emanate from the bow.

- Vessel identity codes and colour schemes within the AIS ship shape/triangle accord to the non-AIS conventions as in para 2 above.  The triangle or ship shape surrounding it remains white (or black for S52 presentation).

- Heading vectors are shown dotted and COG is represented by a solid line (See Review Issues paragraph below).  The length of both vectors is proportional to speed and shows the actual distance travelled in a set time.  The time is selectable by the operator but will apply to all targets.  As AIS does not transmit speed through the water, thus, the heading vector conforms to the SOG data received.

- New tracks that have not been associated in the VTS MIS will be displayed with a circular symbol within an AIS triangle together with a COG and heading vector as selected.  Because it is an AIS target, the name will appear alongside the track.

- Rate of turn is represented by a line at right angles to the heading vector in one of three standard length steps dependent on rate of turn.  It can only be displayed if heading is available and will not be applied to the COG vector.  The length of the rate of turn indicator will be of one, two or three equal lengths (irrespective of the speed vector length) cross hatched at each division to indicate slow, medium or fast rates of turn.  The actual values at which each step changes between slow, medium and fast has been selected on best estimate but, again, we will be able to change the threshold values in the light of experience.  We believe that this will apply sufficient granularity for normal use as the track can always be interrogated to establish the precise value for ROT if required.

- There are a number of AIS display configuration options (see Annex C) that we believe need to be readily available to the operator to both inhibit and adjust the default setting via a simple display toggle button.  These include:

  - enable/disable heading marker (for all or individual tracks)

  - selection of vector length (minutes)

  - selection of historic track and adjust length (minutes)

  - enable/disable ROT (for all or individual tracks)

  - enable/disable AIS (for all or individual tracks)

Note: The ability, to disable AIS tracking within the port VTDS on an individual ship basis is seen as a vital capability to cater for the concern being widely expressed that many ships are transmitting incorrect AIS data.

- Receipt of a Safety Related Message will be indicated by the track blinking until operator incurs the message.

Review Issues

  - Draft IMO proposals for on-board symbology identifies that “the reported position should be located at centre and half the height of the triangle”.  Where a triangle rather than a ship shape is displayed, we would propose that the reported position should be 1/3rd from the base of the triangle to maximise the space available within the triangle for other identifiers.  Course vector would emanate from this position; heading emanates from the apex of the triangle.

  - It will be noted that the selection of dotted or solid lines for heading and COG respectively are the reverse of those stipulated for on-board display.  This was an intentional decision based on the fact that VTS is more concerned with courses and tracks made good rather than actual heading.  There are equally compelling reasons why the on-board display protocol should be used, not least of which is to ensure familiarity by those who may be employed both at sea and within a VTS.

  - It is noted that the draft IMO proposals only show the course prediction facility feature for own ship but not for target symbols.  It is assumed that this is intentional as we would agree that this is potentially highly misleading for VTS use and would not support the inclusion of this capability in a VTDS.

  - We would welcome views on these issues.

Annexes:

# A.	Standard non- AIS Track Classification Symbols

B.	AIS Symbols – Classification, Track, Rate of Turn

C.      	AIS Configuration Options

# Standard Non- AIS Track Classification Symbols

| Type |  | Description | Symbol |  |
| --- | --- | --- | --- | --- |
|  |  |  | VTS | S52 |
| New track |  |  |  |  |
| Fishing vessel |  |  |  |  |
| Radar Tracked | Non-specified P = Piloted X = eXempted N = Non-piloted | London vessel Name label horizontal an d in capitals – can be rotated around symbol | P NAME | P NAME |
|  |  | Medway vessel Label is light green on SYTAR and dark green on S52 | P NAME | P NAME |
|  | Specified P = Piloted X = eXempted N = Non-piloted | London vessel | P NAME | P NAME |
|  |  | Medway vessel Label is light green on SYTAR and dark green on S52 | P NAME | P NAME |

AIS Symbols - Classification

| Type |  | Description | Symbol |  |
| --- | --- | --- | --- | --- |
|  |  |  | SYTAR | S52 |
| AIS Tracked (display scale > 2nm) | Non-specified P = Piloted X = eXempted N = Non-piloted | London vessel Symbol oriented by heading or COG if heading not available. Classification and name labels always typed horizontally | P NAME | P NAME |
|  |  | Medway vessel Label is light green on SYTAR and dark green on S52 |  |  |
|  | Specified P = Piloted X = eXempted N = Non-piloted | Symbol oriented by heading or COG if heading not available Classification and name labels always typed horizontally | P NAME | P NAME |
|  |  | Medway vessel Label is light green on SYTAR and dark green on S52 |  |  |

# AIS Symbols - Track

| Type |  | Description | Symbol |  |
| --- | --- | --- | --- | --- |
|  |  |  | SYTAR | S52 |
| Display scale > 2nm | Vectors | Symbol oriented by heading or COG if heading not available Classification label always typed horizontally Dashed line = heading Solid line = track Line length based on SOG; (operator selectable as present) | P | P |
| Display scale < 2nm | Vessel dimensions | Only if heading available Symbol oriented by heading Classification label always typed horizontally Vectors and classification label originate at antenna location | P | P |

Note: AIS Emergency message notification:  Symbol blinks until operator views message.

# AIS Symbols - Rate-of-Turn

| Type | Description | Symbol |  |
| --- | --- | --- | --- |
|  |  | SYTAR | S52 |
| Rate of turn derived by AIS (only if heading available) | Low rate of turn bar Degrees/min > 3 to < 45 | P | P |
|  | Medium rate of turn arc bar Degrees/min > 45 to < 90 | P | P |
|  | High rate of turn arc bar Degrees/min > 90 | P | P |

Note: Although diagrams show triangles for low and high rates of turn and ship shape for medium rate of turn, the principle of rate of turn indication is the same irrespective of ship symbol.

# AIS Symbols - Configuration Options

### Default Setup

- Adjust default AIS symbol zoom threshold  (transition from triangle to vessel shape symbol)

- Adjust rate-of-turn thresholds

### Operator Display Configuration Functions

- Enable/disable heading marker display (all on/off toggle)

- Enable/disable individual heading marker display (on/off toggle)

- Configure heading & track (COG) vector length

- Enable/disable AIS tracking (global all on/off toggle)

- Enable/disable individual AIS tracks (on/off toggle)

- Enable/disable rate-of-turn display (all on/off toggle).  Must have heading marker displayed.

- Enable/disable individual target rate-of-turn display (on/off toggle). Must have heading marker displayed.

NAV 50/WP.6 Annex 2 dated 7 Jul 04