# CHAPTER 2:   LEGAL FRAMEWORK

0201  Introduction

The successful organization and provision of Vessel Traffic Services generates a self-evident need for international agreement as to how shipping from various flag-states can successfully and harmoniously interact and, equally, a need for domestic national law to reflect universally accepted objectives in relation to the ports that such shipping uses.

Thus there is the need to have a clear and unambiguous route from the global concept, characterized by IMO as ‘Safer Ships, Cleaner Seas’, seen as the strategic imperative and internationally agreed as a good thing, to the local byelaw requirements discharged, perhaps by a single VTS Operator in a small local port.  Generally, the mariner wishes to be part of a regime where for regulatory and procedural purpose all ports ‘look-the-same’ and where he feels equally comfortable; their only principal difference is that of geography.

It is the purpose of this chapter to demonstrate the link between internationally agreed conventions and the successful provision of vessel traffic services at local level that have the potential to be part of the delivery of the safety system envisaged.

0202 The United Nations and International Law

The General Assembly of the United Nations declared the period 1990-1999 to be the United Nations Decade of International Law. The main purposes were, inter alia:

- To promote acceptance of and respect for the principles of international law;

- To promote means and methods for the peaceful settlement of disputes between States, including resort to and full respect for the International Court of Justice;

- To encourage the progressive development of international law and its codification.

- To encourage the dissemination of international law.

Several major developments in international law have occurred under the auspices of the United Nations.  These range from the development in the 1970-80s of the Law of the Sea to, more recently, the negotiation and adoption of several key international treaties in such areas as international environmental law, international economic law, the legal regulation of international terrorism, and the creation of new international organizations and entities.

# 0203  United Nations Convention On The Law Of The Sea

The United Nations Convention on the Law of the Sea (UNCLOS) was adopted in 1982. The United Nations Convention on the Law of the Sea lays down a comprehensive regime of law and order in the world's oceans and seas establishing rules governing all uses of the oceans and their resources. It embodies in one instrument traditional rules for the uses of the oceans and introduces new legal concepts and regimes and addresses new concerns. The text of UNCLOS is available at www.un.org/Depts/los/convention.htm

The Division for Ocean Affairs and the Law of the Sea (DOALOS) of the Office of Legal Affairs of the United Nations serves as the secretariat of the Convention on the Law of the Sea and provides information, advice and assistance to States with a view to providing a better understanding of the Convention and the related Agreements, their wider acceptance, uniform and consistent application and effective implementation.  The Division monitors all developments relating to the Convention, the law of the sea and ocean affairs and reports annually to the General Assembly of the United Nations.

Although the International Maritime Organization (IMO) is explicitly mentioned in only one of the articles of UNCLOS (article 2 of Annex VIII), several provisions in the Convention refer to the "competent international organization" to adopt international shipping rules and standards in matters concerning maritime safety, efficiency of navigation and the prevention of marine pollution from vessels and by dumping. In such cases, the expression "competent international organization", when used in the singular in UNCLOS, applies exclusively to IMO, bearing in mind its global mandate a specialized agency of the United Nations.

0204   International Maritime Organization

The IMO was established as an agency of the UN by the Convention on the International Maritime Organization. When IMO came into existence in 1958, several important international conventions had already been developed, including the International Convention for the Safety of Life at Sea (1948), the International Convention for the Prevention of Pollution of the Sea by Oil (1954) and treaties dealing with load lines and the prevention of collisions at sea. IMO was made responsible for ensuring that these conventions were kept up to date and given the task of developing new conventions as and when the need arose.

The creation of IMO coincided with a period of great change in world shipping and the Organization was kept busy from the start developing new conventions and ensuring that existing instruments kept pace with changes in shipping technology.  It is now responsible for more than 40 international conventions and agreements and numerous protocols.

0205   IMO - Mandate

IMO has a global mandate for, and has focused its activities on the adoption and implementation of international rules and standards for the safety of navigation, prevention of the pollution of the marine environment from vessels’, and maritime security.   Also, it intensified its treaty making activity aimed at ensuring that prompt and adequate compensation is paid to victims of maritime accidents. The wide acceptance and uncontested legitimacy of IMO's universal mandate is evidenced by the following facts:

- 163 sovereign States representing all regions of the world are Members of IMO;

- all Members may participate at meetings of IMO bodies in charge of the elaboration and adoption of recommendations containing safety and antipollution rules and standards. These rules and standards are normally adopted by consensus; and

- all States, irrespective of whether they are or are not Members of IMO or the United Nations, are invited to participate at IMO conferences in charge of adopting new IMO conventions.

At present, between 125 and 147 States (depending on the treaty) have become Parties to the main IMO conventions. Since the general degree of acceptance of these shipping conventions is mainly related to their implementation by flag States, it is of paramount importance to note that States Parties to these Conventions in all cases represent more than 90 per cent of the world's merchant fleet.

Adoption of new treaties, and amendments to existing ones, have been guided by adherence to the philosophy according to which rules and standards should be developed in order to prevent accidents at sea, and not in response to them. Accordingly, operational features are constantly under review in order to ensure that shipping activities conform to the highest possible safety and anti-pollution preventative regulations. The IMO attaches the highest priority to the need of ensuring that its numerous rules and standards contained in these treaties are properly implemented. In order to help ensuring this implementation the Organization focuses on the continuous strengthening of regulations to ensure that flag and port States and ship-owners develop their capacities and exert their responsibility to the fullest.

0206  IMO Conventions

The majority of conventions adopted under the auspices of IMO or for which the Organization is otherwise responsible fall into three main categories. The first group is concerned with maritime safety; the second with the prevention of marine pollution; and the third with liability and compensation, especially in relation to damage caused by pollution.  Outside these major groupings are a number of other conventions dealing with facilitation, tonnage measurement, unlawful acts against shipping and salvage.

0207  IMO Conventions relevant to VTS

The principal Conventions which have applicability to the establishment, organization and conduct of VTS operations are set out in the table below.

| Instrument | Entry into Force |
| --- | --- |
| International Convention for the Safety of Life at Sea , 1974 (SOLAS) Protocol 1978 Protocol 1988 | May 1980 May 1981 February 2000 |
| Convention on the International Regulations for Preventing Collisions at Sea, 1972. | July 1977 (Amended 1981, 1987, 1989, 1993 and by resolution A.910(22) of November 2001. |
| International Convention on Seafarers’ Training, Certification and Watchkeeping, 1978 (STCW) Amendment 1995 | April 1984 February 1997 |
| International Convention on Maritime Search and Rescue (SAR), 1979 Revised by MSC.70(69) | June 1985 January 2000 |
| Convention on the International Maritime Satellite Organization, 1975/76 (INMARSAT) | July 1979 (Amended 1985 & 1989) |
| International Convention for the Prevention of Pollution from Ships, 1973 and amended 1978 ( MARPOL 73/78) Annex I/II Annex III Annex IV Annex V Protocol 1997, Annex VI | October 1983 July 1992 |
| International Convention on Oil Pollution Preparedness, Response and Co-operation (OPRC), 1990 HNS Protocol 2000 | May 1995 Not yet in force (2004) |

IMO Conventions that have relevance to VTS

0206   SOLAS

Vessel Traffic Services were not specifically referred to in the International Convention for the Safety of Life at Sea (SOLAS) 1974, but in June 1997 IMO's Maritime Safety Committee adopted a new regulation to Chapter V (Safety of Navigation), which set out when VTS can be implemented. A revised chapter was adopted in December 2000, and entered into force on 1 July 2002. Regulation 12 ,Vessel traffic services, states:

- Vessel traffic services (VTS) contribute to safety of life at sea, safety and efficiency of navigation and protection of the marine environment, adjacent shore areas, work sites and offshore installations from possible adverse effects of maritime traffic.

- Contracting Governments undertake to arrange for the establishment of VTS where, in their opinion, the volume of traffic or the degree of risk justifies such services.

- Contracting Governments planning and implementing VTS shall, wherever possible, follow the guidelines developed by the Organization. The use of VTS may only be made mandatory in sea areas within the territorial seas of a coastal State.

- Contracting Governments shall endeavour to secure the participation in, and compliance with, the provisions of vessel traffic services by ships entitled to fly their flag.

- Nothing in this regulation or the guidelines adopted by the Organization shall prejudice the rights and duties of Governments under international law or the legal regimes of straits used for international navigation and archipelagic sea lanes.

0206   Safety of Navigation & Maritime Security

The General Assembly of the United Nations invited (Resolution 58/240, December 2003), the IMO to strengthen its functions with regard to port State control in relation to safety and pollution standards as well as maritime security regulations. A new, comprehensive security regime for international shipping is set to enter into force in July 2004 following the adoption by the Conference of a series of measures to strengthen maritime security and prevent and suppress acts of terrorism against shipping. The Conference adopted a number of amendments to the 1974 Safety of Life at Sea Convention (SOLAS), the most far-reaching of which enshrines the new International Ship and Port Facility Security Code (ISPS Code).

0207   Places of Refuge

The General Assembly of the United Nations welcomed (Resolution 58/240, December 2003), the work of the International Maritime Organization in developing guidelines on places of

refuge for ships in need of assistance and encourages States to draw up plans and to establish procedures to implement those guidelines for ships in waters under their jurisdiction.

In the aftermath of a number of tanker incidents which had taken place since 1999, the Maritime Safety Committee decided to consider the issue of places of refuge mainly from the navigational safety viewpoint and commissioned the drafting of guidelines:

1. actions a master of a ship should take when in need of a place of refuge (including actions on board and actions required in seeking assistance from other ships in the vicinity, salvage operators, flag State and coastal States).

2. the evaluation of risks, including the methodology involved, associated with the provision of places of refuge and relevant operations in both a general and a case by case basis; and

3. actions expected of coastal States for the identification, designation and provision of such suitable places together with any relevant facilities.

0207  Standards for Training Certification and Watchkeeping (STCW)

IMO has been vested with the authority to vet the training, examination and certification procedures of Contracting Parties to the International Convention on Standards of Training, Certification and Watchkeeping for Seafarers (STCW), 1978. This an important change made in the 1995 amendments to the Convention which entered into force on 1 February 1997. Governments will be required to provide information to IMO's Maritime Safety Committee for it to judge whether the country concerned meets the requirements of the Convention.

Noting that STCW 1995 conference called for the development, to common standards of those established for mariners, of the training and certification of maritime pilots and VTS personnel, it is to be expected that at some future date IMO authority may similarly be extended. However, it should be further noted that partly in response to STCW 1995 and partly in response to demands from its membership, IALA developed a training regime (V-103) for VTS personnel to match the format and requirements of those established for mariners in STCW 1995.   (Audit?)

0208  Marine Pollution - Particularly Sensitive Sea Areas (PSSA)

A Particularly Sensitive Sea Area (PSSA) is an area that needs special protection through action by IMO because of its significance for recognized ecological or socio-economic or scientific reasons and which may be vulnerable to damage by international maritime activities. The MARPOL Convention defines certain sea areas as "special areas" in which, for technical reasons relating to their oceanographical and ecological condition and to their sea traffic, the adoption of special mandatory methods for the prevention of sea pollution is required. Under the Convention, these special areas are provided with a higher level of protection than other areas of the sea. The criteria for the identification of particularly sensitive sea areas and the criteria for the designation of special areas are not mutually exclusive.

In many cases a Particularly Sensitive Sea Area may be identified within a Special Area and vice versa. When an area is approved as a particularly sensitive sea area, specific measures can be used to control the maritime activities in that area, such as routeing measures, strict application of MARPOL discharge and equipment requirements for ships, such as oil tankers; and installation of Vessel Traffic Services (VTS).

The IMO Marine Environment Protection Committee (MEPC) (49th session) adopted a resolution granting Particularly Sensitive Sea Area (PSSA) status to the Paracas National Reserve of Peru.  The other PSSAs already adopted include:

- the Great Barrier Reef, Australia;

- the Sabana-Camagüey Archipelago in Cuba;

- Malpelo Island (Colombia);

- around the Florida Keys (United States); and

- the Wadden Sea Denmark, Germany and the Netherlands.

In-principle endorsement to a proposal from Australia and Papua New Guinea for the extension of the Great Barrier Reef PSSA to cover the Torres Strait Region, will be further considered in October 2004.

The MEPC has also considered a proposal from Belgium, France, Ireland, Portugal, Spain and the United Kingdom to designate certain areas of the Western European waters as a PSSA. The associated protective measure for the PSSA, therefore, would, at this stage, be a proposed 48-hour reporting system for ships carrying certain cargoes entering the PSSA.  The proposal has been referred for further consideration in July 2004.

0208   Regional Undertakings – European Community

The European Parliament has established (Directive 2002/59/EC dated 27 June 2002) a vessel traffic monitoring and information system along the coasts of Member States.

The purpose of the Directive is to establish within the sea areas subject to the jurisdiction of member States of the European Community a vessel traffic monitoring and information system with a view to enhancing the safety and efficiency of maritime traffic, improving the response of authorities to incidents, accidents or potentially dangerous situations at sea, including search and rescue operations, and contributing to a better prevention and detection of pollution by ships. Member States shall monitor and take all necessary and appropriate measures to ensure that the masters, operators or agents of ships, as well as shippers or owners of dangerous or polluting goods carried on board such ships, comply with the requirements set out in the Directive.

The Directive imposes responsibilities on Member States for the arrangements for, and conduct of the system and, generally, on most commercial shipping, regardless of flag state, whose passage passes through the sea areas concerned.

(Note: Member States of the European Community are Austria, Belgium, Cyprus, Czech Republic, Denmark, Estonia, Finland, France, Germany, Greece, Hungary, Italy, Latvia, Lithuania, Luxemburg, Malta, Netherlands, Poland, Portugal, Romania, Slovak Republic, Slovenia, Spain, Sweden and United Kingdom.)

0209   Enforcement

The enforcement of IMO conventions depends upon the Governments of Member Parties. Contracting Governments enforce the provisions of IMO conventions as far as their own ships are concerned and also set the penalties for infringements, where these are applicable.  They may also have certain limited powers in respect of the ships of other Governments.

In some conventions, certificates are required to be carried on board ship to show that they have been inspected and have met the required standards.  These certificates are normally accepted as proof by authorities from other States that the vessel concerned has reached the required standard, but in some cases further action can be taken.

Should an offence occur within the jurisdiction of another State, however, that State can either cause proceedings to be taken in accordance with its own law or give details of the offence to the flag State so that the latter can take appropriate action.  Under the terms of the 1969 Convention Relating to Intervention on the High Seas, Contracting States are empowered to act against ships of other countries which have been involved in an accident or have been damaged on the high seas if there is a grave risk of oil pollution occurring as a result.

The way in which these powers may be used are very carefully defined, and in most conventions the flag State is primarily responsible for enforcing conventions as far as its own ships and their personnel are concerned.

The IMO has no powers to enforce conventions save for its powers in connection with STCW (see 0207)

0210   National Legislation

Contracting Parties to international conventions are sovereign states that undertake, as part of the accession and ratification process to each convention or protocol, to enact appropriate national legislation to give effect to the provisions that have been agreed. Such enactments will, where appropriate, include provisions for enforcement and sanctions for infringements.

Whilst it is for governments to determine how best to enact international agreements within the framework of national legislation, it is evident that some broad similarities emerge in the way that states undertake this responsibility.  Most governments find it necessary in the maritime context to rely on two broad bodies of primary legislation:

- Marine, or Shipping, or Merchant Shipping Laws or Acts

- Harbour, or Port, or Docks Laws or Acts

With regard to the second, which is normally of a national character with uniform applicability to all port undertaking, it may be accompanied by local legislation that has applicability to only the port to which it refers.

0211    Some examples of National Legislation

Some typical examples of national legislation in connection with VTS are given at Annex A to this chapter. The table provides a synopsis of some of the various methods used by states to implement international obligations.