TASK 3 ;	Develop a recommendation on the performance requirements for VTS equipment

Draft recommendation on Performance Standard for VTS Equipment – Radar

GENERAL

The performance requirements placed on the VTS radar service vary a great deal depending on traffic density, levels of VTS, special regional features and the coverage of the VTS area. The purpose of this document is to describe the general performance requirements of the radar service. By meeting these general requirements, it is possible to obtain the basic level required of VTS radar services in vessel traffic control.

Requirements of radar service may have a high impact on acquisition and life-cycle costs of a VTS and therefore the performance recommendations are divided into three different levels, where:

- standard recommendations are applicable to all levels of VTS as identified by IMO – information service, navigational assistance service and traffic organizational service – for areas with medium to high traffic density and/or without specific navigational hazards.

- Relaxed recommendations are applicable to VTS, information service and/or navigational assistance service for areas with low to medium traffic density and without navigational hazards.

- Extended recommendations are applicable to VTS areas with high traffic density and/or specific navigational hazards.

The three levels of recommendations may be used as applicable within a VTS, e.g. part of a VTS area may call for standard recommendations and another part may call for relaxed recommendations.

CHARACTERISTICS OF THE RADAR TARGET

The radar system’s detection and measurement capacity depends on the characteristics of the radar and the target. These include the target’s average reflective area and its oscillation in relation to the time and the measuring frequency used. The distribution of the target’s reflective parts, the incidence angle and turning speed have a major impact on the power reflected back from the target.

The detection capacity required of the system is determined according to the service level, traffic density and potential navigational hazards. Table 1. describes the typical reflection features and service level requirements of VTS targets.

| TARGET |  | Recommendation level |  |  | Corresponding (point) target model |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Relaxed | Standard | Extended | radar cross section |  | Height |
|  |  |  |  |  | S-band | X-band |  |
| 1 | Navigational aids etc. –without radar reflector. Small open boats, fiberglass, wood or rubber with outboard motor and at least 2 persons onboard, small speedboats, small fishing vessels, small sailing boats and the like. |  |  |  |  | 1 m2 | 1 m ASL |
| 2 | Inshore fishing vessels, sailing boats, speedboats and the like. |  |  | X |  | 3 m2 | 2 m ASL |
| 3 | Navigational aids with radar reflector. |  | X | X | 4 m2 | 10 m2 | 3 m ASL |
| 4 | Small metal ships, fishing vessels, patrol vessels and the like. | X | X | X | 40 m2 | 100 m2 | 5 m ASL |
| 5 | Coasters and the like. | X | X | X | 400 m2 | 1.000 m2 | 8 m ASL |
| 6 | Large coasters, bulk carriers, cargo ships and the like. | X | X | X | 4.000 m2 | 10.000 m2 | 12 m ASL |
| 7 | Container carriers, tankers etc. | X | X | X | 40.000 m2 | 100.000 m2 | 18 m ASL |

Table 1. Types of targets to be detected

RADAR DETECTION PERFORMANCE AND DISTURBANCES

When selecting a radar system, attention should be paid to the fact that the target types defined by the VTS authority should enable reliable detection even in demanding weather and environmental conditions, in visibility of 5 cabels or rain up to 10 mm/hr and in sea conditions of sea state 1. The system should be designed in such a way that the defined target types could be detected reliably in the entire area covered by the VTS service.

Factors disturbing and restricting the performance of radar systems include noise as well as interference and clutter signals from various sources. Every radar should be designed and equipped with devices to reduce the adverse effects of rain and sea clutter and enhance the probability of target detection per scan. The radar should also be designed and installed so as to eliminate to the maximum extent possible, false echoes caused by side lobes or reflections from nearby structures. When selecting a radar system and the measuring frequency, regional special conditions such as heavy rainfall should be taken into account.

There are several technological solutions available to raise the performance of the radar system to the required level. Most typical solutions include higher average transmission power, larger antennas, the use of circular polarization, reducing the receiver noise and more effective processing and filtering of the transmitted and received signal.

RADAR ACCURACY AND DISCRIMINATION

The measuring accuracy and discrimination of the system is determined by the VTS authority based on recommendation levels. The recommendations for target separation for different service levels are listed in Table 2.

| Radar accuracy and physical separation between small point targets for discrimination in display and tracking |  |  | Recommendation level |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relaxed |  | Standard |  | Extended |  |
|  |  |  | Display | Tracking | Display | Tracking | Display | Tracking |
| In range |  | Short range applications (<5 NM coverage – include waterways, harbors etc) | 25 m | 40 m | 20 m | 30 m | 15 m | 25 m |
|  |  | Long range applications (up to 20 NM coverage – littoral waters, offshore etc) | 75 m | 100m | 60 m | 75 m | 50 m | 60 m |
|  |  | Very long range applications (>20 NM coverage) | N/A |  | 100 m | 125 m | 80 m | 100 m |
| In azimuth | X-band | Angle between targets as seen from the radar | 1.2o | 1.3o | 0.7o | 0.8o | 0.55o | 0.6o |
|  |  | Or distance in meters, whichever is the greater | 25 m | 40 m | 20 m | 30 m | 15 m | 25 m |
|  |  | Corresponding –3 dB antenna horizontal beam width | 0.6 - 0.7o |  | 0.4 - 0.45o |  | 0.35 - 0.4o |  |
|  | S-band | Angle between targets as seen from the radar | N/A |  | 3.5o | 4o | 1.8o | 2o |
|  |  | Or distance in meters, whichever is the greater | N/A |  | 20 m | 30 m | 15 m | 25 m |
|  |  | Corresponding –3 dB antenna horizontal beam width | N/A |  | 1.8 - 2o |  | 1 - 1.25o |  |

Table 2. Target Separation and accuracy

The system should be designed in such a way that the defined radar accuracy and discrimination can be achieved in the entire area covered by the VTS service. In long measuring distances, the impact of the height and type of antenna on the measuring accuracy and resolution should be taken into account.

PLOT EXCTRACTION AND TRACKING

Plot extraction shall be automatic. Track initiation should be automatic, automatic except selected areas, automatic in selected areas or manual depending on the concept of operations. The maximum number of confirmed tracks per each individual radar sensor and other tracking performance parameters are defined by local VTS authority based on traffic density and types of tracks.

RELIABILITY AND AVAILABILITY

The requirements for the availability of radar service shall defined  by the VTS authority. The recommendations for availability listed in Table 3.

| Availability and redundancy for individual radar sites |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Recommendation level |  |  |  |  |  |
|  |  | Relaxed |  | Standard |  | Extended |  |
| Recommended availability for the individual radar site |  | 99 % |  | 99,6 % |  | 99,9 % |  |
| Accessibility to site (maintenance) |  | Easy | Difficult | Easy | Difficult | Easy | Difficult |
| Redundancy or overlap in coverage typically required | Yes |  |  |  | X | X | X |
|  | No | X | X | X |  |  |  |

Table 3. Availability and redyndancy