# LONG RANGE IDENTIFICATION AND TRACKING (LRIT)

# A VTS PERSPECTIVE

# BACKGROUND

At the 34th IALA Council meeting in May 2004, the Strategy Group tasked the AIS Committee to review the AIS long range tracking ability and report to Council (VTS 21/4/1 refers).  In response to this, the AIS Committee produced an output paper (AIS 15 Output 04).

In preparation for the IALA Seminar on Long Range Tracking, the VTS Committee took the opportunity to review the following aspects of long-range identification tracking:

- The economic, safety and environmental impacts of LRIT (not limited to long range AIS).

- The impact of LRIT on VTS’ operations and personnel.

- Security considerations of LRIT

- Evaluate technologies that may be used for LRIT.

Fundamental to any debate on LRIT is a clear appreciation as to what is meant by “Long Range”.  The draft SOLAS Amendment for LRIT proposes that access to LRIT information should be worldwide for Flag States, Coastal states up to 2000 miles, and Port States as dictated by consideration of port entry.  For the purpose of this document, the Committee has assumed ‘long range’ to be outside of VHF coverage.

# DISCUSSION

In addition to serving the traditional demands for safety and efficiency of traffic management, VTS are now being increasingly used to respond to offshore environmental concerns and, more recently, to maritime security and monitoring concerns.  The potential impact of LRIT on some aspects of VTS functionality is discussed below.

In the absence of well-defined user requirements, it is difficult to craft technical requirements for LRIT.  However, there is widespread support for the minimum transmitted message – position, identification, and time of position.  Existing equipment (e.g., AIS) may already generate this information (and more) and could be used to supply this long-range identification and tracking message.  To reiterate, it was suggested that the AIS may be used to generate an LRIT message, but this message should comprise the minimum data elements.  It was also suggested that regular reporting of ships was necessary to make LRIT an effective tool for VTS as well as security interests.

# Economic, safety and environmental impact

In many cases, LRIT may be the only method for a coastal state to monitor the transit of vessels within their territorial waters or EEZ.  The following additional benefits are also perceived:

- Knowledge of the presence and movement of shipping could assist in responses to SAR incidents, medical evacuations at sea.

- LRIT could also provide a near real time database, which can be drawn upon to augment national ship reporting systems or provide early warning of the imminent entry of vessels into such systems.

- LRIT information may be used to enforce compliance with national and international regulations, assist with EEZ management, monitor PSSA’s and compliance with traffic routeing measures.  This information could also serve as a deterrent for harm to the environment and assist in pollution investigations.

- LRIT reports may have benefits for traffic planning, port traffic management and pilot scheduling, however these rely more on precise and confirmed times of arrival rather than long range positional information.

# Impact on VTS operations and personnel

- Where the provision of VTS is a national responsibility, designated VTS centres may well be the first recipients of LRIT information, and could therefore make a valuable contribution as a distribution centre providing data to allied services such as those responsible for maritime security, safety and environmental protection.

- In  order that such services do not impact negatively on core VTS functions, data inflow would have to be automated (and its redistribution seamless), in accordance with ‘need-to-know’ and ‘confidentiality’ rules, as set by appropriate national or regional authorities.

- Tasking a VTS centre with processing LRIT information is likely to have resource and training implications.

- Monitoring LRIT reports could require displays specifically dedicated for this purpose.  If these are to be located at a VTS centre, care must be taken that the associated procedures do not adversely affect core VTS operations.

# Security considerations

- Consideration should be given to the need to ensure the security of any communications device used to provide LRIT information.

- Security organisations are likely to benefit significantly from LRIT data.

- The locations selected for receiving LRIT information may also be the recipients of an SSAS (Ship Security Alert System) message.

# Technologies that may be used

- A table summarizing some of the technologies presently available for LRIT purposes is attached at Annex A.

- A variety of existing shipboard systems may be used for generating this information.  For example, Inmarsat-C could be used for vessel polling.  Inmarsat-C GMDSS polling capability is based on Data Network Identifier (DNID) downloads from the polling entity to a polled ship.  The DNID capacity is limited to 256 mobile earth stations per DNID. However, Inmarsat has expressed interest in developing a “global DNID” with unlimited capacity, and this should be encouraged.

- LRIT reports are likely to be achieved most conveniently by polling techniques.

- Tracking systems for commercial and other purposes (e.g. vessel monitoring services for fisheries management and fleet management) already exist, and in some cases already act as a LRIT system.  Consideration may need to be given to incorporating these existing databases into the overall arrangements made for the provision of LRIT data.

# CONCLUSION

The VTS Committee concludes that, in addition to security, LRIT has economic, safety and environmental protection benefits.

The VTS Committee further concludes that, where VTS is a national responsibility, designated VTS centres may perform the LRIT role.  However, care must be taken that such an arrangement does not impact adversely on core VTS operations.  The associated resource, procedural, training and security implications will also need to be addressed.

Clear understanding of what is meant by ‘Long Range’ and LRIT, including its operational requirements, is needed.

Given that many systems could contribute to the provision of LRIT information, an organisational structure for the secure reception and dissemination of data needs to be developed.  This structure could also include cost recovery mechanisms.

The Committee recognizes that the issue is still under active consideration within IMO.  However, the outcomes of the IALA sponsored Long Range Seminar in November 2004 are likely to assist in clarifying any potential role for IALA.

The views of the VTS Committee and the outcomes of the IALA Long Range Seminar should be reflected in an IALA input to COMSAR 9 (Feb 2005).

# REFERENCE

VTS 20- 15 INFO 1:    IALA and Security

## ANNEX A*  Summary of Technologies Presently Available for LRIT

| PARAMETER | ORBCOMM | IRIDIUM | GLOBALSTAR | INMARSAT |
| --- | --- | --- | --- | --- |
| Orbit | Polar | Polar | Polar | Geosynchronous |
| Frequency range | 137MHz 150MHz | 1.6GHz | 1.6GHz 2.5GHz | 1.5GHz 1.6GHz |
| Gateway topology | Store and forward | Space switched | Earth switched | Earth switched |
| Coverage | Global | Global | Requires earth station and mobile within single satellite coverage | Global except for polar regions |
| Data rate per terminal | 2.4kb/s | 9.6kb/s | 9.6kb/s | 64kb/s (New M4) |
| Data cost per kilobit (50k transfer) | ~$0.10 per kilobit | ~$0.20 per kilobit | ~$0.20 per kilobit | ~$0.11 per kilobit |
| Estimated cost of terminal | $500 | $1,000 | $1,500 | $8,000 (New M4) |
| Data interface | AT Hayes at terminal | AT Hayes at terminal E-mail at earth station | AT Hayes and TCP/IP at terminal TCP/IP at earth station | AT Hayes and TCP/IP at terminal TCP/IP at earth station |
| Antenna system | Omni directional VHF monopole | Omni directional microwave monopole | Omni directional microwave monopole | Tracking antenna on mobile equipment |
| Primary constraints | - Low data rates - Data only - Network latencies - Potential on ship interference issues | - Low data rates - E-mail return on earth segment | - Limited coverage | - Complex antenna equipment - M4 said to be power hungry |
| Primary advantages | - Simple interface - Low cost - Global coverage - Low power consumption | - Simple interface - Data and voice - Low cost - Global coverage | - Simple interface - Data and voice - Terminals have GSM and satellite capability | - High data rates - Data and voice - Near global coverage |

*Input to AIS 15 (Sept 2004).  AIS 15/12/1 refers