Input paper:  ENG21-3.1.1.2

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					X  Input

□  DTEC	□  VTS							□  Information

Agenda item 			n.n

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			Joaquín Alarcón from PORT OF BARCELONA

# Introduction to ‘Virtual Floor’ Mooring System

# Summary

This document provides an initial introduction to a specific mooring system, used by Port of Barcelona during more than 15 years, internally called ‘Virtual Floor Mooring’, which it’s useful in different conditions, specially to deploy buoys in protected seabed areas or when a shorter swinging radius is required.

## Purpose of the document

In response to interest in this mooring system during the revision of G1066 The Design of Floating Aids to Navigation Mooring, and taking into account its likely usefulness in several conditions or requirements like described before, the Committee is requested to consider including this information as a part of the referred Guideline, or as a new document avoiding to overload the existing G1066, due to there are a lot of questions to take into account to deploy this mooring system.

A short presentation it’s also proposed to share whit all members during this Committee season.

## Related documents

IALA Guideline G1066 The Design of Floating Aids to Navigation Mooring.

# Background

During IALA ENG 20 as a part of subgroup task revising the Guideline G1066, I introduced this kind of mooring system, that we use in Port of Barcelona from more than 15 years ago, being a good solution to reduce the swinging radius in Entry Channel, deep areas or Isolated Danger buoys for example, and also in protected areas to avoid damages to the seabed. This mooring system called ‘Virtual Floor Mooring’ (internal denomination) covers several needs, and it’s used by Port of Barcelona in places that includes different depths, current, wind and waves conditions.

# Discussion

The ‘Virtual Floor’ Mooring System is divided into two main parts.

First one consists of a taut line composed by a sinker and a chain, which keeps submerged a floating element (intermediate buoy) at a defined depth (around 25 meters), generating an effect similar to elevate the seabed up to this new depth, and gives the name to this kind of mooring.

Second one is a chain line that links this virtual floor with the buoy forming a catenary.

Here you can see a representation of Channel Entry buoys of Port of Barcelona:

Figure 1: Virtual Floor Mooring elements

Figure 2: 29680 'Sierra' buoy, South Entry Channel of Port of Barcelona

In deeper locations like showed, the use of traditional mooring system could be difficult, in order to found the correct balance between resistance requirements and buoyancy limitations. This could happen not only for small buoys but also for big buoys, due to the big efforts generated to the mooring system.

Many times, different kinds of marks require to reduce the swinging radius, for example in channels, Isolated Danger marks, Entry Channel buoys, and anyplace where this factor could affect their effectivity.

In these situations, this kind of mooring allow to reduce this factor and at the same time, but don’t compromise or limit other design or operational requirements.

Here you can see some parts of this system, original images of Entry Channel buoys of Port of Barcelona:

Figure 3: Intermediate buoy

Figure 4: Mooring line element

Figure 5: Mooring line element working

Figure 6: Intermedite buoy working (-25m)

As a reference, in this specific case of ‘Sierra’ buoy, using a transitional mooring system would mean to obtain an estimated swinging radius around 116 meters, in front of around 67 meters using the Virtual Floor mooring system.

Another main advantage is to avoid damages to the seabed, because eliminates the ground or thrash chain sections of traditional mooring systems, helping to protect the marine environment in protected areas, and allowing them to perform their function while respecting this environment.

Here you can see some examples of buoys marking different dangerous points, near to protected areas with depths around 15-20m, using this system to avoid damages to the seabed:

Figure 8: 31135 Cardinal Mark

Figure 7: 30909 Isolated Danger

Figure 9: Virtual Floor Mooring detail

# References

- None

# Action requested of the Committee

The Committee is requested to:

- Consider the information in this proposal.

Leave open if uncertain